3GPP TSG-RAN4 Meeting #103-e	R4-2209666

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.175 | CR | 0020 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.175: updates reflecting modifications in IEC 61000-4-3:2020 for the upper frequency range of the RI test, Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Recently, the IEC 61000-4-3 specification has been updated (IEC 61000-4-3:2020), where one of the modifications introduced was the removal of the upper frequency limit of the RI requirements. RAN4 EMC specifications are referring to the 61000-4-3 and reusing the upper frequency limit for the RI testing up to 6 GHz, so far. Therefore based on the IEC 61000-4-3:2020 content, the related 6GHz limit for the Radiated Immunity testing shall be removed from RAN4 EMC specifications. Related discussion paper was provided in R4-2209668. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of the upper frequency limit of 6 GHz for the Radiated Immunity testing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect upper frequency limit of 6 GHz and misleading testing limitation would be kept for the RI test, not aligned with the external IEC 61000-4-3:2020 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1, 4.4.2, 7.2, 9.2, 9.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.113, TS 38.114, TS 38.124 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified sections ------------------------------

## 4.4	Exclusion bands

### 4.4.1	Transmitter exclusion band

The transmitter exclusion band for IAB is the frequency range over which no tests of radiated immunity of a transmitter are made. As the IAB node may operate its access and backhaul link in different NR IAB operating band, the transmitter exclusion band for IAB applies separately for the access and backhaul link.

The transmitter exclusion band is defined as:

FDL,low – ΔfOBUE <f < FDL,high + ΔfOBUE

Where:

-	Values of FDL,low and FDL,high are defined for each NR IAB operating band in TS 38.174 [2], clause 5.2.

-	The value of ΔfOBUE is derived considering the width of the NR IAB operating band, and is defined as in table 4.4.1-1.

Table 4.4.1-1: Maximum offset ΔfOBUE outside the downlink operating band for IAB-DU

| IAB-DU type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| IAB-DU type 1-O | FDL,high – FDL,low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL,high – FDL,low ≤ 900 MHz | 40 |
| IAB-DU type 2-O | FDL,high – FDL,low ≤ 4000 MHz | 1500 |

Table 4.4.1-2: Maximum offset ΔfOBUE outside the uplink operating band for IAB-MT

| IAB-MT type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| IAB-MT type 1-O | FUL,high – FUL,low < 100 MHz | 10 |
|  | 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz | 40 |
| IAB-MT type 2-O | FUL,high – FUL,low ≤ 4000 MHz | 1500 |

### 4.4.2	Receiver exclusion band

The receiver exclusion band for IAB is the frequency range over which no tests of radiated immunity of a receiver are made. As the IAB node may operate its access and backhaul link in different NR IAB operating band, the receiver exclusion band for IAB applies separately for the access and backhaul link. The receiver exclusion band applies to IAB type 1-O.

The receiver exclusion band is defined as:

FUL,low – ΔfRX <f < FUL,high + ΔfRX

Where:

-	Values of FUL,low and FUL,high are defined for each NR IAB operating band in in TS 38.174 [2], clause 5.2.

-	The value of ΔfRX is derived considering the width of the NR IAB operating band, and is defined as in table 4.4.2-1. Value of the ΔfRX also depends on the RI test setup, i.e. whether or not the spatial exclusion zone (as described in subclause 9.2.2) is considered during the RI test.

Table 4.4.2-1: ΔfRX offset values for IAB

| IAB type | IAB operating band characteristics | RI test setup | ΔfRX (MHz) |
| --- | --- | --- | --- |
| IAB type 1-O | FUL,high – FUL,low < 100 MHz | With exclusion zone | 20 |
|  |  | Without exclusion zone | 60 |
|  | 100 MHz FUL,high – FUL,low 900 MHz | With exclusion zone | 60 |
|  |  | Without exclusion zone | 200 |

NOTE:	there is no receiver exclusion band defined for IAB type 2-O.

------------------------------ Next modified section ------------------------------

## 7.2	Immunity

Table 7.2-1: Immunity requirements applicability

| Phenomenon | Application | Equipment test requirement |  | Reference clause | Reference |
| --- | --- | --- | --- | --- | --- |
|  |  | IAB equipment | Ancillary equipment | in the present document | standard |
| RF electromagnetic field | Enclosure | applicable | applicable | 9.2 | IEC 61000-4-3 [13] |
| Electrostatic discharge | Enclosure | applicable | applicable | 9.3 | IEC 61000-4-2 [12] |
| Fast transients common mode | Signal, telecommunications and control ports, DC and AC power input ports | applicable | applicable | 9.4 | IEC 61000-4-4 [14] |
| RF common mode 0.15 - 80 MHz | Signal, telecommunications and control ports, DC and AC power input ports | applicable | applicable | 9.5 | IEC 61000-4-6 [16] |
| Voltage dips and interruptions | AC mains power input ports | applicable | applicable | 9.6 | IEC 61000-4-11 [17] |
| Surges, common and differential mode | AC power input ports and telecommunications port | applicable | applicable | 9.7 | IEC 61000-4-5 [15] |

------------------------------ Next modified section ------------------------------

## 9.2	RF electromagnetic field

The test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

### 9.2.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to operate as intended in the presence of a radio frequency electromagnetic field disturbance at the enclosure.

### 9.2.2	Test method and level

The test method shall be in accordance with IEC 61000-4-3 [13]. The use of reverberation chamber test method according to IEC 61000-4-21 [18], clause 6.1 and Annex D as alternative method is allowed.

-	For transmitters, receivers and transceivers the following requirements shall apply:

-	The test level shall be 3 V/m amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	The stepped frequency increments shall be 1 % of the momentary frequency;

-	The test shall be performed at fixed frequency test steps starting from 80 MHz, as defined in IEC 61000-4-3 [13]; with the exception of the exclusion band for receivers (see clause 4.4);

-	Responses in stand-alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see clause 4.3;

-	The frequencies selected during the test shall be recorded in the test report.

-	For the test method in accordance with IEC 61000-4-3 [13], the spatial exclusion zone can be chosen to protect the IAB node receiver(s). For the frequency arrange above 690 MHz (according to the test method in ETSI EN 301 489-50 [28]), the EMC RF electromagnetic field immunity requirement applies on the non-radiating faces of the IAB type 1-O, or IAB type 2-O, as depicted on figures 9.2.2-1.

NOTE:	Depending on the IAB implementation, application of the spatial exclusion to all radiating faces of the IAB may not allow proper execution of the RI testing. In such cases as depicted on figures 9.2.2-2, to protect the IAB node receiver(s), exclusion bands shall be considered, as in table 4.4.1-1.

Figure 9.2.2-1: Example of the spatial exclusion application in case of a single instance of the IAB radiating face (horizontal plane depicted)

Figure 9.2.2-2: Example of the spatial exclusion application in case of a 3-panel IAB node (horizontal plane depicted) – proper execution of the RI test may not be feasible

------------------------------ End of modified section ------------------------------