3GPP TSG-RAN WG4 Meeting #103-e	R4-2209693

Electronic Meeting, 9 May – 20 May 2022

Agenda Item:	9.11.2.3.1

Source:	Ericsson

Title:	Discussion on 30kHz SCS scenario for CRS-IM

Document for:	Discussion

# 1	Introduction

After the last RAN4 meeting’s discussion, it was still undermined whether to define CRS-IM requirements for 30kHz SCS scenario. Following agreements have been reached to move forward:

Whether to define CRS-IM requirements for 30 kHz SCS scenario - Only define CRS-IM requirements for scenario with 30 kHz SCS assuming [10%] interference loading, 4 CRS ports and 1+1 DMRS configuration under the condition with enough performance discrimination between CRS-IM on and CRS-IM off i.e. at least 1dB performance difference observed - Channel BW: 20MHz - FFS on special slot configuration - Interested companies can bring results with Rel-15 rate matching (symbol level)

According to the above agreements from the WF [1], whether to have such requirement depends on whether there is at least 1dB performance difference observed from the simulation results.

In this contribution, we provided our simulation results for 30kHz SCS scenario, and gave our observation of these results.

# 2	Discussion

## 2.1 Simulation assumptions

Table 2.1-1 Simulation assumption for 30kHz SCS scenario:

| Parameters | Value |
| --- | --- |
| CBW and SCS | TDD 30kHz/20MHz for target cell and TDD 15kHz/20MHz for interference cells |
| Interference loading | 10% |
| CRS ports for interference cells | 4 CRS ports |
| TDD pattern for target cell | 7DS2U with S=6D+4G+4U |
| DMRS configuration | 1+1 |
| Other parameters | Reuse the parameters from 15kHz Scenario |

Note: Companies are encouraged to provide simulation results with and without CRS-IM in next meeting

## 2.2 Test cases and results

Here are the test cases we have simulated:

Table 2.2-1 Test cases for 30kHz SCS scenario

| Test case number | Configurations | PDSCH loading | Gain |
| --- | --- | --- | --- |
| 1 | MCS 13 with 4x2 ULA low | 0% | 0.92 |
| 2 |  | 10% | 1.19 |
| 3 |  | 20% | 1.26 |
| 4 |  | 30% | 1.68 |
| 5 |  | 100% | 1.25 |

Here are the simulation results:

Figure 2.2-1 Simulation results for TDD 30kHz SCS for different loading levels

From the simulation results we can observe that there is a 1.19dB gain from LLR weighting over the reference at PDSCH loading level 10%, which achieves at least 1dB performance difference set by the WF [1]. In this case, we are positive on defining requirements for TDD 30kHz SCS scenario.

Observation 1: There is a 1.19dB gain from LLR weighting over the reference at PDSCH loading level 10%.

Proposal 1: Define requirements for TDD 30kHz SCS scenario.

Proposal 2: Consider following test cases for defining requirements:

| Test case | MCS | Channel | PDSCH loading level | Antenna configuration |
| --- | --- | --- | --- | --- |
| 1 | MCS13 | TDLA30-10 | 10% | 4x2 |
| 2 |  |  |  | 4x4 |

# 3	Summary

Here, we summarize our proposals:

Observation 1: There is a 1.19dB gain from LLR weighting over the reference at PDSCH loading level 10%.

Proposal 1: Define requirements for TDD 30kHz SCS scenario.

Proposal 2: Consider following test cases for defining requirements:

| Test case | MCS | Channel | PDSCH loading level | Antenna configuration |
| --- | --- | --- | --- | --- |
| 1 | MCS13 | TDLA30-10 | 10% | 4x2 |

# References

[1]	R4-2207240  WF on CRS-IM in scenarios with overlapping spectrum for LTE and NR with 30kHz SCS, CMCC