3GPP TSG-RAN WG4 Meeting #103-e	R4-2209694

Electronic Meeting, 9 May. – 20 May 2022

Agenda Item: 	9.11.2.3.1

Source:	Ericsson

Title:	Simulation results for CRS-IM

Document for:	Information

# 1	Introduction

According to the agreed WF for 15kHz SCS scenario [1], companies are encouraged to bring simulation results for 2 CRS ports to be one of the references for deciding whether to define requirements for 2 CRS ports.

In this contribution, we bring our simulation results for 2 CRS ports LLR weighting for scenario 1. Besides, we also bring our TDD simulation results for both scenarios for alignment. Our simulation results for FDD 4x2, 4x4 are shared as well, since there are some updates comparing with previous submitted results.

In the meantime, as indicated by the WF for 30kHz SCS scenario [2], companies are encouraged to bring simulation results for assuming [10%] interference loading, 4 CRS ports and 1+1 DMRS configuration, in order to see if there is enough performance discrimination between CRS-IM on and off. i.e. at least 1dB performance difference observed. Thus, we also bring our simulation results for 30kHz SCS scenario for information.

# 2	Discussion

## 2.1 15kHz SCS scenario

## 2.1.1 FDD

Summary:

Table 1 Test cases for 15kHz SCS scenario FDD

| Case Number | Scenario | MIMO | SNR @ 70% Max TP | Gain over the reference |
| --- | --- | --- | --- | --- |
| Case 1 | Scenario 1 | 2Tx 2Rx Low | 8.3dB | 1.1 |
| Case 2 |  | 2Tx 4Rx Low | 4.0dB | 1.4 |
| Case 3 |  | 4Tx 2Rx Low | 8.1dB | 0.6 |
| Case 4 |  | 4Tx 4Rx Low | 4.1dB | 0.6 |
| Case 5 | Scenario 2 | 4Tx 2Rx Low | 7.7dB | 0.6 |
| Case 6 |  | 4Tx 4Rx Low | 3.7dB | 0.6 |

## 2.1.2 TDD

Summary:

Table 2 Test cases for 15kHz SCS scenario TDD

| Case Number | Scenario | MIMO | SNR @ 70% Max TP | Gain over the reference |
| --- | --- | --- | --- | --- |
| Case 1 | Scenario 1 | 2Tx 2Rx Low | 9.6dB | 0.8 |
| Case 2 |  | 2Tx 4Rx Low | 5.2dB | 0.8 |
| Case 3 |  | 4Tx 2Rx Low | 10.6dB | 0.5 |
| Case 4 |  | 4Tx 4Rx Low | 6.9dB | 0.4 |
| Case 5 | Scenario 2 | 4Tx 2Rx Low | 7.1dB | 0.8 |
| Case 6 |  | 4Tx 4Rx Low | 3.3dB | 0.8 |

## 2.2 30kHz SCS scenario

Summary:

Table 3 Test cases for 30kHz SCS scenario FDD

| Case Number | Scenario | Interference cell loading level | CBW & SCS | MIMO | Gain |
| --- | --- | --- | --- | --- | --- |
| Case 1 | Scenario 2 | 10% | 20MHz, 30kHz | 4Tx 2Rx Low | 1.19dB |

# 3	Summary

In this contribution, we provided our simulation results for the TDD/FDD for 15kHz SCS scenario and FDD for 30kHz SCS scenario.

# References

[1]	R4-2207239 WF on general part and 15 kHz NR SCS scenario for CRS-IM receiver, China Telecom

# Annex

FDD/TDD 15kHz SCS Scenario

Table 1. NR Cell Parameters

| Parameters | Serving cell FDD |
| --- | --- |
| Network scenario | Sync |
| Number of carriers | single carrier |
| Carrier frequency | 2 GHz |
| PDSCH allocation in frequency domain | Full PRB |
| Physical cell ID | 0 |
| Active DL BWP index | 1 |
| Channel bandwidth | 10MHz |
| TDD configuration | N/A |
| SCS | 15kHz |
| PDCCH allocation | Scenario 1: symbols #2 Scenario 2: symbols #0 and #1 |
| SSB position in burst | SSB index #0 |
| SSB periodicity | 20ms |
| TRS configuration | k0=0 for CSI-RS resource 1,2,3,4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 Density 3, 1 port Periodicity 20 slots Offset: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| CSI-RS configuration | Not necessary of PDSCH Demod |
| PRB bundling size for target PDSCH | 2 |
| Antenna configuration | 2x2, 2x4, 4x2, 4x4 for scenario 1 4x2, 4x4 for scenario 2 |
| Rank | 1 |
| Overhead for TBS determination | 18 |
| PDSCH DMRS ports | port 0 ~3 |
| Maximum number of OFDM symbols for DL front loaded DMRS | 1 |
| Number of additional DMRS | 1 |
| MCS | 16QAM, MCS 13 |
| Precoding granularity | 2 |
| PRB bundling size | 2 |
| PDSCH precoder | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination |
| Antenna Correlation | ULA Low |
| Propagation condition | TDLA30-10 |
| Time Offset relative to serving cell | 0 us |
| Traffic model | Full buffer |
| Number of HARQ Processes | 4 |
| Maximum HARQ transmission | 4 |

Table 2. NR PDSCH Parameters

| PDSCH configuration | Mapping type | Type A |
| --- | --- | --- |
|  | k0 | 0 |
|  | Starting symbol (S) | Scenario 1: 3 Scenario 2: 2 |
|  | Length (L) | Scenario 1: 9 Scenario 2: 12 |
|  | PDSCH aggregation factor | 1 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
|  | Resource allocation type | Type 0 |
|  | RBG size | Config2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Position of the first DM-RS for downlink | 3 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 |

Table 3. LTE Interf. cell parameters

|  | Cell 0(DSS) | LTE Interf. cell 1 | LTE Interf. cell 2 |
| --- | --- | --- | --- |
| carrier centre subcarrier | Same as NR carrier centre subcarrier |  |  |
| BW(MHz) | 10 |  |  |
| Resource allocation | Random full band (50PRB) on/off model, proportional to the average resource utilization in the interfering cells; ON/OFF pattern depends on the Position distribution |  |  |
| Non-full buffer interference Model | Interfering PDSCH transmissions in interfering cells are randomly & independently active over the full band with an activity in time domain equal on average to the targeted resource utilization |  |  |
| Average resource utilization | 20% |  |  |
| CP Type | Normal |  |  |
| Time offset(us) | 0 | 3 | ​-1 |
| Frequency offset(HZ) | 0 | 300 | -100 |
| PDSCH MCS | Random 16QAM |  |  |
| Precoding | TM4 Random PMI |  |  |
| Rank | Randomly changing rank per allocated subband from subframe to subframe: 80% rank-1, 20% rank-2 |  |  |
| Number of antenna ports | 4 | 4 | 4 |
| v-shift | 0 | 1 | 2 |
| INR(dB) | / | 10.45 | 4.6 |
| Antenna Correlation | ULA Low |  |  |
| Note 1: No MBSFN is configured on LTE carrier |  |  |  |

TDD 30kHz SCS Scenario

Table 4. Specific simulation assumption for 30kHz SCS scenario:

| Parameters | Value |
| --- | --- |
| CBW and SCS | TDD 30kHz/20MHz for target cell and TDD 15kHz/20MHz for interference cells |
| NRB | 51 for target cell and 106 for interference cells |
| Interference loading | 10% |
| CRS ports for interference cells | 4 CRS ports |
| TDD pattern for target cell | 7DS2U with S=6D+4G+4U |
| DMRS configuration | 1+1 |
| Other parameters | Reuse the parameters from 15kHz Scenario |

TR 36.863 Table 7.2-1

TR 36.863 Table 7.2-1