3GPP TSG-RAN WG4 Meeting # 103-e 		R4-2209707

# Electronic Meeting, May 9 – 20, 2022

Source:	Nokia, Nokia Shanghai Bell

Title: 	Simulation results for Redcap PDSCH

Agenda item:	9.19.5.2.1

Document for:	Discussion

Introduction

The last eMeeting’s discussion and WF of the UE demodulation and CSI requirements for RedCap topic are captured in the summary [1] and WF [2].

In this contribution, we present a summary of first PDSCH simulation results. Please note that these initial results may be subject to changes in the coming meetings.

PDSCH simulation results

The simulation setup and assumptions have been slightly updated after the last meeting, and our results below conform to the file “REV_R4-2207206 WF RedCap UE demod.docx” as shared on the reflector by the moderator.

PDSCH simulation results summary

Simulation results are reported in this section for simulation assumptions contained in the (revised reflector version of the) WF [2].

FDD 15kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | New | 0.8 |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New | 9.2 |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | New | 13.9 |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | Reuse | -2.6 |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New | 5.0 |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | Reuse | 17.6 |

TDD 30kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New | 0.8 |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New | 9.8 |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New | 13.8 |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New | -2.6 |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New | 4.9 |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New | 18.2 |

TDD 120kHz FR2

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-3 Test 1-1 | Reuse | -2.6 |
| 2Rx UE | 100MHz / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-1 | TDLA30-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-2 | Reuse | 11.3 |
| 2Rx UE | 100MHz / 120kHz | 64QAM 0.46 Rank 2 | FR2.120-2 | TDLA30-75 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-6 | Reuse | 15.3 |

Conclusion

In this contribution we have provided a summary of first PDSCH simulation results for agreed scenarios in the WF [2].

We have not made observation and proposals.

- References

- R4-2207451 Email discussion summary for [102-e][331] NR_RedCap_Demod, Ericsson

- R4-2207206 WF on RedCap demodulation and CQI reporting requirements, Ericsson