3GPP TSG-RAN WG4 Meeting # 103-e 		R4-2209713

Electronic Meeting, May 9 – 20, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on BS demodulation requirements for Additional enhancements for NB-IoT

Agenda item:	10.8.6.2.2

Document for:	Discussion

Introduction

The Rel-17 work item for additional enhancements for NB-IoT and LTE-MTC was approved at TSG RAN#88-e with latest WID revision [1]. At RAN4 #102-e, the technical discussion on demodulation requirements for this WI was continued. The WID1 [1] lists following objective for NB-IoT involving RAN4 effort:

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4] - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4] - Specify necessary performance requirements, measurement accuracy requirements and test cases related to the above-mentioned enhancements and core requirements.

Based on RAN4 #102-e discussions [2] and the approved WF [3], this contribution provides our view on BS demodulation requirements for NB-IoT 16QAM.

Discussion

Regarding BS demodulation requirements, requirements for NPUSCH format 1 with 16QAM need to be defined as agreed at RAN4 #101bis-e [4].

NPUSCH format 1 16QAM requirements

Following agreements were achieved at RAN4 #102-e and following remaining open issues were observed [3]:

Number of allocated subcarriers - Option 1: 3, 6, 12 tones - Option 2: 12 tones - Option 3: 3 tones Number of scheduled RUs and TBS - Option 1: (IRU, ITBS)=(0,15), TBS=280 bits - Option 2: (IRU, ITBS)=(0,14), TBS=256 bits RV: {0, 2, 0, 2} Max number of HARQ transmission: 4

Simulation Results

Link-level simulations have been performed based on the agreed simulation assumptions in [3] with real channel and noise estimation. For TBS size, 256 bits and 280 bits were simulated, each for 3, 6 and 12 tones.

Simulation results for SNR required at 70% of maximum throughput are summarized in Table 1.

Table 1: SNR at 70% of maximum TP of proposed new FRCs for 16QAM in NB-IoT uplink

| Reference channel | 3 tones / 256 bits | 3 tones / 280 bits | 6 tones / 256 bits | 6 tones / 280 bits | 12 tones / 256 bits | 12 tones / 280 bits |
| --- | --- | --- | --- | --- | --- | --- |
| SNR [dB] @TP=70%, ETU1, with HARQ | 4.92 | 5.48 | 5.46 | 6.04 | 5.06 | 5.67 |

It can be seen from Table 1, that the required SNR figures, as observed in the simulations, are quite close to each other, roughly between 5 and 6 dB. There is a difference of about 0.5 dB in average between both TBS’s and a maximum difference of 0.5 dB between different number of tones for each TBS.

The required SNR figures, as observed in the simulations, are quite close to each other, roughly between 5 and 6 dB. There is a difference of about 0.5 dB in average between both TBS’s and a maximum difference of 0.5 dB between different number of tones for each TBS.

Regarding selection of TBS for NPUSCH 16QAM performance requirements, as commented at RAN4 #102-e, we have a preference to use a TBS of 280 bits versus 256 bits, since better corresponding to the expected higher SNR operating point for NB-IoT 16-QAM in general. Thus, performance requirements for 12 tones and TBS of 280 bits should be specified.

To cover deployments with lower number of tones, e.g. cell-edge scenarios as depicted in [5], we propose to define performance requirements also for 3 tones with the same TBS of 280 bits. We can then agree to omit performance requirements for 6 tones.

The following proposal is made.

Define new uplink FRCs for 16QAM NPUSCH format 1 demodulation requirements in TS 36.104, based on TBS of 280 bits, for 3 and 12 tones only and according to simulation assumptions in R4-2207200.

Conclusion

This contribution has provided our view on BS demodulation core requirements for NB-IoT 16QAM, as part of the Rel-17 NB-IoT feature [1].

Following observation was made:

- The required SNR figures, as observed in the simulations, are quite close to each other, roughly between 5 and 6 dB. There is a difference of about 0.5 dB in average between both TBS’s and a maximum difference of 0.5 dB between different number of tones for each TBS.

Following proposal is submitted for discussion:

- Define new uplink FRCs for 16QAM NPUSCH format 1 demodulation requirements in TS 36.104, based on TBS of 280 bits, for 3 and 12 tones only and according to simulation assumptions in R4-2207200.

References

- RP-211340, WID revision: Additional enhancements for NB-IoT and LTE-MTC, Huawei, HiSilicon, RAN #92e

- R4-2207453, Email discussion summary for [102-e][333] NB-IOT_MTC_Demod_NWM, Moderator (Huawei, HiSilicon), RAN4 #102-e

- R4-2207200, WF on Rel-17 NB-IOT and eMTC performance requirements, Huawei, RAN4 #102-e

- R4-2203041,	WF on Rel-17 NB-IOT and eMTC performance requirements, Huawei, RAN4 #101bis-e

- R4-2203549, View on demodulation requirement for Rel-17 NB-IoT, Samsung, RAN4 #102-e