3GPP TSG-RAN WG4 Meeting # 103-e                	R4-2209737

Electronic Meeting, 9-20 May., 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PDSCH simulation results for the extension to 71 GHz

Agenda item: 	9.15.10.2.1

Document for: 	Discussion

Introduction

This paper presents PDSCH simulation results for the definition of demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Simulation assumptions

Simulation assumptions are shown below and are derived considering the agreements on the WF [1].

Table 1 PDSCH Simulation Assumptions

| Parameter | Value |
| --- | --- |
| Scenarios | SA FR2-2 |
| Center Frequency | 70GHz |
| SCS and BW settings | 120 kHz: 100, 400 MHz 480 MHz: 400 MHz, 1600 MHz 960 kHz: 400 MHz, 2000MHz |
| TDD Patterns | 120 kHz: 3D1S1U, S=10D:2G:2U 480 kHz: 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U 960 kHz: 29D3S8U, S1=10D:4G:0U, S2=0D:12G:2U, S3=0D:0G:14U |
| TDL channel model | TDL-A |
| combinations on {SCS, RMS Delay spread} | 120kHz: 10ns 480kHz: 10ns/5ns 960kHz: 10ns/5ns |
| Doppler frequency / FO | 3 km/h UE speed (200 Hz ). 10 km/h (650 Hz) |
| Phase Noise | Without phase noise With phase noise PN model set 1 and 2 in TR 38.808 |
| CP | Normal CP |
| Max modulation order | Up to 64QAM |
| PRB numbers (temporary) | 120kHz, 100MHz: 66 120kHz, 400MHz: 264 480kHz, 400MHz: 66 480kHz, 1600MHz: 264 960kHz, 400MHz: 32 960kHz, 2000MHz: 165 |
| Mapping type | Type A |
| MCS | MCS 4, MCS 13, MCS 20, MCS 22 |
| PTRS Configuration | K=2, L=1 |
| Rank | 1 |
| Correlation matrix and antenna configuration | 2x2 ULA Low |
| Phase noise compensation | PT-RS based CPE |
| PDCCH Frequency | Every slot |

PDSCH Simulation Results

Table 2 Results for PDSCH

| Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition |  | No PN | PN set1 | PN set2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) | SNRBB (dB) | SNRBB (dB) |
| 100 / 120 | MCS 13 | FR2.120-1 | TDLA10-200 | 70 | 7.71 | 7.92 | 8.04 |
|  |  |  | TDLA10-650 |  | 7.97 | 8.17 | 8.31 |
| 400 / 120 | MCS 20 | FR2.120-1 | TDLA10-650 | 70 | 13.51 | 14.31 | 15.57 |
| 400 / 120 | MCS 22 | FR2.120-1 | TDLA10-650 | 70 | 15.61 | 16.86 | 20.44 |
| 400 / 480 | MCS 13 | 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U | TDLA10-200 | 70 | 7.08 | 7.26 | 7.31 |
|  |  |  | TDLA10-650 |  | 7.13 | 7.32 | 7.33 |
| 1600 / 480 | MCS 4 | 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U | TDLA10-650 | 70 | -0.38 | -0.21 | -0.20 |
| 1600 / 480 | MCS 13 | 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U | TDLA5-200 | 70 | 7.13 | 7.31 | 7.35 |
|  |  |  | TDLA10-200 |  | 6.97 | 7.17 | 7.22 |
|  |  |  | TDLA5-650 |  | 7.08 | 7.26 | 7.31 |
|  |  |  | TDLA10-650 |  | 6.95 | 7.15 | 7.21 |
| 1600 / 480 | MCS 20 | 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U | TDLA5-200 | 70 | 13.17 | 13.98 | 14.59 |
| 1600 / 480 | MCS 22 | 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U | TDLA5-200 | 70 | 15.10 | 16.38 | 17.85 |
| 400 / 960 | MCS 13 | 29D3S8U, S1=10D:4G:0U, S2=0D:12G:2U, S3=0D:0G:14U | TDLA10-200 | 70 | 7.14 |  | 7.24 |
| 2000 / 960 | MCS 4 | 29D3S8U, S1=10D:4G:0U, S2=0D:12G:2U, S3=0D:0G:14U | TDLA10-200 |  | -0.05 | 0.11 | 0.13 |
| 2000 / 960 | MCS 13 | 29D3S8U, S1=10D:4G:0U, S2=0D:12G:2U, S3=0D:0G:14U | TDLA5-200 | 70 | 7.29 | 7.42 | 7.41 |
|  |  |  | TDLA10-200 |  | 7.25 | 7.42 | 7.39 |
| 400 / 960 | MCS 20 | 29D3S8U, S1=10D:4G:0U, S2=0D:12G:2U, S3=0D:0G:14U | TDLA10-200 | 70 | 13.30 |  | 13.64 |
| 400 / 960 | MCS 22 | 29D3S8U, S1=10D:4G:0U, S2=0D:12G:2U, S3=0D:0G:14U | TDLA10-200 | 70 | 15.37 |  | 15.89 |

References

- R4-2207223, WF on demodulation performance requirements definition for 52.6 - 71 GHz, Intel Corporation