3GPP TSG-RAN WG4 Meeting #103-e                                         R4-2209740

Electronic Meeting, May 09 - May 20, 2022

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.839: Update for simultaneous Rx/Tx capability

Agenda Item:	8.31.4

Document for:	Approval

# Background

This contribution provides the TP for TR of simultaneous Rx/Tx capability based on [1-3].

# References

[1] R4-2209251, TR38.839 v0.3.0

[2] R4-2206489, TP for TR 38.839: Update for simultaneous Rx/Tx capability, Huawei, HiSilicon

[3] R4-2206479, WF on simultaneous RxTx capability, Huawei, HiSilicon

[4] R4-2204744, draft CR to TS38.101-2[R17] On Simultaneous RxTx capability for FR2 inter-band CA_n257-n259 and CA_n258-n260, ZTE

# Text Proposal

## <Start of Text Poposal>

### 5.2.1	FR1+FR1 FDD-TDD band combination

Editor note: This band combination category also includes SUL band combination where SUL band is derived from FDD band.

For FR1+FR1 FDD-TDD band combination,

- Simultaneous Rx/Tx is the default capability in FDD-TDD FR1 band combinations

- For TDD-FDD combinations, the capability is mandatory without checking if there are no MSD issues and frequency range is not adjacent to each other for the band combinations

- For TDD-FDD combinations with REFSENS degradation due to self-interference, MSD caused by harmonics/IMD, etc. should be defined. For some band combinations that are close in frequency, e.g. bands n7 and n38, other methods to mitigate interference can be considered as well. Simultaneous Rx/Tx capability for this kind of special cases could be handled separately from those having MSD issues.

- Case by case analysis is considered for FR1 FDD-TDD band combination which may have difficulty to support simultaneous Rx/Tx operation, e.g. with large MSD. If a FR1 FDD-TDD band combination is identified which cannot support simultaneous Rx/Tx operation, a note similar to FR1 TDD-TDD band combination shall be indicated in the specification, and for such operation the minimum requirements are not applicable for this band combination. Otherwise, the FR1 FDD-TDD band combination with MSD can support simultaneous Rx/Tx operation.

## <Next Change>

# 6	Specific cases for TDD-TDD band combinations

## 6.1	FR2+FR2 TDD-TDD band combination

For FR2+FR2 TDD-TDD band combinations introduced in Rel-17, it is clarified that the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between inter-band NR carriers.

Table 6.1-1: Inter-band CA operating bands in FR2

| NR CA Band | NR Band |
| --- | --- |
| CA_n257-n2591 | n257, n259 |
| CA_n258-n2601 | n258, n260 |
| CA_n260-n2611,2 | n260, n261 |
| NOTE 1: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between inter-band NR carriers in the current version of this specification. NOTE 2: The band combination was introduced in Rel-16. |  |

## <End of Text Poposal>