3GPP TSG-RAN WG4 # 103-e	R4-2209797

Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.19.5.2.1

Source:	MediaTek Inc.

Title:	Simulation results and discussion on PDSCH requirements for RedCap

Document for:	Discussion

- Introduction

In RAN4#102-e, a way forward on RedCap PDSCH requirements was agreed [1]. In this contribution, we provide our view for the remaining issue and present simulation results on PDSCH requirements for RedCap UE.

- Discussion

In the last meeting, RAN4 discussed whether to define 256QAM PDSCH requirements for RedCap UE or not.

Define 256QAM demodulation requirements or not - Option 1: Specify 256QAM demodulation requirements for FR1 only - Candidate test setup: - TS38.101-4 5.2.2.1.1-3 Test 1-3 for FDD 15kHz - TS38.101-4 5.2.2.2.1-3 Test 1-3 for TDD 30kHz - FFS the MCS for 1Rx case - Option 2: Not to specify 256QAM demodulation requirements. - Option 3: Discuss whether to specify the requirements for FR1 once we stabilize the discussion on requirements for mandatory features.

Although 256QAM is an optional feature, we think it is important to verify higher throughput for RedCap UE. Therefore, we support Option 1 to specify 256QAM demodulation requirements for FR1 only. For existing 256QAM test cases, the SNR requirement is around 24/25 dB for 2 Rx. If we consider 1 Rx UE with the same MCS, it will increase the operating SNR to a higher level. Hence, we suggest using MCS20 to define requirements for 256QAM modulation where the simulation results for FDD and TDD are shown as below.

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Reference from TS38.101-4 | SNR for 70% of max T-put, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | 256QAM 0.67 Rank 1 | TDLA 30-10 | 2x1 low | 5.2.2.1.1-3 Test 1-3 | 22.7 |

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Reference from TS38.101-4 | SNR for 70% of max T-put, [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | 256QAM 0.67 Rank 1 | FR1.30-1A | TDLB100-40022. | 2x1 low | 5.2.2.2.1-3 Test 1-1 | 22.9 |

Proposal 1: Use MCS20 to define 256QAM demodulation requirements for RedCap UE.

- Simulation Results

We provide the simulation results for FDD and TDD in Table 1 and Table 2 respectively.

Table 1. Simulation results for FDD

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Reference from TS38.101-4 Table | SNR for 70% of max T-put, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 5.2.2.1.1-3 Test 1-1 | 1.3 |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 5.2.2.1.1-3 Test 1-4 | 10.0 |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 5.2.2.1.1-4 Test 2-1 | 14.2 |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 5.2.2.1.1-3 Test 1-4 | 5.9 |

Table 2. Simulation results for TDD

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Reference from TS38.101-4 Table | SNR for 70% of max T-put, [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x1 low | 5.2.2.2.1-3 Test 1-1 | 1.4 |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 5.2.2.2.1-3 Test 1-4 | 10.1 |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 5.2.2.2.1-4 Test 2-1 | 14.4 |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x2 ULA low | 5.2.2.2.1-3 Test 1-1 | -2.2 |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 5.2.2.2.1-3 Test 1-4 | 5.9 |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 5.2.2.2.1-4 Test 2-1 | 17.3 |

- Conclusion

In this contribution, we provide simulation results for RedCap PDCSH based on the agreed simulation assumption in [1]. Also, we have a proposal about 256QAM demodulation requirements.

Proposal 1: Use MCS20 to define 256QAM demodulation requirements for RedCap UE.

- Reference

- R4-2207206, “WF on RedCap demodulation and CQI reporting requirements”, Ericsson