3GPP TSG-RAN WG4 Meeting # 103-e	R4-2209803

# Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draft | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.101-4, PDSCH requirements for 1024QAM in FR1 FDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DL1024QAM_FR1-Perf |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Demodulation performance requirements for 1024QAM in FR1 FDD needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The requirements for demodulation performance for 1024QAM in FR1 FDD are introduced in sections 5.2.2.1.1, 5.2.3.1.1 and A.3.2.1.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The demodulation performance requirements for 1024QAM in FR1 FDD are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sections 5.2.2.1.1, 5.2.3.1.1 and A.3.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

##### 5.2.2.1.1	Minimum requirements for PDSCH Mapping Type A

<SKIP UNCHANGED PART>

Table 5.2.2.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 1-2, 1-3, 1-5, 1-6, 1-7, 1-8, 2-1, 2-2 |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 2 receive antenna conditions. | 1-4 |
| Verify the PDSCH mapping Type A performance requirements for Enhanced Receiver Type 1 under 2 receive antenna conditions. | 3-1 |

Table 5.2.2.1.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Test 1-1 2 for other tests |
|  | Resource allocation type |  | Test 1-2: Type 1 with start RB = 23, LRBs = 6 Other tests: Type 0 |
|  | RBG size |  | Test 1-2: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Tests 1-1, 1-5, 1-6, 1-7 1 for other tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | Test 1-5, 1-6, 1-7: 10 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  | CSI-RS offset | Slots | Test 1-5, 1-6, 1-7: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  | 8 for Test 1-4 4 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.2.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | TDLB100-400 | 2x2, ULA Low | 70 | -0.8 |
| 1-2 | R.PDSCH.1-1.2 FDD | 10 / 15 | QPSK, 0.30 | TDLC300-100 | 2x2, ULA Low | 70 | 0.2 |
| 1-3 | R.PDSCH.1-4.1 FDD | 10 / 15 | 256QAM, 0.82 | TDLA30-10 | 2x2, ULA Low | 70 | 24.6 |
| 1-4 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 30 | 1.1 |
| 1-5 | R.PDSCH.1-8.1 FDD | 10 / 15 | 16QAM, 0.48 | HST-750 | 1x2 | 70 | 6.2 |
| 1-6 | R.PDSCH.1-8.2 FDD | 10 / 15 | 64QAM, 0.43 | HST-972 | 1x2 | 70 | [9.9] |
| 1-7 | R.PDSCH.1-8.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-600 | 2x2 | 70 | [8.6] |
| 1-8 | R.PDSCH.1-13.1 FDD | 10 / 15 | 1024QAM, 0.79 | [TBD] | 2x2, ULA Low | 70 | [TBD] |

END OF CHANGE 1

START OF CHANGE 2

##### 5.2.3.1.1	Minimum requirements for PDSCH Mapping Type A

<SKIP UNCHANGED PART>

Table 5.2.3.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 1-2, 1-3, 1-5, 1-6, 1-7, 1-8, 2-1, 2-2, 3-1, 4-1 |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 4 receive antenna conditions. | 1-4 |
| Verify the PDSCH mapping Type A performance requirements for Enhanced Receiver Type 1 under 4 receive antenna conditions. | 5-1 |

Table 5.2.3.1.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Test 1-1 wideband for Test 3-1 2 for other tests |
|  | Resource allocation type |  | Test 1-2: Type 1 with start RB = 23, LRBs = 6 Other test: Type 0 |
|  | RBG size |  | Test 1-2: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Test 1-1, 1-5, 1-6, 1-7 1 for other tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | Test 1-5, 1-6, 1-7: 10 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  | CSI-RS offset | Slots | Test 1-5, 1-6, 1-7: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  | 8 for Test 1-4, 2-1 4 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.3.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | TDLB100-400 | 2x4, ULA Low | 70 | -3.5 |
| 1-2 | R.PDSCH.1-1.2 FDD | 10 / 15 | QPSK, 0.30 | TDLC300-100 | 2x4, ULA Low | 70 | -2.9 |
| 1-3 | R.PDSCH.1-4.1 FDD | 10 / 15 | 256QAM, 0.82 | TDLA30-10 | 2x4, ULA Low | 70 | 21.0 |
| 1-4 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x4, ULA Low | 30 | -1.5 |
| 1-5 | R.PDSCH.1-8.1 FDD | 10 / 15 | 16QAM, 0.48 | HST-750 | 1x4 | 70 | 3.3 |
| 1-6 | R.PDSCH.1-8.2 FDD | 10 / 15 | 64QAM, 0.43 | HST-972 | 1x4 | 70 | [7.0] |
| 1-7 | R.PDSCH.1-8.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-600 | 2x4 | 70 | [5.0] |
| 1-8 | R.PDSCH.1-13.1 FDD | 10 / 15 | 1024QAM, 0.79 | [TBD] | 2x4, ULA Low | 70 | [TBD] |

END OF CHANGE 2

START OF CHANGE 3

#### A.3.2.1.1	Reference measurement channels for SCS 15 kHz FR1

<SKIP UNCHANGED PART>

Table A.3.2.1.1-13: PDSCH Reference Channel for FDD (1024QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-13.1 FDD |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |
| Number of allocated resource blocks | PRBs | 52 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames | Slots | 19 |  |  |  |  |
| MCS table |  | 1024QAM |  |  |  |  |
| MCS index |  | 23 |  |  |  |  |
| Modulation |  | 1024QAM |  |  |  |  |
| Target Coding Rate |  | 0.79 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 1,…, 19 | Bits | 54240 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 1,…, 19 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A |  |  |  |  |
| For Slots i = 1,…, 19 | CBs | 7 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 10, 11 | Bits | 65520 |  |  |  |  |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 68640 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 51.528 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

END OF CHANGE 3