3GPP TSG-RAN WG4 Meeting # 103-e	R4-2209843

# Electronic Meeting, May.9th –20th, 2022

Title: 	Discussion on PDCCHPBCH requirements

Source: 	Huawei, HiSilicon

Agenda item:       9.15.10.2.2

Document for:	Discussion

Background

In last meeting, RAN 4 agreed to introduce PBCH and PDCCH performance requirements for FR2-2 band. In this contribution, we give our analysis on the open issues and initial simulation results.

Discussions

PDCCH requirements

SCS

Based on our understanding, 960kHz SCS is similar to 60kHz SCS for FR1 which need much processing capability and will not be popular in the future. Moreover, the maximum achievable SNRBB for 960kHz is too low which is untestable. Therefore, we propose to not consider 960kHz SCS.

Proposal 1: Define PDCCH requirements for 120kHz and 480kHz SCS.

TDD pattern

For 120kHz, 3D1S2U, S=10D:2G:2U can be reused. For 480kHz, we propose to keep the boundary of UL/DL slot aligned with 120kHz SCS, hence, 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U can be used

Proposal 2: Use 3D1S2U, S=10D:2G:2U for 120kHz SCS and 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U for 480kHz SCS.

Performance requirements for multi-slot PDCCH monitoring

We have following options for this open issue:

Issue 3-4-1: Performance requirements for multi-slot PDCCH monitoring Option 1: Define performance requirements for multi-slot PDCCH monitoring for 480 and 960 kHz SCS Option 2: Do not define performance requirements for multi-slot PDCCH monitoring for 480 and 960 kHz SCS

RAN 1 introduced multi-slot PDCCH monitoring for 480kHz and 960kHz SCS to keep the UE capability of PDCCH decoding aligned with 120kHz SCS with scaling operation. RAN 1 specified it is mandatory for UE to monitor one PDCCH every four slots for 480kHz and optional for UE to monitor two PDCCHs every four slots. Hence, multi-slot PDCCH monitoring should be considered. In order to ensure the coverage, we can define one requirement for UE supporting both these capabilities.

Proposal 3: Define one requirement for both UE supporting monitor one slot and two slots every four slots

PDCCH simulation assumptions.

We agreed to define following assumptions for PDCCH test.

Issue 3-4-2: PDCCH simulation assumptions Use following assumptions for PDCCH performance test. - Antenna configuration: 1T2R and 2T2R - Aggregation level: 2 and 4 for 1T2R; 8 and 16 for 2T2R

For other assumptions we propose to reuse Rel-15 FR2 PDCCH test. As for propagation conditions, we propose to use TDLA5-200 which is same as proposed value in PDSCH test.

The proposed simulation assumptions are captured in Table 2-1 and Table 2-2:

Table 2-1: Test parameters for FR2-2 PDCCH

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | DDDSU S=10D:2G:2U for 120kHz SCS 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U for 480kHz SCS |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1 and 5 6 for test 2 and 6 | 2 |
| Interleaver size |  | 3 for test 1 and 5 2 for test 2 and 6 | 3 |
| Shift index |  | 0 |  |

Table 2-2: Proposed cases for PDCCH

| Test number | SCS (MHz) | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 120 | 100 | 1 | 60 (40) | 2 | TDLA5-200 | 1_0 | 40 | 1x2 Low |
| 2 | 120 | 100 | 1 | 60 (56) | 4 | TDLA5-200 | 1_1 | 56 | 1x2 Low |
| 3 | 120 | 100 | 1 | 60 (56) | 8 | TDLA5-200 | 1_1 | 56 | 2x2 Low |
| 4 | 120 | 100 | 2 | 60 (40) | 16 | TDLA5-200 | 1_0 | 40 | 2x2 Low |
| 5 | 480 | 400 | 1 | 60 | 2 | TDLA5-200 | 1_0 | 40 | 1x2 Low |
| 6 | 480 | 400 | 1 | 60 | 4 | TDLA5-200 | 1_1 | 56 | 1x2 Low |
| 7 | 480 | 400 | 1 | 60 | 8 | TDLA5-200 | 1_1 | 56 | 2x2 Low |
| 8 | 480 | 400 | 2 | 60 | 16 | TDLA5-200 | 1_0 | 40 | 2x2 Low |

Proposal 4: Use Table 2-1 and Table 2-2 as simulation assumptions for FR2-2 PDCCH test.

We provide our initial simulation results in Figure 2-1:

| - Test 1 | - Test 2 |
| --- | --- |
| - Test 3 | - Test 4 |
| - Test 5 | - Test 6 |
| - Test 7 | - Test 8 |

Figure 2-1: Initial simulation results for FR2-2 PDCCH

We provide the comparison of target SNR for PDCCH and maximum achievable SNRBB in Table 2-3:

Table 2-3:  Comparison of target SNR for PDCCH and maximum achievable SNRBB

| Test number | Allocated RB | Maximum achievable SNRBB(dB) | Ideal SNR(dB) @ 1% of BLER | Impairment SNR(dB) @ 1% of BLER |
| --- | --- | --- | --- | --- |
| 1 | 12 | 15.6 | 4.9 | 6.9 |
| 2 | 24 | 12.5 | 2.7 | 4.7 |
| 3 | 48 | 9.3 | -1.4 | 0.6 |
| 4 | 48 | 9.3 | -4.3 | -2.3 |
| 5 | 12 | 9.3 | 3.2 | 5.2 |
| 6 | 24 | 5.9 | 0.5 | 2.5 |
| 7 | 48 | 1.8 | -2.5 | -0.5 |
| 8 | 48 | 1.8 | -5.4 | -3.4 |

Based on the comparison results of Table 2-3, the target SNR for all proposed cases are lower than the corresponding maximum achievable SNR, hence, all proposed cases are testable.

Observation 1: The target SNR for all proposed cases are lower than the corresponding maximum achievable SNR, hence, all cases are testable.

PBCH requirements

SCS

As per [2], only 120 kHz and 480 kHz SCS for PBCH can be used for initial access. Therefore, we propose to not define performance of PBCH for 960 kHz SCS

Observation 2: Only 120 kHz and 480 kHz SCS can be used for initial access.

Proposal 5: Not consider 960 kHz SCS

Propagation conditions

We propose to use TDLA5-200 to keep aligned with that of PDSCH.

Proposal 6: Use TDLA5-200 as propagation conditions

Others

We have following options for PBCH simulation assumptions

PBCH simulation assumptions (if introduced pending on outcome of issues Issue 1-2-2) SSB Index: - Option 1: Only with not known SSB index - Option 2: Both known and not known SSB index - Option 3: Only with known SSB index - Option 4: Not known SSB index for 120 kHz and known SSB index for 480/960 kHz.

For 120kHz and 480kHz SCS, we propose to define PBCH requirements only for not known SSB index to reduce the simulation work. For 960kHz SCS (If introduced), we propose to only consider known SSB index.

Proposal 7: Only consider not known SSB index for 120kHz and 480kHz SCS. Only consider known SSB index for 960kHz SCS (If introduced).

Conclusion

In this contribution, we give our views on test setup for PDCCH and PBCH test. The proposals and observations are:

For PDCCH test:

- Proposal 1: Define PDCCH requirements for 120kHz and 480kHz SCS.

- Proposal 2: Use 3D1S2U, S=10D:2G:2U for 120kHz SCS and 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U for 480kHz SCS.

- Proposal 3: Define one requirement for both UE supporting monitor one slot and two slots every four slots

- Proposal 4: Use Table 2-1 and Table 2-2 as simulation assumptions for FR2-2 PDCCH test.

- Observation 1: The target SNR for all proposed cases are lower than the corresponding maximum achievable SNR, hence, all cases are testable.

For PBCH test:

- Observation 2: Only 120 kHz and 480 kHz SCS can be used for initial access.

- Proposal 5: Not consider 960 kHz SCS

- Proposal 6: Use TDLA5-200 as propagation conditions

- Proposal 7: Only consider not known SSB index for 120kHz and 480kHz SCS. Only consider known SSB index for 960kHz SCS (If introduced).

Reference

[1]   R4-2207223 WF on demodulation performance requirements definition for 52.6 - 71 GHz.Intel