3GPP TSG-RAN WG4 Meeting #103-e	R4-2209877

# Electronic Meeting, 9th – 20th May, 2022

Title: 	Discussion on satellite NTN demod PUSCH

Source: 	Huawei, HiSilicon

Agenda item:	9.12.8.2.1

Document for:	Discussion

Background

During RAN#102-e meeting, WF [1] on NTN satellite demodulation was approved. In this contribution, we share our views about NTN satellite PUSCH demodulation requirements.

Discussion

PUSCH

Scope of PUSCH requirements

Agreements - Not to consider the requirements for mapping Type B with non-slot transmission Candidate options - Option 1: Do not consider 2 step RACH case - Option 2: Consider the 2 step RACH case

2-step RA type is introduced in Rel-16 to reduce the delay of the initial access. We think 2-step RA type is also benefit for NTN scenario since there is very small frequency offset and delay offset after UE pre-compensation so that the performance for PUSCH in Msg3 can be ensured. So we propose to define requirements for 2-step RA type requirement.

For NTN PUSCH, define requirements for 2-step RA type requirement.

SCS/CBW set for PUSCH requirements

Candidate options - Option 1: 15kHz SCS: 5MHz/10MHz/20MHz, 30kHz SCS: 10MHz/20MHz - Option 2: a few of PRBs for all SCS

As per RF agreements, two new NTN satellite bands n256/n255 is introduced with {5, 10, 15, 20}MHz bandwidth configuration for 15kHz SCS and {10, 15, 20}MHz bandwidth configuration for 30kHz/60kHz SCS.

Option 1 is derived from the normal BS performance requirements, i.e. only 5MHz, 10MHz and 20MHz bandwidth configuration case is defined for 15kHz SCS while 10MHz and 20MHz bandwidth configuration case is defined for 30kHz SCS.

In last meeting, Option 2 is proposed by companies considering that there is poor link budget for NTN uplinks. Based on our evaluation results in [2] and link budget evaluation derived from TR 38.863 as Table 2.1.2-1, it is not feasible for 10MHz and 20MHz under current test configuration from the link budget point of view. Also we notice that for different bandwidth with other test configuration same, there is nearly same simulation results, so we think there is negligible influence on performance for different bandwidth. Also to reduce the test effort, only 5MHz requirements for 15kHz SCS and 10MHz requirements for 30kHz is enough.

Table 2.1.2-1 Link budget for NTN PUSCH

|  | LEO600-rural | LEO600-urban | LEO1200-rural | LEO1200-urban | GEO-rural | GEO-urban | LEO600-rural | LEO600-urban | LEO1200-rural | LEO1200-urban | GEO-rural | GEO-urban |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE Max Tx power (dBm) | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| Coupling loss 50%-tile | 125.73 | 125.54 | 131.49 | 131.39 | 139.94 | 140 | 125.73 | 125.54 | 131.49 | 131.39 | 139.94 | 140 |
| Noise figure for NTN satellite (dB) | 4.3 | 4.3 | 4.3 | 4.3 | 7.4 | 7.4 | 4.3 | 4.3 | 4.3 | 4.3 | 7.4 | 7.4 |
| SCS (kHz) | 15 | 15 | 15 | 15 | 15 | 15 | 30 | 30 | 30 | 30 | 30 | 30 |
| Thermal Noise | -103.17 | -103.17 | -103.17 | -103.17 | -100.07 | -100.07 | -103.72 | -103.72 | -103.72 | -103.72 | -100.62 | -100.62 |
| SNR (UL 5MHz) | 0.44 | 0.63 | -5.32 | -5.22 | -16.87 | -16.93 | 0.99 | 1.18 | -4.77 | -4.67 | -16.32 | -16.38 |

Only define 5MHz requirements for 15kHz SCS and 10MHz requirements for 30kHz for NTN PUSCH performance requirements.

Antenna configuration for PUSCH requirements

Agreements - UE 1Tx - SAN 2Rx Candidate options - FFS: SAN 1Rx, SAN 4Rx and SAN 8Rx

For legacy BS performance requirements for PUSCH, 2Rx/4RX/8Rx cases are defined with applicability rule that lowest and highest of supported Rx number shall be tested based on manufacture declaration. Similar method can be considered for NTN performance requirement definition to give manufacture more flexible to only test the configurations they support. Also considering that companies have concern about the test effort, we propose to apply the above applicability with the modification that only highest of supported Rx number shall be tested based on manufacture declaration rather than both lowest and highest.

Also consider SAN 4Rx and 8Rx for NTN PUSCH requirements definition. Applicability rule can be defined so that only highest of supported Rx number shall be tested based on manufacture declaration.

Proposals

In this contribution, we discuss on satellite PUSCH demodulation requirements for NTN. Our observations and proposals are:

- For NTN PUSCH, define requirements for 2-step RA type requirement.

Only define 5MHz requirements for 15kHz SCS and 10MHz requirements for 30kHz for NTN PUSCH performance requirements.

Also consider SAN 4Rx and 8Rx for NTN PUSCH requirements definition. Applicability rule can be defined so that only highest of supported Rx number shall be tested based on manufacture declaration.

Reference

R4-2207198, WF on NTN SAN demodulation requirements, RAN4#102-e, Huawei, HiSilicon

R4-2209878, Simulation results on satellite NTN demod PUSCH, RAN4#103-e, Huawei, HiSilicon