3GPP TSG-RAN WG4 Meeting #103-e	R4-2209880

# Electronic Meeting, 9th – 20th May, 2022

Title: 	Simulation results on satellite NTN demod PUCCH

Source: 	Huawei, HiSilicon

Agenda item:	9.12.8.2.2

Document for:	Information

Background

During RAN4#102-e meeting, WF [1] on NTN SAN demodulation requirements was approved. In this contribution, we provide our initial simulation results about the NTN PUCCH demodulation requirements.

Simulation

PUCCH format 0

Here we provide the initial simulation results for PDCCH format 0 for alignment, as shown in Table 2.1-2 and Figure 2.1-1, based on simulation assumption as shown in Table 2.1-1.

Table 2.1-1 Simulation assumption for PDCCH format 0

| Parameter | Test |
| --- | --- |
| PUCCH format | 0 |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 |
| Channel model | NTN-TDLA100-200 |
| Number of OFDM symbols | 2 |
| Test metric | ACK missed detection probability <1% |

Table 2.1-2 Ideal simulation results for PDCCH format 0

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 15 | 5 | 1x2 | 0.97 |
| 2 | 15 | 5 | 1x4 | -3.36 |
| 3 | 15 | 5 | 1x8 | -6.63 |
| 4 | 30 | 10 | 1x2 | 1.29 |
| 5 | 30 | 10 | 1x4 | -3.60 |
| 6 | 30 | 10 | 1x8 | -7.21 |

Figure 2.1-1 Ideal simulation results for PDCCH format 0

PUCCH format 1

Here we provide the initial simulation results for PDCCH format 1 for alignment, as shown in Table 2.2-2 and Figure 2.2-1, based on simulation assumption as shown in Table 2.2-1.

Table 2.2-1 Simulation assumption for PDCCH format 1

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA100-200 |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

Table 2.2-2 Ideal simulation results for PDCCH format 1

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ACK missed detection | NACK to ACK |
| 1 | 15 | 5 | 1x2 | -7.08 | -5.33 |
| 2 | 15 | 5 | 1x4 | -11.2 | -10.25 |
| 3 | 15 | 5 | 1x8 | -14.06 | -13.45 |
| 4 | 30 | 10 | 1x2 | -6.36 | -4.00 |
| 5 | 30 | 10 | 1x4 | -11.22 | -9.00 |
| 6 | 30 | 10 | 1x8 | -14.59 | -13.07 |

PUCCH format 2

Here we provide the initial simulation results for PDCCH format 2 for alignment, as shown in Table 2.3-2 and Figure 2.3-1, based on simulation assumption as shown in Table 2.3-1.

Table 2.3-1 Simulation assumption for PDCCH format 2

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA100-200 |
| Test metric | UCI block error probability<1% |

Table 2.3-2 Ideal simulation results for PDCCH format 2

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 15 | 5 | 1x2 | -2.00 |
| 2 | 15 | 5 | 1x4 | -6.46 |
| 3 | 15 | 5 | 1x8 | -9.94 |
| 4 | 30 | 10 | 1x2 | -1.98 |
| 5 | 30 | 10 | 1x4 | -6.16 |
| 6 | 30 | 10 | 1x8 | -9.40 |

Figure 2.3-1 Ideal simulation results for PDCCH format 2

PUCCH format 3

Here we provide the initial simulation results for PDCCH format 3 for alignment, as shown in Table 2.4-2 and Figure 2.4-1, based on simulation assumption as shown in Table 2.4-1.

Table 2.4-1 Simulation assumption for PDCCH format 3

| Parameter | Value |
| --- | --- |
| PUCCH format | 3 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DM-RS |
| Channel model | NTN-TDLA100-200 |
| Test metric | UCI block error probability<1% |

Table 2.4-2 Ideal simulation results for PDCCH format 3

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 15 | 5 | 1x2 | -2.28 |
| 2 | 15 | 5 | 1x4 | -6.44 |
| 3 | 15 | 5 | 1x8 | -9.58 |
| 4 | 30 | 10 | 1x2 | 0.00 |
| 5 | 30 | 10 | 1x4 | -5.14 |
| 6 | 30 | 10 | 1x8 | -9.00 |

PUCCH format 4

Here we provide the initial simulation results for PDCCH format 4 for alignment, as shown in Table 2.5-2 and Figure 2.5-1, based on simulation assumption as shown in Table 2.5-1.

Table 2.5-1 Simulation assumption for PDCCH format 4

| Parameter | Value |
| --- | --- |
| PUCCH format | 4 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DMRS |
| Channel model | NTN-TDLA100-200 |
| Test metric | UCI block error probability<1% |

Table 2.5-2 Ideal simulation results for PDCCH format 4

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 15 | 5 | 1x2 | 1.94 |
| 2 | 15 | 5 | 1x4 | -4.00 |
| 3 | 15 | 5 | 1x8 | -7.65 |
| 4 | 30 | 10 | 1x2 | 2.91 |
| 5 | 30 | 10 | 1x4 | -3.17 |
| 6 | 30 | 10 | 1x8 | -7.27 |

PUCCH format 1 (multi-slot)

Here we provide the initial simulation results for PDCCH format 1 (multi-slot) for alignment, as shown in Table 2.6-2 and Figure 2.6-1, based on simulation assumption as shown in Table 2.6-1.

Table 2.6-1 Simulation assumption for PDCCH format 1 (multi-slot)

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 (multi-slot) |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |
| Cyclic Prefix | Normal |
| Channel model | NTN-TDLA100-200 |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

Table 2.6-2 Ideal simulation results for PDCCH format 1 (multi-slot)

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ACK missed detection | NACK to ACK |
| 1 | 15 | 5 | 1x2 | -10.44 | -9.25 |
| 2 | 30 | 10 | 1x2 | -10.10 | -6.33 |

Conclusion

In this contribution, we provide our initial simulation results on NTN PUCCH demodulation requirements.

Reference

R4-2207198, WF on NTN SAN demodulation requirements, RAN4#102-e, Huawei, HiSilicon