3GPP TSG-RAN WG4 Meeting #103-e	R4-2209881

# Electronic Meeting, 9th – 20th May, 2022

Title: 	Discussion on satellite NTN demod PRACH

Source: 	Huawei, HiSilicon

Agenda item:	9.12.8.2.3

Document for:	Discussion

Background

During RAN#102-e meeting, WF [1] on NTN satellite demodulation was approved. In this contribution, we share our views about NTN satellite PRACH demodulation requirements.

Discussion

PRACH

Antenna configuration

For legacy BS performance requirements for PRACH, except for LRA=1151 and LRA=571, 2Rx/4RX/8Rx cases are defined with applicability rule that lowest and highest of supported Rx number shall be tested based on manufacture declaration. Similar method can be considered for NTN performance requirement definition to give manufacture more flexible to only test the configurations they support. Also considering that companies have concern about the test effort, we propose to apply the above applicability with the modification that only highest of supported Rx number shall be tested based on manufacture declaration rather than both lowest and highest.

Preamble format

For the preamble format, down scope is needed to reduce the test effort, we propose to only consider typical A2/B4/C2 preamble format for NTN satellite requirements definition.

Time error tolerance

For the time error tolerance, we propose to use the same method as the legacy BS that is 0.522μ+Tdelay where Tdelay is corresponding to the tap with the second largest delay of the propagation delay channel.

SCS

For the SCS, same configuration as the legacy BS requirements can be reused, i.e. 15kHz for LRA=139 and 1151, 30kHz for LRA=139 and 571.

Doppler

For the Doppler, 200Hz Doppler that is corresponding to 0.1ppm of carrier frequency can be selected for NTN satellite PRACH requirements definition.

Overview

The overview of the test configuration is proposed as following:

Table 2.1-1 Test configuration for NTN PRACH requirements

| Parameter | Value |
| --- | --- |
| Preamble format | A2/B4/C2 |
| Antenna configuration | 2/4/8 Rx for LRA=139, 2Rx for LRA=1151 and 571 |
| SCS | 15kHz for LRA=139 and 1151, 30kHz for LRA=139 and 571 |
| Propagation | AWGN, NTN-TDLA100-200 |
| Time error tolerance | 0.522μ+Tdelay (Same formula as legacy BS requirements) |
| Frequency offset | 200Hz |

Proposals

In this contribution, we discuss on satellite PRACH demodulation requirements for NTN. Our observations and proposals are:

For NTN PRACH requirements, use the following test configuration.

| Parameter | Value |
| --- | --- |
| Preamble format | A2/B4/C2 |
| Antenna configuration | 2/4/8 Rx for LRA=139, 2Rx for LRA=1151 and 571 |
| SCS | 15kHz for LRA=139 and 1151, 30kHz for LRA=139 and 571 |
| Propagation | AWGN, NTN-TDLA |
| Time error tolerance | 0.522μ+Tdelay (Same formula as legacy BS requirements) |
| Doppler | 200Hz |
| Delay spread | 250ns |

Applicability rule can be defined so that only highest of supported Rx number shall be tested based on manufacture declaration.

Reference

R4-2207198, WF on NTN SAN demodulation requirements, RAN4#102-e, Huawei, HiSilicon