3GPP TSG-RAN WG4 Meeting # 103 -e	R4-2209923

Electronic Meeting, May 9th – 20th, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	On SAN Spurious Emission requirements for Radiated Characteristics

Agenda Item: 	9.12.3.1

Document for:	Approval

Introduction

The WI description for RAN4 work related to NR over NTN was presented in [1]. Following the progress of RAN4 on the topic, it was approved an extension for the WI activities, in order to address eventual open issues. The WI exception list provided in [2] includes RAN4 RF core requirements, with provision for clarifications and addressing possible inconsistencies.

Discussion

RAN 4 WF:

- Option 1 (Ericsson):

| Spurious frequency range | Basic limit GEO class (dBm) | Basic limit LEO class (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 9kHz – 12.75 GHz | [-13] | [-13] | 4 kHz |

FFS how to handle limit when satellite output power > 50W.

- Option 2 (THALES):

For satellites operating in MSS S-Band n256 and MSS L-band n255, the frequency range for measurements shall be applied as in Table x.x-1.

Table x.x-1. Frequency range for measurements

| Fundamental frequency range | Frequency range for measurements |  |
| --- | --- | --- |
|  | Lower limit | Upper limit (The test should include the entire harmonic band and not be truncated at the precise upper frequency limit stated) |
| 600 MHz - 5.2 GHz | 800 MHz | 11 GHz (5th harmonic) |

SAN spurious emissions limits for GEO and LEO classes with respect to MSS S-Band n256 and MSS L-band n255 are defined as described in Table x.x-2.

The WF for SAN SEM and Spurious Emission presented in [3] presents a list of options for the spurious emission for the SAN as an open issue:

RAN 4 WF (Continuation):

Table x.x-2. Basic spurious emission limit for GEO and LEO SAN classes

| Spurious frequency range | Basic limit GEO class (dBm) | Basic limit LEO class (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 800 MHz - 1508 MHz | -9 | -7 | 4 kHz | Notes 1,2,3 |
| 1508 MHz - 1518 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1518 MHz - 1559 MHz | NA: L-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 1559 MHz - 1569 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 1569 MHz - 2160 MHz | -9 | -7 | 4 kHz | Notes 1,3 |
| 2160 MHz - 2170 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2170 MHz - 2200 MHz | NA: S-Band transmit Bandwidth |  | 4 kHz | Note 1 |
| 2200 MHz - 2210 MHz | Covered by OBUE requirement |  | 4 kHz | Note 1 |
| 2210 MHz- 11000 MHz | -9 | -7 | 4 kHz | Notes 1, 2 |
| - NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. - NOTE 2: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. - NOTE 3: As per ITU-R SM.329 [x], Spurious requirement is -60 dBc with respect to useful operating power P: 51-60= -9 dBm for GEO class and 53-60= -7 dBm for LEO class. |  |  |  |  |

- Option 3: Other solutions are not precluded.

TS 38.104 Section 6.6.5 (Spurious Emission for the conducted transmitter):

#### 6.6.5.1	General

The transmitter spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in table 6.6.1-1. For some operating bands, the upper limit is higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R recommendation SM.329 [2].

TS 38.104 Section 6.6.5 (Spurious Emission for the conducted transmitter):

#### 6.6.5.1	General

The transmitter spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in table 6.6.1-1. For some operating bands, the upper limit is higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R recommendation SM.329 [2].

We believe that Option 1 is in line with current specifications: it addresses frequencies starting from the edge of the transmitting bandwidth, covering a wide range of frequencies as stated in 38.104[4]

Some of the co-existence aspects are not captured by Option 2. For example, the Table provided in Option 2 assumes the frequency range between 1508-1518 MHz is covered by OBUE requirements. But that would be true only for a SAN operating in band n255, whereas for a SAN operating in band n256 the requirements would not be captured under the OBUE definition. The same is valid for other bands (1518 MHz - 1559 MHz, 1559 MHz - 1569 MHz, 2160 MHz - 2170 MHz, 2170 MHz - 2200 MHz, 2200 MHz - 2210 MHz).

Observation 1: The band definition presented in Option 2 are not agnostic to the SAN operating channel bandwidth.

Observation 2: Option 1 is more flexible for future increments in the specification.

Proposal: RAN 4 to adopt option 1 as the reference table for Spurious Emission.

Conclusion

In this contribution we discuss the SAN spurious emission table:

Observation 1: The band definition presented in Option 2 are not agnostic to the SAN operating channel bandwidth.

Observation 2: Option 1 is more flexible for future increments in the specification.

Proposal: RAN 4 to adopt option 1 as the reference table for Spurious Emission.

- References

- RP-213691, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-220209, Rel-17 Work Item Exception for NR-NTN-solutions, Thales

- R4-2207456, Way Forward on SAN SEM and spurious emission, THALES, ZTE

- TS 38.104, NR; Base Station (BS) radio transmission and reception