3GPP TSG-RAN WG4 Meeting # 103 -e	R4-2209928

Electronic Meeting, May 9th – 20th, 2022

Title:	TP to TR 38.717-02-01: Addition of CA_n20-n40

Source:	Nokia, Telefonica

Agenda item:	8.5.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-02-01 to add CA_n20-n40.

************************************* Start of TP*****************************************

## 6.X		CA_n20-n40

### 6.X.1	Common for 1 band UL and 2 bands UL CA

#### 6.X.1.1 Operating bands for CA

Table 6.X.1.1-1: CA band combination of band n20 + n40

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n20-n40 | n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

#### 6.X.1.2	Channel bandwidths per operating band for CA

Table 6.X.1.2-1: Supported bandwidths per CA band combination of band n20+n40

| CA operating/channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| CA_n20A-n40A | - | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |

#### 6.X.1.3	Co-existence studies

Table 6.X.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n20-n40.

Table 6.X.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n20 | 832 | 862 | 791 | 821 | 1664 | 1724 | 2496 | 2586 | 3328 | 3448 | 4160 | 4310 |
| n40 | 2300 | 2400 | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 | 9200 | 9600 | 11500 | 12000 |

Based on above table, there is no harmonic interference.

Table 6.X.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n20 | 832 | 862 | 791 | 821 | 1582 | 1642 | 2373 | 2463 | 3164 | 3284 | 3955 | 4105 |
| n40 | 2300 | 2400 | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 | 9200 | 9600 | 11500 | 12000 |

Based on above table, the 3rd harmonic of band n20 downlink may fall into band n40 thus there can be harmonic mixing issue for the band combination of n20 and n40.

#### 6.X.1.4	∆TIB and ∆RIB values

For CA_n20-n40, the TIB,c values from similar LB FDD + 40 TDD are re-used. The RIB are re-used from DC_20_n78

Table 6.X.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n20-n40 | n20 | 0.3 |
|  | n40 | 0.3 |

Table 6.X.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n20-n40 | n20 | 0 |
|  | n40 | 0.5 |

#### 6.X.1.5	REFSENs requirements

The MSD due to cross band isolation are used from the similar combinations of CA_n18-n41 and CA_n28-n40.

Table 6.x.1.5-1: Reference sensitivity exceptions due to harmonic mixing from a PC3 aggressor NR UL band for NR CA FR1 for either PC3 or PC2 CA

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz(dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz(dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n40 | n204 | 27.8 | 24.8 | 23 | 20.3 |  |  |  |  |  |  |  |  |  |
| NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.x.1.5-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n40 | n20 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |

#### 6.X.1.6	OOB blocking exception requirements

Since band n20 is a low band and n40 is a wide band, the OOB exception is needed.

Table 6.x.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n20-n40

************************************* End of TP*****************************************