3GPP TSG-RAN WG4 Meeting # 103 -e	R4-21209940

Electronic Meeting, May 9th – 20th, 2022

Title:	TP to TR 38.717-03-01: Addition of CA_n8-n40-n78

Source:	Nokia, OPTUS

Agenda item:	8.6.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-03-01 to add CA_n8-n40-n78.

************************************* Start of TP*****************************************

## 6.x	CA_n8-n40-n78

### 6. x.1	Operating bands for CA

Table 6.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8A-n40A-n78A | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

#### 6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported bandwidths per CA band combination for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n8A-n40A-n78A | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 |  |  |  |
|  |  | n78 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.x.3	Co-existence studies

Co-existence studies of CA_n8-n40-n78 with 1UL are already covered in the constituent fall-back modes.

### 6.x.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n8, n40 and n78, the TIB,c and RIB,c values are shown in table 6.x.4-1 and table 6.x.4-2, respectively. They are taken from the existing combination DC_8_n40-n78.

Table 6.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n8A-n40A-n78A | n8 | 0.6 |
|  | n40 | 0.3 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n8A-n40A-n78A | n8 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

### 6.x.5	REFSENS requirements

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

************************************* End of TP*****************************************