3GPP TSG-RAN WG4 Meeting # 103 -e	R4-2209985

Electronic Meeting, May 9th – 20th, 2022

Source: 	Nokia, AT&T

Title: 		TP for 38.717-04-02 to introduce CA_n2A-n29A-n30A-n77A

Agenda Item: 			8.9.2 [NR_CADC_R17_4BDL_2BUL]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 38.717-04-02 to introduce CA_n2A-n29A-n30A-n77A. Note a TP for the 1UL corresponding CA combination also have been submitted for TR 38.717-04-01.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 5.1.x	CA_n2-n29-n30-n77

#### 5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: Inter-band CA operating bands

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n2A-n29A-n30A-n77A | n2, n29, n30, n77 |

#### 5.1.x.2	Channel bandwidths per operating bands for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration or single uplink carrier4 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n29A-n30A-n77A | CA_n2A-n30A CA_n2A-n77A CA_n30A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |

*********************** End of the TP ***************************************************

# 3	References