3GPP TSG-RAN WG4 Meeting # 103-e              									     R4-2210116

Electronic Meeting, 9st of May – 20th of May, 2022

Agenda item:			9.12.3.3

Source:	THALES

Title:	Tentative draft pCR for Clause 7.3.2.2.4.2 Operating band unwanted emissions - TR 38.863

Document for:	Approval

# Introduction

During RAN WG4 Meeting RAN4#102-e, a work split has been proposed to complete Section 6 and Section 7 of TR 38.863. In this document, two alternatives for the TR 38.863 OBUE Clause are proposed to update some of the subsections related to RF coexistence.

# Discussion

In this contribution is proposed to add information with respect to Section 7.3.2.2.4.2 Operating band unwanted emissions of TR 38.863 [1] by including some information related to agreements for TS 38.108.

Proposal 1: To update the TR with one alternative of the attached TP (2 Possible Alternatives of the same TP)

# Conclusions

In this contribution, the following proposal is made:

Proposal 1: To update the TR with one alternative of the attached TP (2 Possible Alternatives of the same TP)

# Reference

[1]	TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

# Text proposals for TR 38.863

------------------------------------------------<Start of TP Alternative 1>-------------------------------------

7.3.2.2.4.2		Operating band unwanted emissions

The SAN Operating Band Unwanted Emissions (OBUE) requirements for GEO and LEO classes are therefore defined as described in Table 7.3.2.2.4.2-1 and Table 7.3.2.2.4.2-2 below.

Table 7.3.2.2.4.2-1: SAN GEO Class OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | 1dBm-45foffsetMHz-0.002dB | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | -3 dBm | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Not applicable for B<100 MHz | 4kHz |

Table 7.3.2.2.4.2-2: SAN LEO Class OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | -3.0dBm-45foffsetMHz-0.002dB+20×log10A600dB | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | -7.0dBm+20×log10A600dB (NOTE 1) | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Not applicable for B<100 MHz | 4kHz |
| NOTE 1: A is the distance from the SAN to the Earth at SAN nadir direction. For example, A=600 for LEO@600km and A=1200 for LEO@1200km. |  |  |  |

To define the OBUE limits, the noise floor has to be defined taking into account the in-band EVM required value of 17.5% for QPSK, corresponding to a Modulation Error Ratio (MER) or equivalent Signal-to-Noise Ratio (SNR):

-20×log10(EVM[%]/100)=15.14 dB.

The OBUE and EVM are applicable to Satellite Access Node (SAN) transmitter. Moreover, EVM includes the Thermal noise of the feederlink (in reception), other kind of potential interference, the Inter-Modulation (IM) noise of the power amplifiers, inter-symbol interference, phase noise, in-band spurious emission generated by SAN.

The thermal noise has low impact in the out-of-band domain. The out-of-band main contribution therefore comes mainly from the Inter-Modulation (IM) noise, and therefore the other sources are neglected. Please also note that EIRP values correspond to the point of operation providing the required EVM.

The simulation considers 30 MHz of bandwidth (see maximum available bandwidth for MSS S-band n256 and MSS L-band n255) since it is the worst case for OBUE.

Table 7.3.2.2.4.2-3: System Parameters used for Evaluation of OBUE SAN

| Parameter | Unit | GEO | LEO@600 | Comments |
| --- | --- | --- | --- | --- |
| EIRP density | (dBW/MHz) | 59 | 34 | As in TR 38.863 |
| Bandwidth | (MHz) | 30 | 30 | Worst case (see explanation below and Figures 7.3.2.2.4.2-1 and 7.3.2.2.4.2-2) |
| Antenna gain | (dBi) | 51,0 | 30,0 | As in TR 38.863 |
| Carrier frequency f0 | (MHz) | 2185 | 2185 | MSS S-Band n256 |
| SNR (equivalent to Modulation Error Ratio) | (dB) | 15,14 | 15,14 | Corresponding to EVM =17,5% using -20×log10(EVM[%]/100) equation |
| Measurement band | (kHz) | 4,0 | 4,0 | As per ITU-R recommendation SM.1541-6 |
| Output power density | (dBm/4kHz) | 14,02 | 10,02 | EIRP density – antenna gain + 10×log10(4kHz/1MHz) + 30 |
| Margin | dB | 2 | 2 | For the computation of In-band and Out-of-band noise density |
| In-band noise density - equivalent to 1st point derivation at 2170 MHz | (dBm/4kHz) | -1,12+2 ≈1 | -5,12+2 ≈-3 | Output power density – SNR + Margin |
| Out-of-band noise density (at ΔF = ± 5 MHz) - equivalent to 2nd point derivation at 2165 MHz | (dBm/4kHz) | -5,0+2 ≈-3 | -9,0+2 ≈-7 | Based on simulations using non-linear satellite power amplifier characteristics + Margin |

The OBUE basic limits are given with 2 dB margin with respect to above computation, and therefore the out-of-band limit becomes -3dBm/4kHz for GEO and -7dBm/4kHz for LEO@600km.

Out-of-band noise is mainly produced by intermodulation noise generated by the power amplifiers and weakly filtered by the output filter in the region close to the bandwidth. The intermodulation noise is simulated considering the power amplifier non-linear responses (in Amplitude and Phase) and filling the bandwidth with a Gaussian signal to emulate OFDM multi-carrier operation.

The Figure 7.3.2.2.4.2-1 below represents the simulated output spectrum and the proposed limits for the case of LEO 600km satellite.

Figure 7.3.2.2.4.2-1. Output Spectrum for different BandWidths using OFDM signal with transponder optimised for QPSK modulation

In Figure 7.3.2.2.4.2-1 and further in Figure 7.3.2.2.4.2-2, we represent the spectrum of a 30 MHz signal, in blue, and the spectrum of a 5 MHz signal, in orange. Please note that the specified EIRP density, expressed in dBW/MHz, applies to all channel bandwidths. Therefore, the worst case will be the widest channel (since in TR 38.863 we consider the same spectral density in dBW/MHz independently of the BandWidth). The red lines from Figure 7.3.2.2.4.2-1 and Figure 7.3.2.2.4.2-2 represent the proposed limits given in Table 7.3.2.2.4.2-2 for LEO (and Table 7.3.2.2.4.2-1 for GEO) represented above.

Moreover, simulations provided in Figure 7.3.2.2.4.2-2 for with transponder optimised for 64QAM modulation clearly show that the OBUE limit from Figure 7.3.2.2.4.2-1 configuration (with lower order modulation) is also applicable for Figure 7.3.2.2.4.2-2 configuration (with higher order modulation), because of the transponder optimisation. The two configurations with 1) QPSK (Figure 7.3.2.2.4.2-1) and 2) 64QAM (Figure 7.3.2.2.4.2-2) are covering all the range of MCS as defined in TS 38.108 [x] and therefore 16QAM configuration is not presented.

Figure 7.3.2.2.4.2-2. Output Spectrum for different BandWidths using OFDM signal with transponder optimised for 64QAM modulation

Moreover, with respect to LEO 600 km class, 20×log10(A/600) is used to scale the OBUE values for different LEO altitudes. The scaling factor ensures that the on-ground received flux density from SAN will be the same independently of the SAN altitude.

ACLR is defined as the ratio of power in the useful BW and noise power on adjacent BW integrated over the channel BW. On the other hand, OBUE are discrete spurious emissions measured in 4 kHz.

The OBUE mask permits to consider spurious emissions higher than the local average noise density.

General observation: The OBUE mask requirement (and the mathematical operation of integration of the OBUE mask) does not reflect the ACLR performance, and should not consider ACLR requirement in the definition or computation of OBUE. For this reason, it is important to emphasize that respecting the OBUE mask will not guarantee/imply the ACLR requirements. ACLR is an independent requirement and shall be considered separate from OBUE.

------------------------------------------------<End of TP Alterative 1>-------------------------------------

------------------------------------------------<Start of TP Alternative2>-----------------------------------

7.3.2.2.4.2		Operating band unwanted emissions

The SAN Operating Band Unwanted Emissions (OBUE) requirements for GEO and LEO classes are therefore defined as described in Table 7.3.2.2.4.2-1.

Table 7.3.2.2.4.2-1: SAN OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 2* BWChannel | 0.002 MHz f_offset < 2* BWChannel + 0.002 MHz | maxSE limit, PSDchannel – 40*log10 f_offset-0.002BWChannel*2+1dBm | 4kHz |
| NOTE 1: PSDchannel = Prated,c – 10log10(BWChannel) – 24, unit dBm/4kHz. NOTE 2: SE limit is spurious emission limit specified in spurious emission clause. NOTE 3: PSD attenuation as in ITU-R SM.1541-6, Annex 5 OoB domain emission limits for space services. |  |  |  |

ITU-R SM.1541-6 [x] specify the OoB mask requirements. The OoB domain emission as attached Annex 5 should be attenuated below the maximum psd by the following and the attenuation can’t be greater than the spurious emission limit.

The OoB domain emissions of a station operating in the bands allocated to the FSS and MSS should be attenuated below the maximum psd, in a reference bandwidth of 4 kHz (for systems operating above 15 GHz a reference bandwidth of 1 MHz may be used in place of 4 kHz) within the necessary bandwidth, by the following:

where F is the frequency offset from the edge of the total assigned band, expressed as a percentage of necessary bandwidth. It is noted that the OoB emission domain starts at the edges of the total assigned band.

We can get the “F” = Frequency offset / channel bandwidth * 100 as above. Thus, we can specify the OoB mask requirements as below based on the ITU regulation when we use f_offset and BWchannel to replace “F”.

-------------------------------------------<End of TP Alternative2>--------------------------------------