3GPP TSG-RAN WG4 Meeting # 103e	R4-2210235

# Electronic Meeting, May 9 - 20, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Adding 25MHz and 30MHz CBW to Band n26

Agenda Item:	8.21.2.1 Addition of bandwidth and Tx/Rx requirements [NR_bands_R17_BWs -Core]

Document for:	Approval

# Introduction

This document presents our views on supporting 25MHz and 30MHz channel bandwidth (CBW) requested for band n26 at RAN plenary meeting #95 [1]. Considering the similarities between band n26 and band n5, it is proposed to restrict uplink (UL) band operation to a maximum of 20MHz CBW and to introduce asymmetric UL/DL CBW operation for downlink (DL) 25MHz and 30MHz CBW. We also propose 25MHz and 30MHz reference sensitivity (REFSENS) levels based on power amplifier (PA) measurements.

# Discussion

Due to similarities between band n26 and n5, it is proposed to avoid excessive self-desensitization by restricting the band n26 UL CBW to a maximum of 20MHz for DL 25MHz and 30MHz CBW operation.

## Capturing Asymmetric UL/DL CBW Requirements for band n26 20/25MHz and 20/30MHz

We advance proposals 1, 2 and 3 below to align the introduction of band n26 25/30MHz CBW with the previous agreements captured in big CR [2] for bands with asymmetric UL/DL CBW.

Proposal 1: Restrict the uplink channel bandwidth to 20MHz for band n26 DL operation at 25MHz and at 30MHz CBW. Adopt the Table 5.3.5-1 (subclause 5.3.5) changes highlighted in yellow in Table 1 below to reflect this proposal.

Table 1: Proposed changes to Table 5.3.5-1 to capture band n26 asymmetric UL/DL 20/25 and 20/30MHz CBW.

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. NOTE 8: This UE channel bandwidth is applicable only to uplink. NOTE 9: Void. NOTE 10: These UE channel bandwidths are applicable to sidelink operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: For band n26 DL 25MHz and DL 30MHz operation with UL restricted to 20MHz CBW, introduce asymmetric channel bandwidth combination set 0 as highlighted in yellow in Table 2 for Table 5.3.6-1 (subclause 5.3.6).

Table 2: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n5 | 20 | 25 | 0 |
| n8 | 20 | 35 | 0 |
| n24 | 10 | 5 | 0 |
| n25 | 40 | 45 | 0 |
| n26 | 20 | 25,30 | 0 |
| n66 | 5, 10 | 20, 40 | 0 |
|  | 20 | 40 |  |
|  | 5, 10 | 20, 25, 30, 40 | 1 |
|  | 20, 25, 30 | 40 |  |
| n70 | 5, 10 | 15 | 0 |
|  | 5, 10, 15 | 20, 25 |  |
| n71 | 5 | 10 | 0 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 5 | 10 | 1 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 20 | 35 |  |
|  | 20 | 25, 30, 35 | 2 |
| n911 | 10 | 5 | 0 |
| n921 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| n931 | 10 | 5 | 0 |
| n941 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [15], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

Proposal 3: For n26 DL 25MHz and 30MHz CBW REFSENS, adopt NOTE 5 highlighted in yellow in Table 3 for the UL RB allocation configuration of table 7.3.2-3 (sub-clause 7.3.2).

Table 3: Proposed changes to Table 7.3.2-3 to capture the 20MHz UL RB configuration for n26 DL 25 and 30MHz REFSENS

| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n26 | 15 | 25 | 251 | 251 | 251 | Note 5 | Note 5 |  |  |  |  | FDD |
|  | 30 |  | 121 | 121 | 121 | Note 5 | Note 5 |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 2: For Band 20; for 15 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16; for 30 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 6 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 8; for 60 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 3 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 4; Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. Note 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. |  |  |  |  |  |  |  |  |  |  |  |  |

## n26 25MHz and 30MHz REFSENS

### Power Amplifier Calibration and REFSENS Assumptions

PA Tx noise in Rx band measurement conditions:

- PA is calibrated to meet -30dBc ACLR at PC3 1dB MPR with the 20 MHz channel bandwidth (CBW), SCS 15 kHz, DFT-S-OFDM QPSK 100RB0 waveform;

- Power Class 3 (PC3) operation and 4dB post-PA loss;

- Local Oscillator (LO) leakage: -28dBc and IQ Image rejection: -28dB;

- C-IM3: -60dBc, C-IM5: -70dBc;

- Duplexer rejection: 50dB Tx-Rx, 45dB Tx-Antenna;

- n26 UL 20MHz RB configuration: according to Table 3, i.e., Lcrb=25RB at SCS 15kHz, with UL RB positioned closest to the DL band; and,

- The DL carrier is positioned at a frequency separation of 45MHz from the UL 20MHz carrier.

### Measurement Results and REFSENS

For UL RB allocation Lcrb=25RB, the UL 20MHz Tx noise levels measured in Rx band for DL 25MHz and DL 30MHz CBW are -34.5dBm and -33.5dBm respectively. This leads to an MSD after MRC combining of 6.7dB and 6.9dB respectively for 25MHz and 30MHz DL CBW. We therefore propose the following:

Proposal 4: Adopt n26 25MHz and 30MHz CBW REFSENS levels highlighted in yellow in Table 5 for Table 7.3.2-1a (sub-clause 7.3.2).

Table 5: n26 25MHz and 30MHz REFSENS proposals for Table 7.3.2-1a.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n26 | 15 | -97.56 | -94.56 | -92.76 | -87.6 | -83.5 | -82.5 |  |  |  |  |
|  | 30 |  | -94.86 | -92.76 | -87.7 | -83.6 | -82.6 |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. |  |  |  |  |  |  |  |  |  |  |  |

# Conclusions

This contribution proposes to avoid excessive 25MHz and 30MHz self-desensitization by restricting band n26 UL CBW to 20MHz (proposal 1). This restriction requires the introduction of a new asymmetric channel bandwidth combination set 0, and proposal 2 captures the required changes. Finally, proposals 3 and 4 present n26 25MHz and 30MHz REFSENS levels based on PA Tx noise in Rx band measurement data together with UL RB configurations.

Proposal 1: Restrict the uplink channel bandwidth to 20MHz for band n26 DL operation at 25MHz and at 30MHz CBW. Adopt the Table 5.3.5-1 (subclause 5.3.5) changes highlighted in yellow in Table 1 below to reflect this proposal.

Table 1: Proposed changes to Table 5.3.5-1 to capture band n26 asymmetric UL/DL 20/25 and 20/30MHz CBW.

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. NOTE 8: This UE channel bandwidth is applicable only to uplink. NOTE 9: Void. NOTE 10: These UE channel bandwidths are applicable to sidelink operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: For band n26 DL 25MHz and DL 30MHz operation with UL restricted to 20MHz CBW, introduce asymmetric channel bandwidth combination set 0 as highlighted in yellow in Table 2 for Table 5.3.6-1 (subclause 5.3.6).

Table 2: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n5 | 20 | 25 | 0 |
| n8 | 20 | 35 | 0 |
| n24 | 10 | 5 | 0 |
| n25 | 40 | 45 | 0 |
| n26 | 20 | 25,30 | 0 |
| n66 | 5, 10 | 20, 40 | 0 |
|  | 20 | 40 |  |
|  | 5, 10 | 20, 25, 30, 40 | 1 |
|  | 20, 25, 30 | 40 |  |
| n70 | 5, 10 | 15 | 0 |
|  | 5, 10, 15 | 20, 25 |  |
| n71 | 5 | 10 | 0 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 5 | 10 | 1 |
|  | 10 | 15 |  |
|  | 15 | 20 |  |
|  | 20 | 35 |  |
|  | 20 | 25, 30, 35 | 2 |
| n911 | 10 | 5 | 0 |
| n921 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| n931 | 10 | 5 | 0 |
| n941 | 5 | 10, 15, 20 | 0 |
|  | 10 | 15, 20 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [15], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

Proposal 3: For n26 DL 25MHz and 30MHz CBW REFSENS, adopt NOTE 5 highlighted in yellow in Table 3 for the UL RB allocation configuration of table 7.3.2-3 (sub-clause 7.3.2).

Table 3: Proposed changes to Table 7.3.2-3 to capture the 20MHz UL RB configuration for n26 DL 25 and 30MHz REFSENS

| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n26 | 15 | 25 | 251 | 251 | 251 | Note 5 | Note 5 |  |  |  |  | FDD |
|  | 30 |  | 121 | 121 | 121 | Note 5 | Note 5 |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 2: For Band 20; for 15 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16; for 30 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 6 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 8; for 60 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 3 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 4; Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. Note 5: For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used. |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 4: Adopt n26 25MHz and 30MHz CBW REFSENS levels highlighted in yellow in Table 5 for Table 7.3.2-1a (sub-clause 7.3.2).

Table 5: n26 25MHz and 30MHz REFSENS proposals for Table 7.3.2-1a.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n26 | 15 | -97.56 | -94.56 | -92.76 | -87.6 | -83.5 | -82.5 |  |  |  |  |
|  | 30 |  | -94.86 | -92.76 | -87.7 | -83.6 | -82.6 |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. |  |  |  |  |  |  |  |  |  |  |  |

# References

[1] 	RP-220160, Revised Basket WID on adding channel bandwidth support to existing NR bands, 3GPP TSG RAN meeting #95e, Ericsson.

[2]	R4-2115101, bigCR to TS 38.101-1 - Introduction of 35MHz and 45MHz channel bandwidth R4-2112023, 3GPP TSG-RAN WG4 Meeting # 100e, Skyworks Solutions, Inc., Apple.