3GPP TSG-RAN WG4 Meeting # 103-e			                                                R4-2210266

Electronic Meeting, 09 – 20 May 2022

Agenda item:			9.15.1, 9.15.2, spectrum utilization

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [103-e][131] NR_ext_to_71GHz_Part_1

Document for:	Information

# Introduction

This document captures RAN4 discussions on general aspects, band planning, system parameters and spectrum utilization of the NR extension to 71GHz work item. The covered agenda items are: 9.15.1 and 9.15.2.

# Topic #1: General (AI 9.15.1)

Topics include UE feature list, LS reply on minimum guard symbol of SRS, and draft CRs

## 1.1	Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2208045 UE features for NR ext. to 71GHz WI | Intel Corporation | Proposal 1: Adopt the UE features listed in Table 1 for NR_ext_to_71GHz. Improved ON/ON transient period is contingent on the outcome of core discussion (covered in thread [132]). |
| Table 1. NR_ext_to_71GHz feature list Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (V2X WI only)”.; Consequence if the feature is not supported by the UE; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. FR2-2 channel bandwidths for each SCS in each band for DL and UL for a single CC; Support of FR2-2 channel bandwidths 1) 120 kHz SCS: {100, 400} MHz CBW 2) 480 kHz SCS: {400, 800, 1600} MHz CBW 3) 960 kHz SCS: {400, 800, 1600, 2000} MHz CBW; FFS; Yes; No; UE cannot support some UE channel bandwidths; Per band; N/A; Applicable to FR2-2 only; N/A; UE indicating the support of specific SCS per band (RAN1 features X-Y) is required to support all CBWs corresponding to this SCS No additional capability signalling is needed.; NA. [Improved ON/ON transient period]; 1) Support of improved ON/ON transient period of X < 5us (X is FFS); FFS; Yes; No; UE does not support improved ON/ON transient period and support 5us transient period; Per UE; N/A; Applicable to FR2-2 only; N/A; Further RAN4 discussion is required on whether to support improved ON/ON transient period and X value; Optional with capability signalling. |
| R4-2208143 Draft reply LS on the minimum guard period between two SRS resources for antenna switching | CATT | Reply LS content: The evaluation results in the R15 reply LS [R4-1710048] still apply to FR2-2, i.e. the absolute time for the antenna switching is the same as R15 capability. Answer from RAN4: The absolute switching time for FR2-2 is the same as the capability evaluated in R15, i.e., the antenna switching time is 15 usec. The detail evaluation results for every possible switching scenario are included in the R15 reply LS R4-1710048 [1]. The decision on the symbol(s) needed to accommodate the required minimum guard time for SRS antenna switching for 480 and 960 kHz SCS is up to RAN1 discussion based on the absolute switching time in R4-1710048. |
| R4-2208541 Draft CR to TS 38.104: Addition of support for band n264 for licensed operation within 66000 to 71000 MHz | Ericsson | Introduces relevant aspects of licensed NR band n264 to TS 38.104. These include operating band definition (sub-clause 5.2), channel bandwidths and SCS (sub-clause 5.3), nominal channel spacing, channel raster and sync raster (sub-clause 5.4), step frequencies for Tx and Rx spurious emission limits (sub-clause 9.7 and 10.7). |
| R4-2208617 Draft CR for TS 38.101-2: Introduction of system parameters for FR2-2 | vivo | Provides updates to Clause 5 of TS 38.101-2 to include relevant system parameters of FR2-2. Updates include definition of FR2-2, inclusion of NR band n263, NR CA band CA_n263 for intra-band contiguous CA, NRB, minimum guardband, channel bandwidths, channel raster and sync raster. |
| R4-2208763 Reply LS on the minimum guard period between two SRS resources for antenna switching | Huawei, HiSilicon | Reply LS content: In Rel-15 RAN4 had discussed on the guard period between two SRS resources for antenna switching and agreed on the value of 15us regardless of SCS type. For 480 and 960 kHz SCS in question, the same 15us guard period applies. The guard period would have impact on demodulation of the symbol within the period, therefore blanked symbols would be helpful to guarantee the performance. |
| R4-2209322 Multi-band relaxation requirement for band n263 | Apple | Observation 1: Band n263 antenna elements are expected to be highly susceptible to distortions from asymmetries in the module, preventing effective beamforming. Observation 2: mm-Wave modules are required to use a high aspect ratio in order to avoid excessive growth in product thickness. This creates significant limitations for multi-band designs. Observation 3: Large gain swings due to the lensing from electrically thick layers, which is expected to be even more pronounced in co-located configurations with FR2-1 antennas. Observation 4: The impairments coming from the lensing and cavity effect cause pattern distortion resulting in element gain degradation, which needs to be considered for the derivation of MBR. Proposal 1: Introduce the multi-band relaxation requirement for band n263 as provided in Table 1. Table 1: UE multi-band relaxation factors for power class 3 Band; MBP,n (dB); MBS,n (dB). n257; 0.73; 0.73. n258; 0.6; 0.7. n259; 0.5; 0.4. n260; 0.51; 0.41. n261; 0.52,4; 0.74. n262; 0.7; 0.7. n263; 1.0; 1.0. Note 1: n260 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n260 Note 2: n261 peak relaxation is 0 dB for UE that exclusively supports n261+n260 Note 3: n257 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 Note 4: n261 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257. |
| R4-2209509 Further discussion and draft reply LS on minimum guard symbol of SRS | Xiaomi | Observation 1: Since RAN1 only ask for the minimum guard time for SRS antenna switching for 480 and 960 kHz SCS, the guard symbol of 120 kHz as 2 symbols should be left un-changed. Observation 2: As 120 kHz is mandatory supported in FR2-2, it is reasonable to reuse the 15us antenna switching time in FR2-2 for 480 and 960 kHz SCS. Proposal 1: To re-use the 15 us antenna switching time as 120 kHz SCS for both 480 and 960 kHz SCS. Proposal 2: It is proposed faster SRS antenna switching to be considered in Rel-18. Reply LS content: Question to RAN4: How many symbol(s) is/are needed to accommodate the required minimum guard time for SRS antenna switching for 480 and 960 kHz respectively, in FR2-2? Considering the SRS antenna switching for different antenna ports have been agreed as 15us since Rel-15 for both FR1 and FR2-1, it is agreed in RAN4 to reuse the 15us for FR2-2 in Rel-17. Further number of guard symbols are left for RAN1 decision. |
| R4-2209716 UE features for NR in 52.6 GHz - 71 GHz | Nokia, Nokia Shanghai Bell | Proposal 1: As each SCS is optional to support, further optionality on maximum channel bandwidth support is not required. Proposal 2: For optional ON-ON transient time, only one value among 1 us or 2 us is specified. |
| R4-2210080 draft CR 38.101-3 on FR2-2 DC/CA with FR1 anchor | Ericsson | New combinations including NR FR2-2 licensed band [n264] are introduced to FR1 + FR2-2 DC/CA combinations. Added combinations are CA_n41- [n264], CA_n77-[n264], and CA_n79-[n264]. |

## 1.2	Open issues summary

### 1.2.1		Sub-topic 1-1: UE feature list

R4-2208045 proposes two UE features to support FR2-2 operation. These are listed below along with the corresponding NR_ext_to_71GHz feature list table.

- Channel bandwidth for a single CC

- Improved ON/ON transient period capability

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2-2 channel bandwidths for each SCS in each band for DL and UL for a single CC | Support of FR2-2 channel bandwidths 1) 120 kHz SCS: {100, 400} MHz CBW 2) 480 kHz SCS: {400, 800, 1600} MHz CBW 3) 960 kHz SCS: {400, 800, 1600, 2000} MHz CBW | FFS | Yes | No | UE cannot support some UE channel bandwidths | Per band | N/A | Applicable to FR2-2 only | N/A | UE indicating the support of specific SCS per band (RAN1 features X-Y) is required to support all CBWs corresponding to this SCS No additional capability signalling is needed. | NA |
| [Improved ON/ON transient period] | 1) Support of improved ON/ON transient period of X < 5us (X is FFS) | FFS | Yes | No | UE does not support improved ON/ON transient period and support 5us transient period | Per UE | N/A | Applicable to FR2-2 only | N/A | Further RAN4 discussion is required on whether to support improved ON/ON transient period and X value | Optional with capability signalling |

Issue 1-1: UE feature list

- Recommended WF

  - Mandatory channel bandwidth is covered in Sub-topic 2-2 of this document. Therefore, Moderator suggests companies provide feedback on the channel bandwidths for each SCS feature in Issue 2-2: Mandatory channel bandwidth of this document, as well as in the Rel-17 UE feature list dedicated thread [136]. If needed based on discussion progress, we can further discuss this feature in round 2.

  - Comments or feedback for the improved ON/ON transient period feature should be provided to thread [132] NR_ext_to_71GHz_Part_2. Pending the discussion outcome in thread [132], the improved ON/ON transient period can be further discussed in this thread during the second round of discussions.

### 1.2.2	Sub-topic 1-2: LS reply to RAN1 on minimum guard period between two SRS resources for antenna switching

RAN4 received an LS from RAN1 posing the question below (R1-2200796):

Question to RAN4: How many symbol(s) is/are needed to accommodate the required minimum guard time for SRS antenna switching for 480 and 960 kHz respectively, in FR2-2?

For reference, the following was specified in Rel-15 (38.214 Section 6.2.1.2) for subcarrier spacing ∆f up to 120 kHz:

Table 6.2.1.2-1: The minimum guard period between two SRS resources of an SRS resource set for antenna switching

|  |  | Y [symbol] |
| --- | --- | --- |
| 0 | 15 | 1 |
| 1 | 30 | 1 |
| 2 | 60 | 1 |
| 3 | 120 | 2 |

While a reply LS was discussed during RAN4 102e, no agreement was reached as there were different views regarding the SRS antenna switching time in FR2 (R4-2206433).

Issue 1-2: LS reply to RAN1

- Proposals

  - Option 1: CATT, R4-2208143

    - Answer from RAN4: The absolute switching time for FR2-2 is the same as the capability evaluated in R15, i.e., the antenna switching time is 15 µsec. The detail evaluation results for every possible switching scenario are included in the R15 reply LS R4-1710048 [1]. The decision on the symbol(s) needed to accommodate the required minimum guard time for SRS antenna switching for 480 and 960 kHz SCS is up to RAN1 discussion based on the absolute switching time in R4-1710048.

  - Option 2: Huawei, R4-2208763

    - In Rel-15 RAN4 had discussed on the guard period between two SRS resources for antenna switching and agreed on the value of 15us regardless of SCS type. For 480 and 960 kHz SCS in question, the same 15us guard period applies.

    - The guard period would have impact on demodulation of the symbol within the period, therefore blanked symbols would be helpful to guarantee the performance.

  - Option 3: Xiaomi, R4-2209509

    - Considering the SRS antenna switching for different antenna ports have been agreed as 15us since Rel-15 for both FR1 and FR2-1, it is agreed in RAN4 to reuse the 15us for FR2-2 in Rel-17. Further number of guard symbols are left for RAN1 decision.

- Recommended WF

  - Considering the timeline, 15µs can be reported for Rel-17 and future studies to improve this value (faster SRS antenna switching) can be considered in Rel-18 (Xiaomi, R4-2209509).

    - If agreeable, draft LS reply R4-2208143 can be taken as baseline and updated accordingly.

  - Companies are encouraged to share their views on the recommended WF and any comments they may have for the LS reply.

### 1.2.3	Sub-topic 1-3: Specification updates

Issue 1-3a: TS 38.104 updates

Draft CR R4-2208541 introduces relevant licensed band n264 content to various clauses of TS 38.104. These include operating band definition (sub-clause 5.2), channel bandwidths and SCS (sub-clause 5.3), nominal channel spacing, channel raster and sync raster (sub-clause 5.4), step frequencies for Tx and Rx spurious emission limits (sub-clause 9.7 and 10.7).

- Recommended WF

  - Moderator suggests companies provide any feedback on draft CR R4-2208541 directly into Section 1.3.2 CRs/TPs comments collection. Please note that CR contains updates to channel arrangement sub-clauses (nominal channel spacing, channel raster and sync raster).

Issue 1-3b: TS 38.101-2 updates

Draft CR R4-2208617 introduces relevant FR2-2 updates to Clause 5 content of TS 38.101-2, including definition of FR2-2, inclusion of NR band n263, NR CA band CA_n263 for intra-band contiguous CA, NRB, minimum guardband, channel bandwidths, channel raster and sync raster.

- Recommended WF

  - Moderator suggests companies provide any feedback on draft CR R4-2208617 directly into Section 1.3.2 CRs/TPs comments collection. Please note that CR contains updates channel raster and sync raster content.

Issue 1-3c: TS 38.101-3 updates

Draft CR R4-2210080 introduces new combinations with NR FR2-2 licensed band [n264] to FR1 + FR2-2 DC/CA combinations. Added combinations are CA_n41- [n264], CA_n77-[n264], and CA_n79-[n264].

- Recommended WF

  - Moderator suggests companies provide any feedback on draft CR R4-2210080 directly into Section 1.3.2 CRs/TPs comments collection.

## 1.3	Companies views’ collection for 1st round

### 1.3.1		Open issues

Issue 1-1: UE feature list

- Please provide feedback for CBW/SCS feature to Issue 2-2: Mandatory channel bandwidth of this document and dedicated Rel-17 UE feature list thread [136]

- Comments for improved ON/ON transient period feature should be provided to thread [132] NR_ext_to_71GHz_Part_2

Issue 1-2: LS reply to RAN1

| Company | Comments |
| --- | --- |
| vivo | We are OK to use Option 1 as baseline. From these options, at least we can agree on 15us for SRS antenna switching time. The number of guard symbols can be left to RAN1. |
| OPPO | Ok with 15us antenna switching time. Whether optimization of this switching time further in Rel-18 can be further discussed. At this moment it is unclear whether some WID can be used to include this new item or TEI can be leveraged. |
| Nokia, Nokia Shanghai Bell | While 15 us was agreed in FR2-1 in rel-15, there was also discussion on further studying faster SRS antenna switching time. It would make sense to consider faster switching time for FR2-2 given the higher SCS and loss of multiple symbols if 15 us switching time is maintained. |
| Huawei | All options are aligned in terms of the 15us reused for Rel-17. Aspects such as decision on number of guard symbols, or impact on demodulation may be captured in the LS to RAN1. Rel-18 scoping is not RAN4 duty, so we suggest not to spend time on such discussion. |
| ZTE | We are fine with the legacy values 15 us to reply to RAN1. |
| Xiaomi | Considering the timeline we think 15us can be agreed and to be sent to RAN1. For the faster switching time we think it make sense to consider it earlier and try to figure out some place for this work since it is quite important for larger SCS. Otherwise it will be the same as the earlier discussion that we have already agreed to improve the switching time while no further discussion in the group. |
| CATT | For the LS, we think only the agreements can be captured to reply RAN1. For the improvements can be another discussion. For the number of guard symbols, the preference from companies in last meeting was that it should be left to RAN1. We think it’ll be ok. RAN1 can decide the physical layer design details according to RAN4 feedback on the time. |
| Ericsson | The recommended WF is not agreeable. RAN1 is asking for a "real number", otherwise RAN1 will scale the existing (reuse the 15 us = 10 us transient + 5 us switch time for FR1). Comparing to FR1, there is no requirement for SRS antenna switching for FR2-1 and the basic transient period is 5 us for FR2-1. Switching between ports can be different if between polarizations and sub-arrays, for example. RAN4 should do due diligence rather than referring to a pre-R15 estimate from 2017. |
| Apple | We are OK to use 15 us and leave the exact number of guard symbols for RAN1 to decide. |
| QCOM | We are fine with 15us and guard decided by RAN1 |

### 1.3.2	CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest focusing on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2208541 | Nokia, Nokia Shanghai Bell: The contents of this draftCR have been technically endorsed already earlier, there is no need to re-endorse them. |
|  | Huawei: it seems that this draft CR is merging content of two already Endorsed draft CRs – is there any additional delta? However, there is one important agreement missing in this dCR, which is restriction of the 960k SCS for the Initial Access – that agreements was captured in another CR in R4-2209683 (section 2.3.2 of this summary). |
|  | ZTE: We have some concerns on CA nominal channel spacing, the current formula for FR2-2 is not fixed, in other words, from spec itself, it is no clear definition of CA channel spacing defined |
|  | Samsung: we would like to clarify the understanding on sync entries to be specified for licensed band n264. It seems the original online agreement is for unlicensed operation in n263. Not quite understand why this agreement should be extended to licensed case. We would prefer to remove SSB for 960k for n264 even though adding note also works. |
|  | Ericsson: The intention is to capture all n264 aspects that was technically endorsed at previous meeting. The reason is to have a draft CR that is based on latest version of the specification. If technical issues still exists we encourage companies to provide technical input. The information here is already technically endorsed based on previous specification versions. Maybe it would be good with some more detailed instructions from the rapporteur how to handle n264. |
| R4-2208617 | Nokia, Nokia Shanghai Bell: Given the status of no-LBT operation for this operating band, we think the note limiting the band for shared spectrum access defined in TS 37.213 shall be removed. Alternative note could be added to state: “This band is only applicable subject to regional and/or country specific restrictions” similar to what was discussed for n96/n102. Further updates may be needed when channel raster discussion has concluded. This cannot be endorsed as it is now given that discussion is on-going. |
|  | Huawei: table 5.4.3.3-2 seems to be missing. SU values discussion is still ongoing so entries in e.g. Table 5.3.2-1 are not to be considered as final. Table 5.3.5-1: the editor’s note on mandatory/optional CBW: discussion on the UE feature list is not expected to modify this table, irrespective of the decision. There are two other CRs to TS 38.101-2 for n263 in R4-2208047 and R4-2210117 – to be coordinated/aligned to keep single dCR. |
|  | ZTE: Please don’t the renumber the exiting table, since it might have been reference somewhere else, In addition, NR-ARFCN is still under the discussion and it should be updated with the agreement reached in this meeting. Table: 5.3.3-3: Minimum guardband (kHz) of SCS 240 kHz SS/PBCH block in FR2-1 |
|  | Apple: Further updates are needed as both SU and channel/Sync. raster discussions are ongoing On the note in the band table, it seems appropriate for unlicensed band while we are open to further discussion. |
| R4-2210080 | Nokia, Nokia Shanghai Bell: While the content is technically valid, this CR needs to wait for licensed operation for FR2-2 being introduced into specifications. |
|  | Huawei: we would like to get some clarification from the proponents on the need for this CR. Our current understanding is that introduction of inter-band FR2-2 combinations for n263 was motivated by the goal to allow “release independence from Rel-17” in future. Whether, similar approach is needed for n264 requires some clarifications. |
|  | ZTE: This could be added after the licensed band n264 is introduced or all requirements are stable enough. |
|  | Apple: we also share the view that this CR is not needed at this moment as n264 is yet to be introduced. |

## 1.4	Summary for 1st round

### 1.4.1	Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #1-1: UE feature list | Mandatory channel bandwidth agreement (Sub-topic 2-2): Agreement: Mandatory channel bandwidths - 120 kHz: mandatory (100 MHz, 400 MHz) - 480 kHz: mandatory (400 MHz), optional (800 MHz, 1600 MHz) - 960 kHz: mandatory (400 MHz,), optional (800MHz, 1600 MHz, 2000 MHz) Based on the above, the channel bandwidth for a single CC feature was agreed during GTW session: UE support of CBW for 480kHz SCS; Support of {800, 1600} CBW for 480kHz SCS; Support of 480kHz SCS; Yes; No; The network does not know if UE can transmit or receive with a specific CBW; Per Band; N/A; Applicable to FR2-2 only; N/A; 400 MHz is a mandatory CBW if the UE supports 480 kHz SCS; Optional with capability signalling. UE support of CBW for 960kHz SCS; Support of {800, 1600, 2000} CBW for 960kHz SCS; Support of 960kHz SCS; Yes; No; The network does not know if UE can transmit or receive with a specific CBW; Per Band; N/A; Applicable to FR2-2 only; N/A; 400 MHz is a mandatory CBW if the UE supports 960 kHz SCS; Optional with capability signalling. For improved ON/ON transient period, no consensus has been reached in thread [132]. Recommendations for 2nd round: No further discussions needed for channel bandwidth feature. Continue discussions for ON/ON transient period in thread [132]. |
| Sub-topic #1-2: LS reply to RAN1 | Candidate options: - Option 1: CATT, R4-2208143 - Answer from RAN4: The absolute switching time for FR2-2 is the same as the capability evaluated in R15, i.e., the antenna switching time is 15 µsec. The detail evaluation results for every possible switching scenario are included in the R15 reply LS R4-1710048 [1]. The decision on the symbol(s) needed to accommodate the required minimum guard time for SRS antenna switching for 480 and 960 kHz SCS is up to RAN1 discussion based on the absolute switching time in R4-1710048. - Option 2: Huawei, R4-2208763 - In Rel-15 RAN4 had discussed on the guard period between two SRS resources for antenna switching and agreed on the value of 15us regardless of SCS type. For 480 and 960 kHz SCS in question, the same 15us guard period applies. - The guard period would have impact on demodulation of the symbol within the period, therefore blanked symbols would be helpful to guarantee the performance. - Option 3: Xiaomi, R4-2209509 - Considering the SRS antenna switching for different antenna ports have been agreed as 15us since Rel-15 for both FR1 and FR2-1, it is agreed in RAN4 to reuse the 15us for FR2-2 in Rel-17. Further number of guard symbols are left for RAN1 decision. Majority view is that 15 µs can be reported for SRS antenna switching time and Option 1 (R4-2208143) can be taken as baseline for the LS reply. Recommendations for 2nd round: Use Option 1 as baseline for LS reply to RAN1 and further discuss edits needed, wording and necessary aspects to be captured in our reply. |

### 1.4.2	CRs/TPs

Note: The tdoc decisions are provided in Section 4

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2208541 | To be revised CR consolidates technically endorsed content to be included in TS 38.104 for band n264. Content of CR R4-2209683 needs to be merged into this CR. |
| R4-2208617 | To be revised |
| R4-2210080 | Postponed until licensed operation (n264) is introduced in FR2-2 |

# Topic #2: Operation bands and system parameters (AI 9.15.2)

## 2.1	Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207780 Remaining issues on system parameters for NR operation in 52.6GHz – 71GHz | Apple | Observation 1: Since 100.8MHz is a multiple of 960kHz, CA of different CBWs and different SCSs can be supported without issues when larger CBWs are formed by aggregating consecutive 100MHz channels. Proposal 1: Confirm the use of 100.8MHz as the step size to define channel raster. Proposal 2: Use 100MHz CBW with 100.8MHz channel spacing as the building block for larger CBWs. Proposal 3: The channel placement proposal in [R4-2201924] can be used as the baseline. Proposal 4: It is preferred to seek some degree of alignment with IEEE 802.11ad/ay channels when deciding the NR channel placement. If needed, we can consider specifying two flavors of fixed channelization, one with channel alignment with IEEE channels and one without. Observation 2: No clear benefit of letting unlicensed bands use the subset of sync raster of licensed bands. Proposal 5: The optionality of CBW is agreed as follows: - 120 kHz: mandatory (100 MHz), optional (400 MHz) - 480 kHz: mandatory (400 MHz), optional (800 MHz, 1600 MHz) - 960 kHz: mandatory (400 MHz), optional (800 MHz, 1600 MHz, 2000 MHz) |
| R4-2208046 Further views on channelization for FR2-2 | Intel Corporation | Proposal 1: (Channel Raster Option 1) - for 100 MHz channel bandwidth, NREF = {2564083 + 1680*N, N = 0:137} - for 400 MHz channel bandwidth, NREF = {2571643, 2578363, 2585083, 2591803, 2598523, 2606923, 2613643, 2620363, 2627083, 2633803, 2642203, 2648923, 2655643, 2662363, 2669083, 2679163, 2685883, 2692603, 2699323, 2706043, 2714443, 2721163, 2727883, 2734603, 2741323, 2751403, 2758123, 2764843, 2771563, 2778283, 2786683, 2600203, 2640523, 2647243, 2653963, 2660683, 2667403, 2674123} - for 800 MHz channel bandwidth, NREF = {2575003, 2588443, 2603563, 2617003, 2630443, 2645563, 2659003, 2672443, 2689243, 2702683, 2717803, 2731243, 2744683, 2761483, 2774923, 2788363, 2581723, 2595163, 2610283, 2623723, 2638843, 2652283, 2665723, 2682523, 2695963, 2711083, 2724523, 2737963, 2754763, 2768203, 2781643} - for 1600 MHz channel bandwidth, NREF = {2581723, 2623723, 2652283, 2695963, 2724523, 2768203, 2610283, 2637163, 2664043, 2753083, 2781643} - for 2000 MHz channel bandwidth, NREF = {2585083, 2620363, 2655643, 2692603, 2727883, 2764843} Proposal 2: (Sync Raster Option 1) - For 120 kHz Pcell and Pscell, GSCN = {24157 + 6*N – floor((N-2)/6) – 1, N=0:137} - For 480 kHz Pcell and Pscell, GSCN = {24180, 24203, 24227, 24250, 24273, 24279, 24303, 24326, 24349, 24373, 24396, 24402, 24419, 24425, 24443, 24448, 24466, 24472, 24489, 24495, 24513, 24518, 24536, 24553, 24577, 24600, 24623, 24647, 24653, 24676, 24699, 24723, 24746, 24769, 24804, 24828, 24851, 24874, 24898, 24927} - For 960 kHz, no applicable SS raster entries exist for Pcell and Pscell Proposal 3: (Channel Raster Option 2) - for 100 MHz channel bandwidth, NREF = {2564083 + 1680*N, N = 0:137} - for 400 MHz channel bandwidth, NREF = {2566603 + 6720*N*4, N = 0:33} - for 800 MHz channel bandwidth, NREF = {2569963 + 13440*N, N = 0:16; 2569963 + 6720 + 13440*M, M = 0: 15} - for 1600 MHz channel bandwidth, NREF = {2576683 + 6720*M + 26880*N, M=0:2, N =0:7} - for 2000 MHz channel bandwidth, NREF = {2585083, 2620363, 2655643, 2692603, 2727883, 2764843} Proposal 4: (Sync Raster Option 2) - For 120 kHz Pcell and Pscell, GSCN = {24156 + 6*N – 3*floor((N+4)/18), N=0:137} - For 480 kHz Pcell and Pscell, GSCN = {24162 + 24*N – 12*floor((N+4)/18), N = 0:33} - For 960 kHz, no applicable SS raster entries exist for Pcell and Pscell |
| R4-2208047 Draft CR to introduce the channel and sync rasters of band n263 (option 1) | Intel Corporation | Introduces channel and synchronization raster entries of band n263 to TS 38.101-2 based on Option 1 of R4-2208046 |
| R4-2208048 Draft CR to introduce the channel and sync rasters of band n263 (option 1) | Intel Corporation | Introduces channel and synchronization raster entries of band n263 to TS 38.104 based on Option 1 of R4-2208046 |
| R4-2208479 System parameters for a NR band in the range 52.6GHz – 71GHz | Nokia, Nokia Shanghai Bell | Observation 1: Raster based purely on 100.8 MHz step size based on 100 MHz ChBW does not enable carrier aggregation between 100 MHz and other channel bandwidths and wastes spectrum. Observation 2: Adding additional raster points for wider channels to allow placing channel next to band edge does not resolve the issues for CA and spectrum waste. Proposal 1: Adopt synchronization raster points shown in Table 1 for n263 Table 1 GSCNs in fixed channelization SCS; 120 kHz; 480 kHz; 960 kHz. Bandwidth; 100 MHz; 400 MHz; 400 MHz; 800 MHz; 1600 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. 24153; 24153; 24157; 24157; 24157; 24160; 24160; 24166; 24166. 24159; 24181; 24184. 24165; 24205; 24205; 24208; 24208. 24171; 24229; 24232. 24177; 24177; 24253; 24253; 24253; 24256; 24256; 24256. 24183; 24277; 24280; 24280. 24189; 24301; 24301; 24304; 24304. 24195; 24325; 24322. 24201; 24201; 24349; 24349; 24349; 24346; 24346; 24352. 24207; 24373; 24370. 24213; 24397; 24397; 24394; 24394; 24400. 24219; 24421; 24418. 24225; 24225; 24445; 24445; 24445; 24442; 24442; 24442. 24231; 24469; 24466. 24234; 24493; 24493; 24490; 24490. 24240; 24517; 24514; 24514. 24246; 24246; 24529; 24541; 24541; 24532; 24538; 24538. 24252; 24553; 24556. 24258; 24577; 24577; 24580; 24580. 24264; 24601; 24604. 24270; 24270; 24625; 24625; 24625; 24628; 24628; 24628; 24628. 24276; 24649; 24652. 24282; 24673; 24673; 24676; 24676. 24288; 24697; 24700. 24294; 24294; 24721; 24721; 24721; 24724; 24724; 24724. 24300; 24745; 24742; 24748. 24306; 24769; 24769; 24766; 24766. 24312; 24793; 24790. 24318; 24318; 24817; 24817; 24817; 24814; 24814; 24814. 24324; 24841; 24838. 24330; 24865; 24865; 24862; 24862. 24336; 24889; 24886. 24339; 24342; 24913; 24913; 24910; 24910. 24345; 24937; 24934. 24351. 24357. 24363; 24363. 24369. 24375. 24381. 24387; 24387. 24393. 24399. 24405. 24411; 24411. 24417. 24423. 24429. 24435; 24435. 24441. 24444. 24450. 24456; 24456. 24462. 24468. 24474. 24480; 24480. 24486. 24492. 24498. 24504; 24504. 24510. 24516. 24522. 24528; 24528. 24534. 24540. 24546. 24549; 24552. 24555. 24561. 24567. 24573; 24573. 24579. 24585. 24591. 24597; 24597. 24603. 24609. 24615. 24621; 24621. 24627. 24633. 24639. 24645; 24645. 24651. 24654. 24660. 24666; 24666. 24672. 24678. 24684. 24690; 24690. 24696. 24702. 24708. 24714; 24714. 24720. 24726. 24732. 24738; 24738. 24744. 24750. 24756. 24759; 24762. 24765. 24771. 24777. 24783; 24783. 24789. 24795. 24801. 24807; 24807. 24813. 24819. 24825. 24831; 24831. 24837. 24843. 24849. 24855; 24855. 24861. 24864. 24870. 24876; 24876. 24882. 24888. 24894. 24900; 24900. 24906. 24912. 24918. 24924; 24924. 24930. 24936. 24942. 24948. 24954. Proposal 2: RF channel raster is fixed based on 100.8 MHz step size for 100 MHz ChBW and 50.4 MHz step size for wider channel bandwidths, network implementation will take care that SSB and coreset#0 fit within channel bandwidth. |
| R4-2208540 Draft CR to TS 38.104: Addition of FR2-2definition in subclause 5.1, n263 in subclause 5.2 and transmission bandwidth information in subclause 5.3 | Ericsson | Adds missing content related to band n263 to Clause 5 of TS 38.104. Additions includes FR2-2 definition, transmission bandwidth configuration, minimum guardband, channel bandwidth and SCS. |
| R4-2208618 Further discussion on channel raster and sync raster for NR extending to 71GHz | vivo | Licensed operation Observation 1: Based on the newly agreed SU numbers, GSCN step sizes for different SSB SCSes are 3 for 120kHz, 15 for 480kHz, and 8 for 960kHz. Proposal 1: Further check GSCN step size based on the agreed SU numbers for licensed bands. Unlicensed operation Observation 2: The step size of 1680 (100.8MHz) apply for the channel raster of 100MHz fixed channels. Proposal 2: Use the fixed RF channel raster with the step size of 6667 (400.02 MHz), 13334 (800.04 MHz), 26667 (1600.02 MHz), 33334 (2000.04 MHz) to derive the channel raster for 400, 800, 1600, 2000 MHz channel bandwidths for the unlicensed band in FR2-2. Proposal 3: Use GSCN step size of 5 for 120kHz, 23 for 480 kHz, 23 for 960 kHz to derive the sync raster number for unlicensed band. |
| R4-2208649 60GHz channel and synchronization raster | LGE | Proposal 1: Aligned with proposal in previous meeting [R4-2205233] and utilizes the floating 960kHz ARFCN raster and ~100MHz (combination of 99.86/100.8MHz) SSB raster for 120kHz SCS. SSB raster locations for 480k SCS are down selected from 120kHz SCS SSB raster. This proposal is re-submitted as it utilizes the full 14 000MHz of spectrum and provides a lot of freedom for selecting the channel frequencies and flexibility for intra-band CA combinations. Proposal 2: Uses the 100.8MHz fixed ARFCN and SSB raster. SSB raster locations for 480k SCS are down selected from 120kHz SCS SSB raster. RF channels are selected in a way that they target maximization of the spectrum usage and enable flexibility for difference CA combinations. Additional channel locations are added for alignment with 802.11 channels. Proposal #1; Proposal #2. Channel raster type; Floating; Fixed. Channel raster; 960kHz; 100.8MHz for 100MHz CBW Raster steps for wider CBWs are multiples of 100.8MHz. Flexibility to support different intra-band CA combinations; Very high; High. 120kHz SCS SSB raster; Fixed One SSB per 100MHz 140 locations; Fixed One SSB per 100.8MHz 138 locations. 480kHz SCS SSB raster; Fixed Two SSB per 400MHz 70 locations; Fixed One SSB per 400MHz 54 locations. Alignment with 802.11 possible; YES; YES. |
| R4-2209139 52.6-71 GHz System Parameters | Ericsson | Spectrum utilization Observation 1: Given challenges for larger arrays and bandwidths at this frequency range compared to FR2-1, a lower spectral utilization should be considered. Since continuous CA is supported as part of the agreement additional bandwidth can be realized using continuous CA as an approach rather than increase spectral utilization. Proposal 1: Spectral utilization consideration captured in Table 1. Table 1: Spectral utilization for 57 – 71 GHz SCS (kHz); 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB. 120; 62; 248; N/A; N/A; N/A. 480; N/A; 61; 120; 238; N/A. 960; N/A; 30; 60; 119; 148. Fixed channelization – CA support in FR2-2 Observation 2: Contiguous carrier on an FFT-aligned grid is needed to allow for support of continuous CA Fixed channelization – ARFCN and SS raster Observation 3: The proposed fixed channelization design supports intra-band contiguous CA as required by the previous RAN4 agreement. Observation 4: When channels are placed in continuous fashion spectral efficiency is optimal. Proposal 2: Adopt the fixed channelization design detailed in Section 2.1.2. Table 5.4.2.3-2: Applicable NR-ARFCN per operating band in FR2 NR operating band; ΔFRaster (kHz); Uplink and Downlink range of NREF (First – <Step size> – Last). n257; 60; 2054166 – <1> – 2104165. 120; 2054167 – <2> – 2104165. n258; 60; 2016667 – <1> – 2070832. 120; 2016667 – <2> – 2070831. n259; 60; 2270832 – <1> – 2337499. 120; 2270832– <2> – 2337499. n260; 60; 2229166 – <1> – 2279165. 120; 2229167 – <2> – 2279165. n261; 60; 2070833 – <1> – 2084999. 120; 2070833 – <2> – 2084999. n263Note 1; 120; 2563333 <2> 2794999. 480; 2565835 <8> 2792499. 960; 2565835 <16> 2792491. Note1: Applicable NR-ARFCN for band n263 For 120 kHz SCS and 100 MHz channels: 2563341 <1680> 2793501 For 120 kHz SCS and 400 MHz channels: 2565861 <6720> 2787621 For 480 kHz SCS and 400 MHz channels: 2565867 <6720> 2787627 For 480 kHz SCS and 800 MHz channels: 2569227 <13440> 2784267 For 480 kHz SCS and 1600 MHz channels: 2575947 <26880> 2764107 For 960 kHz SCS and 400 MHz channels: 2565867 <6720> 2787627 For 960 kHz SCS and 800 MHz channels: 2569227 <13440> 2784267 For 960 kHz SCS and 1600 MHz channels: 2575947 <26880> 2764107 For 960 kHz SCS and 2000 MHz channels: 2579307 <33600> 2747307. Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2) NR operating band; SS Block SCS; SS Block pattern (note); Range of GSCN (First – <Step size> – Last). n257; 120 kHz; Case D; 22388 – <1> – 22558. 240 kHz; Case E; 22390 – <2> – 22556. n258; 120 kHz; Case D; 22257 – <1> – 22443. 240 kHz; Case E; 22258 – <2> – 22442. n259; 120 kHz; Case D; 23140 – <1> – 23369. 240 kHz; Case E; 23142 – <2> – 23368. n260; 120 kHz; Case D; 22995 – <1> – 23166. 240 kHz; Case E; 22996 – <2> – 23164. n261; 120 kHz; Case D; 22446 – <1> – 22492. 240 kHz; Case E; 22446 – <2> – 22490. n263Note 1; 120 kHz; Case D; 24153 <1> 24960. 480 kHz; Case F; 24157 <4> 24949. 960 kHz; Case G; 24160 <8> 24952. NOTE: SS Block pattern is defined in section 4.1 in TS 38.213 [10]. NOTE 1: The following GSCN are allowed for operation in band n263: For operation with 120 kHz SCS: GSCN = {24154, 24160, 24166, 24172, 24177, 24183, 24189, 24195, 24201, 24207, 24212, 24218, 24224, 24230, 24236, 24242, 24247, 24253, 24259, 24265, 24271, 24277, 24282, 24288, 24294, 24300, 24306, 24312, 24317, 24323, 24329, 24335, 24341, 24347, 24352, 24358, 24364, 24370, 24376, 24382, 24387, 24393, 24399, 24405, 24411, 24417, 24422, 24428, 24434, 24440, 24446, 24452, 24457, 24463, 24469, 24475, 24481, 24487, 24492, 24498, 24504, 24510, 24516, 24522, 24527, 24533, 24539, 24545, 24551, 24557, 24562, 24568, 24574, 24580, 24586, 24592, 24597, 24603, 24609, 24615, 24621, 24627, 24632, 24638, 24644, 24650, 24656, 24662, 24667, 24673, 24679, 24685, 24691, 24697, 24702, 24708, 24714, 24720, 24726, 24732, 24737, 24743, 24749, 24755, 24761, 24767, 24772, 24778, 24784, 24790, 24796, 24802, 24807, 24813, 24819, 24825, 24831, 24837, 24842, 24848, 24854, 24860, 24866, 24872, 24877, 24883, 24889, 24895, 24901, 24907, 24912, 24918, 24924, 24930, 24936, 24942, 24947, 24953, 24959, 24965} For operation with 480 and 960 kHz SCS: GSCN = {24163, 24187, 24210, 24233, 24257, 24280, 24303, 24327, 24350, 24373, 24397, 24420, 24443, 24467, 24490, 24513, 24537, 24560, 24583, 24607, 24630, 24653, 24677, 24700, 24723, 24747, 24770, 24793, 24817, 24840, 24863, 24887, 24910, 24933, 24957}. |
| R4-2209682 Draft CR to TS 38.101-2: Channel arrangement and channel bandwidths for 66-71 GHz | Huawei, HiSilicon | Introduces NR band n264 (licensed operation) and its corresponding channel raster and sync raster to TS 38.101-2 |
| R4-2209683 Draft CR to TS 38.104: Channel arrangement and channel bandwidths for 66-71 GHz | Huawei, HiSilicon | Introduces NR band n264 (licensed operation) and its corresponding channel raster and sync raster to TS 38.104 |
| R4-2210117 Draft CR to introduce the channel and sync rasters of band n263 (option 2) | Intel Corporation | Introduces channel and synchronization raster entries of band n263 to TS 38.101-2 based on Option 2 of R4-2208046 |
| R4-2210118 Draft CR to introduce the channel and sync rasters of band n263 (option 2) | Intel Corporation | Introduces channel and synchronization raster entries of band n263 to TS 38.104 based on Option 2 of R4-2208046 |
| R4-2210188 60GHz UE bandwidths | Qualcomm Incorporated | Proposal: 120 kHz SCS 100 MHz (UE mandatory), 400 MHz (UE optional) Proposal: 480 kHz SCS: 400 MHz (UE mandatory): other CCBWs UE optional Proposal: 960 kHz SCS: 400 MHz (UE mandatory): other CCBWs UE optional |

## 2.2	Open issues summary

### 2.2.1	Sub-topic 2-1: Channelization

Issue 2-1a: Channelization for unlicensed operation

- General proposals

  - Proposal 1: Confirm the use of 100.8MHz as the step size to define channel raster. (Apple, R4-2207780)

  - Proposal 2: Use 100MHz CBW with 100.8MHz channel spacing as the building block for larger CBWs. (Apple, R4-2207780)

  - Proposal 3: The channel placement proposal in [R4-2201924] can be used as the baseline. (Apple, R4-2207780)

  - Proposal 4: It is preferred to seek some degree of alignment with IEEE 802.11ad/ay channels when deciding the NR channel placement. If needed, we can consider specifying two flavors of fixed channelization, one with channel alignment with IEEE channels and one without

  - Proposal 5: The step size of 1680 (100.8MHz) apply for the channel raster of 100MHz fixed channels. (vivo, R4-2208618)

- Channel raster proposals

  - Proposal 1: Channel Raster Option 1 (Intel, R4-2208046)

    - for 100 MHz channel bandwidth, NREF = {2564083 + 1680*N, N = 0:137}

    - for 400 MHz channel bandwidth, NREF = {2571643, 2578363, 2585083, 2591803, 2598523, 2606923, 2613643, 2620363, 2627083, 2633803, 2642203, 2648923, 2655643, 2662363, 2669083, 2679163, 2685883, 2692603, 2699323, 2706043, 2714443, 2721163, 2727883, 2734603, 2741323, 2751403, 2758123, 2764843, 2771563, 2778283, 2786683, 2600203, 2640523, 2647243, 2653963, 2660683, 2667403, 2674123}

    - for 800 MHz channel bandwidth, NREF = {2575003, 2588443, 2603563, 2617003, 2630443, 2645563, 2659003, 2672443, 2689243, 2702683, 2717803, 2731243, 2744683, 2761483, 2774923, 2788363, 2581723, 2595163, 2610283, 2623723, 2638843, 2652283, 2665723, 2682523, 2695963, 2711083, 2724523, 2737963, 2754763, 2768203, 2781643}

    - for 1600 MHz channel bandwidth, NREF = {2581723, 2623723, 2652283, 2695963, 2724523, 2768203, 2610283, 2637163, 2664043, 2753083, 2781643}

    - for 2000 MHz channel bandwidth, NREF = {2585083, 2620363, 2655643, 2692603, 2727883, 2764843}

  - Proposal 2: Channel Raster Option 2 (Intel, R4-2208046)

    - for 100 MHz channel bandwidth, NREF = {2564083 + 1680*N, N = 0:137}

    - for 400 MHz channel bandwidth, NREF = {2566603 + 6720*N*4, N = 0:33}

    - for 800 MHz channel bandwidth, NREF = {2569963 + 13440*N, N = 0:16; 2569963 + 6720 + 13440*M, M = 0: 15}

    - for 1600 MHz channel bandwidth, NREF = {2576683 + 6720*M + 26880*N, , M=0:2, N =0:7}

    - for 2000 MHz channel bandwidth, NREF = {2585083, 2620363, 2655643, 2692603, 2727883, 2764843}

  - Proposal 3: RF channel raster is fixed based on 100.8 MHz step size for 100 MHz ChBW and 50.4 MHz step size for wider channel bandwidths, network implementation will take care that SSB and coreset#0 fit within channel bandwidth. (Nokia, R4-2208479)

  - Proposal 4: Use the fixed RF channel raster with the step size of 6667 (400.02 MHz), 13334 (800.04 MHz), 26667 (1600.02 MHz), 33334 (2000.04 MHz) to derive the channel raster for 400, 800, 1600, 2000 MHz channel bandwidths for the unlicensed band in FR2-2. (vivo, R4-2208618)

  - Proposal 5: Floating channel raster, 960kHz step size (LGE, R4-2208649)

  - Proposal 6: Fixed channel raster step size of 100.8MHz for 100MHz CBW, raster steps for wider CBWs are multiples of 100.8MHz (LGE, R4-2208649)

  - Proposal 7: Ericsson, R4-2209139

| NR operating band | ΔFRaster (kHz) | Uplink and Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n263Note 1 | 120 | 2563333 <2> 2794999 |
|  | 480 | 2565835 <8> 2792499 |
|  | 960 | 2565835 <16> 2792491 |

Discussions:

Nokia: Last time we have baseline 100.8MHz. But we identify the issue. 100.8MHz is not to aggregation of BWs. At least for wider bandwidth, we need addition raster, which is 50.4MHz. It enables the CA between different CBWs. We are OK with proposal 3 and 5.

Intel: For NR-U, we have rough similar issue. We wonder if such consideration is needed. The gap would be much fixed. One thing is that at least for 100MHz, all the companies suggested 1680*N for 100Mhz and 6720*N for 400Mhz. We can start with the simple ones, i.e., 100 and 400.

Vivo: to Nokia, we agreed 100.8MHz as baseline last meeting. This is only considered for 100Mhz. Proposal 2 is a good baseline to define the channel raster. For proposal 2, are proposal 1 and 2 the same? Based on the equation of proposal 2, can we derive proposal 1. For unlicensed band, there is total 14GHz. 2000MHz there should be 7 channel raster. But there are only 6 for 2000MHz. Does Intel consider alignment with IEEE?

Apple: if companies think there is some issue, we can have further adjustment. To Nokia, we do not know if the aggregation with 100MHz is really corner case.

Ericsson: alignment with IEEE is not needed. Why should we consider that? We have agreement that channel raster design should consider enabling continuous CA. Proposal 1 does not provide and leads to wider gap. We would like to support #7 and #5 is also OK.

Intel: To Vivo, proposal 1 is not the same as proposal 2. Proposal 2 does include the additional entries. For 2000MHz, if looking at the channel, it can fit 7 channel. We thought 6 channels would be sufficient. To Ericsson, we do not agree that the channel alignment with IEEE should be precluded. In terms of providing CA, the difference is the gap. I think we are flexible how to define it. #5 is against the previous agreement.

LGE: we propose the approach not to waste any spectrum when aggregating 100MHz. For numbers of channel for 2000MHz, we think more than 6 entries are needed. The previous agreement allows some flexibility to support different combinations.

Nokia: on the need of CA, we have not ruled out the contiguous CA. only 100MHz is mandatory. All the initial access should be done via 100MHz. With simple addition to add the shift to the baseline, we can enable CA well. On the need of 6 vs 7 for wider bandwidth, the simple way to address it is possible to place anywhere 100.8 raster or 50.4 raster.

Ericsson: for unequal channel bandwidth, the gap on either edge will be unequal. The fixed step will lead the unequal internal gap. Option 2 is better. Option 7 can also allow it. We should consider the unequal channel bandwidth aggregation.

Intel: we discussed the contiguous and non-contiguous CA in RAN and made the agreement. Why should we discuss it?

Ericsson: the agreement is not to have non-contiguous CA. I do not see the issue to bring the proposal considering CA here.

Nokia: some entries are needed for adding for 400MHz.

Intel: should we have strict contiguous CA. It is not possible to have contiguous CA for NR-U.

Apple: Given the discussion, it is beneficial to discuss the contiguous and non-contiguous CA. With increasing channel bandwidth, we may modify the channel spacing. There is slightly large gap between CCs. But it should be considered as the contiguous CA.

Ericsson: Regarding contiguous CA, we define in terms of gap between CCs, which should be lower than a certain value. There is no gap in-between the channel bandwidth in MHz. We are using the gap in-between. It is not possible to use the fixed raster to ensure no gap. We can design the raster to ensure no gap.

Apple: to Ericsson, when we define the normal spacing for CA, we did have floating raster points. Now we are going to define the fixed. Should we stick to channel normal spacing? Should we stick to fixed channel raster and design the channel spacing. We should not stick to channel normal spacing which is based on floating raster.

Ericsson: in order to have equal internal gap, we need floating raster.

Nokia: we are OK with fixed raster on the condition that 400Mhz is mandatory.

Ericsson: we can consider N*channel spacing. It is consistent with unlicensed band as well. N*20MHz, which allows us to have contiguous CA. There are case when gap in-between CC is too large.

Agreement

- Channel Raster Option 2 (Intel, R4-2208046)

  - for 100 MHz channel bandwidth, NREF = {2564083 + 1680*N, N = 0:137}

  - for 400 MHz channel bandwidth, NREF = {2566603 + 6720*N, N = 0:33}

- Further discuss the channel raster for wider channel bandwidth, i.e. 800MHz, 1600MHz, 2000MHz

- Sync raster proposals

  - Proposal 1: Sync Raster Option 1 (Intel, R4-2208046)

    - For 120 kHz Pcell and Pscell, GSCN = {24157 + 6*N – floor((N-2)/6) – 1, N=0:137}

    - For 480 kHz Pcell and Pscell, GSCN = {24180, 24203, 24227, 24250, 24273, 24279, 24303, 24326, 24349, 24373, 24396, 24402, 24419, 24425, 24443, 24448, 24466, 24472, 24489, 24495, 24513, 24518, 24536, 24553, 24577, 24600, 24623, 24647, 24653, 24676, 24699, 24723, 24746, 24769, 24804, 24828, 24851, 24874, 24898, 24927}

    - For 960 kHz, no applicable SS raster entries exist for Pcell and Pscell

  - Proposal 2: Sync Raster Option 2 (Intel, R4-2208046)

    - For 120 kHz Pcell and Pscell, GSCN = {24156 + 6*N – 3*floor((N+4)/18), N=0:137}

    - For 480 kHz Pcell and Pscell, GSCN = {24162 + 24*N – 12*floor((N+4)/18), N = 0:33}

    - For 960 kHz, no applicable SS raster entries exist for Pcell and Pscell

  - Proposal 3: Adopt synchronization raster points shown in Table 1 of the Annex – (Nokia, R4-2208479)

  - Proposal 4: Use GSCN step size of 5 for 120kHz, 23 for 480 kHz, 23 for 960 kHz to derive the sync raster number for unlicensed band. (vivo, R4-2208618)

  - Proposal 5: LGE, R4-2208649

    - 120kHz SCS SSB raster fixed, one SSB per 100MHz (140 locations)

    - 480kHz SCS SSB raster fixed, two SSB per 400MHz (70 locations)

    - Alignment with 802.11 possible

  - Proposal 6: LGE, R4-2208649

    - 120kHz SCS SSB raster fixed, one SSB per 100.8MHz (138 locations)

    - 480kHz SCS SSB raster fixed, one SSB per 400MHz (54 locations)

    - Alignment with 802.11 possible

  - Proposal 7: Ericsson, R4-2209139

| NR operating band | SS Block SCS | SS Block pattern (note) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n263Note 1 | 120 kHz | Case D | 24153 <1> 24960 |
|  | 480 kHz | Case F | 24157 <4> 24949 |
|  | 960 kHz | Case G | 24160 <8> 24952 |

- Recommended WF

  - Companies are encouraged to provide their views and preference, if any, on the proposals listed for general, channel raster and sync raster

Intel: Have one GSCN entry for each 100MHz.

Agreement:

- Have one GSCN entry corresponding to each channel with the smallest bandwidth for SCS.

- The total number is [138] entries for 120KHz SCS.

- Further discuss the GSCN for 480KHz and 960KHz SCS

Issue 2-1b: Channelization for licensed operation

- Options

  - Observation 1: Based on the newly agreed SU numbers, GSCN step sizes for different SSB SCSes are 3 for 120kHz, 15 for 480kHz, and 8 for 960kHz. (vivo, R4-2208618)

  - Proposal 1: Further check GSCN step size based on the agreed SU numbers for licensed bands. (vivo, 4-2208618)

- Recommended WF

  - Verify GSCN step size according to agreed SU numbers. Companies may share inputs on the above.

Discussion:

Nokia: For licensed operation, we have agreed on the CRs and sent to RAN1. It is not necessary at this time.

Ericsson: agree with Nokia. We do not know why SU is considered.

ZTE: Related to SSB

Intel: when the original, 90% is assumed. In last meeting, RAN4 agreed the updated SU. It does not mean the previous agreement does not work.

Ericsson: we do not change the agreement. Regarding SU we need discuss in the same way for licensed and unlicensed.

ZTE: we do not have proposal to further update.

### 2.2.2	Sub-topic 2-2: Channel bandwidth

Issue 2-2: Mandatory channel bandwidth

- Proposals

  - Option 1: The optionality of CBW is agreed as follows: (Apple R4-2207780, QCOM R4-2210188)

    - 120 kHz: mandatory (100 MHz), optional (400 MHz)

    - 480 kHz: mandatory (400 MHz), optional (800 MHz, 1600 MHz)

    - 960 kHz: mandatory (400 MHz), optional (800 MHz, 1600 MHz, 2000 MHz)

  - Option 2: As each SCS is optional to support, further optionality on maximum channel bandwidth support is not required. (Nokia R4-2209716, Intel R4-2208045)

  - Option 3: Other or potential compromise

- Recommended WF

  - Companies are encouraged to provide their views on the above options and which, if any, channel bandwidth per SCS should be optional/mandatory.

Discussion:

ZTE: UE vendors prefer to optionality for larger channel bandwidth. We propose mandate 400MHz for 120Khz and leave the other wider optional.

Nokia: We support ZTE proposal. Our compromise proposal is to mandate up to 800MHz channel bandwidth.

Apple: there are a number of challenges. For 480KHz and 960KHz, we have already compromised to mandate 400Mhz for 480KHz and 960KHz. We can compromise to accept mandating 400MHz for 120KHz SCS

Huawei: We are on the similar as Apple.

Mediatek: why should we include 400MHz mandatory?

Apple: to Mediatek, we also prefer option 1. But this is the last meeting. The current compromise would be best the group can get. The burden would be less demanding for UE to consider to support 480KHz and 960KHz SCS.

Agreement: Mandatory channel bandwidths

- 120 kHz: mandatory (100 MHz, 400 MHz)

- 480 kHz: mandatory (400 MHz), optional (800 MHz, 1600 MHz)

- 960 kHz: mandatory (400 MHz,), optional (800MHz, 1600 MHz, 2000 MHz)

### 2.2.3	Sub-topic 2-3: Specification updates

Issue 2-3a: TS 38.101-2 updates

- Draft CR R4-2208047: introduces channel and synchronization raster entries of band n263 based on Option 1 of R4-2208046

- Draft CR R4-2209682: introduces NR band n264 (licensed operation) and its corresponding channel raster and sync raster

- Draft CR R4-2210117: introduces channel and synchronization raster entries of band n263 based on Option 2 of R4-2208046

- Recommended WF

  - How to proceed will depend on outcome of channelization discussions. Moderator suggests companies provide any feedback on the above draft CRs directly into Section 2.3.2 CRs/TPs comments collection.

Issue 2-3b: TS 38.104 updates

- Draft CR R4-2208048: introduces channel and synchronization raster entries of band n263 based on Option 1 of R4-2208046

- Draft CR R4-2208540: adds missing content related to band n263 to Clause 5 (includes FR2-2 definition, transmission bandwidth configuration, minimum guardband, channel bandwidth and SCS

- Draft CR R4-2209683: introduces NR band n264 (licensed operation) and its corresponding channel raster and sync raster

- Draft CR R4-2210118: introduces channel and synchronization raster entries of band n263 based on Option 2 of R4-2208046

- Recommended WF

  - Moderator suggests companies provide any feedback on the above draft CRs directly into Section 2.3.2 CRs/TPs comments collection.

## 2.3	Companies views’ collection for 1st round

### 2.3.1	Open issues

Issue 2-1a: Channelization for unlicensed operation

Issue 2-1b: Channelization for licensed operation

| Company | Comments |
| --- | --- |
| vivo | Issue 2-1a: Channelization for unlicensed operation - General proposals In the last meeting, we agreed to use 100.8MHz to derive the stepsize. However, we thought this agreement only considered for 100MHz fixed channels. For other type of channel bandwidths, 100.8MHz is not suitable. • Channel raster proposals Option 1 and Option 2 are acceptable to us. - Sync raster proposals In the last meeting, we agreed to define GSCN for SSB SCS 960kHz and made a note that this is not used for initial access. For P1 and P2, we need to define GSCN for 960kHz, too. |
| Nokia, Nokia Shanghai Bell | Sub-topic 2-1a: 100.8 MHz step size for channel raster has an issue and cannot support contiguous carrier aggregation between 100 MHz and wider channel bandwidths. This comes from the fact that that 100 MHz CBW needs a raster point in the middle of 100 MHz, whereas the midpoint of 400 MHz CBW falls in between of 100 MHz channels. As a result, there is a ~50 MHz gap created in CA configuration mixing 100 MHz with other CBWs. This is especially problematic as some companies propose to mandatorily support only 100 MHz CBW, effectively meaning that initial access needs to be done with 100 MHz CBW. To resolve the situation, we propose using 50.4 MHz as a baseline step size at least for wider than 100 MHz CBWs. We are also ok with using floating raster with 960 kHz step size. Sub-topic 2-1b: We do not see need for further updates on licensed operation, given that the earlier agreement on channelization for licensed operation has been already communicated to RAN1 and is being implemented there. |
| Huawei | Sub-topic 2-1a: improving spectrum utilization of the intra-band CA sounds like a good motivation to adjust previous agreements. More discussion needed, to ease comparison of all the options listed. Sub-topic 2-1b: SU values are still under discussion this meeting – GSCN updates may need to be postponed a little. If there are further agreements for n264, good to keep them in a single updated Dcr, based on Endorsed content from last meeting. |
| Intel | Issue 2-1a: Not sure if the issue raised by Nokia is an actual problem. The same “issue” exists for NR-U channels in 5GHz for Rel-16, and no company raised any concerns or commented that this is an issue. While our first preference is Option 1 detailed in our contribution (R4-2208046), it appears our Option 2 has many similar aspects proposed by other companies. Therefore, we suggest taking Option 2 outlined in our contribution as basis for further discussions and we can try to adjust/update the proposal accordingly, e.g., channel placement for 2GHz. |
| Ericsson | Agreement: - Have one GSCN entry corresponding to each channel with the smallest bandwidth for SCS. - The total number is [138] entries for 120KHz SCS. - Further discuss the GSCN for 480KHz and 960KHz SCS Regarding the above agreements, we can remove the [ ] around 138, upon further checking this is ok for us. For larger BW, the GSCNs for min BW should be re-usable per SCS. If not re-used then we have multiple GSCNs for the larger SCS then we can have SSB center to align with the center of the channel for the larger BWs. If there is no need to have SSB center aligned to the center of the channel then there is no need to have extra set of GSCNs for larger BW (i.e. they can be reused from min BW) |
| Nokia, Nokia Shanghai Bell | Sub-topic 2-1a: We are ok to expand the agreement to 800 and 1600 MHz channel bandwidths. For 2000 MHz channel bandwidth more raster points are needed both to avoid 2 GHz of unused spectrum at band edge as well as future-proofing the raster for CA. For 2000 MHz we suggest choosing 400 MHz raster points so that each 2000 MHz channel covers 5x 400 MHz channel. This results in NREF = { 2580043+6720*N , N = 0:29} Furthermore, it is necessary to update channel spacing definitions so that CA configurations using the closest available raster points for corresponding ChBWs are considered as contiguous CA. For sync raster: It is necessary to specify sync raster points also for 960 kHz SCS. RAN2 specifies in 38.331 that "The cell-defining SSB of the PCell is always on the sync raster." and without sync raster specification there cannot be PCell neither. RAN1 has also specified RACH procedure for 960 kHz to enable handovers, triggering scheduling request etc. Only cell selection procedure is out of scope. |
| LGE | For wider SCS we propose following where raster is built using 400MHz raster that was agreed in GTW. For 400 and 800MHz the raster step is 400MHz For 1600MHz raster step is 800MHz For 2000MHz raster step is 400 and 1600MHz. ARFCN for 800: 2569963 + 6720*N, N=0:32 (+16 additional channels for IEEE co-existence as in R4-2208649) ARFCN for 1600: 2576683 + 2*6720*N, N=0:15 (+6 additional channels for IEEE co-existence as in R4-2208649) ARFCN for 2000: 2580043, 2606923,2613643, 2640523, 2647243, 2674123, 2680843, 2707723, 2714443, 2741323, 2748043, 2774923 (+6 additional channels for IEEE co-existence as in R4-2208649) Additional channels in blue dotted box enable alignment with 802.11ad/ay channels and intra-band CA also in this scenario. |
| Intel | Issue 2-1a: Channel raster For 800 MHz, 1600 MHz, and 200 MHz, we suggest taking similar approach for 100 and 400 MHz and agree to the following: - for 800 MHz: NREF = {2569963 + 6720*N, N = 0:32} - for 1600 MHz: {2576683 + 6720*N, N =0:30} - for 2000 MHz: NREF = {2580043 + 6720*N, N=0:29; 2585083, 2655643, 2692603, 2764843} We think this should provide sufficient flexibility to maximize spectrum usage for various regulatory domains while maintaining the constant gap between adjacent channels in multiples of 100.8 MHz. Nokia’s suggestion to add 2000 MHz in ~400 MHz interval does not necessarily solve having channel positions that can potentially align with IEEE 802.11ad/ay channels. Therefore, our suggestion is to support extended version of 100/400MHz raster entries, and just additionally support 4 more entries that will allow at least one 2 GHz channel to be aligned with IEEE channels. This is the same for all other CBWs, where there is at least one CHW for each of 400, 800, and 1600 that is aligned with IEEE channels. GSCN From our analysis on required GSCN, we concluded the following: - For 120kHz SCS, any one of the following GSCN would be compatible with RAN1 RB offset design and agreed set of channels for 100 and 400 MHz - Option A1) 24156 + 6*N - 3*floor((N+5)/18), N=0:137 - Option A2) 24157 + 6*N - 3*floor((N+11)/18), N=0:137 - For 480kHz SCS, while we do not have the details for 800MHz, 1600MHz, and 2000 MHz, the following GSCN would be compatible with RAN1 RB offset design and agreed set of channels for 400 MHz - Option B1) 24162 + N*24 - 12*floor((N+4)/18) - Option B2) 24163 + N*24 - 12*floor((N+6)/18) We suggest going with one of the above options. With all options being more or less the same, we recommend taking A1 and B1 as the final conclusion. |
| Apple | On channel raster, now that 400MHz CBW is mandatory for all SCSs, the need for aggregating CBW of 100MHz is less. So we can accept the GTW agreement and Intel’s proposal for 800/1600/2000MHz CBW, as CA between 400/800/1600/2000MHz are all supported. We also agree with Nokia that necessary update to channel spacing definitions are done so that CA configurations using the closest available raster points for corresponding CBWs are considered as contiguous CA. |

Issue 2-2: Mandatory channel bandwidth

| Company | Comments |
| --- | --- |
| vivo | We support Option 1. |
| OPPO | Option 1 and Option 2 are ok. |
| Nokia, Nokia Shanghai Bell | Option 2 is our preference, but we recognize the necessity to conclude the discussion. Therefore we would be ok with seeking a compromise in the middle of option 1 and option 2 as long as both camps are willing to move from their position. The midpoint of the options is to add 2 mandatory bandwidths and leave two bandwidths optional, i.e. channel bandwidths 100, 400 and 800 MHz are mandatory. |
| Huawei | Option 1 as the first priority. Refer to related Huawei paper in R4-2208768 in thread [136]. It looks that there is not much progress on this from the last meeting’s discussion. Therefore we may need to start looking into option 3 as the middle ground, e.g. to compromise some of the CBW listed as optional in option1 to become mandatory. |
| ZTE | We are open to further discuss the optional maximum channel bandwidth considering the implementation difficulty for FR2-2, however for 120khz SCS, we still prefer that 400MHz could be mandatory since this is already been mandated in other SCS. |
| Intel | We are open to discussing a compromise (Option 3). As Nokia suggested, the midpoint between Option 1 and Option 2 is a good place to start discussions. |
| Apple | Follow the GTW agreement. |
| QCOM | This was decided in the GTW, so we have agreement. |

### 2.3.2	CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2208047 | Huawei: depends in topic 2-1. Despite Option 1/2 from Intel, there is another Dcr to TS38.101-2 for n263 in R4-2208617. |
| R4-2208048 | Huawei: depends in topic 2-1. Despite Option 1/2 from Intel, there is another Dcr to TS38.104 for n263 in R4-2208540. |
| R4-2208540 | Huawei: depends in topic 2-1. There are Dcr to TS38.104 for n263 in R4-2208048/ R4-2210118. Moreover, Initial Access restriction for 960k SCS missing. |
|  | Ericsson: We have agreement from Gotoweb session for 120 kHz SCS. A revision is required. Eventually we can also solve 480/960 too. |
| R4-2209682 |  |
| R4-2209683 | Huawei: related to R4-2208541 as discussed in section 1.3.2. |
| R4-2210117 | Huawei: depends in topic 2-1. Despite Option 1/2 from Intel, there is another Dcr to TS38.101-2 for n263 in R4-2208617. |
| R4-2210118 | Huawei: depends in topic 2-1. Despite Option 1/2 from Intel, there is another Dcr to TS38.104 for n263 in R4-2208540. |

## 2.4	Summary for 1st round

### 2.4.1	Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #2-1: Channelization | Agreements reached in GTW session on May 10th: Agreement: - Channel Raster Option 2 (Intel, R4-2208046) - for 100 MHz channel bandwidth, NREF = {2564083 + 1680*N, N = 0:137} - for 400 MHz channel bandwidth, NREF = {2566603 + 6720*N, N = 0:33} - Further discuss the channel raster for wider channel bandwidth, i.e., 800MHz, 1600MHz, 2000MHz Agreement: - Have one GSCN entry corresponding to each channel with the smallest bandwidth for SCS - The total number is [138] entries for 120KHz SCS - Further discuss the GSCN for 480KHz and 960KHz SCS Candidate options: We appear to have two candidate options for channel and sync raster. Option 1 noted by LGE, and Option 2 noted by Intel. Nokia’s proposal seems to be the same as Option 2 proposed by Intel except for 4 extra entries for 2000 MHz. Based on comments received so far, moderator suggests the following: For channel raster, agree on one of the following options - Option 1 - For 800 MHz: {2569963 + 6720*N, N=0:32; 2575003, 2581723, 2588443, 2595163, 2645563, 2652283, 2659003, 2665723, 2682523, 2689243, 2695963, 2702683, 2754763, 2761483, 2768203, 2774923} - For 1600 MHz: {2576683 + 2*6720*N, N=0:15; 2581723, 2623723, 2652283, 2695963, 2724523, 2768203} - For 2000 MHz: {2580043, 2606923,2613643, 2640523, 2647243, 2674123, 2680843, 2707723, 2714443, 2741323, 2748043, 2774923; 2585083, 2620363, 2655643, 2692603, 2727883, 2764843} Moderator Note: the 16/6/6 additional values for 800/1600/2000 MHz were taken from the last 16/6/6 entries of 800/1600/2000 MHz of R4-2208649 (LGE) - Option 2 - For 800 MHz: {2569963 + 6720*N, N = 0:32} - For 1600 MHz: {2576683 + 6720*N, N =0:30} - For 2000 MHz: {2580043 + 6720*N, N=0:29; 2585083, 2655643, 2692603, 2764843} Moderator Note: the yellow highlighted entries are identical values in Option 1 and Option 2. The teal highlighted values in Option 1 are covered by 2580043 + 6720*N. For sync raster, agree on one of the following options: - Option 1 - for 120 kHz SCS: 24156 + 6*N - 3*floor((N+5)/18), N=0:137 - for 480 kHz SCS: 24162 + N*24 - 12*floor((N+4)/18), N=0:33 - Option 2 - For 120 kHz SCS: 24157 + 6*N - 3*floor((N+11)/18), N=0:137 - For 480 kHz SCS: 24163 + N*24 - 12*floor((N+6)/18) , N=0:33 Recommendations for 2nd round: Moderator suggests down-selecting among the options listed above to conclude the channelization. Focus on preference between Option 1 or Option 2 for channel raster, and between Option 1 or Option 2 for sync raster. For additional discussion, Nokia noted we need to define sync raster for 960 kHz as well. Since 960 kHz based SSB are only supported as SCells, there should not be any cell search complexity issue associated with this. If so, agreeing to a sync raster for 960 kHz with either a step size of <1> (full-flexible) or <6> (aligned with n264) is a reasonable approach. Companies are asked to provide further comments on sync raster for 960 kHz applicable for SCells considering the following options: - Option 1: step size <1> - Option 2: step size <4> |
| Sub-topic #2-2: Channel bandwidth | Agreement reached in GTW session on May 10th: Agreement: Mandatory channel bandwidths - 120 kHz: mandatory (100 MHz, 400 MHz) - 480 kHz: mandatory (400 MHz), optional (800 MHz, 1600 MHz) - 960 kHz: mandatory (400 MHz,), optional (800MHz, 1600 MHz, 2000 MHz) Recommendations for 2nd round: No further discussion needed |

### 2.4.2	CRs/TPs

Note: The tdoc decisions are provided in Section 4

| R/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2208047 | Not pursued |
| R4-2208048 | Not pursued |
| R4-2208540 | To be revised |
| R4-2209682 | No feedback received, need to verify content in round 2 |
| R4-2209683 | To be merged into the revision of draft CR R4-2208541 |
| R4-2210117 | To be revised |
| R4-2210118 | To be revised |

## 2.5	Discussion on 2nd round (if applicable)

TBA

# Topic #3: Spectrum utilization

## 3.1	Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2208538 On BS RF receiver requirements for the frequency range 52.6 to 71.0 GHz | Ericsson | Proposal 1: It is proposed for FR2-2 to adopt SU allocation values given in Table 2.1-4. Table 2.1-4: NRB and SU allocation FR2 Frequency range; SCS (kHz); Transmission bandwidth (MHz). 50; 100; 200; 400; 800; 1600; 2000. FR2-1; 60; 66 (95.0%); 132 (95.0%); 264 (95.0%); N/A; N/A; N/A; N/A. 120; 32 (92.2%); 66 (95.0%); 132 (95.0%); 264 (95.0%); N/A; N/A; N/A. FR2-2; 120; N/A; 62 (89.3%); N/A; 248 (89.3%); N/A; N/A; N/A. 480; N/A; N/A; N/A; 61 (87.8%); 120 (86.4%); 238 (85.7%); N/A. 960; N/A; N/A; N/A; 30 (86.4%); 60 (86.4%); 119 (85.7%); 148 (85.2%). |
| R4-2209139 52.6-71 GHz System Parameters | Ericsson | Spectrum utilization Observation 1: Given challenges for larger arrays and bandwidths at this frequency range compared to FR2-1, a lower spectral utilization should be considered. Since continuous CA is supported as part of the agreement additional bandwidth can be realized using continuous CA as an approach rather than increase spectral utilization. Proposal 1: Spectral utilization consideration captured in Table 1. Table 1: Spectral utilization for 57 – 71 GHz SCS (kHz); 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB. 120; 62; 248; N/A; N/A; N/A. 480; N/A; 61; 120; 238; N/A. 960; N/A; 30; 60; 119; 148. |
| R4-2209717 On UE Tx RF aspects for FR2-2 | Nokia, Nokia Shanghai Bell | Proposal 1: Confirm 156 RBs for 2GHz CBW with 960 kHz SCS. SCS (kHz); 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB. 120; 66; 264; N/A; N/A; N/A. 480; N/A; 66; 132; 264; N/A. 960; N/A; 33; 66; 132; [156]. |
| R4-2210166 60GHz UE TX | Qualcomm | Proposal: Use the NRB table values for FR2-2 transmission bandwidth requirements, with [] removed from 156. SCS (kHz); 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB. 120; 66; 264; N/A; N/A; N/A. 480; N/A; 66; 132; 264; N/A. 960; N/A; 33; 66; 132; [156]. |

## 3.2	Open issues summary

### 3.2.1	Sub-topic 3-1: NRB and SU allocation in FR2

Issue 3-1: FR2-2 SU allocation

- Options

  - Proposal 1: It is proposed for FR2-2 to adopt SU allocation values given in Table 2.1-4. (Ericsson, R4-2208538/R4-2209139)

Table 2.1-4: NRB and SU allocation FR2

| Frequency range | SCS (kHz) | Transmission bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 | 800 | 1600 | 2000 |
| FR2-1 | 60 | 66 (95.0%) | 132 (95.0%) | 264 (95.0%) | N/A | N/A | N/A | N/A |
|  | 120 | 32 (92.2%) | 66 (95.0%) | 132 (95.0%) | 264 (95.0%) | N/A | N/A | N/A |
| FR2-2 | 120 | N/A | 62 (89.3%) | N/A | 248 (89.3%) | N/A | N/A | N/A |
|  | 480 | N/A | N/A | N/A | 61 (87.8%) | 120 (86.4%) | 238 (85.7%) | N/A |
|  | 960 | N/A | N/A | N/A | 30 (86.4%) | 60 (86.4%) | 119 (85.7%) | 148 (85.2%) |

  - Proposal 2: Confirm 156 RBs for 2GHz CBW with 960 kHz SCS. (Nokia R4-2209717, Qualcomm R4-2210166)

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 264 | N/A | N/A | N/A |
| 480 | N/A | 66 | 132 | 264 | N/A |
| 960 | N/A | 33 | 66 | 132 | [156] |

- Recommended WF

  - Companies are encouraged to share their views on the two proposals for SU allocation

Discussion:

ZTE: we still prefer option 2. The proposal 1 will have impact on the analysis of BS RF spec. We do not see the further relaxation is needed for FR2-2.

Ericsson: we are proposing the proposal 1 with the smaller SU, because we should align SU between BS and UE. We do not understand why smaller SU will impact the MPR. We pay the price for larger SU for FR2-1. We are proposing the smaller SU for several entries. We also have larger channel bandwidth, which should be considered.

Nokia: The concern is the digital filtering. But based on our study, we do not see the benefit to have narrower allocation. Our preference is proposal 2.

Apple: we do not think this aspect has been considered last year. We look at the spectrum mask. Based on the spectrum mask, if the spectrum is lower… we have the reasonable requirement for UE to meet the mask. At least the current proposal 2 seems to ask for 95% SU, which is bit too high.

ZTE: to Ericsson, digital filer will be used for different bandwidth. When reduce the SU, the impact of digital filter should be evaluated again. To Apple, we have a dedicated email thread to discuss. Almost all the companies agreed to reuse the SU of FR2-1.

Ericsson: we agree with Apple comment. Our analysis below n 80% was provided last year to consider filter complexity and thermal noise.

Agreement: confirm the numbers for 100MHz and 400MHz for 120 SCS

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 264 | N/A | N/A | N/A |
| 480 | N/A | 66 | 132 | 264 | N/A |
| 960 | N/A | 33 | 66 | 132 | [156] |

## 3.3	Companies views’ collection for 1st round

### 3.3.1	Open issues

Issue 3-1: FR2-2 SU allocation

| Company | Comments |
| --- | --- |
| vivo | Proposal 2 is OK for us. We can remove the bracket for SU for 2000MHz in P2. |
| Nokia, Nokia Shanghai Bell | We do not agree with proposal 1. The change is motivated by challenges in filtering and difficulties in meeting emissions. However, the filtering that relates to spectrum utilization is digital filtering, and as such plays much lesser role in meeting emission requirements. We do not recognize the implementation challenges to be at level that they would justify reduced spectrum utilization. Therefore, we support proposal 2 and confirming the working assumption from previous meeting and removing square brackets from 2000 MHz ChBW. |
| Huawei | Considering that the SU topic was not really extensively discussed so far, we would be fine not to rush into conclusions. Considering the emission requirements as the primary problem here, we would like to hear more on the motivation to reduce the (single-carrier) SU. More discussion needed. |
| ZTE | We disagree with proposals and this would have big impacts on UE MPR evaluation and BS RF specifications and it’s too late to reopen the discussion again. |
| Ericsson | We are ok to compromise to confirm the values shown above in green, and update with the values we have proposed for the larger SCS/BW combinations. See complete table below. Table 5.3.2-3: Transmission bandwidth configuration NRB for FR2-2 SCS (kHz); 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB. 120; 66; 264; N/A; N/A; N/A. 480; N/A; 61; 120; 238; N/A. 960; N/A; 30; 60; 119; 148. |
| Apple | We also share the view that for large CBW and higher SCS (480/960kHz), the current proposed 95% SU is too high. |
| QCOM | We agree confirming the GTW values in green. In addition, we agree with proposal 2 and removal of the [] around [156]. It appears companies are not aligned here on the other values. At least we should agree the GTW green values to make some progress on MPR for 120 kHz SCS. |
| Intel | GTW green values can be confirmed Given the timeline, we can discuss a lower value for 800MHz/1600MHz/2000MHz as a compromise to conclude the topic (perhaps somewhere in the middle of the working assumption and Ericsson’s proposal) |

## 3.4	Summary for 1st round

### 3.4.1	Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #3-1 | Outcome of GTW session on May 10th: Agreement: confirm the numbers for 100MHz and 400MHz for 120 SCS SCS (kHz); 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB. 120; 66; 264; N/A; N/A; N/A. 480; N/A; 66; 132; 264; N/A. 960; N/A; 33; 66; 132; [156]. Candidate options: - Option 1: Confirm working assumption and 156 RBs for 2GHz CBW with 960 kHz SCS SCS (kHz); 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB. 120; 66; 264; N/A; N/A; N/A. 480; N/A; 66; 132; 264; N/A. 960; N/A; 33; 66; 132; [156]. - Option 2: Confirm the values in green, and update table with values in yellow proposed for the larger SCS/BW combinations SCS (kHz); 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB. 120; 66; 264; N/A; N/A; N/A. 480; N/A; 61; 120; 238; N/A. 960; N/A; 30; 60; 119; 148. - Option 3: Confirm GTW green values and discuss a lower value for 800MHz/1600MHz /2000MHz as a compromise to conclude the topic (somewhere in the middle of the working assumption and Option 2 above) Recommendations for 2nd round: Consider the three options listed above and focus on reaching a compromise as soon as possible |

## 3.5	Discussion on 2nd round (if applicable)

TBA

# Recommendations for Tdocs

## 4.1	1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on system parameters for NR extension to 71GHz | Intel Corporation |  |
|  | Draft reply LS on the minimum guard period between two SRS resources for antenna switching | CATT | Revision of R4-2208143 |
|  | Draft CR to TS 38.104: Addition of FR2-2definition in subclause 5.1, n263 in subclause 5.2 and transmission bandwidth information in subclause 5.3 | Ericsson | Revision of R4-2208540 |
|  | Draft CR to TS 38.104: Addition of support for band n264 for licensed operation within 66000 to 71000 MHz | Ericsson | Revision of R4-2208541 |
|  | Draft CR for TS 38.101-2: Introduction of system parameters for FR2-2 | vivo | Revision of R4-2208617 |
|  | Draft CR to introduce the channel and sync rasters of band n263 (option 2) | Intel Corporation | Revision of R4-2210117 |
|  | Draft CR to introduce the channel and sync rasters of band n263 (option 2) | Intel Corporation | Revision of R4-2210118 |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2207780 |  | Remaining issues on system parameters for NR operation in 52.6GHz – 71GHz | Apple | Noted |  |
| R4-2208045 |  | UE features for NR ext. to 71GHz WI | Intel Corporation | Noted |  |
| R4-2208046 |  | Further views on channelization for FR2-2 | Intel Corporation | Noted |  |
| R4-2208047 |  | Draft CR to introduce the channel and sync rasters of band n263 (option 1) | Intel Corporation | Not pursued | Discussions will focus on Option 2 |
| R4-2208048 |  | Draft CR to introduce the channel and sync rasters of band n263 (option 1) | Intel Corporation | Not pursued | Discussions will focus on Option 2 |
| R4-2208143 |  | Draft reply LS on the minimum guard period between two SRS resources for antenna switching | CATT | To be revised |  |
| R4-2208479 |  | System parameters for a NR band in the range 52.6GHz – 71GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2208540 |  | Draft CR to TS 38.104: Addition of FR2-2definition in subclause 5.1, n263 in subclause 5.2 and transmission bandwidth information in subclause 5.3 | Ericsson | To be revised |  |
| R4-2208541 |  | Draft CR to TS 38.104: Addition of support for band n264 for licensed operation within 66000 to 71000 MHz | Ericsson | To be revised | Consider comments provided in Section 1.3.2 and include content of draft CR R4-2209683 |
| R4-2208617 |  | Draft CR for TS 38.101-2: Introduction of system parameters for FR2-2 | vivo | To be revised |  |
| R4-2208618 |  | Further discussion on channel raster and sync raster for NR extending to 71GHz | vivo | Noted |  |
| R4-2208649 |  | 60GHz channel and synchronization raster | LGE | Noted |  |
| R4-2208763 |  | Reply LS on the minimum guard period between two SRS resources for antenna switching | Huawei, HiSilicon | Noted |  |
| R4-2209139 |  | 52.6-71 GHz System Parameters | Ericsson | Noted |  |
| R4-2209509 |  | Further discussion and draft reply LS on minimum guard symbol of SRS | Xiaomi | Noted |  |
| R4-2209682 |  | Draft CR to TS 38.101-2: Channel arrangement and channel bandwidths for 66-71 GHz | Huawei, HiSilicon | Return to | No feedback received during 1st round Discuss content in round 2 |
| R4-2209683 |  | Draft CR to TS 38.104: Channel arrangement and channel bandwidths for 66-71 GHz | Huawei, HiSilicon | To be merged into revision of draft CR R4-2208541 |  |
| R4-2209716 |  | UE features for NR in 52.6 GHz – 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2210080 |  | draft CR 38.101-3 on FR2-2 DC/CA with FR1 anchor | Ericsson | Postponed |  |
| R4-2210117 |  | Draft CR to introduce the channel and sync rasters of band n263 (option 2) | Intel Corporation | To be revised |  |
| R4-2210118 |  | Draft CR to introduce the channel and sync rasters of band n263 (option 2) | Intel Corporation | To be revised |  |
| R4-2210188 |  | 60GHz UE bandwidths | Qualcomm | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 4.2	2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| vivo | Zhou Shuai | shuai.zhou@vivo.com |
| Nokia, Nokia Shanghai Bell | Toni Lähteensuo | Toni.h.lahteensuo (at) nokia.com |
| Huawei | Michal Szydelko | michal.szydelko@huawei.com |
| Intel | Aida Vera Lopez | aida.l.vera.lopez@intel.com |
| CATT | Huiping Shan | shanhuiping@catt.cn |
| Samsung | Yankun Li | Yankun.li@sumsuang.com |
| Apple | Steven Chen | steven.x.chen@apple.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)