3GPP TSG-RAN WG4 Meeting #103-e	R4-2210273

Electronic Meeting, 9 – 20 May, 2022

Agenda item:			4.1.5

Source:	Moderator (Huawei)

Title:	Email discussion summary for [103-e][201] Maintenance_R15_R16_RRM

Document for:	Information

Introduction

The scope of this email discussion includes the following agenda items:

4.1.5 RRM requirements [WI code] 4.1.5.1 RRM core requirements (38.133/36.133) [WI code] 4.1.5.2 RRM performance requirements (38.133/36.133)

In providing comments, companies are encouraged to:

- Ensure that the comments are inserted in the latest version of the document by checking the folder before uploading

- Use “Track changes” to help identify added comments/changes

- Pay attention to the rule for shortening file name

- Add your contact information to the table in Annex

Further notes for some Tdocs:

- The following Tdocs are included in the summary based on offline requests and chairman’s assignment

| R4-2208198 | Draft CR on TS 36.171 requirements for support of A-GNSS | CATT, CAICT, CENC |
| --- | --- | --- |
| R4-2208199 | Draft CR on TS 36.171 requirements for support of A-GNSS | CATT, CAICT, CENC |
| R4-2208200 | Draft CR on TS 38.171 requirements for support of A-GNSS | CATT, CAICT, CENC |
| R4-2208201 | Draft CR on TS 38.171 requirements for support of A-GNSS | CATT, CAICT, CENC |
| R4-2207731 | SRS configuration correction | Qualcomm, Inc. |
| R4-2208734 | [dCR] Maintenance for IAB-MT test cases R16 | ZTE Corporation |
| R4-2208735 | [dCR] Maintenance for IAB-MT test cases R17 Cat A | ZTE Corporation |

- The following Tdocs are moved to other email threads based on offline requests and chairman’s assignment

| R4-2210084 | CR: Corrections on NR V2X Resource Selection Test | Qualcomm, Inc. | 201 226 |
| --- | --- | --- | --- |
| R4-2210085 | (mirror R17)CR: Corrections on NR V2X Resource Selection Test | Qualcomm, Inc. | 201 226 |
| R4-2210086 | CR: Corrections on LTE V2X Resource Selection Test | Qualcomm, Inc. | 201 226 |
| R4-2210087 | (mirror R15)Corrections on LTE V2X Resource Selection Test | Qualcomm, Inc. | 201 226 |
| R4-2210088 | (mirror R16)Corrections on LTE V2X Resource Selection Test | Qualcomm, Inc. | 201 226 |
| R4-2210089 | (mirror R17)Corrections on LTE V2X Resource Selection Test | Qualcomm, Inc. | 201 226 |
| R4-2208828 | Reply LS on BWP operation without bandwidth restriction | vivo | 201 234 |

Topic #1: Rel-15 NR RRM maintenance

Companies’ contributions summary

| T-doc | Company | Proposals / Observations |
| --- | --- | --- |
| Core part |  |  |
| R4-2207941 | Qualcomm Incorporated | CR According to RAN1 spec, TS38.214, UE is supposed to drop CQI report before receiving at least one CSI-RS transmission occasion for channel measurement during SCell activation which conflicts with test description and criteria on CQI report during SCell activation. Added a condition where UE does not have to transmit a CSI report during SCell activation. |
| R4-2208909 | Huawei, Hisilicon | CR Clarify the interruption requirements for SCell addition/ activation for the case where SMTC is not configured for the SCell |
| R4-2208912 | Huawei, Hisilicon | CR for 36.133 Clarify the interruption requirements for SCell addition/ activation for the case where SMTC is not configured for the SCell |
| R4-2208915 | Huawei, Hisilicon | CR for 36.133 Maximum interruption in paging reception during LTE – NR inter-RAT cell reselection are added |
| R4-2208916 | Huawei, Hisilicon | CR for 36.133 R16 Maximum interruption in paging reception during LTE – NR inter-RAT cell reselection are added Compared with R4-2208916, additional changes are captured in CR due to difference between Rel-15 and Rel-16 Specs |
| R4-2209186 | Huawei, Hisilicon | CR 1. Update the applicability condition in clause 9.1.5.1 such that the SMTC offsets are same even there are only SCCs in FR2. 2. Add CSSF definition to clause 9.1.5.1.2 for FR1-FR2 CA with PCC in FR2. |
| Perf part |  |  |
| R4-2207646 | Anritsu Corporation | CR We propose to remove specified DCI-based active UL BWP for FDD config in Test Parameters Table A.4.5.6.1.1.1-3/A.6.5.6.1.2.1-3. Update Table A.4.5.6.1.1.1-3/A.6.5.6.1.2.1-3: - Config 1 Active UL BWP-2 Configuration: ULBWP.1.3 N/A |
| R4-2207648 | Anritsu Corporation | CR for R17 1) Update Table A.4.5.6.1.1.1-3/A.6.5.6.1.2.1-3: - Config 1 Active UL BWP-2 Configuration: ULBWP.1.3 N/A 2) Conbine the cell of the parameter value for “EPRE ratio of PDCCH to PDCCH DMRS” in Table A.5.5.1.5.1-3 and A.5.5.1.6.1-3 with the parameters below (e.g. EPRE ratio of PBCH DMRS to SSS, etc.) Changes in 1) are same as the associated Rel-15/16 CRs (R4-2207646, 7647). |
| R4-2207747 | Apple | Proposal 1: add A.5.7.1.3 and A.7.7.1.3 in A.3.13A to allow UE not to pass the tests. Proposal 2: add Gmin and Gmax in A.5.7.1.3 and A.7.7.1.3 test requirements. Proposal 3: Ginter shall be considered for intra-band inter-frequency relative accuracy test as well. Proposal 4: Ginter shall be considered to upper bound as well. Proposal 5: additional margins to the upper bound for FR2 inter-frequency relative RSRP accuracy test requirements shall be: Ginter + E. Proposal 6: margin E shall not be larger than Y. RAN4 can also discuss some smaller value to move forward. |
| R4-2207748 | Apple | CR - Update test applicability in A.3.13A to allow UE skip A.5.7.1.3 and A.7.7.1.3. - Add Gmin and Gmax as in other accuracy test in A.5.7.1.3 and A.7.7.1.3. |
| R4-2207948 | Qualcomm Incorporated | CR According to RAN1 spec, TS38.214, UE is supposed to drop CQI report before receiving at least one CSI-RS transmission occasion for channel measurement during SCell activation which conflicts with test description and criteria on CQI report during SCell activation. Added a condition where UE does not have to transmit a CSI report during SCell activation TCs. |
| R4-2208166 | CATT | CR for R17 Add SMTC pattern 6 in A.3.11.6 as R15 spec. |
| R4-2208167 | CATT | CR 1. For EN-DC FR1 RLM tests based on SSB, delete “and SSB#1” 2. For SA FR1 RLM tests based on SSB, delete “and SSB#1” |
| R4-2208341 | OPPO | CR Modify the incorrect event: from event A4 to event B1 in TC for Addition and Release Delay of known NR PSCell |
| R4-2208469 | MediaTek Inc. | Proposal 1: For intra-band case, at lower bound, to add margin D (5.5 dB) for the uncertainty of mis-alignment between fine beam and rough beam. Proposal 2: For intra-band case, at both lower bound and upper bound, not to add margin Ginter (3 dB). Proposal 3: For intra-band case, at upper bound, not to add margin E. Proposal 4: For inter-band case, at upper bound, to add margin Ginter (3dB) but not to add margin E. |
| R4-2208879 | Anritsu Corporation | Observation 1: As for intra-band inter-frequency case, conditions to apply Ginter should be provided such as a frequency separation between carriers and the actual values for the margin. Observation 2: Ginter for inter-band at the upper bound is agreeable to the group and the remaining part is the intra-band inter-frequency case. Observation 3: To discuss on the margin E, the actual antenna gain at beam peak should be provided from proponents. Observation 4: We should care about the increase of test time due to the modification of measurement procedures to compensate relaxation factors. |
| R4-2208906 | Huawei, Hisilicon | CR it is recommended that SnonintrasearchP be set to not present to ensure that the UE will perform neighboring cell detecting. |
| R4-2208926 | Huawei, Hisilicon | Proposal 1: Add Ginter in lower band and upper bound for both intra-band and inter-band, which is the margin due different antenna gain on different frequency carriers. Proposal 2: Do not modify the test procedure. Whether to have new test procedure can be discussed in future release which may need thorough evaluation on testability. Proposal 3: Add additional E + Ginter margin in upper bound, where E is 3 dB. |
| R4-2209078 | Ericsson | CR for R16 Correction of E-UTRAN PRACH configuration index configuration - For test configurations 1, 2, and 3, set to 53. - For test configurations 4, 5, and 6, set to 4. The correction is applicable from Rel-16 specification. Rel-15 specifiction does not have such errors. |
| R4-2209609 | Rohde & Schwarz | CR In all intra-frequency event triggerend measurement test cases, set the connection-related transmission parameters (RMC, TRS) for the neighbour cell to N/A. Resubmission of R4-2206809 which did not have any technical objections in RAN4#102, but which was postponed as the 3rd DRAFT CR of R&S for the R15 maintenance, while only 2 DRAFT CRs per company were allowed. |
| R4-2209612 | Rohde & Schwarz | CR In FR1 BFD-LR test cases (A.4.5.5, A.6.5.5), dedicated CORESET RMCs have been added, while the current CORESET RMCs have been specified to be RMSI CORESET RMCs. |

Open issues summary

Note: Only issues proposed in discussion papers are listed in this section. For other issues proposed via CR, please provide comments to the CR directly in section 1.3.1 and 1.3.2.

Sub-topic 1-1: Test of relative accuracy for FR2 inter-frequency RSRP

Moderator’s Note: the following additional margins for the relative accuracy have been discussed in companies’ contributions

- D: margin due to mis-alignment between fine beam and rough beam

- Ginter: margin due to different antenna gain on different bands

- E: margin due to difference between Y’ and Z’

  - Y’: actual gain difference between fine and rough beam at peak direction

  - Z’: actual gain difference between fine and rough beam at spherical coverage direction

In RAN4#102-e, the agreements are

- Add (D + Ginter) to the lower bound when two cells are in different bands

- D = [5.5] dB, Ginter = [3] dB

Issue 1-1-1: whether to add Ginter when two cells are in same band

Moderator’s Note: This issue is for both upper bound and lower bound. If you think the application of Ginter is different for upper and lower bound, please indicate this in the comment.

- Proposals

  - Option 1a (Apple, HW)

    - Yes

  - Option 1b (Anritsu)

    - Yes, but conditions should be provided such as a frequency separation between carriers and the actual values.

  - Option 2 (MTK)

    - No

- Recommended WF

  - Further discuss the options

| Company | Comments |
| --- | --- |
| Apple | Support option 1a. According to previous discussion, Ginter comes from RF transceiver gain difference. Separation in frequency between two carriers plays a role here. Regarding option 1b, it may take quite a long time to reach common understanding on the frequency separation limit beyond which the additional margin is needed, since it highly depends on implementation. On the other hand, the issue also exists for inter-band scenario. Operating Band; Uplink (UL) operating band BS receive UE transmit; Downlink (DL) operating band BS transmit UE receive; Duplex Mode. FUL_low – FUL_high; FDL_low – FDL_high. n257; 26500 MHz; –; 29500 MHz; 26500 MHz; –; 29500 MHz; TDD. n258; 24250 MHz; –; 27500 MHz; 24250 MHz; –; 27500 MHz; TDD. As can be observed, there is a large overlapping area between the two low bands. Let’s consider the following two cases: Case 1) Two low bands are configured in the inter-frequency accuracy test and carrier frequencies for the two cells are quite close to each other, e.g. with 100MHz frequency separation. Case 2) only one band is configured in the inter-frequency accuracy test, i.e. intra-band case. There is 2GHz frequency separation between serving cell and target cell. If we go with option 1b or option 2. Ginter will be added for case 1, but not for case 2. This doesn’t make too much sense to us since we consider additional margin for 100MHz frequency separation but not for 2GHz frequency separation. |
| MediaTek | We can compromise to option 1a. |
| Qualcomm | We can compromise to option 1a as well. |
| Anritsu | Thanks Apple for the explanation. It makes sense and thus we agree with option 1a. |
| Huawei | Support option 1a and prefer not to have differentiated Ginter for intra-band/inter-band or conditions about frequency separation. |

Issue 1-1-2: whether to add E to the upper bound

Moderator’s Note: there was no concern in RAN4#102-e to add at least Ginter to the upper bound at least for the case when two cells are in different bands.

- Proposals

  - Option 1 (Apple, HW)

    - Yes

      - E is no larger than Y, and smaller value can be considered (Apple)

      - E = 3dB (HW)

  - Option 2 (MTK)

    - No

  - Option 3 (Anritsu)

    - FFS, actual antenna gain at beam peak should be provided from proponents of option 1

- Recommended WF

  - Further discuss the options

| Company | Comments |
| --- | --- |
| Qualcomm | We would prefer not to add E. |
| Apple | We support option 1. We can consider some smaller margin to move forward. |
| MediaTek | Prefer to option 2. |
| E/// | We also prefer not to add E. |
| Huawei | Support option 1. As analyzed in our paper and also contributions of simulation results from companies, it shown that current margin (-X) is not sufficient. To move forward, we propose a margin smaller than Y (3 dB) as compromised solution. Regarding option 3, we don’t think there is clear reliable solution to obtain actual gain difference of rough beam. |

Issue 1-1-3: whether to modify the test procedure to compensate the relaxation margins

- Proposals

  - Option 1 (Anritsu, HW)

    - No

- Recommended WF

  - Agree on option 1

| Company | Comments |
| --- | --- |
| MediaTek | Support option 1. It is not a good way to change the R15 test procedure at this moment. |
| Huawei | Support option 1. First, we don’t think there is reliable solution to obtain the actual gain difference of rough beam to compensate the error. Second, for legacy test cases, we prefer not to modify the test procedure. |

Issue 1-1-4: margin for the lower bound when two cells are in same band

- Proposals

  - Option 1 (MTK)

    - For intra-band case, at lower bound, to add margin D (5.5 dB) for the uncertainty of mis-alignment between fine beam and rough beam

- Recommended WF

  - For intra-band case, at lower bound, at least add margin D = [5.5] dB

  - Whether Ginter should be also added depends on the outcome of Issue 1-1-1.

| Company | Comments |
| --- | --- |
| Qualcomm | OK but would like to see if we can compromise to a margin smaller than 5.5 dB. Clarification: We can compromise to adding both D + Ginter to the lower bound as indicated under issue 1-1-1. Our comment here is that we would like to see if a smaller value of D would work. Leave it in [] for now. |
| Apple | Support option 1. |
| MediaTek | Support option 1. To our understanding, regardless of intra-band or inter-band, the margin D should be considered. |
| E/// | Option 1 is fine |
| Anritsu | Option 1 with brackets is fine. |
| Huawei | Support recommended WF. Regarding D, if the motivation is to consider the misalignment for UE implementation based on the simulation results, we believe it is fair to consider the reasonable implementation where the fine peak and rough peak are aligned, which needs extra margin in upper bound based on the same set of simulation results. |

Sub-topic 1-2: Other issues with test case of FR2 inter-frequency RSRP accuracy

Issue 1-2-1: Applicability of the test considering FR1+FR2 testability

- Proposals

  - Option 1 (Apple)

    - add A.5.7.1.3 and A.7.7.1.3 in A.3.13A to allow UE not to pass the tests

- Recommended WF

  - Check if option 1 is agreeable

| Company | Comments |
| --- | --- |
| Apple | Support option 1. |
| MediaTek | More discussion is needed. As following requirement, UE is only required to do measurement on cell2 (FR2) and it has nothing to do with cell1 (LTE/FR1). Thus, it seems that UE can do the measurement on FR2 cell and report on FR1 cell. (note: If the report is not transmitted successfully, the report can be transmitted repeatedly to TE on LTE/FR1 cell until the network received the report.) ##### A.5.7.1.3.3 Test Requirements The SS-RSRP measurement accuracy for Cell 3 shall fulfil the Absolute requirement in clause 10.1.5.1.1.. |
| E/// | Should not add A.5.7.1.3 and A.7.7.1.3 in A.3.13A. Criteria for OTA test problem is applied for the case UE need to transmit the specified signals on/by the specified time, e.g., the number of HARQ-ACK signals or RRC response messages. On the other hand, the measurement accuracy is verified with a lot of measurement reports. We don’t think it does not affect to the measurement accuracy test even if TE cannot receive a few measurement reports due to the unreliable OTA links. We also need to check with TE vendors how FR1/LTE+FR2 measurement accuracy test is evaluated. |
| Huawei | Agree with option 1. |
| R&S | This is an accuracy TC where only the absolute accuracy of FR2 cell is evaluated, while only the report is sent on FR1 cell. However, the test criteria is not the timing of reporting, but its content. In addition, the report is transmitted periodically and there will not be any evaluation until a report have been successfully received. The test case is in additional statistical, there will be many events required to come to a pass or fail. Thus, since the reporting is not time critical, we don’t see a reason to skip the test case. |

Issue 1-2-2: Test requirements for absolute accuracy

- Proposals

  - Option 1 (Apple)

    - add Gmin and Gmax in A.5.7.1.3 and A.7.7.1.3 test requirements.

- Recommended WF

  - Check if option 1 is agreeable

| Company | Comments |
| --- | --- |
| Apple | Support option 1. |
| MediaTek | Support option 1. |
| Huawei | Agree with option 1. |

Comments to the CRs

Cat-A draftCRs are not listed for comments.

CRs for the Core part

| CR/TP number | Comments collection |
| --- | --- |
| R4-2207941 (QC) | draft Cat-F CR (R15) to SCell Activation Core |
|  | MTK: OK E///: We are ok with principle. May be wording needs to be modified a bit. May beon the below lines. Starting from the first CSI-RS transmission for the channel measurement after the reception of SCell activation command at slot n (as specified in clause 4.3 of TS 38.213 [3]) and until the SCell activation completion at UE, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell. Apple: we are fine with CR and prefer to keep the wording as we discussed from last meeting. QC: Thank MediaTek, Ericsson, Apple for your support and suggestions. We modified the wording based on Ericsson’s suggestion. And a reference section in RAN1 spec is also added to avoid any further misunderstanding. Please see below and let us know if there is still anything you want to modify or if you want to keep the original version. Starting from the first available CSI reference resource for the channel measurement and report (specified in clause 5.2.2.5 of TS 38.214 [26]) after the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the SCell activation completion at UE, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell. Huawei: we are fine with the CR and also prefer to use the wording as discussed in last meeting. Nokia: The CR is not agreeable. From our view, the existing text is clear on the definition of the starting slot, and by the addition proposed the condition for OOR might be invalidated. RAN4 has supported this behaviour since LTE-Rel10 timeframe and from beginning of NR. Changing this behaviour now may cause problems in the field. We do not see any behaviour is broken with current definition. |
| R4-2208909 (HW) | Correction to NR SCell activation interruption requirements 38133_r15 |
|  | MTK: OK vivo: If the UE is provided with SSB configuration (absoluteFrequencySSB) when no SMTC is configured for the SCell, wouldn’t be more reasonable to follow SSB periodicity rather than to assume 5ms periodicity? E///: OK Apple: fine with CR Huawei: Thanks MTK, vivo, E/// and Apple for the comments. To vivo, in the SCell activation delay requirements, it was specified that when no SMTC is configured, the requirements are based on 5ms SSB periodicity. We understand the interruption requirements should be based on the same assumption. Trs is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5ms assuming the SSB transmission periodicity is 5ms. There are no requirements if the SSB transmission periodicity is not 5ms Nokia: Wording is still not clear. We can work with Huawei together on the new TP and some additional corrections. Here is our proposal of new TP: If the UE is not provided SMTC configuration, the SSB transmission periodicity is assumed to be 5ms and TSMTC_duration = [x]ms. If the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, TSMTC_duration = 0ms |
| R4-2208912 (HW) | Correction to NR SCell activation interruption requirements 36133_r15 |
|  | MTK: OK vivo: same comment as for R4-2208909 E///: OK Apple: fine with CR Huawei: to vivo, please see our feedback for R4-2208909. Nokia: same comments as R4-2208909. |
| R4-2208915 (HW) | Correction to paging interruption during reselection requirements_r15 |
|  | Qualcomm: - The CR adds requirements for Inter-RAT LTE to NR paging interruption. Seems okay, but this does not seem like a Cat. F CR. MTK: OK E///: OK Apple: fine Huawei: Thank QC, MTK, E/// and Apple for the comments. To QC, the paging monitoring interruption time requirement during reselection to NR is already captured in 38.133 while it's missing 36.133. Technically, in 36.133, RAN4 had defined paging monitoring interruption requirement during reselection to 2G/3G/4G/Non-3GPP. It makes sense to define clear interruption requirements for reselection to NR, otherwise it would be unclear whether and how much interruption are allowed ,so this was clearly an oversight, but not intentional. So The changes made in R4-2208915 are not to add a new requirement but it’s just an fix to previous oversight, so we understand Cat.F is the appropriate category. Nokia: We think this change is not essential in R15. We could introduce this correction in later release. |
| R4-2208916 (HW) | Correction to paging interruption during reselection requirements_r16 |
|  | Moderator: Cat-F CR for R16 due to difference between R15 and R16 specs. Qualcomm: - The same comment as R4-2208915 MTK: OK E///: OK Apple: one small comment for clarification: measTimingConfig-r15 can be absent, and when it’s absent, the UE assumes that SSB periodicity is 5ms in this frequency. So it needs also to clarify this case when measTimingConfig-r15 is absent. We found the corresponding requirement in TS38.133 didn’t clarify this case either, maybe TS38.133 could be revised in next meeting or in other CR. Huawei: To QC, please refer to our feedback for R4-2208915. To Apple, yes, we agree that the case measTimingConfig is absent should be addressed. We'll handle it in revision. And for 38.133, we're fine to bring 38.133 CR next meeting. Nokia: In general, we are fine with the proposal, however, the wordings need some clarification. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP-r16, the SMTC periodicity follows the periodicity configured in smtc2-LP-r16; otherwise, the SMTC periodicity follows measTimingConfig-r15 Revised version will be uploaded. |
| R4-2209186 (HW) | CR on CSSF outside MG R15 |
|  | E///: We’re fine for intra-band FR2 case, but for SCCs across different FR2 bands it’s better to keep the flexibility of different SMTC offset since there may not be even any coordination between non-co-located BSs which may be using different bands. Huawei: thanks E/// for the comments. To E///, in Rel-15 we have only intra-band FR2 CA, so we guess the change should be fine based on the comments. For Rel-16, for inter-band FR2 CA the PCell must be in FR2 according to the band combinations from RF, while the CR is addressing the scenario with only SCCs in FR2. Would you please let us know if the CR is agreeable based on this clarification? Nokia: Change is fine in Rel15, but the wording has changed from R16 v16.7.0. R15 & R16 should be agreed at same time. |

CRs for the Perf part

| CR/TP number | Comments collection |
| --- | --- |
| R4-2207646 (Anritsu) | R4-2207646 Draft CR to FR1 DCI-based BWP switch TCs |
|  | MTK: OK Nokia: the change is not necessary. this test case is to verify both DCI-based BWP switch and timer-based BWP switch. |
| R4-2207648 (Anritsu) | Draft CR to FR1 DCI-based BWP switch TCs and FR2 CSI-RS based RLM |
|  | Moderator: Cat-F CR for R17 due to difference between R15/16 and R17 specs. MTK: OK Nokia: Same comments as R4-2207646 on the change#1. |
| R4-2207748 (Apple) | draftCR on applicabiltiy for test Cases involving E-UTRA/FR1 and FR2 carriers (R15) |
|  | MTK: OK |
|  | R&S: Depends on Sub-topic 1-2. TC 5.7.1.3 This is an accuracy TC where only the absolute accuracy of FR2 cell is evaluated, while only the report is sent on FR1 cell. However, the test criteria is not the timing of reporting, but its content. In addition, the report is transmitted periodically and there will not be any evaluation until a report have been successfully received. The test case is in additional statistical, there will be many events required to come to a pass or fail. Thus, since the reporting is not time critical, we don’t see a reason to skip the test case. |
| R4-2207948 (QC) | R4-2207948 draft Cat-F CR (R15) to SCell Activation Test Cases |
|  | MTK: OK Nokia: The CR is not agreeable. Same comments as core part R4-2207941. |
| R4-2208166 (CATT) | R4-2208166 Draft CR to add missing SMTC pattern |
|  | Moderator: Cat-F CR for R17 due to difference between R15/16 and R17 specs. MTK: OK Nokia: OK. |
| R4-2208167 (CATT) | Draft CR on radio link monitoring test cases in FR1 |
|  | MTK: OK Nokia: The change is not agreeable. The related part is introduced in R4-2001613 in 94e meeting that UE use other RS instead of the SSB for RLM as BFD-RS to avoid BFD process to kick in the interfere with the test. |
| R4-2208341 (OPPO) | CR to maintain test case of PScell addition and release delay (A4.5.7)_R15 |
|  | Moderator: Title of the CR in the cover sheet is wrong. OPPO: Thanks. A revision number is requested. MTK: OK Nokia: OK. |
| R4-2208906 (HW) | Correction to cell reselection test case_r15 |
|  | MTK: OK Nokia: OK. |
| R4-2209078 (E///) | draft CR: Correction of PRACH configuration parameter for inter-RAT test |
|  | Moderator: Cat-F CR for R16 due to difference between R15 and R16 specs. MTK: OK Nokia: OK. |
| R4-2209609 (R&S) | Draft CR to TS 38.133: Corrections to intra-frequency event triggered test cases (Rel 15) |
|  | Moderator: Resubmission of R4-2206809 which did not have any technical objections in RAN4#102, but which was postponed due to CR number cap. MTK: OK Nokia: OK. |
| R4-2209612 (R&S) | Draft CR to TS 38.133: Corrections to beam failure and link recovery test cases (Rel 15) |
|  | MTK: OK Nokia: OK. |

Summary for 1st round

Open issues

Sub-topic 1-1: Test of relative accuracy for FR2 inter-frequency RSRP

- Issue 1-1-1: whether to add Ginter when two cells are in same band Tentative agreements: Add Ginter = [3] dB also when two cells are in same band, for both upper bound and lower bound Candidate options: None Recommendations for 2nd round: Closed, no discussion needed.

- Issue 1-1-2: whether to add E to the upper bound Tentative agreements:None Candidate options: - Option 1 (Apple, HW) - Yes - E is no larger than Y, and smaller value can be considered (Apple) - E = 3dB (HW) - Option 2 (MTK, QC, E///) - No Recommendations for 2nd round: Further discuss the options

- Issue 1-1-3: whether to modify the test procedure to compensate the relaxation margins Tentative agreements: RAN4 not modify the test procedure to compensate the relaxation margins for Rel-15 Candidate options: None Recommendations for 2nd round: Closed, no discussion needed.

- Issue 1-1-4: margin for the lower bound when two cells are in same band Tentative agreements: For intra-band case, at lower bound, add margin D ([5.5]dB) + Ginter ([3]dB) Candidate options: None Recommendations for 2nd round: Closed, no discussion needed.

Sub-topic 1-2: Other issues with test case of FR2 inter-frequency RSRP accuracy

- Issue 1-2-1: Applicability of the test considering FR1+FR2 testability Tentative agreements: None Candidate options: - Open issue: - Whether to add A.5.7.1.3 and A.7.7.1.3 in A.3.13A to allow UE not to pass the tests - Option 1 (Apple, HW) - Yes - Option 2 (MTK, E///, R&S) - No Recommendations for 2nd round: Further discuss the options

- Issue 1-2-2: Test requirements for absolute accuracy Tentative agreements: add Gmin and Gmax in A.5.7.1.3 and A.7.7.1.3 test requirements. Candidate options: None Recommendations for 2nd round: Closed, no discussion needed.

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #2: Rel-16 NR RRM maintenance

Companies’ contributions summary

| T-doc | Company | Proposals / Observations |
| --- | --- | --- |
| Core part |  |  |
| R4-2207877 | MediaTek (Shenzhen) Inc. | CR Corret the requirement on transmission timing error for sidelink transmissions when NR Cell is synchronization reference source with 30kHz SCS of SSB and SL signal. The correction is aligned with the agreements in R4-1909993. |
| R4-2208829 | vivo | CR • Changed ‘inter-RAT’ to ‘inter-frequency’ for beam level measurements • Corrected reference sections in Rel-16 IDLE CA/DC measurements requirements • Editorial corrections |
| R4-2208830 | vivo | CR for R17 • Corrected SSB index reading time requirements • Corrections on reference sections in Rel-17 IDLE CA/DC measurements requirements |
| R4-2208831 | vivo | CR for 36.133 • Corrected TBDs with reference clauses for corresponding accuracy requirements |
| R4-2209189 | Huawei, Hisilicon | CR for 36.133 - Add the scaling factor NNR_carrier_HST + NNR_carrier to TSSB_index - Update TBD to the acutal the reference section numbers |
| R4-2209191 | Huawei, Hisilicon | CR - Add the scaling factor Kcarrier to TSSB_index - Add the clause number for the accuracy requirements |
| R4-2207787 | Apple | CR In RAN#101-e the applicability fo MRTD requirements for mTRP was agreed and draftCR R4-2120265 was endorsed. Sub-clause was captured in “[]” Delete “[]” around section number in Introduction sub-clause. |
| R4-2208848 | Samsung | CR Editorial correction is needed for the scheduling restriction requirement for L1-SINR measurement on FR2, i.e., “CSI-RS for L1-RSRP measurement” cause confusion and should be removed. |
| R4-2208987 | Huawei, Hisilicon | Proposal 1: For PL-RS switching in FR2, the following two options can be considered to define the PL-RS switching delay when the target PL-RS is SSB and used for L1-RSRP measurements. - Option 1: (Preffered) - To clarify that longer PL-RS switching delay is expected, which can be captured in the note. - Option 2: - To define the PL-RS switching delay as 5*TL1-RSRP_SSB, where TL1-RSRP_SSB is SSB based L1-RSRP measurement period with the assumption of M=1. |
| R4-2208988 | Huawei, Hisilicon | CR To clarify that longer application time is expected if in FR2 the target PL-RS is a SSB on which UE performs L1-RSRP measurements. |
| R4-2208922 | Huawei, Hisilicon | CR for 36.133 impact of smtc2-LP-r16 is not considered in cell-reselection in HST case due to the parallel nature of TEI16 WI and NR HST enhancement WI 1. It's clarified that periodicity of the SMTC window used by the cell being identified shall be used to determine cell-reselection related requirements. just as what RAN4 had done to relaxed measurements. 2. Copy and paste typo is corrected. |
| R4-2208924 | Huawei, Hisilicon | CR impact of smtc2-LP-r16 is not considered in cell-reselection in HST case due to the parallel nature of TEI16 WI and NR HST enhancement WI 1. It's clarified that periodicity of the SMTC window used by the cell being identified shall be used to determine cell-reselection related requirements. just as what RAN4 had done to relaxed measurements. 2. Copy and paste typo is corrected. |
| R4-2208931 | Huawei, Hisilicon | CR for 38.174 Remove gap related aspects in core and performance requirements. 1. Remove gap in 12.3.2.3.2 2. Remove gap configuration in G.2.3.2.1 and G.2.3.2.3 |
| R4-2208836 | vivo | CR • Specified UE behavior for Rx-Tx due to UE autonomous timing adjustment. • Editorial changes. |
| R4-2209193 | Huawei, Hisilicon | Proposal: Update the start point of PRS measurement period to account for the deferred MT-LR. |
| R4-2209194 | Huawei, Hisilicon | CR Clarify the start point of PRS measurement period for deferred MT-LR. |
| R4-2207944 | Qualcomm Incorporated | CR According to RAN1 spec, TS38.214, UE is supposed to drop CQI report before receiving at least one CSI-RS transmission occasion for channel measurement during SCell activation which conflicts with test description and criteria on CQI report during SCell activation. Added a condition where UE does not have to transmit a CSI report during SCell activation. |
| R4-2209199 | Huawei, Hisilicon | CR Clarify that when none of the conditions are met, requirements with TCI indication and L1-RSRP apply for multiple SCell activation. - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, |
| R4-2209201 | Huawei, Hisilicon | CR 1. Clarify that the interruption length Y1 and Y2 during SIB1 decoding are also for each interruption. 2. Clarify that the lower bound for SIB decoding interval is 20ms for SSB/RMSI MUX pattern |
| R4-2207946 | Qualcomm Incorporated | CR According to RAN1 spec, TS38.214, UE is supposed to drop CQI report before receiving at least one CSI-RS transmission occasion for channel measurement during SCell activation which conflicts with test description and criteria on CQI report during SCell activation. Added a condition where UE does not have to transmit a CSI report during SCell activation. |
| R4-2208344 | OPPO | Remove the cell-ranking criteria for inter-RAT measurements subject to CCA. |
| R4-2208927 | Huawei, Hisilicon | CR for 36.133 In current requirements, UE is required to monitor first two successive candidate SSB positions without differentiation between cell detection and other requirements. Change the definition of not available SMTC in 4.2.2.5.7 and 8.1.2.4.21A |
| R4-2208929 | Huawei, Hisilicon | CR In current requirements, UE is still required to monitor two candidate SSB positions in cell detection. Change the definition of not available SMTC in 4.2A for cell detection. |
| R4-2207875 | MediaTek (Shenzhen) Inc. | CR - Revise the maximum timing changed as 3200/2µ Tc, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211. - Delete the corresponding descriptions of SMTC in the minimum requirement for SSB Based RLM for NR-U. - Add the interruption requirements due to gap for single carrier on NR side under EN-DC & NE-DC. |
| Perf part |  |  |
| R4-2208920 | Huawei, Hisilicon | CR for eMIMO BFD TC 1. Missing test parameters are added for eMIMO BFD TCs: - initial/dedicated BWP RMC; - dedicated CORESET RMC; - PDSCH RMC; - BW and data allocated RBs - CSI reporting configuration 2. Test configuration 2 (LTE TDD + NR FDD, 120k SCS) is added for EN-DC TCs 3. 2X2 Antenna configuration is removed. 4. CSI reporting periodicity is changed to 5ms. 5. PRACH RMC is changed to Config.4 FR2. 6. SR offset for SCell BFR is changed to 4. 7. Test parameters are updated. |
| R4-2208990 | Huawei, Hisilicon | CR for L1-SINR TC 1. To define the beam assumption in sections A.5.6.6.1 and A.5.6.6.2. 2. To correct the value of Io in some FR2 L1-SINR measurement test cases |
| R4-2207649 | Anritsu Corporation | CR for HST Cell reselection TC Change the test time value(T2,T3) to match the value described in the A.6.1.1.7.3 Test Requirements. |
| R4-2208162 | CATT | CR for HST FR1 L1-RSRP TC Change the test requirements for delay by using DRX.3 but not DRX.8 configuration. |
| R4-2210091 | MediaTek Inc. | CR for DAPS TC Including missing information on CSI report used for DAPS handover test case and correcting some typos in the same test case. |
| R4-2208024 | Qualcomm Incorporated | Proposal 1: For UE Rx-Tx measurement accuracy requirements in FR1, add group delay calibration margins listed in the table below. Min(PRS BW, SRS BW) (MHz); Margin (Tc). ≥ 5. Proposal 2: For RSTD measurement accuracy requirements in FR1 when the reference and target PRS resources are in the same PFL (single PFL case), add group delay calibration margins listed in the table below. PRS BW (MHz); Margin (Tc). ≥ 5. ≥ 10. Proposal 3: For RSTD measurement accuracy requirements in FR2 when the reference and target PRS resources are in the same PFL (single PFL case), add group delay calibration margins listed in the table below. PRS BW (MHz); Margin (Tc). ≥ 20. Proposal 4: For PRS-RSTD measurements in FR1 with neighbor and reference PRS resources in different PFLs, the value of the group delay calibration margin is FFS. Proposal 5: For PRS-RSTD measurements in FR2 with neighbor and reference PRS resources in different PFLs, the value of the group delay calibration margin is FFS. Proposal 6: If the uplink transmission timing changes due to UE autonomous timing adjustment during the UE Rx-Tx measurement period, - UE Rx-Tx measurement accuracy requirements apply for a cell/TRP that is also the downlink reference cell (defined in TS 38.133, section 7.1.1) for SRS transmission. - UE Rx-Tx measurement accuracy requirements do not apply for a cell/TRP that is not the downlink reference cell (defined in TS 38.133 section 7.1.1) for SRS transmission. In such case, the UE may restart the UE Rx-Tx measurement period. |
| R4-2208202 | CATT | CR for PRS measurement delay TCs 1. Correct the PRS configuration in A.3.31.2. 2. Correct the value of expected RSTD uncertainty in table A.6.6.12.1.1-2. 3. Correct the reference in table A.7.6.9.2.1-2. 4. Update the side condition for PRS-RSRP and UE Rx-Tx in clause B.2.14. |
| R4-2208204 | CATT | CR for PRS measurement accuracy TCs and side condition of PRS measurement in Annex B 1. Add PRS muting information for accuracy requirements test cases 2. Correct the PRS configurations and the power configurations for PRS-RSRP accuracy requirements test cases. 3. Add a sub-test for UE Rx-Tx time difference accuracy requirements test cases. 4. Correct the PRS power configuration for UE Rx-Tx time difference accuracy requirements test cases. 5. Other corrections. |
| R4-2208833 | vivo | Proposal 1: UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. Proposal 2: FFS how to define test to verify the accuracy requirements, or not to define test to verify accuracy requirements on non-reference cell. |
| R4-2208834 | vivo | CR for UE Rx-Tx accuracy • Specified that UE shall meet UE Rx-Tx time difference measurement accuracy requirements due to UE autonomous timing adjustment. • Removed UE behaviour related to core requirements |
| R4-2209196 | Huawei, Hisilicon | Proposal 1: Add the following group delay calibration margin for RSTD accuracy for single PFL case. Table 1: Calibration margin for RSTD FR1 single PFL case PRS BW (MHz); Margin (Tc). ≥ 5; [80]. ≥ 10; [64]. Table 2: Calibration margin for RSTD FR2 single PFL case PRS BW (MHz); Margin (Tc). ≥ 20; [64]. Proposal 2: Add the following group delay calibration margin for UE Rx-Tx accuracy. Table 3: Calibration margin for UE Rx-Tx FR1 Min{PRS BW, SRS BW} (MHz); Margin (Tc). ≥ 5; [128]. Proposal 3: Applicability of Rx-Tx accuracy requirements with autonomous timing adjustment is defined as: - UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) - UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. UE may restart the measurement period in such case Proposal 4: RAN4 not to define new test case for UE-based positioning. Proposal 5: Update the PRS repetition number to 4 for the smallest BW in RSTD and UE Rx-Tx accuracy requirements for FR2 AWGN. |
| R4-2209197 | Huawei, Hisilicon | CR for RSTD and UE Rx-Tx accuracy 1. Add the group delay calibration margin for remaining BWs in RSTD and UE Rx-Tx requirements. 2. Add the applicability of UE Rx-Tx accuracy requirements with autonomous timing adjustment. 3. Update the PRS repetition number to 4 for the smallest BW in RSTD and UE Rx-Tx accuracy requirements for FR2 AWGN. 4. It is noted that in UE Rx-Tx accuracy FR2 fading channel, the repetition number for 120kHz and 32 RB are mistakenly captured as 1, but it should be 4 based on agreements in R4-2108040. |
| R4-2210181 | Ericsson | - Proposal 1: Do not define any DL-TDOA reporting delay tests for UE-based DL-TDOA. - Proposal 2: Do not define any DL-TDOA accuracy tests for UE-based DL-TDOA. - Proposal 3: Do not modify any existing PRS measurement delay or accuracy test to include any aspect related to UE-based DL-TDOA. - Proposal 4: If the applicability of Rx-Tx accuracy requirements with autonomous timing adjustment is defined based on Option 1 (WF in R4-2206821) then X = 64 Tc for FR1 and X=32 Tc for FR2. - Proposal 5: If no consensus is reached on value of X in Option 1 (WF in R4-2206821), then the pplicability of accuracy requirements under TA adjustment if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment is specified based on Option 2 (WF in R4-2206821). |
| R4-2210182 | Ericsson | CR for UE Rx-Tx accuracy The following aspect of the UE Rx-Tx measurement requirements are added: UE Rx-Tx measurement accuracy applicability under uplink transmission timing changes during the UE Rx-Tx measurement period due to the autonomous timing adjustment. The UE behavior is defined based on WF in R4-2206821. |
| R4-2210184 | Ericsson | CR for side condition of PRS measurement in Annex B According to sections 10.1.24 and 10.1.25 the PRS-RSRP and UE Rx-Tx accuracy requirements are applicable for PRS Ês/Iot ≥ -3 dB and for Ês/Iot ≥ -13 dB. These conditions are introduced in annex B. |
| R4-2210225 | Qualcomm Incorporated | CR for UE Rx-Tx accuracy 1. Correct the UE Rx-Tx group delay calibration margins. |
| R4-2208985 | Huawei, Hisilicon | CR for UL switching TCs 1. Remove the power boosting of CSI-RS 2. Some editorial changes are made. |
| R4-2207789 | Apple | CR for spatial relation RMC Added Pathloss reference RS to PUCCH spatial relation |
| R4-2207951 | Qualcomm Incorporated | CR for multiple SCell activation TC and SRS RMC - Added a condition where UE does not have to transmit a CSI report during SCell activation. - Correct startPosition in A.3.24 from 0 to 5 |
| R4-2208918 | Huawei, Hisilicon | CR for SRS RMC Correct startPosition in A.3.24 from 0 to 3 |
| R4-2208164 | CATT | CR for cell reselection with power saving TC 1. Change cell power levels in A.7.1.1.5 and A.7.1.1.6, Es/Io and Io also update accordingly. 2. Change to correct parameter names |
| R4-2207953 | Qualcomm Incorporated | CR for NR-U SCell activation TC - Added a condition where UE does not have to transmit a CSI report during SCell activation. |
| R4-2209076 | Ericsson | CR for NR-U TCs Change 1: Correction of parameter table for NR-U BFD test. Changes 2 and 3: Removal of []. |
| R4-2207731 | Qualcomm, Inc. | CR for SRS RMC Correct startPosition in A.3.24 from 0 to 5 |
| R4-2208734 | ZTE Corporation | CR for 38.174 Correct in the corresponding places that the test configuration / requirements are for IAB-MT, not UE. |

Open issues summary

Note: Only issues proposed in discussion papers are listed in this section. For other issues proposed via CR, please provide comments to the CR directly in section 2.3.1 and 2.3.2.

Sub-topic 2-1: eMIMO

Issue 2-1-1: FR2 PL-RS switching delay when the target PL-RS is SSB and used for L1-RSRP measurements

- Proposals

  - Option 1 (HW)

    - To clarify that longer PL-RS switching delay is expected, which can be captured in the note.

  - Option 2 (HW)

    - To define the PL-RS switching delay as 5*TL1-RSRP_SSB, where TL1-RSRP_SSB is SSB based L1-RSRP measurement period with the assumption of M=1.

- Recommended WF

  - Further discuss the two options

| Company | Comments |
| --- | --- |
| MediaTek | Support option 1. we prefer to keep the same agreement as WF in the last meeting. Because what is the exact value may have to do with the discussion in R17 feMIMO. |
| vivo | No strong view. But we think current R16 PL-RS requirements may also work without this change. Given it is already late even for R17, we slightly prefer not to adopt either option, so that R17 feMIMO requirements are not impacted. |
| Apple | In our understanding if PL-RS is not maintained, additional time for Pl-RS measurement is needed and if the PL-RS is SSB in FR2, time for RX beam sweep should also be allowed. We should not have extension only if the RS for L1-RSRP measurement is SSB, but if the PL-RS is SSB. For PL-RS activation delay, we don’t explicitly capture the delay requirement when PL-RS is unknown – only note that longer delay is expected. For known PL-RS, but not maintained the delay should be Tfirst_SSB + 4*TL1-RSRP_SSB, where Tfirst_SSB is the time to first SSB after MAC-CE decoding, and we need 4 samples for measurement also including beam sweep. With option 2 the delay may be longer than needed. With option 1, the note doesn’t provide a delay requirement while it can be specified, hence we don’t prefer it. Also, the note suggests that longer delay is when target PL-RS is the SSB on which UE performs L1-RSRP measurement, but what happens when SSB based L1-RSRP is not configured on target PL-RS and target PL-RS is SSB and not maintained? |
| Huawei | We prefer option 1, but we can compromise to option 2. |
| Samsung | Option 1 is supported. Option 2 is not preferred. |
| Nokia | This is also discussed in FeMIMO, we need discuss what the PL-RS is in general, it is not clear. |

Sub-topic 2-2: Positioning

Issue 2-2-1: Start of measurement period

- Proposals

  - Option 1 (HW)

    - Update the start point of PRS measurement period to account for the deferred MT-LR.

- Recommended WF

  - Check if option 1 is agreeable

| Company | Comments |
| --- | --- |
| Qualcomm | We would like a different requirement depending on whether the time of the event for deferred LR is known (or predictable) ahead of time. Here's our proposal: - For scheduled LR and deferred LR with periodic reporting: Since the target time T is known by the UE ahead of time, the UE shall start the measurements no later than T_start = T – T_meas_period – max(T_available_PRS), where max(T_available_PRS) is the maximum over all the configured PFLs, so that the UE will have finished the measurements by time T. NOTE: Subtracting max(T_available_PRS) is necessary because T is an arbitrary time that may not be aligned with a time instant where PRS is available for measurement. This requirement would apply only when the target time T is known to the UE before T_start. - For deferred LR with events other than periodic location: Since the time (T) of the triggering event is not known to the UE ahead of time, the UE shall start the measurements no later than T_start = T + max(T_available_PRS), where max(T_available_PRS) is the maximum over all the configured PFLs. Adding max(T_available_PRS) is necessary because T is an arbitrary time that may not be aligned with a time instant where PRS is available for measurement. |
| vivo | In general, it is better to follow the conclusion of R17 ePos if the starting time for scheduled location time can be reused. In our view, the starting time for deferred MT-LR should be up to UE implementation. It is difficult to specify a fixed time. We don’t think UE needs to start the position measurement before the entire measurement period, which could be very long if large number of frequency layers for RRM measurement are configured. |
| E/// | Support option 1. Since scheduledLocationTime T indicates time T when the location measurement or location estimate is to be obtained, the measurement shall start at T or at least at the first instance of MG that is aligned with DL PRS resource. From 37.355: scheduledLocationTime indicates the time T when the location measurements or location estimate is to be obtained. Furthermore, following is defined in section 4.1c Scheduled Location Time of TS 23.273: “…The location preparation phase ends at or near to the time T and is followed by a location execution phase in which the UE location is obtained and returned to the external LCS Client, AF or the UE…” |
| CATT | Support option 1. Same view as Ericsson that the measurement should start from T. Considering the alignment with MG occasion, the start of measurement period can be the first MG occasion covering the DL PRS resources after T. |
| Huawei | We support option 1 which can be a common requirements for all cases of deferred LR. This is our first preference. We are also open to consider the alternative proposed by QC. Case 2 is aligned with option 1 so we are basically fine with it. On Case 1 we are not sure if we need to specify T_start, since a UE may be able to complete the measurement in a shorter time than T_meas_period (RAN4 requirement), and in this case the UE should be allowed to start measurement later than T_start. If we understand QC’s suggestion for case 1 correctly, what is important is that UE can complete the measurement by T, so we can capture this as a requirement but leave it to UE implementation when to start the measurement. |
| Nokia | We support option 1. We observe that the reference is TS 23.273 rather than TS 23.237. |

Issue 2-2-2: group delay calibration margin for RSTD

- Proposals

  - Option 1 (QC)

    - Calibration margin for RSTD FR1 single PFL case

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 5 | 160 |
| ≥ 10 | 80 |

    - Calibration margin for RSTD FR2 single PFL case

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 20 | 80 |

    - For PRS-RSTD measurements in FR1 with neighbor and reference PRS resources in different PFLs, the value of the group delay calibration margin is FFS.

    - For PRS-RSTD measurements in FR2 with neighbor and reference PRS resources in different PFLs, the value of the group delay calibration margin is FFS.

  - Option 2 (HW)

    - Calibration margin for RSTD FR1 single PFL case

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 5 | [80] |
| ≥ 10 | [64] |

    - Calibration margin for RSTD FR2 single PFL case

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 20 | [64] |

- Recommended WF

  - Further discuss the options

  - It seems no value is listed in Proposal 2 and 3 in R4-2208024, could QC please check?

| Company | Comments |
| --- | --- |
| MTK | Support option 2. |
| Qualcomm | We support option 1. For FR1 BW=10 MHz and FR2 BW=20 MHz our proposals are reasonably close to the corresponding values in option 2. For FR1 BW=5 MHz, the two options remain quite far apart. However, it’s not clear to us why the difference between the margins for 5 MHz and 10 MHz in option 2 is only 16 Tc. |
| E/// | Support option 2. |
| CATT | Support option 2. |
| Huawei | For FR1 10MHz and FR2 20MHz we are fine with either option since two options are close. For FR1 5MHz, we suggest 120Tc as a compromise. To QC, We were assuming that somehow an improved calibration performance for the small BW. Of course, this comes with increased implementation costs, so we are open to compromised values. |
| Qualcomm2 | Thanks to Huawei for offering to compromise. For 5 MHz, we can accept 120 Tc as a compromise. For the others, since they are close either the max or average would be OK for us. |

Issue 2-2-3: group delay calibration margin for UE Rx-Tx

- Proposals

  - Option 1 (QC)

    - Calibration margin for UE Rx-Tx FR1

| Min{PRS BW, SRS BW} (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 5 | 160 |

  - Option 2 (HW)

    - Calibration margin for UE Rx-Tx FR1

| Min{PRS BW, SRS BW} (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 5 | [128] |

- Recommended WF

  - Further discuss the options

  - It seems no value is listed in Proposal 1 in R4-2208024, could QC please check?

| Company | Comments |
| --- | --- |
| MTK | Support option 2. |
| Qualcomm | In option 1 our proposal is 160 Tc. Based on our internal data this is feasible. |
| E/// | Support option 2. |
| CATT | Support option 2. |
| Huawei | We are fine with either option since two options are close. |
| Qualcomm2 | Can other companies compromise as well? |

Issue 2-2-4: Applicability of UE Rx-Tx accuracy requirements in case of UE autonomous timing adjustment

- Proposals

  - Option 1 (E///)

    - If the autonomous timing adjustment is below threshold X

      - UE Rx-Tx measurement accuracy requirements shall apply

    - Otherwise

      - UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1)

      - UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. UE shall restart the measurement period in such case

    - X = 64 Tc for FR1 and X=32 Tc for FR2

  - Option 2 (QC, HW, E///)

    - UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1)

    - UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. UE may restart the measurement period in such case

  - Option 3 (vivo)

    - UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

    - FFS how to define test to verify the accuracy requirements, or not to define test to verify accuracy requirements on non-reference cell.

- Recommended WF

  - Further discuss the options

| Company | Comments |
| --- | --- |
| Qualcomm | Support Option 2. |
| vivo | Firstly, supporting UE Rx-Tx time difference measurement on neighbor cells are critical for multi-RTT positioning. Core requirements are already specified for the neighbor cell measurements. If the accuracy requirements cannot be met on a non-reference cell if UE autonomous timing adjustment happens, UE may restart the measurement period. However, the UE autonomous adjustment of uplink transmit timing could happen quite frequently, e.g., every 200ms. It is much shorter than PRS measurement period in typical measurement scenarios, especially considering if there are other RRM measurements that would compete measurement gaps. Consequently, there would be no valid measurement results available at all due to UE may restart the measurements over and over. It effectively means UE Rx-Tx time difference measurement on neighbor cells cannot be supported. Secondly, according to the definition of UE Rx-Tx time difference, the time difference should be based on neighbor cell Rx timing and reference cell Tx timing. Therefore, UE should be able to meet accuracy requirements even if there is Tx timing change on reference cell due to UE autonomous adjustment. In other words, the accuracy should be based on the Tx timing for each measurement after UE autonomous timing adjustment. Thirdly, in the test there would be no UE autonomous timing adjustment impact since DL timing is not supposed to change during the measurement period. There should be no issue for UE to pass the test. Lastly, in the field, UE Rx-Tx time difference measurements and gNB Rx-Tx time difference measurements are combined for RTT-based positioning. The Tx timing change due to UE autonomous timing adjustment will be cancelled out as it will be included in both UE and gNB measurements. In addition, according to WF R4-2105851 in the RAN4#98-bis-e meeting, following agreements were made. - UE continues UE Rx-Tx time difference measurement and the current measurement requirements apply when UL timing change due to UE autonomous adjustment occurs during the measurement period Therefore, we still think option 3 is reasonable and agreements in the RAN4#98-bis-e should be kept. |
| E/// | We can compromise to Option 2. After many meetings of debate, RAN4 already agreed to choose between Option 1 and Option 2 at RAN4#102-e (R4-2206821, WF on maintenance to R16 POS requirements). So in our view Option 3 proposed by Vivo was already ruled out. RAN4 should respect previous agreements. |
| CATT | Our understanding is more aligned with option 3, since the UE autonomous adjustment is quite frequent, the UE will always restart the Rx-Tx measurement and not meet the accuracy requirements for neighbor cell. Then how to guarantee the positioning performance. Is option 2 reasonable for supporting Multi-RTT positioning? |
| Huawei | Option 2 |
| Nokia | We support option 2. |
| Ericsson2 | To CATT: Option 2 is already a compromise since it says: “UE may restart the measurement period in such case”. This means UE has an option not to restart the measurement e.g. if the autonomous change is small. But it can restart when e.g. autonomous change is large. When the autonomous change is large then timing error can be more than the accuracy and will make the measurement useless anyway. |
| Qualcomm2 | Agree with Ericsson’s comment. Option 2 is a reasonable compromise and it took us many meetings to get here. |

Issue 2-2-5: Test for UE based DL-TDOA positioning

- Proposals

  - Option 1 (HW, E///)

    - Do not define any DL-TDOA reporting delay tests for UE-based DL-TDOA.

    - Do not define any DL-TDOA accuracy tests for UE-based DL-TDOA.

    - Do not modify any existing PRS measurement delay or accuracy test to include any aspect related to UE-based DL-TDOA.

- Recommended WF

  - Agree on option 1

| Company | Comments |
| --- | --- |
| E/// | We support Option 1. |
| CATT | Fine with option 1. |
| Huawei | Support the Recommended WF |
| Nokia | We support the recommended WF. |

Issue 2-2-6: PRS repetition number for small BW in accuracy requirements

- Proposals

  - Option 1 (HW, E///)

    - Update the PRS repetition number to 4 for the smallest BW in RSTD and UE Rx-Tx accuracy requirements for FR2 AWGN.

- Recommended WF

  - Check if option 1 is agreeable

| Company | Comments |
| --- | --- |
| Qualcomm | Support option 1. |
| vivo | Support option 1. |
| E/// | We support Option 1. |
| Huawei | Support option 1 |
| Nokia | We support option 1. |

Comments to the CRs

Cat-A draftCRs are not listed for comments.

CRs for the Core part

| CR/TP number | Comments collection |
| --- | --- |
| R4-2207877 (MTK) | Maintenance CR for V2X Te requirements on 38.133 R16 |
|  | vivo: The change is ok, |
| R4-2208829 (vivo) | Draft CR to TS 38.133 Correction to Rel-16 IDLE CA&DC measurements requirements |
|  | MTK: OK Nokia: OK |
| R4-2208830 (vivo) | Draft CR to TS 38.133 Correction to Rel-17 IDLE CA&DC measurements requirements |
|  | Moderator: Cat-F CR for R17 due to difference between R16 and R17 specs. MTK: OK Nokia: OK |
| R4-2208831 (vivo) | Draft CR to TS 36.133 Correction to IDLE DC measurements requirements_R16 |
|  | MTK: OK Apple: fine Nokia: OK |
| R4-2209189 (HW) | CR on beam level EMR requirements 36133 R16 |
|  | MTK: As beamMeasConfigIdle-r16 is configured per frequency layer, only NR inter-frequency carriers configured with idle mode CA measurements and beamMeasConfigIdle should be counted in but not all the NR inter-frequency carriers. vivo: Agree with MTK that only the frequency carriers configured for beam measurement should be accounted. Apple: Agree with MediaTek only the carriers in measIdleCarrierListNR with beamMeasConfigIdle->reportRS-IndexResultsNR-r16 = ‘true’ should be counted here. Huawei: Thanks MTK, vivo and Apple for the comments. We have a bit different view. In our understanding, the scaling factor is accounting for sharing between measurement on multiple frequency layers, or in other words, how often the UE would measure a particular carrier. Therefore, all the frequency layers should be considered. In an example, if there is 1 EMR carrier and N RRM carriers, following the comments the scaling factor for SSB index reading on the EMR carrier will be 1, which means UE is required to perform SSB index reading on the EMR carrier and at the same time perform RRM measurement on one of the RRM carriers, which is clearly not the intention. Please also note that the same approach as proposed in the CR has been used from Rel-15 for inter-frequency measurement, i.e. in clause 9.3.4 CSSF (instead of number of carriers with beam level reporting) is applied in defining TSSB_time_index_inter. Nokia: Same comments as R4-2209191 as below |
| R4-220919 (HW) | CR on beam level EMR requirements 38133 R16 |
|  | MTK: As beamMeasConfigIdle-r16 is configured per frequency layer, only NR inter-frequency carriers configured with idle mode CA measurements and beamMeasConfigIdle should be counted in but not all the NR inter-frequency carriers. vivo: Same comment as for 9189. Apple: same comment as for 9189 Huawei: To MTK, vivo and Apple, please see our reply for R4-2209189 Nokia: the CR is not agreeable with current change. Generally, the correction should be considered since current requirements is for one cell, however it is not clear how scaling is accounted. We think we shall only account carriers which are configured with beamMeasConfigIdle. TP should be updated to adapt: - Index reading for 1 cell (as now) - When multiple carriers are configured with beamMeasConfigIdle |
| R4-2207787 (Apple) | CR for eMIMO requirements maintenance (Rel-16) |
|  | MTK: OK Nokia: OK |
| R4-2208848 (SS) | CR to TS38.133 for the editorial correction on L1-SINR scheduling restriction |
|  | MTK: OK Nokia: OK |
| R4-2208988 (HW) | DraftCR on maintaining PL-RS switching delay requirements R16 |
|  | MTK: depends on discussion in sub-topic 2-1. Apple: Please refer to our comments for Issue 2-1-1 Huawei: to MKT/Apple, we can revise the CR based on outcome of 2-1-1 if needed. Nokia: The CR is not agreeable. Same comments in last meeting are applied. |
| R4-2208922 (HW) | Correction to HST inter-RAT NR cell reselection requirements_r16 |
|  | Qualcomm: - HST enhancement speeds up the measurement, and therefore the power saving feature which slows down the measurement should not be applicable to HST. We propose to have "smtc2-LP-r16 is not applicable to the NR carriers configured with highSpeedCarrierNR-r16" instead. E///: The correction shall cover non-HST and HST, we don’t recommend to add the note in each HST table. Huawei: Thanks QC and E/// for the comments. To QC, after further checking we think it may not be a good idea to note "smtc2-LP is not applicable to NR HST carriers". The reasons are: - It has RAN2 spec impacts. In 38.331 cl.6.3.1 it's explicitly specified that cells in pci-list of smtc2-LP shall follow the measurement timing configuration given in smtc2-LP, which has nothing to do with HST or not. Surely confusion will be caused if we note that smtc2-LP is not applicable for HST in 38.133 while 38.331 requiring UE to follow smtc2-LP. - It limits NW implementations. In our opinion, the concern that measurement will be slowed down is unnecessary. - On one side, the changes made in R4-2208922/R4-2208924 are only to clarify which SMTC periodicity to be used when determining measurement requirements. It doesn't relax measurement delay. NW still has the freedom to control measurement delay. NW still can force UE following smtc1 if it doesn't want measurement to be slowed down (e.g. not configuring smtc2-LP or not include candidate cells in pci-list). - On the other side, if NW does include target cell in pci-list, it means that the NW is currently more concerned with other factors (e.g. power saving) than with measurement delay. We think this is a reasonable implementation, and there is no reason why smtc1 must be used. To E///, We understand there is no need to update non-HST table since non-HST requirements (Table 4.2.2.5.6-1) don't concern SMTC periodicity. Also HST requirements (Table 4.2.2.5.6-2) is the only place that concerns SMTC periodicity. Nokia: The change is ok in principle. The wording needs to be clarified. If high layer signalling smtc2-LP-r16 is configured, for cells indicated in the pci-List parameter in smtc2-LP-r16, the SMTC periodicity corresponds to the periodicity in smtc2-LP-r16; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc Revised version will be uploaded. |
| R4-2208924 (HW) | Correction to HST intra-NR cell-reselection requirements_r16 |
|  | Qualcomm: - HST enhancement speeds up the measurement, and therefore the power saving feature which slows down the measurement should not be applicable to HST. We propose to have "smtc2-LP-r16 is not applicable to UE configured with highSpeedMeasFlag-r16" instead. E///: The correction shall cover non-HST and HST, we don’t recommend to add the note in each HST table. Huawei: to QC and E///, please see our reply for R4-2208922. Nokia: The change is ok in principle. The wording needs to be clarified. Same TP proposal given for R4-2208922 is applied for this CR. |
| R4-2208931 (HW) | Draft CR on maintenance for IAB R16 |
|  | Nokia: OK |
| R4-2208836 (vivo) | Draft CR to 38.133 correction to NR positioning measurement requirements |
|  | Qualcomm: - OK to capture the previous RAN4 agreement. However, there is a related issue in NR_pos maintenance and, depending on the outcome of that issue, this CR may need to be revised. vivo: It can be updated based on conclusion for UE Rx-Tx time difference accuracy requirements. E///: This is related to issue 2-2-4. We do not agree with the proposed changes. Huawei: pending on outcome from 2-2-4 Nokia: The CR depends on the outcome for issue 2-2-4. We do not agree with the changes. |
| R4-2209194 (HW) | CR on PRS meausurement period R16 |
|  | Qualcomm: - Please see our comments on Issue 2-2-1. vivo: Depending on conclusion of issue 2-2-1. Huawei: to QC, we provided our reply to 2-2-1, and we can revise the CR based on the outcome. Nokia: The CR is agreeable in general. The reference to deferred MT-LR needs correction: TS 23.273. |
| R4-2207944 (QC) | draft Cat-F CR (R16) to SCell Activation Core |
|  | MTK: OK E///: Principle is OK. May be text needs bit of fine tuning. Suggestion from us is provided below (entire paragraph may be replaced with the below quoted text). Starting from the first CSI-RS transmission for the channel measurement after the reception of SCell activation command at slot n (as specified in clause 4.3 of TS 38.213 [3]) and until the SCell activation completion at UE, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell. Apple: same comment as to R15: we are fine with CR and prefer to keep the wording as we discussed from last meeting. QC: Thank MediaTek, Ericsson, Apple for your support and suggestions. We modified the wording based on Ericsson’s suggestion. And a reference section in RAN1 spec is also added to avoid any further misunderstanding. Please see below and let us know if there is still anything you want to modify or if you want to keep the original version. Starting from the first available CSI reference resource for the channel measurement and report (specified in clause 5.2.2.5 of TS 38.214 [26]) after the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the SCell activation completion at UE, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell. Huawei: same comment as to R15 CR Nokia: Same comments as R15 CR (R4-2207941) |
| R4-2209199 (HW) | CR to multiple SCell activation requirements R16 |
|  | MTK: OK Apple: fine Nokia: The change looks fine but clarifications are needed. The proposed change in this CR makes the UE requirements unclear and we propose some changes as below: - if none of the following conditions are met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells+Trs*N1 +Trs +5ms Otherwise: Tactivation_time_multiple_scells is: - 6ms + TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells + Trs*N1 + TL1-RSRP,measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC_multiple_scells + TFineTiming + 2ms, Tuncertainty_SP_multiple_scells), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells + Trs*N1 + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC_multiple_scells + 5ms + TFineTiming, Tuncertainty_RRC_multiple_scells + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - otherwise, TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells+Trs*N1 +Trs +5ms Revised version will be uploaded |
| R4-2209201 (HW) | CR on CGI reading requirements R16 |
|  | MTK: generally fine. But, we would like to know why is 20 ms when SSB and RMSI CORESET multiplexing pattern is 1. Huawei: thanks MTK for the comments To MTK, 20ms is the default RMSI scheduling periodicity for multiplexing pattern 1 according to the following texts from 38.331 the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB; In addition, the interruption requirements e.g. in 8.2.2.2.14 are also defined based on 20ms assumption. - K1 = 6 for the target cell carrier frequency on FR1 and K1 = 25 for the target cell carrier frequency on FR2, and - L1 = TSIB1/20 and - L2 = TSIB1/TSMTC, where TSMTC is the periodicity of the SMTC occasion configured for the target cell carrier. Nokia: OK |
| R4-2207946 (QC) | draft Cat-F CR (R16) to SCell Activation Core NR-U |
|  | MTK: OK, same change as R15 Core Nokia: Same comments as R15 CR (R4-2207941) |
| R4-2208344 (OPPO) | CR to maintain inter-RAT measurements subject to CCA in TS 36.133 |
|  | OPPO: The same changes for inter-RAT measurements (to remove the cell-ranking criteria) have been agreed in CR R4-2204308 in last meeting. MTK: OK E///: Reselection based on ranking is supported in 38.133 (e.g. See inter-frequency section). So the inter-RAT should be aligned, so no need for this CR. OPPO: To Ericsson, this CR is for 36.133. And similar changes had been agreed in last meeting (endorsed R15 CR R4-2204308). We think it is a simple CR to just reusing the agreement for CCA which starts from Rel-16. The technical concern has also been solved. Inter-RAT NR measurement may decouple from inter-frequency NR measurement. We cannot find that the margins for Ranking when Treselection = 0 in inter-RAT E-UTRAN measurement in TS 38.133 as well. If we look into TS 36304 clause 5.2.4.5, it could be clear that the ranking is defined for only equal priority E-UTRA frequency (i.e., inter-frequency). The equal priority NR frequency based on ranking belongs to NR inter-frequency measurement, but not inter-RAT measurement. Nokia: OK |
| R4-2208929 (HW) | Draft CR on requirements maintenance for NR-U 38133 R16 |
|  | MTK: OK Nokia: OK |
| R4-2207875 (MTK) | Maintenance CR for RRM requirements on 38.133 R16 |
|  | Qualcomm: - Changes #1, #2, #4, #6 and #7 refer to SSB measurements. Why do we need to change requirements based on SSB measurements? For change #8, it may be agreeable, but it may not be sufficient for the UE to meet measurement accuracy requirements, which are only met if the absolute timing difference is within one CP. E///: - Change 1: µ is the SCS configuration Fine - Change 2: Fine to delete - Change 3: redundant. - Single carrier no interruption. - For E-UTRA-NR dual connectivity (with NR single carrier, NR CA configuration), new addition wording is not needed MTK: Thank QC and E/// for the comments. To QC: If not scaled by 1/2u , the time offset would be larger than one CP when SCS is larger than 30kHz as shown below, then extra cell detection would be needed. We think limiting to ± 3200 Tc/2u is sufficient, the percentage is 35% which aligns with 15kHz SCS and also aligns with LTE. SSB SCS; SSB CP length; ± 3200 Tc; ± 3200 Tc/2u. kHz; us; Ts; in CP%; In CP%. 15; 4.7; 142.1; ± 35%; ± 35%. 30; 2.3; 71.1; ± 70%. 60; 1.2; 35.5; ± 141%. 120; 0.6; 17.8; ± 281%. 240; 0.3; 8.9; ± 563%. To E///: We have a bit different opinion on change#3. This is about the interruptions caused by MG. Even for single carrier, there will be interruptions. So we suggest to keep the revision. For the last bullet, we admit it may be a bit redundant but clearer. So we suggest to keep it. |

CRs for the Perf part

| CR/TP number | Comments collection |
| --- | --- |
| R4-2208920 (HW) | Correction to eMIMO BFD test cases_r16 |
|  | MTK: the configuration of CSI reporting periodicity in Table A.5.5.5.6.1-2, Table A.5.5.5.7.1-2, Table A.7.5.5.6.1-2 and Table A.7.5.5.7.1-2 seems duplicated. CSI reporting periodicity; 1; slot; 40. E///: Table A.5.5.5.7.1-3: should be applicable for 'Test 1 and 2' instead of only for 'Test 1', because test 2 is added in Table A.5.5.5.7.1-1. Huawei: thanks MTK and E/// for the comments To MTK, after some offline checking, it seems there is no duplication. To E///, We think it may not be right to use "test 1 & 2". "test 1" means the test parameters for the 1st sub-test (e.g. TC A.6.3.2.2). In R4-2208920 we are not adding any new sub-tests, but adding new test configurations. What needs to be modified is here： We will update in the revised version, and hope it is fine. |
| R4-2208990 (HW) | DraftCR on maintaining L1-SINR measurement test cases R16 |
|  | MTK: OK E///: - Table A.5.6.6.2.2-3: What is the reason the Io for CSI-RS#0 in T1 is rounded to -64 instead of -63.97? - It looks Io for CSI-RS#1 is different between Table A.5.7.6.2.2-3 (-59.00dBm/95.04MHz) and Table A.7.7.6.3.2-2 (-59.86dBm/95.04MHz). Is it intentional? Huawei: thanks MTK and E/// for the comments To E///, for the Io for CSI-RS#0 in T1 in Table A.5.6.6.2.2-3, either -64 or -63.97 is fine for us since these two values are quite close. For the Io for CSI-RS#1 in Table A.5.7.6.2.2-3, it is a typo. The correct value should be -59.86. Thanks for Ericsson’s carefully checking. |
| R4-2207649 (Anritsu) | Draft CR to Cell reselection to FR1 intra-frequency NR case |
| R4-2208162 (CATT) | Draft CR on HST FR1 L1-RSRP test cases |
|  | Qualcomm: - The change should be 640ms instead of 640 CATT: to Qualcomm, yes, unit of ms is missing. Thank you. Anritsu: It is appreciated if more detailed calculation procedures are provided since the proposed number didn’t match with ours. CATT: to Anritsu, DRX is changed from DRX.8 (320ms) to DRX. 3(40ms) in R4-2203574, according to DRX cycle ≤ 320ms; max(TReport, ceil(K *M*P)*max(TDRX,TSSB)). TSSB = 20ms. K=1, M=1 before R4-2203574, DRX = 320ms. So 1920ms=1600ms+320ms after R4-2203574, DRX = 40ms, so 640ms = 600ms+40ms Your proposal value is still 1920ms with no change when DRX is changed from 320ms to 40ms, is it? |
| R4-2210091 (MTK) | Draft CR 38.133 on DAPS handover test case |
|  | Nokia: OK |
| R4-2208202 (CATT) | Draft CR on R16 NR positioning test cases of general configurations and measurement delay requirements |
|  | Qualcomm: - Changes 1, 2 & 3: OK - Change 4: Why Es/Iot = -3dB was not added for FR2 @ Rx beam peak? - Merge with Ericsson's R4-2210184. [CATT] thanks for the comments, on change #4, it is missing accidently and can be added. Huawei: OK, but the last change needs to be merged with R4-2210184 Nokia: the CR is agreeable. |
| R4-2208204 (CATT) | Draft CR on R16 NR positioning test case of accuracy requirements |
|  | Qualcomm: - Change 1 & 4: For RSTD dual PFL test no muting is needed. In fact, even for the single PFL test we prefer not to have different muting patterns for the two cells. The reason is that the resources from the two TRPs will be farther apart in time and the accuracy requiremen will be more relaxed. Instead we can add a different PRS resource offset for each TRP. Note that in LTE RSTD accuracy test both cells transmit PRS at the same time. - Change 2: PRS config change OK. But similar to change 1, we prefer no muting. Instead, add a slot offset between TRPs. - Change 3 & 6: OK to add sub-test with smaller PRS BW. Prefer no muting. Instead, add a slot offset between TRPs. - Change 5: Test A.7.7.11 needs more changes. There are leftover references to T1 and T2. Relative RSRP accuracy requirements do not apply between PRS resources from two TRPs. [CATT] thanks for the comments, regarding muting, we are just trying to align the configuration for different test cases, if other companies are OK, we are also fine to use different slot offset or just configure at the same time, but need to align the configuration in different test cases. On change #5, the leftover reference to T1 and T2 can be updated in the revision. Regarding relative RSRP accuracy, since it only applies to the measurements within the same resource set, we may need a separate test or add a new PRS resource configuration in each cell in existing test to verify the relative accuracy. We would like to check companies’ view on the verification of relative RSRP accuracy. Nokia: the CR is agreeable. |
| R4-2208834 (vivo) | Draft CR to 38.133 correction to NR positioning accuracy requirements |
|  | E///: This is related to issue 2-2-4. We do not agree with the proposed changes. See our CR in R4-2210182. Huawei: pending on the outcome of 2-2-4, and needs to be merged with R4-2210182 Nokia: The CR depends on the outcome for issue 2-2-4. We do not agree with the changes. |
| R4-2209197 (HW) | CR on accuracy requirements for positioning measurement R16 |
|  | Qualcomm: Pending issues in sub-topic 2-2. vivo: depending on issue 2-2-4 Huawei: to QC and vivo, we can revise the CR based on outcome from 2-2. Nokia: We support the changes related to UE autonomous timing adjustments in clause 10.1.25.2, which is related to issue 2-2-4.The other changes depend on the outcome of issues 2-2-2 and 2-2-3. |
| R4-2210182 (E///) | Updates to accuracy requirements for UE positioning measurements in TS 38.133 |
|  | Qualcomm: - When there's autonomous UE timing adjustment, the CR says "UE shall restart." We support “UE may restart” instead. vivo: depending on issue 2-2-4 E///: We can support Option 2 in issue 2-2-4. So we are fine to revise the CR and change "UE shall restart” to “UE may restart” as suggested by QC. Huawei: pending on the outcome of 2-2-4, and needs to be merged with R4-2208834 Nokia: The CR is agreeable and depends on the outcome of issue 2-2-4. |
| R4-2210184 (E///) | Correction to conditions for NR PRS-based measurements in TS 38.133 |
|  | CATT: OK, overlapped with change #4 in our CR R4-2208202 Huawei: OK, but needs to be merged with R4-2208202 Nokia: The CR is agreeable. |
| R4-2208985 (HW) | R4-2208985 Update to UL switching test cases |
|  | MTK: We want to align the calculation for Io first. Because CSI-RS has density 3, only 3 out of 12 subcarriers in an RB will be power-boosted. In our calculation (as shown below), the Io is -50.51 dBm, which is not higher than -50dBm. Not sure if we made any mistake in the calculation. dB; linear. Power of SSB and data RE; -84dBm; 10^(-84/10). Power of CSI-RS with 6dB power boost; -78dBm; 10^(-78/10). Power of noise; -104dBm; 10^(-104/10). Power in one RB with 12 sub-carrier; 9*10^(-84/10) + 3*10^(-78/10) + 12*10^(-104/10). Power in 106 RBs; -50.51dBm; 106 * (9*10^(-84/10) + 3*10^(-78/10) + 12*10^(-104/10)). BTW, the Io in the test case actually varies with time, but we only keep a single value in the test case setup. Maybe this is one of the reasons causing the confusion? Huawei: thanks MTK for the comments. We will further check the comments and provide replies later. Huawei2: to MTK, from our understanding, in the CSI-RS symbol, not only the 3 subcarriers are power-boosted. Instead the whole symbol would be power-boosted due to the exsiting of OCNG. As a result the Io would be raised to higher than -50dBm. |
| R4-2207789 (Apple) | CR for Spatial relation info switch testcase maintenance (Rel-16) |
|  | Moderator: WI code in the cover sheet is wrong. MTK: OK Apple: WI code will be updated in the revision Nokia: The CR is not agreeable. title & WID need to be corrected. For the change in this CR, further discussion is needed. What’s the PL-RS assumptions? Apple3: The title will be updated to: CR for Spatial relation info switch testcase maintenance (Rel-16) and WI code to: NR_eRRM-Perf. @Nokia: The PUCCH Spatial relation info also includes PL-RS and that is missing from current spatial relation info configuration. We assume that the PL-RS is maintained. |
| R4-2207951 (QC) | draft Cat-F CR (R16) to SCell Activation Test Cases and SRS configuration |
|  | MTK: OK E///: - Change#1: Start position correction is fine but number of symbols may not be correct for SCS of 120kHz. SRS can be in last 6 symbols of the slot and there is always one symbol guard period between two SRS symbols if SCS is higher than 60kHz or if SRS is for Antenna port switching. That means for SCS of 120kHz, SRS can be on symbols number [9 11 13] or [10 12 14]. Considering that maximum SRS symbols in a slot are 3. May need to update that too. - Other changes are OK. Apple: fine with CR R&S: Conflicts with R4-2208918 Nokia: The CR is not agreeable. For change#1 on SCell activation same comments as core part. Fine with change#2 |
| R4-2208918 (HW) | Correction to SRS reference configuration_r16 |
|  | Qualcomm - Overlapped with R4-2207951, and we prefer 7951 since the original intended start position is the first symbol among the last 6 symbols. We should align to the original intended configuration when updating it according to the correct RAN2 IE definition Huawei: we can merge this CR to R4-2207951 and further discuss the SRS configuration there. R&S: Conflicts with R4-2207951 Nokia: we have concern on the proposal of startPosition in SRS configuration, we think it should be 5 for the startposition. The change is overlapping with change#2 in R4-2207951 from QC. |
| R4-2208164 (CATT) | Draft CR on test case for cell reselection for power saving |
|  | Qualcomm: - What is 7.5dB margin referring to? Power saving is coming from the measurement delay extension, not additional margin in measurement accuracy. vivo: Regarding UE gain G agreements from RAN4 99 are copied below. - Agreements: - Follow the release 15 approach in defining the FR2 inter-frequency test cases. To our understanding the same test configuration as Rel-15 should be used. CATT: to Qualcomm and vivo, thank you for comments. It is not related to UE gain G. the 7.5 margin is in 6dB margin for FR1 and 7.5dB margin for FR2 in Ch.4.2.2.4. It follows the same approach as that in legacy R15 test in A.7.1.1.2 changed from v15.11.0 to v15.12.0. The test doesn’t meet the 7.5dB margin compared to thresholds in R15 test before v15.12.0. After legacy R15 test fixed the issue, the same fixing should be applied to R16 power saving test case as well. |
| R4-2207953 (QC) | draft Cat-F CR (R16) to SCell Activation Test Cases NR-U |
|  | MTK: OK, related to R4-2207946 E///: depends on outcome of R4-2207946. Nokia: The CR is not agreeable. Same comments as core part. |
| R4-2209076 (E///) | draft CR: Correction of NR-U RRM test cases |
|  | MTK: OK Nokia: Ok |
| R4-2207731 (QC) | SRS configuration correction |
|  | Moderator: same change as in R4-2207951. The CR was originally submitted to AI 9.9.2.1. |
| R4-2208734 (ZTE) | [dCR] Maintenance for IAB-MT test cases R16 |
|  | Nokia: OK. |
| R4-2210225 (QC) | draft CR (R16) CR for UE Rx-Tx accuracy |
|  | vivo: In general, it is good approach. We support. Huawei: the motivation of the CR is not very clear to us. We would prefer to only define the final value for the accuracy by adding the BB error and the calibration margin together, so instead of capturing “30+” or “30+56”, we prefer to simply capture “86” in the spec. With the change suggested by the CR, RAN4 is implicitly defining separate requirements for BB error and calibration error. We think this is not a good approach because it is sufficient for UE to meet the final accuracy, e.g. UE can have BB error than 30Tc with calibration error smaller than 56Tc, or the other way around, as long as the total error is smaller than 86Tc. Nokia: The CR is agreeable and depends on outcome of issues 2-2-2 and 2-2-3. |

Summary for 1st round

Open issues

Sub-topic 2-1: eMIMO

- Issue 2-1-1: FR2 PL-RS switching delay when the target PL-RS is SSB and used for L1-RSRP measurements Tentative agreements: None Candidate options: - Option 1 (HW, MTK) - To clarify that longer PL-RS switching delay is expected, which can be captured in the note. - Option 2 (HW) - To define the PL-RS switching delay as 5*TL1-RSRP_SSB, where TL1-RSRP_SSB is SSB based L1-RSRP measurement period with the assumption of M=1. - Option2a (Apple) - To define the PL-RS switching delay as Tfirst_SSB + 4*TL1-RSRP_SSB, where Tfirst_SSB is the time to first SSB after MAC-CE decoding, and TL1-RSRP_SSB is SSB based L1-RSRP measurement period with the assumption of M=1. - The delay requirements apply when the target PL-RS is SSB, i.e. not necessarily SSB used for L1-RSRP measurement. - Option 3 (vivo, Nokia) - No spec change for this scenario, or FFS Recommendations for 2nd round: Further discuss the options. There seems to be 2 issues: - Whether the concerned scenario is where target PL-RS is SSB, or where target PL-RS is SSB and used for L1-RSRP measurements - Whether and how to clarify the PL-RS switching delay in the concerned scenario.

Sub-topic 2-2: Positioning

- Issue 2-2-1: Start of measurement period Tentative agreements: None Candidate options: - Open issue - Start point of PRS measurement period for deferred MT-LR - Option 1 (HW, E///, CATT, Nokia) - Update the start point of PRS measurement period to account for the deferred MT-LR. - Measurement period starts from the first MG instance aligned with PRS resource(s) after the associated event(s) defined in 23.273 occur(s). - Option 2 (QC) - For scheduled LR and deferred LR with periodic reporting: the UE shall start the measurements no later than T_start = T – T_meas_period – max(T_available_PRS) - For deferred LR with events other than periodic location: the UE shall start the measurements no later than T_start = T + max(T_available_PRS) - Option 3 (vivo) - Up to UE implementation Recommendations for 2nd round: Further discuss the options.

- Issue 2-2-2: group delay calibration margin for RSTD Tentative agreements: - Calibration margin for RSTD FR1 single PFL case PRS BW (MHz); Margin (Tc). ≥ 5; [120]. ≥ 10; [72]. - Calibration margin for RSTD FR2 single PFL case PRS BW (MHz); Margin (Tc). ≥ 20; [72]. Candidate options: None Recommendations for 2nd round: Further check if the tentative agreements can be agreed.

- Issue 2-2-3: group delay calibration margin for UE Rx-Tx Tentative agreements: - Calibration margin for UE Rx-Tx FR1 Min{PRS BW, SRS BW} (MHz); Margin (Tc). ≥ 5; [160]. Candidate options: None Recommendations for 2nd round: Further check if the tentative agreements can be agreed.

- Issue 2-2-4: Applicability of UE Rx-Tx accuracy requirements in case of UE autonomous timing adjustment Tentative agreements: None Candidate options: - Option 2 (QC, HW, E///, Nokia) - UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) - UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. UE may restart the measurement period in such case - Option 3 (vivo, CATT) - UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. - FFS how to define test to verify the accuracy requirements, or not to define test to verify accuracy requirements on non-reference cell. Recommendations for 2nd round: Further discuss the options.

- Issue 2-2-5: Test for UE based DL-TDOA positioning Tentative agreements: - Do not define any DL-TDOA reporting delay tests for UE-based DL-TDOA. - Do not define any DL-TDOA accuracy tests for UE-based DL-TDOA. - Do not modify any existing PRS measurement delay or accuracy test to include any aspect related to UE-based DL-TDOA. Candidate options: None Recommendations for 2nd round: Closed, no discussion needed.

- Issue 2-2-6: PRS repetition number for small BW in accuracy requirements Tentative agreements: Update the PRS repetition number to 4 for the smallest BW in RSTD and UE Rx-Tx accuracy requirements for FR2 AWGN. Candidate options: None Recommendations for 2nd round: Closed, no discussion needed.

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #3: Maintenance for LTE and A-GNSS

Companies’ contributions summary

| T-doc | Company | Proposals / Observations |
| --- | --- | --- |
| Core part |  |  |
| R4-2208956 | Huawei, Hisilicon | CR for R16 LTE DAPS 36.133 1. Correct note 2, 3 in Table 5.7.1-1. 2. Some editorial changes. The same issue was also solved in NR DAPS. It was endorsed in [R4-2113814]. |
| R4-2208933 | Huawei, Hisilicon | CR for R16 NB-IoT 36.133 Add supporting NR bands in bands groups for NB-IoT |
| R4-2208198 | CATT, CAICT, CENC | CR for R16 36.171 1. Correct the BDS ICD reference for B1I signal in TS 36.171. 2. Table C.2: Ref. BDS ICD, update the value in the BDS entry as: Signal = B1C, X [m] = 0.5 * c / 1.023 Mcps = 150 m. 3. Table C.3: Ref. BDS ICD, update the value in BDS entry in this table as: Signal = B1C, Ratio N = 1575.420 MHz (Carrier Freq) / 1.023 Mcps (Chipping rate) = 1540. |
| R4-2208200 | CATT, CAICT, CENC | CR for R16 38.171 1. Correct the BDS ICD reference in TS 38.171. 2. Correct the applicable clause in TS 38.171 section B.1.13. 3. Table C.2: Ref. BDS ICD, update the value in the BDS entry as: Signal = B1C, X [m] = 0.5 * c / 1.023 Mcps = 150 m. 4. Table C.3: Ref. BDS ICD, update the value in BDS entry in this table as: Signal = B1C, Ratio N = 1575.420 MHz (Carrier Freq) / 1.023 Mcps (Chipping rate) = 1540. |

Open issues summary

No open issue, please provide comments to the CR directly in section 3.3.

Comments to the CRs

Cat-A draftCRs are not listed for comments.

| R4-2208956 (HW) | Clarification on asynchronous DAPS handover R16 |
| --- | --- |
|  | Qualcomm: OK. E///: We prefer to state explicitly that when the 20 µs condition is not met, then the UE may perform autonomous interruption. This is, for example outlined in draft CR R4-2205415 RAN4#102-e. Table 5.7.1-2: Autonomous interruptions related to DL to UL switching for syncrounous TDD DAPS handover in the same band Scenario; Allowed interruption. Target cell earlier than source cellNote 1, prior to start of random access; Not applicable. Target cell later than source cell Note 1, prior to start of random access; The UE may stop receiving the target DL up to 20µS prior to the start of the source UL. Target cell earlier than source cell Note 1, after start of random access; The UE may stop receiving the source DL up to 20µS prior to the start of the target UL. Target cell later than source cell Note 1, after start of random access; The UE may start transmitting the source UL up to 20µS after the start of the source DL. Note 1: As observed by UE at antenna connector. Table 5.7.1-3: Autonomous interruptions related to UL to DL switching for syncrounous TDD DAPS handover in the same band Scenario; Allowed interruption. Target cell earlier than source cell Note 1, prior to start of random access; The UE may start receiving the target DL up to 20µS after the end of the source UL. Target cell later than source cell Note 1 prior to start of random access; Not applicable. Target cell earlier than source cell Note 1, after start of random access; The UE may stop transmissions of the source UL up to 20µS prior to the start of target DL reception.. Target cell later than source cell Note 1, after start of random access; The UE may start receiving the source DL up to 20µS after the end of the target UL. Note 1: As observed by UE at antenna connector. Huawei: thanks QC and E/// for the comments. To E///, RAN4 has agreed to follow the agreement on NR DAPS handover for LTE DAPS handover (see R4-2115271). Hence, the solution captured in TS38.133 for NR DAPS HO also needs to be captured into TS36.133 for LTE DAPS HO. The original version in Huawei's CR is aligned with the agreement on NR DAPS handover. Nokia: ‘after starting RACH’ somehow contradict the ‘not required to transmit in the uplink to any of source and target cells’. we assume that the intention is that after RACH has been triggered towards the target cell there is a 20us time for UE to switch? |
| R4-2208933 (HW) | Draft CR on adding NR bands groups for NB-IoT R16 |
|  | Nokia: The CR is agreeable. |
| R4-2208198 (CATT, CAICT, CENC) | Draft CR on TS 36.171 requirements for support of A-GNSS |
|  | Qualcomm: - BDS B1C signal has similar characteristics as GPS L1C and Galilei E1. Therefore, in Table C.2 the value for X should be 125 and for Y -4.5. Otherwise OK. [CATT] thanks for the comments, we propose this value based on X [m] = 0.5 * c / 1.023 Mcps = 150 m which follows the calculation principle when initially introducing BDS signal. But we are not sure how this value X for GPS L1C is derived. Could you indicate how to derive the value for GPS L1C and why the value can be reused for BDS B1C? Nokia: The CR is agreeable. |
| R4-2208200 (CATT, CAICT, CENC) | Draft CR on TS 38.171 requirements for support of A-GNSS |
|  | Qualcomm: - BDS B1C signal has similar characteristics as GPS L1C and Galilei E1. Therefore, in Table C.2 the value for X should be 125 and for Y -4.5. Otherwise OK. Nokia: The CR is agreeable. |

Summary for 1st round

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on R15 and R16 RRM maintenance | Huawei, Hisilicon | WF to capture agreements for open issues |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2207787 |  | CR for eMIMO requirements maintenance (Rel-16) | Apple | Agreeable |  |
| R4-2207875 |  | Maintenance CR for RRM requirements on 38.133 R16 | MediaTek (Shenzhen) Inc. | Return to |  |
| R4-2207877 |  | Maintenance CR for V2X Te requirements on 38.133 R16 | MediaTek (Shenzhen) Inc. | Agreeable |  |
| R4-2207941 |  | draft Cat-F CR (R15) to SCell Activation Core | Qualcomm Incorporated | Revised |  |
| R4-2207944 |  | draft Cat-F CR (R16) to SCell Activation Core | Qualcomm Incorporated | Revised |  |
| R4-2207946 |  | draft Cat-F CR (R16) to SCell Activation Core NR-U | Qualcomm Incorporated | Revised |  |
| R4-2208829 |  | Draft CR to TS 38.133 Correction to Rel-16 IDLE CA&DC measurements requirements | vivo | Agreeable |  |
| R4-2208830 |  | Draft CR to TS 38.133 Correction to Rel-17 IDLE CA&DC measurements requirements | vivo | Agreeable |  |
| R4-2208831 |  | Draft CR to TS 36.133 Correction to IDLE DC measurements requirements_R16 | vivo | Agreeable |  |
| R4-2208836 |  | Draft CR to 38.133 correction to NR positioning measurement requirements | vivo | Revised |  |
| R4-2208848 |  | CR to TS38.133 for the editorial correction on L1-SINR scheduling restriction | Samsung | Agreeable |  |
| R4-2208909 |  | Correction to NR SCell activation interruption requirements 38133_r15 | Huawei, Hisilicon | Revised |  |
| R4-2208912 |  | Correction to NR SCell activation interruption requirements 36133_r15 | Huawei, Hisilicon | Revised |  |
| R4-2208915 |  | Correction to paging interruption during reselection requirements_r15 | Huawei, Hisilicon | Return to |  |
| R4-2208916 |  | Correction to paging interruption during reselection requirements_r16 | Huawei, Hisilicon | Revised |  |
| R4-2208922 |  | Correction to HST inter-RAT NR cell reselection requirements_r16 | Huawei, Hisilicon | Revised |  |
| R4-2208924 |  | Correction to HST intra-NR cell-reselection requirements_r16 | Huawei, Hisilicon | Revised |  |
| R4-2208927 |  | Draft CR on requirements maintenance for NR-U 36133 R16 | Huawei, Hisilicon | Return to | Missed from first round discussion |
| R4-2208929 |  | Draft CR on requirements maintenance for NR-U 38133 R16 | Huawei, Hisilicon | Agreeable |  |
| R4-2208931 |  | Draft CR on maintenance for IAB R16 | Huawei, Hisilicon | Agreeable |  |
| R4-2208933 |  | Draft CR on adding NR bands groups for NB-IoT R16 | Huawei, Hisilicon | Agreeable |  |
| R4-2208956 |  | Clarification on asynchronous DAPS handover R16 | Huawei, Hisilicon | Return to |  |
| R4-2208988 |  | DraftCR on maintaining PL-RS switching delay requirements R16 | Huawei, Hisilicon | Revised |  |
| R4-2209186 |  | CR on CSSF outside MG R15 | Huawei, Hisilicon | Return to |  |
| R4-2209189 |  | CR on beam level EMR requirements 36133 R16 | Huawei, Hisilicon | Revised |  |
| R4-2209191 |  | CR on beam level EMR requirements 38133 R16 | Huawei, Hisilicon | Revised |  |
| R4-2209194 |  | CR on PRS meausurement period R16 | Huawei, Hisilicon | Revised |  |
| R4-2209199 |  | CR to multiple SCell activation requirements R16 | Huawei, Hisilicon | Revised |  |
| R4-2209201 |  | CR on CGI reading requirements R16 | Huawei, Hisilicon | Agreeable |  |

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2207646 |  | Draft CR to FR1 DCI-based BWP switch TCs | Anritsu Corporation | Return to |  |
| R4-2207648 |  | Draft CR to FR1 DCI-based BWP switch TCs and FR2 CSI-RS based RLM | Anritsu Corporation | Return to |  |
| R4-2207649 |  | Draft CR to Cell reselection to FR1 intra-frequency NR case | Anritsu Corporation | Agreeable |  |
| R4-2207748 |  | draftCR on applicabiltiy for test Cases involving E-UTRA/FR1 and FR2 carriers (R15) | Apple | Revised |  |
| R4-2207789 |  | CR for Spatial relation info switch testcase maintenance (Rel-16) | Apple | Revised |  |
| R4-2207948 |  | draft Cat-F CR (R15) to SCell Activation Test Cases | Qualcomm Incorporated | Return to |  |
| R4-2207951 |  | draft Cat-F CR (R16) to SCell Activation Test Cases and SRS configuration | Qualcomm Incorporated | Revised |  |
| R4-2207953 |  | draft Cat-F CR (R16) to SCell Activation Test Cases NR-U | Qualcomm Incorporated | Return to |  |
| R4-2208162 |  | Draft CR on HST FR1 L1-RSRP test cases | CATT | Revised |  |
| R4-2208164 |  | Draft CR on test case for cell reselection for power saving | CATT | Return to |  |
| R4-2208166 |  | Draft CR to add missing SMTC pattern | CATT | Agreeable |  |
| R4-2208167 |  | Draft CR on radio link monitoring test cases in FR1 | CATT | Return to |  |
| R4-2208202 |  | Draft CR on R16 NR positioning test cases of general configurations and measurement delay requirements | CATT | Revised |  |
| R4-2208204 |  | Draft CR on R16 NR positioning test case of accuracy requirements | CATT | Revised |  |
| R4-2208341 |  | CR to maintain test case of PScell addition and release delay (A4.5.7)_R15 | OPPO | Revised | Technically agreeable, revised to correct cover sheet |
| R4-2208344 |  | CR to maintain inter-RAT measurements subject to CCA in TS 36.133(R16) | OPPO | Return to |  |
| R4-2208834 |  | Draft CR to 38.133 correction to NR positioning accuracy requirements | vivo | Merged | Merged to R4-2210182 |
| R4-2208906 |  | Correction to cell reselection test case_r15 | Huawei, Hisilicon | Agreeable |  |
| R4-2208918 |  | Correction to SRS reference configuration_r16 | Huawei, Hisilicon | Merged | Merged to R4-2207951 |
| R4-2208920 |  | Correction to eMIMO BFD test cases_r16 | Huawei, Hisilicon | Revised |  |
| R4-2208985 |  | Update to UL switching test cases | Huawei, Hisilicon | Revised |  |
| R4-2208990 |  | DraftCR on maintaining L1-SINR measurement test cases R16 | Huawei, Hisilicon | Revised |  |
| R4-2209076 |  | draft CR: Correction of NR-U RRM test cases | Ericsson | Agreeable |  |
| R4-2209078 |  | draft CR: Correction of PRACH configuration parameter for inter-RAT test | Ericsson | Agreeable |  |
| R4-2209197 |  | CR on accuracy requirements for positioning measurement R16 | Huawei, Hisilicon | Revised | Capture all changes for RSTD accuracy |
| R4-2209609 |  | Draft CR to TS 38.133: Corrections to intra-frequency event triggered test cases (Rel 15) | Rohde & Schwarz | Agreeable |  |
| R4-2209612 |  | Draft CR to TS 38.133: Corrections to beam failure and link recovery test cases (Rel 15) | Rohde & Schwarz | Agreeable |  |
| R4-2210091 |  | Draft CR 38.133 on DAPS handover test case | MediaTek Inc. | Agreeable |  |
| R4-2210182 |  | Updates to accuracy requirements for UE positioning measurements in TS 38.133 | Ericsson | Revised | Capture all changed for UE Rx-Tx accuracy |
| R4-2210184 |  | Correction to conditions for NR PRS-based measurements in TS 38.133 | Ericsson | Merged | Merged to R4-2208202 |
| R4-2210225 |  | DraftCR - Correction of margins for UE Rx-Tx accuracy requirements | Qualcomm Incorporated | Merged | Merged to R4-2210182 |
| R4-2207731 |  | SRS configuration correction | Qualcomm Incorporated | Not Pursued | Same change is proposed in R4-2207951 |
| R4-2208734 |  | [dCR] Maintenance for IAB-MT test cases R16 | ZTE Corporation | Agreeable |  |
| R4-2208198 |  | Draft CR on TS 36.171 requirements for support of A-GNSS | CATT, CAICT, CENC | Return to |  |
| R4-2208200 |  | Draft CR on TS 38.171 requirements for support of A-GNSS | CATT, CAICT, CENC | Return to |  |

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei | Li Zhang | zhangli164@huawei.com |
| Qualcomm | CH Park | chparkqc@qti.qualcomm.com |
| MediaTek | Hsuanli Lin | Hsuanli.Lin@mediatek |
| Apple | Jie Cui | Jie_cui@apple.com |
| CATT | Qiuge Guo | guoqiuge@catt.cn |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)