3GPP TSG RAN WG4 Meeting #103-e	                                                                  R4-2210282

Electronic Meeting, May 9-20, 2022

Agenda item:			9.10.1.1, 9.10.2.1

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [103-e][210] NR_MG_Part_2

Document for:	Information

# Introduction

The scope of this email discussion is UE RRM requirements for NR positioning from the following agenda items:

- AI 9.10.1.1, 9.10.2.1 Pre-configured MG pattern

In providing comments, companies are encouraged to:

- Be concise

- Provide comments on all topics/sub-topics of interest

- Ensure that comments are inserted in the latest version of the document by checking the folder before uploading

- Use “Track changes” to help identify added comments/changes

# Topic #1: Pre-configured MG pattern(s) core requirements (AI9.10.1.1)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2208030 | Qualcomm | Proposal 1: When the network has provided a pre-configured gap and the UE notifies the network via LocationMeasurementIndication that it needs an always-ON gap to perform NR positioning measurements, the UE will determine that the pre-configured MG is always-ON only if the network updates the status of the pre-configured MG accordingly via RRC. Observation 1: RAN4 needs to clarify whether simultaneous BWP switches on multiple CCs are precluded (restriction on the network) when pre-configured MG are provided to the UE. Proposal 2: Simultaneous BWP switches on multiple CCs are precluded (restriction on the network) when pre-configured MG are provided to the UE. |
| R4-2209203 | MediaTek inc. | Observation 1: The current Pre-MG activaton/deactivation delay depends on the SCell activation delay requirement, not on whether the SCell has been successfully activated. Observation 2: The Pre-MG activation delay needs to consider latest SCell being activated by UE in the case of multiple SCell activation. Proposal 1: Modify the requirement on the Pre-MG activation/deactivation delay trigged by SCell activation/deactivation as: UE shall be able to finish pre-configured activation or deactivation within [5] ms after the report of the latest first valid CQI among all SCell(s) being activated. |
| R4-2208295 | MediaTek inc. | CR |
| R4-2208354 | OPPO | Proposal 1: To determine the state of pre-configured MG when switching to initial DL BWP: - option 1: signaling-based activation mechanism cannot be used for initial DL BWP - option 2: add ON/OFF indication for initial DL BWP when designing signaling in RAN2 |
| R4-2208355 | OPPO | CR to maintain pre-configured measurement gap in TS 38.133 |
| R4-2208459 | Apple | draftCR |
| R4-2208772 | ZTE | Proposal 1: Based on the approved trigger events, the restriction on CA case can be enlarged so as to prohibit the existence of simultaneous multiple trigger events happening across all serving cells. Otherwise, we need to identify the start point of activation/deactivation delay and when the activation/deactivation take effects, not only for the trigger event of BWP switching by DCI/timer, but also for other trigger events. Proposal 2: RAN4 can conclude that pre-MG for MR-DC/EN-DC/NE-DC is not supported in Rel-17. |
| R4-2209203 | Huawei, HiSilicon | Proposal 1: Remove initiation of LocationMeasurementIndication as a trigger event for rule based pre-MG activation/deactivation, and remove PRS measurement as a criterion to activate pre-MG. Proposal 2: Define delay for activation/deactivation after pre-MG configuration as [5]ms after the RRC processing time. |
| R4-2209204 | Huawei | CR |
| R4-2210231 | Nokia, Nokia Shanghai Bell | Specify the proposed changes in section 2 in clause 9.1.7.3.1. “For Pre-MG deactivation: The UE shall autonomously determine the status of the per-UE Pre-MG pattern as activated immediately after the configuration of the per-UE Pre-MG pattern provided that at least one of the configured measurements cannot be performed without measurement gaps. In case of concurrent trigger events, the UE shall determine the status of the per-UE Pre-MG pattern, taking into account all trigger events. The UE shall determine the status of the per-UE Pre-MG pattern based on every new received triggering condition. In case of concurrent triggering conditions, the UE shall determine the status of the per-UE Pre-MG pattern based on all these triggering conditions. The UE shall autonomously determine the status of the per-FR Pre-MG pattern as activated immediately after the configuration of the per-FR Pre-MG pattern provided that at least one of the configured measurements in the same FR cannot be performed without measurement gaps. The UE shall determine the status of the per-FR Pre-MG pattern based on every new received triggering condition. In case of concurrent triggering conditions, the UE shall determine the status of the per-FR Pre-MG pattern based on all these triggering conditions. For Pre-MG activation: The UE shall autonomously determine the status of the per-UE Pre-MG pattern as activated immediately after the configuration of the per-UE Pre-MG pattern provided that at least one of the configured measurements cannot be performed without measurement gaps. The UE shall determine the status of the per-UE Pre-MG pattern based on every new received triggering condition. In case of concurrent triggering conditions, the UE shall determine the status of the per-UE Pre-MG pattern based on all these triggering conditions. The UE shall autonomously determine the status of the per-FR Pre-MG pattern as activated immediately after the configuration of the per-FR Pre-MG pattern provided that at least one of the configured measurements in the same FR cannot be performed without measurement gaps. The UE shall determine the status of the per-FR Pre-MG pattern based on every new received triggering condition. In case of concurrent triggering conditions, the UE shall determine the status of the per-FR Pre-MG pattern based on all these triggering conditions. “ |
| R4-2210167 | Ericsson | CR |

## Open issues summary and companies views’ collection for 1st round

### Sub-topic 1-1 Pre-MG activation/deactivation

#### Issue 1-1-1 Additional trigger events for pre-MG activation/deactivation

[Moderator notes: In R4#102-e the following agreements on Pre-MG used for PRS measurement achieved:

- RAN4 considers that a Pre-configured MG that is not always ON (activated) as determined from the signalling provided by the network, or if no such signalling is provided (i.e. autonomous rules are applied to determine the status of the Pre-configured MG), is not sufficient to perform PRS measurements.

- In the above scenario, the UE will inform the network that it is going to start/stop PRS measurements with the configured Pre-configured MG by initiating the existing LocationMeasurementIndication procedure.

In this meeting, some clarification on this issue was proposed.]

- Option 1(Huawei): Remove initiation of LocationMeasurementIndication as a trigger event for rule based pre-MG activation/deactivation, and remove PRS measurement as a criterion to activate pre-MG.

- Option 2(Qualcomm): When the network has provided a pre-configured gap and the UE notifies the network via LocationMeasurementIndication that it needs an always-ON gap to perform NR positioning measurements, the UE will determine that the pre-configured MG is always-ON only if the network updates the status of the pre-configured MG accordingly via RRC.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Support Option 1. These two options can resolve the problem when there is misunderstanding between UE and gNB on the pre-MG status if pre-MG status is to be changed being tiggered by LocationMeasurementIndication. However, according to Option 2, the network need to update the status of pre-MG. But in the last meeting discussion, RAN4 would not intend to ask RAN2 update the necessary signaling. That is the network behavior is completely up to themselves. So Option 2 may revert this conclusion and more standardization works from RAN2 is required. Regarding to core part was completed, we prefer Option 1 by which the uncertain issue can be removed by RAN4. The other reason to support to remove PRS trigger events is in Rel17 the other similar mechanism to enable the pre-configured MG for positioning was separately discussed in ePos WI indeed. |
| Nokia | We support option 2. PRS measurement was agreed to be supported using pre-MG and was liaised to RAN2. Pre-MG patterns including #24 and #25 were agreed. In our view, the confirmation of the pre-MG activation status (always ON) by the network is needed after UE sends the LocationMeasurementIndication. In such case, an RRC reconfiguration, configuring a legacy MG, can be avoided. |
| E/// | We support to keep LocationMeasurementIndication as a trigger event for rule based pre-MG activation/deactivation. PRS measurement as a criterion to activate pre-MG should be kept. Otherwise PRS measurements cannot be done if the pre-configured gaps are already configured. When LocationMeasurementIndication is sent by the UE then both UE and gNB should assume the pre-MG is ON (activated). We are fine to do necessary clarification. We prefer not to introduce the new signaling from gNB to UE. |
| Huawei | We understand the two options are not conflicting with each other. Option 1 is our proposal, and we support it. It addresses the rule based pre-MG activation/deactivation. The reason behind is that NW may have different re-actions to the LocationMeasurementIndication which is up to NW implementation, while the current spec implicitly requires that NW must activate the pre-MG, which limits the NW flexibility. On option 2, if the contexts is for RRC based pre-MG activation/deactivation, we can also support it, since it is proposed for the same reason as option 1. For RRC based pre-MG activation/deactivation, NW can either configure a legacy MG or update the status of pre-MG such that it is always ON. |
| Qualcomm | We support option 2. We have a similar view as Huawei that the UE cannot assume that the gap status is changed after sending LocationMeasurementIndication unless it receives some explicit response/action from the network. However, the UE should be allowed to send LocationMeasurementIndication regardless of whether the autonomous rules or RRC-based signalling is used to determine the status of the pre-configured MG. It is up to the network what to do in response. We would like to confirm that option1 does not preclude the UE initiating LocationMeasurementIndication when autonomous rules are used. |
| ZTE | It is really hard to resolve the misunderstanding issue between UE and gNB on the activated/deactivated status when triggered by PRS request under UE autonomous mechanism. So Option 1 can be a choice, but which will override the approved trigger events in 102 meeting. Compared with Option 2, UE determines the pre-configued gap status when it receives the RLC acknowledgement to LocationMeasurementIndication message, we think which is better than Option 2. But since the RLC acknowledge is not the response to the content of LocationMeasurementIndication message, it is just an acknowledge of receiving message, so no matter original Option 2 or such modified Option 2’, both can not guarantee consistent understanding for the PRS measurement request between UE and gNB. So if we can not find better option, we have to apply Option 1. |
| Apple | In our view option 1 and 2 are not mutual exclusive. Option 1 is for UE autonomous activation/deactivation while option 2 is for network-controlled activation/deactivation. Option 2 is fine for us. Option 1 is also fine in general. However, we would like to clarify that even if option 1 is agreed (UE won’t automatically change Pre-MG status), UE shall still inform NW about PRS measurement using LocationMeasurementIndication. |
| CATT | We prefer to keep the agreements in last meeting including LocationMeasurementIndication as a trigger event. But if there are technical issues to implement this event and all companies agree, we are also fine to remove it since RAN1 has also defined Pre-MG for PRS measurement in R17 which can take the same effect. For option 2, we can understand the intention, but we have concern on this suggestion. We prefer not to introduce the new RRC signaling for gNB confirmation. In our understanding, for UE autonomous activation, when UE report the LocationMeasurementIndication, UE and gNB will have the same understanding to activate the Pre-MG. And even if Pre-MG is configured, NW has the flexibility to reconfigure the legacy gap when there is gap-based measurement. If additional RRC signaling is introduced for gNB to update the Pre-MG status as suggested in option 2, then what is the difference with gap reconfiguration? |
| OPPO | Prefer option 2. UE should be allowed to send LocationMeasurementIndication regardless of autonomous rules or RRC-based signalling. Option 2 provides a possible solution, since Pre-MG is agreed to be used for PRS measurement. |
| MTK | Support Option 1. In addition to the analysis above provided by companies. We see some additional problem to support Pre-MG with positioning measurement. - The Pre-MG activation/deactivation delay is still missing. As mentioned by some companies, UE cannot naively assume Pre-MG status change after sending LocationMeasurementIndication. It needs to check the network response. And depending on the network response, RAN4 needs to define UE’s behaviors correspondingly. As this WE is already in the maintenance phase, we do not think some new network or UE behavior/procedure can still be introduced now. - UE only indicates LocationMeasurementIndication when Pre-MG is sufficient for PRS measurement. This means UE will not inform network about the start of PRS measurement if the Pre-MG is sufficient. Since network has no idea when the PRS measurement starts, network may trigger BWP switch to UE, causing Pre-MG status change. When this happens, network may think the gap should be OFF, but UE will keep the gap ON, leading to problem to the link. Therefore, we suggest removing PRS m initiation of LocationMeasurementIndication as a trigger event. |
| Moderator | Upon the received comments above, firstly for the RRC signaling based pre-MG activation/deactivation, Option 2 is correct because NW will indicate UE whether pre-MC is (de)activated by updated RRC singaling for pre-MG status or the legacy MG configured by RRC reconfiguration message. . However, for the autonomous pre-MG activation/deactivation mechanism, as many companies mentioned, one of important concerns for option 2 whether the additional new signaling to indicate the severing gNB’s decision on pre-MG activation/deactivation or fall back to the legacy MG to UE needed. Therefore, there are also have different alternatives below. - Option 2-1: When the network has provided a pre-configured gap and the UE notifies the network via LocationMeasurementIndication that it needs an always-ON gap to perform NR positioning measurements, the UE will determine that the pre-configured MG is always-ON only if the network updates the status of the pre-configured MG accordingly via RRC That is with this option 2-1, the new RRC signaling needed to update pre-MG activation status in case of autonomous pre-MG activation UE. This is completely contradicted with RAN4’s purpose to introduce autonomous pre-MG activation mechanism. Moreover, this is very significant changes on RAN2’s spec. Currently in RAN2 spec there is no any signaling needed when NW granted UE required measurement gap for positioning. As the core part was completed in the last meeting, it is infeasible to ask RAN2 to implement such signaling. “TS38.133 v16.5.0 Figure 5.5.5.1-1: Location measurement indication The purpose of this procedure is to indicate to the network that the UE is going to start/stop location related measurements towards E-UTRA or NR (eutra-RSTD, nr-RSTD, nr-UE-RxTxTimeDiff, nr-PRS-RSRP) which require measurement gaps or start/stop detection of subframe and slot timing towards E-UTRA (eutra-FineTimingDetection) which requires measurement gaps. UE shall initiate this procedure only after successful AS security activation. NOTE: It is a network decision to configure the measurement gap.” - Option 2-2: When the network has provided a pre-configured gap and the UE notifies the network via LocationMeasurementIndication that it needs an always-ON gap to perform NR positioning measurements, the UE will determine that the pre-configured MG is always-ON only if the network did NOT configure any legacy MG by the existing RRCReconfiguation IE. That is for Option 2-2, UE can implicitly know whether the pre-MG or legacy MG is used after it indicated its PRS measurement to NW by “LocationMeasurementIndication”. |

### Sub-topic 1-2 Pre-MG in CA mode

#### Issue 1-2-1 Clarifications on simultaneous BWP switches on multiple

- Option 1(Qualcomm): Simultaneous BWP switches on multiple CCs were precluded in Rel17 when pre-configured MG are provided to the UE.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Option 1 is fine for us which is also the common understanding of RAN4. “Using of pre-configured MG for CA - In this meeting, RAN4 continue focus on the single carrier case and evaluate the additional working efforts to support the CA case with pre-configured MG activation/deactivation based on BWP switching on a single CC “ |
| vivo | Ok with option 1 based on the previous common understanding/objectives of RAN4 although we believe this could be supported with reasonable working effort. |
| Nokia | We support option 1. |
| E/// | Support Option 1. It is also our understanding that the simultaneous BWP switches on multiple CCs is precluded in R17. |
| Huawei | Support option 1 from RAN4 requirement perspective, and we think it is already reflected by the following statement in clause 8.19.2.1: The requirements in this clause only apply to the case that the DCI/timer-based BWP switch is performed on a single CC with more than one BWP configurations configured on the CC. For the agreement the proponent is concerned with, we understand it is for the case where UE is configured with CA and BWP switch occurs on a single CC, and in this case the pre-MG activation/deactivation delay should be same as the case where UE is configured with single CC. |
| Qualcomm | Option 1. |
| ZTE | Fine with Option 1. It is aligned with previous common understanding. |
| Apple | Fine with option 1. |
| CATT | Fine with option 1 which was already agreed in previous meeting. |
| OPPO | Option 1 is fine. |
| MTK | We understand the intention of Option 1. But since this is UE requirement, it is better to say that “No requirements apply to simultaneous BWP switches on multiple CCs in Rel17 when pre-configured MG are provided to the UE”. This means UE may take a longer time to finish the BWP switch on multiple CCs, rather than rejecting the network request. |
| Moderator | Option 1 can be agreed with the clarification of MTK |

#### Issue 1-2-2 Whether need indicate the state of pre-configured MG when switching to initial DL BWP

- Option 1(OPPO): yes

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | The indication of pre-MG status per BWP shall include all for the candidates BWPs of UE which also included that for the initial BWP. |
| Vivo | Same view as Intel |
| Nokia | We support option 1, i.e. adding ON/OFF indication for switching to initial DL BWP for signalling-based Pre-MG (de-)activation. And we share Intel’s view. |
| E/// | Agree with Intel and no further clarification is needed. |
| Huawei | We think the issue is valid, but based on the current 38331, BWP-DownlinkDedicated can be also configured for initial DL BWP in ServingCellConfig, so we are not sure if any signaling change in RAN2 is needed. |
| ZTE | For CONNECTED UE, deactivatedMeasGapList can be configured for initial DL BWP. So there is no difference for the status indication between initial DL BWP and other BWP. Not any additional signalling is needed. |
| Apple | In our view option 1 has already been covered by existing spec. |
| CATT | Same view as Intel that the per-BWP configuration should include the initial BWP. |
| OPPO | Firstly, we think that initial BWP is considered as all other candidate BWPs. We also agree with Huawei’s observation that BWP-DownlinkDedicated can be also configured for initial DL BWP in ServingCellConfig. But it is only applicable to Spcell or Scell of an SCG or an MCG, except PCell of MCG. Based on current RAN2 design, we are not sure whether deactivatedMeasGapList indicated in BWP-DownlinkDedicated can be configured for initial DL BWP of all cases, especially of PCell. Therefore, we propose this issue in RAN4 to fix the hole of indicating the state of pre-configured MG when switching to initial DL BWP, and then trigger the corresponding signaling design in RAN2 if necessary. |
| OPPO2 | We would like to clarify more to try to reach a common understanding for UE on this, Based on the latest TS 38.331, Pre-MG ON/OFF indication is carried by each BWP configuration, i.e. deactivatedMeasGapList-r17 in BWP-DownlinkDedicated to indicate a list of pre-configured gap IDs which will be deactivated upon the switch to this BWP. We focus on PCell’s BWP configuration based on 38.331. Two options are allowed to configure initial BWP(BWP#0) for PCell, as figure B2-1 and B2-2 below. For option 2, initial BWP(BWP#0) can be taken as one dedicated BWP, and there is no problem based on current design. It could usually happen to Pcell in case of HO. However, for option 1, we can see BWP-downlinkdedicated is not configured in BWP#0 and UE can perform BWP switching among 5 BWPs, e.g., timer based BWP switching. The current design of deactivatedMeasGapList-r17 in BWP-DownlinkDedicated for Pre-MG on/off cannot work in this case. In addition, if UE can stay in one cell without any mobility issue, the switching to such initial BWP could be very common case. |
| Moderator | It can be FFS. We can firstly check the following scenarios: - For DCI-based BWP switching, the bwp switching could be not happened from the initial BWP to another BWPs indicated by DCI because the BWP indicator in DCI0-1/1-1 as in table 7.3.1.1.2-1 [38.212] below shall be configured by RRC but initial BWP(BWP#0) is pre-defined by spec indeed. Value of BWP indicator field; Bandwidth part. 2 bits. 00; First bandwidth part configured by higher layers. 01; Second bandwidth part configured by higher layers. 10; Third bandwidth part configured by higher layers. 11; Fourth bandwidth part configured by higher layers. - For RRC-based BWP switching, the bwp switching from initial BWP to another BWP requires RRC reconfiguration indeed. - For timer-based BWP switching, the bwp switching happened from initial BWP to another BWP only if the default BWP was not configured. |

### Sub-topic 1-3 Activation/Deactivation Delay

#### Issue 1-3-1 Pre-MG activation/deactivation delay trigged by Scell activation/deactivation

- Option 1(MTK ): Modify the requirement on the Pre-MG activation/deactivation delay trigged by Scell activation/deactivation as: UE shall be able to finish pre-configured activation or deactivation within [5] ms after the report of the latest first valid CQI among all Scell(s) being activated.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Option 1 is fine for us. |
| Vivo | Ok with option 1. |
| Nokia | We agree the current wording in TS 38.133 in clause 8.19.3 Pre-MG activation/deactivation upon Scell activation/deactivation: “…[5] ms after for the completion of Scell(s) activation / deactivation” and referring to clause 8.3 is not crystal clear. For instance, in clause 8.3.2 the Scell activation delay is defined as Tactivation_time which does not take into account the delay due to subsequent CSI_reporting. Thus, we propose to clarify in 8.19.3 that this delay is included: “…[5] ms after for the completion of Scell(s) activation/deactivation including CSI reporting delay for all Scell(s) being activated, as specified in clause 8.3.2”. |
| Huawei | Support option 1. UE may be able to complete Scell activation faster than the RAN4 requirements, so it is reasonable to base the pre-MG activation/deactivation on the first valid CQI. |
| Qualcomm | Our understanding is that option 1 proposes an optimization of the pre-configured MG de-activation timeline when a Scell is activated. The requirement for pre-configured MG activation when a Scell is de-activated would remain the same. We are not sure if it would be worth making such an optimization. Could there be issues if the network does not receive the first CQI? |
| ZTE | Fine with Option 1. |
| Apple | Fine with option 1. |
| CATT | Fine with option 1. |
| OPPO | Option 1 is fine. |
| MTK | We support Option 1 and also fine with Nokia’s version (i.e., the completion of Scell activation/deactivation including CSI reporting delay) This is not an optimization in our view. Actually in real field, UE may have SCell activation failure. The proposal can also address this issue. |

#### Issue 1-3-2 How to determine pre-MG (de)activation status when multiple trigger events happened simultaneously

- Option 1 (Nokia, ZTE ): The UE shall determine the status of the Pre-MG pattern based on all trigger events

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | This was also the common understanding of RAN4 in the last meeting. We are not sure whether and how this agreement is necessary to be captured in the spec. |
| Nokia | We support option 1. Pre-MG (de-)activation should consider multiple simultaneous trigger events, otherwise this is a strong network limitation for Pre-MG usage, if scenarios like multiple Scells (de-)activation or concurrent Scell (de-)activation and MO addition / release are not supported. We have provided proposed wording for the spec in R4-2210231. |
| E/// | Option 1 is fine |
| Huawei | Support option 1. |
| Qualcomm | We agree that this issue requires further consideration and clarification. RAN4 may start by clarifying what constitutes “simultaneous events.” In our view, since triggering events are relevant in that they can cause a change in the status of a pre-configured gap, two or more triggering events are considered to be “simultaneous” if they would cause a change in the status of the gap at the same time T. For all triggering events (ignoring LocationMeasurementIndication, which we argue in issue 1-1-1 is not a real triggering event), RAN4 has defined a gap activation/deactivation delay of 5 ms. If two or more events can cause a change is status at time T, it means that all the information needed to decide the status of the gap based on all the events is known by time T-5 ms. |
| ZTE | Support Option 1. Referring to how to capture this in the spec, agree with Nokia’s proposal in R4-2210231. Further more, maybe we can add the description “happened in all serving cell(s)”. |
| Apple | Option 1 is fine in general. Discussion may be needed in the CR. |
| CATT | Generally fine with option 1. |
| MTK | Option 1 is fine. But we are not sure whether and how to capture this in the spec. |

#### Issue 1-3-3 Pre-MG activation/deactivation delay after pre-MG being configured initially

- Option 1(Huawei ): Define delay for activation/deactivation after pre-MG configuration as [5]ms after the RRC processing time.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | If the pre-MG was initially configured as ON, the activation delay requirements is needed. And we also thought the requirements for other RRC trigger events as Option 1 can be reused. |
| Vivo | Ok with option 1 |
| Nokia | We support option 1. It should be clarified it is applicable to the rule-based Pre-MG activation/deactivation mechanism only. |
| E/// | Option 1 is ok |
| Huawei | Support option 1. In our view, the pre-MG activation/deactivation after configuration is same as activation/deactivation triggered by RRC procedures. |
| Qualcomm | Configuration of measurement gaps (including pre-configured gaps) is done via RRC so this should be covered already by prior RAN4 agreement. |
| ZTE | Fine with Option 1. This shold be the same handling as delay for pre-MG activation/deactivation triggered by RRC procedures. |
| Apple | option 1 is fine. In our view this has already been covered by previous RAN4 agreement wrt RRC. |
| CATT | Fine with option 1 and fine with Nokia’s clarification that this is for UE autonomous activation/deactivation only. |
| MTK | OK with Option 1 |

## CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2208295 | Nokia: We think the changes in 8.19.3 are not needed. As commented under issue 1-3-1, we propose following change in this clause: “…[5] ms after for the completion of Scell(s) activation/deactivation including CSI reporting delay for all Scell(s) being activated, as specified in clause 8.3.2”. There are typos in clause 9.1.7.3.2. |
|  | E///: generally OK |
|  | Huawei: 1. there are two clauses with same clause number 8.19.2. 2. RAN2 is still discussing how to indicate the pre-MG status for RRC based activation/deactivation, so the change in 9.1.7.3.2 may need to be revisited. 3. some changes in 9.1.7 are conflicting with R4-2208355, R4-2209204 and R4-2210167 |
| R4-2208355 | Nokia: In clause 9.1.7.3, the RRC indication is still to be defined in the RAN2 signalling. In our view, it needs to be indicated in the RRC message containing MeasGapConfig field configuring Pre-MG rather than by the RRC indication preconfiguredNW-ControlledMeasGap-r17. For clause 9.1.7.3.2, the wording in Ericsson’s CR in R4-2210167 is preferred, as it is clearer. |
|  | E///: The updates to NW controlled activation/deactivation are quite unclear. We prefer changes in 8295 (MTK) or in our CR in 10167. |
|  | Huawei: some changes in 9.1.7 are conflicting with R4-2208295, R4-2209204 and R4-2210167. RAN2 is still discussing how to indicate the pre-MG status for RRC based activation/deactivation, so the change in 9.1.7.3.2 may need to be revisited. |
|  | OPPO: how to indicate the pre-MG status for RRC based activation/deactivation can be hold until RAN2 has clear conclusion. We are ok to merge this CR with Ericsson’s CR in R4-2210167. |
|  | MTK: Same view as Huawei. |
| R4-2208459 | Nokia: The CR is agreeable. In section 8.19.3 Pre-MG activation/deactivation upon Scell activation/deactivation, we propose to add a clarification for the condition, when completion of Scell activation / deactivation occurs (see issue 1-3-1). |
|  | E///: generally OK |
|  | MTK: OK |
| R4-2209204 | Nokia: We support proposed changes #1 and #2 to clauses 8.19.3 and 9.1.7.2. We do not support change #3 to clause 9.1.7.3.1 on the removal of initiation of LocationMeasurementIndication procedure for PRS / E-UTRA RSTD and E-CID measurement purpose and on the addition of the pre-configured MG configuration as an additional triggering condition, as the entire clause refers to the time interval after Pre-MG configuration when the already configured Pre-MG pattern is activated or deactivated (see introduction phrase in 9.1.7.3.1). |
|  | E///: It is related to issue 1-1-1. We don’t agree to remove PRS measurements as a trigger. Also inter-RAT RSTD and E-CID should not be removed. There is only one pre-configured MG pattern. There is no configuration of Pre-MG if the UE is already configured with Pre-MG. So why Configuration of pre-configured measurement gap should change the status? |
|  | Huawei: To Nokia and Ericsson, we agree that whether to keep or remove PRS measurements as a trigger depends on outcome of issue 1-1-1. For the change related to configuration of Pre-MG, the intention is to capture the following agreement from RAN4#101-bis (R4-2202614): How to indicate the initial status (activation/deactivation) of Pre-MG at or during configuration for the rule-based activation/deactivation pre-MG - For autonomous activation/deactivation of Pre-MG, the capable UE can determine the pre-MG status autonomously by Pre-MG (de)activation rules define in TS38.133 without any additional signalling. Basically UE needs to determine the initial status of pre-MG after configuration using the same rules as for rule based activation/deactivation of Pre-MG. |
|  | MTK: CR is fine to us. Just that we may need moderator’s help to partition the sections to be handled by different company CRs. |
| R4-2210167 | Nokia: We support changes in 9.1.7.1 and 9.1.7.3.2. For clause 9.1.7.2, we prefer to merge this with Huawei’s draftCR in R4-2209204, as it better distinguished dedicated BWP and Scell cases. The last two changes on Pre-MG status change should be taken from this CR. For all subclauses of 9.1.7, the Ies should have the “-r17” postfix as used in OPPO’s draftCR. There are typos in 9.1.7.3.2: “in not present” should read “is not present”. |
|  | E///: Thanks to Nokia for the comments. We are fine to update the CR and add “-r17” postfix and correct the typos. |
|  | Huawei: some changes in 9.1.7 are conflicting with R4-2208295, R4-2208355 and R4-2209204. RAN2 is still discussing how to indicate the pre-MG status for RRC based activation/deactivation, so the change in 9.1.7.3.2 may need to be revisited. |
|  | MTK: Fine except the part related to ongoing RAN2 discussion |

## Summary for 1st round

### Open issues

Status summary

#### Issue 1-1-1 Additional trigger events for pre-MG activation/deactivation Tentative agreements: None Candidate options: - Option 1(Huawei, Intel, ZTE, Apple,MTK): Remove initiation of LocationMeasurementIndication as a trigger event for rule based pre-MG activation/deactivation, and remove PRS measurement as a criterion to activate pre-MG. - Option 2 (Qualcomm, Nokia,Apple,OPPO): When the network has provided a pre-configured gap and the UE notifies the network via LocationMeasurementIndication that it needs an always-ON gap to perform NR positioning measurements, the UE will determine that the pre-configured MG is always-ON only if the network updates the status of the pre-configured MG accordingly via RRC. Recommendations for 2nd round: Can be further discussed in 2nd round. Companies can also check whether the following harmonized options below which can avoid the additional RRC signaling issue be acceptable. - Option 3: When the network has provided a pre-configured gap and the UE notifies the network via LocationMeasurementIndication that it needs an always-ON gap to perform NR positioning measurements, the UE will determine that the pre-configured MG is always-ON only if the network did NOT updates RRCReconfiguration to configure the legacy MG for UE’s PRS measurements.

#### Issue 1-2-1 Clarifications on simultaneous BWP switches on multiple Tentative agreements: From RAN4 perspective, there is no any requirements applied to simultaneous BWP switches on multiple CCs in Rel17 when pre-configured MG is configured to UE. Candidate options: Recommendations for 2nd round: Companies can check the tentative agreement above.

#### Issue 1-2-2 Whether need indicate the state of pre-configured MG when switching to initial DL BWP Tentative agreements: No Candidate options: - Option 1(OPPO): yes - Option 2: NO #### Recommendations for 2nd round: Further discussion needed. Companies can check whether the BWP switching happened between the initial BWP and other active BWP

#### Issue 1-3-1 Pre-MG activation/deactivation delay trigged by Scell activation/deactivation Tentative agreements: Modify the requirement on the Pre-MG activation/deactivation delay trigged by Scell activation/deactivation as: UE shall be able to finish pre-configured activation or deactivation within [5] ms after the report of the latest first valid CQI among all Scell(s) being activated. Candidate options: #### Recommendations for 2nd round: No further discussion needed.

#### Issue 1-3-2 How to determine pre-MG (de)activation status when multiple trigger events happened simultaneously Tentative agreements: The general principle on how to determine Pre-MG (de)activation status when multiple trigger events happened simultaneously can be: “The UE shall determine the status of the Pre-MG pattern based on all trigger events” Candidate options: Recommendations for 2nd round: The exact wording to reflect this principle can be discussed in the corresponding draft CR.

#### Issue 1-3-3 Pre-MG activation/deactivation delay after pre-MG being configured initially Tentative agreements: Define delay for activation/deactivation after pre-MG configuration as [5]ms after the RRC processing time. Candidate options: #### Recommendations for 2nd round: No further discussion needed.

## Discussion on 2nd round (TBA)

Please only comment on topics that are selected for discussion in 2nd round.

Issue 2-1

| Company | Comments |
| --- | --- |

## Summary on 2nd round

No further agreement was reached in the 2nd round.

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

# Topic #2: Test cases (AI 9.10.2.1)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207755 | Apple | Proposal 1: the following major functionalities of Pre-MG need to be verified: - Pre-MG activation/deactivation mechanism - Network based Pre-MG activation/deactivation - UE autonomous Pre-MG activation/deactivation - Pre-MG activation/deactivation delay - Activation/deactivation upon DCI/timer-based BWP switch Proposal 2: only test cases for SA need to be specified in R17. Proposal 3: for gap pattern configuration, use #0 in FR1 and #13 for FR2. Proposal 4: RAN4 only introduces Pre-MG related test cases for single carrier. |
| R4-2208031 | Qualcomm | Proposal 1: The test cases for concurrent MG cover the following scenarios - NR-SA (not supported in MR-DC) - Single carrier mode only - Measurement procedure only - Non-DRX only - Per-UE gap type - Inter-frequency and inter-frequency measurements - FR1 only Proposal 2: The test cases for pre-configured MG verify the following feature aspects - Activation/deactivation mechanism - Signalling-based or autonomous rules, subject to UE capability - Triggering events - BWP switch - Addition/removal of measurement objects Proposal 3: Introduce an intra-frequency test to verify pre-configured MG activation/deactivation caused by BWP switch. The test can be skipped for Ues that support bwp-withoutRestriction. - Configure two DL BWPs, where only one of them contains the CD SSB - Configure A3 event on an intra-frequency MO - Pre-configure MG with autonomous rule-based or signaling-based activation/deactivation (subject to UE capability) - Switch to BWP w/o CD SSB - Bring up the intra-frequency neighbor cell Proposal 4: Introduce an inter-frequency test to verify pre-configured MG activation/deactivation caused by addition of a measurement object. - No inter-frequency MO configured at the start of the test - Pre-configure MG with autonomous rule-based or signaling-based activation/deactivation (subject to UE capability) - Configure A3 event on inter-frequency MO outside the active DL BWP |
| R4-2208067 | Intel | Proposal 1: It is necessary to define the test cases for NR standalone scenario only. Proposal 2: Test cases for SSB measurement with Pre-MG shall be defined at least. Proposal 3: Test cases for the intra-frequency with Pre-MG gap can be defined with high priority. Proposal 4-1: When the pre-MG activation is based on the network signaling, the test cases under both single carrier and CA test cases shall be defined Proposal 4-2: When the pre-MG activation is based on UE autonomous rules, the test cases under single carrier test cases needed only. Proposal 5: BWP switching trigger event can be tested firstly. Proposal 6: Non-DRX cases will be tested only in Rel17. Proposal 7: The following test cases for core requirement of Pre-MG are listed in Table 1 below. Table 1 Test cases for Pre-MG core requirements No; Type of Test; Description; Test purpose. 1-1; Pre-configured measurement gap activation delay upon DCI/timer-based BWP switching for UE support the autonomous pre-MG activation; FDD/ TDD Pre-configured gap configuration ON, No network signaling to indicate pre-MG activation/deactivation status Gap#0 BWP switching trigger No DRX cycle AWGN; Core requirements in section 8.19.2 which is also rely on UE’s capability to be verified. UE completes the pre-configured MG activation/deactivation within the requirements. 1-2; Pre-configured measurement gap activation delay upon DCI/timer-based BWP switching for UE support the signaling pre-MG activation; FDD /TDD Pre-configured gap configuration ON, Network signaling to indicate pre-MG activation/deactivation status Gap#0 BWP switching trigger No DRX cycle AWGN; Core requirements in section 8.19.2 which is also rely on UE’s capability to be verified. UE completes the pre-configured MG activation/deactivation within the requirements. 2-1; Intra-freq measurement without gap reporting; TDD/FDD, FR1/FR2 SSB Pre-MG deactivated Gap#0 No DRX cycle Alignment b/w cells = synchronous 2 cells in total AWGN; Core requirements in section 9.9.2.4 which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay for certain number of cells. 2-2; Intra-freq measurement with gap reporting; TDD/FDD, FR1/FR2 SSB Pre-MG activated Gap#0 No DRX cycle Alignment b/w cells = synchronous 2 cells in total AWGN; Core requirements in section 9.9.2.4 which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay for certain number of cells. Proposal 8: RAN4 can combine the test cases for Pre-MG activation/deactivation delay and measurement reporting by the testing procedure in Figure 1. The testing procedure for measurements with activated Pre-MG can consist of three successive time periods, with time durations of T1, T2 and T3 respectively. - During the duration of T1, UE can be configured with Pre-MG but being deactivated. - At the start of time duration T2, the serving gNB can trigger Pre-MG activation. And UE is expected to complete the Pre-MG activation within T2. - At the start of time duration T3, the UE may not have any timing information of eighbour cell to be measured (e.g. cell 2).. |
| R4-2208207 | CATT | Proposal 1: Only the test cases for UE autonomous activation/deactivation delay are needed. Proposal 2: Discuss how to decide the ending point of pre-configured MG activation/deactivation delay in the test. Proposal 3: For the test case of Pre-configured MG deactivation delay, use ACK/NACK on the gap occasion as the feedback of the test. Proposal 4: For the test case of Pre-configured MG activation delay, use the measurement report after pre-configured MG activation as the feedback of the test. Proposal 5: The following test cases for Pre-configured MG activation/deactivation are expected: No.; Test case; Note. 1; Pre-configured MG deactivation delay upon DCI, timer based BWP switching; Test in inter-frequency SSB based measurement. 2; Pre-configured MG deactivation delay upon RRC based active BWP switching; Test in inter-frequency SSB based measurement. 3; Pre-configured MG deactivation delay upon activation/deactivation of Scell(s); Test in intra-frequency SSB based measurement. 4; Pre-configured MG deactivation delay upon addition/removal of any measurement object(s); Test in inter-frequency CSI-RS based measurement. 5; Pre-configured MG deactivation delay upon addition/release/change of a Scell in carrier aggregation; Test in intra-frequency SSB based measurement. 6; Pre-configured MG deactivation delay upon Initiation of LocationMeasurementIndication procedure; Test in PRS-RSRP measurement. 7; Pre-configured MG activation delay upon DCI, timer based BWP switching; Test in inter-frequency SSB based measurement. 8; Pre-configured MG activation delay upon RRC based active BWP switching; Test in inter-frequency SSB based measurement. 9; Pre-configured MG activation delay upon activation/deactivation of Scell(s); Test in intra-frequency SSB based measurement. 10; Pre-configured MG activation delay upon addition/removal of any measurement object(s); Test in inter-frequency CSI-RS based measurement. 11; Pre-configured MG activation delay upon addition/release/change of a Scell in carrier aggregation; Test in intra-frequency SSB based measurement. 12; Pre-configured MG activation delay upon Initiation of LocationMeasurementIndication procedure; Test in PRS-RSRP measurement. Note: Both FR1 and FR2 are needed for each test case.. |
| R4-2208300 | MediaTek inc. | Observation 1: The test case scope for Pre-MG should target to cover the following 3 main functionalities: 1) Two activation/deactivation mechanisms, 2) Additional delay for pre-configured MG activation/deactivation and 3) Updated UE requirements. Proposal 1: The Pre-MG test cases should also be limited to NR-SA, i.e., Section A.6 and A.7, while Section A.3 can also be updated to capture new configurations if needed. Proposal 2: Introducing test cases to verify the Pre-MG activation/deactivation delay, e.g., one for network-controlled mechanism and the other for UE autonomous mechanism. Proposal 3: Do not introduce test case to verify UE behaviour regarding Pre-MG impact on L1 measurements. Proposal 4: If RAN4 agrees to introduce Pre-MG test cases for L3 measurement delay, the Pre-MG activation/deactivation delay can also be verified within the test cases. There is no need to additionally introduce test cases in signalling characteristic to only check the activation/de-activation delay. Proposal 5: Adopt the DCI-based BWP switch as the only triggering event in the test cases. |
| R4-2208524 | CMCC | Proposal 1: considering that all existing MG patterns #0~25 in Rel-16 are applicable for the pre-configured MG, it is proposed to define test cases for both per-UE Pre-MG and per-FR Pre-MG. Proposal 2: as for the detailed gap pattens for test cases, gap pattern #0 for per-UE Pre-MG, gap pattern #4 for FR1 Pre-MG and gap pattern #13 for FR2 Pre-MG, can be considered, similar as the test cases for legacy measurement gaps. Proposal 3: it is suggested to discuss and decide whether to define test cases for following combination operations: - Pre-configured FR1 gap + Pre-configured FR2 gap - legacy FR1 gap + Pre-configured FR2 gap - Pre-configured FR1 gap + legacy FR2 gap Proposal 4: it is proposed to define test cases for NW-controlled pre-configured MG activation/deactivation mechanism and autonomous pre-configured MG activation/deactivation mechanism respectively. Proposal 5: For UE autonomous activation/deactivation mechanism, since multiple events could change the pre-MG activation/deactivation status, it is necessary to discuss based on which events to design the test cases. Proposal 6: it is suggested to discuss and decide whether to define test cases for Pre-MG with the use cases of PRS measurement and CSI-RS L3 inter-frequency measurement. Proposal 7: it is proposed to design test case for pre-MG under CA. |
| R4-2208779 | ZTE Corporation | Proposal 1: Between the network-controlled mechanism and UE autonomous mechanism, since network-controlled mechanism is semi-static activation/deactivation, similar as legacy MG configuration/de-configuration, so directly re-using the performance test of legacy MG is fine. Proposal 2: It is necessary to test pre-configured MG based on UE autonomous mechanism. The test purpose is to verify whether UE can perform measurement without/with gap from the first occasion after all switching delay. Proposal 3: In order to test pre-configured MG based on UE autonomous mechanism, the test should be performed under DCI based/timer BWP switching triggered, RRC triggered and MAC CE triggered pre-MG status transition. Proposal 4: Event A3 can be used as the trigger event similar as legacy measurement test. |
| R4-2209209 | Huawei, Hisilicon | Proposal 1: Define TCs for pre-MG with pre-MG deactivation triggered by DCI based BWP switch in a single CC setup. Proposal 2: Define the following testing requirements for all the TCs: - UE performs pre-MG deactivation within the delay requirements - UE performs data Tx/Rx when pre-MG is deactivated - UE performs measurement outside MG when pre-MG is deactivated Proposal 3: Define the following TCs for pre-MG. - TC1: rule based pre-MG deactivation for FR1 - TC2: rule based pre-MG deactivation for FR2 - TC3: RRC based pre-MG deactivation for FR1 TC4: RRC based pre-MG deactivation for FR2 |
| R4-2210169 | Ericsson | - Proposal # 1: Verify cell search/measurement period and data reception in serving cell for FR1 and FR2 during deactivated gaps for autonomous activation/deactivation of Pre-MG under the following scenarios: - Pre-MG status change under DCI based active BWP switching separately in FR1 and FR2. - Pre-MG status change triggered by inter-RAT LTE measurement configuration in FR1 - Pre-MG status change triggered by SSB based inter-frequency measurement configuration in FR2 - Proposal # 2: Verify cell search search/measurement period and data reception in serving cell for FR1 and FR2 during deactivated gaps for RRC based activation/deactivation of Pre-MG. - Proposal # 3: In all tests, UE is configured with only one serving cell (Pcell) i.e. in non-CA scenario. - Proposal # 4: All tests are conducted in AWGN and in non-DRX. - Proposal # 5: Proposed test cases are given in Table: Table 1: Propose test case scenario to verify requirements based on Pre-MG pattern No.; Test case; Test setup and scenario; Purpose of test. 1; SA event triggered reporting tests with autonomous activation/deactivation of Pre-MG by BWP switching in FR1; - Two intra-frequency cells (Cell1 and Cell2 on F1), per-UE Pre-MG, non-DRX, AWGN - Test times: T1, T2 and T3 - Pre-MG status is changed by DCI based active BWP - Pre-MG status: deactivated in T2 and activated in T3. - Data scheduled during T2; - During T2: SSB based intra-frequency cell search delay is met for Cell2, and UE receives data in Cell1 - During T3: SSB based intra-frequency measurement period is met for Cell2.. 2; SA event triggered reporting tests with autonomous activation/deactivation of Pre-MG by BWP switching in FR2; Same as in test case scenario 1; Same as in test case scenario 1. 3; SA event triggered reporting tests with autonomous activation of Pre-MG by inter-RAT LTE in FR1; - Two intra-frequency cells (Cell1 and Cell2 on F1), one LTE cell (Cell3) on F2, per-UE P-MGP, non-DRX, AWGN - Test times: T1, T2 and T3 - Pre-MG status is deactivated by configuration of inter-RAT LTE measurements - Pre-MG status: deactivated in T2 and activated in T3. - Data scheduled during T2 - Inter-RAT LTE (Cell3) configured beginning of T3; - During T2: SSB based intra-frequency cell search delay is met for Cell2, and UE receives data in Cell1 - During T3: inter-RAT cell search delay is met for Cell3. 4; SA event triggered reporting tests with autonomous activation of Pre-MG by SSB based inter-frequency in FR2; - Two intra-frequency cells (Cell1 and Cell2 on F1), one inter-frequency cell (Cell3) on F2, per-UE P-MGP, non-DRX, AWGN - Test times: T1, T2 and T3 - Pre-MG status is deactivated by configuration of SSB based inter-frequency measurements requiring gaps - Pre-MG status: deactivated in T2 and activated in T3. - Data scheduled during T2 - Inter-frequency (Cell3) configured beginning of T3; - During T2: SSB based intra-frequency cell search delay is met for Cell2, and UE receives data in Cell1 - During T3: SSB based inter-frequency cell search delay is met for Cell3. 5; SA event triggered reporting tests with network-controlled activation/deactivation of Pre-MG in FR1; - Two intra-frequency cells (Cell1 and Cell2 on F1), per-UE Pre-MG, non-DRX, AWGN - Test times: T1, T2 and T3 - Pre-MG status is changed by RRC command. - Pre-MG status: deactivated in T2 and activated in T3. - Data scheduled during T2; - During T2: SSB based intra-frequency cell search delay is met for Cell2, and UE receives data in Cell1 - During T3: SSB based intra-frequency measurement period is met for Cell2.. 6; SA event triggered reporting tests with network-controlled activation/deactivation of Pre-MG in FR2; Same as in test case scenario 5; Same as in test case scenario 5. |

## Open issues summary and companies’ views collection for 1st round

### Sub-topic 2-1 Test cases configuration

#### Issue 2-1-1 Test cases for the different deployment scenarios

- Option 1 (Apple, Qualcomm, Intel, MTK, Ericsson ):  Only need to define the test cases for SA

Recommended WF: Agree Option 1.

| Company | Comments |
| --- | --- |
| Intel | Support recommended WF |
| vivo | Support recommended WF |
| Nokia | We support the recommended WF. |
| E/// | Support recommended WF |
| Huawei | Support the Recommended WF. |
| Qualcomm | Support the recommended WF. |
| ZTE | Support recommended WF |
| Apple | Support the recommended WF. |
| CATT | Support the recommended WF. |
| OPPO | Support the recommended WF. |
| MTK | Support the recommended WF. |

#### Issue 2-1-2 Non-DRX or DRX

- Option 1 (Intel, Qualcomm, Ericsson) Only define the test cases for Non-DRX cases.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Option 1 |
| vivo | Ok with option 1 |
| Nokia | We support option 1. |
| E/// | Option 1 is fine |
| Huawei | Support option 1 |
| Qualcomm | Option 1 |
| ZTE | Fine with Option 1. |
| Apple | Option 1. |
| CATT | Support option 1. |
| OPPO | Option 1. |
| MTK | Option 1 |

#### Issue 2-1-3 CA or single carrier

- Option 1 (Apple, Qualcomm, Ericsson, HW) : Only single carrier

- Option 2( Intel , CMCC, CATT ): both single carrier and CA

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Both options are fine for us. But for Option 2, some method to reduced TCs can be considered. |
| Vivo | Option 1 is preferred |
| Nokia | We support option 1. |
| E/// | Prefer Option 1 |
| Huawei | Option 1. In our view, there are indeed some additional work that the UE should do for CA either for rule based or RRC based activation/deactivation, but they are functional while the test should focus on the performance of the pre-MG. |
| Qualcomm | Option 1. Single carrier is sufficient to test the triggering events that most companies want to cover in the tests. |
| ZTE | Prefor Option 2. For the network signaling based mechanism, referring to deactivated Scell, CA case should be verified. |
| Apple | Option 1, assuming not all UE have to support CA. |
| CATT | Slightly prefer option 2 to test the CA case. Before deciding this issue, we would like to clarify the test cases for the trigger event. Whether the Scell activation/deactivation triggered Pre-MG activation need to be tested? |
| OPPO | Option 1. |
| MTK | Support Option 1. This makes the tests also applicable to the CA-incapable UEs, e.g., Redcap. |
| CMCC | Prefer option 2. For CA, an additional ON/OFF indication for each SCell is introduced to indicate the Pre-configured MG status when the SCell is de-activated, which need to be tested. And this test can only apply to CA capable UE. |

#### Issue 2-1-4 Pre-MG activation mechanism

- Option 1 (Apple, Intel, Qualcomm, MTK, CMCC) UE autonomous pre-MG activation and network signalling Pre-MG activation mechanism shall be tested

- Option 2(ZTE, CATT  ): UE autonomous pre-MG activation only

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Option1. And we can test the two mechanisms within the different cases to reduce the total TCs number. |
| Vivo | Option 1. |
| Nokia | We support option 1. Both mechanisms have been specified for Pre-MG and hence require to be tested. |
| E/// | Prefer Option 1 since these are different UE capabilities. If UE supports both mechanisms then applicability rules can be defined. |
| Huawei | Option 1. The two mechanisms are for different UE capabilities and NW implementations, so they should be both tested. |
| Qualcomm | Option1. There are separate UE capabilities for each mechanism. |
| ZTE | Fine with option 1 since some new requirements also introduced to NW signalling mechanism. |
| Apple | Option 1. |
| CATT | We propose option 2 because for NW-controlled mechanism the measurement requirements are same as the existing requirements as long as the UE follow the NW indication and no need to be tested. But if all companies want to introduce the test case, we are fine to compromise. |
| OPPO | Option 1. |
| MTK | Option 1 |
| CMCC | Option 1 |

#### Issue 2-1-5 Which types of measurement reference signal measured

- Option 1 (Qualcomm, Intel, E///) SSB

- Option 2(CATT): SSB, PRS, CSI-RS

- Option 2a( CMCC ): FFS on whether CSI-RS and PRS measurement TCs shall be defined.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| vivo | Option 1 is ok. PRS could be considered. |
| Nokia | We prefer option 1. |
| E/// | Option 1. |
| Huawei | Option 1. The test should focus on the performance of pre-MG related operations, and testing all the RS-es is not the key point. |
| Qualcomm | Option 1. |
| Apple | Option 1. CSI-RS and PRS are optional. UE supporting Pre-MG may not support CSI-RS or PRS. |
| CATT | Prefer option 2. But to reduce the number of the test cases, we suggest defining different test cases with different reference signals, e.g. SSB for NW-controlled mechanism and CSI-RS for UE autonomous mechanism. |
| OPPO | Option 1. |
| MTK | Support Option 1. Anyway, we should start from SSB first. |
| CMCC | Except SSB, we also prefer to consider CSI-RS and PRS measurement TCs, but if companies have strong preference on option 1, we are also fine. |

#### Issue 2-1-6 Which types of trigger events

- Option 1 (Intel, MTK, HW) BWP switching

- Option 2( Qualcomm, E///): BWP switching , addition/removal of measurement objects

- Option 3(CATT, ZTE): all trigger events

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Both Option 1 and 2 are fine for us. For Option 2, the different trigger events shall be coupled with the different TCs. |
| Vivo | Ok with option 1 |
| Nokia | We prefer option 2. |
| E/// | Option 2. In our view BWP switching and addition/removal of measurement objects can be tested in the same test e.g. in different test times. |
| Huawei | Option 1. In our view, the typical use case for pre-MG is BWP switching, and it is also the most different and challenging use case compared to legacy MG. |
| Qualcomm | We support option 2, which includes addition/removal of MOs in addition to BWP switch. Perhaps other companies think that testing MO addition is not as valuable because it’s done by RRCReconfig? We would like to hear the views of others. |
| ZTE | Our original view is not to verify each trigger event one by one, but to verify all types of trigger events, i.e. triggered by DCI, MAC CE and RRC signalling, since each of them has individual transition delay starting point. |
| Apple | Option 1. This is the most typical use case. |
| CATT | Option 3. New UE behavior is defined for the trigger events and should be verified. |
| OPPO | Option 1. This is the most typical use case. |
| MTK | Option 1. This is the typical case, which is also mentioned in WID. Also, BWP switch is the most stringent case which provides UE the shortest time to activate/deactivate the gap. |

#### Issue 2-1-7 per-UE/per-FR gap patterns

- Option 1(Qualcomm, E///): per-UE gap pattern

- Option 2(CMCC): both per-UE and per-FR gaps could be configured in TCs.

- Option 2a(CMCC):

    - discuss and decide whether to define test cases for following combination operations: Pre-configured FR1 gap + Pre-configured FR2 gap

    - legacy FR1 gap + Pre-configured FR2 gap

    - Pre-configured FR1 gap + legacy FR2 gap

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Prefer Option 1. |
| Vivo | Prefer option 1 |
| Nokia | We prefer option 1. |
| E/// | Option 1. |
| Huawei | Option 1. Alternatively, we can also follow the same approach as existing TCs, i.e. we define sub-tests for per-UE gap and per-FR gap, and UE supporting per-FR gap only needs to pass the sub-test with per-FR gap. |
| Qualcomm | Support option 1. |
| ZTE | Option 1. |
| Apple | Option 1. |
| CATT | Both option 1 and option 2 are fine. We are also fine with Huawei’s suggestion to define sub-tests for per-UE gap and per-FR gap. |
| OPPO | Option 1 |
| MTK | Option 1, which is applicable to every UE. |
| CMCC | Option 2, and we are OK with Huawei’ suggestion which guarantee the test coverage and not increase test burden. For Option 2a, in our contribution, it is proposed to further discuss whether to have tests for these combinations. We do not have strong preference, just considering that we have these combinations and would like to hear companies’ view whether we need to define test. |

#### Issue 2-1-8 Gap pattern

- Option 1 (Apple) : #0 in FR1 and #13 for FR2.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Option 1 is fine. |
| Vivo | OK with option 1. |
| Nokia | We support option 1. |
| E/// | Option 1 is fine. |
| Huawei | Support option 1. |
| Qualcomm | OK with option 1 if FR2 test cases are defined. |
| ZTE | Fine with Option 1. |
| Apple | Option 1. |
| CATT | Fine with option 1. |
| OPPO | Option 1. |
| MTK | Option 1 is fine |
| CMCC | This issue is related with Issue 2-1-7. If per-FR gap is considered, i.e. define sub-tests for per-UE gap and per-FR gap, and UE supporting per-FR gap only needs to pass the sub-test with per-FR gap, except #0, #13, we also need to consider gap pattern #4. For FR1, if a UE supports per-FR gap, only need to pass test with gap pattern #4, otherwise, UE only need to pass test with gap pattern #0, similar as existing TCs. |

#### Issue 2-1-9 Testing procedure

- Proposal 1 (Intel, E///): Both the activation/deactivation and measurement reporting can be fulfilled within one testing procedure.

- Proposal 2(Qualcomm, ZTE): Configure A3 event can be used as the measurement reporting trigger events as the test cases with the legacy MGs

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Proposal 1 and 2 are fine for us. |
| vivo | Ok with option 1. |
| Nokia | We prefer proposal 2, but proposal 1 is also ok. |
| E/// | Option 1. We are fine to use one of the event (e.g. A3) used in the existing tests for measurement reporting. |
| Huawei | Both proposals are agreeable to us. |
| Qualcomm | The two proposals are not mutually exclusive. Both can be supported. |
| ZTE | Fine with Proposal 1 and 2. |
| Apple | Fine with both proposal 1 and 2. |
| CATT | Fine with proposal 1. |
| OPPO | Fine with Proposal 1 and 2. |
| MTK | Start from Proposal 1 first |

#### Issue 2-1-10 Test cases on FR1/FR2

- Option 1(Qualcomm): Only define TCs for FR1

- Option 2. (Intel, CATT, Huawei, Ericsson): Define the different TCs for FR1 and FR2.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Option 2. For FR1/FR2 the testing method could be different. |
| Nokia | We support option 2. |
| E/// | Option 2. It is important to have similar tests for FR1 and FR2. FR1 and FR2 should not be mixed in the same test due to testability issue. |
| Huawei | Option 2. There seems to be no clear reason to exclude FR2 test. |
| Qualcomm | Prefer option1 but would compromise to define some test cases for each FR. |
| ZTE | Option 2. |
| Apple | Option 2. |
| CATT | Option 2. |
| OPPO | Option 2 |
| MTK | Option 2 |
| CMCC | Option 2 |

### Sub-topic 2-2 Testing purpose and test cases list

[Moderator notes:

Companies’ view on TC’s list are quite diverse. In order converge this issue clearly, we can firstly decouple this issue to the following sub-issues.

]

#### Issue 2-2-1 Test cases for Pre-configuration MG activation/deactivation delay

- Option 1 (Intel, CATT):  Define TCs for both activation and deactivation delay

- Option 2 (Huawei): Define TCs for pre-MG deactivation delay

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Option 1 |
| Nokia | We support option 1. |
| E/// | Option 1. This can be done in the same test during different test times. |
| Huawei | Option 2, in our view, the pre-MG deactivation delay can be verified by checking whether UE causes interruption due to MG, but activation is not so easy to test. We can also compromise to option 1 if there are clear approach to verify the activation delay. |
| Qualcomm | We do not agree fully with either option. We agree that each the test case should feature a change in the status of the pre-configured gap during the test. Otherwise, the test would be similar to a legacy test for measurements with gaps or without gaps. We do not think it is necessary to verify both activation and activation delay in the same test case. Verifying one of them per test case should be sufficient. We do think meaningful test cases can be defined where the measurements are performed within the pre-configured gaps and the gaps would not be activated for the whole duration of the test. We included proposals in our paper. |
| ZTE | Option 1. |
| Apple | Option 1 is fine. They can be verified in one test. |
| CATT | Support option 1. We can further discuss the test ending point for activation and deactivation delay. |
| MTK | Support Option 1. We think the activation is also testable if we configure CQI report for every 2 slots. |
| CMCC | Option 1 |

#### Issue 2-2-2 Test cases for the reporting delay of intra-frequency measurement with gap

- Option 1 (CATT):  No need

- Option 2 (ZTE): Define TCs to verify whether UE can perform measurement without/with gap from the first occasion after all switching delay

- Option 3(Intel, Qualcomm, E///): Combine the test cases for the intra-frequency measurement reporting with that for Pre-MG activation/deactivation delay which is triggered by BWP switching

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | In order to achieve the better balance between the testing coverage and efforts, Option 3 shall be supported. |
| Vivo | Ok with option 3 and option 2. |
| Nokia | We support options 2 and 3. |
| E/// | Option 3. |
| Huawei | Option 2 and 3. In our view, in the test, a BWP switch will be triggered, and the intra-freq measurement will change from a measurement with MG to a measurement without MG, and the pre-MG will be deactivated. We can verify the deactivation delay (by checking the data interruption) and measurement delay in the same test. |
| Qualcomm | Support option 2. |
| ZTE | Support Option 2 and 3. |
| Apple | Fine with option 2 and 3. |
| CATT | Follow the GTW agreement. For option 2, whether the measurement without gap is needed should be further discussed. |
| OPPO | Support option 3. BTW, we can focus on intra-frequency measurement with gap. |

#### Issue 2-2-3 Test cases for the reporting delay of inter-frequency measurement with gap

- Option 1 (Qualcomm, E///):  Inter-frequency measurement reporting which is triggered by MO remove/addition

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Prefer to test intra-frequency measurement report test case only because in case of inter-frequency measurement, there is no any pre-MG status switching which is same as the legacy measurement TC. |
| Nokia | We support option 1. |
| E/// | Option 1. Our suggestion is to verify in the same test case: - intra-frequency measurements without gaps (during deactivated Pre-MG) in one test time (T2) and - inter-frequency measurements with gaps (during activated Pre-MG) in another test time (T3) The configuration of inter-frequency MO during T3 will trigger the UE to activate the Pre-MG. |
| Huawei | We do not support option 1. In our view, the test is similar to existing inter-freq TCs with a legacy MG configured. |
| Qualcomm | Support option 1. |
| ZTE | Support Option 1. |
| Apple | We don’t see the necessity of this. Similar view as HW, this is similar to existing inter-f test with legacy MG. |
| CATT | Option 1 is not clear to us. What is the purpose of the suggested test? For inter-frequency measurement reporting or for Pre-MG activation/deactivation delay triggered by MO addition? If it is for measurement reporting, we don’t think it is needed. But if it is for pre-MG activation/deactivation delay, then we are fine. |
| MTK | We prefer to start from intra-frequency and BWP switch trigger event. Others can be low priority |

#### Issue 2-2-4 Test cases for data Tx/Rx when pre-MG deactivated

- Option 1 (Huawei):  Define TCs for pre-MG to verify whether UE can perform the data Tx/Rx when  pre-MG deactivated.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Nokia | We support option 1. |
| E/// | We support Option 1. This can be tested in the same test where UE measurement reporting delay is tested. |
| Huawei | Support option 1. Data Tx/Rx when pre-MG is deactivated is the main benefit of pre-MG, so it should be verified. |
| Qualcomm | Support option 1. |
| ZTE | We support Option 1. |
| Apple | Option 1 is fine. It can be verified together with other funcitionalities. |
| CATT | Fine with option 1. But we would like to check whether it can be included in the Pre-MG deactivation delay test. |
| MTK | OK with Option 1 |
| CMCC | Support option 1 |

#### Issue 2-2-5 Test cases list (TBA)

[Moderator notes:

Given the comments on TCs list received in 1st round and GTW, the following test cases can be defined

- Proposal 1: test cases for the intra-frequency measurement reporting with that for Pre-MG activation/deactivation delay which is triggered by BWP switching

And for the other test cases (e.g. inter-frequency measurement reporting, the (de)activation delay with more trigger events) can be FFS in 2nd round. From the moderator perspective, the inter-frequency measurement reporting with Pre-MG is same as these with the legacy MG because the pre-MG shall be always ON

If the main purpose to introduce the inter-frequency measurement triggered by MO remove/addition is to cover more trigger events, it is better to couple more triggered events  with intra-frequency measurement test cases. Therefore, please companies check the following proposals also.

- Proposal 2: FFS on the other test cases (e.g. inter-frequency measurement reporting, the (de)activation delay with more trigger events)

  - Option 2-1: test cases for the inter-frequency measurement reporting with that for Pre-MG activation/deactivation delay which is triggered by MO remove/addition

  - Option 2-2: test cases for the intra-frequency measurement reporting with that for Pre-MG activation/deactivation delay which is triggered by MO remove/addition

]

### CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |

## Summary for 1st round

### Open issues

Status summary

#### Issue 2-1-1 Test cases for the different deployment scenarios Tentative agreements: Only need to define the test cases for SA Candidate options: Recommendations for 2nd round: No further discussion needed.

#### Issue 2-1-2 Non-DRX or DRX Tentative agreements: Only define the test cases for Non-DRX cases Candidate options: Recommendations for 2nd round: No further discussion needed

#### Issue 2-1-3 CA or single carrier Tentative agreements: The single carrier test cases can be defined firstly. Candidate options: #### Recommendations for 2nd round: Further discussion on whether the test cases for CA is needed.

#### Issue 2-1-4 Pre-MG activation mechanism Tentative agreements: Both UE autonomous pre-MG activation and network signalling Pre-MG activation mechanism shall be tested Candidate options: #### Recommendations for 2nd round: No further discussion needed.

#### Issue 2-1-5 Which types of measurement reference signal measured Tentative agreements: The SSB reference signal can be configured for Pre-MG TCs. Candidate options: #### Recommendations for 2nd round: No further discussion needed.

#### Issue 2-1-6 Which types of trigger events Tentative agreements: The test case for Pre-MG activation triggered by DCI-based BWP switching can be define firstly. FFS on the test case for Pre-MG activation triggered by other events Candidate options: #### Recommendations for 2nd round: Further discussion on whether other trigger events test cases needed.

#### Issue 2-1-7 per-UE/per-FR gap patterns Tentative agreements: per-UE gap pattern can be configured in Pre-MG testing Candidate options: #### Recommendations for 2nd round: No further discussion needed.

#### Issue 2-1-8 Gap pattern Tentative agreements: #0 in FR1 and #13 for FR2 Candidate options: #### Recommendations for 2nd round: No further discussion needed.

#### Issue 2-1-9 Testing procedure Tentative agreements: Both the activation/deactivation and measurement reporting can be fulfilled within one testing procedure. Candidate options: #### Recommendations for 2nd round: Further discussion on Proposal 2(Qualcomm, ZTE): Configure A3 event can be used as the measurement reporting trigger events as the test cases with the legacy MGs

#### Issue 2-1-10 Test cases on FR1/FR2 Tentative agreements: Define the different TCs for FR1 and FR2 Candidate options: #### Recommendations for 2nd round: No further discussion needed.

#### Issue 2-2-1 Test cases for Pre-configuration MG activation/deactivation delay Tentative agreements: GTW agreements: Define TCs for both activation and deactivation delay Candidate options: #### Recommendations for 2nd round: No further discussion needed.

#### Issue 2-2-2 Test cases for the reporting delay of intra-frequency measurement with gap Tentative agreements: GTW Agreement: - Combine the test cases for the intra-frequency measurement reporting with that for Pre-MG activation/deactivation delay which is triggered by BWP switching Candidate options: Recommendations for 2nd round: No further discussion needed.

#### Issue 2-2-3 Test cases for the reporting delay of inter-frequency measurement with gap Tentative agreements: None Candidate options: Option 1: Inter-frequency measurement reporting which is triggered by MO remove/addition #### Recommendations for 2nd round: Can be further discussed in 2-2-5

#### Issue 2-2-4 Test cases for data Tx/Rx when pre-MG deactivated Tentative agreements: Define TCs for pre-MG to verify whether UE can perform the data Tx/Rx when pre-MG deactivated. Candidate options: #### Recommendations for 2nd round: No further discussion needed

#### Issue 2-2-5 Test cases list Tentative agreements: - Test cases for the intra-frequency measurement reporting with that for Pre-MG activation/deactivation delay which is triggered by BWP switching Candidate options: Recommendations for 2nd round: Further discussion on the other test cases (e.g. inter-frequency measurement reporting, the (de)activation delay with more trigger events) - Option 2-1: test cases for the inter-frequency measurement reporting with that for Pre-MG activation/deactivation delay which is triggered by MO remove/addition - Option 2-2: test cases for the intra-frequency measurement reporting with that for Pre-MG activation/deactivation delay which is triggered by MO remove/addition

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

## Discussion on 2nd round

Please only comment on topics that are selected for discussion in 2nd round.

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | R17 NR MG enhancements – Pre-configured MG | Intel |  |
|  | LS on Pre-configured MG | OPPO | To: RAN_2; Cc: Up to the issue 1-2-2 |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2208295 |  | Maintenance CR on TS38.133 for pre-MG | MTK | Merged | The change part of 8.19. can be merged with R4-2208459 The change part of 9.1.7.1& 9.1.7.2 can be merged with R4-2209204 The change part of 9.1.7.3 can be merged with R4-2210167 |
| R4-2208355 |  | CR to maintain pre-configured measurement gap in TS 38.133 | Oppo | Merged | The change part of 9.1.7.1& 9.1.7.2 can be merged with R4-2209204 The change part of 9.1.7.3 can be merged with R4-2210167 |
| R4-2208459 |  | draftCR on Pre-MG core maintenance (8.19) | Apple | Revised | Combined all changes on 8.19 from R4-2208295, R4-2209204. |
| R4-2209204 |  | CR on pre-MG related requirements (9.1.7.1 &9.1.7.2) | Huawei | Revised | Combine all changes on 9.1.7.1 &9.1.7.2 from R4-2208295, R4-2208355, R4-2210167 Remove the change part of 9.1.7.3 which can be merged with R4-2210167 Remove the change part of 8.19. which can be merged with R4-2208459 |
| R4-2210167 |  | Correction to Pre-MG requirements in TS 38.133 (9.1.7.3) | Ericsson | Revised | Combine all changes on 9.1.7.3 from R4-2208295, R4-2208355, R4-2209204 Remove the change part of 9.1.7.1 &9.1.7.2 which can be merged with R4-2209204 |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Nokia | Juergen Hofmann | juergen.hofmann@nokia.com |
| Apple | Qiming Li | Li_qiming@apple.com |
| CATT | Qiuge Guo | guoqiuge@catt.cn |
| OPPO | Roy Hu | hurongyi@oppo.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)