3GPP TSG-RAN WG4 Meeting # 103-e												R4-2210302

Electronic Meeting, 09 – 20 May 2022

Agenda item:			9.22

Source:	Nokia, Nokia Shanghai Bell

Title:	Email discussion summary for [103-e][230] NR_IIOT_URLLC_enh

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

WF for RAN4#103 as agreed in RAN4#102 (R4-2207020):

- UE Rx-Tx time difference measurement requirement for PRS:

  - FFS for requirements for fading conditions

- gNB Rx-Tx time difference measurement requirement for SRS:

  - FFS for requirements for fading conditions

- UE Rx-Tx time difference measurement requirements for TRS

  - FFS for channel model for requirements definition and whether to include both models

  - Number of samples assumed for deriving the accuracy requirements [1 or 4]

- Interested companies are encouraged to provide simulation results for TRS based PDC for RAN4#103

- Test case work for PDC RTT UE Rx-Tx time difference measurement accuracy requirement with TRS/PRS and SRS

Agenda items for RAN4#103:

9.22	Enhanced IIoT and URLLC support	[NR_IIOT_URLLC_enh]

9.22.1	RRM core requirement maintenance	[NR_IIOT_URLLC_enh-Core]

9.22.1.1	Propagation delay compensation enhancements	[NR_IIOT_URLLC_enh-Core]

9.22.1.2	Reference point for Te requirements	[NR_IIOT_URLLC_enh-Core]

9.22.1.3	Others	[NR_IIOT_URLLC_enh-Core]

9.22.2	RRM performance requirements	[NR_IIOT_URLLC_enh-Perf]

Following Topics will be handled in this email discussion:

- Topic#1 Propagation delay compensation enhancements

- Topic#2 Reference point for Te requirements

- Topic#3 RRM performance requirements

# Topic #1: Propagation delay compensation enhancements

Discussion papers and CR submitted for the meeting 103-e addressing remaining Core requirements:

- 4 Tdoc’s

- 3 CRs

7 open Issues to discuss and agree in this meeting.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2208820 | vivo | Proposal 1: No accuracy requirements under fading channel are specified for both UE Rx-Tx time difference measurement based on PRS/TRS and gNB Rx-Tx time difference measurement based on SRS. Proposal 1a: Accuracy requirements under fading channel are specified for both UE Rx-Tx time difference measurement based on PRS/TRS and gNB Rx-Tx time difference measurement based on SRS. Proposal 2: Measurement period requirements for PRS based UE Rx-Tx time difference measurement is TUERxTx,PRS=Nsample *TPRS if UE capability {N, T} is not needed for RTT-based PDC. Proposal 3: Accuracy requirements for RTT-based PDC with TRS are based on measurements with one sample under AWGN channel. |
| R4-2209235 | Huawei, HiSilicon | Proposal 1: RAN4 waits for RAN1 conclusion on the applicable capabilities for PRS based PDC, and to revisit the requirements if necessary. Proposal 2: Sample number for TRS based PDC is 4. Proposal 3: Introduce a scaling factor Kgap to account for the PDC resources occasions dropped due to collision with MG. Proposal 4a: No Rx beam sweeping factor is needed if QCL information is provided for the RS. Proposal 4b: Define scheduling restriction if PDC RS is not QCL-ed with PDCCH or PDSCH. Proposal 4c: Define measurement restriction if PDC RS is not QCL-ed with L1 RS. Proposal 5: Define DRX requirements for PDC such that UE is assumed to take one sample per DRX cycle. |
| R4-2209506 | Nokia, Nokia Shanghai Bell | Proposal 1: Consider AWGN channel and the fading channel condition defined RTT based PDC with PRS. Observation 1: It has been agreed to only consider AWGN for the RTT based PDC gNB Rx-Tx time difference measurement requirement. Proposal 2: Define UE Rx-Tx time difference measurement accuracy with TRS in both AWGN and fading channels for FR1 and FR2. Proposal 3: Define UE Rx-Tx time difference measurement accuracy with 1 TRS measurement sample for FR1 and FR2. Observation 2: Given the latest RAN1/2 specification for IIOT_URLLC_enh, the measurement for PDC in TS 38.133 -9.12 should be updated/corrected. Proposal 4: Introduce new SRS type in TS 38.133 A.3.24 for usagePDC-r17 Proposal 5: Define test case for RTT-based PDC UE Rx-Tx time difference measurement period with PRS and TRS. Proposal 6: Define test case for RTT-based PDC UE Rx-Tx time difference measurement accuracy with PRS and TRS. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description

Measurement period requirements for PRS based UE Rx-Tx time difference measurement:

Background (from R4-2208820):

In our understanding, UE capability {N, T} for PRS processing for supporting positioning is not needed for RTT-based PDC as the measurement is only on one serving cell and within active BWP. The processing capability would be similar between TRS and PRS. Thus, measurement period requirements for PRS should be similar as for TRS if {N, T} is not needed.

Issue 1-1: Measurement period requirements for PRS should be similar as for TRS if {N, T} is not needed?

- Proposals

- Recommended WF

  - Discuss together with Sub-topic 1-2.

### Sub-topic 1-2

Sub-topic description

Applicable capabilities for PRS based PDC:

Background (from R4-2209235):

The current measurement period requirements for PRS based PDC are mostly re-used from PRS based positioning measurement, and in particular, some of the scaling factors are based on the assumption that feature group 13-1 (PRS processing capability for positioning) are applicable for PDC, e.g. {N,T} and max number of PRS resources per slot (N’)

Issue 1-2: RAN4 waits for RAN1 conclusion on the applicable capabilities for PRS based PDC?

- Proposals

  - Option 1: RAN4 keep the impacted parameters in [] until RAN1 has reached agreement.

  - Option 2: Other

- Recommended WF

  - RAN4 keep the impacted parameters in [] until RAN1 has reached agreement. RAN4 can additionally add an editor’s note: ‘Pending agreement on UE processing capability discussion’.

### Sub-topic 1-3

Sub-topic description

The number of samples assumed for deriving the accuracy requirements:

Issue 1-3: The number of samples assumed for deriving the accuracy requirements [1 or 4]?

- Proposals

  - Option 1: samples assumed for deriving the accuracy requirements: 1 [vivo, Nokia]

  - Option 2: samples assumed for deriving the accuracy requirements: 4 [Huawei]

- Recommended WF

  - More discussion needed

### Sub-topic 1-4

Sub-topic description

Other aspects raised.

Background (from R4-2209235):

A number of open issues were raised on this paper:

- Need for scaling factor Kgap when the PDC resources occasions collide with MG?

- Need for Rx beam sweeping factor if no QCL information is provided for the RS?

- Need for scheduling restriction if PDC RS is not QCL-ed with PDCCH or PDSCH?

- Need for defining measurement restriction if PDC RS is not QCL-ed with L1 RS?

- Need for defining DRX requirements for PDC?

Next these issues are listed for companies to comment.

Issue 1-4: Is there a need to introduce scaling factor Kgap when the PDC resources occasions collide with MG?

- Proposals

  - Option 1: Yes, there is a need for scaling factor Kgap when the PDC resources occasions collide with MG.

  - Option 2: Other

- Recommended WF

  - More discussion needed.

Issue 1-5: Is there a need for Rx beam sweeping factor if no QCL information is provided for the RS?

- Proposals

  - Option 1: Yes, there is a need for Rx beam sweeping factor if no QCL information is provided for the RS.

  - Option 2: Other

- Recommended WF

  - More discussion needed.

Issue 1-6: Is there a need for scheduling restriction if PDC RS is not QCL-ed with PDCCH or PDSCH?

- Proposals

  - Option 1: Yes, there is a need for scheduling restriction if PDC RS is not QCL-ed with PDCCH or PDSCH.

  - Option 2: Other

- Recommended WF

  - More discussion needed.

Issue 1-7: Is there a need for defining measurement restriction if PDC RS is not QCL-ed with L1 RS?

- Proposals

  - Option 1: Yes, there is a need for defining measurement restriction if PDC RS is not QCL-ed with L1 RS.

  - Option 2: Other

- Recommended WF

  - More discussion needed.

Issue 1-8: Is there a need to define DRX requirements for PDC measurement?

- Proposals

  - Option 1: Yes, define DRX requirements for PDC such that UE is assumed to take one sample per DRX cycle.

  - Option 2: Other

- Recommended WF

  - More discussion needed.

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1: Issue 1-1: Measurement period requirements for PRS should be similar as for TRS if {N, T} is not needed?

| Company | Comments |
| --- | --- |
| Moderator | No comments needed. Comment together with next Sub-topic 1-2 |

Sub topic 1-2: Issue 1-2: RAN4 waits for RAN1 conclusion on the applicable capabilities for PRS based PDC?

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support the Recommended WF. |
| Ericsson | WF is fine. |
| Nokia | Support option 1. As it is mentioned that the UE capability and the related parameters for PDC measurement with PRS and TRS is not yet defined from RAN1. Also check R2-2203766 and TS38.306—17.0.0. |
| vivo | We are fine with the recommended WF. Issue 1-1 is next step if RAN1 decision is that no UE capability is needed for PDC. |
| Qualcomm | Support the recommended WF. |

Sub topic 1-3: Issue 1-3: The number of samples assumed for deriving the accuracy requirements [1 or 4]?

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 2. It was agreed that for PRS based PDC the sample number is 4, and in our view it is better to have aligned measurement period for two RS-es. |
| Ericsson | For AWGN 1 sample is fine based on our simulation results. For fading channels 4 samples do improve performance. |
| Nokia | Option 1. Based on our simulation results and the ones from other companies with comparisons between 1 sample and 4 samples TRS, the UE TRS TUE-RX error difference is marginal and, in most cases, especially AWGN channel, the difference is negligible. Therefore, the TRS measurement accuracy can base on 1 sample TRS. |
| vivo | Similar observation as Ericsson is also made from our simulation results. If requirements are specified for AWGN only, option 1 should be used. |
| Qualcomm | Option 2. Same view as Huawei regarding alignment of the requirements. |

Sub topic 1-4: Issue 1-4: Is there a need to introduce scaling factor Kgap when the PDC resources occasions collide with MG?

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1. In our view UE is not supposed to do PDC measurement during MG, so a scaling factor Kgap should be introduced to account for the measurement opportunities punctured by MG. |
| Ericsson | Option 1. |
| Nokia | Option 1 |
| vivo | Our preference is NW guarantees that PDC RS is not collided with MG. If impact of gap is considered, is it based on legacy one gap (or one gap per FR) configuration or would multiple concurrent gaps also be considered? For the legacy gap, it would be feasible for NW to avoid colliding. |
| Qualcomm | Support option 1. We should also address collisions with PPW, although for that several factors may need to be considered (i.e. priority states). Also, PPW may not be applicable for PDC measurements with PRS. |

Sub topic 1-4: Issue 1-5: Is there a need for Rx beam sweeping factor if no QCL information is provided for the RS?

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 2. In our view since PDC is a serving cell measurement, NW should be able to provide QCL information for the PRS TRS. We suggest to define this as an applicability condition for the requirements, and the requirements should be defined without Rx beam sweeping factor. |
| Ericsson | Option 2, No Rx beam sweeping factor is needed if QCL information is provided for the RS, since . Since PDC is only for serving cell, as analyzed in R4-2209235 from Huawei. |
| Nokia | Option 2. |
| vivo | Option 2. QCL information should be provided for PDC-RS. |
| Qualcomm | Agree there would be a need for Rx beam sweeping if QCL information is not provided for the RS. |

Sub topic 1-4: Issue 1-6: Is there a need for scheduling restriction if PDC RS is not QCL-ed with PDCCH or PDSCH?

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1. In our view PDC measurement is similar as L1 measurement. When the PDC-RS is not QCL-ed with PDCCH or PDSCH, there will be scheduling restrictions. |
| Ericsson | Option 1, there is a need for scheduling restriction if PDC RS is not QCL-ed with PDCCH or PDSCH. in R4-2209235 from Huawei: “...We agree, in case the PDC RS is not QCLed with PDCCH or PDSCH, there may be scheduling restrictions. This is also similar as L1-RSRP measurement based on CSI-RS. We suggest to follow the same principle to allow scheduling restriction for this case...” |
| Nokia | Option 1 |
| vivo | It may need further discussion. In general, option 1 is fine. Wouldn’t it also be fine that PDC measurement is dropped if it is not QCL-ed with PDxCH? With 1 sample measurement requirement, dropping would also be fine in our view. |
| Qualcomm | This is a valid question. We would like to keep this open for now and keep discussing into the second round on how to address it. |

Sub topic 1-4: Issue 1-7: Is there a need for defining measurement restriction if PDC RS is not QCL-ed with L1 RS?

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1. In our view PDC measurement is similar as L1 measurement. When the PDC-RS is not QCL-ed with RS for L1 measurement, there will be measurement restrictions because UE can only measure one of them but not both. |
| Ericsson | Option 1, yes (same reason as Issue 1-6). |
| Nokia | Option 1 |
| vivo | Option 1 is fine. Clarification is needed that it is up to UE implementation which RS is measured if PDC RS is not QCL-ed with L1 RS. |
| Qualcomm | Same comment as for issue 1-6. |

Sub topic 1-4: Issue 1-8: Is there a need to define DRX requirements for PDC measurement?

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1. In our view PDC measurement is similar as L1 measurement, for which the measurement period is based on DRX cycle when DRX is in use. |
| Ericsson | Is long DRX cycles compatible with accurate time distribution and services integrated with TSN (Time Sensitive Networks)? For integration with TSN networks then periodic delivery of timing packets are expected and accurate 5GS clock is needed for packet residence time compensation. Accurate 5GS timing would require regular connection to gNB for time and frequency stability. Waking up after DRX is an inaccurate process the Te value alone is much bigger than the whole control-to-control budget of +/- 145-275 ns. For PDC, specifically, it needs to reflect the delay and channel present when the 5GS time was distributed and hence there is a dependency between the 5GS delivery and PDC occasion (when PDC measurement is executed). Furthermore we have a question to Huawei: What is meant with “…PDC measurement which is not time critical. …”. |
| vivo | Prefer to have a study on the impact of measurement delay on PDC accuracy before decision is made on whether DRX is needed. When UE is in higher mobility, then measurement delay cannot be too long. However, if the typical use case is low mobility of stationary, then longer measurement delay would be fine. |
| Qualcomm | Measurement period for RRM is scaled with DRX cycle whereas the same is not true for positioning measurements in connected state. Would PDC have issues with long DRX cycle? Needs more discussion. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2208822, CR to TS 38.133: Correction to measurement requirements for PDC, vivo | Huawei: in general OK but 1. Clause 9.12.4.1, it seems there is no definition of K in clause 9.1 of 38.214. 2. Clause 9.12.4.2, number of samples for TRS Measurement Period is pending on sub-topic 1-3. |
|  | vivo: Reply to Huawei: - In our understanding, K is related UE capability which is under discussion in RAN1. Since clause 9.1 of 38.214 is for PDC, it would be captured in this section if K is defined in RAN1. If there is no UE capability and K is defined in RAN1 spec, then RAN4 needs to refine the definition of Lavailable_PRS. It is fine to keep this open for now and make changes after everything is clear. - Agree number of samples for TRS measurement is pending on sub-topic 1-3. |
|  | Qualcomm: In addition to Huawei’s comments, it may be premature to remove the note in section 9.12.4.2. We’re still discussing several issues regarding the PDC measurement period in this meeting. |
| R4-2209236, CR on requirements for UE Rx-Tx measurement for PDC, Huawei, HiSilicon | Ericsson: Where is Kgap defined? Higher layer? It is not in CR. CR talks about Kp. |
|  | vivo: Depending on sub-topic 1-4. |
|  | Qualcomm: The text about Kp needs to be revised. Several one issues related to this CR are still being discussed. |
| R4-2209615, CR on correction for RTT-based PDC measurement requirements in 38.133, Nokia, Nokia Shanghai Bell | Huawei: in general OK but 1. Clause 9.12.4.1, it seems there is no definition of K in clause 9.1 of 38.214. 2. Clause 9.12.4.1, we suggest to keep the notation as TPRS instead of TperPRS since we only have one PRS periodicity. |
|  | Ericsson: Ericsson, OK, but some changes has to be merged with R4-2208822. |
|  | vivo: - We also don’t see the necessity of changing the notation of TPRS - Part of the changes overlapped with our CR R4-2208822. - Not clear whether this capability below is needed. The gNB side Rx-Tx time difference measurement capability is as indicated by the UE in gNB-SideRTT-BasedPDC-r17, according to TS 38.306. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-2 | Issue 1-2: RAN4 waits for RAN1 conclusion on the applicable capabilities for PRS based PDC? Tentative agreements: Yes Agreement: RAN4 keep the impacted parameters in [] until RAN1 has reached agreement. RAN4 can additionally add an editor’s note: ‘Pending agreement on UE processing capability discussion’ Candidate options: Not needed Recommendations for 2nd round: Issue is closed |

|  | Status summary |
| --- | --- |
| Sub-topic #1-3 | Issue 1-3: The number of samples assumed for deriving the accuracy requirements [1 or 4]? Tentative agreements: No Candidate options: 4 samples: Huawei, Ericsson, Qualcomm 1 sample: Ericsson, vivo, Nokia Moderator comment to facilitate the 2nd round discussion: The measurement accuracy is defined using AWGN channel conditions. However, for defining the number of samples RAN4 also need to consider a non-AWGN channel with fading. The group need to account fading channel characteristics, which are more realistic in actual field conditions. RAN4 should discuss whether it is reliable to rely on single sample results or there is a need to use more than sample result. Moderator suggest to account these aspects in the 2nd round discussion. Recommendations for 2nd round: Continue discussion in 2nd round |

|  | Status summary |
| --- | --- |
| Sub-topic #1-4 | Issue 1-4: Is there a need to introduce scaling factor Kgap when the PDC resources occasions collide with MG? Tentative agreements: Yes Agreement: It has to be discussed and agreed how to handle PDC when measurement gaps are configured. Candidate options: It is moderator understanding that RAN4 do not need discuss how network configures the measurement gap (overlapping or not with PDC), but RAN4 will focus on the scenario where the measurement gap does overlap with PDC and how the related UE requirements are to be defined in such scenarios. Then new additional questions were raised. Which of following scenarios should be considered: - consider legacy gaps only - consider concurrent gaps - consider PPW Moderator suggest the group to initially focus only on 1) collision between PDC and legacy gaps. Once decided the group can then potentially consider additional impact from concurrent gaps and PPW Recommendations for 2nd round: Continue discussion in 2nd round with focus on: Scaling factor Kgap when the PDC resources occasions collide with legacy MG? |

|  | Status summary |
| --- | --- |
| Sub-topic #1-4 | Issue 1-5: Is there a need for Rx beam sweeping factor if no QCL information is provided for the RS? Tentative agreements: Yes Agreement: If no QCL information is provided for the PDC RS there would be need for UE Rx sweep. Candidate options: It is proposed by several companies to define requirements under the condition that QCL information is provided by the network. It is also proposed that RAN4 should only define requirements for the scenario where no Rx sweeping is needed. Moderator propose to continue 2nd round discussion addressing following possible agreement: - if no QCL information is provided it is assumed that UE Rx beam sweeping is needed for PDC measurements - RAN4 only define requirement conditioned: - QCL information is provided - UE Rx sweeping is not needed. Recommendations for 2nd round: Continue discussion with focus on: - if no QCL information is provided it is assumed that UE Rx beam sweeping is needed for PDC measurements - RAN4 only define requirement conditioned: - QCL information is provided - UE Rx sweeping is not needed. |

|  | Status summary |
| --- | --- |
| Sub-topic #1-4 | Issue 1-6: Is there a need for scheduling restriction if PDC RS is not QCL-ed with PDCCH or PDSCH? Tentative agreements: No Candidate options: - There is there a need for scheduling restriction if PDC RS is not QCL-ed with PDCCH or PDSCH (3 companies support). - Further clarifications and suggest keeping this open for 2nd round discussion (2 companies support). It is moderator understanding that it also depends on Issue 1-5. Recommendations for 2nd round: Continue discussion with focus on: - There is there a need for scheduling restriction if PDC RS is not QCL-ed with PDCCH or PDSCH (3 companies support). - Further clarifications and suggest keeping this open for 2nd round discussion (2 companies support). |

|  | Status summary |
| --- | --- |
| Sub-topic #1-4 | Issue 1-7: Is there a need for defining measurement restriction if PDC RS is not QCL-ed with L1 RS? Tentative agreements: No Candidate options: Most companies support option 1: - There a need for defining measurement restriction if PDC RS is not QCL-ed with L1 RS However, one company prefer to keep this open and discuss further. Additionally, a new issue was raised: - Which RS is measured if PDC RS is not QCL-ed with L1 RS Recommendations for 2nd round: Continue discussion on: - There a need for defining measurement restriction if PDC RS is not QCL-ed with L1 RS - Which RS is measured if PDC RS is not QCL-ed with L1 RS |

|  | Status summary |
| --- | --- |
| Sub-topic #1-4 | Issue 1-8: Is there a need to define DRX requirements for PDC measurement? Tentative agreements: No Candidate options: Companies prefer more discussion on this Issue. Continue discussion. Recommendations for 2nd round: Continue discussion regarding: - Is there a need to define DRX requirements for PDC measurement? |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2208822, CR to TS 38.133: Correction to measurement requirements for PDC, vivo | To be revised |
| R4-2209236, CR on requirements for UE Rx-Tx measurement for PDC, Huawei, HiSilicon | Merge to R4-2208822 |
| R4-2209615, CR on correction for RTT-based PDC measurement requirements in 38.133, Nokia, Nokia Shanghai Bell | Merge to R4-2208822 |

## Discussion on 2nd round (if applicable)

# Topic #2: Reference point for Te requirements

Only 1 CR submitted to clarify the UE UL timing requirements with more accurate wording to avoid misinterpretation and confusion.

No open issues raised by any companies. Hence, only need to comment the CR.

Main technical topic overview. The structure can be done based on sub-agenda basis.

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2208055, draftCR to clarify timing reference point for UE UL timing test cases, Intel | Huawei: OK |
|  | Ericsson: we support the CR, which aligns the wording in the test case with the core requirements which were updated in RAN4#102-e. |
|  | Apple: OK. The CR aligns with the agreement on core requirements. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2208055, draftCR to clarify timing reference point for UE UL timing test cases, Intel | Agreeable |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #3: RRM performance requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2208820 | vivo | Proposal 1: No accuracy requirements under fading channel are specified for both UE Rx-Tx time difference measurement based on PRS/TRS and gNB Rx-Tx time difference measurement based on SRS. Proposal 1a: Accuracy requirements under fading channel are specified for both UE Rx-Tx time difference measurement based on PRS/TRS and gNB Rx-Tx time difference measurement based on SRS. Proposal 2: Measurement period requirements for PRS based UE Rx-Tx time difference measurement is TUERxTx,PRS=Nsample *TPRS if UE capability {N, T} is not needed for RTT-based PDC. Proposal 3: Accuracy requirements for RTT-based PDC with TRS are based on measurements with one sample under AWGN channel. |
| R4-2208823 | vivo | Proposal 1: UE Rx-Tx time difference measurement accuracy requirements for RTT-based PDC with PRS are specified based on existing requirements in section 10.1.25.2 under conditions: - For FR1: AWGN, Es/Iot=-3dB, SCS = 15kHz, 30kHz, 60kHz. - For FR2: AWGN, Es/Iot=-3dB, SCS = 60kHz, 120kHz. Proposal 2: UE Rx-Tx time difference measurement accuracy requirements for RTT-based PDC with TRS are specified under conditions: - For FR1: AWGN, Es/Iot=-3dB, SCS = 15kHz, 30kHz, 60kHz. - For FR2: AWGN, Es/Iot=-3dB, SCS = 60kHz, 120kHz. Proposal 3: The same RF group delay calibration error as for PRS based UE Rx-Tx time difference measurement can be reused for TRS based measurement. Proposal 4: Whether to specify UE Rx-Tx time difference measurement accuracy requirements, for both PRS-based and TRS-based, for fading channels following outcome of core part discussion. Proposal 5: Test cases for RTT-based PDC are specified for both FR1 and FR2 under AWGN. Proposal 6: Test case list for measurement period requirements is provided in Table 1. Proposal 7: Test case list for measurement accuracy requirements is provided in Table 2. |
| R4-2208821 | vivo | Observation 1: Measurement accuracy with 4 samples and 1 sample are almost the same under AWGN channel. Observation 2: Measurement accuracy can be improved with 4 samples compared to 1 sample measurement under TDL-A fading channel. |
| R4-2208651 | Ericsson | Simulation results. Collected in Excel |
| R4-2208652 | Ericsson | Observation 1: Existing pos UE Rx-Tx tests should not be modified or added to, in order to include PDC etc. The best is to define new PDC tests in new sections dedicated to PDC but many aspects can be reused. Observation 2: The basic structure of Existing Measurement procedure and Measurement Performance requirements (based on PRS/SRS) for positioning can be reused for TRS/SRS |
| R4-2209237 | Huawei, HiSilicon | Simulation results. Collected in Excel. The simulation are conducted based on agreed simulation assumption in [1], which are copied below for convenience. Some differences: - For fading channel, we are using TDL-C (60 ns delay spread, 300 Hz) as used in PRS requirements. - Since it was agreed in [1] to derive requirements based on 2-slot TRS, we only simulated TRS resource number 4. - Only 4-sample are simulated. Proposal: Take simulation results in Table 1 and Table 2 into account for defining the accuracy requirements for TRS based PDC. |
| R4-2209238 | Huawei, HiSilicon | Proposal 1: Define accuracy requirements for UE and gNB Rx-Tx in AWGN only. Proposal 2: Use same BW groups from PRS to define accuracy for TRS based PDC. Proposal 3: Use same group delay calibration margin from PRS to accuracy for TRS based UE Rx-Tx. Proposal 4: Define the following TCs for PDC. - TC1: PRS based UE Rx-Tx time difference measurement for FR1 in SA - TC2: PRS based UE Rx-Tx time difference measurement for FR2 in SA - TC3: TRS based UE Rx-Tx time difference measurement for FR1 in SA - TC4: TRS based UE Rx-Tx time difference measurement for FR2 in SA Proposal 5: In each TC, include sub-tests for two different PRS/TRS BWs, and verify both delay and accuracy requirements. |
| R4-2209506 | Nokia, Nokia Shanghai Bell | Proposal 1: Consider AWGN channel and the fading channel condition defined RTT based PDC with PRS. Observation 1: It has been agreed to only consider AWGN for the RTT based PDC gNB Rx-Tx time difference measurement requirement. Proposal 2: Define UE Rx-Tx time difference measurement accuracy with TRS in both AWGN and fading channels for FR1 and FR2. Proposal 3: Define UE Rx-Tx time difference measurement accuracy with 1 TRS measurement sample for FR1 and FR2. Observation 2: Given the latest RAN1/2 specification for IIOT_URLLC_enh, the measurement for PDC in TS 38.133 -9.12 should be updated/corrected. Proposal 4: Introduce new SRS type in TS 38.133 A.3.24 for usagePDC-r17 Proposal 5: Define test case for RTT-based PDC UE Rx-Tx time difference measurement period with PRS and TRS. Proposal 6: Define test case for RTT-based PDC UE Rx-Tx time difference measurement accuracy with PRS and TRS. |
| R4-2209519 | Nokia, Nokia Shanghai Bell | Simulation results. Collected in Excel. Observation 1: The TRS measurement sample number has minor effect on the TUE-RX error. Observation 2: The TRS TUE-RX error changes significantly between AWGN fading channels in both FR1 and FR2. |
| R4-2210226 | Qualcomm | Simulation results. Collected in Excel. Observation 1: Simulation results for UE Rx-Tx time difference using TRS are shown in Table 1 and Table 2. The results do not include any margin to account for UE Rx/Tx group delay calibration error. |

Moderator: All simulation results have been collected into an excel sheet and shared in the 230 ftp-folder

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1

Sub-topic description:

UE Rx-Tx time difference measurement requirement for PRS:

Issue 3-1: Define UE Rx-Tx time difference measurement requirement for PRS under fading channel conditions?

- Proposals

  - Option 1: No, RAN4 should not define UE Rx-Tx time difference measurement requirement for PRS under fading channel conditions [vivo, Huawei]

  - Option 2: Yes, RAN4 should define UE Rx-Tx time difference measurement requirement for PRS under fading channel conditions [Nokia]

  - Option 2a: Yes, RAN4 should define UE Rx-Tx time difference measurement requirement for PRS under fading channel conditions if corresponding requirements are specified for gNB Rx-Tx time difference either [vivo]

- Recommended WF

  - More discussion needed

### Sub-topic 3-2

Sub-topic description

gNB Rx-Tx time difference measurement requirement for SRS:

Issue 3-2: Define gNB Rx-Tx time difference measurement requirement for SRS under fading channel conditions?

- Proposals

  - Option 1: No, RAN4 should not define gNB Rx-Tx time difference measurement requirement for SRS under fading channel conditions. [Nokia, Huawei]

  - Option 2: Yes, RAN4 should define gNB Rx-Tx time difference measurement requirement for SRS under fading channel conditions if corresponding requirements are specified for UE Rx-Tx time difference measurement. [vivo]

- Recommended WF

  - More discussion needed

### Sub-topic 3-3

Sub-topic description

UE Rx-Tx time difference measurement requirements for TRS:

Issue 3-3: The channel model used when defining UE Rx-Tx time difference measurement requirements for TRS?

- Proposals

  - Option 1: RAN4 should define UE Rx-Tx time difference measurement requirement for TRS under fading channel conditions. [Nokia]

  - Option 2: RAN4 should define UE Rx-Tx time difference measurement requirement for TRS under AWGN channel conditions. [vivo, Nokia, Huawei]

- Recommended WF

  - More discussion needed

Issue 3-4: Should RAN4 define requirements for UE Rx-Tx time difference measurement requirements for TRS under one one channel model or both channel models (fading and AWGN)?

- Proposals

  - Option 1: RAN4 should define requirements for UE Rx-Tx time difference measurement requirement for TRS under both AWGN and fading channel conditions if corresponding requirements are specified for gNB Rx-Tx time difference measurement either. [vivo]

  - Option 2: RAN4 should define requirements for UE Rx-Tx time difference measurement requirement for TRS only under AWGN channel conditions. [Huawei, vivo]

  - Option 3: RAN4 should define requirements for UE Rx-Tx time difference measurement requirement for TRS under both fading and AWGN channel conditions. [Nokia]

- Recommended WF

  - More discussion needed.

### Sub-topic 3-4

Sub-topic description:

BW groups used for PRS to define accuracy for TRS based PDC.

Issue 3-5: RAN4 to use same BW groups from PRS to define accuracy for TRS based PDC?

- Proposals

  - Option 1: RAN4 to use same BW groups from PRS to define accuracy for TRS based PDC

- Recommended WF

  - Agree on Option 1

### Sub-topic 3-5

Sub-topic description:

Group delay calibration margin from PRS to accuracy for TRS based UE Rx-T.

Issue 3-6: RAN4 to use same group delay calibration margin from PRS to accuracy for TRS based UE Rx-Tx?

- Proposals

  - Option 1: RAN4 to use same group delay calibration margin from PRS to accuracy for TRS based UE Rx-Tx

- Recommended WF

  - Agree on Option 1

### Sub-topic 3-6

Sub-topic description:

New SRS type in TS 38.133 A.3.24 for usagePDC-r17.

Issue 3-7: RAN4 to introduce new SRS type in TS 38.133 A.3.24 for usagePDC-r17?

- Proposals

  - Option 1: RAN4 to introduce new SRS type in TS 38.133 A.3.24 for usagePDC-r17

- Recommended WF

  - Agree on Option 1

### Sub-topic 3-7

Sub-topic description

test cases – discuss directly based on the table

Issue 3-8: Test case list for measurement core requirements

- Proposals

| TC No. | TC | Purpose | Clause | Note |
| --- | --- | --- | --- | --- |
| 1 | UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA | To verify measurement period requirements for UE Rx-Tx time difference measurement with PRS for RTT-based PDC | A.6.6.x1.1 | A.6.6.x1 UE Rx-Tx time difference measurement for RTT-based PDC |
| 2 | UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA | To verify measurement period requirements for UE Rx-Tx time difference measurement with TRS for RTT-based PDC | A.6.6.x1.2 |  |
| 3 | UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR2 SA | To verify measurement period requirements for UE Rx-Tx time difference measurement with PRS for RTT-based PDC | A.7.6.x1.1 | A.7.6.x1 UE Rx-Tx time difference measurement for RTT-based PDC |
| 4 | UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR2 SA | To verify measurement period requirements for UE Rx-Tx time difference measurement with TRS for RTT-based PDC | A.7.6.x1.2 |  |

| A.3.30.W | CSI-RS configurations for RRM | SCS ∈ FR1 set {15,30,60} kHz for FR1 SCS ∈ FR1 set {60,120} kHz for FR2 |
| --- | --- | --- |

In each TC, include sub-tests for two different PRS/TRS BWs, and verify both delay and accuracy requirements.

- Recommended WF

  - Please comment directly on the list in the table if there are any need for changes

Issue 3-9: Test case list for measurement accuracy requirements

- Proposals

| TC No. | TC | Purpose | Clause | Note |
| --- | --- | --- | --- | --- |
| 1 | UE Rx-Tx time difference measurement accuracy with PRS for RTT-based PDC in FR1 SA | To verify measurement accuracy requirements for UE Rx-Tx time difference measurement with PRS for RTT-based PDC | A.6.7.x1.1 | A.6.7.x1 UE Rx-Tx time difference measurement for RTT-based PDC |
| 2 | UE Rx-Tx time difference measurement accuracy with TRS for RTT-based PDC in FR1 SA | To verify measurement accuracy requirements for UE Rx-Tx time difference measurement with TRS for RTT-based PDC | A.6.7.x1.2 |  |
| 3 | UE Rx-Tx time difference measurement accuracy with PRS for RTT-based PDC in FR2 SA | To verify measurement accuracy requirements for UE Rx-Tx time difference measurement with PRS for RTT-based PDC | A.7.7.x1.1 | A.7.7.x1 UE Rx-Tx time difference measurement for RTT-based PDC |
| 4 | UE Rx-Tx time difference measurement accuracy with TRS for RTT-based PDC in FR2 SA | To verify measurement accuracy requirements for UE Rx-Tx time difference measurement with TRS for RTT-based PDC | A.7.7.x1.2 |  |

| A.3.30.W | CSI-RS configurations for RRM | SCS ∈ FR1 set {15,30,60} kHz for FR1 SCS ∈ FR1 set {60,120} kHz for FR2 |
| --- | --- | --- |

In each TC, include sub-tests for two different PRS/TRS BWs, and verify both delay and accuracy requirements.

- Recommended WF

  - Please comment directly on the list in the table if there are any need for changes

## Companies views’ collection for 1st round

### Open issues

Sub topic 3-1: Issue 3-1: Define UE Rx-Tx time difference measurement requirement for PRS under fading channel conditions?

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1 In our view, the typical scenario for PDC is IIOT, where fading channel is not very relevant. Also, the accuracy under fading channel is clearly worse than AWGN, making it more challenging to meet the error budget for PDC. Last, in Rel-16 for positioning there is no corresponding gNB measurement requirements for fading, and we prefer to not re-open the discussion for gNB. |
| Ericsson | Option 2. |
| Nokia | we can accept prioritizing defining the requirement for AWGN channel. |
| vivo | Option 1. Option 2a is also fine if the group agrees to define requirements under fading channel. |
| Qualcomm | Option 1. |

Sub topic 3-2: Issue 3-2: Define gNB Rx-Tx time difference measurement requirement for SRS under fading channel conditions?

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 1. Same reasons as above for 3-1. |
| Ericsson | Option 1 |
| Nokia | Option 1 It has been agreed in RAN4#102e to reuse the current gNB Rx-Tx time difference measurement requirement, which is only based on AWGN channel. |
| vivo | Option 2. Also related to Issue 3-1. |
| Qualcomm | Option 1. |

Sub topic 3-3: Issue 3-3: The channel model used when defining UE Rx-Tx time difference measurement requirements for TRS?

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 2. Same reasons as above for 3-1. |
| Ericsson | Option 1. |
| Nokia | we can accept prioritizing defining the requirement for AWGN channel. |
| vivo | Prefer option 2. Can follow conclusion for PRS. |
| Qualcomm | Option 2 |

Sub topic 3-3: Issue 3-4: Should RAN4 define requirements for UE Rx-Tx time difference measurement requirements for TRS under one one channel model or both channel models (fading and AWGN)?

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Option 2. Same reasons as above for 3-1. |
| Ericsson | Option 3. |
| Nokia | we can accept prioritizing defining the requirement for AWGN channel. |
| vivo | Option 2. Option 1 is also fine if the group agrees to define requirements under fading channel. Can follow conclusion for PRS. |
| Qualcomm | Prefer option 2. |

Sub topic 3-4: Issue 3-5: RAN4 to use same BW groups from PRS to define accuracy for TRS based PDC?

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support the Recommended WF. |
| Ericsson | WF is fine, agree on option 1. |
| Nokia | Support option 1 |
| vivo | Option 1 is supported. |
| Qualcomm | Support the recommended WF. |

Sub topic 3-5: Issue 3-6: RAN4 to use same group delay calibration margin from PRS to accuracy for TRS based UE Rx-Tx?

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support the Recommended WF. |
| Ericsson | WF is fine, agree on option 1. |
| Nokia | Support option 1 |
| vivo | Support the Recommended WF |
| Qualcomm | Support the recommended WF. |

Sub topic 3-6: Issue 3-7: RAN4 to introduce new SRS type in TS 38.133 A.3.24 for usagePDC-r17?

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support the Recommended WF. |
| Ericsson | We are not against developing a new SRS type in TS 38.133 A.3.24 for usagePDC-r17, if needed. |
| Nokia | Support option 1 |
| vivo | Fine with option 1. |
| Qualcomm | Need more time to check this proposal. We’ll comment further in the second round. |

Sub topic 3-7: Issue 3-8: Test case list for measurement core requirements

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Our preference is to use one set of test cases to verify both the delay and accuracy of PDC measurement, i.e. the test cases as listed for issue 3-8 and 3-9 can be merged. The reason is that if we look into the test setup of UE Rx-Tx test cases defined in Rel-16 for positioning, it can be seen that the test conditions are almost the same in delay and accuracy TCs, and in the delay TCs both delay and accuracy are verified according to the test requirements. On the CSI-RS RMC, we agree that we need a new RMC for TRS based PDC. TRS RMC is defined in A.3.17 instead of A.3.30. We suggest to remove 60kHz SCS for both FR1 and FR2 because we did not test 60kHz SCS in any existing test case. We support to include sub-tests for two different PRS/TRS BWs because of different accuracies. |
| Nokia | In principle we support to define TCs for measurement period and measurement accuracy for FR1 and FR2 respectively. Regarding how to group the TCs, it can be further discussed. |
| vivo | As proponent of the test cases list for both measurement delay and accuracy, we are fine to merge the test if it is feasible. Since there are separated test cases for core and accuracy for existing requirements, it needs further discussion whether or not it can be merged for PDC measurement, |
| Qualcomm | We agree with Huawei’s comments that the measurement delay and accuracy may be merged in the same test case. The test configuration would be pretty much the same anyway. And we should reuse most of the test configuration in the test cases for PRS and TRS. |

Sub topic 3-7: Issue 3-9: Test case list for measurement accuracy requirements

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Please see our comments to issue 3-8. |
| Nokia | Find our comments in 3-8. |
| Qualcomm | See issue 3-8. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2208824, Draft CR to TS 38.133: Introuduction of accuracy requirements for PDC, vivo | Huawei: the changes are OK, but some applicability conditions from existing requirements are not included. We may need to discuss whether they are applicable for PDC measurement accuracy. Need to be merged with R4-2209641 |
|  | Ericsson, same comment as Huawei. |
|  | vivo: Applicability conditions can be captured if there is conclusion. |
| R4-2209641, draftCR on UE Rx-Tx time difference measurement accuracy requirements for RTT-based PDC, Nokia, Nokia Shanghai Bell | Huawei: In general OK, but whether to define accuracy requirements for fading depends on the outcome of sub-topic 3-1 and 3-3. Need to be merged with R4-2208824 |
|  | vivo: Applicability conditions needs further discussion on whether or not it is also applicable to PDC measurement. Requirements for fading is under discussion. |
| R4-2209642, draftCR on test cases for RTT-based PDC UE Rx-Tx time difference measurement requirements, Nokia, Nokia Shanghai Bell | Huawei: as commented f or issue 3-8 and 3-9, we suggest to merge the delay and accuracy test cases. Also, in A.6.7.15.2, it seems one test case is defined for both for PRS and TRS based PDC, but we understand we should define separate test cases for them. We suggest to remove the changes related to the test cases in the CR. For this meeting we suggest to focus on the test case list, and we can work on the CRs for the test cases in next meeting, |
|  | Ericsson, In A.6.6.14.4.2 Test requirements there is no rate of correct events specified. Ericsson also prefers to continue work on test procedure CR in next meeting. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Issue 3-1: Define UE Rx-Tx time difference measurement requirement for PRS under fading channel conditions? Tentative agreements: No Candidate options: - 3 companies support option 1. - 2 companies support option 2. (1 company supporting option 2 can compromise prioritizing AWGN channel.) - 1 company support 2a (also supporting option 1).cc Recommendations for 2nd round: Continue the discussion considering following compromise: - At least define UE Rx-Tx time difference measurement requirement for PRS under AWGN channel conditions. - Continue to discuss whether RAN4 additionally should define UE Rx-Tx time difference measurement requirement for PRS under fading channel conditions |

|  | Status summary |
| --- | --- |
| Sub-topic #3-2 | Issue 3-2: Define gNB Rx-Tx time difference measurement requirement for SRS under fading channel conditions? Tentative agreements: Yes Candidate options: - 4 companies support option 1 - 1 company support option 2 It is pointed out that it has been agreed in RAN4#102e to reuse the current gNB Rx-Tx time difference measurement requirement, which is only based on AWGN channel. Based on the RAN4 agreement moderator suggest agreeing on option 1. Agreement: RAN4 should not define gNB Rx-Tx time difference measurement requirement for SRS under fading channel conditions. Recommendations for 2nd round: Issue is closed |

|  | Status summary |
| --- | --- |
| Sub-topic #3-3 | Issue 3-3: The channel model used when defining UE Rx-Tx time difference measurement requirements for TRS? Tentative agreements: No Candidate options: - 3 companies support option 2. - 2 companies support option 1. (1 company can agree to prioritizing AWGN channel conditions.) Recommendations for 2nd round: - At least define UE Rx-Tx time difference measurement requirement for TRS under AWGN channel conditions. - Continue to discuss whether RAN4 additionally should define UE Rx-Tx time difference measurement requirement for TRS under fading channel conditions |

|  | Status summary |
| --- | --- |
| Sub-topic #3-3 | Issue 3-4: Should RAN4 define requirements for UE Rx-Tx time difference measurement requirements for TRS under one channel model or both channel models (fading and AWGN)? Tentative agreements: No Candidate options: - 3 companies support option 2. - 2 companies support option 3. (1 company can agree to prioritizing AWGN channel conditions.) - 1 company support option 1 (support also option 2) Recommendations for 2nd round: - At least define UE Rx-Tx time difference measurement requirement for TRS under AWGN channel conditions. - Continue to discuss whether RAN4 should define UE Rx-Tx time difference measurement requirement for both channel models (fading and AWGN)? |

|  | Status summary |
| --- | --- |
| Sub-topic #3-4 | Issue 3-5: RAN4 to use same BW groups from PRS to define accuracy for TRS based PDC? Tentative agreements: Yes No company raised concerns against the recommended WF. Agreement: - RAN4 will use same BW groups from PRS to define accuracy for TRS based PDC Candidate options: Recommendations for 2nd round: Issue is closed |

|  | Status summary |
| --- | --- |
| Sub-topic #3-5 | Issue 3-6: RAN4 to use same group delay calibration margin from PRS to accuracy for TRS based UE Rx-Tx? Tentative agreements: Yes No company raised concerns against the recommended WF. Agreement: - RAN4 will use same group delay calibration margin from PRS to accuracy for TRS based UE Rx-Tx Candidate options: Recommendations for 2nd round: Issue is closed |

|  | Status summary |
| --- | --- |
| Sub-topic #3-6 | Issue 3-7: RAN4 to introduce new SRS type in TS 38.133 A.3.24 for usagePDC-r17? Tentative agreements: No No company objected Option 1 but one company requested more time to study in 2nd round. Candidate options: - RAN4 to introduce new SRS type in TS 38.133 A.3.24 for usagePDC-r17? Recommendations for 2nd round: Continue discussion from 1st round: - RAN4 to introduce new SRS type in TS 38.133 A.3.24 for usagePDC-r17? |

|  | Status summary |
| --- | --- |
| Sub-topic #3-7 | Issue 3-8: Test case list for measurement core requirements Issue 3-9: Test case list for measurement accuracy requirements Tentative agreements: Yes No companies had concerns regarding the proposal of merging the verification both the delay and accuracy of PDC measurement into same set of test cases. Agreement: - RAN4 agree to use one set of test cases to verify both the delay and accuracy of PDC measurement. Candidate options: - RAN4 need to agree on a list of test cases. Recommendations for 2nd round: Continue discussion based on: TC No.; TC; Purpose; Clause; Note. 1; UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA; To verify measurement period requirements for UE Rx-Tx time difference measurement with PRS for RTT-based PDC; A.6.6.x1.1; A.6.6.x1 UE Rx-Tx time difference measurement for RTT-based PDC. 2; UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA; To verify measurement period requirements for UE Rx-Tx time difference measurement with TRS for RTT-based PDC; A.6.6.x1.2. 3; UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR2 SA; To verify measurement period requirements for UE Rx-Tx time difference measurement with PRS for RTT-based PDC; A.7.6.x1.1; A.7.6.x1 UE Rx-Tx time difference measurement for RTT-based PDC. 4; UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR2 SA; To verify measurement period requirements for UE Rx-Tx time difference measurement with TRS for RTT-based PDC; A.7.6.x1.2. A.3.30.W; CSI-RS configurations for RRM; SCS ∈ FR1 set {15,30,60} kHz for FR1 SCS ∈ FR1 set {60,120} kHz for FR2. Address at least raised issue raised by Huawei: 1) On the CSI-RS RMC, we agree that we need a new RMC for TRS based PDC. TRS RMC is defined in A.3.17 instead of A.3.30? 2) Remove 60kHz SCS for both FR1 and FR2 because 60kHz SCS is not tested in any existing test case? 3) Include sub-tests for two different PRS/TRS BWs because of different accuracies? |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2208824, Draft CR to TS 38.133: Introuduction of accuracy requirements for PDC, vivo | Merged to R4-2209641 |
| R4-2209641, draftCR on UE Rx-Tx time difference measurement accuracy requirements for RTT-based PDC, Nokia, Nokia Shanghai Bell | To be revised |
| R4-2209642, draftCR on test cases for RTT-based PDC UE Rx-Tx time difference measurement requirements, Nokia, Nokia Shanghai Bell | Postponed |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on NR_IIOT_URLLC_enh | Nokia, Nokia shanghai Bell | Capture agreement and further WF |
| Simulation Results for NR_IIOT_URLLC_enh | Nokia, Nokia shanghai Bell | Tdoc for Capturing the simulations results provided the different companies |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2208055 | draftCR to clarify timing reference point for UE UL timing test cases | Intel | Agreeable |  |
| R4-2208822 | CR to TS 38.133: Correction to measurement requirements for PDC | vivo | Revised | Revised and account changes from R4-2209236 and R4-2209615 |
| R4-2209236 | CR on requirements for UE Rx-Tx measurement for PDC | Huawei, HiSilicon | Merged | Merged to R4-2208822 |
| R4-2209615 | CR on correction for RTT-based PDC measurement requirements in 38.133 | Nokia, Nokia Shanghai Bell | Merged | Merged to R4-2208822 |
| R4-2208824 | Draft CR to TS 38.133: Introuduction of accuracy requirements for PDC | vivo | Merged | Merged to R4-2209641 |
| R4-2209641 | draftCR on UE Rx-Tx time difference measurement accuracy requirements for RTT-based PDC | Nokia, Nokia Shanghai Bell | Revised | Revised and account changes from R4-2208824 |
| R4-2209642 | draftCR on test cases for RTT-based PDC UE Rx-Tx time difference measurement requirements | Nokia, Nokia Shanghai Bell | Postponed | Come back in next meeting |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-22xxxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)