3GPP TSG-RAN WG4 Meeting #103-e 													R4-2210326

Electronic Meeting, 9th May 2022 - 20th May 2022

Agenda item:			9.8.4.2

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	Email discussion summary for [103-e][320] NR_HST_FR2_Demod_Part2

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

## Scope

This T-doc will be used to guide and summarize the email discussion for the topic of Rel-17 NR FR2 HST BS Demod requirements (AI 9.8.4.2), with the email thread identifier “[103-e][320] NR_HST_FR2_Demod_Part2”.

The scope of this email discussion are the Rel-17 NR FR2 HST BS Demod requirements, and in particular the agenda items:

9.8.4.2	BS demodulation requirements	[NR_HST_FR2-Perf]

9.8.4.2.1	PUSCH requirements 	 [NR_HST_FR2-Perf]

9.8.4.2.2	PUSCH with UL timing adjustment requirements 	[NR_HST_FR2-Perf]

9.8.4.2.3	PRACH requirements

Priority topics are marked directly in the open issues’ summaries.

## Notes on email discussions

From the previous meeting arrangements:

- Delegates are strongly encouraged to provide comments/concerns asap - Silence within a reasonable timeframe means no objection - It is strongly encouraged that each company/delegate consolidate their comments/views and send them out in one email for each email thread - Length of file names shall be reduced, e.g. - At the beginning of first round, moderators share / ftp / tsg_ran / WG4_Radio / TSGR4_98_e / Inbox / Drafts / [98e][101] NR_NewRAT_SysParameters\Summary_101_1st round_v01.docx - After update by company A: Summary_101_1st round_v02_companyA - After update by company B: Summary_101_1st round_v03_companyA_companyB - After update by company C: Summary_101_1st round_v04_companyB_companyC

# Topic #1: General (9.8.4.2)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-20xxxxx | Company A | Proposal 1: Observation 1: |
| R4-2208075 | Samsung | Title: Simulation results summary for Rel-17 FR2 HST BS demod Moderator: Simulation summary, only reserved. |
| R4-2208078 | Samsung | Title: Big CR on FR2 HST BS demodulation requirement for TS 38.104 Moderator: BigCR, only reserved. |
| R4-2209870 | Huawei, HiSilicon | Title: Draft CR on HST FR2 BS applicability rule (38.141-2, Rel-17) Moderator: This CR is limited to PUSCH related applicability rules, as such it will be treated in the second topic (PUSCH/9.8.4.2.1) in the section 2.2.4. |
| R4-2210148 | Nokia, Nokia Shanghai Bell | Title: Big CR to TS 38.141-2 on HST FR2 BS Demodulation Performance Requirements Moderator: BigCR, only reserved. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Interested companies are expected to add their views directly under the respective issues in a dialogue-like form, i.e., identical to how the chair would record views during a f2f meeting.

Please add further table rows as required and do not change previous comments of your company or other companies. Answering to questions from other companies is encouraged.

No non-CR contributions submitted to the “general” category.
Though, we will use this section to capture the previously agreed CR split for reference:

| Section number | Section title | Responsible company |
| --- | --- | --- |
| TS 38.104 |  |  |
|  | Big CR | Samsung |
| 11 | Radiated performance requirements |  |
| 11.2 | Performance requirements for PUSCH |  |
| 11.2.2 | Requirements for BS type 2-O |  |
| 11.2.2.x | Requirements for PUSCH for high speed train | Intel |
| 11.2.2.y | Requirements for UL timing adjustment | CATT |
| 11.4 | Performance requirements for PRACH |  |
| 11.4.2 | Requirements for BS type 2-O |  |
| 11.4.2.2 | PRACH detection requirements |  |
| 11.4.2.2.x | Minimum requirements for high speed train | Huawei |
| Annex A | Reference measurement channels | Intel |
| Annex G.3 | High speed train condition | Nokia |
| Annex G.4 | Moving propagation conditions | CATT |
| TS 38.141-2 |  |  |
|  | Big CR | Nokia |
| 4.6 | Manufacturer's declarations | Samsung, Nokia |
| 8 | Radiated performance requirements |  |
| 8.1.2 | Applicability rule |  |
| 8.1.2.4 | Applicability of PUSCH for high speed train performance requirements | Huawei |
| 8.2 | OTA performance requirements for PUSCH |  |
| 8.2.4 | Performance requirements for PUSCH for high speed train | Ericsson, Samsung |
| 8.2.5 | Performance requirements for UL timing adjustment | CATT |
| 8.4 | OTA performance requirements for PRACH |  |
| 8.4.1 | PRACH false alarm probability and missed detection |  |
| 8.4.1.6 | Test requirement for high speed train | Huawei |
| Annex A | Reference measurement channels | Intel |
| Annex E | OTA measurement system set-up | Ericsson |
| Annex J.3 | High speed train condition | Nokia |
| Annex J.4 | Moving propagation conditions | CATT |

### Sub-topic 1-1: Other

Sub-topic description:

In this sub-topic companies are invited to bring issues to the attention of the group, which have not been captured in the previous sub-topics.

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Title, Source |
|  | Company A |
|  | Company B |
| R4-2208078 | Big CR on FR2 HST BS demodulation requirement for TS 38.104, Samsung |
|  | Moderator: BigCR, only reserved. For email approval. |
|  | Moderator: Please continue discussion in second round discussion section! |
| R4-2210148 | Big CR to TS 38.141-2 on HST FR2 BS Demodulation Performance Requirements, Nokia, Nokia Shanghai Bell |
|  | Moderator: BigCR, only reserved. For email approval. |
|  | Moderator: Please continue discussion in second round discussion section! |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic 1-1 | Sub-topic 1-1: Other No further issues have been brought to the attention of the group. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |
| #1 | WF on BS demodulation requirement for FR2 HST | Nokia, Nokia Shanghai Bell |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
|  | None. |

## Discussion on 2nd round

### Sub-topic 1-1: Other

Sub-topic description:

In this sub-topic companies are invited to bring issues to the attention of the group, which have not been captured in the previous sub-topics.

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Title, Source |
|  | Company A |
|  | Company B |
| R4-2208078 | Big CR on FR2 HST BS demodulation requirement for TS 38.104, Samsung |
|  | Moderator: BigCR, only reserved. For email approval. |
| R4-2210148 | Big CR to TS 38.141-2 on HST FR2 BS Demodulation Performance Requirements, Nokia, Nokia Shanghai Bell |
|  | Moderator: BigCR, only reserved. For email approval. |

## Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: PUSCH (9.8.4.2.1)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-20xxxxx | Company A | Proposal 1: Observation 1: |
| R4-2207907 | Nokia, Nokia Shanghai Bell | Title: draftCR to TS 38.104 on HST FR2 Channel Conditions Moderator: draft CR, no proposals/observations. |
| R4-2207908 | Nokia, Nokia Shanghai Bell | Title: draftCR to TS 38.141-2 on HST FR2 Channel Conditions Moderator: draft CR, no proposals/observations. |
| R4-2207984 | Ericsson | Title: PUSCH simulation results Moderator: Simulation results, no proposals/observations. |
| R4-2207985 | Ericsson, Samsung | Title: Draft CR to 38.141-2: Inttroduction of HST PUSCH requirements Moderator: draft CR, no proposals/observations. |
| R4-2208077 | Samsung | Title: Discussion and simulation results of PUSCH requirement for Rel-17 FR2 HST Proposal 1: The ideal result alignment threshold as [3.0] dB for requirement derivation can be considered, in case the results has still large gap existed. |
| R4-2208080 | Samsung | Title: Draft CR on BS Manufacturer declaration for FR2 HST for TS 38.141-2 Moderator: draft CR, no proposals/observations. |
| R4-2208223 | CATT | Title: Simulation results for PUSCH demodulation requirements for FR2 HST Moderator: Simulation results, no proposals/observations. |
| R4-2209865 | Huawei, HiSilicon | Title: Simulation results on PUSCH demodulation requirements for FR2 HST Moderator: Simulation results, no proposals/observations. |
| R4-2210120 | Nokia, Nokia Shanghai Bell | Title: On HST FR2 PUSCH Requirements On PUSCH requirement selection: Observation 1: More impairment results are still needed to make the final concussion on the PUSHC demodulation requirements in HST FR2 scenarios. Based on the exiting ideal PUSCH simulations results reported by the companies, the ideal span for MCS19 can be in the alignment threshold of 2.5 dB if only one outlier is excluded. After that, there is still enough results from more than three companies that are aligned. Proposal 1: RAN4 to follow standard requirement selection procedure to (post-FFT) PUSCH results at MCS19 with the ideal result alignment threshold as at most 2.5 dB and impairment threshold as 4dB. On HST manufacturer declaration: Observation 2: There is already existing Manufacturer declaration D.109 for High Speed train defined in Rel-16 for FR1 that can be confused with a new declaration for HST FR2. Proposal 2: RAN4 to change the title of exiting high speed train manufacture declaration in TS 38.141-1 and TS 38.141-2 to reference FR1, e.g., FR1 high speed train. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Interested companies are expected to add their views directly under the respective issues in a dialogue-like form, i.e., identical to how the chair would record views during a f2f meeting.

Please add further table rows as required and do not change previous comments of your company or other companies. Answering to questions from other companies is encouraged.

### Sub-topic 2-1: Requirements selection

Sub-topic description:

In the last meeting the following GtW agreement was reached:

Issue 2-3-1: Requirement selection

Apply standard requirement selection to (post-FFT) results with outlier selection, as in Rel-15 [R4-1904713] [R4-19004714]. Choose ideal result alignment threshold as [2.5dB], and impairment threshold as [4dB].

It’s encouraged that companies can further update their results in future meetings to specify final performance requirements.

Two proposals have been received on the [] numbers

Open issues and candidate options before e-meeting:

Issue 2-1-1: Requirement selection procedure

- Previous agreements (RAN4#102e)

  - Apply standard requirement selection to (post-FFT) results with outlier selection, as in Rel-15 [R4-1904713] [R4-19004714].

  - MCS 19.

- Proposals

  - Option 1 (Samsung): Ideal result alignment threshold as [3.0] dB.

  - Option 2 (Nokia): Ideal result alignment threshold as at most 2.5 dB and impairment threshold as 4dB.

  - Option 3 (Moderator): Ideal threshold = 2.5dB. Impaired threshold = 4dB.

  - Other options not precluded.

- Recommended WF

  - Please try to find a compromise in the first few days of the discussion, so we can adapt draftCRs and close the SNR number topics in the second round.

  - Can the compromise of option 3 be acceptable to all?

| Company | Comments |
| --- | --- |
| Nokia | After the preliminary check of the PUSCH simulation results submitted to this and previous meetings, only 50MHz DMRS 1+0 and DMRS 1+1 results ideal span is above 3dB. However, if one outlier is excluded, then the results are well aligned with the span well below 2.5dB. The span of impaired results is far below the threshold. Thus, we believe that the standard alignment procedure can be used, and there is no need to consider threshold above 2.5dB. Option 3 by Moderator is OK. |
| Ericsson | We also prefer option 3; the results now seem reasonably converged |
| Samsung | We have updated the latest simulation summary, encourage companies to further check whether the results are captured correctly. Regarding the requirement definition, As checking with companies, we are ok with option 3 with [2.5]dB |
| CATT | We also prefer Option 3. |
| Huawei | We are OK with Option 3 proposed by operator to finalize this issue, with the ideal span is at most 2.5dB and follow the standard alignment procedure to derive the final SNR value for performance requirements definition. |

### Sub-topic 2-2: Manufacturer’s declarations and applicability rules

Sub-topic description:

There are existing manufacturer declarations (D.109) for High-Speed train, that were define in Rel-16 NR_HST and are applicable to FR1, albeit that those declarations do not mention this specifically in 38.141-2.
Hence, there is some potential confusion with a new declaration for HST FR2.

[38.141-2]:

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.109 | High speed train | Declaration of high speed train scenario support, i.e. HST support or no HST support | c | x | n/a |

The same it true for the applicability rules [38.141-2]:

8.1.2.4	Applicability of PUSCH for high speed train performance requirements

##### 8.1.2.4.1	Appliability of requirements for different speeds

Unless otherwise stated, a BS that declares to support 500km/h (see D.110 in table 4.6-1) and passes the tests for 500km/h, can also consider the tests for 350km/h as passed.

However, the text to be clarified was introduced and is only used by Rel-16 NR_HST. 
Usually, such changes should go thus through TEI16/NR_HST.
Please comment, if such changes are acceptable and/or if it is appropriate to be endorsed such changes in NR_HST_FR2.

Open issues and candidate options before e-meeting:

Issue 2-2-1: Clarify Rel-16 manufacturer’s declarations and applicability rules to explicitly apply to FR1

- Proposals

  - Option 1 (Samsung, Nokia): Yes, for manufacturer’s declarations.

  - Option 2 (Huawei): Yes, for applicability rules.

  - Option 3: Not needed.

- Recommended WF

  - Please comment early in first round.

| Company | Comments |
| --- | --- |
| Nokia | We think that a clearer split in between HST FR1 and FR2 both in manufacturer declaration and applicability rules will be beneficial. Thus, we support both Option 1 and Option2. However, there is no need to change Rel-16 specifications as such, because HST FR2 is not present there. All the changes can be kept in Rel-17 TSs only. Additionally, we see a need to clarify an applicability rule 8.1.2.1.6 Applicability of UL timing adjustment requirements for different scenarios in TS 38.141-2 that takes into account only HST FR1. For HST FR2, only scenario Y needs to be tested when the corresponding declaration is supported. |
| Ericsson | We are OK with both option 1 and option 2 |
| Samsung | We are ok with option 1 and option 2 |
| CATT | We are OK with both option 1 and option 2. |
| Huawei | We think that the existing manufacture declaration D.109 can be reused for FR2 HST by updating the applicability for BS type 2-O from n/a to x to make it also applicable for FR2 from Rel-17, we did not observe the necessity to define additional manufacture declaration for support FR2 HST. Only 350km/h velocity is supported for FR2 HST in Rel-17, different applicability rule should be specified. |

Issue 2-2-2: Implement Issue 2-2-1 in NR_HST or NR_HST_FR2

- Proposals

  - Option 1 (Samsung, Nokia): NR_HST/TEI16

  - Option 2 (Huawei): NR_HST_FR2

- Recommended WF

  - Please comment early in first round.

| Company | Comments |
| --- | --- |
| Nokia | In general, specification changes to HST FR1 requirements shall not be made in HST FR2 WI. However, if the changes do not impact the meaning of former definitions, are kept in Rel-17 specifications, and are not objected by the companies in this discussion, we think that such changes are acceptable to avoid unnecessary overhead. Follow the rule of no changes in HST FR1 can be easily followed for Manufacturer declarations but is harder for other clauses. Thus, Option 2 is also acceptable to us. |
| Ericsson | Whichever WI code that can work is OK. Since the change is discussed under the FR2 HST item and the change to FR1 is editorial, probably the HST_FR2 WID code is OK. But no strong view. |
| Samsung | This change is to distinguish FR1 and FR2, since change for FR1 is editorial due to FR2 HST introduced, we are ok to up them, both option 1 and option 2 are acceptable, if go option 2, we probably need to ask one tdoc for 38.141-1? |
| CATT | Both options are acceptable and share the similar view with Ericsson and Samsung. |
| Huawei | We prefer Option 2 since the change is only for clarification the difference between HST FR1 and HST FR2. |

### Sub-topic 2-3: Other

Sub-topic description:

In this sub-topic companies are invited to bring issues to the attention of the group, which have not been captured in the previous sub-topics.

| Company | Comments |
| --- | --- |
| Nokia | We would like to introduce a common logic in the definition of HST FR2 requirements and the way how these requirements are distinguished from HST FR1 requirements. In TS 38.141-2, the common practice to distinguish FR1 and FR2 is to reference the BS type, i.e., either type 1-O or type 2-O. We propose to follow the same rule in Applicability rules and Requirements itself. |
| Ericsson | OK with the Nokia proposal above |
| CATT | Fine with Nokia proposal above. |
| Huawei | We are OK with Nokia’s proposal to use BS type 1-O and BS type 2-O to distinguish FR1 and FR2 in TS 38.141-2. |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Title, Source |
|  | Company A |
|  | Company B |
| R4-2209870 | Draft CR on HST FR2 BS applicability rule (38.141-2, Rel-17), Huawei |
|  | Moderator: This CR is limited to PUSCH related applicability rules, as such it will be treated in the second topic (PUSCH/9.8.4.2.1) in the section 2.2.4. However, this CR proposes change to text introduced and only used by Rel-16 NR_HST. Usually, such changes should go through TEI16/NR_HST. Please comment, if change1 is acceptable and/or if it is appropriate to be endorsed in NR_HST_FR2. |
|  | Nokia: - As we commented in the Sub-topic 2-3 above, we do not think that usage of FR1/FR2 is the usual way to distinguish requirements in TS 38.141-2. For example, in 8.1.2.1.3 Applicability of requirements for different configurations, BS type 1-O and BS type 2-O applicability rules are together. For example, “Unless otherwise stated, a BS type 1-O that declares to support 500km/h (see D.110 in table 4.6-1) and…” Therefore, we propose not to introduce a new clause for HST FR2 and keep 8.1.2.4 only. The new clause for Applicability of requirements for different DMRS configurations clause should be introduced though for type 2-O BSs only. - It was agreed to introduce PUSCH requirements for 50MHz and 200 MHz CBW. However, other CBWs, e.g., 100 MHz might be present in the deployments. As far as a separate clause is defined for high speed PUSCH performance requirements, it will be necessary to add Applicability of requirements for different channel bandwidth under 8.1.2.4 similarly to 8.1.2.1.2. Additionally, 8.1.2.1.6 Applicability of UL timing adjustment requirements for different scenarios in TS 38.141-2 that takes into account only HST FR1. This applicability rule should be extended to cover the UL timing adjustment requirements in HST FR2 (BS type 2-O) with corresponding declaration. |
|  | Moderator: Please continue discussion in second round discussion section! |
| R4-2207907 | draftCR to TS 38.104 on HST FR2 Channel Conditions, Nokia, Nokia Shanghai Bell |
|  | Nokia: We proposed to rename propagation condition from HST Scenario 1-NR500 to HST Scenario 1-NR500, FR1. However, we noticed, that this will require changes in all Test requirements where these propagation conditions are referenced. Thus, we prefer to use the old names. Scenario B-NR350, FR2 can be still used though. |
|  | Ericsson: Maybe Scenario B could be renamed to Scenario 4 for consistency in the table. No strong view though. |
|  | Samsung: regarding with name, we think we need to align the channel name referred with UE side, meanwhile, the deployment should be included it, as agreed as Define HST FR2 model based on Bi-directional scenario-B model. Scenario B-BI-NR350, FR2 : Open space |
|  | Huawei: RAN4 agreed to use the following channel Model names for FR2 HST in TS 38.101-4 for UE demodulation requirements o Bi-directional scenario B : HST-DPS-FR2-BI-B o Uni-directional scenario A: HST-DPS-FR2-UNI-A To easily differentiate the channel model for FR1 HST and FR2 HST, it is better to include word FR2, such as FR2-BI-B-NR350 |
|  | Moderator: Please continue discussion in second round discussion section! |
| R4-2207908 | draftCR to TS 38.141-2 on HST FR2 Channel Conditions, Nokia, Nokia Shanghai Bell |
|  | Nokia: The same comment as above. |
|  | Ericsson: Maybe Scenario B could be renamed to Scenario 4 for consistency in the table. No strong view though. |
|  | Samsung: the same comment as above |
|  | Huawei: same comments as above |
|  | Moderator: Please continue discussion in second round discussion section! |
| R4-2207985 | Draft CR to 38.141-2: Inttroduction of HST PUSCH requirements, Ericsson, Samsung |
|  | Nokia: - In Table 8.2.4.6-1/2 Additional DM-RS position: it is better to change Pos1 to pos1 for consistency. - The requirements can be adjusted with the references to propagations conditions (Scenario B-NR350, FR2) and FRCs. - The PUSCH requirements are agreed to be the same both for DM-RS pos1 and pos2 (i.e., based on pos1). Thus, relevant FRCs and DM-RSs should be listed, e.g., pos1, pos2, in the same requirement. - Following TS 38.141-2 logics, it is OK to introduce different sections for HST FR1 and FR2, but BS type 1-O and type 2-O should be used instead for FR1 and FR2. - Caluse - > Clause |
|  | Moderator: Please continue discussion in second round discussion section! |
| R4-2208080 | Draft CR on BS Manufacturer declaration for FR2 HST for TS 38.141-2, Samsung |
|  | Nokia: Conditioned on the opinions of the other companies it should be also OK for us to clarify D.109 declaration to include reference to FR1 only in Rel-17 TS. |
|  | Huawei: In essential, D.109 and D.117(new) are same; D.101 and D.118(new) are same. We did not observe the any difference between them. |
|  | Moderator: Please continue discussion in second round discussion section! |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic 2-1 | Sub-topic 2-1: Requirements selection Issue 2-1-1: Requirement selection procedure Tentative agreements: Ideal threshold = 2.5dB. Impaired threshold = 4dB. Candidate options: None. Recommendations for 2nd round: Tentative agreement seems agreeable. |
| Sub-topic 2-2 | Sub-topic 2-2: Manufacturer’s declarations and applicability rules Issue 2-2-1: Clarify Rel-16 manufacturer’s declarations and applicability rules to explicitly apply to FR1 Tentative agreements: Yes, for applicability rules. Candidate options: - Option 1: Yes, for manufacturer’s declarations. - Option 4: Existing manufacture declaration D.109 can be reused for FR2 HST. - Option 5 (Moderator): Existing manufacture declaration D.109 shall be modified, from Rel-17 on, to declare support for FR1 and/or FR2 HST. Recommendations for 2nd round: It is the moderator’s understanding that option 4 means that, starting from Rel-17, declaration to support HST requires to support both FR1 and FR2. I.e., No possibility to declare HST support independently for both FRs. Unless the text of the declaration is also changed to explicitly allow this. Please clarify and discuss in the second round. The moderator has combined option 1 and 4 in option 5, which should allow for declaration of support in FR1 and FR2, while keeping the existing manufacturer declaration, which is already referenced in D.110, D.111 (TS 38.141-2 Table 4.6-1). The distinction for Rel-16/Rel-17 in applicability rules (e.g., for max velocity) seems agreeable for all. Please discuss the wording in the corresponding draftCR (revision of R4-2209870). Issue 2-2-2: Implement Issue 2-2-1 in NR_HST or NR_HST_FR2 Tentative agreements: Use NR_HST_FR2 to implement any required FR1/FR2 distinctions starting from Rel-17. Candidate options: - Option 1: Replicate FR1 clarification in 38.141-1. - Option 2: Do not add FR1 clarification in 38.141-1. Recommendations for 2nd round: Tentative agreement seems agreeable. The question has come up if we need a draftCR for 38.141-1 to also add “FR1” in this specification (at least applicability rules). It is the moderator’s understanding that 38.141-1 would remain valid and well defined also without this clarification, but text coherence between -1 and -2 might be an advantage. Please comment in the second round. |
| Sub-topic 2-3 | Sub-topic 2-3: Other Issue 2-3-1: Terminology to use for FR1 and FR2 distinction Tentative agreements: Use either BS type 1-O or type 2-O to distinguish FR1 and FR2 in applicability rules and requirements. Candidate options: None Recommendations for 2nd round: Tentative agreement seems agreeable. Issue 2-3-2: Treatment of additional new draftCRs Recommendations for 2nd round: There was an offline request to obtain new tdoc numbers to upload and treat the following draftCRs in the second round: DraftCR to TS 38.104: FRC for HST FR2 PUSCH performance requirements, Intel Corporation DraftCR to TS 38.104: HST FR2 PUSCH performance requirements, Intel Corporation DraftCR to TS 38.141-2: FRC for HST FR2 PUSCH performance requirements, Intel Corporation After checking with co-chair, “It's Ok to have any t-doc request if it's helpful to proceed the work based on the consensus from companies. Please include the request into 1st round summaries. And whether these new draftCRs for information only or for endorsement, it's pending on all the experts' discussion.”. Hence, the above draftCRs have been requested and added CR discussion. If late addition is not acceptable to a contributor please propose to postpone or change to “for information”, in the CR discussion of the second round. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |
|  | None. |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2209870 | Revised |
| R4-2207907 | Revised |
| R4-2207908 | Revised |
| R4-2207985 | Revised |
| R4-2208080 | Revised |
| TBA | New Tdoc (see section 5) |
| TBA | New Tdoc (see section 5) |
| TBA | New Tdoc (see section 5) |

## Discussion on 2nd round

### Sub-topic 2-1: Requirements selection

No open issues after first round.

### Sub-topic 2-2: Manufacturer’s declarations and applicability rules

Issue 2-2-1: Clarify Rel-16 manufacturer’s declarations and applicability rules to explicitly apply to FR1

- Agreement in first round (informative repetition)

  - Yes, for applicability rules.

- Proposals

  - Option 1: Yes, for manufacturer’s declarations.

  - Option 4: Existing manufacture declaration D.109 can be reused for FR2 HST.

  - Option 5 (Moderator): Existing manufacture declaration D.109 shall be modified, from Rel-17 on, to declare support for FR1 and/or FR2 HST.

- Recommended WF

  - It is the moderator’s understanding that option 4 means that, starting from Rel-17, declaration to support HST requires to support both FR1 and FR2. I.e., No possibility to declare HST support independently for both FRs. Unless the text of the declaration is also changed to explicitly allow this.

  - Please clarify and discuss in the second round.

  - The moderator has combined option 1 and 4 in option 5, which should allow for declaration of support in FR1 and FR2, while keeping the existing manufacturer declaration, which is already referenced in D.110, D.111 (TS 38.141-2 Table 4.6-1).

| Company | Comments |
| --- | --- |

Issue 2-2-2: Implement Issue 2-2-1 in NR_HST or NR_HST_FR2

- Agreement in first round (informative repetition)

  - Use NR_HST_FR2 to implement any required FR1/FR2 distinctions starting from Rel-17.

- Proposals

  - Option 1: Replicate FR1 clarification in 38.141-1.

  - Option 2: Do not add FR1 clarification in 38.141-1.

- Recommended WF

  - The question has come up if we need a draftCR for 38.141-1 to also add “FR1” in this specification (at least applicability rules).

  - It is the moderator’s understanding that 38.141-1 would remain valid and well defined also without this clarification, but text coherence between -1 and -2 might be an advantage.

  - Please comment in the second round..

| Company | Comments |
| --- | --- |

### Sub-topic 2-3: Other

Sub-topic description:

In this sub-topic companies are invited to bring issues to the attention of the group, which have not been captured in the previous sub-topics.

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Title, Source |
|  | Company A |
|  | Company B |
| revisedR4-2209870 | Draft CR on HST FR2 BS applicability rule (38.141-2, Rel-17), Huawei |
|  | Moderator: This CR is limited to PUSCH related applicability rules, as such it will be treated in the second topic (PUSCH/9.8.4.2.1) in the section 2.2.4. However, this CR proposes change to text introduced and only used by Rel-16 NR_HST. Usually, such changes should go through TEI16/NR_HST. Please comment, if change1 is acceptable and/or if it is appropriate to be endorsed in NR_HST_FR2. |
|  | Nokia: - As we commented in the Sub-topic 2-3 above, we do not think that usage of FR1/FR2 is the usual way to distinguish requirements in TS 38.141-2. For example, in 8.1.2.1.3 Applicability of requirements for different configurations, BS type 1-O and BS type 2-O applicability rules are together. For example, “Unless otherwise stated, a BS type 1-O that declares to support 500km/h (see D.110 in table 4.6-1) and…” Therefore, we propose not to introduce a new clause for HST FR2 and keep 8.1.2.4 only. The new clause for Applicability of requirements for different DMRS configurations clause should be introduced though for type 2-O BSs only. - It was agreed to introduce PUSCH requirements for 50MHz and 200 MHz CBW. However, other CBWs, e.g., 100 MHz might be present in the deployments. As far as a separate clause is defined for high speed PUSCH performance requirements, it will be necessary to add Applicability of requirements for different channel bandwidth under 8.1.2.4 similarly to 8.1.2.1.2. Additionally, 8.1.2.1.6 Applicability of UL timing adjustment requirements for different scenarios in TS 38.141-2 that takes into account only HST FR1. This applicability rule should be extended to cover the UL timing adjustment requirements in HST FR2 (BS type 2-O) with corresponding declaration. |
| revisedR4-2207907 | draftCR to TS 38.104 on HST FR2 Channel Conditions, Nokia, Nokia Shanghai Bell |
|  | Nokia: We proposed to rename propagation condition from HST Scenario 1-NR500 to HST Scenario 1-NR500, FR1. However, we noticed, that this will require changes in all Test requirements where these propagation conditions are referenced. Thus, we prefer to use the old names. Scenario B-NR350, FR2 can be still used though. |
|  | Ericsson: Maybe Scenario B could be renamed to Scenario 4 for consistency in the table. No strong view though. |
|  | Samsung: regarding with name, we think we need to align the channel name referred with UE side, meanwhile, the deployment should be included it, as agreed as Define HST FR2 model based on Bi-directional scenario-B model. Scenario B-BI-NR350, FR2 : Open space |
|  | Huawei: RAN4 agreed to use the following channel Model names for FR2 HST in TS 38.101-4 for UE demodulation requirements o Bi-directional scenario B : HST-DPS-FR2-BI-B o Uni-directional scenario A: HST-DPS-FR2-UNI-A To easily differentiate the channel model for FR1 HST and FR2 HST, it is better to include word FR2, such as FR2-BI-B-NR350 |
| revisedR4-2207908 | draftCR to TS 38.141-2 on HST FR2 Channel Conditions, Nokia, Nokia Shanghai Bell |
|  | Nokia: The same comment as above. |
|  | Ericsson: Maybe Scenario B could be renamed to Scenario 4 for consistency in the table. No strong view though. |
|  | Samsung: the same comment as above |
|  | Huawei: same comments as above |
| revisedR4-2207985 | Draft CR to 38.141-2: Inttroduction of HST PUSCH requirements, Ericsson, Samsung |
|  | Nokia: - In Table 8.2.4.6-1/2 Additional DM-RS position: it is better to change Pos1 to pos1 for consistency. - The requirements can be adjusted with the references to propagations conditions (Scenario B-NR350, FR2) and FRCs. - The PUSCH requirements are agreed to be the same both for DM-RS pos1 and pos2 (i.e., based on pos1). Thus, relevant FRCs and DM-RSs should be listed, e.g., pos1, pos2, in the same requirement. - Following TS 38.141-2 logics, it is OK to introduce different sections for HST FR1 and FR2, but BS type 1-O and type 2-O should be used instead for FR1 and FR2. - Caluse - > Clause |
| revisedR4-2208080 | Draft CR on BS Manufacturer declaration for FR2 HST for TS 38.141-2, Samsung |
|  | Nokia: Conditioned on the opinions of the other companies it should be also OK for us to clarify D.109 declaration to include reference to FR1 only in Rel-17 TS. |
|  | Huawei: In essential, D.109 and D.117(new) are same; D.101 and D.118(new) are same. We did not observe the any difference between them. |
| TBA | DraftCR to TS 38.104: HST FR2 PUSCH performance requirements, Intel Corporation |
| TBA | DraftCR to TS 38.104: FRC for HST FR2 PUSCH performance requirements, Intel Corporation |
| TBA | DraftCR to TS 38.141-2: FRC for HST FR2 PUSCH performance requirements, Intel Corporation |

## Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: PUSCH with UL timing adjustment requirements (9.8.4.2.2)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-20xxxxx | Company A | Proposal 1: Observation 1: |
| R4-2208195 | CATT | Title: Draft CR for TS 38.104, Introduce performance requirements for UL TA Moderator: draft CR, no proposals/observations. |
| R4-2208224 | CATT | Title: Discussion on UL TA demodulation requirements for FR2 HST Observation 1: SRS resource allocation for FDD is not needed. Proposal 1: To configure SRS transmission comb and SRS periodic for FR HST as KTC = 2 and TSRS = 10 respectively. |
| R4-2208225 | CATT | Title: Draft CR for TS 38.141-2, Introduce performance requirements for UL TA Moderator: draft CR, no proposals/observations. |
| R4-2209866 | Huawei, HiSilicon | Title: Simulation results on PUSCH with UL timing adjustment requirements for FR2 HST Moderator: Simulation results, no proposals/observations. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Interested companies are expected to add their views directly under the respective issues in a dialogue-like form, i.e., identical to how the chair would record views during a f2f meeting.

Please add further table rows as required and do not change previous comments of your company or other companies. Answering to questions from other companies is encouraged.

### Sub-topic 3-1: SRS configuration

Sub-topic description:

Concerning the SRS configuration, the following prior agreements have been made (among others)

- Prior agreements [R4-2120703]

  - GTW agreement on channel model for UL timing adjustment requirement:

    - Follow same approach as existing PUSCH UL timing adjustment requirement demodulation requirement specified in TS 38.104

  - On test parameters for timing offset:

    - Use A=1.25us, Δw= 1.04s-1 corresponding to 120KHz SCS for HST FR2 UL timing adjustment requirements

  - On SRS transmission configuration:

    - The SRS transmission location is the last symbol in slot#3 in radio frame with TDD pattern as DDDSU, S=10:2:2

  - Simulation assumption for UL timing adjustment requirement

    - SRS resource allocation

      - 50MHz CBW (32RBs)~C_SRS =9, B_SRS =0

      - 200MHz CBW (132RBs)~ C_SRS=33, B_SRS=0

- Prior agreements [R4-2108637]:

  - SRS Transmission comb: KTC=2

  - SRS Transmission periodicity : KSRS=10

  - Slots in which sounding RS is transmitted

    - The last symbol in slot#3 in radio frames for 120KHz SCS

It is the understanding of the moderator that only 120kHz requirements for FR2 HST UL TA have been agreed.

Open issues and candidate options before e-meeting:

Issue 3-1-1: SRS Transmission comb

- Proposals

  - Option 1 (CATT): KTC=2

- Recommended WF

  - Option 1 is aligned with agreement in [R4-2108637].
It does not seem necessary to the moderator to confirm this prior agreement again.

| Company | Comments |
| --- | --- |
| Nokia | Agree with the WF by the Moderator. |
| Ericsson | OK with WF |
| Samsung | Ok with WF |
| CATT | Agree with the recommended WF. |
| Huawei | We are OK with the recommended WF. |

Issue 3-1-2: SRS Transmission periodicity

- Proposals

  - Option 1 (CATT): 10 slots.

- Recommended WF

  - Option 1 is aligned with agreement in [R4-2108637].
It does not seem necessary to the moderator to confirm this prior agreement again.

  - However, the wording of “slot#3 in radio frame” in the prior agreement of “The SRS transmission location is the last symbol in slot#3 in radio frame with TDD pattern as DDDSU, S=10:2:2”, seems confusing as there are 80 slots in a radio frame in 120kHz. Which seemingly conflicts with the t1 slot transmission periodicity.
Can each company comment on the moderator’s understanding of the intention being 
“The SRS transmission location is the last symbol in slot#3 in radio frame with the TDD pattern as DDDSU, S=10:2:2”?
I.e., the target is to transmit SRS in each second special slot.

| Company | Comments |
| --- | --- |
| Nokia | Agree with the WF and Moderator’s understanding. |
| Samsung | We suggest to change the SRS transmission periodicity as 80 slots accordance with SCS, since the intention should be only SRS transmission during one radio frame, we may also need to update the Tsrs =20 for 30KHz SCS |
| Huawei | Updated: The SRS is configured optional used for frequency estimation, so we think 10 ms is more reasonable as SRS Transmission periodicity. For the transmission slots, we are OK to change the wording to “The SRS transmission location is the last symbol in slot#3 in radio frame with the TDD pattern as DDDSU, S=10:2:2” for HST FR2. Same issue exists for HST FR1 30kHz, maybe we can update it in maintenance part. |

### Sub-topic 3-2: Other

Sub-topic description:

In this sub-topic companies are invited to bring issues to the attention of the group, which have not been captured in the previous sub-topics.

| Company | Comments |
| --- | --- |
| Nokia | An applicability rule 8.1.2.1.6 Applicability of UL timing adjustment requirements for different scenarios shall be clarified to take HST FR2 (i.e., for BS type 2-O with HST FR2 declared) into account. |
| Ericsson | Agree an applicability rule clarification would be good. |
| Huawei | We are OK to consider the applicability rule for UL timing for HST FR2. |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Title, Source |
|  | Company A |
|  | Company B |
| R4-2208195 | Draft CR for TS 38.104, Introduce performance requirements for UL TA, CATT |
|  | Nokia: - We do not think that MCS shall be listed in parameters. It was not resent in SHT FR1 parameters. [CATT] agree. - At RAN4#101-e it was agreed to define the same requirements for DM-RS pos1 and pos2 both for PUSCH and UL timing adjustment requirements [R4-2120703]: - Define requirement based on the simulation results with 2 DMRS+ PT_RS (L=1, K=2) configuration, but the final requirements are applicable for both 2 DMRS+ PT_RS (L=1, K=2) and 3 DMRS + PT_RS (L=1, K=2) - Define FRC for 2 DMRS + PT_RS (L=1, K=2) - Define FRC for 3 DMRS + PT_RS (L=1, K=2) In our understanding this means that only two sets of requirements shall be defined, i.e., two requirements per CBW in table 11.2.2.7.2-1. [CATT] The SNR for DMRS 1+1 and 1+1+1 for 50MHz are 8.8 and 9.2 respectively, how to process the difference? we can use the maximum value. e.g. 9.2. (See below) Table 11.2.2.7.2-1 Minimum requirements for UL timing adjustment with mapping type B for high speed train Number of TX antennas; Number of RX antennas; Cyclic prefix; Channel Bandwidth [MHz]; SCS [kHz]; Moving propagation conditions and correlation matrix (Annex G); FRC (Annex A); SNR [dB]. 1; 2; Normal; 50; 120; Scenario Y; G-FR2-Ax-xx; [9.1]. Scenario Y; G-FR2-Ax-xx/ G-FR2-Ax-xx; [9.2]. 200; 120; Scenario Y; G-FR2-Ax-xx; [9.0]. Scenario Y; G-FR2-Ax-xx/ G-FR2-Ax-xx; [9.1]. - We do not see a strong need to make a specific split in between FR1 and FR2 in G.4 and introduce a new table G4-2 for FR2 since the same Scenario Y is still used. 120MHz SCS should be enough to distinguish, and can be added to Scenario Y. [CATT] splitting can clearly differentiate FR1 and FR2, but we are also OK with your proposal. Editorial: SRS resource allocation, on one cell “-” are used in bullets and in the following one not. [CATT] OK, remove “ - ”. |
|  | Moderator: Please continue discussion in second round discussion section! |
| R4-2208225 | Draft CR for TS 38.141-2, Introduce performance requirements for UL TA, CATT |
|  | Nokia: The same comments as above. [CATT] OK. Additionally, we observe that in TS 38-141.2 it is common to have two different sections for BS type 1-O and BS type 2-O. We propose to follow the same approach here as well. [CATT] It was observed that the existing Table 8.2.5.5-1 and Table 8.2.5.5-2 need to be void if two different subclauses are used. If other companies have no concern on this, it is acceptable to us. |
|  | Moderator: Please continue discussion in second round discussion section! |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic 3-1 | Sub-topic 3-1: SRS configuration Issue 3-1-1: SRS Transmission comb Tentative agreements: No need to confirm prior agreement again (KTC=2). Issue 3-1-2: SRS Transmission periodicity Tentative agreements: None. Candidate options: - Option 1: Intention of prior agreement was to transmit SRS in each second special slot. Modify prior agreement and clarify in specification. - Option 2: Intention of prior agreement was to transmit SRS once per radio frame, i.e., in “the last symbol in slot#3 in radio frame with the TDD pattern as DDDSU, S=10:2:2”. Modify prior agreement and clarify in specification. Recommendations for 2nd round: The prior moderator understanding (option 1) is not shared by all companies. Please discuss in second round and align on one option. The same issue exists for HST FR1 30kHz. It should be updated in Rel-16 maintenance, potentially with an unanimously co-signed CR. |
| Sub-topic 3-2 | Sub-topic 3-2: Other Issue 3-2-1: Clarification of UL TA applicability rule Tentative agreements: “Applicability of UL timing adjustment requirements for different scenarios” (TS 38.141-2, 8.1.2.1.6) shall be clarified to take HST FR2 (i.e., for BS type 2-O with HST FR2 declared) into account. Candidate options: None Recommendations for 2nd round: Tentative agreement seems agreeable. Please also take into the account the manufacturer declaration discussion in the previous section. There may or may not be a way to declare FR2 independently of FR1 HST support. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |
|  | None. |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2208195 | Revised |
| R4-2208225 | Revised |

## Discussion on 2nd round

### Sub-topic 3-1: SRS configuration

Issue 3-1-2: SRS Transmission periodicity

- Proposals

  - Option 1: Intention of prior agreement was to transmit SRS in each second special slot.
Modify prior agreement and clarify in specification.

  - Option 2: Intention of prior agreement was to transmit SRS once per radio frame, i.e., in “the last symbol in slot#3 in radio frame with the TDD pattern as DDDSU, S=10:2:2”.
Modify prior agreement and clarify in specification.

- Recommended WF

  - The prior moderator understanding (option 1) is not shared by all companies.
Please discuss in second round and align on one option.
Option 2 seems like it can be a way forward.

  - Please also propose the corresponding specification change wording.

| Company | Comments |
| --- | --- |

### Sub-topic 3-2: Other

Sub-topic description:

In this sub-topic companies are invited to bring issues to the attention of the group, which have not been captured in the previous sub-topics.

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Title, Source |
|  | Company A |
|  | Company B |
| revisedR4-2208195 | Draft CR for TS 38.104, Introduce performance requirements for UL TA, CATT |
|  | Nokia: - We do not think that MCS shall be listed in parameters. It was not resent in SHT FR1 parameters. [CATT] agree. - At RAN4#101-e it was agreed to define the same requirements for DM-RS pos1 and pos2 both for PUSCH and UL timing adjustment requirements [R4-2120703]: - Define requirement based on the simulation results with 2 DMRS+ PT_RS (L=1, K=2) configuration, but the final requirements are applicable for both 2 DMRS+ PT_RS (L=1, K=2) and 3 DMRS + PT_RS (L=1, K=2) - Define FRC for 2 DMRS + PT_RS (L=1, K=2) - Define FRC for 3 DMRS + PT_RS (L=1, K=2) In our understanding this means that only two sets of requirements shall be defined, i.e., two requirements per CBW in table 11.2.2.7.2-1. [CATT] The SNR for DMRS 1+1 and 1+1+1 for 50MHz are 8.8 and 9.2 respectively, how to process the difference? we can use the maximum value. e.g. 9.2. (See below) Table 11.2.2.7.2-1 Minimum requirements for UL timing adjustment with mapping type B for high speed train Number of TX antennas; Number of RX antennas; Cyclic prefix; Channel Bandwidth [MHz]; SCS [kHz]; Moving propagation conditions and correlation matrix (Annex G); FRC (Annex A); SNR [dB]. 1; 2; Normal; 50; 120; Scenario Y; G-FR2-Ax-xx; [9.1]. Scenario Y; G-FR2-Ax-xx/ G-FR2-Ax-xx; [9.2]. 200; 120; Scenario Y; G-FR2-Ax-xx; [9.0]. Scenario Y; G-FR2-Ax-xx/ G-FR2-Ax-xx; [9.1]. - We do not see a strong need to make a specific split in between FR1 and FR2 in G.4 and introduce a new table G4-2 for FR2 since the same Scenario Y is still used. 120MHz SCS should be enough to distinguish, and can be added to Scenario Y. [CATT] splitting can clearly differentiate FR1 and FR2, but we are also OK with your proposal. Editorial: SRS resource allocation, on one cell “-” are used in bullets and in the following one not. [CATT] OK, remove “ - ”. |
| revisedR4-2208225 | Draft CR for TS 38.141-2, Introduce performance requirements for UL TA, CATT |
|  | Nokia: The same comments as above. [CATT] OK. Additionally, we observe that in TS 38-141.2 it is common to have two different sections for BS type 1-O and BS type 2-O. We propose to follow the same approach here as well. [CATT] It was observed that the existing Table 8.2.5.5-1 and Table 8.2.5.5-2 need to be void if two different subclauses are used. If other companies have no concern on this, it is acceptable to us. |

## Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #4:PRACH requirements (9.8.4.2.3)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-20xxxxx | Company A | Proposal 1: Observation 1: |
| R4-2209867 | Huawei, HiSilicon | Title: Simulation results on PRACH demodulation requirements for FR2 HST Moderator: Simulation results, no proposals/observations. |
| R4-2209868 | Huawei, HiSilicon | Title: Draft CR on PRACH minimum requirements for high speed train (38.104, Rel-17) Moderator: draft CR, no proposals/observations. |
| R4-2209869 | Huawei, HiSilicon | Title: Draft CR on PRACH test requirement for high speed train (38.141-2, Rel-17) Moderator: draft CR, no proposals/observations. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Interested companies are expected to add their views directly under the respective issues in a dialogue-like form, i.e., identical to how the chair would record views during a f2f meeting.

Please add further table rows as required and do not change previous comments of your company or other companies. Answering to questions from other companies is encouraged.

No proposals or observations have been received for PRACH.

### Sub-topic 4-1: Other

Sub-topic description:

In this sub-topic companies are invited to bring issues to the attention of the group, which have not been captured in the previous sub-topics.

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Title, Source |
|  | Company A |
|  | Company B |
| R4-2209868 | Draft CR on PRACH minimum requirements for high speed train (38.104, Rel-17), Huawei, HiSilicon |
| R4-2209869 | Draft CR on PRACH test requirement for high speed train (38.141-2, Rel-17), Huawei, HiSilicon |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic 4-1 | Sub-topic 4-1: Other No further issues have been brought to the attention of the group. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |
|  | None. |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2209868 | Endorsed |
| R4-2209869 | Endorsed |

## Discussion on 2nd round

### Sub-topic 4-1: Other

Sub-topic description:

In this sub-topic companies are invited to bring issues to the attention of the group, which have not been captured in the previous sub-topics.

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

All PRACH draftCRs have been endorsed after the first round.

## Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
|  | None. |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on … | YYY |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
|  | WF on BS demodulation requirement for FR2 HST | Nokia, Nokia Shanghai Bell |  |
|  | DraftCR to TS 38.104: HST FR2 PUSCH performance requirements | Intel Corporation | As per email exchange “(offline) FR2 HST Demod CR for BS demod” |
|  | DraftCR to TS 38.104: FRC for HST FR2 PUSCH performance requirements | Intel Corporation | As per email exchange “(offline) FR2 HST Demod CR for BS demod” |
|  | DraftCR to TS 38.141-2: FRC for HST FR2 PUSCH performance requirements | Intel Corporation | As per email exchange “(offline) FR2 HST Demod CR for BS demod” |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2209870 |  | Draft CR on HST FR2 BS applicability rule (38.141-2, Rel-17) | Huawei | Revised |  |
| R4-2207907 |  | draftCR to TS 38.104 on HST FR2 Channel Conditions | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2207908 |  | draftCR to TS 38.141-2 on HST FR2 Channel Conditions | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2207985 |  | Draft CR to 38.141-2: Inttroduction of HST PUSCH requirements | Ericsson, Samsung | Revised |  |
| R4-2208080 |  | Draft CR on BS Manufacturer declaration for FR2 HST for TS 38.141-2 | Samsung, [Nokia, Nokia Shanghai Bell] | Revised | @Haijie/MCC: Could you please add “Nokia, Nokia Shanghai Bell” as a further co-sourcing company? |
| R4-2208195 |  | Draft CR for TS 38.104, Introduce performance requirements for UL TA | CATT | Revised |  |
| R4-2208225 |  | Draft CR for TS 38.141-2, Introduce performance requirements for UL TA | CATT | Revised |  |
| R4-2209868 |  | Draft CR on PRACH minimum requirements for high speed train (38.104, Rel-17) | Huawei, HiSilicon | Endorsed |  |
| R4-2209869 |  | Draft CR on PRACH test requirement for high speed train (38.141-2, Rel-17) | Huawei, HiSilicon | Endorsed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Moderator (Nokia) | Axel Mueller | axel.mueller@nokia-bell-labs.com |
| Nokia, Nokia Shanghai Bell | Dmitry Petrov | Dmitry.a.petrov@nokia-bell-labs.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add your name as suffix after company name, when making comments, i.e. Company A (XX, XX).