3GPP TSG-RAN WG4 Meeting # 103-e														R4-2210368

Electronic Meeting, May 09 – 20, 2022

Source:	Verizon, Samsung

Title:	TP for TR 37.717-21-11: DC_2-48_n2

Agenda Item:	8.12.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11to include DC_2-48_n2 according to the request in [1].

# 2. Reference

[1] RP-220689, Revised WID: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	DC_2-48_n2

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | DC_48A_n2Ax |
| Note x: The downlink DC_2_n2 RESSENS requirements only apply when the band n2 downlink carrier is configured closer to the uplink operating band than the E-UTRA Band 2 downlink carrier. |  |

### 5.x.2	Co-existence studies

Coexistence studies have been performed for the lower order combination of DC_2A_n48A, where,

- For UL DC_48A_n2 configuration, IMD4 products fall into the band 2 Rx

### 5.x.3	∆TIB and ∆RIB values

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n2 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n2 | 0.6 |

Table 5.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-48_n2 | 2 | 0.2 |
|  | 48 | 0.5 |
|  | n2 | 0.2 |

### 5.x.4	Reference sensitivity exceptions

Based on co-existence analysis, it is found that MSD is needed due to 4th IMD falling into own Rx band of 2. Therefor additional MSD is needed defined in Table 7.3B.2.3.5.2-1 of 38.101-3 along with the requirement of table 5.x.4-1.

Table 5. x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | n2 | 1853 | 5 | 25 | 1933 | N/A | N/A |
|  | 48 | 3590 | 20 | 100 | 3590 | N/A | N/A |
|  | 2 | 1889 | 5 | 25 | 1969 | 12 | IMD4 |

<End of Text Proposal>