3GPP TSG RAN WG4 Meeting #103-e	R4-2210406

Online, 9th – 20th May 2022

Agenda item:	8.8.2

Source:	SoftBank Corp.

Title:	TP for TR 38.717-03-02: CA_n28-n41-n257

Document for:	Approval

1	Introduction

NR-CA of 3 Band DL and 2Band UL, CA_n28-n41-n257, was approved in RAN [1]. This TP is to capture the basic aspects for the NR-CA.

2	Text Proposal

[Unchanged Parts Skipped]

5.3.x	CA_n28-n41-n257

5.3.x.1	Operating bands for CA

Table 5.3.x.1-1: CA band combination of band n28+n41+n257

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

5.3.x.2	Channel bandwidths per operating band for CA

Table 5.3.x.2-1: Supported bandwidths per CA band combination of band n28+n41+n257

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n41A-n257A | CA_n28A-n41A CA_n28A-n257A CA_n41A-n257A | n28 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n28A-n41A-n257G | CA_n28A-n41A CA_n28A-n257G CA_n41A-n257G | n28 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n257 | n257G |  |
| CA_n28A-n41A-n257H | CA_n28A-n41A CA_n28A-n257H CA_n41A-n257H | n28 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n257 | n257H |  |
| CA_n28A-n41A-n257I | CA_n28A-n41A CA_n28A-n257I CA_n41A-n257I | n28 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n257 | n257I |  |

5.3.x.3	UE co-existence studies

Co-existence studies can be omitted because harmonic and intermodulation impact between FR1 bands have been already studied for CA_n28-n41, and harmonic and intermodulation impact between FR1 bands and FR2 band are negligible.

5.3.x.4	REFSENS requirements

There is no additional REFSENS requirement for this combination.

[Unchanged Parts Skipped]

3	Reference

[1]  RP-211060  Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL, ZTE