GPP TSG-RAN WG4 Meeting #103-e                                                    													R4-2210429

Online, 9th – 20th May, 2022

Source:	Vodafone

Title:	TP for TR 37.717-11-21: DC_20_n7-n78

Agenda item:			8.15.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-11-21 to include DC_20_n7-n78.

The proposed TIB,c, RIB and MSD values were derived from those present in 38.101-3 for DC_7A-20A_n78A and DC_20A_n41A-n78A.

# 2. Reference

# 3. Text Proposal

<Start of Text Proposal>

## 6.x	DC_20_n7-n78

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20A_n7A-n78A | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_20A_n7A-n78A | DC_20A_n7A DC_20A_n78A | 20 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n7 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.x.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_20A_n7A-n78A with dual uplink on DC_20A_n7A and DC_20A_n78A were calculated and listed in Table 6.x.3-1 and 6.x.3-2 respectively.

Table 6.x.3-1: Harmonic and IMD analysis for DC_20_n7-n78 with UL on DC_20_n7

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 2500 | 2570 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 5000 – 5140 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 7500 – 7710 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1638 – 1738 |  |  | 3332 – 3432 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 776 – 906 |  |  | 4138 – 4308 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4164 – 4294 |  |  | 5832 – 6002 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 782 – 912 |  |  | 2480 – 2590 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 – 86 |  |  | 6638 – 6878 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 3276 – 3476 |  |  | 6664 – 6864 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4996 – 5156 |  |  | 8332 – 8572 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9138 – 9448 |  |  | 758 – 948 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5776 – 6046 |  |  | 2414 – 2644 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10832 – 11142 |  |  | 5828 – 6018 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9164 – 9434 |  |  | 7496 – 7726 |  |  |

Table 6.x.3-2: Harmonic and IMD analysis for DC_20_n7-n78 with UL on DC_20_n78

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2438 – 2968 |  |  | 4132 – 4662 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1576 – 2136 |  |  | 5738 – 6768 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4964 – 5524 |  |  | 7432 – 8462 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 732 – 962 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 714 – 1304 |  |  | 9038 – 10568 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 4876 – 5936 |  |  | 8264 – 9324 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 – 6386 |  |  | 10732 – 12262 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12338 – 14368 |  |  | 148 – 472 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8176 – 9736 |  |  | 4014 – 5104 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 – 16062 |  |  | 6628 – 7248 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 – 13124 |  |  | 9096 – 10186 |  |  |

Based on Table 6.x.3-1 and Table 6.x.3-2:

- 2nd order IMD generated by dual uplink of Band 20 + Band n7 may fall into own Rx of band n78 – it could reuse the MSD value of DC_20A_n41A-n78A

- 4th order IMD generated by dual uplink of Band 20 + Band n7 may fall into own Rx of band n78

- 3rd and 5th order IMD generated by dual uplink of Band 20 + Band n7 may fall into own Rx of band 20

- 5th order IMD generated by dual uplink of Band 20 + Band n7 may fall into own Rx of band n7

- 2nd order IMD generated by dual uplink of Band 20 + Band n78 may fall into own Rx of band n7 – it could reuse the MSD value of DC_7A-20A_n78A

- 2nd order IMD generated by dual uplink of Band 20 + Band n78 may fall into own Rx of band 20

- 4th order IMD generated by dual uplink of Band 20 + Band n78 may fall into own Rx of band n7 and band 20

### 6.x.4	∆TIB and ∆RIB values

### For DC_20A_n7A-n78A, the TIB,c and RIB,c values can reuse the values of DC_7A-20A_n78A and DC_20A_n41A-n78A as given in the tables below

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n7-n78 | 20 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.8 |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n7-n78 | n78 | 0.5 |

### 6.x.5	Reference sensitivity exceptions

Table 6.x.5-1 shows the required MSD.

Table 6.x5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A_n7A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n7 | 2555 | 5 | 25 | 2675 | 30.8 | IMD2 |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
|  | 20 | 850 | 5 | 25 | 809 | N/A | N/A |
|  | n7 | 2550 | 10 | 50 | 2675 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | 28.8 | IMD21 |
| NOTE 1: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

<End of Text Proposal>