3GPP TSG-RAN WG4 Meeting # 103-e 													R4-2210476

Electronic Meeting, 9th May – 20th May, 2022

Agenda item:			9.9

Source:	Moderator (vivo)

Title:	Email discussion summary for [103-e][207] NR_RRM_enh2_2

Document for:	Information

Introduction

This email discussion summary covers agendas 9.9.1.2 and 9.9.2.2 for core requirement maintenance and performance requirements for topic HO with PSCell.

Topic #1: HO with PSCell core requirement maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207769 | Apple | Proposal 1: the RO transmission failure due to both UL LBT failure and RO collision with PCell UL channel shall be limited by PREAMBLE_TRANSMISSION_COUNTER, and the UE behaviour when PREAMBLE_TRANSMISSION_COUNTER reaches the preambleTransMax should follow TS38.321. |
| R4-2207770 | Apple | CR on HO with PSCell for NE-DC to NE-DC in TS38.133 R17 |
| R4-2207771 | Apple | CR on HO with PSCell for EN-DC to EN-DC in TS36.133 R17 |
| R4-2208170 | CATT | Proposal 1: The requirements for PSCell addition for handover with PSCell from NR SA to EN-DC are modified to include parallel processing. |
| R4-2208171 | CATT | CR: Completing requirement of HO with PSCell |
| R4-2208500 | Nokia, Nokia Shanghai Bell | CR: Correction on HO with PSCell requirements in 38133 |
| R4-2208501 | Nokia, Nokia Shanghai Bell | CR: Correction on HO with PSCell requirements in 36133 |
| R4-2208530 | CMCC | CR on Handover with PSCell |
| R4-2208937 | Huawei, Hisilicon | CR on HO with PSCell requirements |
| R4-2209494 | vivo | CR on R17 core requirements for HO with PSCell |
| R4-2210131 | Ericsson | Proposal 1: Separate end points are assumed for PCell HO and PSCell addition/change and PRACH preamble transmission is the end point for PCell HO and PSCell addition/change. Proposal 2: Both parallel and sequential cell search cases are considered for HO with PSCell for NR-U Proposal 3: For NR PSCell change with target NR PSCell under CCA (band n46), Tprocessing for the NR PSCell addition/change is 25ms. Proposal 4: When PSCell is under CCA, if UE is incapable of simultaneous PRACH preamble transmission in PCell and PSCell, and RACH occasions in PCell and PSCell collide, then UE shall prioritize PRACH preamble transmission on the carrier with CCA. An additional uncertainty term is introduced for the PCell HO delay. |
| R4-2210132 | Ericsson | CR on requirements for HO with PSCell when PSCell is on CCA in EN-DC to EN-DC scenario |
| R4-2210133 | Ericsson | CR on requirements for HO with PSCell when PSCell is on CCA in NR SA to EN-DC scenario |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1 Core requirements for HO with PSCell for NR-U

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1: Scenarios for HO with PSCell for NR-U

- Proposals

  - Option 1: (Ericsson)

    - For HO with PSCell when NR PSCell is in FR1 unlicensed spectrum (band n46), requirements shall be defined for NR SA to EN-DC and EN-DC to EN-DC cases

- Recommended WF

  - Discuss option 1.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Fine with option 1. |
| Intel | OK with Option 1. |
| Ericsson | Support Option 1. Since Licensed carrier supports both the configurations, do not see why unlicensed carrier should be treated differently. |
| Qualcomm | Support Option 1. |
| OPPO | OK with Option 1. |
| Nokia | Fine with Option 1. |

Issue 1-2: Cell search and timing acquisition

- Proposals

  - Option 1: (Ericsson)

    - Both parallel and sequential cell search cases are considered for HO with PSCell for NR-U

- Recommended WF

  - Discuss in the 1st round.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | For EN-DC to EN-DC, only parallel processing is considered, and for NR-SA to EN-DC, both parallel and sequential processing shall be considered. |
| Intel | Fine with Option 1. |
| Ericsson | We are fine with Apple proposal. |
| Qualcomm | We are okay with Apple comment |
| OPPO | OK with Option 1. |
| Nokia | We are fine with Apple comment. |
| vivo | OK to option 1 and Apple’s comment |

Issue 1-3: SW processing and RF warm up

- Proposals

  - Option 1: (Ericsson)

    - For NR PSCell change with target NR PSCell under CCA (band n46), Tprocessing for the NR PSCell addition/change is 25ms.

- Recommended WF

  - Discuss in the 1st round.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | For EN-DC to EN-DC case, the Tprocessing is 25ms, but for NR-SA to EN-DC with sequential processing the Tprocessing is 30ms. |
| Ericsson | Fine with Apple’s proposal. |
| Qualcomm | Agree with Apple, Tprocessing should follow as same as the requirement in licensed band. |
| OPPO | Fine with Apple’s proposal. |
| Nokia | Tprocessing should be the same in licensed and unlicensed band. We agree with Apple’s and Qualcomm’s comments. |
| vivo | Fine with Apple’s proposal |

Issue 1-4: Ending points for PCell HO and PSCell change

- Proposals

  - Option 1: (Ericsson)

    - Separate end points are assumed for PCell HO and PSCell addition/change and PRACH preamble transmission is the end point for PCell HO and PSCell addition/change.

- Recommended WF

  - Discuss in the 1st round.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Agree with option 1. |
| Intel | OK with Option 1. |
| Ericsson | Support option 1. |
| Qualcomm | Support option 1. |
| Ericsson | Fine with option l. |
| Nokia | Fine with option 1. |
| vivo | OK with option 1. |

Issue 1-5: Handling of RACH occasion collision between PCell and PSCell

- Proposals

  - Option 1: (Ericsson)

    - When PSCell is under CCA, if UE is incapable of simultaneous PRACH preamble transmission in PCell and PSCell, and RACH occasions in PCell and PSCell collide, then UE shall prioritize PRACH preamble transmission on the carrier with CCA. An additional uncertainty term is introduced for the PCell HO delay.

  - Option 2: (Apple)

    - The RO transmission failure due to both UL LBT failure and RO collision with PCell UL channel shall be limited by PREAMBLE_TRANSMISSION_COUNTER, and the UE behaviour when PREAMBLE_TRANSMISSION_COUNTER reaches the preambleTransMax should follow TS38.321.

- Recommended WF

  - Discuss in the 1st round.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Regarding option 1, we think RAN4 shall follow the RAN1 conclusion in TS38.213, i.e., LTE RACH shall be prioritized. Support option 2. |
| Ericsson | May be a clarification question to Apple. Is RAN1 conclusion referred here is regarding the power allocation issue? Can you please refer the clause number if it is not the power allocation issue. Our understanding of RAN1 conclusion of prioritizing the MCG transmission is for power allocation limitation issue and not for the collision case. However, we are not sure if power allocation issue is applicable between NR-U PSCell and LTE PCell. |
| Qualcomm | We don’t agree with Option 1. In worst case at option1, Pcell RACH timing can be significantly delayed if PScell RACH is continuously failed from either LBT failure or collision. If any prioritization for NR-U needs to be defined, we think it should be discussed at RAN1. We can support Option2 to define the maximum limit of trial. |
| OPPO | We share the similar view as QC. Option 2 is preferred. |
| Apple2 | To Ericsson, yes, it’s power limitation issue (UE cannot simultaneously transmit on both LTE and NR CCs), and the section we referred to is TS38.213 section 7.6.1. |
| Nokia | We do not support prioritizing the PRACH preamble transmission on the carrier with CCA. We support option 2, to define a collision limit also for this case, and reuse the UE behavior when the PREAMBLE_TRANSMISSION_COUNTER reaches the maximum value. |
| Ericsson2 | Thank you, Apple, for clarification. We can keep the collision case in [] in the revised CR and discuss further in the second round. |

Companies’ views collection for 1st round

Open issues

Comments are collected in section 1.2.

CRs/TPs comments collection

Companies are encouraged to provide comments on CRs.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2207770 Apple | Ericsson: Agree with the principle. Wording can be discussed further. May be something on these lines. TIU_PCell zero if there is no collision between PCell RACH and PSCell RACH occasion. In other cases, it is the uncertainty in acquiring the next available RACH occasion for PCell RACH transmission. Exact wording can be further discussed in second round. |
|  | Qualcomm : we are fine with this CR |
|  | MTK: ok with this change |
| R4-2207771 Apple | Ericsson: Similar comment as above. |
|  | Qualcomm : we are okay with this CR. |
|  | MTK: ok with this change |
|  | Nokia: The CR is not agreeable. The addition of Longer TPSCell_ DU is not needed since TPCell_DU has captured the collision case for LTE + FR1 EN-DC case. |
| R4-2208171 CATT | Apple: fine with this CR |
|  | Huawei： Similar change in 8500/8530/8937/9494 |
|  | Ericsson: In principle OK. May be few minor editorial changes may be required and can be done in 2nd round |
|  | Qualcomm : fine with this CR. |
|  | CMCC: one minor comment, Tsearch_HO is in use in 6.1.5.2.1, while Tsearch_PCell is in use in 6.1.5.2.2, the terminology needs to be aligned in order to avoid the ambiguity. [CATT] Since the time for obtaining the timing reference of target PCell is not always needed, to make it clear, we use a new terminology Tsearch_PCell and refer to Tsearch_HO when it is needed. But we are also fine to use Tsearch_HO directly. |
|  | Nokia: we are fine with this CR, there are still some editorial error to be corrected. We have the similar change in 8500, we can merge together. |
| R4-2208500 Nokia, Nokia Shanghai Bell | Apple: fine with this CR and could be merged with CATT CR 8171. |
|  | Huawei: similar change in 7770. Prefer the wording in 7770 Huawei2: Sorry for the mistake, the comment above is to 8501 |
|  | Ericsson: CATT and Nokia CR can be merged. |
|  | Qualcomm : Agree with CR |
|  | MTK: ok with this CR, could be merged with CATT. |
|  | Nokia: OK, we can merge with CATT’s CR 8171 |
| R4-2208501 Nokia, Nokia Shanghai Bell | Apple: fine with this CR |
|  | Ericsson: Do we need to capture the collision case of PCell RACH and PSCell RACH? |
|  | Qualcomm : agree with CR |
|  | MTK: same comment as Ericsson. |
|  | Nokia: To Ericsson & MTK, the TPCell_DU has captured the collision case. |
| R4-2208530 CMCC | Apple: fine with this CR and could be merged with CATT CR 8171. |
|  | Ericsson: Agree with the change. |
|  | Qualcomm : Agree with CR |
|  | CMCC: We are fine to merge to CR 8171 |
|  | MTK: ok with this CR, could be merged with CATT. |
| R4-2208937 Huawei, Hisilicon | Apple: fine with this CR and could be merged with CATT CR 8171. |
|  | Qualcomm : Agree with CR |
|  | MTK: ok with this CR, could be merged with CATT. |
| R4-2209494 vivo | Apple: for NE-DC to NE-DC HO with PSCell, the requirement shall refer to LTE PSCell addition but not NR PSCell addition, so it shall refer to section 8.8.2 of TS38.133. But the UE processing time delay is unclear in section 8.8.2 (because we stated that “except that UE processing delay for software processing and RF warmup is same as processing delay specified for PCell HO in 6.1.5.3.2.”), shall extra 25ms of processing be added into that equation from section 8.8.2? |
|  | Ericsson: I think 2nd change should refer to 8.8.2 of TS 38.133. To Apple: From our understanding it is the 50ms component is UE processing delay from the equation of 8.8.2. |
|  | Qualcomm : 8.8.2 of TS38.133 instead of 8.9.2 for NE-DC to NE-DC Legacy T processing is defined as 20ms for same FR and 40ms for different FR. Thus, we don’t think 50ms is pointing just T_processing but it may T_processing+other_delay. Since it is not clear what 50ms stands for, we suggest clarifying the definition of 50ms. |
|  | Apple2: to Ericsson and Qualcomm, the 50ms was explained in R4-147869 during DC discussion in LTE, - Total pScell configuration/activation time Delay Items; Subframe level sync (up to 33us); Subframe level async (up to 500us). Known pSCell; Unknown pSCell; Known pSCell; Unknown pSCell. RRC Procedure; 15; 15; 15; 15. pSCell activation time; 20ms; 30ms; 20ms; 30ms. SFN acquisition; 50; 50; 50; 50. Pcell RACH uncertainty; 20; 20; 20; 20. PScell RACH uncertainty; 30; 30; 30; 30. Total delay; 135; 145; 135; 145. Where 50ms is SFN acquisition time based on previous agreement in LTE. |
|  | Qualcomm2 : To Apple, Thanks for the table. However, we still think T_processing is included in 50ms but not explicitly defined. At 38.133. V17.5, A.4A.1.1, PScell addition for NE-DC is already defined with following values. The UE shall transmit the PRACH to PSCell at latest 120 msNote1 into T3. Tconfig_EUTRAN-PSCell = 20ms + Tactivation_time + 50ms + TPCell_ DU + TE-UTRAN-PSCell_ DU Tactivation_time = 20ms TPSCell_ DU = 0ms TE-UTRAN-PSCell_ DU = 30ms Although T_processing is not explicitly defined, we think this test requirement implies T_processing is already included in this test requirements. Thus, we think extra 5ms is required at 8.8.2. |
|  | Nokia: the 1st change is fine and can be merged to 8171. The 2nd change should refer to 8.8.2 which is for NE-DC PSCell addition, and the processing time need to be updated to refer to PCell HO part |
|  | Vivo: Sorry for the typo. It should be 8.8.2. We are fine to merge CRs Ericsson2: We also share the RF warm-up and software processing is included in the 50ms and additional 25ms is not needed. |
| R4-2210132 Ericsson | Apple: based on conclusions from the above issues for HO with PSCell in NR-U |
|  | Ericsson: We shall update it based on outcome of discussion |
| R4-2210133 Ericsson | Apple: based on conclusions from the above issues for HO with PSCell in NR-U |
|  | Ericsson: We shall update it based on outcome of discussion |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Sub-topic 1 Core requirements for HO with PSCell for NR-U

|  | Status summary |
| --- | --- |
| Issue 1-1: Scenarios for HO with PSCell for NR-U | Tentative agreements: - For HO with PSCell when NR PSCell is in FR1 unlicensed spectrum (band n46), requirements shall be defined for NR SA to EN-DC and EN-DC to EN-DC cases Recommendations for 2nd round: The issue is closed. |
| Issue 1-2: Cell search and timing acquisition | Tentative agreements: - For HO with PSCell from EN-DC to EN-DC, only parallel processing is considered for NR-U. - For HO with PSCell from NR-SA to EN-DC, both parallel and sequential processing shall be considered for NR-U. Recommendations for 2nd round: The issue is closed. |
| Issue 1-3: SW processing and RF warm up | Tentative agreements: - For HO with PSCell from EN-DC to EN-DC and from NR-SA to EN-DC with parallel processing, Tprocessing is 25ms. - For HO with PSCell from NR-SA to EN-DC with sequential processing, Tprocessing is 30ms. Recommendations for 2nd round: The issue is closed. |
| Issue 1-4: Ending points for PCell HO and PSCell change | Tentative agreements: - PRACH preamble transmission is the end point for PCell HO and PSCell addition/change. - Separate end points are assumed for PCell HO and PSCell addition/change Recommendations for 2nd round: The issue is closed. |
| Issue 1-5: Handling of RACH occasion collision between PCell and PSCell | Tentative agreements: None Candidate options: - Proposals - Option 1: (Ericsson) - When PSCell is under CCA, if UE is incapable of simultaneous PRACH preamble transmission in PCell and PSCell, and RACH occasions in PCell and PSCell collide, then UE shall prioritize PRACH preamble transmission on the carrier with CCA. An additional uncertainty term is introduced for the PCell HO delay. - Option 2: (Apple) - The RO transmission failure due to both UL LBT failure and RO collision with PCell UL channel shall be limited by PREAMBLE_TRANSMISSION_COUNTER, and the UE behaviour when PREAMBLE_TRANSMISSION_COUNTER reaches the preambleTransMax should follow TS38.321. Recommendations for 2nd round: Discuss in the GTW. Continue discussion in the 2nd round if necessary. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2210131 | to be Revised |
| R4-2207770 | to be Revised |
| R4-2207771 | to be Revised |
| R4-2208171 | to be Merged |
| R4-2208500 | to be Merged |
| R4-2208501 | to be Return to |
| R4-2208530 | to be Merged |
| R4-2208937 | to be Revised |
| R4-2209494 | to be Revised |
| R4-2210132 | to be Revised |
| R4-2210133 | to be Revised |

Discussion on 2nd round

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Issue 1-5: Handling of RACH occasion collision between PCell and PSCell

- Proposals

  - Option 1: (Ericsson)

    - When PSCell is under CCA, if UE is incapable of simultaneous PRACH preamble transmission in PCell and PSCell, and RACH occasions in PCell and PSCell collide, then UE shall prioritize PRACH preamble transmission on the carrier with CCA. An additional uncertainty term is introduced for the PCell HO delay.

  - Option 2: (Apple, QC, OPPO)

    - The RO transmission failure due to both UL LBT failure and RO collision with PCell UL channel shall be limited by PREAMBLE_TRANSMISSION_COUNTER, and the UE behaviour when z reaches the preambleTransMax should follow TS38.321.

- Recommended WF

  - Agree on option 2.

  - Ericsson is encouraged to check if option 2 is agreeable.

- 2nd round Comment collection:

| Company | Comments |
| --- | --- |
| Ericsson | We checked internally and we are not sure that the prioritization rules defined in RAN1 spec due to transmit power limitation is applicable for only licensed carrier operation (that means LTE and NR) or applicable for NR-U and LTE also. Considering this we think clarification from RAN1 may be needed to clarify things. To make progress, we can agree to option 2 as the working agreement and meanwhile we can send LS to RAN1 to get it clarified. In the CR we can keep the collision case in [] while we wait for RAN1 LS reply. |
| Apple | We support option 2 and agree with Ericsson’s suggestion. |
| Nokia | We support option 2, and we also agree with sending an LS to RAN1 to get this issue clarified. |
| Qualcomm | We support option2 and LS to RAN1. |
| OPPO | Support option 2. |

Summary on 2nd round (if applicable)

Topic #2: HO with PSCell performance requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207772 | Apple | Draft CR on TC for HO with PSCell from NR-SA to EN-DC with parallel processing and known FR2 PSCell in TS38.133 R17 |
| R4-2208064 | Intel Corporation | DraftCR to TS 38.133: Handover with PSCell from NR-DC to NR-DC with sequential processing |
| R4-2208106 | Xiaomi | CR on test case for Handover with PSCell from NR SA to EN-DC with sequential processing |
| R4-2208176 | CATT | Proposal 1: The configurations for this test case can refer to the configurations for E-UTRA intra-frequency handover and for NR intra-frequency handover. Proposal 2: There are four cells in the test case, two original cells on E-UTRA carrier and NR carrier and two target cells on the same carriers respectively. Proposal 3: The test can consist of three successive time periods, T1, T2 and T3. The target cells turn off during T1, and turn on during T2. UE will send Event A3 reports and receive RRC message implying handover with PSCell during T2, and perform the handover with PSCell during T3. Proposal 4: The test handover requirement for handover with PSCell is defined as 60ms. Proposal 5: The test requirement for PSCell change delay is defined as 117ms. |
| R4-2208177 | CATT | Test case of handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1 |
| R4-2208351 | OPPO | draft CR on HO with PSCell from NR SA to EN-DC with parallel processing(TC2) |
| R4-2208458 | Qualcomm Incorporated | N/A |
| R4-2208502 | Nokia, Nokia Shanghai Bell | - Remove TC3 and TC4 in NR SA to EN-DC since it is similar as TC7 and TC8 in NR-DC to NR-DC - Update the target PCell to unknown cell in TC6 to raise the test coverage for HO with PSCell. - Only focus on PSCell addition / change delay verification in TC7, TC8, TC9 and TC10. - TC4, TC7 and TC10 could be tested with the LTE/FR1+FR2 test setup defined in A.3.7, A.3.7A and A.3.7C. - TC3 and TC8 need further discuss since it will be hard to test because of this FR1+FR2 test limitation |
| R4-2208503 | Nokia, Nokia Shanghai Bell | dratCR on test case for HO with PSCell - TC10 |
| R4-2208518 | CMCC | Proposal 1: for HO with PSCell, it is proposed to define test cases for the four scenarios including from NR SA to EN-DC, from EN-DC to EN-DC, from NE-DC to NE-DC, and from NR-DC to NR-DC. Proposal 2: it is proposed to define test cases for both parallel processing and sequential processing. Proposal 3: for the test configuration of test cases for HO with PSCell, it is proposed to consider following configurations: - For FR1 - 15 kHz SSB SCS with 10 MHz bandwidth for FDD - 15 kHz SSB SCS with 10 MHz bandwidth for TDD - 30 kHz SSB SCS with 40 MHz bandwidth for TDD - For FR2 - 120 kHz SSB SCS with 100 MHz bandwidth for TDD |
| R4-2208942 | Huawei, Hisilicon | Observation 1: The requirement for HO with PSCell can be verified by testing the transmission time of PRACH to target PCell and target PSCell. Observation 2: For HO with PSCell from NR SA to EN-DC, to verify the sequential processing case, both target PCell and target PSCell should be unknown; otherwise, the delay may be not significantly different from parallel case. Proposal 1: For the test case of HO with PSCell from NR SA to EN-DC, the sequential processing case should be verified when target PCell and target PSCell are unknown and the target PSCell is in FR1. Proposal 2: For the test case of HO with PSCell from NR-DC to NR-DC, do not define test cases for sequential processing case. Observation 3： For the test case of HO with PSCell from NR-DC to NR-DC, it doesn't make much sense to only verify the requirements of PSCell change when the link level in PCell cannot be guaranteed. |
| R4-2208943 | Huawei, Hisilicon | Draft CR on TC for HO with PSCell from NR SA to EN-DC |
| R4-2209495 | vivo | Proposal 1 Test cases are designed to only verify the known cases for HO w PSCell requirements when parallel processing is assumed. Proposal 2 For test cases related to FR1-FR2 joint testing, test case design are delayed until testability issues are solved. Proposal 3 Do not verify all intra-frequency and inter-frequency HO scenarios in the HO w PSCell test cases. Proposal 4 For the case when both target PCell and target PSCell are known, the test system sends RRC message implying HO with PSCell only after it receives 2 MRs. |
| R4-2209496 | vivo | draft CR on test cases for Handover with PSCell from NE-DC to NE-DC with known target PSCell |
| R4-2209991 | MediaTek Inc. | Draft CR 38.133 on the test case of handover with PSCell from NR-DC to NR-DC with parallel processing |
| R4-2210114 | Qualcomm Incorporated | Proposal 1: Test cases cover the following cases NR SA to EN-DC, NR EN-DC to EN-DC, NR NE-DC to NE-DC, and NR-DC to NR-DC (FR1+FR2 only) Proposal 2. Both sequential processing cases and parallel processing cases are covered. Proposal 3. Target PCell and Target Pscell are both known or both unknown. Proposal 4. If target PSCell is unknown, single SMTC configuration of target unknown PScell from either source Pcell MO or source PSCell MO can be configured. |
| R4-2210136 | Ericsson | TC for HO with PSCell from NE-DC to NE-DC with unknown target PSCell |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1 Test cases coverage, design and configurations

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: Test cases design principle

- Proposals

  - Proposal 1: Define test cases for the scenarios including from NR SA to EN-DC, from EN-DC to EN-DC, from NE-DC to NE-DC, and from NR-DC to NR-DC (FR1+FR2 only).

  - Proposal 2: Define test cases to cover both parallel processing and sequential processing cases.

  - Proposal 3: Define test cases with target PCell and target Pscell are both known or both unknown.

  - Proposal 5: Test cases are designed to only verify the known cases for HO w PSCell requirements when parallel processing is assumed.

  - Proposal 6:  For test cases related to FR1-FR2 joint testing, test case design is delayed until testability issues are solved.

  - Proposal 7: Do not verify all intra-frequency and inter-frequency HO scenarios in the HO w PSCell test cases.

- Recommended WF

  - Discuss the proposals in the 1st round.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Proposal 1: may not need ‘NR-DC to NR-DC case’ and ‘LTE + FR2 EN-DC case’ due to FR1+FR2 testability issue. We are fine to test other scenarios. Proposal 2: agree with proposal 2. Proposal 3: We support to selectively test the unknown case only to reduce the TC amount. Proposal 5: if proposal 3 can go with only unknown case, we may not need to test known case for parallel processing. Proposal 6: agree with proposal 6. Proposal 7: agree with proposal 7, either intra-frequency or inter-frequency is selected for testing. |
| Huawei | For p2-p5. Fine with these proposals but they are quite generic. Suggest to discuss in the test case list in a case by case manner. For p6, as also commented in other thread, it is recommended to handle the issue in a consistent manner cross WIs. For p3, may be clarifications are needed. |
| Intel | Proposal 1: For NR-DC to NR-DC FR1+FR2 with sequential processing, there may be some problem due to FR1+FR2 testability issue. Proposal 2: Fine. Proposal 3: Fine. it’s better to balance the workload and test coverage. Proposal 6: Further discuss. In legacy, we have test case for NR PSCell addition(FR2) delays under EN-DC. Therefore, for FR1+FR2 paralleling processing, is it possible that we only test PSCell change delay which is only on FR2? Since HO in FR1 may be problematic. Proposal 7: Fine. |
| Ericsson | P1: Same view as Apple. P2: OK P3 and p5 may be it can be as generic as possible without limiting it to any particular cases while maintaining the number of test cases to as minimum as possible. P6 agree with Huawei that consistent methodology can be followed. P7: It can be mix of them among test cases. |
| Qualcomm | We are fine all proposals. We would like further discuss how to minimize test cases |
| CMCC | P1: we are fine not to consider NR-DC to NR-DC (FR1+FR2 only), support to define test for other scenarios. P2, P3: support |
| CATT | Fine with P2, P3, P6 and P7. For P1, fine to remove FR1+FR2 case. P5 need further discussion in the exact test case whether parallel processing is applicable for unknown case. |
| OPPO | P1: Same view as Apple. P2, P3, P6, P7: OK |
| MTK | P1: We are fine not consider the (FR1+FR2) scenarios due to testability issue. Ok with P2,6,7. P3,5 can be discussed further in the test cases. |
| Nokia | Generally, we are fine with the principles in the proposals, however we should further discuss how to reduce test cases to cover all the core requirements, only to define necessary test cases. |
| vivo | Support P2, P3, P5, P6, P7. For P1 and P6, we are also OK if test cases are defined in this release, but add applicability to it. |

Issue 2-2: General test cases configuration

- Proposals

  - Proposal 1: If target PSCell is unknown, single SMTC configuration of target unknown PScell from either source Pcell MO or source PSCell MO can be configured.

  - Proposal 2: For the case when both target PCell and target PSCell are known, the test system sends RRC message implying HO with PSCell only after it receives 2 MRs.

  - Proposal 3: The requirement for HO with PSCell can be verified by testing the transmission time of PRACH to target PCell and target PSCell.

  - Proposal 4: For the test configuration of test cases for HO with PSCell, following configurations is considered:

  - For FR1

    - 15 kHz SSB SCS with 10 MHz bandwidth for FDD

    - 15 kHz SSB SCS with 10 MHz bandwidth for TDD

    - 30 kHz SSB SCS with 40 MHz bandwidth for TDD

  - For FR2

    - 120 kHz SSB SCS with 100 MHz bandwidth for TDD

- Recommended WF

  - Discuss the proposals in the 1st round.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Proposal 1: fine with proposal 1. Proposal 2: related with the discussion of proposal 3 in issue 2-1. Proposal 3: fine with proposal 3. Proposal 4: fine with configurations for FR1. Regarding FR2, it’s also related with the conclusion in proposal 1/6 in issue 2-1, i.e., if no FR2 would be involved in HO with PSCell test, we don’t need to discuss FR2 configuration. |
| Huawei | Fine with all proposals |
| Intel | Proposal 1: OK with proposal 1. Proposal 3: OK with proposal 3. Proposal 4: For FR1, fine with the configuration. For FR2, depends on the conclusion of issue 2-1. |
| Ericsson | P2 do not understand the proposal. May be clarification is needed. Other proposals are fine. |
| Qualcomm | We are okay with proposal 1,3,4. |
| CMCC | We are OK with P1,3,4. And for P4, we are fine to further discuss FR2 |
| CATT | Fine with all the proposals. |
| OPPO | P1,P3,P4: OK |
| MTK | We are fine with Proposals 1,3, and 4. Proposal 2 not clear. |
| Nokia | Similar view as Ericsson, P1, P3, P4 are fine. P2 needs to be clarified. |
| vivo | Support all proposals. For proposal 2, since both PCell and PSCell are considered as known, valid MRs need to be sent for both PCell frequency layer and PSCell frequency layer. In legacy test cases, when testing HO to known cell, MR is transmitted before HO command. However, in HO w PSCell test case design, the transport channels for MRs needs to be specified. UE needs to send 2 MRs to either PCell only, or to both PCell and PSCell. This is the necessary detail to complete the test case design. Please refer to our CR R4-2209496 and check whether the following is agreeable. [Note: Some typos in R4-2209496 corrected as below. Sorry for the confusion] |

Issue 2-3: Test cases configuration for HO with PSCell from EN-DC to EN-DC

- Proposals

  - Proposal 1: The configurations for this test case can refer to the configurations for E-UTRA intra-frequency handover and for NR intra-frequency handover.

  - Proposal 2: There are four cells in the test case, two original cells on E-UTRA carrier and NR carrier and two target cells on the same carriers respectively.

  - Proposal 3: The test can consist of three successive time periods, T1, T2 and T3. The target cells turn off during T1, and turn on during T2. UE will send Event A3 reports and receive RRC message implying handover with PSCell during T2, and perform the handover with PSCell during T3.

  - Proposal 4: The test handover requirement for handover with PSCell is defined as 60ms.

  - Proposal 5: The test requirement for PSCell change delay is defined as 117ms.

- Recommended WF

  - Discuss the proposals in the 1st round.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Proposal 1: fine with proposal 1 if in proposal 7 of issue 2-1 we conclude to choose intra-frequency for testing. Proposal 2: fine with proposal 2 if in proposal 7 of issue 2-1 we conclude to choose intra-frequency for testing. Proposal 3: fine with proposal 3. Proposal 4: fine, RRC procedure delay is 20ms based on RAN2 LS R2-2104580, TIU= 15ms based on LTE HO testing, Tprocessing= 25ms (LTE+ FR1 EN-DC), so in total is 60ms. Proposal 5: have different calculation from our side. DHOwithPSCel_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + TPCell_DU + 2 ms =20ms+25ms+0+20ms+20ms(based on PSCell addition test in TS38.133 A.4.5.7)+0(no clolliding with PCell RACH)+2ms = 87ms |
| Huawei | Fine with p1-p3. For p4 and p5, conditions are needed (known/unknown FR1/FR2) |
| Intel | Proposal 1~4: Fine. Proposal 5: if the target PSCell is known, the delay for PSCell change is 87ms. Same view as Apple. |
| Ericsson | P1 needs conclusion from other issues. P2 to P3 are fine. P4 and P5 can be discussed in test case. |
| Qualcomm | We are fine with Proposal 1, 2, 3 agree with apple for Proposal 4,5 if target PScell is known. |
| CATT | Support proposal 1, 2, 3, 4. And proposal 5 can be modified to: “The test requirement for PSCell change delay is defined as 87ms.” For P5, to Apple, Intel and QC, yes, the delay should be 87ms and it seems we make TPCell_DU = 30ms in the calculation. We are also fine to further discuss in the test case. |
| OPPO | Fine with P1, P2, P3. P4 and P5 can be discussed in test case. |
| MTK | Fine with Proposal 1,2,3,4. Proposal 5 should be 87 ms as Apple pointed out. |
| Nokia | P1 needs to be discussed together with other scenarios, we can cross config between test cases to reduce the test cases load. We are fine with P2 & P3. P4 & P5 will depend on the known/unknown target cells. |
| vivo | Generally fine with all the proposals, with the corrected delay in Proposal 5. |

Issue 2-4: Test cases configuration for HO with PSCell from NR SA to EN-DC

- Proposals

  - Proposal 1: For the test case of HO with PSCell from NR SA to EN-DC, the sequential processing case should be verified when target PCell and target PSCell are unknown and the target PSCell is in FR1.

- Recommended WF

  - Discuss the proposals in the 1st round.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Fine with proposal 1. |
| Huawei | Support proposal 1. |
| Intel | OK with proposal 1. |
| Ericsson | OK with proposal 1. |
| Qualcomm | We are fine with proposal 1. |
| CATT | Fine with option 1. |
| OPPO | Fine with option 1. |
| MTK | Fine with Proposal 1. |
| Nokia | We are fine with proposal 1. |
| vivo | We are fine with proposal 1. |

Issue 2-5: Test cases list for HO with PSCell

Following list of test cases were proposed by moderator for work split before the meeting.

| TC No. | TC | PCell Handover | PSCell addition/change | Purpose | Clause | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Handover with PSCell from NR SA to EN-DC with sequential processing | NR SA-EUTRA handover with known target PCell Source Pcell in FR1 | SMTC of target unknown PSCell is present in RRCConnectionReconfiguration Target PSCell in FR1 | To verify delay requirements for handover with PSCell from NR SA to EN-DC with sequential processing | A.6.3.1.x1 |  |
| 2 | Handover with PSCell from NR SA to EN-DC with parallel processing | NR SA-EUTRA handover with known target PCell Source Pcell in FR1 | SMTC of target known PSCell is NOT present in RRCConnectionReconfiguration Target PSCell in FR1 | To verify delay requirements for handover with PSCell from NR SA to EN-DC with parallel processing | A.6.3.1.x2 |  |
| 3 | Handover with PSCell from NR SA to EN-DC with sequential processing | NR SA-EUTRA handover with known target PCell Source Pcell in FR1 | SMTC of target unknown PSCell is present in RRCConnectionReconfiguration Target PSCell in FR2 | To verify delay requirements for handover with PSCell from NR SA to EN-DC with sequential processing | A.7.3.1.x1 | Option 1: No test case Option 2: Only requirements for PSCell are verified |
| 4 | Handover with PSCell from NR SA to EN-DC with parallel processing | NR SA-EUTRA handover with known target PCell Source Pcell in FR1 | SMTC of target known PSCell is NOT present in RRCConnectionReconfiguration Target PSCell in FR2 | To verify delay requirements for handover with PSCell from NR SA to EN-DC with parallel processing | A.7.3.1.x2 | Option 1: No test case Option 2: Only requirements for PSCell are verified |
| 5 | Handover with PSCell from NE-DC to NE-DC with known target PSCell | Intra-frequency handover from FR1 to FR1 with known target PCell | Target known E-UTRA PSCell | To verify delay requirements for handover with PSCell from NE-DC to NE-DC | A.4A.1.x1 |  |
| 6 | Handover with PSCell from NE-DC to NE-DC with unknown target PSCell | Inter-frequency handover from FR1 to FR1 with known target PCell | Target unknown E-UTRA PSCell | To verify delay requirements for handover with PSCell from NE-DC to NE-DC | A.4A.1.x2 |  |
| 7 | Handover with PSCell from NR-DC to NR-DC with parallel processing | Inter-frequency handover from FR1 to FR1 with unknown target PCell | SMTC of target known PSCell is present in reconfigurationWithSync Target PSCell in FR2 | To verify delay requirements for handover with PSCell from NR-DC to NR-DC with parallel processing | A.7.3.1.x3 | Option 1: No test case Option 2: Only requirements for PSCell are verified |
| 8 | Handover with PSCell from NR-DC to NR-DC with sequential processing | Inter-frequency handover from FR1 to FR1 with unknown target PCell | SMTC of target unknown PSCell is present in targetcellSMTC-SCG-r16 but not in reconfigurationWithSync Target PSCell in FR2 | To verify delay requirements for handover with PSCell from NR-DC to NR-DC with sequential processing | A.7.3.1.x4 | Option 1: No test case Option 2: Only requirements for PSCell are verified |
| 9 | Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1 | E-UTRA intra-frequency handover | Source PSCell in FR1 Target known PSCell in FR1 | To verify delay requirements for handover with PSCell from EN-DC to EN-DC | A.4.3.x1 |  |
| 10 | Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR2 | E-UTRA inter-frequency handover | Source PSCell in FR1 Target known PSCell in FR2 | To verify delay requirements for handover with PSCell from EN-DC to EN-DC | A.5.3.x1 | Option 1: No test case Option 2: Only requirements for PSCell are verified |
| 11 | Are additional test cases needed? |  |  |  |  |  |

- Proposals

  - Proposal 1: Remove TC3 and TC4 in NR SA to EN-DC since it is similar as TC7 and TC8 in NR-DC to NR-DC

  - Proposal 2: Update the target PCell to unknown cell in TC6 to raise the test coverage for HO with PSCell.

  - Proposal 3: Only focus on PSCell addition / change delay verification in TC7, TC8, TC9 and TC10.

  - Proposal 4: For the test case of HO with PSCell from NR-DC to NR-DC, do not define test cases for sequential processing case.

- Recommended WF

  - Discuss the proposals in the 1st round. Additional comments on adding/removing/changing/confirming test cases in the list are encouraged.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Proposal 1: TC 7 and 8 shall be removed due to FR1+FR2 testability issue Proposal 2: fine with proposal 2. Proposal 3: fine with proposal 3. Proposal 4: same comment as to proposal 1, the whole test for FR1+FR2 NR-DC could be removed due to FR1+FR2 testability issue. |
| Huawei | For TC1, as discussed in issue 2-4, both target PCell and target PSCell should be unknown, For TC3, if the target PScell is in FR2, it seems sequential case cannot be performed due to testability issue. For TC4, due to testability issue, if the RRC command is received in FR1, then UE may fail the test due to missing the RRC command. For TC 7, the test case may be performed when UE receives the RRM command in FR2 SCG via SRB split. But the performance of HO (RACH transmission in FR1) can not be tested. Thus, we don’t think it make much sense to only test PSCell change delay as it may lead to some unpredictable behavior when the SNR condition in PCell is not controllable. For TC 8, similar as TC 3. For TC 10, when the source PScell is in FR1 and the target PSCell is in FR2, due to the testability issue, TE may miss the measurement report sent by UE, and UE may miss the RRC command. So, there may be problem even only PScell change is to be defined. |
| Intel | Proposal 1: Depends on discussion of issue 2-1. Proposal 2: Fine. Proposal 3: For TC8 with sequential processing, there may be some problem due to FR1+FR2 testability issue. Proposal 4: Agree. |
| Ericsson | Depends on the other issues conclusions. |
| Qualcomm | It depends on the other issues, but we prefer to remove all TC where Pcell and PSCell status are not matched (e.g Pcell known, PsCell unknown or vice versa) to reduce the amount of test cases. |
| OPPO | OK with P2 and P3. We are ok that FR1+FR2 NR-DC tests and tests where Pcell and PSCell status are not matched are removed. |
| MTK | Proposal 1,3,4: TC3,4,7,8,10 might need to be removed due to FR1+FR2 testability issue. Proposal 2: ok |
| Nokia | We are fine with the proposals. We should define the minimum test cases to cover all core requirements instead of list all possible test cases. |
| vivo | P1: Depends on issue 2-1 P2: For parallel processing HO w PSCell, we prefer to test known case only. P3: Depends on issue 2-1 for TC 7,8,10. For TC9, if only PSCell change/addition is verified, it can be verified in EN-DC PSCell change testing. Why do we test is again? P4: We are fine to define test case for sequential case. Moreover, TC 6 can be changed to known PSCell, if P5 in 2-1 is agreeable. |

Companies’ views collection for 1st round

Open issues

Comments are collected in section 2.2.

CRs/TPs comments collection

Companies are encouraged to provide comments on the structure and technical part of requirements.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2207772 Apple |  |
| R4-2208064 Intel Corporation |  |
| R4-2208106 Xiaomi |  |
| R4-2208177 CATT |  |
| R4-2208351 OPPO |  |
| R4-2208503 Nokia, Nokia Shanghai Bell |  |
| R4-2208943 Huawei, Hisilicon |  |
| R4-2209496 vivo |  |
| R4-2209991 MediaTek Inc. |  |
| R4-2210136 Ericsson |  |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Sub-topic 2-1 Test cases coverage, design and configurations

|  | Status summary |
| --- | --- |
| Issue 2-1: Test cases design principle | Tentative agreements: - Define test cases to cover both parallel processing and sequential processing cases. - Define test cases for the scenarios including from NR SA to EN-DC, from EN-DC to EN-DC, from NE-DC to NE-DC - FFS test cases for scenarios with FR1+FR2. Candidate options: Issue 2-1-1: Test cases for scenarios with FR1+FR2 - Proposals - Option 1: - For test cases related to FR1-FR2 joint testing, test case design is delayed until testability issues are solved. - Option 2: - For test cases related to FR1-FR2 joint testing, test cases are introduced in Rel-17 with additional applicability rules. Recommendations for 2nd round: Confirm tentative agreements. Discuss and conclude Issue 2-1-1 in the GTW session. Proposal 3, Proposal 5 and Proposal 7 will be discussed in the GTW. If no conclusion can be made, it is recommended to discuss in the test cases list. |
| Issue 2-2: General test cases configuration | Tentative agreements: - If target PSCell is unknown, single SMTC configuration of target unknown PScell from either source Pcell MO or source PSCell MO can be configured. - The requirement for HO with PSCell can be verified by testing the transmission time of PRACH to target PCell and target PSCell. - For the test configuration of test cases for HO with PSCell, following configurations is considered: - For FR1 - 15 kHz SSB SCS with 10 MHz bandwidth for FDD - 15 kHz SSB SCS with 10 MHz bandwidth for TDD - 30 kHz SSB SCS with 40 MHz bandwidth for TDD - For FR2 - 120 kHz SSB SCS with 100 MHz bandwidth for TDD - Note：Whether FR1+FR2 test cases will be introduced in Rel-17 are discussed separately. Candidate options: - Proposals - Proposal 2: For the case when both target PCell and target PSCell are known, the test system sends RRC message implying HO with PSCell only after it receives 2 MRs. Recommendations for 2nd round: Further discuss Proposal 2 in the 2nd round |
| Issue 2-3: Test cases configuration for HO with PSCell from EN-DC to EN-DC | Tentative agreements: - The test can consist of three successive time periods, T1, T2 and T3. The target cells turn off during T1, and turn on during T2. UE will send Event A3 reports and receive RRC message implying handover with PSCell during T2, and perform the handover with PSCell during T3. - For known target PCell and known target PSCell - The test handover requirement for handover with PSCell is defined as 60ms. - The test requirement for PSCell change delay is defined as 87ms. Candidate options: - Proposals - Proposal 1: The configurations for this test case can refer to the configurations for E-UTRA intra-frequency handover and for NR intra-frequency handover. - Proposal 2: There are four cells in the test case, two original cells on E-UTRA carrier and NR carrier and two target cells on the same carriers respectively. Recommendations for 2nd round: Discuss Proposal 1 and Proposal 2 together with test cases list. |
| Issue 2-4: Test cases configuration for HO with PSCell from NR SA to EN-DC | Tentative agreements: - For the test case of HO with PSCell from NR SA to EN-DC, the sequential processing case should be verified when target PCell and target PSCell are unknown and the target PSCell is in FR1. Recommendations for 2nd round: The issue is closed. |
| Issue 2-5: Test cases list for HO with PSCell | No obvious agreement. Whether test cases for scenarios with FR1+FR2 should be introduced depending on Issue 2-1-1 Tentative agreements: None Candidate options: - Proposals TC No.; TC; PCell Handover; PSCell addition/change; Purpose; Clause. 1; Handover with PSCell from NR SA to EN-DC with sequential processing; NR SA-EUTRA handover with unknown target PCell Source Pcell in FR1; SMTC of target unknown PSCell is present in RRCConnectionReconfiguration Target PSCell in FR1; To verify delay requirements for handover with PSCell from NR SA to EN-DC with sequential processing; A.6.3.1.x1. 2; Handover with PSCell from NR SA to EN-DC with parallel processing; NR SA-EUTRA handover with known target PCell Source Pcell in FR1; SMTC of target known PSCell is NOT present in RRCConnectionReconfiguration Target PSCell in FR1; To verify delay requirements for handover with PSCell from NR SA to EN-DC with parallel processing; A.6.3.1.x2. [3]; Handover with PSCell from NR SA to EN-DC with sequential processing; NR SA-EUTRA handover with known target PCell Source Pcell in FR1; SMTC of target unknown PSCell is present in RRCConnectionReconfiguration Target PSCell in FR2; To verify delay requirements for handover with PSCell from NR SA to EN-DC with sequential processing; A.7.3.1.x1. [4]; Handover with PSCell from NR SA to EN-DC with parallel processing; NR SA-EUTRA handover with known target PCell Source Pcell in FR1; SMTC of target known PSCell is NOT present in RRCConnectionReconfiguration Target PSCell in FR2; To verify delay requirements for handover with PSCell from NR SA to EN-DC with parallel processing; A.7.3.1.x2. 5; Handover with PSCell from NE-DC to NE-DC with known target PSCell; Intra-frequency handover from FR1 to FR1 with known target PCell; Target known E-UTRA PSCell; To verify delay requirements for handover with PSCell from NE-DC to NE-DC; A.4A.1.x1. 6; Handover with PSCell from NE-DC to NE-DC with unknown target PSCell; Inter-frequency handover from FR1 to FR1 with known target PCell; Target unknown E-UTRA PSCell; To verify delay requirements for handover with PSCell from NE-DC to NE-DC; A.4A.1.x2. [7]; Handover with PSCell from NR-DC to NR-DC with parallel processing; Inter-frequency handover from FR1 to FR1 with unknown target PCell; SMTC of target known PSCell is present in reconfigurationWithSync Target PSCell in FR2; To verify delay requirements for handover with PSCell from NR-DC to NR-DC with parallel processing; A.7.3.1.x3. [8]; Handover with PSCell from NR-DC to NR-DC with sequential processing; Inter-frequency handover from FR1 to FR1 with unknown target PCell; SMTC of target unknown PSCell is present in targetcellSMTC-SCG-r16 but not in reconfigurationWithSync Target PSCell in FR2; To verify delay requirements for handover with PSCell from NR-DC to NR-DC with sequential processing; A.7.3.1.x4. 9; Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1; E-UTRA intra-frequency handover; Source PSCell in FR1 Target known PSCell in FR1; To verify delay requirements for handover with PSCell from EN-DC to EN-DC; A.4.3.x1. [10]; Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR2; E-UTRA inter-frequency handover; Source PSCell in FR1 Target known PSCell in FR2; To verify delay requirements for handover with PSCell from EN-DC to EN-DC; A.5.3.x1. 11; Are additional test cases needed?. Recommendations for 2nd round: Discuss the test cases in a case by case manner by taking agreements in the 1st round into consideration. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2208106 | to be Revised |
| R4-2207772 | to be Revised |
| R4-2208064 | to be Revised |
| R4-2208177 | to be Revised |
| R4-2208351 | to be Revised |
| R4-2208503 | to be Revised |
| R4-2208943 | to be Revised |
| R4-2209496 | to be Revised |
| R4-2209991 | to be Revised |
| R4-2210136 | to be Revised |

Discussion on 2nd round

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Following agreements were made in the GTW session.

Agreement:

- Define test cases for the scenarios including from NR SA to EN-DC, from EN-DC to EN-DC (LTE+FR1 NR), from NE-DC to NE-DC.

- Define test cases to cover both parallel processing and sequential processing cases.

- For test cases related to FR1-FR2 joint testing,

  - Option 1: Test case design is delayed until testability issues are solved

  - Option 2: Introduce the test case and define the applicability

  - Option 3: Do not introduce the test

- Do not verify all intra-frequency and inter-frequency HO scenarios in the HO w PSCell test cases.

Following tentative agreements were made during the 1st round discussion.

Issue 2-2: General test cases configuration

- If target PSCell is unknown, single SMTC configuration of target unknown PScell from either source Pcell MO or source PSCell MO can be configured.

- The requirement for HO with PSCell can be verified by testing the transmission time of PRACH to target PCell and target PSCell.

- For the test configuration of test cases for HO with PSCell, following configurations is considered:

  - For FR1

    - 15 kHz SSB SCS with 10 MHz bandwidth for FDD

    - 15 kHz SSB SCS with 10 MHz bandwidth for TDD

    - 30 kHz SSB SCS with 40 MHz bandwidth for TDD

  - For FR2

    - 120 kHz SSB SCS with 100 MHz bandwidth for TDD

    - Note：Whether FR1+FR2 test cases will be introduced in Rel-17 are discussed separately.

Issue 2-3: Test cases configuration for HO with PSCell from EN-DC to EN-DC

- The test can consist of three successive time periods, T1, T2 and T3. The target cells turn off during T1, and turn on during T2. UE will send Event A3 reports and receive RRC message implying handover with PSCell during T2, and perform the handover with PSCell during T3.

- For known target PCell and known target PSCell

  - The test handover requirement for handover with PSCell is defined as 60ms.

  - The test requirement for PSCell change delay is defined as 87ms.

Issue 2-4: Test cases configuration for HO with PSCell from NR SA to EN-DC

- For the test case of HO with PSCell from NR SA to EN-DC, the sequential processing case should be verified when target PCell and target PSCell are unknown and the target PSCell is in FR1.

If any further comments to the above tentative agreements, it can be provided in the table below.

- 2nd round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Fine with tentative agreements. |
| Huawei | Fine with tentative agreements. |
| Intel | Fine with tentative agreements. |
| Nokia | We are fine with the tentative agreements. |
| CATT | Fine with the tentative agreements. |
| Ericsson | Agree with tentative agreements |
| vivo | Support the tentative agreements |
| MTK | Fine with tentative agreements. |
| Qualcomm | We are fine with tentative agreements. |
| OPPO | Fine with the tentative agreements |

Issue 2-1-1: Test cases design principle - FR1+FR2 test cases

- Proposals

  - Option 1: Test case design is delayed until testability issues are solved

  - Option 2: Introduce the test case and define the applicability

  - Option 3: Do not introduce the test

- Recommended WF

  - Discuss the proposals in the 2nd round.

- 2nd round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1. We still think the FR1+FR2 TC shall be introduced after we finalize the FR1+FR2 testability issue: configuration may come from FR1 and it would probably cause problem for UE to receive such configuration in the test. |
| Huawei | Fine option 1/3. But share different understanding as Apple. I think the issue is not from the configuration but for PRACH transmission. If there is serving cell in FR2 (i.e. EN-DC with FR2 PSCell or NR-DC, or NR SA with FR2 SCell), the RRC command can be transmitted via FR2. We think the problem is UE needs to transmit PRACH in both FR1 and FR2. It doesn’t make much sense to only verify FR2 PSCell delay. |
| Intel | Prefer option 1. However, similar comments as the 1st round, in legacy, we have test case for NR PSCell addition(FR2) delays under EN-DC, which is FR1+FR2 test. Therefore, for FR1+FR2 paralleling processing, is it possible that we only test PSCell change delay which is only on FR2? |
| Nokia | We are fine with Option 1, FR1+FR2 test cases need to be specified to verify the core requirements. However, with the FR1+FR2 testability issues, currently it is hard for test vendors to test FR1+FR2. We can come back when the testability issues are solved. Furthermore, the FR1+FR2 testability issues may impact other WIs, RAN4 would need to have a same rule for this same situation. |
| Ericsson | There are pros and cons to proposal 2. If the testability issue is resolved, then there may be large number of tests to be defined in one WI. However, without knowing a time frame for a test method readiness, defining test case may consume current time resources in RAN4. We are fine with all the options. But we can follow same approach for all the test cases across WI for FR1+FR2. |
| vivo | Support option 1. |
| MTK | We are fine with options 1 and 3. |
| Qualcomm | We support option2. Test can be designed to verify the core requirements, but test requirement can be delayed until testability issue is resolved. We think it will be difficult to design new test cases in the future for R17 test cases. Once we design test cases at this WI and delay to test until testability issue is resolved, it can be easier to maintain the test cases after it is resolved rather than creating new test cases in the future. |
| OPPO | Prefer option 1. |

Issue 2-2: General test cases configuration

- Proposals

  - Proposal 2: For the case when both target PCell and target PSCell are known, the test system sends RRC message implying HO with PSCell only after it receives 2 MRs.

- Recommended WF

  - Discuss the proposal in the 2nd round.

- 2nd round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We prefer to select TC for unknown only as the worst case, but can compromise to proposal 2 if majority companies agree on it. |
| Huawei | Fine with proposal 2. |
| Nokia | For proposal 2, are the 2 MRs only configured in test cases and impact test cases? If it is only for test cases, we are fine. |
| CATT | Fine with proposal 2. |
| Ericsson | Similar view as Nokia. |
| vivo | Support proposal 2. To Nokia and Ericsson, Yes. In our view, in test case, normally the most typical scenarios for this feature are tested. Unknown case seems not the typical scenario but we may need confirmation from NW vendors and operators. We are also fine with other ways to implement the test. We have no strong view on this. |
| MTK | Fine with proposal 2. |
| Qualcomm | We support proposal 2 for known target cell scenarios. |
| OPPO | Proposal 2 is fine. |

Issue 2-5: Test cases list for HO with PSCell

- Proposals

  - TC #1: From SA to EN-DC (All cells are in FR1)

    - PCell: unknown target PCell

    - PSCell: unknown target PSCell

    - Purpose: To verify delay requirements for sequential processing

  - TC #2: From SA to EN-DC (All cells are in FR1)

    - PCell: known target PCell

    - PSCell: known target PSCell

    - Purpose: To verify delay requirements for parallel processing

  - TC #5: From NE-DC to NE-DC (All cells are in FR1)

    - PCell: Intra-frequency handover from FR1 to FR1, known target PCell

    - PSCell: known target PSCell

    - Purpose: To verify delay requirements for known PCell and known PSCell combination

  - TC #6: From NE-DC to NE-DC (All cells are in FR1)

    - PCell: Inter-frequency handover from FR1 to FR1, known target PCell

    - PSCell: unknown target PSCell

    - Purpose: To verify delay requirements for known PCell and unknown PSCell combination

  - TC #9: From EN-DC to EN-DC (All cells are in FR1)

    - PCell: E-UTRA intra-frequency handover, known target PCell

    - PSCell: known target PSCell

    - Purpose: To verify delay requirements for known PCell and known PSCell combination

- Recommended WF

  - Companies are encouraged to provide comments on each test case whether it is agreeable, or any revision is needed, or additional test cases are needed for test coverage, etc.

- 2nd round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Fine to narrow down the TC list as in proposal. Even though we think the unknown case should be tested for the worst case, we can compromise to the above list. |
| Huawei | Share similar view as Apple. If both PCell and PSCell are known, then there is no much procedure needs to be down in “parallel”. For TC9, slightly prefer to change to both unknown case. |
| Intel | No strong view. Fine with Recommended WF. |
| Nokia | We share similar view as Apple and Huawei, for the TC#6, Is it the case listed in email sent to RAN4 reflector before meeting? If it is the same case, TC#6 should be NE-DC to NE-DC instead of SA to EN-DC. Considering the test coverage, we would suggest updating to unknown target PCell in TC#6. |
| CATT | Considering the test coverage, we would suggest defining two test cases for each DC mode. Based on this understanding, we agree with Nokia to update TC#6 as unknown PCell and unknown PSCell combination from NE-DC to NE-DC. And suggest adding another test case TC#X as: unknown PCell and unknown PSCell combination from EN-DC to EN-DC |
| Ericsson | We have same comment as Nokia regarding TC#6. It should be for NE-DC to NE-DC. |
| vivo | To Nokia, Ericsson and CATT, yes it is a typo. TC#6 should be NE-DC to NE-DC Support the proposal. |
| MTK | No strong view. Fine with Recommended WF. |
| Qualcomm | We are fine with test coverages. |
| OPPO | Fine with Recommended WF. |

Summary on 2nd round (if applicable)

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on further RRM enhancement for NR and MR-DC – HO with PSCell | vivo | Capture agreements in the meeting. Capture open issues, if any. |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2207769 |  | On remaining issues of HO with PScell | Apple | Noted |  |
| R4-2208170 |  | Discussion on issues for requirement of HO with PSCell | CATT | Noted |  |
| R4-2210131 |  | Discussion RRM requirements for handover with PSCell for NR-U | Ericsson | Noted |  |
| R4-2207770 |  | CR on HO with PSCell for NE-DC to NE-DC in TS38.133 R17 | Apple | Revised |  |
| R4-2207771 |  | CR on HO with PSCell for EN-DC to EN-DC in TS36.133 R17 | Apple | Revised |  |
| R4-2208171 |  | Completing requirement of HO with PSCell | CATT | Revised |  |
| R4-2208500 |  | Correction on HO with PSCell requirements in 38133 | Nokia, Nokia Shanghai Bell | Merged | Merged to CR R4-2208171 |
| R4-2208501 |  | Correction on HO with PSCell requirements in 36133 | Nokia, Nokia Shanghai Bell | Return to |  |
| R4-2208530 |  | CR on Handover with PSCell | CMCC | Merged | Merged to CR R4-2208171 |
| R4-2208937 |  | CR on HO with PSCell requirements | Huawei, Hisilicon | Merged | Merged to CR R4-2208171 |
| R4-2209494 |  | CR on R17 core requirements for HO with PSCell | vivo | Revised |  |
| R4-2210132 |  | CR on requirements for HO with PSCell when PSCell is on CCA in EN-DC to EN-DC scenario | Ericsson | Revised |  |
| R4-2210133 |  | CR on requirements for HO with PSCell when PSCell is on CCA in NR SA to EN-DC scenario | Ericsson | Revised |  |
| R4-2208176 |  | Discussion on test for HO with PSCell | CATT | Noted |  |
| R4-2208458 |  | RRM performance requirements for HO with PSCELL | Qualcomm Incorporated | Withdraw |  |
| R4-2208502 |  | discussion on HO with PSCell performance requirements | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2208518 |  | Discussion on test cases for HO with PSCell | CMCC | Noted |  |
| R4-2208942 |  | Discussion on performance requirements for HO with PSCell | Huawei, Hisilicon | Noted |  |
| R4-2209495 |  | Discussion on test cases for R17 requirements for HO with PSCell | vivo | Noted |  |
| R4-2210114 |  | RRM performance requirements for HO with PSCELL | Qualcomm Incorporated | Noted |  |
| R4-2208106 |  | CR on test case for Handover with PSCell from NR SA to EN-DC with sequential processing | Xiaomi | Revised |  |
| R4-2207772 |  | Draft CR on TC for HO with PSCell from NR-SA to EN-DC with parallel processing and known FR2 PSCell in TS38.133 R17 | Apple | Revised |  |
| R4-2208064 |  | DraftCR to TS 38.133: Handover with PSCell from NR-DC to NR-DC with sequential processing | Intel Corporation | Revised |  |
| R4-2208177 |  | Test case of handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1 | CATT | Revised |  |
| R4-2208351 |  | draft CR on HO with PSCell from NR SA to EN-DC with parallel processing(TC2) | OPPO | Revised |  |
| R4-2208503 |  | dratCR on test case for HO with PSCell – TC10 | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2208943 |  | Draft CR on TC for HO with PSCell from NR SA to EN-DC | Huawei, Hisilicon | Revised |  |
| R4-2209496 |  | draft CR on test cases for Handover with PSCell from NE-DC to NE-DC with known target PSCell | vivo | Revised |  |
| R4-2209991 |  | Draft CR 38.133 on the test case of handover with PSCell from NR-DC to NR-DC with parallel processing | MediaTek Inc. | Revised |  |
| R4-2210136 |  | TC for HO with PSCell from NE-DC to NE-DC with unknown target PSCell | Ericsson | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2210583 |  | WF on further RRM enhancement for NR and MR-DC – HO with PSCell | vivo | Agreeable |  |
| R4-2207770 | R4-2210997 | CR on HO with PSCell for NE-DC to NE-DC in TS38.133 R17 | Apple | Agreeable |  |
| R4-2207771 | R4-2210998 | CR on HO with PSCell for EN-DC to EN-DC in TS36.133 R17 | Apple | Agreeable |  |
| R4-2208171 | R4-2210999 | Completing requirement of HO with PSCell | CATT | Agreeable |  |
| R4-2208501 | R4-2211191 | Correction on HO with PSCell requirements in 36133 | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2209494 | R4-2211000 | CR on R17 core requirements for HO with PSCell | vivo | Agreeable |  |
| R4-2210132 | R4-2211001 | CR on requirements for HO with PSCell when PSCell is on CCA in EN-DC to EN-DC scenario | Ericsson | Agreeable |  |
| R4-2210133 | R4-2211002 | CR on requirements for HO with PSCell when PSCell is on CCA in NR SA to EN-DC scenario | Ericsson | Agreeable |  |
| R4-2211170 |  | LS on handling of RACH occasion collision between PCell and PSCell for HO with PSCell for NR-U | Ericsson | Agreeable |  |
| R4-2208106 | R4-2211003 | CR on test case for Handover with PSCell from NR SA to EN-DC with sequential processing | Xiaomi | Postponed | R4-2211003 should be withdraw. |
| R4-2207772 | R4-2211004 | Draft CR on TC for HO with PSCell from NR-SA to EN-DC with parallel processing and known FR2 PSCell in TS38.133 R17 | Apple | Postponed | R4-2211004 should be withdraw. |
| R4-2208064 | R4-2211005 | DraftCR to TS 38.133: Handover with PSCell from NR-DC to NR-DC with sequential processing | Intel Corporation | Postponed | R4-2211005 should be withdraw. |
| R4-2208177 | R4-2211006 | Test case of handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1 | CATT | Agreeable |  |
| R4-2208351 | R4-2211007 | draft CR on HO with PSCell from NR SA to EN-DC with parallel processing(TC2) | OPPO | Agreeable |  |
| R4-2208503 | R4-2211008 | dratCR on test case for HO with PSCell – TC10 | Nokia, Nokia Shanghai Bell | Postponed | R4-2211008 should be withdraw. |
| R4-2208943 | R4-2211009 | Draft CR on TC for HO with PSCell from NR SA to EN-DC | Huawei, Hisilicon | Agreeable |  |
| R4-2209496 | R4-2211010 | draft CR on test cases for Handover with PSCell from NE-DC to NE-DC with known target PSCell | vivo | Agreeable |  |
| R4-2209991 | R4-2211011 | Draft CR 38.133 on the test case of handover with PSCell from NR-DC to NR-DC with parallel processing | MediaTek Inc. | Postponed | R4-2211011 should be withdraw. |
| R4-2210136 | R4-2211012 | TC for HO with PSCell from NE-DC to NE-DC with unknown target PSCell | Ericsson | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Apple | Jie Cui | Jie_cui@apple.com |
| Huawei | Zhongyi Shen | shenzhongyi3@huawei.com |
| Ericsson | Venkat | Venkatarao.gonuguntla@ericsson.com |
| CATT | Qiuge Guo | guoqiuge@catt.cn |
| MTK | Ogeen Toma | Ogeen.hanna@mediatek.com |
| Qualcomm | Hyunwoo Cho | hyuncho@qti.qualcomm.com |
| OPPO | Roy Hu | hurongyi@oppo.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)