3GPP TSG-RAN WG4 Meeting # 103-e 												R4-2210477

Electronic Meeting, May 9 - May 20, 2022

Agenda item:			9.9.1.3, 9.9.2.3

Source:	Moderator (CATT)

Title:	Email discussion summary for [103-e][208] NR_RRM_enh2_3

Document for:	Information

Introduction

This document includes the discussions in agenda item 9.9.1.3 and 9.9.2.3 which contain the following topics

- Topic #1: PUCCH SCell activation/deactivation core requirements maintenance

- Topic #2: PUCCH SCell activation/deactivation performance requirements

Topic #1: PUCCH SCell activation/deactivation core requirements maintenance

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207773 | Apple | Proposal 1: The known condition of PL-RS for known PUCCH SCell could be defined as (the different part form legacy definition is highlighted in yellow): The pathloss reference signal is known for known PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. - Pathloss reference signal activation command is received within 1280 ms upon the last transmission of the RS resource for L3 measurement - The UE has sent at least one L3 RSRP report for the target pathloss reference signal before the pathloss reference signal activation command - The target pathloss reference signal remains detectable during the PUCCH SCell activation period - SNR of the target pathloss reference signal≥-3dB - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period - SNR of the associated SSB ≥-3dB Otherwise, the pathloss reference signal is unknown. Proposal 2: The known condition of PL-RS for unknown PUCCH SCell could be defined as (the different part form legacy definition is highlighted in yellow): The pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L1-RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. - Pathloss reference signal activation command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least one L1-RSRP report for the target pathloss reference signal before the pathloss reference signal activation command - The target pathloss reference signal remains detectable during the PUCCH SCell activation period - SNR of the target pathloss reference signal≥-3dB - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period - SNR of the associated SSB ≥-3dB Otherwise, the pathloss reference signal is unknown. Proposal 3: when PL-RS of target PUCCH SCell is known, the X=5 sample measurement time is always considered and no need to consider condition of ‘maintain’ or ‘not maintain’. |
| R4-2207774 | Apple | CR on PUCCH SCell activation in TS38.133 R17 |
| R4-2207956 | Qualcomm Incorporated | PUCCH SCell activation without concurrently to be activated multiple SCells Proposal 1: When PL-RS of target FR1 PUCCH SCell is known, the same number of samples for PL-RS measurement as FR2 should be added to the FR1 PUCCH SCell activation latency requirement. And the number of samples is 5. Proposal 2: Update the previous agreement about FR2 PL-RS measurement, from a sequential processing to a parallel processing, i.e. PL-RS can be measured in parallel with “DL CSI-RS reception/processing” and “UL TA acquisition”. - Previous agreement: THARQ+Tactivation_time+[X]+max{(Tfirst_available_CSI+TCSI_processing),(T1+T2+T3)}+TCSI_reporting_after - Proposed udpate: THARQ+Tactivation_time+max{(Tfirst_available_CSI+TCSI_processing),(T1+T2+T3),[X]}+TCSI_reporting_after Proposal 3: RAN4 does not define PUCCH SCell activation requirements that require UE to maintain a measurement of PL-RS configured in a different activated serving cell in the same band as the deactivated PUCCH SCell. PUCCH SCell activation in parallel with multiple DL SCell Proposal 4-1: Multiple DL SCells to be activated along with PUCCH SCell shall be in the same PUCCH group as the PUCCH SCell. Any to be activated DL SCell in parallel with PUCCH SCell should not be activated earlier than the PUCCH SCell. Proposal 4-2: For the activation latency requirement of multiple DL SCells being activated in parallel with PUCCH SCell, the legacy requirements are reused with the following update. - TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] which cannot be earlier than the PUCCH SCell activation completion Proposal 5-1: For the activation latency requirement of multiple DL SCells being activated in parallel with PUCCH SCell, do not consider the following scenarios: - FR1 unknown non-contiguous CA scenario which includes N1*Trs in the latency requirement - One of serving cells is activated in the same band as to be activated FR2 PUCCH SCell - Unknown PUCCH SCell with known to-be-activated DL SCells Proposal 5-2: If FR2 PUCCH SCell is known, DL SCells to be activated in parallel with the PUCCH SCell in the same band are considered known FR2 SCell in terms of latency requirement, i.e. no L1-RSRP measurement and report are included in the requirement. |
| R4-2208063 | Intel Corporation | Proposal 1: When PL-RS of target PUCCH Scell is known, 5 sample measurement time is always needed. Proposal 2: No need to have restrictions on the relation between the associated RS for TCI state, PL-RS and spatial relation indication. Proposal 3: The delay requirements for PUCCH SCell activation with multiple DL Scells are: - the normal SCell activation delay requirement for deactivated SCell with multiple Downlink SCells defined in clause 8.3.7 of current specification 38.133 apply for other downlink Scells. - PUCCH Scell activation delay requirements can be derived from single PUCCH SCell activation delay by replacing Tactivation_time with Tactivation_time_multiple_scells |
| R4-2208087 | Nokia, Nokia Shanghai Bell | Observation #1: The PL-RS configuration or UL spatial relation configuration are not mandatory for pathloss estimation in FR2. Observation #2: The PL-RS i.e. the RS resource used for calculating the downlink pathloss reference is among the RS resources used for L3 or L1-RSRP measurement reporting, or QCLed (with Type D) to the RS resources. Proposal 1: If PUCCH SCell is known in FR2, the reported L3 measurement results can be reused for pathloss estimation and additional PL-RS measurement is not needed during PUCCH SCell activation. Observation #3: PL-RS is maintained if the UE has performed intra-frequency L3 measurement on deactivated PUCCH SCell according to TS 38.133 section 9.2.5.1. Proposal 2: If PUCCH SCell is unknown in FR2, additional PL-RS measurement delay needs to be introduced only when the PL-RS is not maintained. Proposal 3: If PUCCH SCell is unknown in FR2, the additional PL-RS measurements are performed in parallel with CSI measurements and random access procedure if applicable, and the PUCCH SCell activation is updated as below: Tdelay_PUCCH_SCell = Tactivation_time + [X] + max ([X], (TFirst_available_CSI + TCSI_processing), (T1+T2+T3)) + TCSI_reporting_after where [X=5] sample measurement time is introduced in FR2 when PL-RS of target PUCCH SCell is unknown in FR2 - [X] = 0 if PL-RS is maintained and [X] = 5 if PL-RS is not maintained. Proposal 4: The PUCCH SCell activation shall be prioritized over the other being-activated DL SCells when they are activated by a single MAC command. Proposal 5: The SCell activation delay for PUCCH SCell with multiple DL SCells is the same as the single PUCCH SCell activation delay in clause 8.3.12 i.e. Tdelay_PUCCH_SCell. Proposal 6: For the other being-activated DL SCells in the same MAC command, the existing activation delay requirement for deactivated SCell with multiple DL SCells defined in clause 8.3.7 i.e. Tactivation_time_multiple_scells can be reused. Observation #4: For intra-band CA, the UE is not expected to be scheduled with PRACH on PUCCH SCell and PUCCH/PUSCH/SRS on the other active serving cells simultaneously. Proposal 7: The PUCCH SCell activation delay requirement (Tdelay_PUCCH SCell) shall apply provided that: - the RA on PUCCH SCell is not interrupted by the PUCCH/PUSCH/SRS on other activated serving cells otherwise additional delay to activate the SCell is expected. |
| R4-2208088 | Nokia, Nokia Shanghai Bell | 38.133 draftCR on PUCCH SCell activation delay |
| R4-2208172 | CATT | Proposal 1: Replace “FFS under what condition the [X] = 0 or [X] = 5” with “X =5 if the last transmission of the RS resource used for L3 RSRP/L1-RSRP measurement reporting is configured as aperiodic measurement reporting, or the last one measurement reporting of periodic/semi-persistent measurement reporting, otherwise, X=0.” Proposal 2: Not need to discuss further the Issue 1-2-6 (Relation between the associated RS for TCI state, PL-RS and spatial relation indication?). Proposal 3: Agree the requirements listed in WF [1] for the case that the downlink SCells are within different PUCCH group from the activated PUCCH SCell. For the case that downlink SCells are within the same PUCCH group with the activated PUCCH SCell, the normal SCell activation delay requirement for deactivated SCells will be specified same as that for PUCCH SCell with replacing Tactivation_time with Tactivation_time_multiple_scells. Proposal 4: Deleting the editor notes of “FFS on whether to define interruptions due to PUCCH SCell RACH has different SCS from spCell data/control channel” from specification. |
| R4-2208173 | CATT | CR on completing PUCCH SCell activation requirement |
| R4-2208349 | OPPO | Proposal 1: Prefer extra delay time could be 5 samples no matter whether PL-RS is not maintained or not before Scell is activated. Observation 1: CSI report of other SCells except PUCCH SCell could be in different PUCCH group. Proposal 2: Define the PUCCH Scell activation delay requirements and SCell activation delay for other downlink Scells separately. |
| R4-2208462 | MediaTek Inc. | Observation 1: For Tuncertainty_MAC of single PUCCH SCell activation, compared with R15/R16 legacy SCell, the main difference is the UL spatial relation indication is additionally considered. Proposal 1: For PUCCH SCell activation with multiple SCells, the Tuncertainty_MAC_multiple_scells is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and spatial relation, and SCell activation command of this unknown PUCCH SCell. Proposal 2: For PUCCH SCell activation with multiple Scells, the TSMTC_MAX_multiple_scells and TFirstSSB_MAX_multiple_scells should additionally consider PUCCH SCell . Observation 2: The PUCCH SCell should be activated no later than other SCell(s) in the same PUCCH group. Proposal 3: For PUCCH SCell activation with multiple Scells, the delay of TCSI_reporting for three target cells to be activated are as below: - PUCCH SCell: - same as single PUCCH SCell requirement, i.e. max ((TFirst_available_CSI + TCSI_processing), (T1+T2+T3)) + TCSI_reporting_after. - SCell in the different PUCCH group from PUCCH SCell: - same as legacy R15/R16 SCell. - SCell in the same PUCCH group as PUCCH SCell: - the summation of TCSI_reporting defined in legacy R15/R16 SCell and the PUCCH SCell’s CSI reporting time (max ((TFirst_available_CSI + TCSI_processing), (T1+T2+T3)) + TCSI_reporting_after). Observation 3: For R16 legacy multiple SCells activation requirement, no UE requirement applies when the target unknown cells to be activated does not have active serving cell(s) or known to-be-activated SCell(s) on the same band. Proposal 4: For PUCCH SCell activation with multiple Scells, no UE requirement applies when the unknown SCells (not including PUCCH SCell) to be activated does not have active serving cell(s) or known to-be-activated SCell(s) on the same band. Proposal 5: For PUCCH SCell activation with multiple Scells, RAN4 further study whether the unknown PUCCH SCell should be activated only on the band which contains active serving cell(s) or known to-be-activated SCell(s). Proposal 6: In the inter-band CA case, for the issue of interruption, it is up to UE implementation if network schedules concurrent transmission of PRACH and PUSCH/PUCCH/SRS for inter-band CA, and UE does not support parallelTxPRACH-SRS-PUCCH-PUSCH. Proposal 7: In the intra-band CA case, for the issue of interruption, no UE requirement applies when PRACH transmission is colliding with PUSCH/PUCCH/SRS. Proposal 8: For the PUCCH SCell activation, extra five measurement samples are needed regardless of the PL-RS is maintained or not. |
| R4-2208464 | MediaTek Inc. | CR on PUCCH SCell activation and deactivation delay requirements |
| R4-2208938 | Huawei, Hisilicon | Observation 1: Whether UE can simultaneously transmit PRACH on PUCCH SCell and other physical channels/signals on other activated serving cell is irrelative to SCS configuration. Observation 2: UE may not be able to simultaneously transmit PRACH on PUCCH SCell and other physical channels/signals on other activated serving cell for inter-band CA when UE is not capable of parallelTxPRACH-SRS-PUCCH-PUSCH. Proposal 1: Clarify that there could be interruption on UL transmission on other serving Cells when colliding with RACH transmission on PUCCH SCell if UE is not capable of parallelTxPRACH-SRS-PUCCH-PUSCH. Proposal 2: When PL-RS of target PUCCH SCell is known, the 5 sample measurement time is introduced for both FR1 and FR2. Proposal 3: No need to have restrictions on the relation between the associated RS for TCI state, PL-RS and spatial relation indication. Observation 3: SCell to be activated with PUCCH SCell could be within the primary PUCCH group or secondary PUCCh group. Proposal 4: For SCell to be activated with PUCCH SCell which is in the primary PUCCH group, legacy requirements for SCell activation with multiple SCells can apply. For SCell to be activated with PUCCH SCell which is in the secondary PUCCH group, extra delay to wait for UE accomplishing activation of PUCCH SCell needs to be considered in addition to legacy requirements for SCell activation with multiple SCells. |
| R4-2208939 | Huawei, Hisilicon | CR on PUCCH SCell activation |
| R4-2210134 | Ericsson | Proposal 1: RAN4 not to specify any relation or restriction between the associated RS for TCI state, PL-RS and spatial relation indication. Proposal 2: If the to be activated PUCCH SCell share the same PL-RS with the active serving cell or the PL-RS of active serving cell and the PL-RS of the to be activated PUCCH SCell are configured to be QCL Type-A, PL-RS can consider to be maintained for the to be activated PUCCH SCell. In this case X value is 0. Proposal 3: RAN4 to agree that X=4 when the PUCCH SCell activated is unknown at the reception of PUCCH SCell activation command and the RS used for L1-RSRP is same as PL-RS. For all other cases where PL-RS not maintained is X=5. Proposal 4: RAN4 to consider the scenarios considered for Rel-16 multiple SCell activation case for multiple SCell activation when one SCell is PUCCH SCell. Proposal 5: For scenario 1 and valid TA case, total activation delay of multiple SCells when one of the SCell is PUCCH SCell is THARQ+Tdelay_PUCCH_multiple_SCellsNR slot length, where, Tdelay_PUCCH_ multiple_SCells = Tactivation_time_multiple_scells + [X] + TCSI_Reporting. Where: - Tactivation_time_multiple_scells is the target SCell activation delay in millisecond in multiple SCell activation scenario as specified in section 8.3.7. - [X] sample measurement time is introduced in FR2 when PL-RS of target PUCCH SCell is known - FFS under what condition the [X] = 0 or [X] = 5 - TCSI_Reporting is same as specified in clause 8.3.7 Proposal 6: For scenario 2 and valid TA case, total activation delay of multiple SCells when one of the SCell is PUCCH SCell is THARQ+Tdelay_PUCCH_multiple_SCellsNR slot length, where, Tdelay_PUCCH_ multiple_SCells = Tactivation_time_multiple_scells + [X] + max (TCSI_Reporting_Primary_Group, TCSI_Reporting_Secondary_Group). Where: - Tactivation_time_multiple_scells is the target SCell activation delay in millisecond in multiple SCell activation scenario as specified in section 8.3.7. - [X] sample measurement time is introduced in FR2 when PL-RS of target PUCCH SCell is known - FFS under what condition the [X] = 0 or [X] = 5 - TCSI_Reporting_SpCell is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources on primary PUCCH group, as specified in TS 38.331 [2]. - TCSI_Reporting_PUCCH_SCell is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources on secondary PUCCH group, as specified in TS 38.331 [2]. Proposal 7: When the PRACH and CSI report partially overlap in time during multiple SCell activation involving PUCCH SCell, UE should transmit PRACH and postpone the CSI report to next CSI reporting instance. Proposal 8: For scenario 2 and invalid TA case, total activation delay of multiple SCells when one of the SCell is PUCCH SCell is THARQ+Tdelay_PUCCH_multiple_SCellsNR slot length, where, Tactivation_time_multiple_scells + [X] + max (max ((TFirst_available_CSI_PUCCH_SCell + TCSI_processing_PUCCH_SCell), (T1+T2+T3)) + TCSI_reporting_after_PUCCH_SCell, TCSI_Reporting_SpCell + Tuncertainity) Where: - Tactivation_time_multiple_scells is the target SCell activation delay in millisecond in multiple SCell activation scenario as specified in section 8.3.7. - [X] sample measurement time is introduced in FR2 when PL-RS of target PUCCH SCell is known - FFS under what condition the [X] = 0 or [X] = 5 - Tfirst_available_CSI_ PUCCH_SCell: the delay uncertainty in acquiring the first available downlink CSI reference resource for PUCCH SCell. - TCSI_processing_ PUCCH_SCell: the UE processing time for CSI reporting of PUCCH SCell. - TCSI_reporting_after_ PUCCH_SCell the delay uncertainty in acquiring the first available CSI reporting resource after T3 - T1 is the delay uncertainty in acquiring the first available PDCCH triggered PRACH occasion in the PUCCH SCell after Tactivation_time. - T1 is up to the summation of a delay uncertainty for reception of PDCCH order, SSB to PRACH occasion association period and 10 ms, where SSB to PRACH occasion association period is defined in the table 8.1-1 of TS 38.213 - T2 is the delay from slot n + (THARQ + Tactivation_time +T1)/NR slot length until UE has obtained a valid TA command for the target PUCCH SCell being activated. Slot n is the slot where the UE receives PUCCH SCell activation command. - T3 is the delay for applying the received TA for uplink transmission on target PUCCH SCell being activated, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213. - TCSI_Reporting_SpCell: is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources on primary PUCCH group, as specified in TS 38.331 [2]. - Tuncertainity is uncertainty of first available CSI reporting resource, if CSI report on primary PUCCH group collides with PRACH on PUCCH SCell. In other cases, it is zero. |
| R4-2210135 | Ericsson | CR on SCell activation/deactivation with PUCCH |

Open issues summary

Sub-topic 1-1 PUCCH SCell activation delay requirements

Agreements in RAN4#102e meeting:

- the known condition of PL-RS for known PUCCH SCell could be defined as:

  - The pathloss reference signal is known for known PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

    - The target pathloss reference signal determination is based on the L3 RSRP measurement reporting

    - The target pathloss reference signal remains detectable during the PUCCH SCell activation period

      - SNR of the target pathloss reference signal≥-3dB

    - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

      - SNR of the associated SSB ≥-3dB

    - Otherwise, the pathloss reference signal is unknown.

- the known condition of PL-RS for unknown PUCCH SCell could be defined as:

  - The pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L1-RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

    - The target pathloss reference signal determination is based on the L1-RSRP measurement reporting

    - The target pathloss reference signal remains detectable during the PUCCH SCell activation period

      - SNR of the target pathloss reference signal≥-3dB

    - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

      - SNR of the associated SSB ≥-3dB

    - Otherwise, the pathloss reference signal is unknown.

Issue 1-1-1: The known condition of PL-RS

Proposals:

- Option 1: Update the known condition of PL-RS signals: (Apple)

  - the known condition of PL-RS for known PUCCH SCell could be defined as:

    - The pathloss reference signal is known for known PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

      - Pathloss reference signal activation command is received within 1280 ms upon the last transmission of the RS resource for L3 measurement

      - The UE has sent at least one L3 RSRP report for the target pathloss reference signal before the pathloss reference signal activation command

      - The target pathloss reference signal remains detectable during the PUCCH SCell activation period

        - SNR of the target pathloss reference signal≥-3dB

      - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

        - SNR of the associated SSB ≥-3dB

    - Otherwise, the pathloss reference signal is unknown.

  - the known condition of PL-RS for unknown PUCCH SCell could be defined as:

    - The pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L1-RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

      - Pathloss reference signal activation command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

      - The UE has sent at least one L1-RSRP report for the target pathloss reference signal before the pathloss reference signal activation command

      - The target pathloss reference signal remains detectable during the PUCCH SCell activation period

        - SNR of the target pathloss reference signal≥-3dB

      - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

        - SNR of the associated SSB ≥-3dB

    - Otherwise, the pathloss reference signal is unknown.

- Recommended WF

  - Need more discussion

| Issue 1-1-1: The known condition of PL-RS |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Okay with the proposal. |
| Apple | We still think it’s necessary to make the definition of known clear to both UE and network sides, i.e., using the report timing to determine the known condition is a clear criterion to both sides. For instance, with current description, how network can determine if this PL-RS is known when it receives L3 measurement report of this PL-RS long times ago? The time range of reporting is quite important to make network and UE on the same page for such known/unknown condition. |
| Nokia | We have some concerns/questions on the proposed conditions in Option 1. Firstly, there is no PL-RS activation command defined in MAC spec. The PL-RS may be configured by RRC or implicitly indicated in PUCCH-SpatialRelationInfo, but neither of them is mandatory for pathloss estimation. According to RAN1, the UE is able to determine a default PL-RS even without any PL-RS indication. Hence we should not link the PL-RS condition to an unclear message which is optionally transmitted. In addition, we wonder why 1280ms time restriction is introduced. We understood 1280ms is the time range to the last transmission of the RS resource for L3 measurement. As the RS resource i.e. SSB is periodically transmitted every 20ms, this can ensure the time range always within 1280ms in case PL-RS is indicated explicitly. In any case, the known condition of PL-RS is expected to reflect the agreements below. We think existing known condition is sufficient to reflect the agreements, and we should avoid any ambiguity on the condition or any implication on the optional UE behaviour. - For Tactivation_time, - For known PUCCH SCell, - TCI sate, PL-RS and spatial relation indication are assumed to be based on the L3 measurement. - For unknown PUCCH SCell, - TCI sate, PL-RS and spatial relation indication are assumed to be based on L1-RSRP measurement. |
| Huawei | Fine with option 1 |
| MediaTek | Support option 1. Without 1280 ms time restriction may lead to confusion between UE and network. If we have no explicit time restriction, can UE regard PL-RS as known when the report is transmitted one hour ago before receiving pathloss reference signal activation command? |
| Ericsson | We kind of agree with Nokia comments. Since TCI and PL-RS are supposed to be on same RS, can the following proposal is agreeable to companies? - the known condition of PL-RS for known PUCCH SCell could be defined as: - The pathloss reference signal is known for known PUCCH SCell during activation if the following conditions are met. - Last transmission of the RS resource used for L3 RSRP measurement reporting is less than 1280ms from the PUCCH SCell activation command. Where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. - The target pathloss reference signal remains detectable during the PUCCH SCell activation period - SNR of the target pathloss reference signal≥-3dB - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period - SNR of the associated SSB ≥-3dB - Otherwise, the pathloss reference signal is unknown. - the known condition of PL-RS for unknown PUCCH SCell could be defined as: - The pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met. - The last transmission of the RS resource used for L1-RSRP measurement reporting is less than 1280ms from the PUCCH SCell activation command. Where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. - The target pathloss reference signal remains detectable during the PUCCH SCell activation period - SNR of the target pathloss reference signal≥-3dB - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period - SNR of the associated SSB ≥-3dB Otherwise, the pathloss reference signal is unknown. |
| Intel | OK with option 1. It seems that there is no agreement that TCI and PL-RS are supposed to be on same RS. |
| CATT | Support option 1. Same view as MTK that 1280ms is needed. The RS transmission and measurement report are valid within a certain time. |
| vivo | OK with option 1 |
| Nokia2 | It seems we are missing common understanding on PL-RS activation command. As questioned, what is PL-RS activation command? It is not defined in any spec. We can understand the concerns on PL-RS validity if the RS resource used for reporting is too long time ago. But we should not base the timing on PL-RS activation command which is unclear. In order to move forward, E/// proposal sounds reasonable to link the timing to PUCCH SCell activation command. Or some rephrase like: PUCCH SCell activation command is received within 1280 ms upon the last transmission of the RS resource for L3 measurement reporting |

Issue 1-1-2: Whether the PL-RS will introduce extra delay time when the known condition is met in FR2 (the value of [X] in 8.3.12)?

Proposals

- Option 1: (Apple, Qualcomm, Intel, OPPO, MTK, Huawei)

  - When PL-RS of target PUCCH SCell is known, the X=5 sample measurement time is always considered and no need to consider condition of ‘maintain’ or ‘not maintain’.

- Option 2: (Nokia)

  - If PUCCH Scell is known in FR2, the reported L3 measurement results can be reused for pathloss estimation and additional PL-RS measurement is not needed during PUCCH Scell activation.

  - If PUCCH Scell is unknown in FR2, [X] = 0 if PL-RS is maintained and [X] = 5 if PL-RS is not maintained.

- Option 3: (CATT)

  - X =5 if the last transmission of the RS resource used for L3 RSRP/L1-RSRP measurement reporting is configured as aperiodic measurement reporting, or the last one measurement reporting of periodic/semi-persistent measurement reporting, otherwise, X=0.

- Option 4: (Ericsson)

  - If the to be activated PUCCH Scell share the same PL-RS with the active serving cell or the PL-RS of active serving cell and the PL-RS of the to be activated PUCCH Scell are configured to be QCL Type-A, PL-RS can be considered to be maintained for to be activated PUCCH Scell.  In this case X value can be 0.

  - X=4 when the PUCCH Scell activated is unknown at the reception of PUCCH Scell activation command and the RS used for L1-RSRP is same as PL-RS.

  - For all other cases where PL-RS not maintained is X=5.

- Recommended WF

  - Need more discussion

| Issue 1-1-2: Whether the PL-RS will introduce extra delay time when the known condition is met in FR2 (the value of [X] in 8.3.12)? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support Option 1. The second bullet of Option 4 (X=4) is acceptable. Any idea of defining requirements based on the premise that PL-RS measurement can be maintained is not acceptable to us. The supporting arguments have been provided for the last a couple of meetings from multiple companies. We should not expect UE to maintain any measurements for deactivated Scell. |
| Apple | We still think option 1 makes more sense as we discussed in our paper. We understand the scenario Ericsson mentioned, i.e., PL-RS is maintained on other active serving cell, we doubt if that’s a likely case that pathloss estimation on one CC is based on the RS measurement on other CC. |
| Nokia | Option 2. For known Scell, the most controversial issue is if L3 measurement results can be reused for pathloss estimation. We have been assuming L3 measurements for pathloss estimation and we only defined L3 measurement requirements without differentiating the usage. It seems very natural to reuse the available L3 results for pathloss estimation. We’d like to understand why additional L3 PL-RS measurement is considered. There may be some UE implementation to optimize the PL estimation e.g. using narrow beam, but this should not unnecessarily delay the Scell activation. For unknown Scell, the network may not receive any valid L3 measurement results before sending the Scell activation command. However, the UE is required to perform L3 measurements in deactivated Scell i.e. the PUCCH Scell according to clause 9.2.5 Table 9.2.5.2-4. This means the UE may also have acquired L3 measurement results but not yet send it to the network. If this is the case, the PL-RS determined based on L1-RSRP may have been L3 measured at the UE and can be used for PL estimation without additional PL-RS measurement. Therefore, we can differentiate the cases considering PL-RS is maintained or not (though it is not explicitly defined), and additional PL-RS measurement is needed only when PL-RS is not maintained. We are also fine to consider the case where PUCCH Scell is sharing the PL-RS with the active serving cell i.e the first bullet in Option 4. If PL-RS is considered as maintained in this case, we agree additional PL-RS measurement is not needed i.e. X = 0. |
| Huawei | Support option 1. As explained in previous meeting, if the RS is measured before (L1 or L3), it is not equivalent to the PL-RS is maintained by UE. 5 samples time is defined according to RAN1’s definition. We should not try to speed up the procedure by violating RAN1’s definition in LS in Rel-16. The only exception is when the PL-RS is configured with RS in other active serving cell and it happens to be configured as PL-RS of other cells, which is quite corner case to be considered. Besides, there is discussion in thread 201 about “5 samples” in FR2. The conclusion should also apply here. |
| MediaTek | Support option 1. To our understanding, the UL channel of PUCCH Scell is important. Thus, we tend to believe the UL power should base on the PL-RS transmitted on PUCCH Scell. We think it is a bit strange that UE is required to maintain a PL-RS on the other cell for a deactivated Scell before receiving activation command. |
| Ericsson | We understand UE vendors clarification that before receiving the activation command, UE cannot store the measurements made on deactivated Scell as UE do not know when the particular Scell is going to be activated. Only case where the UE can be skipped with pathloss measurements without having impact on the power control performance is PL-RS of active serving cell and PUCCH Scell being same. This can happen in two cases - PL-RS of to be activated PUCCH Scell and other active serving cell PL-RS are QCL Type-A. I think this may not be possible as UE need not get the QCL type A relation from other CC. - When the same RS is configured as PL-RS for active serving cell and PUCCH Scell. - For inter-band CA case due to frequency difference, it may not be possible to reuse the PL from other band due to propagation difference. - For intra-band CA case, we think this is possible to use PL from other carriers if they share same PL-RS. In this case, X=0; We think X=4 is also a valid case as mentioned in our paper. |
| Intel | Prefer option 1. It’s possible that PL-RS may be re-used by other active Scell. However, we still have concern that whether dual PUCCH in intra-cell CA is real deployment. |
| CATT | We can compromise to option 4. From configuration perspective, it is not precluded that the PUCCH Scell share the same PL-RS as active cell. For the number of samples, it can be updated if there are conclusions in email thread #201. |
| OPPO | Support option 1. It could be possible that the PL-RS is configured associated with the active serving cell and PL-RS can assumed to be maintained. But to move forward, we prefer to define the generic requirements and not to differentiate maintain and not maintain cases. For FR2 case, we prefer extra delay time could be 5 samples no matter whether PL-RS is not maintained or not before Scell is activated. |
| vivo | Prefer option 1. Regarding option 4 x= 4 could be valid for this case however option 1 is preferred for simplicity. |
| Nokia2 | We are a bit confused on the concept “PL-RS is maintained”. It has been understood 5 samples are needed for PL estimation. But now even the UE has acquired L3 measurements, and the PL-RS is determined based on the measurements, the PL-RS is still considered as “not maintained”. Could UE vendors clarify what is “PL-RS is maintained”? What is the difference from PL-RS is L3 measured? |

Issue 1-1-3: Update the PUCCH Scell activation delay requirements about PL-RS measurement in FR2 for invalid TA case?

Proposals

- Option 1: (Qualcomm, Nokia)

  - Update the previous agreement about FR2 PL-RS measurement, from a sequential processing to a parallel processing, i.e. PL-RS can be measured in parallel with “DL CSI-RS reception/processing” and “UL TA acquisition”.

    - Previous agreement: THARQ+Tactivation_time+[X]+max{(Tfirst_available_CSI+TCSI_processing),(T1+T2+T3)}+TCSI_reporting_after

    - Proposed update: THARQ+Tactivation_time+max{(Tfirst_available_CSI+TCSI_processing),(T1+T2+T3),[X]}+TCSI_reporting_after

- Recommended WF

  - Need more discussion

| Issue 1-1-3: Update the PUCCH Scell activation delay requirements about PL-RS measurement in FR2 for invalid TA case? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support Option 1. We do not see an issue with PL-RS measurement in parallel with “DL CSI-RS reception/processing and UL TA acquisition.” Or it will lead to a much longer latency than necessary. As can be seen from Fig 1, a PL-RS measurement window does not have to be separated out from the window for DL CSI-RS reception/processing and UL TA acquisition. Figure 1. FR2 known Scell activation |
| Apple | We are fine with the proposed update since PL-RS measurement is not for RACH transmission and it could be in parallel with TA acquisition procedure. |
| Nokia | Option1. The additional PL-RS measurements if needed can be in parallel with CSI-RS measurements and RACH procedure, hence [x] shall be moved inside the bracket. |
| Huawei | Fine with option 1 |
| MediaTek | More discussion is needed. One question for clarification, what is the beam assumption of the PRACH? To our understanding, if the fine beam is used for PRACH transmission, then it should wait for the PL-RS measurement complete. Otherwise, they can be done in parallel. |
| Ericsson | We agree with option 1. |
| Intel | Agree with option 1. |
| CATT | Support option 1. |
| Huawei2 | Regarding MTK’s comments, we think it is a valid point. But from our understanding, the PL-RS measurement here is for PUCCH. The beam for PRACH is configured in PDCCH ordered which is based on latest reported L1/L3 measurement, and the Tx power of PRACH can be determined accordingly. Then it seems they can be down in parallel. |
| vivo | Ok with option 1. |

Issue 1-1-4: Whether the PL-RS will introduce extra delay time when the known condition is met in FR1?

Moderator: In moderator’s understanding, the previous discussions on spatial relation and PL-RS are only for FR2. This issue is to discuss whether the PL-RS measurement time is applied for FR1.

Proposals

- Option 1: (Qualcomm, Huawei)

  - When PL-RS of target FR1 PUCCH SCell is known, the same number of samples for PL-RS measurement as FR2 should be added to the FR1 PUCCH Scell activation latency requirement. And the number of samples is 5.

- Recommended WF

  - Need more discussion

| Issue 1-1-4: Whether the PL-RS will introduce extra delay time when the known condition is met in FR1? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support Option 1. We believe this is just something that companies, including us, overlooked and was confused with UL spatial relation issue. |
| Apple | Support Option 1, FR1 shall also use PL-RS to determine the Tx power allocation for PUCCH. |
| Nokia | We tend to agree that pathloss estimation is also needed in FR1, hence the principle if/when additional PL-RS measurement is needed can be applied to FR1. But Option 1 is not agreeable as this implies additional 5 samples PL-RS measurement is always needed. |
| Huawei | Support option 1 |
| MediaTek | Support option 1. Regardless of FR1 and FR2, the UL power will be determined by PL-RS. Thus, extra delay for PL-RS is needed. |
| Ericsson | We agree that pathloss estimation is needed for FR1 too. X value conclusion from other issue can be applied here as well. |
| Intel | Fine with option 1 |
| CATT | Support option 1. For the number of samples, the conclusion in FR2 (in issue 1-1-2) can also apply to FR1. |
| vivo | Ok with option 1 |

Issue 1-1-5: Interruption due to PRACH transmission of PUCCH Scell (related to RAN1 reply LS R4-2206147(R1-2202599))?

- Option 1: (CATT)

  - Not define interruption due to PUCCH Scell RACH has different SCS from spCell data/control channel.

    - Deleting the editor notes of “FFS on whether to define interruptions due to PUCCH Scell RACH has different SCS from spCell data/control channel” from specification.

- Option 2: (MTK)

  - In the inter-band CA case, it is up to UE implementation if network schedules concurrent transmission of PRACH and PUSCH/PUCCH/SRS for inter-band CA, and UE does not support parallelTxPRACH-SRS-PUCCH-PUSCH.

  - In the intra-band CA case, no UE requirement applies when PRACH transmission is colliding with PUSCH/PUCCH/SRS.

- Option 3: (Nokia)

  - The PUCCH Scell activation delay requirement (Tdelay_PUCCH Scell) shall apply provided that:

    - The RA on PUCCH Scell is not interrupted by the PUCCH/PUSCH/SRS on other activated serving cells otherwise additional delay to activate the Scell is expected.

- Option 4: (Huawei)

  - Clarify that there could be interruption on UL transmission on other serving Cells when colliding with RACH transmission on PUCCH Scell if UE is not capable of parallelTxPRACH-SRS-PUCCH-PUSCH .

- Recommended WF

  - Need more discussion.

| Issue 1-1-5: Interruption due to PRACH transmission of PUCCH Scell (related to RAN1 reply LS R4-2206147(R1-2202599))? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support Option 2. It should be left to UE implementation, i.e. no requirement, if such a UE is scheduled with the concurrent transmissions. If any clarification is needed or the requirements should be defined in a specific way, the clarification shall be first made by RAN1. |
| Apple | We are fine with option 4 and 1st bullet of option 2. Based on RAN1 reply “it seems RAN4’s question is the case where the UE transmits PRACH on PUCCH Scell in the secondary PUCCH group and PUCCH/PUSCH/SRS on a UL cell in the primary PUCCH group; then to RAN1’s understanding this must be inter-band simultaneous transmissions”, only inter-band CA case shall be considered in our requirement. |
| Nokia | Option 3. As the interruption may or may not happen, we need to at least identify the impact to PUCCH Scell activation delay. We can clarify the activation delay requirement is applied only in case of no interruption. Option 3 is quite general to reflect both intra-band and inter-band cases, irrespective of the UE capability. We understood the UE ehavior in case of collision is left to UE implementation, hence seems no need to define the interruption in this case. |
| Huawei | Support option 4. But also fine to no requirements for such case (concurrent transmission of PRACH and PUSCH/PUCCH/SRS for inter-band CA, and UE does not support parallelTxPRACH-SRS-PUCCH-PUSCH). |
| MediaTek | Support option 2. For intra-band CA case, we are ok not to mention in 133 which is not considered in RAN1. |
| Ericsson | First of all, NW may not schedule two dynamic grants in overlapping manner if the UE is not capable of simultaneous transmissions. However, it may so happen that one dynamic grant and periodic or semi-persistent grant may overlap in scheduling and in that case UE behavior has to be defined so that all UE follow same behavior. If the contention-based RACH (CBRA) and other UL overlaps, NW may not know about the PRACH transmission, and it is fine to leave it to UE implementation in the CBRA case. However, contention free RA (CFRA) case, since PRACH on PUCCH Scell is NW scheduled through PDCCH order, we do not think it should be left to UE implementation. Overlapping case may occur between PRACH and periodic or semi-persistent transmissions in the UL. We can assume that it may not occur between two dynamic grants. Considering this PDCCH order can overlap with periodic transmissions such as CSI report or SRS. When this happens, UE shall prioritize PRACH as NW scheduled it knowing there is an overlapping transmission on PUCCH/PUSCH/SRS (if the UE do not have capability for simultaneous transmission capability of PRACH and CSI/SRS) Based on the above analysis, we propose option 5. Option 5 (Ericsson): For inter-band CA case, if network schedules overlapping transmission of PRACH and PUSCH/PUCCH/SRS, and UE does not support parallelTxPRACH-SRS-PUCCH-PUSCH, UE should transmit PRACH. |
| CATT | Firstly, based on RAN1 response, for intra-band CA case, PRACH and PUSCH/PUCCH/SRS are not transmitted at the same symbol, so no further discussion on this case is needed since there is no interruption issue. Our discussion should focus on the inter-band case and for the UE not supporting parallelTxPRACH-SRS-PUCCH-PUSCH. For this case, we are fine with option 4 to clarify that there could be interruption on the UL transmission. In our understanding, option 5 proposed by Ericsson seems same as option 4 since that UE prioritize PRACH means there will be interruption on other UL transmission. |
| vivo | Ok with option 2 |

Sub-topic 1-2 PUCCH Scell activation delay requirements with multiple DL Scells

Issue 1-2-1: The scenarios of PUCCH Scell activation with multiple DL Scells?

- Proposal 1: (Qualcomm)

  - Multiple DL Scells to be activated along with PUCCH Scell shall be in the same PUCCH group as the PUCCH Scell. Any to be activated DL Scell in parallel with PUCCH Scell should not be activated earlier than the PUCCH Scell.

- Proposal 2: (Qualcomm)

  - For the activation latency requirement of multiple DL Scells being activated in parallel with PUCCH Scell, do not consider the following scenarios:

    - FR1 unknown non-contiguous CA scenario which includes N1*Trs in the latency requirement

    - One of serving cells is activated in the same band as to be activated FR2 PUCCH Scell

    - Unknown PUCCH Scell with known to-be-activated DL Scells

- Proposal 3: (MTK)

  - For PUCCH Scell activation with multiple Scells, no UE requirement applies when the unknown Scells (not including PUCCH Scell) to be activated does not have active serving cell(s) or known to-be-activated Scell(s) on the same band.

  - For PUCCH Scell activation with multiple Scells, RAN4 further study whether the unknown PUCCH Scell should be activated only on the band which contains active serving cell(s) or known to-be-activated Scell(s).

- Proposal 4 (Ericsson):

  - RAN4 to consider the scenarios considered for Rel-16 multiple Scell activation case for multiple Scell activation when one Scell is PUCCH Scell.

  - Legacy multiple Scell activation requirements are defined for EN-DC, NE-DC, NR SA and NR-DC.

- Recommended WF

  - Need more discussion

| Issue 1-2-1: The scenarios of PUCCH Scell activation with multiple DL Scells? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support Proposal 1 and 2. For Proposal 3: - The first bullet is okay with us, and one example can be “If FR2 PUCCH Scell is known, DL Scells to be activated in parallel with the PUCCH Scell in the same band are considered known FR2 Scell in terms of latency requirement, i.e. no L1-RSRP measurement and report are included in the requirement.” - We do not support the first part of the second bullet (unknown PUCCH Scell should be activated only on the band which contains active serving cell(s)). We have never seen any commercial filed deployment with intra-band CA between dual PUCCH groups. We do not see the need of discussing the issue. - For the second part of the second bullet (unknown PUCCH Scell should be activated only on the band which contains known to-be-activated Scell(s).), it is very unlikely to see a scenario where PUCCH Scell is unknow while one of Scells in the same PUCCH group is known. We do not support this case either. For Proposal 4: - We believe NR-DC is not an applicable case for dual PUCCH. Please see the text excerpted from TS38.213. |
| Apple | We are fine with proposal 1 for sake of simplicity. If multiple Scell activation includes DL Scell in different PUCCH group from to-be-activated PUCCH Scell, the scenario would be too complicated (Scells in different PUCCH Scell groups would have additional limitation on the activation timeline, e.g., RACH of PUCCH Scell may collide with CSI reporting for being-activated Scells on other PUCCH group) and we recommend to firstly focus on the multiple Scell activation in same PUCCH group. Regarding proposal 2, we are fine with bullet 2 and 3, but don’t understand the rationale behind bullet 1. Regarding proposal 3, we don’t think there could be an active serving cell on the same band as target PUCCH Scell. As pointed out in RAN1 LS, one single PUCCH group is expected for Scells on the same band, and then in same PUCCH group, the PUCCH Scell shall be the first one to be activated. So for PUCCH Scell it shall not be any active serving cell on the same band as target PUCCH Scell. Regarding “known to-be-activated Scell(s) on the same band”as target PUCCH Scell, we think it’s not a practical case that other Scell(s) is known in the same PUCCH group but the PUCCH Scell in that PUCCH group is unknown. Proposal 4 is for general scenario, we could discuss proposal 1 and proposal 4 are similar, we are fine to go with proposal 1. |
| Nokia | P1: For activating the multiple Scells with PUCCH Scell, the Scell may or may not be in the same PUCCH group of the PUCCH Scell. We can define the delay requirements for these Scells separately but better not exclude the scenario. P2: We do not agree with the proposal. In the context of multiple Scell activation, the case of multiple FR2 unknown Scells are excluded. We may drop the same scenario where multiple FR2 unknown Scells are being activated in parallel with PUCCH Scell, and other scenarios need to be considered. P3: We are fine with the first bullet and can discuss the 2nd bullet. In our views, the PUCCH Scell may be on inter-band from the SpCell and it is likely there is no active serving cells or known to-be-activiated Scells on it. We may need consider the worse case for PUCCH Scell. P4: We are fine with it. |
| Huawei | For option 1, we think when the to-be-active Scells are in different PUCCH group should also be considered. Support option 3. We agree with the first bullet. For the second the bullet, we share the same views as comments from companies that PUCCH Scell without intra-band serving cell or known to -be-activated Scell should be considered. |
| MediaTek | For proposal 1, we are ok if RAN4 explicitly defines no UE requirement applies as long as any one of the Scells to be activated is in the different PUCCH group from PUCCH Scell. For proposal 2, more discussion is needed. For first sub-bullet, could proponent explain more? For proposal 3, support first bullet. For the second bullet, we are not sure is there any evidence to proof no intra-band CA with dual PUCCH group from spec perspective? For proposal 4, as QC’s comment, the PUCCH Scell activation requirement is not applicable in NR-DC scenario. |
| Ericsson | Proposal 1: We think Scells can be in different PUCCH group. If there is a restriction on this, from the following excerpt, - UE only receives one single MAC command for multiple Scell activation within the activation period defined in this clause NW need to wait till the ongoing Scell activation is completed to activate Scells from the other PUCCH group. Proposal 2: Considering the other Scells in the same group cannot be activated earlier than PUCCH Scell, last two bullets are fine. First bullet needs more clarification. Proposal 3: Our understanding is proponent support multiple Scells can be from different PUCCH group. Only then first bullet may be valid. We do not agree with second bullet. Proposal 4: Thank you QC and MTK for pointing about NR-DC. Other cases anyway are considered so I guess companies are OK with it by removing NR-DC. |
| Intel | Support Proposal 1. We prefer to define requirement when multiple Scells are in the same PUCCH group with to-be-activated PUCCH Scell first. For Proposal 2, for the first bullet, suggest to further clarify the case. It seems to preclude some scenarios which will cause large delay. Fine with 2nd and 3rd bullet. For Proposal 3, need to further clarify whether Scell is in the same or different PUCCH group from PUCCH Scell. If Scell to be activated is in different PUCCH group from PUCCH Scell, fine with the first bullet. If Scell is in the same PUCCH group from PUCCH Scell, we think that PUCCH Scell will be activated first. For proposal 4, fine to further discuss. |
| CATT | For proposal 1, the Scells can be with different PUCCH group. We think this is valid case and the requirements can be defined. For proposal 2, we don’t quite understand what the concern is on defining requirements for the listed cases. More clarification is needed to preclude these cases. For proposal 3, fine with the first bullet. For the 2nd bullet, we think there is no need to have the restriction that active serving cell is contained on the same band as PUCCH Scell. For single PUCCH Scell activation, we defined the requirements for the case that no active serving existing on the same band and in this case, cross PUCCH group reporting is needed. For proposal 4, fine with it by removing NR-DC. |
| Qualcomm2 | Adding 2nd comment to elaborate on the following bullet under Proposal 2. - FR1 unknown non-contiguous CA scenario which includes N1*Trs in the latency requirement This is, to us, kind of multiple SCell activation with a bunch of completely unknown SCells. In such a case, the requirement does not apply to those known SCells in the legacy spec. And in any case, the latency is expected pretty long. Therefore, we do not see a need of defining the requirement for such a case. |

Issue 1-2-2: The activation delay requirements for the PUCCH Scell to be activated?

Proposals

- Option 1: (Intel, CATT, OPPO)

  - PUCCH Scell activation delay requirements can be derived from single PUCCH Scell activation delay by replacing Tactivation_time with  Tactivation_time_multiple_scells

- Option 1a: (MTK)

  - PUCCH Scell activation delay requirements can be derived from single PUCCH Scell activation delay by replacing Tactivation_time with Tactivation_time_multiple_scells. And the following updates are made for Tactivation_time_multiple_scells:

    - Tuncertainty_MAC_multiple_scells is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and spatial relation, and Scell activation command of this unknown PUCCH Scell.

    - TSMTC_MAX_multiple_scells and TFirstSSB_MAX_multiple_scells should additionally consider PUCCH Scell.

- Option 2: (Nokia)

  - The PUCCH Scell activation shall be prioritized over the other being-activated DL Scells when they are activated by a single MAC command.

  - The Scell activation delay for PUCCH Scell with multiple DL Scells is the same as the single PUCCH Scell activation delay in clause 8.3.12 i.e. Tdelay_PUCCH_Scell.

- Option 3: (Ericsson)

  - Scenario 1: CSI report of all Scells transmitted on SpCell

    - Only valid TA case can be assumed

    - Tactivate_total = THARQ+Tdelay_PUCCH_multiple_SCellsNR slot length, where:

      - Tdelay_PUCCH_ multiple_Scells = Tactivation_time_multiple_scells + [X] + TCSI_Reporting, where:

        - Tactivation_time_multiple_scells is the target Scell activation delay in millisecond in multiple Scell activation scenario as specified in section 8.3.7.

        - TCSI_Reporting is same as specified in clause 8.3.7

  - Scenario 2: CSI report of primary PUCCH group transmitted on Pcell and CSI report of secondary PUCCH group transmitted on PUCCH Scell

    - For valid TA case, Tactivate_total = THARQ+Tdelay_PUCCH_multiple_SCellsNR slot length, where:

      - Tdelay_PUCCH_ multiple_Scells = Tactivation_time_multiple_scells + [X] + max (TCSI_Reporting_Primary_Group, TCSI_Reporting_Secondary_Group), where:

        - TCSI_Reporting_SpCell is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources on primary PUCCH group, as specified in TS 38.331 [2].

        - TCSI_Reporting_PUCCH_Scell is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources on secondary PUCCH group, as specified in TS 38.331 [2].

    - For invalid TA case, Tactivate_total = THARQ+Tdelay_PUCCH_multiple_SCellsNR slot length, where:

      - Tdelay_PUCCH_ multiple_Scells = Tactivation_time_multiple_scells + [X] + max (max ((TFirst_available_CSI_PUCCH_Scell + TCSI_processing_PUCCH_Scell), (T1+T2+T3)) + TCSI_reporting_after_PUCCH_Scell, TCSI_Reporting_SpCell + Tuncertainity), where:

        - Tfirst_available_CSI_ PUCCH_Scell: the delay uncertainty in acquiring the first available downlink CSI reference resource for PUCCH Scell.

        - TCSI_processing_ PUCCH_Scell: the UE processing time for CSI reporting of PUCCH Scell.

        - TCSI_reporting_after_ PUCCH_Scell the delay uncertainty in acquiring the first available CSI reporting resource after T3

        - Tuncertainity is uncertainty of first available CSI reporting resource, if CSI report on primary PUCCH group collides with PRACH on PUCCH Scell. In other cases, it is zero.

    - When the PRACH and CSI report partially overlap in time during multiple Scell activation involving PUCCH Scell, UE should transmit PRACH and postpone the CSI report to next CSI reporting instance.

- Recommended WF

  - Need more discussion

| Issue 1-2-2: The activation delay requirements for the PUCCH Scell to be activated? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support Option 1a with the following modification: - For the second sub-bullet “TSMTC_MAX_multiple_scells and TFirstSSB_MAX_multiple_scells should additionally consider PUCCH Scell,” we do not think it is necessary. As PUCCH Scell is just one of Scells to be concurrently activated by the same MAC CE, the definition of the current parameters already include the PUCCH Scell. The sub-bullet should be removed. For Option 3: - To start with, we are not in favor of the Scell activation scenario with “mixture of Scells from primary PUCCH group and secondary PUCCH group.” - We fail to understand “Only valid TA case can be assumed” for Scenario 1. For Scenario 2, the requirements for multiple Scell activation with PUCCH Scell can be defined in such a way that the requirement applies to respective Scells, not the total latency for all to-be-activated Scells. |
| Apple | Support option 1a, but just one question to option 1a: why remove ‘UL’ before the ‘spatial relation’, is that intentional or typo? |
| Nokia | Option 2. As the PUCCH Scell is supposed to also transmit CSI reports of other Scells, we think it shall be activated with higher priority over other Scells. This can minimize the PUCCH Scell activation delay, while other Scells activation can be done in parallel without any negative impact. |
| Huawei | For the delay of PUCH Scell, we are fine with the 1st bullet in option 2 where the activation of PUCCH Scell should be prioritized. For option 3, we think there is no need to further differentiate the cases when the CSI is reported in PUCCH Scell or Pcell. For single cell activation requirements, even UE supporting cross group report, the requirements are defined as UE can report CSI on PUCCH Scell. For the last bullet, we think it should be aligned with 1-1-5. |
| MediaTek | Support option 1a. Additional comment to QC: We understand your point that “PUCCH Scell is also a Scell”. However, our intention is to avoid mis-understanding and want to emphasize the PUCCH Scell should be additionally considered. Maybe we can have this proposal first and to further think about how to capture in spec. Response to Apple: thanks for bringing this up. It is a typo. It should be “UL spatial relation”. |
| Ericsson | Thank you, QC for the comments. Regarding the scenario 1, our thinking was UE needs to maintain UL TA of Pcell or PSCell always. If companies think invalid TA case should be considered, we are fine with it too. To Huawei: We think we need to differentiate further into scenarios because in single PUCCH Scell case it is the CSI report of the single PUCCH Scell transmitted on Pcell or PUCCH Scell. However, in this case based on scenario 1 or scenario 2, there can be one or two CSI reports and they need not be at same time for scenario 2. This additional uncertainty component needs to be added to delay requirements for scenario 2. For scenario 1, only single CSI report will be there, and it may be similar as single PUCCH Scell requirements with DL activation delay replaced by multiple downlink Scell activation requirements. In this case also CSI reporting collision with RACH needs to be considered and shall define the prioritization rules. |
| Intel | Fine with option 1a. |
| CATT | We are also fine with option 1a which further clarifies the details. For option 3, we think there is no need to differentiate the scenario into such details. And we think the requirements can be separately discussed for PUCCH Scell and other DL Scells. For PUCCH Scell, the CSI reporting should be the same as that in single PUCCH Scell activation and the requirements except Tactivation can be reused. For the other DL Scells, the requirements can be differentiated based on whether they are in the same PUCCH group with PUCCH Scell (i.e. whether the CSI reporting is transmitted on PUCCH Scell), if yes, the existing requirements in 8.3.7 can be reused except CSI reporting time should be no earlier than PUCCH Scell activation completion, and if no, the existing requirements in 8.3.7 can be reused including CSI reporting time. |
| OPPO | Fine with option 1a. |
| Huawei2 | @Ericsson, thanks for the explanation. We agree with the observation there could be CSI reporting on both PCell and PUCCH SCell up to NW configuration, but from requirement’s perspective, what’s special is the UL preparation time of PUCCH SCell compared with ordination SCell activation. Then in these, case the requirements are verified by the points when UE report CSI on PUCCH SCell. So if we further consider the case when CSI report is configured in PCell, then it seems no difference with ordinary SCell activation as there is no UL preparation time. |

Issue 1-2-3: The activation delay requirements for the other DL SCells to be activated?

Proposals

- Option 1: (Qualcomm)

  - For the activation latency requirement of multiple DL Scells being activated in parallel with PUCCH Scell, the legacy requirements (in 8.3.7) are reused with the following update.

    - TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] which cannot be earlier than the PUCCH Scell activation completion

  - If FR2 PUCCH Scell is known, DL Scells to be activated in parallel with the PUCCH Scell in the same band are considered known FR2 Scell in terms of latency requirement, i.e. no L1-RSRP measurement and report are included in the requirement.

- Option 2: (Intel, Nokia, OPPO)

  - The existing Scell activation delay requirement for deactivated Scell with multiple Downlink Scells defined in clause 8.3.7 of current specification 38.133 can be reused.

- Option 3: (CATT)

  - For the case that the downlink Scells are within different PUCCH group from the activated PUCCH Scell, the existing Scell activation delay requirement for deactivated Scell with multiple Downlink Scells defined in clause 8.3.7 of current specification 38.133 can be reused.

  - For the case that downlink Scells are within the same PUCCH group with the activated PUCCH Scell, the activation delay requirements are same as that for PUCCH Scell (as defined in issue 1-2-2).

- Option 4: (Huawei, MTK)

  - For Scell to be activated with PUCCH Scell which is in the primary PUCCH group, legacy requirements for Scell activation with multiple Scells can apply.

  - For Scell to be activated with PUCCH Scell which is in the secondary PUCCH group, extra delay to wait for UE accomplishing activation of PUCCH Scell needs to be considered in addition to legacy requirements for Scell activation with multiple Scells.

- Recommended WF

  - Need more discussion

| Issue 1-2-3: The activation delay requirements for the other DL Scells to be activated? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support Option 1. - If companies agree that the sub-bullet under the first main bullet of Option 1 is not needed, we can modify the first bullet as “For the activation latency requirement of multiple DL Scells being activated in parallel with PUCCH Scell, the legacy requirements (in 8.3.7) are reused.” The reason why the sub-bullet may not need to explicitly mention “which cannot be earlier than the PUCCH SCell activation completion” is because it is obvious if CSI of the to-be-activated DL SCells needs to be reported to the PUCCH SCell. Besides, as a couple of companies proposed, the requirement spec does not have to explicitly preclude the case where CSI of a group to-be-activated SCells can be reported to SpCell. As “TCSI_reporting” already says “first available CSI reporting resources,” the current definition of the parameter looks clear enough. - For the second bullet of Option 1, if companies agree that PUCCH SCell is also one of SCells to be concurrently activated by the same MAC CE, hence, the case described in the second bullet is already included in the legacy multiple SCell activation requirements, the second bullet can be removed. For Option 3 and Option 4: We are not in favor of the scenario described in the first bullet “downlink SCells are within different PUCCH group from the activated PUCCH SCell” and “SCell to be activated with PUCCH SCell which is in the primary PUCCH group” |
| Apple | Fine with option 1 bullet 1, but not sure if bullet 2 means different definition from legacy activation requirement, because we had following requirement in legacy multiple DL SCell activation (the target SCell is still defined as unknown): If the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time_multiple_scells is: - max(Tuncertainty_MAC_multiple_scells + 5ms + TFineTiming, Tuncertainty_RRC_multiple_scells + TRRC_delay-THARQ), if on the same band UE also has at least one parallel to-be-activated SCell which is FR2 known Scell . Tuncertainty_MAC_multiple_scells =0 if UE receives the SCell activation command and TCI state activation commands at the same time. We have different view on scenarios mentioned in Option 4 bullet #1, we have provided our view to issue 1-2-1 proposal 1. So here if to issue 1-2-1 proposal 1 is agreeable, we may not need to discuss 1st bullet in option 4. We are generally fine with 2nd bullet in option 4. |
| Nokia | We don’t see much difference on the activation delay for the SCells belonging to the same PUCCH group of PUCCH SCell. This seems to be agreeable. We are fine to define the activation delays for the SCells belonging to different PUCCH group in Option 3 and 4. It could be beneficial to discuss the activation delay separately. In addition, for the SCells within the same PUCCH group, we think PUCCH SCell shall be prioritized over the other SCells. |
| Huawei | We support option 4. It depends on whether to consider Scell to-be-activated in primary PUCCH group. We are fine with 1st bullet in option1 for SCell will in the same group with PUCCH SCell. |
| MediaTek | More discussion is needed. For option 1 which is modified by QC and option 2, we think some clarification is needed. For example, the UL spatial relation, the SCell including PUCCH SCell,…, etc. For option 3 and 4, to simplify spec, we support second bullet in option 4 if RAN4 agreed to define the case only when all SCells (including PUCCH SCell) to be activated in the same PUCCH group. |
| Ericsson | We prefer to treat all SCells (including PUCCH SCell) as multiple SCells of Rel-16 framework and reuse the requirements of Rel-16. No need to differentiate PUCCH SCell as it was not easy discussion in Rel-16 and we do not prefer to discuss it again 😊. |
| Intel | Fine with option 1 if the SCell belongs to the same PUCCH group of PUCCH SCell. |
| CATT | Fine with option 3 and option 4. We slightly prefer to define the requirements separately for PUCCH Scell and other DL Scells since there are some differences when the SCells are within the different PUCCH groups with PUCCH Scell and the reporting can be independent. |

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2207774 (Apple) | ericsson: Pending on the discussion |
|  | Company B |
| R4-2208088 (Nokia) | Company A |
|  | Huawei: For the changes related to PL-RS, it should wait for the conclusion of pending issues and discussion in 201. |
|  | MediaTek: depends on discussion in open issue. |
| R4-2208173 (CATT) | Nokia: Corrections to interruption clauses are fine. The other clauses are up to conclusion on Sub-topic 1-1 and 1-2. |
|  | Huawei: similar changes in 8939. For the changes related to PL-RS, it should wait for the conclusion of pending issues and discussion in 201 |
|  | MediaTek: depends on discussion in open issue. Ericsson: First three changes are fine. Other changes are not OK. |
| R4-2208464 (MTK) | Nokia: OK to clarify the SSB-less case. |
|  | MediaTek: agree the CR to capture SSB-less case. |
|  | Ericsson: “There is no requirement if the target PUCCH Scell is not configured with SSB”, this change is OK. Other changes need further discussion. |
| R4-2208939 (Huawei) | Nokia: Corrections to interruption clauses are fine. The PUCCH Scell activation is up to conclusion on Sub-topic 1-1 and 1-2. |
|  | MediaTek: generally fine. |
|  | Ericsson: Pending on open issues |
| R4-2210135 (Ericsson) | Huawei: For the changes related to PL-RS, it should wait for the conclusion of pending issues and discussion in 201. |
|  | MediaTek: depends on discussion in open issue. |

Summary for 1st round

Open issues

Sub-topic 1-1 PUCCH Scell activation delay requirements

|  | Status summary |
| --- | --- |
| Issue 1-1-1 | Status: 7 companies support option 1, Nokia and Ericsson suggest replacing “Pathloss reference signal activation command” with “PUCCH SCell activation command” Tentative agreements: None. Candidate options: - Option 1 (same as that in first round) - Option 2: - the known condition of PL-RS for known PUCCH SCell could be defined as: - The pathloss reference signal is known for known PUCCH SCell during activation if the following conditions are met. - Last transmission of the RS resource used for L3 RSRP measurement reporting is less than 1280ms from the PUCCH SCell activation command. Where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. - The target pathloss reference signal remains detectable during the PUCCH SCell activation period - SNR of the target pathloss reference signal≥-3dB - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period - SNR of the associated SSB ≥-3dB - Otherwise, the pathloss reference signal is unknown. - the known condition of PL-RS for unknown PUCCH SCell could be defined as: - The pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met. - The last transmission of the RS resource used for L1-RSRP measurement reporting is less than 1280ms from the PUCCH SCell activation command. Where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. - The target pathloss reference signal remains detectable during the PUCCH SCell activation period - SNR of the target pathloss reference signal≥-3dB - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period - SNR of the associated SSB ≥-3dB Recommendations for 2nd round: Further discuss the two options. |
| Issue 1-1-2 | Status: 7 companies support option 2, 2 companies support option 4 and 1 company supports option 2. Tentative agreements: None. Candidate options: - Option 1: (Apple, Qualcomm, Intel, OPPO, MTK, Huawei) - When PL-RS of target PUCCH SCell is known, the X=5 sample measurement time is always considered and no need to consider condition of ‘maintain’ or ‘not maintain’. - Option 2: (Nokia) - If PUCCH Scell is known in FR2, the reported L3 measurement results can be reused for pathloss estimation and additional PL-RS measurement is not needed during PUCCH Scell activation. - If PUCCH Scell is unknown in FR2, [X] = 0 if PL-RS is maintained and [X] = 5 if PL-RS is not maintained. - Option 4: (Ericsson) - If the to be activated PUCCH Scell share the same PL-RS with the active serving cell or the PL-RS of active serving cell and the PL-RS of the to be activated PUCCH Scell are configured to be QCL Type-A, PL-RS can be considered to be maintained for to be activated PUCCH Scell. In this case X value can be 0. - X=4 when the PUCCH Scell activated is unknown at the reception of PUCCH Scell activation command and the RS used for L1-RSRP is same as PL-RS. - For all other cases where PL-RS not maintained is X=5. Recommendations for 2nd round: Further discuss the three options. |
| Issue 1-1-3 | Status: 8 companies support option 1 and one company asks for clarification. Tentative agreements: - Update the previous agreement about FR2 PL-RS measurement, from a sequential processing to a parallel processing, i.e. PL-RS can be measured in parallel with “DL CSI-RS reception/processing” and “UL TA acquisition”. - Previous agreement: THARQ+Tactivation_time+[X]+max{(Tfirst_available_CSI+TCSI_processing),(T1+T2+T3)}+TCSI_reporting_after - Proposed update: THARQ+Tactivation_time+max{(Tfirst_available_CSI+TCSI_processing),(T1+T2+T3),[X]}+TCSI_reporting_after Candidate options: None. Recommendations for 2nd round: Please MTK check if the tentative agreement can be acceptable based on Huawei’s response. |
| Issue 1-1-4 | Status: All companies agree that the same number of samples delay for PL-RS is needed for FR1 and two companies clarified that the exact number should follow the conclusion in issue 1-1-2. So moderator would suggest to agree on option 1 by removing the second sentence which is about the exact number. Tentative agreements: - When PL-RS of target FR1 PUCCH SCell is known, the same number of samples for PL-RS measurement as FR2 should be added to the FR1 PUCCH Scell activation latency requirement. Candidate options: None. Recommendations for 2nd round: Agree the tentative. |
| Issue 1-1-5 | Status: The clarification on this issue is mainly for the inter-band CA case when UE doesn’t support parallelTxPRACH-SRS-PUCCH-PUSCH. There are two aspects of clarification, one for interruption requirements and the other one for delay requirements. 4 companies think it should be up to UE implementation when PRACH transmission is collided with SRS/PUCCH/PUSCH in which Nokia suggest to further clarify that the delay requirements only apply when there is no interruption on PRACH transmission. 2 companies think PRACH transmission should be prioritized in case of collision. 2 companies are fine with both approaches. Tentative agreements: None. Candidate options: - Option 1: - In the inter-band CA case, it is up to UE implementation if network schedules concurrent transmission of PRACH and PUSCH/PUCCH/SRS for inter-band CA, and UE does not support parallelTxPRACH-SRS-PUCCH-PUSCH. - Option 1a: (Nokia) - The PUCCH Scell activation delay requirement (Tdelay_PUCCH Scell) shall apply provided that: - The RA on PUCCH Scell is not interrupted by the PUCCH/PUSCH/SRS on other activated serving cells otherwise additional delay to activate the Scell is expected. - Option 2: - Clarify that there could be interruption on UL transmission on other serving Cells when colliding with RACH transmission on PUCCH Scell if UE is not capable of parallelTxPRACH-SRS-PUCCH-PUSCH (i.e. PRACH transmission is prioritized). Recommendations for 2nd round: Further discuss the options. |

Sub-topic 1-2 PUCCH Scell activation delay requirements with multiple DL Scells

|  | Status summary |
| --- | --- |
| Issue 1-2-1 | Status: For P1: 4 companies support; 4 companies don’t support and think the case when PUCCH Scell and other DL Scells are within different PUCCH group should not be excluded. For P2: There is no consensus on the proposal. 3 companies are fine with the last two bullets, two companies ask clarification for all the 3 bullets and one company doesn’t agree with proposal. For P3, 6 companies support the 1st bullet, one company asks for clarification and one company doesn’t agree. The second bullet need further study. For P4, companies clarify that NR-DC case is not applicable. Tentative agreements: Updated proposal 4: - The delay requirements for PUCCH SCell activation with multiple DL SCells are defined for EN-DC, NE-DC, NR SA. Candidate options: - Proposal 1: - Multiple DL Scells to be activated along with PUCCH Scell shall be in the same PUCCH group as the PUCCH Scell. Any to be activated DL Scell in parallel with PUCCH Scell should not be activated earlier than the PUCCH Scell. - Proposal 2: - For the activation latency requirement of multiple DL Scells being activated in parallel with PUCCH Scell, do not consider the following scenarios: - FR1 unknown non-contiguous CA scenario which includes N1*Trs in the latency requirement - One of serving cells is activated in the same band as to be activated FR2 PUCCH Scell - Unknown PUCCH Scell with known to-be-activated DL Scells - Proposal 3: - For PUCCH Scell activation with multiple Scells, no UE requirement applies when the unknown Scells (not including PUCCH Scell) to be activated does not have active serving cell(s) or known to-be-activated Scell(s) on the same band. - For PUCCH Scell activation with multiple Scells, RAN4 further study whether the unknown PUCCH Scell should be activated only on the band which contains active serving cell(s) or known to-be-activated Scell(s). Recommendations for 2nd round: Check the tentative agreements and further discuss the 3 proposals. |
| Issue 1-2-2 | Status: 6 companies support option 1a, 1 company supports option 2 and 1 company supports option 3. Tentative agreements: None. Candidate options: Same as first round. Recommendations for 2nd round: Further discuss option 1a, option 2 and option 3. |
| Issue 1-2-3 | Tentative agreements: None. Candidate options: For the case when PUCCH SCell and other DL Scell are within same PUCCH groups (case 1), there are the following options: - Option 1A: (Qualcomm) - For the activation latency requirement of multiple DL Scells being activated in parallel with PUCCH Scell, the legacy requirements (in 8.3.7) are reused with the following update. - TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] which cannot be earlier than the PUCCH Scell activation completion - Option 1B: - For Scell to be activated with PUCCH Scell which is in the secondary PUCCH group, extra delay to wait for UE accomplishing activation of PUCCH Scell needs to be considered in addition to legacy requirements for Scell activation with multiple Scells. - Option 1C: - Do not define separate requirements for PUCCH Scell and other DLSCells. (i.e. no need to differentiate PUCCH SCell). For the case when PUCCH SCell and other DL Scell are within different PUCCH groups (case 2) - Option 2A: - For the case that the downlink Scells are within different PUCCH group from the activated PUCCH Scell, the existing Scell activation delay requirement for deactivated Scell with multiple Downlink Scells defined in clause 8.3.7 of current specification 38.133 can be reused. - Option 2B: - Do not define requirements for this case. Recommendations for 2nd round: Further discuss the options for the two cases. |

CRs/TPs

Discussion on 2nd round (if applicable)

Sub-topic 1-1 PUCCH SCell activation delay requirements

Issue 1-1-1: The known condition of PL-RS

Candidate options:

- Option 1: Update the known condition of PL-RS signals: (same as that in first round)

  - the known condition of PL-RS for known PUCCH SCell could be defined as:

    - The pathloss reference signal is known for known PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

      - Pathloss reference signal activation command is received within 1280 ms upon the last transmission of the RS resource for L3 measurement

      - The UE has sent at least one L3 RSRP report for the target pathloss reference signal before the pathloss reference signal activation command

      - The target pathloss reference signal remains detectable during the PUCCH SCell activation period

        - SNR of the target pathloss reference signal≥-3dB

      - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

        - SNR of the associated SSB ≥-3dB

    - Otherwise, the pathloss reference signal is unknown.

  - the known condition of PL-RS for unknown PUCCH SCell could be defined as:

    - The pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L1-RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

      - Pathloss reference signal activation command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

      - The UE has sent at least one L1-RSRP report for the target pathloss reference signal before the pathloss reference signal activation command

      - The target pathloss reference signal remains detectable during the PUCCH SCell activation period

        - SNR of the target pathloss reference signal≥-3dB

      - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

        - SNR of the associated SSB ≥-3dB

    - Otherwise, the pathloss reference signal is unknown.

- Option 2:

  - the known condition of PL-RS for known PUCCH SCell could be defined as:

    - The pathloss reference signal is known for known PUCCH SCell during activation if the following conditions are met.

      - Last transmission of the RS resource used for L3 RSRP measurement reporting is less than 1280ms from the PUCCH SCell activation command. Where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

      - The target pathloss reference signal remains detectable during the PUCCH SCell activation period

        - SNR of the target pathloss reference signal≥-3dB

      - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

        - SNR of the associated SSB ≥-3dB

    - Otherwise, the pathloss reference signal is unknown.

  - the known condition of PL-RS for unknown PUCCH SCell could be defined as:

    - The pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met.

      - The last transmission of the RS resource used for L1-RSRP measurement reporting is less than 1280ms from the PUCCH SCell activation command. Where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

      - The target pathloss reference signal remains detectable during the PUCCH SCell activation period

        - SNR of the target pathloss reference signal≥-3dB

      - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

        - SNR of the associated SSB ≥-3dB

Recommendations for 2nd round: Further discuss the two options.

| Issue 1-1-1: The known condition of PL-RS |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Okay with Option 2. |
| Ericsson | We prefer Option 2. |
| Huawei | Fine with option 1/2 |
| Apple | We understand PL-RS activation is not very clear, actually here it could be uplink spatial relation activation(together with PL-RS). The only problem to use option 2 is: if the uplink spatial relation activation timing-point has big time offset from SCell activation timing-point, the L3 measurement report or L1 measurement report would be more than 1280ms earlier than the timing when this uplink spatial relation(together with PL-RS) is activated. We propose option 1a as following: - Option 1: Update the known condition of PL-RS signals: (same as that in first round) - the known condition of PL-RS for known PUCCH SCell could be defined as: - The pathloss reference signal is known for known PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. - Uplink spatial relation activation command is received within 1280 ms upon the last transmission of the RS resource for L3 measurement - The UE has sent at least one L3 RSRP report for the target pathloss reference signal before the uplink spatial relation activation command - The target pathloss reference signal remains detectable during the PUCCH SCell activation period - SNR of the target pathloss reference signal≥-3dB - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period - SNR of the associated SSB ≥-3dB - Otherwise, the pathloss reference signal is unknown. - the known condition of PL-RS for unknown PUCCH SCell could be defined as: - The pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L1-RSRP measurement reporting and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. - Uplink spatial relation activation command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least one L1-RSRP report for the target pathloss reference signal before the uplink spatial relation activation command - The target pathloss reference signal remains detectable during the PUCCH SCell activation period - SNR of the target pathloss reference signal≥-3dB - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period - SNR of the associated SSB ≥-3dB - Otherwise, the pathloss reference signal is unknown. |
| CATT | Fine with both options. |
| Nokia | We are fine with Option 2. Could we reword the sentence a bit to align with other conditions? And the latter part “where the RS resource is the target ….” is already in the first paragraph, hence seems no need to repeat it? The reworded condition is as below and in the draft CR: Option2: - Last transmission of the RS resource used for L3 RSRP measurement reporting is less than 1280ms from the PUCCH SCell activation command. Where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. Now reworded to: - PUCCH SCell activation command is received within 1280 ms upon the last transmission of the RS resource for L3 measurement reporting To Apple, the concern here is PL-RS may be derived based some default RS as agreed in previous meeting, i.e. UL spatial relation is not mandatory command for PL estimation. Additionally, we agreed in 1st round that PL-RS discussion also applies to FR1. The conditions here should be general enough to apply to both FR1 and FR2, but UL spatial relation is not needed for FR1 either? |
| MediaTek | Support option 2. |

Issue 1-1-2: Whether the PL-RS will introduce extra delay time when the known condition is met in FR2 (the value of [X] in 8.3.12)?

Candidate options:

- Option 1: (Apple, Qualcomm, Intel, OPPO, MTK, Huawei)

  - When PL-RS of target PUCCH SCell is known, the X=5 sample measurement time is always considered and no need to consider condition of ‘maintain’ or ‘not maintain’.

- Option 2: (Nokia)

  - If PUCCH Scell is known in FR2, the reported L3 measurement results can be reused for pathloss estimation and additional PL-RS measurement is not needed during PUCCH Scell activation.

  - If PUCCH Scell is unknown in FR2, [X] = 0 if PL-RS is maintained and [X] = 5 if PL-RS is not maintained.

- Option 3: (Ericsson)

  - If the to be activated PUCCH Scell share the same PL-RS with the active serving cell or the PL-RS of active serving cell and the PL-RS of the to be activated PUCCH Scell are configured to be QCL Type-A, PL-RS can be considered to be maintained for to be activated PUCCH Scell.  In this case X value can be 0.

  - X=4 when the PUCCH Scell activated is unknown at the reception of PUCCH Scell activation command and the RS used for L1-RSRP is same as PL-RS.

  - For all other cases where PL-RS not maintained is X=5.

Recommendations for 2nd round: Further discuss the three options.

| Issue 1-1-2: Whether the PL-RS will introduce extra delay time when the known condition is met in FR2 (the value of [X] in 8.3.12)? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support Option 1. |
| Ericsson | In the existing intra-band CA configurations from 38.101, maximum of 8 CC can be configured. We agree that PUCCH may not be configured commonly for intra-band CA as the number of CC supported is only 8 and dual PUCCH may not be a common case. With this there may not be a case where PL-RS can be shared between the to be activated PUCCH SCell and active serving cell. We are fine to drop first bullet of proposal 3. With this our revised proposal is - X=4 when the PUCCH SCell activated is unknown at the reception of PUCCH SCell activation command and the RS used for L1-RSRP is same as PL-RS. - For all other cases where PL-RS not maintained is X=5. |
| Huawei | We support option 1. |
| Apple | Support option 1. |
| CATT | Support option 3 but can compromise to option 1 to move forward. |
| Intel | Prefer option 1. We prefer to define more generic requirement. |
| Nokia | We support Option 2. It seems our questions were not fully solved in the 1st round discussion. To simplify the discussion, we understood the key issues are the following: - In what conditions can PL-RS be considered as “maintained”? - What is the difference between “PL-RS is maintained” and “PL-RS is L3 measured”? It has been understood 5 samples are needed for PL estimation. But now even the UE has acquired L3 measurements, and the PL-RS is determined based on the measurements, the UE vendors still think PL-RS is “not maintained”. It would be good if we could align the understanding on above questions so as to move forward on the PL-RS measurement delay discussion. |
| MediaTek | Support option 1 as our comment in 1st round. |
| vivo | Ok with option 1. We think Ericsson’s new proposal is ok however a generic requirement is preferred. |

Issue 1-1-3: Update the PUCCH Scell activation delay requirements about PL-RS measurement in FR2 for invalid TA case?

Tentative agreements:

- Update the previous agreement about FR2 PL-RS measurement, from a sequential processing to a parallel processing, i.e. PL-RS can be measured in parallel with “DL CSI-RS reception/processing” and “UL TA acquisition”.

  - Previous agreement: THARQ+Tactivation_time+[X]+max{(Tfirst_available_CSI+TCSI_processing),(T1+T2+T3)}+TCSI_reporting_after

  - Proposed update: THARQ+Tactivation_time+max{(Tfirst_available_CSI+TCSI_processing),(T1+T2+T3),[X]}+TCSI_reporting_after

Recommendations for 2nd round: Please MTK check if the tentative agreement can be acceptable based on Huawei’s response.

| Issue 1-1-3: Update the PUCCH Scell activation delay requirements about PL-RS measurement in FR2 for invalid TA case? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support the tentative agreements. |
| Huawei | Fine with the tentative agreements |
| Apple | Fine with tentative agreement. |
| CATT | Support the tentative agreement. |
| Intel | OK with the tentative agreements. |
| Nokia | Support the tentative agreements. This has been captured in the draft CR of PUCCH SCell activation delay. |
| MediaTek | Thanks for the HW’s comment in 1st round. We can support the tentative agreement if RAN4 agreed to add a note that “For known PUCCH Scell, the PDCCH order (when applicable) and the activation command for TCI, UL spatial relation, and PL-RS are based on latest valid L3-RSRP reporting.” The reason is because, as HW’s 1st comment, the beam for PRACH is configured in PDCCH ordered which is based on latest reported L1/L3 measurement, and the Tx power of PRACH can be determined accordingly. However, according to the agreement in the last meeting as follows, RAN4 only agreed the unknown case and the PDCCH order is based on valid L1-RSRP. It seems no explicit agreement for the known case. - For unknown case where beam indication is needed, the requirements only apply when: - And the PDCCH order (when applicable) and the activation command for TCI, UL spatial relation, and PL-RS are based on latest valid L1-RSRP reporting via Primary PUCCH group.. |
| vivo | Ok with the tentative agreement |
| CATT2 | To MTK: Based on the following agreement in previous meeting, for known PUCCH Scell, no need to indicate beam information to trigger the PDCCH order. So we guess the note is not needed. - If the target PUCCH Scell is known, no need to indicate the beam information to network for determining the associated SSB in PDCCH order for RA, i.e., no additional SSB based beam measurement is needed. |

Issue 1-1-4: Whether the PL-RS will introduce extra delay time when the known condition is met in FR1?

Tentative agreements:

- When PL-RS of target FR1 PUCCH SCell is known, the same number of samples for PL-RS measurement as FR2 should be added to the FR1 PUCCH Scell activation latency requirement.

Recommendations for 2nd round: Check the tentative.

| Issue 1-1-4: Whether the PL-RS will introduce extra delay time when the known condition is met in FR1? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support the tentative agreements. |
| Ericsson | May be a small correction. When PL-RS of target FR1 PUCCH SCell is known or unknown, the same number of samples for PL-RS measurement as FR2 should be added to the FR1 PUCCH SCell activation latency requirement. |
| Huawei | Support tentative agreements. To Ericsson: we think if the PL-RS is unknown, then longer delay is expected. |
| Apple | Fine with tentative agreement. We don’t need to include unknown case here, because RAN4 previously agreed that we would not consider exact delay requirement for PL-RS unknown condition in requirement: - If the PL-RS of target PUCCH SCell is unknown, clarify that “longer activation time is expected if the pathloss reference signal is unknown.” in the spec. |
| CATT | Support the tentative agreement. And agree with Huawei and Apple that we have agreed not to define detailed requirements when PL-RS is unknown. |
| Intel | Fine with the tentative agreements. Same view as Apple. |
| Nokia | Support the tentative agreement. |
| MediaTek | Support tentative agreement. |
| vivo | Ok with the tentative agreement |

Issue 1-1-5: Interruption due to PRACH transmission of PUCCH Scell (related to RAN1 reply LS R4-2206147(R1-2202599))?

Candidate options:

- Option 1:

  - In the inter-band CA case, it is up to UE implementation if network schedules concurrent transmission of PRACH and PUSCH/PUCCH/SRS for inter-band CA, and UE does not support parallelTxPRACH-SRS-PUCCH-PUSCH.

- Option 1a: (Nokia)

  - The PUCCH Scell activation delay requirement (Tdelay_PUCCH Scell) shall apply provided that:

    - The RA on PUCCH Scell is not interrupted by the PUCCH/PUSCH/SRS on other activated serving cells otherwise additional delay to activate the Scell is expected.

- Option 2:

  - Clarify that there could be interruption on UL transmission on other serving Cells when colliding with RACH transmission on PUCCH Scell if UE is not capable of parallelTxPRACH-SRS-PUCCH-PUSCH (i.e. PRACH transmission is prioritized).

Recommendations for 2nd round: Further discuss the options.

| Issue 1-1-5: Interruption due to PRACH transmission of PUCCH Scell (related to RAN1 reply LS R4-2206147(R1-2202599))? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Okay with both Option 1 and 2. |
| Ericsson | We support option 2 for the following reason. In general, dual PUCCH SCell may be deployed in inter-band CA case. For example, between FR1+FR2 CA or FR2 inter-band CA. One of the main use case as per our understanding is FR1+FR2 CA where the FR1 provides umbrella coverage and FR2 cells are deployed for capacity. In this case, FR1 and FR2 cells are generally non-collocated. Due to the non-colocation ideal backhaul may or may not be available. In case of non-ideal backhaul, PDCCH order for PRACH on PUCCH SCell can always collide with UL transmissions like SRS or PUSCH or PUCCH to other cells. Since PRACH is more important compared to other UL signals, we think it is necessary for UE to prioritize the PRACH if UE cannot transmit both the transmissions. |
| Huawei | We support option 2. |
| Apple | Fine with option 1 and option 2. |
| CATT | Support option 2 and clarify that this is for inter-band case when drafting CR. |
| Intel | Prefer option 2. |
| Nokia | We understand the UE prioritization rule is not clearly defined in RAN1 in case of collision, so it is up to UE implementation how to behave so Option 1 was preferred. And under this option, we can only state the activation delay requirement applies only if no interruption occurs. But we can also go for Option 2 if the prioritization can be clarified in RAN4. |
| MediaTek | Either option 1 or option 2 is fine to us. In our view, if we want to introduce the interruption requirement, it can be introduced in 8.2.1.2.20, 8.2.2.2.18 and 8.2.3.2.19. |
| vivo | Fine with option 1. Fine with option 2 without (PRACH transmission is prioritized). Prioritization rules need be checked with RAN1. |

Sub-topic 1-2 PUCCH Scell activation delay requirements with multiple DL Scells

Issue 1-2-1: The scenarios of PUCCH Scell activation with multiple DL Scells?

Tentative agreements:

- The delay requirements for PUCCH SCell activation with multiple DL SCells are defined for EN-DC, NE-DC, NR SA.

Candidate options:

- Proposal 1:

  - Multiple DL Scells to be activated along with PUCCH Scell shall be in the same PUCCH group as the PUCCH Scell. Any to be activated DL Scell in parallel with PUCCH Scell should not be activated earlier than the PUCCH Scell.

- Proposal 2:

  - For the activation latency requirement of multiple DL Scells being activated in parallel with PUCCH Scell, do not consider the following scenarios:

    - FR1 unknown non-contiguous CA scenario which includes N1*Trs in the latency requirement

    - One of serving cells is activated in the same band as to be activated FR2 PUCCH Scell

    - Unknown PUCCH Scell with known to-be-activated DL Scells

- Proposal 3:

  - For PUCCH Scell activation with multiple Scells, no UE requirement applies when the unknown Scells (not including PUCCH Scell) to be activated does not have active serving cell(s) or known to-be-activated Scell(s) on the same band.

  - For PUCCH Scell activation with multiple Scells, RAN4 further study whether the unknown PUCCH Scell should be activated only on the band which contains active serving cell(s) or known to-be-activated Scell(s).

Recommendations for 2nd round: Check the tentative agreements and further discuss the 3 proposals.

| Issue 1-2-1: The scenarios of PUCCH Scell activation with multiple DL Scells? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Do not support Proposal 3. For Proposal 1, regarding whether DL SCells to be activated in parallel with PUCCH SCell can belong to different PUCCH groups, we are not positive. However, if the requirement can be defined without much complication due to inter-PUCCH group interruptions, we are open to the scenario. We support Proposal 2, and for the clarification on the first bullet of Proposal 2, what is meant by “N1*Trs” is that we do not want to consider the case where multiple unknown to-be activated SCells are not contiguous with each other, i.e. those SCells need to be more or less sequentially activated. And it does not even include L1-RSRP based TCI activation procedure if we are not wrong. In addition, the activation delay is already so long that we do not see the value of defining requirements for such a case. |
| Ericsson | We think proposal 1 is restrictive and we prefer to keep generic as possible. We think we need to consider following two scenarios - All SCells belonging to secondary PUCCH group - I think all companies are fine with this case. - SCells belonging to primary plus secondary PUCCH group - We think only invalid TA case may be bit complicated and we can keep it FFS till next meeting if companies need more time for analyzing it. |
| Huawei | Fine with two scenarios proposed by Ericsson. Regarding the first bullet in option 2. We think this case (multiple unknown scell within same band)is not precluded in legacy requirements. So can QC clarify the different here? |
| Apple | Fine with proposal 1, or if no interruption is assumed between PUCCH groups (PUCCH SCell activation interrupts SCell activation in primary PUCCH group), we can accept to have SCells in both PUCCH groups. We agree with bullet #2/#3 of proposal 2. Regarding bullet #1 of proposal 1, we understand the motivation from Qualcomm is to preclude some less likely scenarios, then we have no strong view on it. We agree with bullet #1 of proposal 3. But for bullet #2 of proposal 3, as we commented in the first round, we don’t think there could be an active serving cell on the same band as target PUCCH Scell. |
| CATT | Support the tentative agreement. For proposal 1, support the two scenarios proposed by Ericsson. For proposal 2, we are fine with the 3rd bullet but we don’t understand why the case in 2nd bullet needs to be precluded. For proposal 3, the first bullet seems contrary with the 2nd bullet of proposal 2. For the second bullet, it is already under discussion in proposal 2 and proposal 3 and no need to have this bullet. |
| Intel | Prefer Option 1. We are open to discuss the scenario that multiple SCells are in different PUCCH group where requirement is not too complicated. |
| Nokia | We also prefer not restricting the scenario to Option1. We can support the scenarios from Ericsson. |
| MediaTek | For proposal 1, fine with Ericsson’s proposal. For proposal 2, support 3th bullet. For proposal 3, support 1st bullet and compromise not to consider 2nd bullet. |

Issue 1-2-2: The activation delay requirements for the PUCCH Scell to be activated?

Candidate options:

- Option 1a: (MTK)

  - PUCCH Scell activation delay requirements can be derived from single PUCCH Scell activation delay by replacing Tactivation_time with Tactivation_time_multiple_scells. And the following updates are made for Tactivation_time_multiple_scells:

    - Tuncertainty_MAC_multiple_scells is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and spatial relation, and Scell activation command of this unknown PUCCH Scell.

    - TSMTC_MAX_multiple_scells and TFirstSSB_MAX_multiple_scells should additionally consider PUCCH Scell.

- Option 2: (Nokia)

  - The PUCCH Scell activation shall be prioritized over the other being-activated DL Scells when they are activated by a single MAC command.

  - The Scell activation delay for PUCCH Scell with multiple DL Scells is the same as the single PUCCH Scell activation delay in clause 8.3.12 i.e. Tdelay_PUCCH_Scell.

- Option 3: (Ericsson)

  - Scenario 1: CSI report of all Scells transmitted on SpCell

    - Only valid TA case can be assumed

    - Tactivate_total = THARQ+Tdelay_PUCCH_multiple_SCellsNR slot length, where:

      - Tdelay_PUCCH_ multiple_Scells = Tactivation_time_multiple_scells + [X] + TCSI_Reporting, where:

        - Tactivation_time_multiple_scells is the target Scell activation delay in millisecond in multiple Scell activation scenario as specified in section 8.3.7.

        - TCSI_Reporting is same as specified in clause 8.3.7

  - Scenario 2: CSI report of primary PUCCH group transmitted on Pcell and CSI report of secondary PUCCH group transmitted on PUCCH Scell

    - For valid TA case, Tactivate_total = THARQ+Tdelay_PUCCH_multiple_SCellsNR slot length, where:

      - Tdelay_PUCCH_ multiple_Scells = Tactivation_time_multiple_scells + [X] + max (TCSI_Reporting_Primary_Group, TCSI_Reporting_Secondary_Group), where:

        - TCSI_Reporting_SpCell is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources on primary PUCCH group, as specified in TS 38.331 [2].

        - TCSI_Reporting_PUCCH_Scell is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources on secondary PUCCH group, as specified in TS 38.331 [2].

    - For invalid TA case, Tactivate_total = THARQ+Tdelay_PUCCH_multiple_SCellsNR slot length, where:

      - Tdelay_PUCCH_ multiple_Scells = Tactivation_time_multiple_scells + [X] + max (max ((TFirst_available_CSI_PUCCH_Scell + TCSI_processing_PUCCH_Scell), (T1+T2+T3)) + TCSI_reporting_after_PUCCH_Scell, TCSI_Reporting_SpCell + Tuncertainity), where:

        - Tfirst_available_CSI_ PUCCH_Scell: the delay uncertainty in acquiring the first available downlink CSI reference resource for PUCCH Scell.

        - TCSI_processing_ PUCCH_Scell: the UE processing time for CSI reporting of PUCCH Scell.

        - TCSI_reporting_after_ PUCCH_Scell the delay uncertainty in acquiring the first available CSI reporting resource after T3

        - Tuncertainity is uncertainty of first available CSI reporting resource, if CSI report on primary PUCCH group collides with PRACH on PUCCH Scell. In other cases, it is zero.

    - When the PRACH and CSI report partially overlap in time during multiple Scell activation involving PUCCH Scell, UE should transmit PRACH and postpone the CSI report to next CSI reporting instance.

Recommendations for 2nd round: Further discuss option 1a, option 2 and option 3.

| Issue 1-2-2: The activation delay requirements for the PUCCH Scell to be activated? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support Option 1a with the modification that Apple and MediaTek mentioned in the first round. Besides, the wording “additionally consider PUCCH Scell” in the second bullet of the option needs to be clarified, i.e. TSMTC_MAX_multiple_scells and TFirstSSB_MAX_multiple_scells should additionally consider also include PUCCH Scell. |
| Ericsson | Option 3 as it covers most of the cases |
| Huawei | We support option 2. For option 1, it seems PUCCH SCell activation has same priority as other Scell. Then the delay of PUCCH Scell activation is derived by N1*Trs. Then for all other Scells, the TCSI_reporting will be unnecessary long as discussed in 1-2-3. |
| Apple | Support option 1a with “UL spatial relation”, and fine with Qualcomm’s revision. |
| CATT | Option 1a is only for the case when PUCCH Scell and other SCells are within the same PUCCH group. We are fine to consider the scenarios mentioned in option 3, but for the case the PUCCH SCell and other DL Scells are within different PUCCH groups, no need to consider the collision between the CSI reporting in different PUCCH groups and the requirements for different PUCCH group should be defined separately. |
| Intel | Support the first bullet of option 2. For option1a, it’s fine if SCell activation delay is modified to make sure that the delay is longer than PUCCH SCell activation delay. Otherwise, option 1a needs modification. |
| Nokia | We support Option2. The intention of Option 2 is to prioritize the activation of PUCCH SCell. At least it shall be prioritized over SCells in the same PUCCH group. Option 1a considers the worst case where PUCCH SCell activation may be delayed by all the other SCells. This seems not very reasonable as it makes no sense to prioritize the other SCells whose CSI reporting depends on the PUCCH SCell activation. |
| MediaTek | Support option 1a, and fine with QC’s version. Besides, we think the requirement in proposal 1a may also be applicable to different PUCCH SCell case, i.e. all SCell share the same Tactivation_time_multiple_scells. The main difference compared with R16 is the CSI reporting time. Because, some report should be transmitted on SpCell while some report should be transmitted on PUCCH SCell. |

Issue 1-2-3: The activation delay requirements for the other DL SCells to be activated?

Candidate options:

For the case when PUCCH SCell and other DL Scell are within same PUCCH groups (case 1), there are the following options:

- Option 1A: (Qualcomm)

  - For the activation latency requirement of multiple DL Scells being activated in parallel with PUCCH Scell, the legacy requirements (in 8.3.7) are reused with the following update.

    - TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] which cannot be earlier than the PUCCH Scell activation completion

- Option 1B:

  - For Scell to be activated with PUCCH Scell which is in the secondary PUCCH group, extra delay to wait for UE accomplishing activation of PUCCH Scell needs to be considered in addition to legacy requirements for Scell activation with multiple Scells.

- Option 1C:

  - Do not define separate requirements for PUCCH Scell and other DLSCells. (i.e. no need to differentiate PUCCH SCell).

For the case when PUCCH SCell and other DL Scell are within different PUCCH groups (case 2)

- Option 2A:

  - For the case that the downlink Scells are within different PUCCH group from the activated PUCCH Scell, the existing Scell activation delay requirement for deactivated Scell with multiple Downlink Scells defined in clause 8.3.7 of current specification 38.133 can be reused.

- Option 2B:

  - Do not define requirements for this case.

Recommendations for 2nd round: Further discuss the options for the two cases.

| Issue 1-2-3: The activation delay requirements for the other DL SCells to be activated? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Perhaps we can simply reuse the legacy multiple SCell activation requirement because TCSI_reporting already says “available CSI reportin resources.” In other words, the parameter already allows UE to wail until PUCCH SCell is activated and the CSI report resource is made available. Note that the CSI can be also reported to primary PUCCH group. If we add something specific to PUCCH SCell, it may not be correct for the case where non-zero CQI needs to be reported to the other PUCCH group. Besides, if RAN4 defines requirements for PUCCH SCell activation with multiple parallel SCells belonging to different PUCCH groups, the suggested wording in the options above won’t be applicable. - TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2]. |
| Ericsson | To avoid rediscussing many scenarios, we prefer not to treat PUCCH SCell differently. Case 1: Option 1C. Case 2: option 2A for downlink part of the delay. Exact equations are proposed in our paper. |
| Huawei | We think QC’s comment is a good approach, “first available CSI reporting resources” can serve both case 1 and case 2. |
| Apple | Case 1: option 1A and QC’s suggestion. Case 2: prefer option 2B. But if companies agreed with this scenario, we are wondering if the RACH on PUCCH SCell would also interrupt the L1-RSRP reporting on other PUCCH group. If no interruption is assumed, then we think the option 2A or QC’s suggestion could be used. |
| CATT | We are fine with QC’s suggestion and we also think it can apply for both case 1 and case 2. |
| Intel | Case 1: Fine with Option 1A. case 2: depends on issue 1-2-1. If yes, fine with option 2A. |
| Nokia | Case 1: Option 1A. We still prefer some clarification like the last sentence. Although Tcsi-reporting is defined quite general, it would be clearer to indicate the dependency on PUCCH SCell activation. Case 2: Option 2A. |
| MediaTek | Case 1: support option 1c Case 2: support 2A. One question to QC for clarification: For option 1A, we would like to further check as our comment in GTW. The activated in parallel you mentioned, does it mean one additional searcher is needed for PUCCH SCell activation? |

Topic #2: PUCCH Scell activation/deactivation performance requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207775 | Apple | TC for PUCCH Scell activation and deactivation delay requirements of FR1 unknown PUCCH Scell and one FR1 unknown Scell (All NR cells in FR1) R17 |
| R4-2207957 | Qualcomm Incorporated | Proposal 1: For PUCCH Scell activation without a valid TA, the cell should belong to a different TAG from SpCell. Proposal 2: PL-RS, TCI state, and spatial relation if applicable shall be associated with one SSB ID. And an explicit PL-RS indication can be skipped in the test cases. Proposal 3: For performance test cases, we propose to make modifications as presented in Table 1. |
| R4-2208089 | Nokia, Nokia Shanghai Bell | draftCR on TC3 PUCCH Scell activation of FR2 known cell in SA |
| R4-2208107 | Xiaomi | CR on TC for PUCCH Scell activation and deactivation delay requirements |
| R4-2208178 | CATT | Proposal 1: The test cases indicated ‘N/A’ are considered not needed to develop due to the PUCCH Scell activation and deactivation delay in these test cases can be tested in other similar test cases. Proposal 2: For unknown PUCCH Scell activation in FR2, the test cases can be specified based on UE having capability of supporting cross PUCCH group CSI reporting and SS supporting control UE cross cells. Proposal 3: The configurations and process for the test cases of PUCCH Scell activation/deactivation with multiple Scells can be discussed and specified firstly and the test requirements can be [TBD] and to be completed when the core requirements are specified. Proposal 4: Further discuss the presented test cases and endorse the test cases. |
| R4-2208179 | CATT | Test case for PUCCH Scell activation and deactivation delay requirements |
| R4-2208352 | OPPO | Proposal 1: RAN4 decide whether to introduce test cases for PUCCH Scell activation/deactivation delay requirements with multiple DL Scells. Proposal 2: The tests with valid TA cases are not necessary. |
| R4-2208353 | OPPO | draft CR for PUCCH Scell activation and deactivation delay requirements of FR1 known cell for EN-DC (TC10) |
| R4-2208463 | MediaTek Inc. | Draft CR on TC for PUCCH Scell activation and deactivation delay requirements of FR1 unknown cell |
| R4-2208519 | CMCC | Proposal 1: it is proposed to define test cases for following scenarios: - Specify Scell Activation Delay Requirement for Deactivated PUCCH Scell - Specify Scell Activation Delay Requirement for Deactivated PUCCH Scell with Multiple Scells - Specify Scell Deactivation Delay Requirement for Activated PUCCH Scell - Specify Scell Deactivation Delay Requirement for Activated PUCCH Scell with Multiple Scells Proposal 2: PUCCH Scell activation/deactivation, it is proposed to define test cases for interruption requirements. Proposal 3: For Scell activation delay requirements, it is proposed to define test cases for both valid TA and invalid TA. Proposal 4: For Scell activation delay requirements, it is proposed to define test cases for both known case and unknown case. |
| R4-2208944 | Huawei, Hisilicon | Observation 1: For PUCCH Scell activation test cases, UE should be tested whether it can transmit valid CSI via PUCCH Scell. Proposal 1: Define two sub test in each test cases for valid TA and invalid TA scenarios, where PUCCH Scell is configured with sTAG. TimeAlignmentTimer should expires before UE receives the activation command for invalid TA case, and UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment for sTAG for valid TA case. Proposal 2: For NR SA case, when the to-be-activated PUCCH Scell is in FR2 and Pcell is in FR1, only define test cases for known case with valid TA. Another FR2 serving cell should be configured in the test, and TE shall guaranteed that the FR2 serving cell is successfully activated before activation the PUCCH Scell. Observation2: For EN-DC, if the PSCell is in FR2, UE cannot be configured with a PUCCH Scell in FR2. Proposal 3: For EN-DC case, when the to-be-activated PUCCH Scell is in FR2, only define test cases for known case with valid TA. Another FR2 serving cell should be configured in the test, and TE shall guaranteed that the FR2 serving cell is successfully activated before activation the PUCCH Scell. |
| R4-2208945 | Huawei, Hisilicon | Draft CR on TC for PUCCH Scell activation and deactivation |
| R4-2210137 | Ericsson | TC for PUCCH Scell activation and deactivation delay requirements of FR2 known el with FR1 Pcell |

Open issues summary

Sub-topic 2-1 Test configuration

Issue 2-1-1: Reference signals configuration for PL-RS, TCI state, and spatial relation

- Proposal 1: (Qualcomm)

  - PL-RS, TCI state, and spatial relation if applicable shall be associated with one SSB ID. And an explicit PL-RS indication can be skipped in the test cases.

- Recommended WF

  - Need more discussion

| Issue 2-1-1: Reference signals configuration for PL-RS, TCI state, and spatial relation |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Although we agree that an explicit restriction on the association among PL-RS, TCI state, and spatial relation does not have to be specified in core requirement spec, those should be associated with one SSB ID, which we believe is the most prevalent configuration in the real field. Besides, an explicit PL-RS indication can be skipped in the tests which is also in line with how it is configured in the real field. |
| Apple | Fine with proposal 1 to simplify the TC. |
| Nokia | We are fine with the proposal. |
| Huawei | Fine with proposal 1. |
| MediaTek | Ok with proposal 1. |
| Ericsson | Fine with the proposal |
| Intel | OK with proposal 1. |
| CATT | Fine with the proposal. |
| OPPO | Fine with proposal 1. |

Issue 2-1-2: Whether to define test cases for both known and unknown case

- Proposal 1: (CMCC, CATT)

  - Define test cases for both known case and unknown case.

- Proposal 1a: (CATT)

  - For unknown PUCCH SCell activation in FR2, the test cases can be specified based on UE having capability of cross PUCCH group CSI reporting and SS capable of controlling UE cross cells.

- Recommended WF

  - Need more discussion

| Issue 2-1-2: Whether to define test case for both known and unknown case |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support Proposal 1. For FR1 unknown PUCCH Scell with other cells in a different frequency band in FR1: - There should be two different test cases. One out of the two can be selected by UE capability on L1-RSRP report across PUCCH groups. - Case#1-1: ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted. - Both ‘with’ and ‘without’ a valid TA - Case#1-2: ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is not provided in same MAC PDU with Scell activation, i.e. L1-RSRP measurement and report are needed as part of PUCCH Scell activation - ‘without’ a valid TA - For UE capable of CSI report across PUCCH groups, Case#1-2 is selected and Case#1-1 is skipped. For the other Ues, case#1 is selected for the test. For FR2 unknown PUCCH Scell with FR1 Pcell: - Depending on UE capability of CSI report across PUCCH groups and the cells are configured as single TAG vs. dual TAG, one of the following cases can be selected for the test. - Case#2-1: With a valid TA - Case#2-2: Without a valid TA for UE capable of CSI report across PUCCH groups. The UE can skip Case#1. Please see the diagram below. |
| Apple | Fine with proposal 1 and 1a but the necessity of specific TC shall be checked case by case considering FR1+FR2 testability issue. |
| Nokia | Agree with proposal 1. We also agree that the unknown test cases shall be specified assuming UE supporting the cross PUCCH group CSI reporting. We may need also consider the time relaxation of transmitting CSI reporting on SpCell which is indicated in UE capability. |
| Huawei | Agree with proposal 1. |
| MediaTek | Support proposal 1. How to reduce the test case can be further discuss in the following discussion. |
| Ericsson | Support proposal 1 |
| Intel | OK with Proposal 1. |
| CMCC | Support proposal 1. |
| CATT | Support proposal 1 and 1a. For the unknown cell, the requirements are defined only for the UE supporting cross PUCCH group reporting, so we are not sure whether the sub-tests proposed by QC is needed. |
| OPPO | Fine with proposal 1. |

Issue 2-1-3: Whether to define test cases for both valid TA and invalid TA case

- Proposal 1: (OPPO)

  - The tests with valid TA cases are not necessary.

- Proposal 2: (CMCC, Qualcomm, Huawei)

  - Define test cases for both valid TA and invalid TA.

- Proposal 2a: (Qualcomm)

  - For PUCCH SCell activation without a valid TA, the cell should belong to a different TAG from SpCell.

- Proposal 2b: (Huawei)

  - Define two sub tests in each test case for valid TA and invalid TA scenarios, where PUCCH Scell is configured with sTAG.

  - TimeAlignmentTimer should expires before UE receives the activation command for invalid TA case

  - UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment for sTAG for valid TA case.

- Recommended WF

  - Need more discussion

| Issue 2-1-3: Whether to define test case for both valid TA and invalid TA case |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support Proposal 2 and 2a. For Proposal 2b: It does not seem to be really about “whether to define test cases for both valid TA and invalid TA case” but how to configure parameters when the answer to Issue 2-1-3 is “yes” |
| Apple | Fine with option 2/2a/2b. |
| Nokia | Agree with proposal 2. 2a and 2b are not conflicting, but 2b is more elaborated hence 2b is preferred. PUCCH Scell may be configured for different purposes, so both valid and invalid cases shall be considered. |
| Huawei | Support option 2/2a/2b |
| MediaTek | Support proposal 2/2a/2b. The UE behavior before and after timer expired is different. Thus, we prefer to test both valid and invalid TA case. |
| Intel | OK with option 2/2a/2b |
| CMCC | Support option 2/2a/2b |
| CATT | Support option 2, 2a and 2b. |
| OPPO | Ok to define two sub tests in each test case for valid TA and invalid TA scenario if most companies agreed. |

Issue 2-1-4: Test cases for interruption requirements due to PUCCH Scell activation/deactivation

- Proposal 1: (CMCC)

  - Define test cases for interruption requirements due to PUCCH Scell activation/deactivation.

- Recommended WF

  - Need more discussion

| Issue 2-1-4: Test cases for interruption requirements due to PUCCH Scell activation/deactivation |  |
| --- | --- |
| Company | Comments |
| Qualcomm | If the purpose of interruption testing is more or less the same as legacy Scell activation, Proposal 1 is okay with us. |
| Apple | Different views from proposal 1. The interruption could be verified during the activation TC and no need to have individual TC for interruption. |
| Nokia | Not sure what the proposal intends for. In legacy Scell activation test cases, interruption is also verified in the same test case for activation delay. No additional test case is needed. |
| MediaTek | Same view as Nokia |
| Ericsson | Same view as Apple. |
| Intel | Fine to verify interruption in the same activation delay test. |
| CMCC | If the similar approach of legacy Scell activation test is adopted, i.e. interruption is verified during the activation TC, we are also fine. |
| CATT | Fine with proposal 1 and the interruption is verified in the delay TC. |
| OPPO | Same view as Nokia. |

Issue 2-1-5: Whether the test cases for the following band combinations are needed?

Moderator: please add your preference (Yes or no) in the table directly and provide your technical reasons in the comment table.

| Company | FR1 Pcell/PSCell + FR1 PUCCH SCell | FR1 Pcell/PSCell + FR2 PUCCH SCell | FR2 Pcell/PSCell + FR1 PUCCH SCell | FR2 Pcell/PSCell + FR2 PUCCH SCell(intra-band) | FR2 Pcell/PSCell + FR2 PUCCH SCell(inter-band) |
| --- | --- | --- | --- | --- | --- |
| Qualcomm | Yes | Yes | No | No | Yes and No |
| Apple | Yes | No, due to FR1+FR2 testability issue | No, due to FR1+FR2 testability issue | No, intra-band SCell shall be in same PUCCH group as Pcell/PSCell (as also mentioned in RAN1 LS) | Yes |
| Nokia | Yes | Yes | Yes | Yes | Yes. This depends on the discussion on FR2 inter-band CA in Rel17 RF FR2 WI. |
| Huawei | Yes | Yes | Low priority | No | Yes |
| MediaTek | Yes | More discussion is needed | More discussion is needed | Open to discuss | Open to discuss |
| Ericsson | Yes | Yes, If the CSI is sent on PUCCH SCell then FR1+FR2 testability may not be an issue. | Same views as Huawei | No | Yes |
| CMCC | Yes | Open to discuss | Open to discuss | Open to discuss | Yes |
| CATT | Yes | Yes | Open to discuss | Open to discuss. | Yes |
| OPPO | Yes | Open to discuss | No | No | Yes |

- Recommended WF

  - Need more discussion

Technical comments on the band combination:

| Issue 2-1-5: Band combination configuration in the test cases |  |
| --- | --- |
| Company | Comments |
| Qualcomm | FR1 Pcell/PSCell + FR1 PUCCH Scell: Yes - FR1 known PUCCH Scell (with SpCell in a different frequency band in FR1): - For the detailed configurations, the measurement period of the Scell being activated is equal to or smaller than 2400ms, and both ‘with’ and ‘without’ a valid TA can be tested. For ‘without a valid TA’, the PUCCH Scell shall belong to a secondary TAG. - FR1 unknown cell (with SpCell in a different frequency band in FR1): - There should be two different test cases. One out of the two can be selected by UE capability on L1-RSRP report across PUCCH groups. - Case#1: ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted. - Both ‘with’ and ‘without’ a valid TA - Case#2: ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is not provided in same MAC PDU with Scell activation, i.e. L1-RSRP measurement and report are needed as part of PUCCH Scell activation - ‘without’ a valid TA - For UE capable of CSI report across PUCCH groups, Case#2 above is selected and Case#1 is skipped. For the other Ues, case#1 is selected for the test. FR1 Pcell/PSCell + FR2 PUCCH Scell: Yes - FR2 known PUCCH Scell with FR1 SpCell : - Both ‘with’ and ‘without’ a valid TA can be tested based on either SP CSI-RS or P CSI-RS for CSI acquisition. - FR2 unknown PUCCH Scell with FR1 SpCell - Case#1: With a valid TA - Case#2: Without a valid TA for UE capable of CSI report across PUCCH groups. The UE can skip Case#1. FR2 Pcell/PSCell + FR1 PUCCH Scell: No - It is a very unlikely deployment. FR2 Pcell/PSCell + FR2 PUCCH Scell(intra-band): No - FR2 known PUCCH Scell with intra-band FR2 SpCell: - We haven’t seen any commercial deployment with intra-band CA between dual PUCCH groups. - FR2 unknown PUCCH Scell with intra-band FR2 SPCell: - It is an invalid scenario according to the legacy requirements. FR2 Pcell/PSCell + FR2 PUCCH Scell(inter-band): Yes and No - FR2 known PUCCH Scell with inter-band FR2 SpCell - It is okay for us to not consider the case because it may require other UE capabilities to support this case, e.g. IBM. - FR2 unknown PUCCH Scell with inter-band FR2 SpCell With the same reason above, it is okay for us to not consider the case. For unknown FR2 PUCCH Scell, it can be tested under ‘FR2 unknown PUCCH Scell with FR1 SpCell’ scenario. |
| Apple | Inter-band shall be as baseline configuration for Pcell/PSCell and target PUCCH Scell; and FR1+FR2 CA shall be avoided due to FR1+FR2 testability issue. |
| MediaTek | For FR1 SpCell + FR2 PUCCH SCell, more discussion is needed. For known PUCCH SCell, because L3 measurement report can be transmitted on SpCell, it seems that as long as prior to the start of test case L3 report can be received by NW, and the starting point of the test case is UE receives one MAC CE containing activation command, TCI state and spatial relation. The following PUCCH SCell activation procedures, e.g. Tactivation_time on PUCCH SCell, T1, T2, T3 (if the case is invalid TA) and CSI reporting, can be performed in FR2. Thus, it seems testable. For unknown PUCCH SCell, UE is required to transmit L1-RSRP report via SpCell. Thus, UE may need to report L1-RSRP measurement report via FR1 SpCell after receiving activation command. Because the report on FR1 SpCell is unstable, the time between activation and L1-RSRP measurement report may be uncertain. Therefore, to our understanding, unknown PUCCH SCell is untestable. |
| CATT | For FR2 intra-band, we are not sure if there is explicit agreement in RAN1 that the SCell shall be in same PUCCH group as Pcell/PSCell. |

Issue 2-1-6: Testability for FR1 +FR2 combination

- Proposal 1: (Huawei)

  - For NR SA (or EN-DC) case, when the to-be-activated PUCCH Scell is in FR2 and Pcell (or PSCell) is in FR1, only define test cases for known case with valid TA.

  - Another FR2 serving cell should be configured in the test, and TE shall guarantee that the FR2 serving cell is successfully activated before activation the PUCCH Scell.

- Recommended WF

  - Need more discussion

| Issue 2-1-6: Testability for FR1 +FR2 combination |  |
| --- | --- |
| Company | Comments |
| Qualcomm | We do not support Proposal 1. RAN4 should define test cases as if there is no testability issue, and wait until the issue is resolved and tested. |
| Apple | We have different view from proposal 1, as we don’t think it’s essential to test FR1+FR2 CA case. Even though the proposal 1 could somehow avoid the FR1+FR2 testability issue (e.g., MAC CE command transmitted by FR2 active Scell instead of FR1 Pcell/PSCell), the similar situation could also be verified by FR2+FR2 CA and therefore it’s not necessary to test FR1+FR2 case. On the other hand, the interruption on FR1 Pcell/PSCell cannot be verified accurately in this case either (FR1 connection is not guaranteed due to FR1+FR2 testability issue). |
| Nokia | We are open to discuss this testability issue. If another FR2 serving cell is needed, could the cross PUCCH Scell group CSI reporting be transmitted on this FR2 cell then the unknown case can also be verified? |
| Huawei | We support option 1. We don’t think all FR1+FR2 test cases should be skipped even it can be performed without the so-called testability issue. Based on the conclusion in R4-2115240 which is shown as follow, the test cases should be defined with necessary modification. - Some of the LTE/FR1+FR2 tests can be performed with some modification: - Modify each individual test e.g. remove testing of LTE/FR1 cell in the test And A.5.5.3.1 SCell Activation and deactivation of SCell in FR2 intra-band is same as the case here (LTE PCell and FR2 SCell). The modification is to remove the interruption verification on PCell and clarify that RRC messages are received in SCG via signaling on SRB3. So think same principle shall be followed. |
| MediaTek | Same comment as Issue 2-1-5 |
| Intel | Fine to discuss the testability issue first. |

Issue 2-1-7: Whether the following test cases are needed for PUCCH Scell activation/deactivation with multiple DL Scells in FR1?

Moderator: please add your preference (Yes or no) in the table directly and provide your technical reasons in the comment table.

| Company | FR1 known PUCCH SCell + FR1 known SCell | FR1 known PUCCH SCell + FR1 unknown SCell | FR1 unknown PUCCH SCell + FR1 known SCell | FR1 unknown PUCCH SCell + FR1 unknown SCell |
| --- | --- | --- | --- | --- |
| Qualcomm | Yes and No | Yes | No | No |
| Apple | No | Yes | No | Yes |
| Nokia | Yes | Yes | Yes | Yes |
| Huawei | Yes | Yes | NO | Yes |
| MediaTek | No | yes | Open to discuss | No |
| Ericsson | No | Yes | No | Yes |
| CATT | Yes | Yes | Yes | Yes |
| OPPO | No | Yes | No | Yes |

- Recommended WF

  - Need more discussion

Technical comments on the test cases for PUCCH SCell activation/deactivation with multiple DL Scells in FR1:

| Issue 2-1-7: Whether the following test cases are needed for PUCCH Scell activation/deactivation with multiple DL Scells in FR1? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | FR1 known PUCCH Scell + FR1 known Scell: Yes - If the case below “FR1 known PUCCH Scell + FR1 unknown Scell” is defined, we may not need to define this test case. FR1 known PUCCH Scell + FR1 unknown Scell: Yes - We further propose to consider FR1 unknown Scell to be activated in parallel with known PUCCH Scell is in the same band as the PUCCH Scell. Besides, the measurement period of the Scells being activated is equal to or smaller than 2400ms, and both ‘with’ and ‘without’ a valid TA can be tested. For ‘without a valid TA’, the PUCCH Scell shall belong to a secondary TAG. FR1 unknown PUCCH Scell + FR1 known Scell: No - It is unlikely that PUCCH Scell is unknown while to-be-activated DL Scells in the same PUCCH group are known. FR1 unknown PUCCH Scell + FR1 unknown Scell: No This case already requires a long latency. We do not see the need of defining test cases for this. |
| Apple | To control the TC amount, the DL Scell could be also tested for unknown case for the worst scenario. DL Scell known case has already been verified by legacy multiple Scell activation TCs, and here the key point is to verify the PUCCH Scell activation delay when multiple DL Scells are also being activated at the same time. If PUCCH SCell is unknown, we don’t think the Scells in the same PUCCH group could be known in a practical scenario. |
| Nokia | This also depends on the conclusion on Sub-topic 1-2. If the separate delay requirements are defined for Scells belonging to same or different PUCCH group, we may need also consider the condition which PUCCH group the SCell belongs to? |
| MediaTek | To our understanding, the main motivation is to test the PUCCH SCell and we prefer to test two cases among these four cases to avoid too many test case. Thus, we prefer to test case 2 and 3. The reason is known and unknown PUCCH SCell should be tested. And the question is how to determine the known condition of the another SCell. However, as other companies’ comment, case 1 can be covered by the existing requirement. Therefore, case 2 and 3 are suggested. Besides, we are ok not to test case 3 if other companies think this is not typical case. |
| Ericsson | Same view as Apple. Unless we are testing something unique, we should try to limit the number of TC. |
| CATT | We think all the cases can be considered, but to control the number of test case, we can split the cases into FR1 and FR2, e.g. test FR1 known PUCCH SCell + FR1 known SCell and FR1 unknown PUCCH SCell + FR1 known SCell, while test FR2 known PUCCH SCell + FR2 unknown SCell + FR2 known SCell and FR2 unknown PUCCH SCell + FR2 unknown SCell in FR2. |

Issue 2-1-8: Whether the following test cases are needed for PUCCH Scell activation/deactivation with multiple DL Scells in FR2?

Moderator: please add your preference (Yes or no) in the table directly and provide your technical reasons in the comment table.

| Company | FR2 known PUCCH SCell + FR2 known SCell | FR2 known PUCCH SCell + FR2 unknown SCell | FR2 unknown PUCCH SCell + FR2 known SCell | FR2 unknown PUCCH SCell + FR2 unknown SCell |
| --- | --- | --- | --- | --- |
| Qualcomm | No | Yes | No | Yes |
| Apple | No | Yes | No | Yes |
| Nokia | Yes | Yes | Yes | This can be further discussed. |
| Huawei | Yes | Yes | No | Yes |
| MediaTek | No | yes | Open to discuss | No |
| Ericsson | No | Yes | No | Yes |
| CATT | Yes | Yes | Yes | Yes |
| OPPO | No | Yes | No | Yes |

- Recommended WF

  - Need more discussion

Technical comments on the test cases for PUCCH SCell activation/deactivation with multiple DL Scells in FR2:

| Issue 2-1-8: Whether the following test cases are needed for PUCCH SCell activation/deactivation with multiple DL Scells in FR2? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | FR2 known PUCCH SCell + FR2 known SCell: No - We can skip this case, and use the following case “FR2 known PUCCH SCell + FR2 unknown SCell.” The scenario below would effectively turn “FR2 unknown SCell in the same band as the PUCCH SCell” into “FR2 known SCell” in terms of the latency requirement. FR2 known PUCCH SCell + FR2 unknown SCell: Yes - FR2 known PUCCH SCell and one FR2 unknown SCell in the same band as the PUCCH SCell. And Pcell is in FR1 FR2 unknown PUCCH SCell + FR2 known SCell: No - It is unlikely that PUCCH SCell is unknown while to-be-activated DL Scells in the same PUCCH group are known. FR2 unknown PUCCH SCell + FR2 unknown SCell: Yes FR2 unknown PUCCH SCell and one FR2 unknown SCell in the same band as the PUCCH SCell. And Pcell is in FR1 |
| Apple | Same comment as for issue 2-1-7. |
| Nokia | For FR2 unknown Scells other than PUCCH SCell, we may assume the SCell has active serving cell(s) or known to-be-activated SCell(s) on the same band. |
| MediaTek | Suggest to follow the same principle in Issue 2-1-7. |
| Ericsson | Same comments as previous issue |

Issue 2-1-9: For the test cases in issue 2-1-8 if agreed, whether the PUCCH Scell and the other Scell should be at the same band?

- Proposal 1: (Qualcomm)

  - Yes

- Recommended WF

  - Need more discussion

| Issue 2-1-9: For the test cases in issue 2-1-8 if agreed, whether the PUCCH SCell and the other SCell should be at the same band? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support Proposal 1. Otherwise, the testability will be also subject to a separate/independent UE capability, IBM. |
| Apple | We agree with proposal 1. This proposal is also related with some previous issues, e.g., issue 1-2-1. We think the requirement applies when multiple DL SCell shall be in the PUCCH group as target PUCCH SCell. Based on current multiple SCell activation side condition that “any to-be-activated unknown SCell has active serving cell(s) or known to-be-activated SCell(s) on the same band”, it’s simple to have FR2 DL SCell on the same band as target FR2 PUCCH SCell. Otherwise, the DL being-activated SCell in different band from target PUCCH SCell shall be in the same band as Pcell/PSCell because in R17 FR2 inter-band CA only have up to 2 bands, which means the DL being-activated SCell cannot be in the same PUCCH group as target PUCCH SCell; and that’s conflicted with our understand in the issue 1-2-1. |
| Nokia | We understood they don’t have to be on the same band. But we can assume for unknown SCell on a band different from PUCCH SCell, there is active serving cell(s) or known to-be-activated SCell(s) on that band. |
| Huawei | We assume that it means issue 2-1-8 (PUCCH Scell and other SCell in FR2). Then we are fine with proposal 1. |
| MediaTek | This issue seems depends on the discussion of proposal 1 in Issue 1-2-1. |
| CATT | Fine with proposal 1. |
| OPPO | Proposal 1 is fine. It’s simple to consider FR2 DL SCell on the same band as target FR2 PUCCH SCell. |

Sub-topic 2-2 Test case list

Moderator: Moderator would suggest focusing on the general discussion in the first round and moderator will try to update the test case list at the start of 2nd round based on the discussion.

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2207775 (Apple) | Company A |
|  | Company B |
| R4-2208089 (Nokia) | Company A |
|  | Company B |
| R4-2208107 (Xiaomi) |  |
| R4-2208179 (CATT) |  |
| R4-2208353 (OPPO) |  |
| R4-2208463 (MTK) |  |
| R4-2208945 (Huawei) |  |
| R4-2210137 (Ericsson) |  |

Summary for 1st round

Open issues

Sub-topic 2-1 Test configuration

|  | Status summary |
| --- | --- |
| Issue 2-1-1 | Status: all companies support proposal 1. Tentative agreements: - PL-RS, TCI state, and spatial relation if applicable shall be associated with one SSB ID. And an explicit PL-RS indication can be skipped in the test cases. Candidate options: None. Recommendations for 2nd round: No more discussion. |
| Issue 2-1-2 | Status: all companies support proposal 1 and 2 companies support proposal 1a. Tentative agreements: - Define test cases for both known case and unknown case. Candidate options: - Proposal 1a: - For unknown PUCCH SCell activation in FR2, the test cases can be specified based on UE having capability of cross PUCCH group CSI reporting and SS capable of controlling UE cross cells. Recommendations for 2nd round: Further discuss proposal 1a. |
| Issue 2-1-3 | Status: all companies support option 2/2a/2b, but only one company (QC) raised concern on option 2b that it should be decided after option 2 is agreed. Moderator would suggest QC to check in the 2nd round. Tentative agreements: - Define two sub tests in each test case for valid TA and invalid TA scenarios, where PUCCH Scell is configured with sTAG. - TimeAlignmentTimer should expires before UE receives the activation command for invalid TA case - UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment for sTAG for valid TA case. Candidate options: None. Recommendations for 2nd round: Please QC check if the tentative is agreeable. |
| Issue 2-1-4 | Status: no objection to define the test case for interruption requirements but all companies think the interruption can be verified together in the delay test cases. Tentative agreements: - The interruption requirements due to PUCCH Scell activation/deactivation can be verified in the test cases for PUCCH Scell activation/deactivation delay requirements. Candidate options: None. Recommendations for 2nd round: Check the tentative agreements. |
| Issue 2-1-5 | Status: all companies support FR1 Pcell/PSCell + FR1 PUCCH SCell and FR2 Pcell/PSCell + FR2 PUCCH SCell(inter-band); Majority support FR1 Pcell/PSCell + FR2 PUCCH SCell; Majority don’t support or make it low priority for FR2 Pcell/PSCell + FR1 PUCCH SCell and FR2 Pcell/PSCell + FR2 PUCCH SCell(intra-band). Tentative agreements: Define test cases for: - FR1 Pcell/PSCell + FR1 PUCCH SCell - FR2 Pcell/PSCell + FR2 PUCCH SCell(inter-band) - FR1 Pcell/PSCell + FR2 PUCCH SCell Candidate options: None. Recommendations for 2nd round: Check the tentative agreements. |
| Issue 2-1-6 | Status: no consensus. 1 company supports and 2 companies don’t support. Other companies need further study. Tentative agreements: None. Candidate options: - Proposal 1: - For NR SA (or EN-DC) case, when the to-be-activated PUCCH Scell is in FR2 and Pcell (or PSCell) is in FR1, only define test cases for known case with valid TA. - Another FR2 serving cell should be configured in the test, and TE shall guarantee that the FR2 serving cell is successfully activated before activation the PUCCH Scell. Recommendations for 2nd round: Further discuss. |
| Issue 2-1-7 | Status: majority support FR1 known PUCCH SCell + FR1 unknown SCell and FR1 unknown PUCCH SCell + FR1 unknown SCell Majority don’t support FR1 unknown PUCCH SCell + FR1 known SCell; Almost equal support for FR1 known PUCCH SCell + FR1 known SCell. Tentative agreements: Define test case for: - FR1 known PUCCH SCell + FR1 unknown SCell - FR1 unknown PUCCH SCell + FR1 unknown SCell FFS: whether to define test case for FR1 known PUCCH SCell + FR1 known SCell Candidate options: Whether to define test case for FR1 known PUCCH SCell + FR1 known SCell? - Option 1: - Yes - Option 2: - No Recommendations for 2nd round: check the tentative agreements and further discuss the candidate options. |
| Issue 2-1-8 | Status: same as that for FR1 (issue 2-1-7) Tentative agreements: Define test case for: - FR2 known PUCCH SCell + FR2 unknown SCell - FR2 unknown PUCCH SCell + FR2 unknown SCell FFS: whether to define test case for FR2 known PUCCH SCell + FR2 known SCell Candidate options: Whether to define test case for FR2 known PUCCH SCell + FR2 known SCell? - Option 1: - Yes - Option 2: - No Recommendations for 2nd round: check the tentative agreements and further discuss the candidate options. |
| Issue 2-1-9 | Status: 5 companies support proposal 1, 1 company doesn’t support and 1 company thinks it depends on other issues. Tentative agreements: None. Candidate options: Issue 2-1-9: For the test cases in issue 2-1-8 if agreed, whether the PUCCH SCell and the other SCell should be at the same band? - Proposal 1: - Yes Recommendations for 2nd round: Further discuss. |

Sub-topic 2-2 Test case list

The test case list is summarized as below based on 1st round discussion and can be further discussed and assigned in 2nd round.

| Test No. | Test | DC/CA mode | Section | Company |
| --- | --- | --- | --- | --- |
| TC for SA |  |  |  |  |
| TC 1-1 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 known cell (All NR cells in FR1) | NR SA with all cells in FR1 | 6.5.3.x1 |  |
| TC 1-2 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown cell (All NR cells in FR1) | NR SA with all cells in FR1 | 6.5.3.x2 |  |
| TC 1-3 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with FR1 PCell | For FR2 | 7.5.3.x1 |  |
| TC 1-4 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell |  |  |  |
| TC 1-5 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with inter-band FR2 PCell |  | N/A |  |
| TC 1-6 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with inter-band FR2 PCell |  | 7.5.3.x3 |  |
| TC 1-7 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 known PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) | NR SA with all cells in FR1 | 6.5.3.x4 |  |
| TC 1-8 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) | NR SA with all cells in FR1 | 6.5.3.x4 |  |
| TC 1-9 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell | NR SA with one or more cells in FR2 | 7.5.3.x5 |  |
| TC 1-10 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown PUCCH SCell and one FR2 unknown SCell with FR2 PCell | NR SA with one or more cells in FR2 | 7.5.3.x5 |  |
| TC for EN-DC |  |  |  |  |
| TC 2-1 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 known cell (All NR cells in FR1) | EN-DC with all NR cells in FR1 |  |  |
| TC 2-2 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown cell (All NR cells in FR1) |  |  |  |
| TC 2-3 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with FR1 PSCell | For FR2 |  |  |
| TC 2-4 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PSCell |  |  |  |
| TC 2-5 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with inter-band FR2 PSCell |  |  |  |
| TC 2-6 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with inter-band FR2 PSCell |  |  |  |
| TC 2-7 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 known PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) |  |  |  |
| TC 2-8 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) |  |  |  |
| TC 2-9 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PSCell |  |  |  |
| TC 2-10 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown PUCCH SCell and one FR2 unknown SCell with FR2 PSCell |  |  |  |
| Note1: Both valid TA and invalid TA cases are included in each test case if applicable. Note2: TC 1-7~TC 1-10 and TC 2-7~TC 2-10 are for the PUCCH SCell activation/deactivation delay requirements with multiple DL Scells. |  |  |  |  |

CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |
|  | Company A |
|  | Company B |

Discussion on 2nd round (if applicable)

Sub-topic 2-1 Test configuration

Issue 2-1-2: Whether to define test cases for both known and unknown case

Candidate options:

- Proposal 1a:

  - For unknown PUCCH SCell activation in FR2, the test cases can be specified based on UE having capability of cross PUCCH group CSI reporting and SS capable of controlling UE cross cells.

Recommendations for 2nd round: Further discuss proposal 1a.

| Issue 2-1-2: Whether to define test cases for both known and unknown case |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support Proposal 1a, and the test cases for known need to be also defined. One clarification question about ‘SS capable of controlling UE cross cells’ Does SS stand for ‘System Simulator’? If so, why does it need to be considered here? |
| Ericsson | Ok with proposal 1a. |
| Huawei | Fine with proposal 1a, and agree with QC’s view about “SS” |
| Apple | Option 1a. |
| CATT | Support proposal 1a. For SS, our thinking is that for unknown case, UE need to report beam information through Pcell but send PDCCH order through PUCCH SCell based on reported beam information which means Pcell and PUCCH SCell need to interact to each other on this information. But it is not required in existing SCell activation test case generally. So we clarified that SS need to have this capability to interact the beam information between cells. |
| Nokia | Fine with Proposal 1a. |
| MediaTek | Support option 1a |

Issue 2-1-3: Whether to define test cases for both valid TA and invalid TA case

Tentative agreements:

- Define two sub tests in each test case for valid TA and invalid TA scenarios, where PUCCH Scell is configured with sTAG.

- TimeAlignmentTimer should expires before UE receives the activation command for invalid TA case

- UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment for sTAG for valid TA case.

Recommendations for 2nd round: Please QC check if the tentative is agreeable.

| Issue 2-1-3: Whether to define test cases for both valid TA and invalid TA case |  |
| --- | --- |
| Company | Comments |
| Qualcomm | We are not against the tentative agreements. But we don’t think PUCCH SCell shall belong to sTAG even for valid TA scenario because this may require the UE to support dual TAG on top of dual PUCCH groups. If there is no compelling reason to tie the two different UE capabilities together, we prefer to remove sTAG condition from the valid TA scenario. |
| Huawei | Agree with QC’s observation. For valid TA case, single TAG is enough. |
| Apple | Fine with tentative agreement. And fine with QC’s view for valid TA case. |
| CATT | Fine with the tentative agreement. And to address QC’s concern, we can modify the first bullet as: - Define two sub tests in each test case for valid TA and invalid TA scenarios. And for invalid TA scenario, PUCCH Scell is configured with sTAG. |
| Nokia | Fine with the tentative agreements. And QC’s proposal is also agreeable. |
| MediaTek | Fine with the tentative agreement and modified proposal |

Issue 2-1-4: Test cases for interruption requirements due to PUCCH SCell activation/deactivation

Tentative agreements:

- The interruption requirements due to PUCCH Scell activation/deactivation can be verified in the test cases for PUCCH Scell activation/deactivation delay requirements.

Recommendations for 2nd round: Check the tentative agreements.

| Issue 2-1-4: Test cases for interruption requirements due to PUCCH Scell activation/deactivation |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Okay with the tentative agreements. |
| Ericsson | Ok with tentative agreement |
| Apple | Fine with tentative agreement. |
| CATT | Support the tentative agreement. |
| Intel | Support with tentative agreement. |
| Nokia | Fine with tentative agreement. |
| MediaTek | Support the tentative agreement |

Issue 2-1-5: Whether the test cases for the following band combinations are needed?

Tentative agreements:

Define test cases for:

- FR1 Pcell/PSCell + FR1 PUCCH Scell

- FR2 PCell/PSCell + FR2 PUCCH SCell(inter-band)

- FR1 PCell/PSCell + FR2 PUCCH SCell

Recommendations for 2nd round: Check the tentative agreements.

| Issue 2-1-5: Whether the test cases for the following band combinations are needed? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Okay with the tentative agreements. |
| Ericsson | Okay with the tentative agreements. |
| Huawei | Fine with the tentative agreements |
| Apple | Fine with FR1 only or FR2 only case; but not fully convinced on FR1+FR2 TC. We don’t know how much FR1+FR2 testability issue would impact the CSI reporting on FR1, and if the beam information cannot be successfully reported to PCell, it would impact the whole TC. |
| CATT | Support the tentative agreement. For Apple’s concern, maybe we can discuss in the CR drafting. If technical issues are identified in the test configuration or procedure, we can further revisit it. |
| Nokia | We’d like to clarify, does the first bullet include the TCs in intra-band and inter-band? We are fine to drop the FR2 intra-band case, but FR1 intra-band seems to be valid scenario. - FR1 Pcell/PSCell + FR1 PUCCH Scell |
| MediaTek | Have the same concern as Apple. But we are ok to discuss in the CR drafting stage. |

Issue 2-1-6: Testability for FR1 +FR2 combination

Candidate options:

- Proposal 1:

  - For NR SA (or EN-DC) case, when the to-be-activated PUCCH Scell is in FR2 and Pcell (or PSCell) is in FR1, only define test cases for known case with valid TA.

  - Another FR2 serving cell should be configured in the test, and TE shall guarantee that the FR2 serving cell is successfully activated before activation the PUCCH Scell.

Recommendations for 2nd round: Further discuss.

| Issue 2-1-6: Testability for FR1 +FR2 combination |  |
| --- | --- |
| Company | Comments |
| Qualcomm | If the serving cell in FR2 added for an interruption requirement test is inter-band with respect to the FR2 PUCCH SCell, the proposal is okay to us. Otherwise, we do not support it. |
| Huawei | To QC: The purpose of having a FR serving cell is not for interruption requirement but for reliable activation command receiving. We are fine to have the serving cell as inter-band wrt PUCCH SCell. |
| Apple | Not sure about the feasibility of proposal 1: we assume the ‘another FR2 serving cell’ is on the different band from FR2 PUCCH SCell because PUCCH SCell shall be the first one in the secondary PUCCH group. But we don’t have combination of FR1 band + FR2 band +FR2 band so far. |
| Nokia | We support defining the test case for FR1+FR2 scenario. Would be good if the proponent company can explain in details (e.g. CR) the purpose of each network node. This may help align the understanding and evaluation of the testability. |
| MediaTek | Suggest keep it FFS and come back in the next meeting. |

Issue 2-1-7: Whether the following test cases are needed for PUCCH SCell activation/deactivation with multiple DL SCells in FR1?

Tentative agreements:

Define test case for:

- FR1 known PUCCH SCell + FR1 unknown SCell

- FR1 unknown PUCCH SCell + FR1 unknown SCell

FFS: whether to define test case for FR1 known PUCCH SCell + FR1 known SCell

Candidate options:

Whether to define test case for FR1 known PUCCH SCell + FR1 known SCell?

- Option 1:

  - Yes

- Option 2:

  - No

Recommendations for 2nd round: check the tentative agreements and further discuss the candidate options.

| Issue 2-1-7: Whether the following test cases are needed for PUCCH SCell activation/deactivation with multiple DL SCells in FR1? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Although we do not prefer “FR1 unknown PUCCH SCell + FR1 unknown SCell” because of its long latency, we won’t be strongly against the case if the case is agreed to support in core requirement part. Support Option 2 assuming UE passes “FR1 known PUCCH SCell + FR1 unknown SCell” would not have an issue with “FR1 known PUCCH SCell + FR1 known SCell” |
| Huawei | Support option 2 based on the tentative agreements. |
| Apple | Support tentative agreement and option 2. |
| CATT | Support the tentative agreement and fine with option 2. |
| Nokia | We prefer Option 1 as this seems to be simplest scenario. There should not be big efforts to test known case if unknown case can be well verified? |
| MediaTek | Fine with option 2 and the tentative agreement. |

Issue 2-1-8: Whether the following test cases are needed for PUCCH SCell activation/deactivation with multiple DL Scells in FR2?

Tentative agreements:

Define test case for:

- FR2 known PUCCH SCell + FR2 unknown SCell

- FR2 unknown PUCCH SCell + FR2 unknown SCell

FFS: whether to define test case for FR2 known PUCCH SCell + FR2 known SCell

Candidate options:

Whether to define test case for FR2 known PUCCH SCell + FR2 known SCell?

- Option 1:

  - Yes

- Option 2:

  - No

Recommendations for 2nd round: check the tentative agreements and further discuss the candidate options.

| Issue 2-1-8: Whether the following test cases are needed for PUCCH SCell activation/deactivation with multiple DL Scells in FR2? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | For “FR2 unknown PUCCH SCell + FR2 unknown SCell” we prefer one FR2 unknown SCell in the same band as the PUCCH SCell, andnd Pcell is in FR1 Support Option 2 assuming UE passes “FR2 known PUCCH SCell + FR2 unknown SCell” would not have an issue with “FR2 known PUCCH SCell + FR2 known SCell” |
| Huawei | Support option 2 based on the tentative agreements. |
| Apple | Support tentative agreement and option 2. |
| CATT | Support the tentative agreement and fine with option 2. |
| Nokia | Same comments as in Issue 2-1-7. |
| MediaTek | Fine with option 2 and the tentative agreement. |

Issue 2-1-9: For the test cases in issue 2-1-8 if agreed, whether the PUCCH SCell and the other SCell should be at the same band?

Candidate options:

- Option 1:

  - Yes

- Option 2:

  - No

Recommendations for 2nd round: Further discuss.

| Issue 2-1-9: For the test cases in issue 2-1-8 if agreed, whether the PUCCH SCell and the other SCell should be at the same band? |  |
| --- | --- |
| Company | Comments |
| Qualcomm | Support Option 1. This is to decouple two different UE capabilities for the test. |
| Apple | Support option 1. |
| Nokia | Fine with Option 1. |

Sub-topic 2-2 Test case list

| Test No. | Test | DC/CA mode | Section | Company |
| --- | --- | --- | --- | --- |
| TC for SA |  |  |  |  |
| TC 1-1 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 known cell (All NR cells in FR1) | NR SA with all cells in FR1 | 6.5.3.x1 | Huawei |
| TC 1-2 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown cell (All NR cells in FR1) | NR SA with all cells in FR1 | 6.5.3.x2 | MTK |
| TC 1-3 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with FR1 PCell | NR SA with one or more cells in FR2 | 7.5.3.x1 | Ericsson |
| TC 1-4 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell | NR SA with one or more cells in FR2 | 7.5.3.x2 | Qualcomm |
| TC 1-5 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with inter-band FR2 PCell | NR SA with one or more cells in FR2 | 7.5.3.x3 | Nokia |
| TC 1-6 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with inter-band FR2 PCell | NR SA with one or more cells in FR2 | 7.5.3.x4 | CATT |
| TC 1-7 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 known PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) | NR SA with all cells in FR1 | 6.5.3.x3 | Xiaomi |
| TC 1-8 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) | NR SA with all cells in FR1 | 6.5.3.x4 | Apple |
| TC 1-9 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell | NR SA with one or more cells in FR2 | 7.5.3.x5 |  |
| TC 1-10 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown PUCCH SCell and one FR2 unknown SCell with FR2 PCell | NR SA with one or more cells in FR2 | 7.5.3.x6 |  |
| TC for EN-DC |  |  |  |  |
| TC 2-1 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 known cell (All NR cells in FR1) | EN-DC with all NR cells in FR1 | 4.5.3.x1 | OPPO |
| TC 2-2 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown cell (All NR cells in FR1) | EN-DC with all NR cells in FR1 | 4.5.3.x2 | CATT |
| TC 2-3 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with FR1 PSCell | EN-DC with one or more NR cells in FR2 | 5.5.3.x1 |  |
| TC 2-4 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PSCell | EN-DC with one or more NR cells in FR2 | 5.5.3.x2 |  |
| TC 2-5 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with inter-band FR2 PSCell | EN-DC with one or more NR cells in FR2 | 5.5.3.x3 |  |
| TC 2-6 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with inter-band FR2 PSCell | EN-DC with one or more NR cells in FR2 | 5.5.3.x4 |  |
| TC 2-7 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 known PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) | EN-DC with all NR cells in FR1 | 4.5.3.x3 | vivo |
| TC 2-8 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) | EN-DC with all NR cells in FR1 | 4.5.3.x4 |  |
| TC 2-9 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PSCell | EN-DC with one or more NR cells in FR2 | 5.5.3.x5 |  |
| TC 2-10 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown PUCCH SCell and one FR2 unknown SCell with FR2 PSCell | EN-DC with one or more NR cells in FR2 | 5.5.3.x6 |  |
| Note1: Both valid TA and invalid TA cases are included in each test case if applicable. Note2: TC 1-7~TC 1-10 and TC 2-7~TC 2-10 are for the PUCCH SCell activation/deactivation delay requirements with multiple DL SCells. |  |  |  |  |

Recommendations for 2nd round: Discuss the test case list and volunteer on the test case (the volunteers before meeting are included in the table).

| Sub-topic 2-2 Test case list |  |
| --- | --- |
| Company | Comments |
| Qualcomm | We can volunteer for TC 1-4. |
| Ericsson | If needed, we can volunteer for one more test case. Moderator can assign it to us based on the need. |
| Huawei | We can volunteer for TC 2-5 if needed. |
| Nokia | We can take additional TC2-6 if needed. |

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on … | YYY |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
|  | WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements | CATT |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2207774 |  | CR on PUCCH SCell activation in TS38.133 R17 | Apple | Merged | Merged R4-2208088 |
| R4-2208088 |  | 38.133 draftCR on PUCCH SCell activation delay | Nokia | Revised | Capture the agreement for PUCCH Scell activation delay |
| R4-2208173 |  | CR on completing PUCCH SCell activation requirement | CATT | Merged | Merged with other CRs |
| R4-2208464 |  | CR on PUCCH SCell activation and deactivation delay requirements | MTK | Revised | Capture the SSB-less case. Other changes merged with R4-2208088 |
| R4-2208939 |  | CR on PUCCH SCell activation | Huawei | Revised | Capture the agreements for interruption. Other changes merged with R4-2208088 |
| R4-2210135 |  | CR on SCell activation/deactivation with PUCCH | Ericsson | Revised | Capture the agreement for PUCCH Scell with multiple DL Scells. Other changes merged with R4-2208088 |
| R4-2207775 |  | TC for PUCCH Scell activation and deactivation delay requirements of FR1 unknown PUCCH Scell and one FR1 unknown Scell (All NR cells in FR1) R17 | Apple | Postponed | Focus on the test case list and test configuration, and treat the CR in next meeting |
| R4-2208089 |  | draftCR on TC3 PUCCH Scell activation of FR2 known cell in SA | Nokia | Postponed |  |
| R4-2208107 |  | CR on TC for PUCCH Scell activation and deactivation delay requirements | Xiaomi | Postponed |  |
| R4-2208179 |  | Test case for PUCCH Scell activation and deactivation delay requirements | CATT | Postponed |  |
| R4-2208353 |  | draft CR for PUCCH Scell activation and deactivation delay requirements of FR1 known cell for EN-DC (TC10) | OPPO | Postponed |  |
| R4-2208463 |  | Draft CR on TC for PUCCH Scell activation and deactivation delay requirements of FR1 unknown cell | MTK | Postponed |  |
| R4-2208945 |  | Draft CR on TC for PUCCH Scell activation and deactivation | Huawei | Postponed |  |
| R4-2210137 |  | TC for PUCCH Scell activation and deactivation delay requirements of FR2 known cell with FR1 Pcell | Ericsson | Postponed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |
| R4-2210584 | R4-2211208 | WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements | CATT | Agreeable |  |
| R4-2211013 | R4-2211209 | 38.133 draftCR on PUCCH SCell activation delay | Nokia | Agreeable |  |
| R4-2211014 | N/A | CR on PUCCH SCell activation and deactivation delay requirements | MTK | Agreeable |  |
| R4-2211015 | N/A | CR on PUCCH SCell activation | Huawei | Agreeable |  |
| R4-2211016 | N/A | CR on SCell activation/deactivation with PUCCH | Ericsson | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| CATT | Qiuge Guo | guoqiuge@catt.cn |
| Qualcomm | CH Park | chparkqc@qti.qualcomm.com |
| Apple | Jie Cui | Jie_cui@apple.com |
| Nokia | Lei Du | lei.du@nokia-sbell.com |
| Huawei | Zhongyi Shen | shenzhongyi3@huawei.com |
| MediaTek | ChihKai Yang | ck.yang@mediatek.com |
| Ericsson | Venkat | Venkatarao.gonuguntla@ericsson.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)