3GPP TSG-RAN WG4 Meeting # 103-e												R4-2210481

Electronic Meeting, 09 – 20 May 2022

Agenda item:			9.15.8.1, 9.15.8.2, 9.15.9

Source:	Moderator (Qualcomm)

Title:	Email discussion summary for [103-e][212] NR_ext_to_71GHz_RRM_1

Document for:	Information

# Introduction

In RAN #89e meeting a new WI on Extending current NR operation to 71GHz was approved. During the RAN4 #99 the initial scope of RRM work for NR_ext_to_71GHz WI was defined and captured in the way forward R4-2108354. Further discussion was split into two email threads. The discussion during RAN4 #100-e, RAN4 #101-e, RAN4 #101bis-e and RAN4 #102-e in the first email thread can be found in – R4-2115405, R4-2120370, R4-2202733, R4-2207065 with corresponding WFs in – R4-2115351, R4-2120316, R4-2202657, R4-2206919 and for the second email thread in – R4-2115406, R4-2120371, R4-2202734, R4-2207066 with corresponding WFs in – R4-2115352, R4-2120317, R4-2202659, R4-2206924 respectively.

Current email discussion document focuses on the general requirements, timing requirements, scheduling restrictions and measurement procedures based on the documents submitted under AIs - 9.15.8. and 9.15.8.2.

This document also discusses the initial scope for performance requirements based on the documents submitted under AI – 9.15.9.

Draft CRs are accepted in this meeting

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: The following list of open issues was identified, based on the contributions, for the 1st round

  - General

    - RX beam sweeping scaling factor

    - deriveSSB-IndexFromCell

    - Measurement procedures

  - Timing requirements

    - UE transmit timing

    - MRTD

    - MTTD

  - Performance requirements

    - General

    - Test-cases

    - Work-plan

- 2nd round: Based on the discussion during the 1st round, the following list of topics are to be discussed during the 2nd round

  - General

    - RX beam sweeping scaling factor

    - deriveSSB-IndexFromCell

    - Measurement procedures

  - Timing requirements

    - UE transmit timing

    - MRTD

  - Performance requirements

    - General

    - Test-cases

    - Work-plan

# Topic #1: General

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4- 2208144 | CATT | Observations: For FR2-2, it is necessary to judge whether the current number of Rx beams can meet the basic coverage requirements. Proposal 1: If necessary, it is suggested to specify new RX beam sweeping scaling factor for FR2-2. Proposal 2: It is suggested to use a RX beam sweeping scaling factor scaling factor of 12 for FR2-2. Proposal 3: If the new RX beam sweeping scaling factor is agreed, it is necessary to additionally specify whether some FR2-2 UEs could use the existing scaling factor for measurement. Proposal 4: For some scenarios with strict measurement delay requirements (UE moves fast) and loose coverage requirements, the existing scaling factor in FR2-1 could also be used for measurement in FR2-2. |
| R4-2208308 | LGE | Proposal 1: Based on the number of antenna elements for FR2-2 power classes, up to 16 scaling factors for Rx beam sweeping needs to be considered for serving cell realated measurements. And introduce both 8 and 16 scaling factor for the measurement requirements depending on UE implementation. Proposal 2: maxNumberRxBeam should be extended from 8 to 16 for CSI-RS based L1-RSRP and L1-SINR measurements and send LS to RAN1 and RAN2. |
| R4-2208732 | ZTE | Proposal 1: Reuse the existing FR2-1 scaling factor for Rx beam sweeping for FR2-2. |
| R4-2208809 | Vivo | Proposal 1: SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended by 1 sample compared with FR2-1 if using the channel model from RAN1, i.e., 6 samples is needed for SSB index acquisition delay for inter-frequency measurement for FR2-2. Proposal 2: The requirements of SSB index detection for FR2-2 for inter-frequency measurement can be defined as below: Table 2: Time period for time index detection for inter-frequency measurements for FR2-2 Condition NOTE1,2; TSSB_time_index_inter. No DRX; Max(200ms, MSSB_index_inter Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(200ms, (1.5 MSSB_index_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; MSSB_index_inter DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group.. MSSB_index_inter = 6*N, N is Rx beam sweeping factor for FR2-2. Proposal 3: 5 samples is needed for SSB index acquisition delay for intra-frequency measurement for FR2-2 when deriveSSB-IndexFromCell is not enabled. Proposal 4: When deriveSSB-IndexFromCell is not enabled, the requirements of SSB index detection for FR2-2 for intra-frequency measurement can be defined as below: Table 3: Time period for time index detection for intra-frequency measurements for FR2-2 DRX cycle; Without measurement gaps; With measurement gaps. No DRX; Max(200ms, ceil(MSSB_index_intra Kp) SMTC period) CSSFintra; Max(200ms, ceil(MSSB_index_intra Kp) Max(MGRP, SMTC period)) CSSFintra. DRX cycle≤ 320ms; Max(200ms, ceil(1.5 MSSB_index_intra s Kp) Max(SMTC period, DRX cycle)) CSSFintra; Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(MGRP, SMTC period, DRX cycle)) CSSFintra. DRX cycle>320ms; ceil(MSSB_index_intra Kp) DRX cycle CSSFintra; ceil(MSSB_index_intra Kp) DRX cycle CSSFintra. MSSB_index_inter = 5*N, N is Rx beam sweeping factor for FR2-2. |
| R4-2208946 | Huawei | Observation 1: The antenna elements assumptions for FR2-2 is higher than that for FR2-1. Observation 2: The trade-off between coverage and delay are already considered for legacy requirements for FR2-1. Observation 3: More antenna elements with narrower beam is used to overcome the overage issue. Delay caused by more beam sweeping could be investigated in beam management design in RAN1 instead of simply ignoring the implementation issue Proposal 1: For PC2 and PC3, increase the beam sweeping factor to 16 for FR2-2 for corresponding RRM requirements. And the beam sweeping factor shall be updated in corresponding requirements where beam sweeping is included. |
| R4-2209095 | Ericsson | Proposal 1: Reuse the existing FR2-1 scaling factor for Rx beam sweeping for FR2-2. Proposal 2: In RRM specifications, frame boundary alignment tolerance shall be treated as UE’s capability, no network signaling. Proposal 3: DeriveSSB_IndexFromCell is not always enabled in unlicensed band in FR2-2. Proposal 4: For 480kHz SCS: - Option 1a: 3 SSB symbols For 960kHz SCS: when deriveSSB-IndexFromCell is enabled - Option 2a: 3 SSB symbols For 960kHz SCS: when deriveSSB-IndexFromCell is disabled - Option 3a: 6 SSB symbols Proposal 5: Define frame boundary alignment tolerance of PDSCH when deriveSSB-IndexFromCell is disabled as below: SSB SCS (KHz); Data SCS (KHz); Frame boundary alignment tolerance of PDSCH symbols (deriveSSB-IndexFromCell disabled). 960; 120; 1 120KHz symbol. 960; 480; 3 480KHz symbols. 960; 960; 7 960KHz symbols. Proposal 6: MSSB_index_intra = 6*N and N reuses RX beam sweeping number for different power classes, e.g. power class 1, power class 2 and power class 3. Proposal 7: Add QCL-D type assumption for L3-RSSI measurements. |
| R4-2209384 | Nokia | Observation 1: RX beam sweeping impacts directly RLM in sync and out of sync requirements, where if N=12 and N=16 TEvaluate_out_SSB could reach up to 460 s and 614 s respectively. Observation 2: Rx beam sweeping scaling factor impacts the requirements listed on Table 2. Proposal 1: Do not change Rx beam sweeping scaling factor for FR2-2 in comparison to FR2-1. Proposal 2: If RAN4 agrees to define new Rx beam sweeping scaling factor for FR2-2, consider the requirements on the table below for defining CR work split. Requirements; Impact due to Rx Beam Sweeping. Cell reselection; Intra-frequency; Yes. Inter-frequency; Yes. Handover; Yes. RRC Connection Mobility Control; RRC re-establishment; Yes. RRC release with redirection; Yes. Signalling characteristics; SCell activation and deactivation delay; Yes. Interruption; No. PSCell addition and release delay; No. Active TCI state switching delay; No. Active BWP switching delay; No. PSCell change; No. Conditional PSCell change; Yes. Radio link monitoring; Yes. Link recovery procedures; Yes. Uplink spatial relation switch delay; Indirect impact from TL1_Measurement_Period in 9.5.4. Pathloss reference signal switch delay; No. Measurement requirements; UE measurement capability; No. Intra-frequency; Yes. Inter-frequency; Yes. L1-RSRP measurements for reporting; Yes. L1-SINR measurements for reporting; Yes. CSI-RS based measurement; Yes. Observation 3: From our simulation results 2 samples are enough to detect SSB index with TDL-A 30 and TDL-A 5 at -6dB for the following SCS: 120 kHz, 240 kHz, 480 kHz and 960 kHz. Proposal 3: RAN4 to reuse the SSB index identification time from FR2-1. |
| R4-2210223 | Qualcomm | Observation 1: Increase in the UE bandwidth for 480/960kHz SCS implies UE processing at a higher sampling frequency. Observation 2: Increase in the frame boundary alignment tolerance means that the UE must start the cell-search way earlier leading to processing a large number of samples at a much higher sampling frequency. Observation 3: The hardware and memory requirements for cell search procedure grows by a huge margin for a UE operating in the FR2-2 band. Proposal 1: RAN4 to relax the cell-detection delay requirements by a factor of 2 for 480kHz SCS and by a factor of 4 for 960kHz SCS to avoid increasing the UE HW and memory requirements. |
| R4-2207783 | Apple | Proposal 1: Rx beam sweeping scaling factor is set to 12. Proposal 2: To decide the scaling factor N1 for RRC_IDLE state, either option 1 or option 2 can be considered. If needed, UE capability can be used to signal different values for other power classes or for a certain power class. |
| R4-2208811 | Vivo | Draft CR - Introduction of scheduling restriction requirements in FR2-2 |
| R4-2209050 | Nokia | Draft CR to TS 38.133 Corrections on scheduling availability in FR2-2 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Rx beam sweeping scaling factor

Sub-topic description: Discussion whether a new Rx beam sweeping scaling factor is needed or not and related values.

Open issues and candidate options before e-meeting:

Issue 1-1-1: Rx beam sweeping scaling factor

- Proposal 1: Re-use the existing scaling factor from FR2-1 (ZTE, Nokia, Ericsson)

- Proposal 2: Define a new scaling factor:

  - Option 1: 12 (CATT, Apple)

  - Option 2: 16 (Huawei, LGE)

- Proposal 3: Define two scaling factors:

  - Option 1: 8 and 12 (CATT)

  - Option 2: 8 and 16 (LGE)

- Recommended WF

  - Discuss the proposals.

  - Note: Impact on RRM requirements would be further discussed in the second round if an agreement is reached on the scaling factor value.

| Company | Comments |
| --- | --- |
| Ericsson | Even antenna elements are doubled in RF discussion, but the purpose is to provide assumption to evaluate spherical coverage, there isn’t directly proportional to beam width which is also relevant to linkbudget request. Considering SSB index still is 64, it implies beam width from network perspective, we lean towards keep RX beam number in terms of less coverage scenario compared to FR2-1. |
| Nokia | We prefer Proposal 1. As discussed in our paper, the measurement times are already very long in FR2 with N=8. We have not seen any deep technical analysis justifying why narrower beams are needed. Additionally, there will be a very extensive specification impact if we change the Rx beam sweeping. It is not only core requirements that will need to be updated, but also the vast majority of FR2-1 test cases cannot be reused for FR2-2 when using 120 kHz SCS. Not clear how we can use the two scaling factors. Would we introduce a UE capability? For example, for the TCI state switch, the network would need to know when the UE can be scheduled, and this time depends on the Rx beam sweeping. If we have 2 scaling factors the network won’t know when the UE is expected to conclude the procedure and that it can be scheduled. |
| Moderator | - GTW agreement: Further discuss the two alternative values for scaling factor - Alternative 1: Define the new scaling factor of 12 - Alternative 2: The scaling factor is 8 - Alternative 3: If there is no agreement on alternative 1 or alternavie 2, then define the new UE capability and apply the scaling factor of 8 or [16] depending on UE capability Please provide further comments considering Alternative 1 and Alternative 2. |
| ZTE | We think in RRM we should care more about mobility management and delay. RF agreement means the UE is capable of forming narrower beams, but not necessarily have to use fine beams to do RRM measurements. We still prefer Alternative 2. On low mobility: in such case the low mobility criteria is met and the UE can save power by entering relaxation mode. |
| Huawei | We can compromise to alternative 1 though we prefer 16. Operating in higher frequency range, more elements are assumed to guarantee the coverage performs, which will result in narrower beam width. It is straightforward that UE needs to sweep more beams. Otherwise, there is no need to double the number of elements. And we don’t think SSB index of 64 is relevant to UE Rx beam sweeping. And the arguments for keeping the scaling factor as 8 are not convincing to us in some degree. We don’t think the discussion about the antenna array assumption is only related to RF requirements. It seems to imply that the antenna used for RRM measurement is independent to the antenna array framework of UE. The WF about this issue is captured showed as follows. It is the common understanding that the Rx beam sweeping depends on the RF conclusion on antenna size, and now RF session has concluded with doubled antenna elements. We don’t think it is reasonable to say that RRM requirements can be considered independently. Rx beam sweeping scaling factor - Rx beam scaling factor for FR2-2 need to continue waiting for more RF conclusion on antenna size assumption and power class.. If it means the rough beam and fine beam, then we should note legacy FR2-1, we already assumed rough beam for RRM measurement. Then how could UE overcome the path loss in high frequency if we assumed all the same as FR2-1. |
| Ericsson | Updates: In RF session, 2 x 4 was not agreed in the end (as reference architecture for deriving the output-power and spherical coverage requirement) and companies did not agree to assuming 2 panels for spherical coverage, ultimately the requirements did not reflect these cases. It’s worthy noting that the cell detection time period is increased too much, taking this issue, Issue Issue 1-3-1 and Issue 1-3-2, the delay makes any mobility useless. - For instance, when DRX=320ms, the detection delay is 16x6x4x320ms=122.88seconds for PC1 and 960KHz, compared to 8x5x320ms=12.8 seconds in FR2-1. If CSSF is added, the delay is dramatically increased. |
| CATT | We prefer alternative 2, but considering the delay we can compromise as alternative 1. We don’t think it should consider RRM measurement separately, indeed, the path loss and the coverage is very crucial for FR2-2 and should be considered. |
| LGE | We prefer to consider new scaling factor as 16, but 12 is fine for us as commented in GTW. In FR2-2, the number of antenna elements for PC1, PC2, and PC3 UEs has been doubled in order to operate in higher frequency, and UEs would operate larger number of narrow beams. So, new scaling factor for Rx beam sweeping in RRM requirements should be considered. If Alt 3 is considered, we prefer to keep 16. |
| MTK | We can support alternative 1, and it would be a good compromise. Larger number of RX beam is required for the more antenna elements and FR2-2 is expected for low mobility scenario. |
| Apple | We support Alt. 1. Note the value of 12 reflects the consideration of limiting measurement delay to be not too long, which is reasonable given the low mobility scenario for FR2-2. |
| Qualcomm | We support alt 1. Increasing the number of antenna elements imply that beams will be narrow, and UE needs to sweep more beams for measurements. Although we prefer a factor of 16, but given the impact on measurement delay, we are ready to support alt 1. |
| Nokia | As for alt 3, we think this is not really acceptable. Alt 4 means in practice that the requirement is N=16, and that will pose a great burden on measurement times. Out preference if Alt 2, since there was never simulations or analysis showing the real benefit on increasing the number of Rx beams. |
| vivo | We support Alternative 1. It is necessary to make a balance between measurement delay and coverage. We believe 12 is a reasonable value. |

Issue 1-1-2: maxNumberRxBeam

- Proposal 1 (LGE): maxNumberRxBeam should be extended from 8 to 16 for CSI-RS based L1-RSRP and L1-SINR measurements and send LS to RAN1 and RAN2.

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| Ericsson | Don’t support proposal1, it depends on Issue 1-1-1. |
| Nokia | We can wait for the agreement on 1-1-1 Rx Beam sweeping scaling factor before deciding on this issue. |
| ZTE | Depending on 1-1-1. |
| Huawei | Suggest to conclusion on issue 1-1-1 first. |
| LGE | Agree to wait for the conclusion of issue 1-1-1. And if new scaling factor is agreed in issue 1-1-1, the extended value for maxNumberRxBeam should be the same as the value of the new scaling factor |
| Apple | We are open to discussion. It is OK to wait for the conclusion of issue 1-1-1 first. |
| Qualcomm | Suggest waiting for conclusion on issue 1-1-1. |
| vivo | We can follow the same value with the scaling factor of issue 1-1-1 for the maxNumberRxbeam. |

### Sub-topic 1-2: deriveSSB-IndexFromCell

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2-1: deriveSSB-IndexFromCell

- Proposal 1 (Ericsson): In RRM specifications, frame boundary alignment tolerance shall be treated as UE’s capability, no network signalling.

- Proposal 2 (Ericsson): deriveSSB_IndexFromCell is not always enabled in unlicensed band in FR2-2.

- Proposal 3 (Ericsson): Frame boundary alignment tolerance – SSB

  - For 480kHz SCS:

    - Option 1a: 3 SSB symbols

  - For 960kHz SCS: when deriveSSB-IndexFromCell is enabled

    - Option 2a: 3 SSB symbols

  - For 960kHz SCS: when deriveSSB-IndexFromCell is disabled

    - Option 3a: 6 SSB symbols

- Proposal 4 (Ericsson): Define frame boundary alignment tolerance of PDSCH when deriveSSB-IndexFromCell is disabled as below:

| SSB SCS (KHz) | Data SCS (KHz) | Frame boundary alignment tolerance of PDSCH symbols (deriveSSB-IndexFromCell disabled) |
| --- | --- | --- |
| 960 | 120 | 1 120KHz symbol |
| 960 | 480 | 3 480KHz symbol |
| 960 | 960 | 7 960KHz symbol |

- Recommended WF

  - Discuss proposal 4.

  - Proposals 1, 2 and 3 have already been agreed during RAN4#102e. No further discussion is needed.

| Company | Comments |
| --- | --- |
| Ericsson | The table is derived by calculation on non-aliasing SSB indexes with known 3 least significant bits. |
| Huawei | From our understanding, issue in proposal 4 was also agreed in last meeting. Is the motivation to change 6 to 7 for 960 and 960 case? |
| Ericsson | Update: Answer Huawei’ question: yes. 6 is derived by hypothetical propagation delay difference, it shall not be the assumption of UE capability to identify correct SSB index, instead we suggest to use SSB symbol offset between maximal and minimal of 3 least significant bits of SSB index as reference to calculation. |
| CATT | We agree with proposal 4. Using SSB symbol offset between maximal and minimal of 3 least significant bits of SSB index as reference to calculation is more reasonable than deriving by hypothetical propagation delay difference. |
| Apple | We want to understand the reason for the revision from 6 960 symbols to 7 960 symbols. |
| Qualcomm | We prefer to keep the previous agreement. This directly translates from the requirements for 480kHz SCS. SSB SCS (KHz); Data SCS (KHz); Frame boundary alignment tolerance of PDSCH symbols (deriveSSB-IndexFromCell disabled). 960; 120; 1 120KHz symbol. 960; 480; 3 480KHz symbol. 960; 960; 6 960KHz symbol. |
| Huawei2 | @Ericsson Thanks for the explanation. We think changing 6 to 7 can also satisfy deriving the SSB index with DMRS sequence index and can have more flexibility to NW. Thus, we are fine with the changing. |
| vivo | For Proposal 4, for the combination of 960kHz SSB SCS and 960kHz Data SCS, the frame boundary alignment tolerance of PDSCH symbols is 6 considering the coverage is 1km and cell phase synchronization accuracy is 3us during RAN4#102e. In our understanding, according to the SSB design from RAN1, if the UE determine the SSB index by only decoding the PBCH DMRS, the timing difference between reference cell and target cell may be up to dozens of symbols as long as the SSB index number difference between reference cell and target cell is less than 7. In addition, if the value is changed from 6 to 7, the L3 measurement scheduling restriction requirements for FR2-2 also need further corrections, i.e., one more symbol need to be performed scheduling restriction before and after SSB transmission. |

### Sub-topic 1-3: Measurement procedures

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-3-1: Cell (PSS/SSS) detection

- Proposal 1 (Qualcomm): RAN4 to relax the cell-detection delay requirements by a factor of 2 for 480kHz SCS and by a factor of 4 for 960kHz SCS to avoid increasing the HW and memory requirements.

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Ericsson | We have not strong view, but not sure 3 additional SSB symbols need double of total detection time. Another view is if it is agreed to add scaling factor, we suggest to only apply this kind of relaxation on shorter DRX cycle cases and gradually abbreviate relaxation on longer DRX cycle cases. |
| Nokia | It is not clear the intention of the Proposal 1. Is it for the intra-frequency cell identification requirements? We understand that since deriveSSB-IndexFromCell is not assumed for 480 and 960 the Tidentify_intra should consider TSSB_time_index_intra, and this was already agreed in the last meeting. |
| Moderator | GTW agreement: RAN4 to relax the cell-detection delay requirements by a factor and further discuss the number of the factor to avoid increasing the HW and memory requirements - Option 1: a factor of 2 for 480kHz SCS and by a factor of 4 for 960kHz SCS - Option 2: a factor of 1.5 for 480KHz and 2 for 960KHz SCS, and also apply the different factors depending on DRX cycles Please provide further comments considering option 1 and option 2. |
| Ericsson | Update: Another question to Proponents of Propsoal1: the scaling factor is only for ‘TPSS/SSS_sync_intra’ and ‘TPSS/SSS_sync_inter’ in connected mode or other measurement delay, e.g. Tdetect,NR_Intra in idle mode also shall be covered? As updated comments on Issue 1-1-1, the detection delay is too long to perform mobility correctly taking this issue into account, especially with long DRX cycle. We suggest to restrict the scaling factor number and applicable scope of DRX. |
| MTK | In our understanding, it also includes Tdetect,NR_Intra and Tdetect,NR_Inter |
| Apple | We are OK with Option 1. |
| Qualcomm | Support option 1. We think this should apply to Tdetect,NR_Intra as well |
| Huawei | We can understanding the motivation of the proposal. Regarding proposal 2, we think if the intention is to let UE process in different window, then 1.5 seems doesn’t make much sense (should be at least 2?). But we should be careful about directly doubling the total time. From our understanding, with 480 and 960 KHz, when UE is not required to monitor the whole SMTC window, the number of samples that UE is required to process depends on the number of symbols and FFT size. For the time being, it seems the situation is not that sever. So we need more time to consider it as it is the first time for discussion. |
| Nokia | We prefer Option 2. I would like to point out that TPSS/SSS_sync_intra is already being proposed to be increase depending on the outcome of the Rx beam sweeping and the number of samples for PSS/SSS detection, see current requirements below: That means that if N is creased to 16, and the number of samples is increased by one, MPSS/SSS would be potentially increased from 40 to 96, and TPSS/SSS, which is already increasing this time 2.4 times. If we use Option 1, if we use the factor of 4 this will be potentially increased by 9.6 times for 960 kHz. |

Issue 1-3-2: Inter-frequency SSB index identification time

- Proposal 1 (Vivo): SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended by 1 sample compared with FR2-1 if using the channel model from RAN1, i.e., 6 samples is needed for SSB index acquisition delay for inter-frequency measurement for FR2-2.

  - MSSB_index_inter = 6 *N in Table 9.3.4-4

  - N is Rx beam sweeping factor for FR2- 2

- Proposal 2 (Nokia): RAN4 to reuse the SSB index identification time from FR2-1.

- Proposal 3 (Moderator): (Based on the GTW discussion) the SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended to

  - 4*NRxBeam samples for power class 2 and 3

  - 6*NRxBeam samples for power class 1

  - NRxBeam is the Rx beam sweeping factor under discussion in Issue 1-1-1

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Ericsson | As proposed in proposal 2, we generally lead to a shorter identification time. |
| Nokia | We prefer Proposal 2. From our simulation results the number of samples can be maintained. |
| Moderator | Check if proposal 3 can be agreed |
| Huawei | Fine with proposal 3 |
| CATT | Proposal 3 is ok for us. |
| LGE | Fine with proposal 3 |
| MTK | Fine with Proposal 3 |
| Apple | We are OK with proposal 3. |
| Qualcomm | Fine with proposal 3 |
| Nokia | Update after proposal 3 was introduced We think proposal 3 is only acceptable if NRxBeam is kept 8 in the Issue 1-1-1 |
| vivo | We are fine with Proposal 3. |

Issue 1-3-3: Intra-frequency SSB index identification time when deriveSSB-IndexFromCell is not enabled

- Proposal 1: When deriveSSB-IndexFromCell is not enabled, the requirements of SSB index detection for FR2-2 for intra-frequency measurement can be defined as below:

Time period for time index detection for intra-frequency measurements for FR2-2

| DRX cycle | Without measurement gaps | With measurement gaps |
| --- | --- | --- |
| No DRX | Max(200ms, ceil(MSSB_index_intra Kp) SMTC period) CSSFintra | Max(200ms, ceil(MSSB_index_intra Kp) Max(MGRP, SMTC period)) CSSFintra |
| DRX cycle≤ 320ms | Max(200ms, ceil(1.5 MSSB_index_intra s Kp) Max(SMTC period, DRX cycle)) CSSFintra | Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(MGRP, SMTC period, DRX cycle)) CSSFintra |
| DRX cycle>320ms | ceil(MSSB_index_intra Kp) DRX cycle CSSFintra | ceil(MSSB_index_intra Kp) DRX cycle CSSFintra |

  - Option 1: MSSB_index_intra = 5*N

  - Option 2: MSSB_index_intra = 6*N

  - Option 3 (new): MSSB_index_intra = 3*N

  - N is Rx beam sweeping factor for FR2-2.

- Recommended WF

  - Discuss the options.

| Company | Comments |
| --- | --- |
| Ericsson | We slightly support option 2. |
| Nokia | We are fine with Proposal 1 and created Option 3 based on the previous issue. As for MSSB_index_intra it depends on the outcome of issue 1-3-2. If we agree on Proposal 2 than MSSB_index_intra = 3*N |
| Moderator | GTW agreement: Follow the conclusion for inter-frequency. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2208811 Draft CR – Introduction of scheduling restriction requirements in FR2-2 | Company A |
|  | Company B |
| R4-2209050 Draft CR to TS 38.133 Corrections on scheduling availability in FR2-2 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 Rx beam sweeping scaling factor | Issue 1-1-1: Rx beam sweeping scaling factor Companies’ views: Except for two companies, all companies can support Alternative 1. GTW agreements: Further discuss the two alternative values for scaling factor - Alternative 1: Define the new scaling factor of 12 - Alternative 2: The scaling factor is 8 - Alternative 3: If there is no agreement on alternative 1 or alternative 2, then define the new UE capability and apply the scaling factor of 8 or [16] depending on UE capability Candidate options: - Option 1: Define the new scaling factor of 12 - Option 2: The scaling factor is 8 - Option 3: If there is no agreement on option 1 or option 2, then define the new UE capability and apply the scaling factor of 8 or [16] depending on UE capability Recommendations for 2nd round: This is the last meeting to complete the core requirements, and this issue affects other pending issues as well, can the companies agree on Option 1. Issue 1-1-2: maxNumberRxBeam Tentative agreements: No agreement Candidate options: - Option1: If Rx Beam Sweeping Scaling factor is extended, maxNumberRxBeam should be extended by the number - Option 2: Do not extend maxNumberRxBeam Recommendations for 2nd round: Companies to check if they can agree to Option 1 |
| Sub-topic #1-2 deriveSSB-IndexFromCell | Issue 1-2-1: deriveSSB-IndexFromCell Tentative agreements: No agreement Candidate options: - Option 1: SSB SCS (KHz); Data SCS (KHz); Frame boundary alignment tolerance of PDSCH symbols (deriveSSB-IndexFromCell disabled). 960; 120; 1 120KHz symbol. 960; 480; 3 480KHz symbols. 960; 960; 6 960KHz symbols. - Option 2: SSB SCS (KHz); Data SCS (KHz); Frame boundary alignment tolerance of PDSCH symbols (deriveSSB-IndexFromCell disabled). 960; 120; 1 120KHz symbol. 960; 480; 3 480KHz symbols. 960; 960; 7 960KHz symbols. Recommendations for 2nd round: Companies to check if they can compromise to Option 1. |
| Sub-topic #1-3: Measurement procedures | Issue 1-3-1: Cell (PSS/SSS) detection GTW agreements: RAN4 to relax the cell-detection delay requirements by a factor and further discuss the number of the factor to avoid increasing the HW and memory requirements - Option 1: a factor of 2 for 480kHz SCS and by a factor of 4 for 960kHz SCS - Option 2: a factor of 1.5 for 480KHz and 2 for 960KHz SCS, and apply the different factors depending on DRX cycles Candidate options: - Option 1: a factor of 2 for 480kHz SCS and by a factor of 4 for 960kHz SCS - Option 2: a factor of 1.5 for 480KHz and 2 for 960KHz SCS, and apply the different factors depending on DRX cycles Recommendations for 2nd round: Continue discussion in the second round. Issue 1-3-2: Inter-frequency SSB index identification time Tentative agreements: The SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended to - 4*NRxBeam samples for power class 2 and 3 - 6*NRxBeam samples for power class 1 - NRxBeam is the Rx beam sweeping factor under discussion in Issue 1-1-1 Candidate options: Recommendations for 2nd round: Companies to confirm the tentative agreement. Issue 1-3-3: Intra-frequency SSB index identification time when deriveSSB-IndexFromCell is not enabled GTW agreements: When deriveSSB-IndexFromCell is not enabled, the requirements of SSB index detection for FR2-2 for intra-frequency measurement can be defined as below: Time period for time index detection for intra-frequency measurements for FR2-2 DRX cycle; Without measurement gaps; With measurement gaps. No DRX; Max(200ms, ceil(MSSB_index_intra Kp) SMTC period) CSSFintra; Max(200ms, ceil(MSSB_index_intra Kp) Max(MGRP, SMTC period)) CSSFintra. DRX cycle≤ 320ms; Max(200ms, ceil(1.5 MSSB_index_intra s Kp) Max(SMTC period, DRX cycle)) CSSFintra; Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(MGRP, SMTC period, DRX cycle)) CSSFintra. DRX cycle>320ms; ceil(MSSB_index_intra Kp) DRX cycle CSSFintra; ceil(MSSB_index_intra Kp) DRX cycle CSSFintra. - MSSB_index_intra to apply agreements from inter-frequency SSB index identification time in Issue 1-3-2 Candidate options: Recommendations for 2nd round: No further discussion in 2nd round |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2208811 Draft CR – Introduction of scheduling restriction requirements in FR2-2 | Agreeable |
| R4-2209050 Draft CR to TS 38.133 Corrections on scheduling availability in FR2-2 | Agreeable |

## Discussion on 2nd round

### Sub-topic 1-1: Rx beam sweeping scaling factor

Issue 1-1-1: Rx beam sweeping scaling factor

- Option 1: Define the new scaling factor of 12

- Option 2: The scaling factor is 8

- Recommended WF

  - This is the last meeting to complete the core requirements, and this issue affects other pending issues as well, can the companies agree on Option 1?

| Company | Comments |
| --- | --- |
| Ericsson | We can not support Option1. We support Option2 or can compromise to capability support 8 or [16]. The [16] shall be checked together with scaling factor on 480KHz/960KHz to prevent too long detection time which make mobility useless. |
| Nokia | Our preference is Option 2. As said, proponents of option 1 did not bring simulations or supporting technical analysis to justify that N=8 is not feasible. As for the capability, we think that this doesn’t solve the problem, since the minimum UE capability will anyway be the largest N. If we agree on Option 1, we also have to discuss spec impact and work split. Our suggestion is to use the following table as reference: - Cell reselection - Intra-frequency - Inter-frequency - Handover - RRC Connection Mobility Control - RRC re-establishment - RRC release with redirection - Signalling characteristics - SCell activation and deactivation delay - Conditional PSCell change - Radio link monitoring - Link recovery procedures - Uplink spatial relation switch delay - Measurement requirements - Intra-frequency - Inter-frequency - L1-RSRP measurements for reporting - L1-SINR measurements for reporting - CSI-RS based measurement |
| vivo | We support Option 1. Based on the RF conclusion, the number of antenna size for FR2-2 need to be doubled. We understand the value of 12 has been a good and compromised choice when considering the effects of both measurement delay and coverage. As for the spec impact, we agree with the Nokia’s comment. The requirements related to Rx beam sweeping factor need to be revised. |
| CATT | We prefer option1since it is a good compromise of 8 and 16, and we suggest to discuss spec impact and work split as Nokia commented. |
| Huawei | We support option 1. Companies supporting 8 is only trying to emphasize “what UE should do” but ignore “What UE can do”. We didn’t observe convincing argument about how UE can maintain the same Rx beam sweeping factor when operating in FR2-2. If so, why RF spends quite many meeting to enlarge the number of antenna elements to achieve equivalent or even poorer EIRP requirements. And we also cannot agree with the argument that the RF agreement is independent to RRM requirements. As shown in 1st round, it was agreed that the Rx beam scaling factor depends on RF conclusion on antenna size assumption and power class. Rx beam sweeping scaling factor - Rx beam scaling factor for FR2-2 need to continue waiting for more RF conclusion on antenna size assumption and power class. |
| LGE | We support to increase the scaling factor as option 1. And we are also fine with both 8 and [16] with capability as commented by Ericsson. |
| Qualcomm | We support Option 1. A scaling factor of 12 offers a good compromise between delay and UE Rx beam sweeping. We don’t think the impact on requirements will be huge as it mostly involves updating the scaling factor from 8 to 12 for FR2-2. We are okay with discussing the impact and work-split. |
| Apple | We support Option 1, which is already a compromise that aims to minimize the delay impact. |

Issue 1-1-2: maxNumberRxBeam

- Option1: If Rx Beam Sweeping Scaling factor is extended, maxNumberRxBeam should be extended by the number

- Option 2: Do not extend maxNumberRxBeam.

- Recommended WF

  - Can companies agree to Option 1?

| Company | Comments |
| --- | --- |
| Ericsson | It’s straightforward to accept Option1 unless any issue caused by Option1 is revealed. |
| vivo | Option 1. |
| CATT | Option 1. |
| LGE | Support option 1. |
| Qualcomm | Support option 1 |
| Apple | We support Option 1. |

### Sub-topic 1-2: deriveSSB-IndexFromCell

Issue 1-2-1: deriveSSB-IndexFromCell

- Option 1:

| SSB SCS (KHz) | Data SCS (KHz) | Frame boundary alignment tolerance of PDSCH symbols (deriveSSB-IndexFromCell disabled) |
| --- | --- | --- |
| 960 | 120 | 1 120KHz symbol |
| 960 | 480 | 3 480KHz symbols |
| 960 | 960 | 6 960KHz symbols |

- Option 2:

| SSB SCS (KHz) | Data SCS (KHz) | Frame boundary alignment tolerance of PDSCH symbols (deriveSSB-IndexFromCell disabled) |
| --- | --- | --- |
| 960 | 120 | 1 120KHz symbol |
| 960 | 480 | 3 480KHz symbols |
| 960 | 960 | 7 960KHz symbols |

- Recommended WF

  - Can companies agree to Option 1?

| Company | Comments |
| --- | --- |
| Ericsson | Support Option 2. With respect to discussion in last meeting, the number is derived by 1.5Km propagation difference among gNB. To our understanding, propagation delay difference or ISD difference shall not be prerequisite of UE’s capability. |
| vivo | We prefer to keep the previous agreement. As commented in the first round, if the value is changed from 6 to 7, the L3 measurement scheduling restriction requirements for FR2-2 also need further corrections. However, the corresponding CR is already been agreed. |
| CATT | Prefer option 2 and Ericsson’s comment seems like more reasonable. And it is also more flexible for the network. |
| Huawei | We are fine with option 2. |
| Qualcomm | We prefer to keep option 1 which was agreed in the last meeting. We have defined most of the RRM requirements for FR2-2, such as timing requirements, the frame boundary tolerance requirements, scheduling restrictions etc., considering a cell size of 1km. As far as we know, RAN1 also used a cell size of around 1km in their study. |
| Apple | We also prefer to keep Option 1. |

### Sub-topic 1-3: Measurement procedures

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-3-1: Cell (PSS/SSS) detection

GTW agreements: RAN4 to relax the cell-detection delay requirements by a factor and further discuss the number of the factor to avoid increasing the HW and memory requirements

- Option 1: a factor of 2 for 480kHz SCS and by a factor of 4 for 960kHz SCS

- Option 2: a factor of 1.5 for 480KHz and 2 for 960KHz SCS, and apply the different factors depending on DRX cycles

- Recommended WF

  - Discuss the options.

| Company | Comments |
| --- | --- |
| Ericsson | We keep cautious view on the issue. Option1 may cause 4 times of detection time and it makes mobility useless if RX beam sweep number is taken into account simultaneously. In this regard, we support Option 2 and open to discuss the exact number. |
| Nokia | We support Option 2. The beam sweeping scaling factor and the intra/inter-frequency identification time are still under discussion, and together with this factor they will all influence the PSS/SSS detection time, As we commented in the first round, if Option 1 is adopted, and the Proposal 3 in Issue 1-3-2 is approved, and N=16, this is equivalent to increasing Mpss/sss_syncfrom - Mpss/sss_sync = 5*8 = 40 to 4*Mpss/sss_sync = 4*(6*16) = 384 That would mean increasing the PSS/SSS identification time 9.6 times. |
| Huawei | As commented in 1st round, if the intention is to let UE process in different window, then 1.5 seems doesn’t make much sense (should be at least 2?). But we share the same concerns on doubling the total time. From our understanding, with 480 and 960 KHz, when UE is not required to monitor the whole SMTC window, the number of samples that UE is required to process depends on the number of symbols and FFT size. For the time being, it seems the situation is not that sever. I am not sure whether it is fine to discuss this in maintenance stage or performance stage to give more time to analyze. |
| Qualcomm | The Rx beam sweeping scaling factor being discussed in Issue 1-3-2 seems to have a majority support of 12. Under the assumption that a factor of 12 is agreed, we have Mpss/sss_sync_w/o_gaps = 120 (12*2 *5) for 480kHz SCS and 240 (12*4*5) for 960kHz SCS As mentioned by Huawei, the goal of this relaxation is to allow the UE to process the samples in a TDM fashion. And a factor of 1.5 may not work well. Being said that we understand the concerns on long detection delays and therefore can accept the following compromise: Option 3 (Compromise): a factor of 2 for 480kHz SCS and by a factor of 3 for 960kHz SCS |
| Nokia | What about? Option 4: a factor of 2 for 480kHz SCS and by a factor of 2 for 960kHz SCS |
| Apple | We are OK with Option 3 proposed by QC as compromise. |

Issue 1-3-2: Inter-frequency SSB index identification time

- Proposal 3 (Moderator) (Based on the GTW discussion): The SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended to

  - 4*NRxBeam samples for power class 2 and 3

  - 6*NRxBeam samples for power class 1

  - NRxBeam is the Rx beam sweeping factor under discussion in Issue 1-1-1

- Recommended WF

  - Agree on proposal 3.

| Company | Comments |
| --- | --- |
| Ericsson | Agree on Proposal 3. |
| Nokia | If we agree with NRxBeam=8 in Issue 1-1-1 we are fine with Proposal 3. |
| vivo | Support Proposal 3. |
| Huawei | Support proposal 3. |
| LGE | Support proposal 3. |
| Qualcomm | We support proposal 3. |
| Apple | Proposal 3 is OK. |

# Topic #2: Timing requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2208145 | CATT | Observations 1: The basic principles for MRTD requirements in FR2-2 contradict the definition of MRTD in legacy spec. Proposal 1: Change the definition of MRTD for synchronous NR DC in FR2-2 as below, so that it could be larger than 0.5 slot - For FR2-2 operation, a UE shall be capable of handling a relative receive timing difference between slot timing boundary of a cell belonging to MCG and slot timing boundary from the same slot index of a cell belonging to the SCG to be aggregated for NR DC operation. A UE shall be capable of handling a relative receive timing difference among the closest slot timing boundaries of different carriers to be aggregated in NR carrier aggregation. Proposal 2: RAN4 to change the definition of receive timing difference between carriers in case of NR CA to address the case when MRTD is larger than one slot. For instance, RTD can be considered between the slot boundaries. Proposal 3: The existing MRTD requirements for FR1 and FR2-1 synchronous NR-DC can be reused for FR1 and FR2-2, i.e., MRTD = 33 µs Proposal 4: MRTD = [0.26] µs for non-contiguous intra band CA in FR2-2 Proposal 5: The existing MRTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2, i.e., MRTD = 25 µs Proposal 6: Define MTTD requirements in FR2-2 based on the following rule: - MTTD = MRTD + (TA step size / 2+ TA adjustment accuracy + Te) in cc1 + ( TA step size / 2 + TA adjustment accuracy +Te) in cc2. Proposal 7: The existing MTTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2, i.e., MTTD = 26.1 us Proposal 8: The existing MTTD requirements for inter-band synchronous NR DC for FR1 and FR2-1 can be reused for inter-band synchronous NR DC for FR1 and FR2-2, i.e., MTTD = 34.1 us |
| R4-2208650 | Ericsson | Proposal 1: MRTD = 0.26 µs for non-contiguous intra band CA in FR2-2, if 260 ns requirement is applicable for intra-band non-contiguous CA TAE and pending on the conclusion in main and RRM sessions for the supporting on non-contiguous CA in Rel-17 FR2-2 52.6-71 GHz. Observation 1: It is an important observation that to be able to reuse FR2-1 sites, for FR2-2 sites, then MRTD has to be unchanged, since the new sites will have the same distance to FR1 site as FR2-1. Proposal 2: MRTD = 8 µs for FR2-2 CA inter band. Proposal 3: MRTD = 25 µs for inter band carrier aggregation between FR1 and FR2-2. Proposal 4: MTTD = 8.5 µs for FR2-2 CA inter band. Proposal 5: MTTD = 26.1 µs for inter band carrier aggregation between FR1 and FR2-2. Proposal 6: MRTD = 33 µs for inter-band asynchronous NR-DC, for SCG SCS = 480 kHz and SCG SCS = 960 kHz. Proposal 7: MTTD = 34.1 µs for inter-band asynchronous NR-DC, for SCG SCS = 480 kHz and SCG SCS = 960 kHz. |
| R4-2208810 | Vivo | Proposal 1: Considering the lower RF margin and the shorter timing adjustment period for FR2-2 for gradual timing adjustment requirements. Proposal 2: The existing MRTD requirements for inter-band CA for FR1 and FR2-1 should be reused for inter-band CA for FR1 and FR2-2. Proposal 3: Update the definition of MRTD for inter-band CA for FR1 and FR2-2. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’. Proposal 4: The existing MRTD requirements for inter-band NR DC for FR1 and FR2-1 should be reused for inter-band NR DC for FR1 and FR2-2. Proposal 5: Update the definition of MRTD for inter-band NR DC for FR1 and FR2-2. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’. Proposal 6: The existing MTTD requirements for inter-band CA for FR1 and FR2-1 should be reused for inter-band CA for FR1 and FR2-2. Proposal 7: Update the definition of MTTD for inter-band CA for FR1 and FR2-2. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’. Proposal 8: The existing MTTD requirements for inter-band NR DC for FR1 and FR2-1 should be reused for inter-band NR DC for FR1 and FR2-2. Proposal 9: Update the definition of MTTD for inter-band NR DC for FR1 and FR2-2. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’. |
| R4-2208947 | Huawei | Proposal 1: For UL SCS of 120 kHz in FR2-2, a UE is required to meet the UL timing accuracy requirements if an SSB is available in the last 160 ms, and Te is proposed to be 2.7*64*Tc when SSB is 480 KHz and 2.5*64*Tc when SSB is 960 KHz. Proposal 2: Define gradual timing adjustment in finer granularity with 100 ms, which will leads to total 12.8 ns timing drift, and take the TA adjustment accuracy as baseline for the RF margin. Proposal 3: Modify the definition of MRTD for FR1 and FR2-2 inter-band CA as the a relative transmission timing difference among the closest frame timing boundaries excluding slot offset if configured. Proposal 4: No need to modify the definition of MRTD for NR-DC. The MRTD for FR1 and FR2-2 can be defined as 33 us or min (33us, 0.5 slot) Proposal 5: There is no need to define MRTD/MTTD requirements for intra-band non-contiguous CA within FR2-2. Observation 1: MTTD for inter-band FR1 and FR2-2 CA and inter-band FR1 and FR2-2 NR-DC can be calculated as 26.0 us and 34.0 us, which may subject to conclusion on MRTD and TA adjustment accuracy. |
| R4-2209385 | Nokia | Observation 1: Accuracy of SSB-based DL timing detection improves by 4.4 ns = 0.14*64*Tc for 480 kHz in comparison to 240 kHz SSB. Observation 2: Accuracy of SSB-based DL timing detection improves by 6.5 ns = 0.2*64*Tc for 480 kHz in comparison to 240 kHz SSB. Proposal 1: Determine Te UL accuracy for 120 kHz for different SSB SCS as Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te. 2-2; 120; 120; 3.5*64*Tc. 480; 120; 2.86*64*Tc. 960; 120; 2.80*64*Tc. Observation 3: If 20 km/h is considered, timing offset variation, clock drift, and sampling rate account for a Tq value of about 0.79*64*Tc. Observation 4: Latest agreements on Te values include a minimum value of 0.86*64*Tc for 960 kHz. Proposal 2: Adopt Tq=Tp=0.8*64*Tc for the gradual timing adjustment parameters for 480 kHz and 960 kHz SCS in FR2-2 |
| R4-2207781 | Apple | Proposal 1: Te = 2.88*64*Tc for (480kHz, 120kHz) and Te = 2.82*64*Tc for (960kHz, 120kHz). Proposal 2: The UE timing advance adjustment accuracy for 480/960kHz SCS is defined as shown in the table below. UL Sub Carrier Spacing(kHz); 15; 30; 60; 120; 480; 960. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc; [±12 Tc]; [±8 Tc]. |
| R4-2208812 | Vivo | Draft CR – Introduction of deriveSSB-IndexFromCell tolerance requirements in FR2-2 |
| R4-2209386 | Nokia | Draft CR adding timing requirements for FR2-2 |
| R4-2207782 | Apple | Draft CR on UE transmit timing for NR operation in 52.6GHz – 71GHz |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: UE transmit timing

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1-1: UE transmit timing error

- Proposal 1 (Huawei): For UL SCS of 120 kHz in FR2-2, a UE is required to meet the UL timing accuracy requirements if an SSB is available in the last 160 ms, and Te is proposed to be 2.7*64*Tc when SSB is 480 KHz and 2.5*64*Tc when SSB is 960 KHz

- Proposal 2 (Nokia): Determine Te UL accuracy for 120 kHz for different SSB SCS as:

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 2-2 | 120 | 120 | 3.5*64*Tc |
|  | 480 | 120 | 2.86*64*Tc |
|  | 960 | 120 | 2.80*64*Tc |

- Proposal 3 (Apple): Te = 2.88*64*Tc for (480kHz, 120kHz) and Te = 2.82*64*Tc for (960kHz, 120kHz).

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Ericsson | Proposals 1,2 and 3 are quite similar and all ok. We have a preference for proposal 1, since it is marginally stricter, but again all values are fine. |
| Nokia | Our preference is proposal 2, as we have calculated on our discussion paper based on the improvement of SSB DL timing accuracy. |
| Moderator | GTW agreement: Agree on proposal 2 |

Issue 2-1-2: Gradual timing adjustment

- Proposal 1 (Vivo): Considering the lower RF margin and the shorter timing adjustment period for FR2-2 for gradual timing adjustment requirements.

- Proposal 2 (Huawei): Define gradual timing adjustment in finer granularity with 100 ms, which will leads to total 12.8 ns timing drift, and take the TA adjustment accuracy as baseline for the RF margin.

- Proposal 3 (Nokia): Adopt Tq=Tp=0.8*64*Tc for the gradual timing adjustment parameters for 480 kHz and 960 kHz SCS in FR2-2

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Ericsson | Again, proposals 2 and 3 are quite similar since in proposal 2 we have Tq=Tp=0.8*64*Tc and if we scale proposal 3 to 200 ms T_gradual we get 2*12.8 ns ≈ 0.79*64*Tc. We are not against a T_gradual = 100 ms. We support considering a lower RF margin in proposal 1 and (again) a we can discuss shorter T_gradual. |
| Nokia | We agree with Proposal 3 if it considers 200 ms granularity. Fine with Proposal 2. If we agree on proposal 2 the values of Proposal 3 should be revised to Tp=Tq=0.46*64*Tc, therefore we included a new Proposal 4, which considers both at the same time. |
| Moderator | GTW Agreement: Consider the RF margin for gradual timing adjustment. Further discuss the proposals, based on GTW agreement |
| Huawei | From our understanding, the timing error that UE need to compensate depends on following factors: - Time granularity - Timing drift due to frequency error: 0.1 ppm - Timing drift due to movement - RF margin It is recognized that current Tp and Tq is too large compared with Te requirements. Then to has a smaller Tp and Tq, only factor 1 and 4 can be further considered (finer granularity and/or smaller RF margin). With finer adjustment granularity, the timing drift caused by factor 2 and 3 will be reduced by half. Regarding the RF margin, current assumption is 1.5 Ts, we don’t think it is relay needed have such big margin for FR2-2. So we also like to hear more views from companies about the RF margin. |
| Apple | Given the tight Te requirements are based on reduced SSB availability time period, we are OK to take proposal 2 as a baseline. Besides considering a finer granularity with 100ms, we can consider 80ms. |
| Qualcomm | We are okay with proposal 3 |

Issue 2-1-3: UE timing advance adjustment accuracy

- Proposal 1 (Apple): The UE timing advance adjustment accuracy for 480/960kHz SCS is defined as shown in the table below:

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc | [±12 Tc] | [±8 Tc] |

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Magnus | We prefer the already agreed 480 kHz [±8 Tc] 960 kHz [±4 Tc]. |
| Nokia | We prefer to keep the previous agreement. |
| CATT | We prefer the previous agreement, that is, [±8 Tc] for 480 kHz and [±4 Tc] for 960 kHz. |
| Apple | Based on our analysis, the previous tentative agreement was indeed challenging from UE implementation’s point of view and we raised this issue multiple times. To move forward, we offered proposal 1 as a compromise and hope companies can accept and thus RAN4 can close this issue. |
| Qualcomm | We can agree with proposal 1 |

### Sub-topic 2-2: MRTD

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-2-1: MRTD definition – NR-DC

- Proposal 1 (CATT): Change the definition of MRTD for synchronous NR DC in FR2-2 as below, so that it could be larger than 0.5 slot

  - For FR2-2 operation, a UE shall be capable of handling a relative receive timing difference between slot timing boundary of a cell belonging to MCG and slot timing boundary from the same slot index of a cell belonging to the SCG to be aggregated for NR DC operation. A UE shall be capable of handling a relative receive timing difference among the closest slot timing boundaries of different carriers to be aggregated in NR carrier aggregation

- Proposal 1a: Change the definition of MRTD for synchronous NR DC in FR2-2 as below, so that it could be larger than 0.5 slot

  - For FR2-2 operation, a UE shall be capable of handling a relative receive timing difference between slot timing boundary of a cell belonging to MCG and slot timing boundary from the same slot index of a cell belonging to the SCG to be aggregated for synchronous NR DC operation.

- Proposal 2 (Vivo): Update the definition of MRTD for inter-band NR DC for FR1 and FR2-2. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’

- Proposal 3 (Huawei): No need to modify the definition of MRTD for NR-DC. The MRTD for FR1 and FR2-2 can be defined as 33 us or min (33us, 0.5 slot).

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Ericsson | We support proposal 2, The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’, or even frame timing. |
| Nokia | We are fine with Proposal 1 in general, but we proposed a correction in Proposal 1a, since it was also including carrier aggregation. |
| Moderator | GTW Agreement: Down-select to proposal 2 and proposal 3. Further discuss proposal 2 and 3. |
| Huawei | Regarding proposal 2, we would like to know the necessary to update slot timing to subframe timing. Is it only to eliminate the ambiguity in RRM spec because 33us maybe always large than half slot? We would like here more views from companies regarding following capability. We don’t think frame synchronization or subframe synchronization is needed for NR-DC. Sfn-SyncNRDC Indicates the UE supports NR-DC only with SFN and frame synchronization between Pcell and PSCell. If not included by the UE supporting NR-DC, the UE supports NR-DC with slot-level synchronization without condition on SFN and frame synchronization. In this release of the specification, the UE shall not report this UE capability.; UE; No; No; No. |
| Apple | Proposal 2 is OK, as it does not require any change in the MRTD definition. |
| Qualcomm | We are fine with proposal 3. But this basically means that FR1 +FR2-2 NR-DC will always be asynchronous, which makes sense. So why not just define requirements for async case? @Apple – Do you mean proposal 3? Proposal 2 indicates change in MRTD definition. |
| Nokia | We still think update in definition is needed. |

Issue 2-2-2: MRTD definition – NR-CA

- Proposal 1 (CATT): RAN4 to change the definition of receive timing difference between carriers in case of NR CA to address the case when MRTD is larger than one slot. For instance, RTD can be considered between the slot boundaries

- Proposal 2 (Vivo): Update the definition of MRTD for inter-band CA for FR1 and FR2-2. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’

- Proposal 3 (Huawei): Modify the definition of MRTD for FR1 and FR2-2 inter-band CA as the relative transmission timing difference among the closest frame timing boundaries excluding slot offset if configured.

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Ericsson | We support proposal 2, The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’, or even frame timing. |
| Nokia | We are fine with Proposal 1. |
| Huawei | We prefer option 3. Frame time of subframe timing can both work. Regarding the slot offset, more clarification as shown below (excerpted from 38300): CA is supported for both contiguous and non-contiguous CCs. When CA is deployed frame timing and SFN are aligned across cells that can be aggregated, or an offset in multiples of slots between the Pcell/PSCell and an Scell is configured to the UE. The maximum number of configured CCs for a UE is 16 for DL and 16 for UL. |
| Apple | We wonder if we can reuse the approach of min (Xus, 0.5 slot) without having to change the MRTD definition. |
| Qualcomm | We are fine with proposal 3. Also, @Apple’s suggestion makes sense to us. |
| Huawei2 | We share slightly different views as Apple and Qualcomm. Apple’s suggestion imply that there is no any synchronization restriction for CA (the definition of MRTD is between closet slots). From our understanding, things are different from the DC case, SFN and frame timing alignment is needed for CA as shown in our last comments. |
| vivo | We support proposal 2. We are also fine with proposal 3. |

Issue 2-2-3: MRTD for non-contiguous intra-band CA

- Proposal 1 (CATT, Ericsson): MRTD = [0.26] µs for non-contiguous intra band CA in FR2-2

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| Ericsson | Actually proposal 1 depends on the status of the feature NC CA in WI ext to 71 GHz. This depends on outcome in RAN4 RF session, as we stated in Ericsson R4-2208650: “MRTD = 0.26 µs for non-contiguous intra band CA in FR2-2, if 260 ns requirement is applicable for intra-band non-contiguous CA TAE and pending on the conclusion in main and RRM sessions for the supporting on non-contiguous CA in Rel-17 FR2-2 52.6-71 GHz.” If the feature I included then we agree to MRTD = [0.26] µs for non-contiguous intra band CA in FR2-2, with collocated assumption. |
| Huawei | From our understanding, NC CA is already down scoped. |
| CATT | Support proposal 1. |
| MTK | We would like to point out that for the higher SCS, if MRTD = [0.26] us, additional scheduling restriction, due to RX beam switch, AGC tuning, will be needed to accommodate for the MRTD. |
| Apple | Our understanding NC CA has been removed from the scope and there is no need to define the requirement. |
| Qualcomm | Agree with Huawei. This case has been downscoped, so RAN4 doesn’t need to discuss the MRTD for this case |
| vivo | We share the same understanding with Huawei. It seems that the non-contiguous intra band CA scenario was down-scope in Release 17 in the RP #95e meeting. |

Issue 2-2-4: MRTD for FR1 and FR2-2 inter-band CA

- Proposal 1 (CATT, Ericsson, Vivo): The existing MRTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2, i.e., MRTD = 25 µs

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Ericsson | We support proposal 1, MRTD = 25 µs |
| Nokia | We are fine with the proposal 1 |
| Huawei | Fine with proposal 1 |
| CATT | Support proposal 1. |
| Apple | Given the large CBW in FR2-2 and the high data rate, reusing the same MRTD 25us from FR2-1 may pose some issue for UE and warrant further study. |
| Qualcomm | We see some serious issues with using MRTD of 25us for this case. MRTD of 25us translates to the timing misalignment of two slots for 960kHz SCS. For example, how would the UE transmit HARQ ACK/NACK configured in the next slot when there’s only one uplink carrier. The network is not aware of the MRTD, so it cannot avoid such scenarios. There may be many other scenarios in the field where the UE may not be able to transmit/receive with such large MRTD. RAN4 should consider smaller MRTD values for FR1+FR2-2 inter-band CA. We think limiting the MRTD values to half slot length should be considered for all the cases. |

Issue 2-2-5: MRTD for FR1 and FR2-2 NR DC – Synchronous

- Proposal 1 (CATT, Ericsson, Vivo, Huawei): The existing MRTD requirements for FR1 and FR2-1 synchronous NR-DC can be reused for FR1 and FR2-2, i.e., MRTD = 33 µs

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Ericsson | We support proposal 1, MRTD = 33 µs |
| Nokia | We are fine with Proposal 1 |
| Huawei | Fine with proposal 1 |
| CATT | Support proposal 1. |
| Apple | Given the large CBW (up to 2000MHz) in FR2-2 and the high data rate, reusing the same MRTD 33us from FR2-1 may pose some issue for UE and warrant further study. |
| Qualcomm | Agree with @Apple. As mentioned in the previous issue, we should limit the MRTD to half slot length for all cases. |

### Sub-topic 2-3: MTTD

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-3-1: MTTD definition

- Proposal 1 (Vivo): Update the definition of MTTD for inter-band CA for FR1 and FR2-2. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’

- Proposal 2 (Vivo): Update the definition of MTTD for inter-band NR DC for FR1 and FR2-2. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’.

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Ericsson | Proposals 1 ans 2 are fine. |
| Nokia | We prefer to conclude the definition as in MRTD and reuse the same wording for MTTD. |
| Huawei | Same comments as MRTD |
| CATT | Depends on MRTD, we would like to reuse the same wording for MTTD. |
| Apple | We also prefer to wait for the conclusion of MRTD. |
| Qualcomm | Prefer to wait for conclusions on MRTD |
| vivo | We are OK to wait for the outcome of MRTD. |

Issue 2-3-2: MTTD for FR1 and FR2-2 inter-band CA

- Proposal 1 (CATT, Ericsson, Vivo, Huawei): The existing MTTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2, i.e., MTTD = 26.1 us

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| Ericsson | Proposal 1. |
| Nokia | Fine with the proposal |
| CATT | Support proposal 1. |
| Qualcomm | This depends on MRTD values |
| vivo | Support proposal 1. |

Issue 2-3-3: MTTD for FR1 and FR2-2 NR DC - Synchronous

- Proposal 1 (CATT, Ericsson, Vivo, Huawei): The existing MTTD requirements for inter-band synchronous NR DC for FR1 and FR2-1 can be reused for inter-band synchronous NR DC for FR1 and FR2-2, i.e., MTTD = 34.1 us

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| Ericsson | Proposal 1. |
| Nokia | Fine with the proposal |
| CATT | Support proposal 1. |
| Qualcomm | This depends on MRTD values |
| vivo | Support proposal 1. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2207782 Draft CR on UE transmit timing for NR operation in 52.6GHz - 71GHz | Ericsson: We already have [8 Tc] and [4 Tc] in agreement, We prefer to adhere to previous agreement in brackets. The UL UE time budget is already very relaxed due to large Te values for SCS = 480 and 960 kHz. If we further relax adjustment accuracy as well, there is no margin left for BS RX demodulation to cope with any radio delay dispersion in the channel. |
|  | Nokia: We share the same opinion as Ericsson regarding TAC accuracy. |
| R4-2208812 Draft CR - Introduction of deriveSSB-IndexFromCell tolerance requirements in FR2-2 | Company A |
|  | Company B |
| R4-2209386 Draft CR adding timing requirements for FR2-2 | Ericsson: Need to merge Te values with R4-2207782. Should be ok, they are rather similar. |
|  | Huawei: Depends on pending issues |
|  | Nokia: Considering the outcome of 2-1-1 is in line with this CR, may we use R4-2209386 for the Te requirements? |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 UE transmit timing error | Issue 2-1-1: UE transmit timing error GTW agreements: Determine Te accuracy for 120 kHz for different SSB SCS as: Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te. 2-2; 120; 120; 3.5*64*Tc. 480; 120; 2.86*64*Tc. 960; 120; 2.80*64*Tc. Candidate options: Recommendations for 2nd round: No further discussion in 2nd round. Issue 2-1-2: Gradual timing adjustment GTW agreements: Consider the RF margin for defining the gradual timing adjustment requirements for FR2-2 Candidate options: - Proposal 1: The maximum aggregate adjustment rate shall be Tq per 200 ms. - Option 1a: Define gradual timing adjustment parameters, Tp, Tq for 480 kHz and 960 kHz SCS in FR2-2 as Tq=Tp=0.8*64*Tc - Proposal 2: The maximum aggregate adjustment rate shall be Tq per 100 ms. - Option 2a: Define gradual timing adjustment parameters, Tp, Tq for 480 kHz and 960 kHz SCS in FR2-2 as Tq=Tp=0.46*64*Tc - Option 2b: Define gradual timing adjustment parameters, Tp, Tq for 480 kHz and 960 kHz SCS in FR2-2 as Tq=Tp=0.8*64*Tc - FFS: Discuss whether the minimum aggregate adjustment rate shall be Tp per second or Tp per 0.5 second Recommendations for 2nd round: Continue discussion in 2nd round. Issue 2-1-3: UE timing advance adjustment accuracy Tentative agreements: No agreement Candidate options: The UE timing advance adjustment accuracy for 480/960kHz SCS is defined as shown in the table below: - Option 1: UL Sub Carrier Spacing(kHz); 15; 30; 60; 120; 480; 960. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc; [±12 Tc]; [±8 Tc]. - Option 2 (previous agreement): UL Sub Carrier Spacing(kHz); 15; 30; 60; 120; 480; 960. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc; [±8 Tc]; [±4 Tc]. Recommendations for 2nd round: Continue discussion in 2nd round. If no agreement reached in this round, this issue will not be discussed any further and previous agreement (Option 2) will hold. |
| Sub-topic #2-2: MRTD | Issue 2-2-1: MRTD definition – NR-DC GTW agreements: Down select to two options. Candidate options: - Option 1: Update the definition of MRTD for inter-band NR DC for FR1 and FR2-2. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’ - Option 2: No need to modify the definition of MRTD for NR-DC. The MRTD for FR1 and FR2-2 can be defined as 33 us or min (33us, 0.5 slot). Recommendations for 2nd round: Continue discussion in 2nd round Issue 2-2-2: MRTD definition – NR-CA Tentative agreements: No agreements. Candidate options: - Option 1: RAN4 to change the definition of receive timing difference between carriers in case of NR CA to address the case when MRTD is larger than one slot. For instance, RTD can be considered between the same slot boundaries - Option 2: Update the definition of MRTD for inter-band CA for FR1 and FR2-2. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’ - Option 3: Modify the definition of MRTD for FR1 and FR2-2 inter-band CA as the relative transmission timing difference among the closest frame timing boundaries excluding slot offset if configured. Recommendations for 2nd round: Continue discussion in 2nd round Issue 2-2-3: MRTD for non-contiguous intra-band CA Tentative agreements: Do not define MRTD requirements for non-contiguous intra band CA in FR2-2 as the scenario has been down scoped. Candidate options: Recommendations for 2nd round: Confirm the tentative agreement Issue 2-2-4: MRTD for FR1 and FR2-2 inter-band CA Tentative agreements: No agreement. Candidate options: - Option 1: The existing MRTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2, i.e., MRTD = 25 µs - Option 2: MRTD cannot be greater than half slot length. Recommendations for 2nd round: Continue discussion in 2nd round Issue 2-2-5: MRTD for FR1 and FR2-2 NR DC – Synchronous Tentative agreements: No agreement. Candidate options: - Option 1: The existing MRTD requirements for FR1 and FR2-1 synchronous NR-DC can be reused for FR1 and FR2-2, i.e., MRTD = 33 µs - Option 2: MRTD cannot be greater than half slot length. Recommendations for 2nd round: Continue discussion in 2nd round |
| Sub-topic#2-3 MTTD | Issue 2-3-1: MTTD definition Issue 2-3-2: MTTD for FR1 and FR2-2 inter-band CA Issue 2-3-3: MTTD for FR1 and FR2-2 NR DC - Synchronous Tentative agreements: No agreement on all MTTD issues Candidate options: Recommendations for 2nd round: All MTTD related issues depend on MRTD related issues under discussion. Moderator’s recommendation is to revisit MTTD related issues after agreements on MRTD and try to re-use the conclusions from MRTD. No further discussion needed in 2nd round unless agreements are reached on MRTD issues. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2207782 Draft CR on UE transmit timing for NR operation in 52.6GHz - 71GHz | To be revised |
| R4-2208812 Draft CR - Introduction of deriveSSB-IndexFromCell tolerance requirements in FR2-2 | Agreeable |
| R4-2209386 Draft CR adding timing requirements for FR2-2 | To be revised |

## Discussion on 2nd round

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### Sub-topic 2-1: UE transmit timing

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1-2: Gradual timing adjustment

- Proposal 1: The maximum aggregate adjustment rate shall be Tq per 200 ms.

  - Option 1a: Define gradual timing adjustment parameters, Tp, Tq for 480 kHz and 960 kHz SCS in FR2-2 as Tq=Tp=0.8*64*Tc

- Proposal 2: The maximum aggregate adjustment rate shall be Tq per 100 ms.

  - Option 2a: Define gradual timing adjustment parameters, Tp, Tq for 480 kHz and 960 kHz SCS in FR2-2 as Tq=Tp=0.46*64*Tc

  - Option 2b: Define gradual timing adjustment parameters, Tp, Tq for 480 kHz and 960 kHz SCS in FR2-2 as Tq=Tp=0.8*64*Tc

  - FFS: Discuss whether the minimum aggregate adjustment rate shall be Tp per second or Tp per 0.5 second

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 (per 200 ms, Tq=Tp=0.8*64*Tc) or Option 2a (per 100 ms Tq=Tp=0.46*64*Tc) |
| Nokia | We are fine with Option 1a or Option 2a |
| vivo | We support Option 2b. When the shorter timing adjustment period is considered (i.e., 100ms), the total time drift can be reduced. In addition, according to the GTW agreement, the RF margin need to be considered for gradual timing adjustment. However, for Option 2a, only about 1ns of RF margin is considered. Compared with the legacy RF margin (i.e., 1.5*64*Tc) for FR2-1, it would be very small. |
| Huawei | We support option 2b. We would like to known whether RF margin is already included? Regarding FFS point, per second is fine. |
| Qualcomm | We support Option 2b. |
| Apple | Among all options, we prefer Option 2b and would like to put the numbers in brackets for further checking. |

Issue 2-1-3: UE timing advance adjustment accuracy

The UE timing advance adjustment accuracy for 480/960kHz SCS is defined as shown in the table below:

- Option 1:

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc | [±12 Tc] | [±8 Tc] |

- Option 2 (previous agreement):

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc | [±8 Tc] | [±4 Tc] |

- Recommended WF

  - Check whether option 1 can be agreed.

| Company | Comments |
| --- | --- |
| Ericsson | We prefer the already agreed 480 kHz [±8 Tc] 960 kHz [±4 Tc]. The UL timing uncertainty budget is down to the last few ns when it comes to allowed RF channel dispersion. There is no margin for further relaxations. |
| Nokia | We prefer to keep the agreement of Option 2. |
| CATT | Option 2. |
| Apple | We proposed Option 1, which is already a compromise after considering implementation challenges. |

### Sub-topic 2-2: MRTD

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-2-1: MRTD definition – NR-DC

- Option 1: Update the definition of MRTD for inter-band NR DC for FR1 and FR2-2. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’

- Option 2: No need to modify the definition of MRTD for NR-DC. The MRTD for FR1 and FR2-2 can be defined as 33 us or min (33us, 0.5 slot).

- Recommended WF

  - Discuss the options.

| Company | Comments |
| --- | --- |
| Ericsson | Option 1. Update the definition of MRTD for inter-band NR DC for FR1 and FR2-2. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’. [TS 38.133 7.6.6] (synchronous NR DC): “The UE shall be capable of handling at least a relative receive timing difference between slot timing of signal from a cell belonging to the MCG and slot subframe timing of signal from a cell belonging to the SCG at the UE receiver as shown in Table 7.6.6-1 provided that the UE indicates that it is capable of synchronous NR DC only [16].” We need the same update for Carrier Aggregation. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’. [TS 38.133 7.6.4] (CA): For inter-band NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference between slot subframe timing of all pairs of carriers to be aggregated at the UE receiver as shown in Table 7.6.4-2 below. Since existing definitions work up to and including FR2-1 SCS, we can restrict proposed change of definition to rel-17 SCS = 480 kHz and higher. |
| Nokia | We prefer Option 1. We prefer that the final text is written such that only FR2-2 is impacted by the modification, not the legacy requirements as Ericsson proposed. |
| vivo | Prefer Option 1. Update the definition of MRTD for inter-band NR DC for FR1 and FR2-2. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’. Only in this way, the MRTD value for FR1 and FR2-2 inter-band CA/DC can reuse the MRTD value for FR1 and FR2-1 inter-band CA/DC. |
| CATT | Prefer option 1, it is better to reuse the MRTD value for FR1 and FR2-1 inter-band CA/DC. |
| Huawei | For NR-DC We support option 2. From my understanding, companies support option 1 want to reuse the value of legacy requirements, but does UE really need subframe synchronization for DC? We quoted following capability but didn’t receive any comments. For option 1, are we going to change the definition of this capability? Sfn-SyncNRDC Indicates the UE supports NR-DC only with SFN and frame synchronization between Pcell and PSCell. If not included by the UE supporting NR-DC, the UE supports NR-DC with slot-level synchronization without condition on SFN and frame synchronization. In this release of the specification, the UE shall not report this UE capability.; UE; No; No; No. For CA The definition should be updated. |
| Qualcomm | We support Option 2 |
| Ericsson | Option 1, as stated before, above, in order to have a definition compatible with existing MRTD requirements MRTD = 33 µs, since a slot at SCS = 480 kHz is 31.25 µs and a slot at SCS = 960 kHz is 15.62 µs. With regards to Nokia’s comment: Ericsson is fine to restrict any specification updates of the definition to FR2-2 only. (This was also our intention with original comment above, but perhaps this statement is clearer). With regards to MRTD definition for NR-DC: We think it is an advantage to use similar definitions for both NR-CA and NR-DC. |
| Nokia | @Huawei My understanding from that capability is that the UE has to support both synchronous and asynchronous. And in that case the requirements for synchronous NR-DC are still necessary. |
| Apple | Option 2 is OK |

Issue 2-2-2: MRTD definition – NR-CA

- Option 1: RAN4 to change the definition of receive timing difference between carriers in case of NR CA to address the case when MRTD is larger than one slot. For instance, RTD can be considered between the slot boundaries

- Option 2: Update the definition of MRTD for inter-band CA for FR1 and FR2-2. The legacy definition ‘slot timing’ can be replaced with ‘subframe timing’

- Option 3: Modify the definition of MRTD for FR1 and FR2-2 inter-band CA as the relative transmission timing difference among the closest frame timing boundaries excluding slot offset if configured.

- Recommended WF

  - Discuss the options

| Company | Comments |
| --- | --- |
| Qualcomm | Option 3 makes more sense. |
| Ericsson | Option 2 is fine, since it is the same as for NR DC. Option 3 works as well, if it helps bring consensus. With regards to MRTD definition for N- DC: We think it is an advantage to use similar definitions for both NR-CA and NR-DC. We are prepared to limit any redefinitions of MRTD to FR2-2. |
| Nokia | Option 3 is fine |
| Apple | We would like to reflect the absolute MTRD value in the requirement for CA, therefore Option 1 is not preferred. Both Option 2 and option 3 seem OK. We also share the view that if possible, the definition of MRTD for DC and CA should be kept similar. |

Issue 2-2-3: MRTD for non-contiguous intra-band CA

- Proposal 1: Do not define MRTD requirements for non-contiguous intra band CA in FR2-2 as the scenario has been down scoped.

- Recommended WF

  - Agree on proposal 1.

| Company | Comments |
| --- | --- |
| Ericsson | The recommended WF is fine. |
| Nokia | We are fine with the WF. |
| vivo | We agree with the Recommended WF. |
| CATT | Agree with the Recommended WF. |
| Huawei | Fine with the recommended WF |
| Qualcomm | Support the recommended WF |
| Apple | We are OK with the WF. |

Issue 2-2-4: MRTD for FR1 and FR2-2 inter-band CA

- Option 1: The existing MRTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2, i.e., MRTD = 25 µs

- Option 2: MRTD cannot be greater than half slot length.

- Recommended WF

  - Discuss the options

| Company | Comments |
| --- | --- |
| Ericsson | We support proposal 1, MRTD = 25 µs. One reason for our proposal is that operators which reuse existing FR2-1 sites under an FR1 umbrella for FR2-2, would get exactly the same MRTD between FR2-2 and FR1, as for FR2-1 and FR1. |
| Nokia | We prefer Option 1. |
| vivo | Support Option 1. We understand the MRTD for FR1 and FR2-2 NR CA depends on the maximum propagation delay difference which is up to the maximum distance between FR1 BS and FR2-2 BS. For Option 2, it means FR2-2 BS must be placed very close to FR1 BS and the deployment would be limited. |
| CATT | Prefer option 1 and we share the same view with vivo. |
| Huawei | Prefer option 1. The motivation of option 2 is valid, but is put strong restriction on deployment. |
| Qualcomm | We are fine with Option 1 |
| NTT DOCOMO, INC. | Support option 1. As vivo commented, the shoter MRTD limits deployment flexibility. If FR2-2 BS can be only deployed next to the FR1 BS due to the MRTD limitation, the location may be not suitable for FR2-2, i.e. it may be NLOS location. It results in CA incapable deployment. |
| AT&T | Support Option 1. |
| Apple | Option 1 is OK |

Issue 2-2-5: MRTD for FR1 and FR2-2 NR DC – Synchronous

- Option 1: The existing MRTD requirements for FR1 and FR2-1 synchronous NR-DC can be reused for FR1 and FR2-2, i.e., MRTD = 33 µs

- Option 2: MRTD cannot be greater than half slot length.

- Recommended WF

  - Discuss the options

| Company | Comments |
| --- | --- |
| Ericsson | We support proposal 1, MRTD = 33 µs. One reason for our proposal is that operators which reuse existing FR2-1 sites under an FR1 umbrella for FR2-2, would get exactly the same MRTD between FR2-2 and FR1, as for FR2-1 and FR1. |
| Nokia | We support option 1. MRTD=33 us. |
| Vivo | Support Option 1. We understand the MRTD for FR1 and FR2-2 NR DC depends on the maximum propagation delay difference which is up to the maximum distance between FR1 BS and FR2-2 BS. For Option 2, it means FR2-2 BS must be placed very close to FR1 BS and the deployment would be limited. |
| CATT | Prefer option 1 and we share the same view with vivo. |
| Huawei | Support option 1. The motivation of option 2 is not valid for DC case. |
| Qualcomm | We are fine with Option 1 |
| NTT DOCOMO, INC. | Support option 1 as same opinion as issue 2-2-4. |
| AT&T | Support Option 1. |
| Apple | Option 1 is OK |

# Topic #3: Performance requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2208146 | CATT | Proposal 1: Approve the proposed test case list for performance part of NR extension to 71 GHz RRM requirements to complete performance requirements in Rel-17 timeline. Proposal 2: It is recommended to discuss whether to define new test cases for those requirements in FR2-2 that reuse the current specification for FR2-1. - If not to define new test cases for those requirements in FR2-2 that reuse the current specification for FR2-1, the description of frequency bands in the current FR2 test case should be modified to include FR2-2. |
| R4-2208731 | ZTE | Proposal 1: The rate of UE meeting the Te requirement observed during repeated tests shall be at least 95%. |
| R4-2208813 | Vivo | Proposal 1: The new SSB pattern for 480kHz and 960kHz need to be introduced for FR2-2 in RRM test cases. The SSB configurations may be as follows: SSB pattern number; SSB SCS; Channel bandwidth; Number of SSB per SS-burst; Symbol numbers containing SSBs; SSB index; Slot number containing SSBs. 1; 480kHz; 400MHz; 2; 2-5 9-12; 0 1; 0 0. 2; 960kHz; 400MHz; 2; 2-5 9-12; 0 1; 0 0. 3; 480kHz; 400MHz; 1; 2-5; 0; 0. 4; 960kHz; 400MHz; 1; 2-5; 0; 0. 5; 480kHz; 400MHz; 2; 2-5 9-12; 2 3; 1 1. 6; 960kHz; 400MHz; 2; 2-5 9-12; 2 3; 1 1. 7; 480kHz; 400MHz; 1; 9-12; 1; 0. 8; 960kHz; 400MHz; 1; 9-12; 1; 0. |
| R4-2208950 | Huawei | Observation 1: For the requirements without explicit updating for FR2-2, the test cases shall also be considered. Proposal 1: Define test cases for operation in FR2-2 in dedicated sections. Observation 2: For test cases where no particular behavior to be verified, an unavailable SSB/SMTC group can be modeled as that there is one SSB not transmitted by TE in N consecutive SSB/SMTC occasions. Observation 3: When a particular UE behavior is to be verified when exceeding L, UE may be not aware of LBT failure due to beam sweeping. Proposal 2: For test cases where no particular behavior to be verified, an unavailable SSB/SMTC group can be modeled as that there is one SSB not transmitted by TE in N consecutive SSB/SMTC occasions Proposal 3: If particular UE behavior is to be verified when exceeding the limitation of unavailable SBS/SMTC groups, the unavailable SSB/SMTC group can be modeled as none of SSB is transmitted by TE. |
| R4-2209052 | Nokia | Observation 1: Most of the existing RRM performance requirements cannot be assumed to cover the UE behaviour with 480 and 960 kHz SCS. Proposal 1: Define RRM performance requirements for 480 kHz and 960 kHz SCS. Observation 2: No definition on RRM core requirements was reached so far regarding Rx beam sweeping. Observation 3: Open options for Rx beam sweeping are N=8 (unchanged) or in the range from N=12 to N=16. Observation 4: If beam sweeping scaling factor is changed, most of the RRM performance requirements from FR2-1 with 120 kHz cannot be reused for FR2-2. Observation 5: Some FR2-1 test cases may be reused when the configuration does not include SSB SCS which is not supported by FR2-2. Proposal 2: Attempt to reuse FR2-1 RRM performance requirements for 120 kHz SCS that do not depend on the Rx Beam sweeping scaling factor. Proposal 3: If Rx beam sweeping scaling factor is changed in RRM core requirements, define new RRM performance requirements for 120 khz SCS considering the decision on the number of Rx Beam for RRM requirements. Proposal 4: RAN4 not to reuse FR2-1 RRM performance requirements for 120 kHz SCS with SSB using 240 kHz SCS. Proposal 5: RAN4 not to reuse FR2-1 RRM performance requirements for 120 kHz SCS for test cases whose RRM core requirements changed, for example due to new Rx beam sweeping scaling factor. Proposal 6: RAN4 to define RRM performance requirements for the following scenarios: -FR2-2 only standalone -FR2-2 non-contiguous CA -FR1+FR2-2 CA (FR1 is PCell) -FR1+FR2-2 DC (FR1 is PCell) Proposal 7: RAN4 to use as a baseline for the RRM performance without CCA the Rel 15 test cases. Proposal 8: RAN4 to discuss the list of test cases for RRM performance without CCA based on Table 2 Proposal 9: RAN4 to discuss whether the LBT model defined in NR-U needs to be adjusted to support the SMTC/SSB/DRX occasion group definition in FR2-2. Proposal 10: Create in TS 38.133 the following new top-level sections for NR-test cases with CCA in FR2-2: A.14 NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR2-2 A.15 NR Standalone Tests with NR Scell under CCA and All Other Cells in FR2-2 Proposal 11: For requirements that are the same for operation with CCA and without CCA, tests defined for operation without CCA can be reused to test the requirements with CCA. Proposal 12: RAN4 to discuss the list of test cases for RRM performance with CCA based on Table 3 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: General

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 3-1-1: FR2-2 performance scenarios

- Proposal 1 (Nokia): RAN4 to define RRM performance requirements for the following scenarios:

  - FR2-2 only standalone

  - FR2-2 non-contiguous CA

  - FR1+FR2-2 CA (FR1 is PCell)

  - FR1+FR2-2 DC (FR1 is PCell)

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Ericsson | Agree on proposal 1. |
| Nokia | As proponent, we agree with Proposal 1, the intention was to use the same scenarios for which we defined the core requirements. |
| Huawei | For the second bullet, it depends on whether to considerer NC CA. |
| CATT | Proposal 1 is ok for us. |
| Apple | FR2-2 NC CA has been removed from the scope. |
| Qualcomm | Agree with Huawei and Apple, FR2-2 non-contiguous CA can be removed from the scope. |

Issue 3-1-2: Performance requirements for FR2-2

- Proposal 1 (Nokia): Attempt to reuse FR2-1 RRM performance requirements for 120 kHz SCS that do not depend on the Rx Beam sweeping scaling factor.

- Proposal 2 (Nokia): If Rx beam sweeping scaling factor is changed in RRM core requirements, define new RRM performance requirements for 120 khz SCS considering the decision on the number of Rx Beam for RRM requirements.

- Proposal 3 (Nokia): RAN4 not to reuse FR2-1 RRM performance requirements for 120 kHz SCS with SSB using 240 kHz SCS.

- Proposal 4 (Nokia): RAN4 not to reuse FR2-1 RRM performance requirements for 120 kHz SCS for test cases whose RRM core requirements changed, for example due to new Rx beam sweeping scaling factor.

- Proposal 5 (CATT): It is recommended to discuss whether to define new test cases for those requirements in FR2-2 that reuse the current specification for FR2-1

  - If not to define new test cases for those requirements in FR2-2 that reuse the current specification for FR2-1, the description of frequency bands in the current FR2 test case should be modified to include FR2-2

- Proposal 6 (Nokia): For requirements that are the same for operation with CCA and without CCA, tests defined for operation without CCA can be reused to test the requirements with CCA.

- Recommended WF

  - Discuss the proposals individually

| Company | Comments |
| --- | --- |
| Ericsson | We agree on proposals generally. |
| Nokia | For this issue we have few proposals that attempt to reuse and reduce workload for the RRM performance requirements. We agree with Proposals 1, 2, 3, 4, 5 and 6 |
| Huawei | Generally fine with these proposal in principle. But we would like to invite companies to consider following questions: - Whether the test cases are defined for operation in FR2-2 or operation with new SCS (480,960)? If the purpose is the former one, the 120 KHz shall be considered with higher priority as it is mandatory capability. - Whether to have dedicated sections for test cases for FR2-2 or new test configurations in existing FR2 test cases? |
| Apple | Those high level principles are in general OK. Meanwhile, we may need to think hard how to minimize the number of test cases while ensuring good test coverage, and thus more time is needed. |
| Qualcomm | Generally fine with the proposals. |
| vivo | For the first question from Huawei, we prefer the former one, i.e., the test cases are defined for operation in FR2-2. For the second question from Huawei, we prefer to define the new sections for FR2-2 to distinguish the different frequency groups. However, considering the simpler method, we are also fine with only defining the new SCS configurations in the existing FR2 test case. |

Issue 3-1-3: Performance requirements for 480/960 kHz SCS

- Proposal 1 (Nokia): Define RRM performance requirements for 480 kHz and 960 kHz SCS.

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Ericsson | Agree on proposal 1. |
| Nokia | Agree with Proposal 1 We think it is important to have test cases for the new SCSs. |
| Huawei | As commented in issue 3-1-2, if the purpose of defining test cases is to verify operation in FR2-2, then performance requirements for 120KHz shall also be define even without change compared with that for legacy FR2. |
| CATT | Agree with proposal 1. We also believe it is necessary to define performance requirements for 120KHz. |
| Apple | Proposal 1 is OK |
| Qualcomm | Fine with proposal 1 |
| vivo | We share the same view with Huawei and CATT. The performance requirements for 120kHz for new frequency bands also need to be defined. |

Issue 3-1-4: Specification structure for test-cases

- Proposal 1 (Huawei): Define test cases for operation in FR2-2 in dedicated sections.

- Proposal 2 (Nokia): Create in TS 38.133 the following new top-level sections for NR-test cases with CCA in FR2-2

  - A.14 NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR2-2

  - A.15 NR Standalone Tests with NR Scell under CCA and All Other Cells in FR2-2

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Ericsson | Regarding proposal1, we suggest to follow approach in core part, using same chapter for FR2, add SCS and differentiate FR2-2 only when necessary. We agree on proposal 2, since CCA in FR1 is using top-level sections. |
| Nokia | Agree with Proposal 2. For Proposal 1, we are a bit unsure if we could reuse some of the test cases in A.7, and add additional test configurations in those. For example, if we don’t define new Rx beam sweeping scaling factor, many test cases from A.7 could be reused for FR2-2. @Huawei, do you think that would be possible as well? |
| Huawei | Fine with proposal 1 and proposal 2. For Ericsson’s comments, we are not sure whether the intention is to add new test configurations in legacy FR2 TC. To Nokia: Thanks for the further clarification. We think adding one configurations for 480KHz and 960KHz is in the test configuration in existing TC is one approach. But as comments in previous issues, it seems that for 120KHz, it cannot be differentiated in the test cases whether it is for FR2-1 or FR2-2. |
| CATT | We agree with proposal 2. For option 1, we have the same consideration as Ericsson, that is, add test configurations for FR2-2 in the same chapter only when necessary since it is a more concise method. |
| Nokia | To Huawei: what we thought could be done is to add a 120 kHz configuration in FR2-2, and specify that this configuration only applies for UE that declares support for FR2-2. Anyway, if we change the Rx Beam sweeping it will be very hard to reuse any 120 kHz test case, so it could be worth creating a new clause as in Proposal 1. |
| vivo | We are fine with proposal 1 and proposal 2. |

Issue 3-1-5: Baseline for test-cases

- Proposal 1 (Nokia): RAN4 to use as a baseline for the RRM performance without CCA the Rel 15 test cases.

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Ericsson | Agree on proposal 1. |
| Nokia | We agree with Proposal 1. We intend with that proposal to prioritize FR2-2 test cases for which there are already FR2-1 test cases in Rel 15. |
| CATT | Agree with proposal 1. |
| Qualcomm | Fine with the proposal |
| vivo | Fine with Proposal 1. |

Issue 3-1-6: UE transmit timing error test

- Proposal 1 (ZTE): The rate of UE meeting the Te requirement observed during repeated tests shall be at least 95%.

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Ericsson | We have not strong view, but we suggest to reuse 90% in timing advance adjustment test case. |
| Nokia | We are fine with proposal 1. We agree with ZTE argument. |
| ZTE | Proposal 1. |
| Huawei | Similar view as Ericsson. |
| Apple | Similar view as Huawei and Ericsson. Reuse 90% |
| Qualcomm | Agree with Ericsson. Reuse 90%. |
| vivo | Similar view as Ericsson. |

Issue 3-1-7: SSB pattern for 480/960 kHz SCS

- Proposal 1 (vivo): The new SSB pattern for 480kHz and 960kHz need to be introduced for FR2-2 in RRM test cases. The SSB configurations may be as follows:

| SSB pattern number | SSB SCS | Channel bandwidth | Number of SSB per SS-burst | Symbol numbers containing SSBs | SSB index | Slot number containing SSBs |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 480kHz | 400MHz | 2 | 2-5 9-12 | 0 1 | 0 0 |
| 2 | 960kHz | 400MHz | 2 | 2-5 9-12 | 0 1 | 0 0 |
| 3 | 480kHz | 400MHz | 1 | 2-5 | 0 | 0 |
| 4 | 960kHz | 400MHz | 1 | 2-5 | 0 | 0 |
| 5 | 480kHz | 400MHz | 2 | 2-5 9-12 | 2 3 | 1 1 |
| 6 | 960kHz | 400MHz | 2 | 2-5 9-12 | 2 3 | 1 1 |
| 7 | 480kHz | 400MHz | 1 | 9-12 | 1 | 0 |
| 8 | 960kHz | 400MHz | 1 | 9-12 | 1 | 0 |

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Ericsson | We’re fine with proposal1. |
| CATT | Ok with proposal 1. |
| vivo | Support Proposal 1. Based on the previous SSB pattern of FR2-1 and the SSB design from RAN1, we introduce the new SSB patterns for 480kHz and 960kHz. |

Issue 3-1-8: Modelling unavailable SSB/SMTC

- Proposal 1a (Huawei): For test cases where no particular behaviour to be verified, an unavailable SSB/SMTC group can be modelled as that there is one SSB not transmitted by TE in N consecutive SSB/SMTC occasions

- Proposal 1b (Huawei): If particular UE behaviour is to be verified when exceeding the limitation of unavailable SBS/SMTC groups, the unavailable SSB/SMTC group can be modelled as none of SSB is transmitted by TE

- Proposal 2 (Nokia): RAN4 to discuss whether the LBT model defined in NR-U needs to be adjusted to support the SMTC/SSB/DRX occasion group definition in FR2-2.

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Ericsson | We agree on proposal 1a. Regarding proposal1b, we suggest to mute one SSB transmitted by TE in N SSB/SMTC occasions also, but the index of the one SSB is changed, e.g. SSB#1 in occasion1, SSB#2 in occasion 2. We agree on proposal 2. |
| Nokia | We think that more discussion is needed. We believe that just one model should be defined, and this model should consider: SSBs of different indexes and SSBs transmitted in different candidate positions. Is the intention of proposal 1a that “… one SSB index not transmitted by TE in N consecutive SSB/SMTC occasions”? We agree with Proposal 1b and 2. |
| Huawei | The intention of proposal 1, for instance two SSB index SSB1 and SSB2 are configured. Then in the test cases, only one of the SSB within N SSB of SSB1 is not transmitted, then the delay is extended by 1 SSB group. If all SSB of SSB1 within N SSBs are muted, the then it is proposal 1b, that some particular UE behavior is to be verified. For other requirements, in most real scenarios, the LBT failure randomly happens like proposal 1a. |
| CATT | We agree with Proposal 1a and 2. In our opinion, proposal 1a is a good model in the most real scenarios. And whether to add “at least one SSB”? For proposal 1b, we understand some particular UE behavior in Huawei’s comment, but why model this scenario separately and not be incorporated in proposal 1a? From our understanding, the model in proposal 1a is also suitable for this particular scenario. |
| Apple | Both proposal 1a and 1b seem reasonable. |
| Qualcomm | Proposal 1 seems okay. We can further discuss issues identified (if any) with the models in the next meeting. |
| vivo | We slight prefer Proposal 1b. If the unavailable SSB/SMTC group can be modelled as the method from Proposal 1a, the UE may complete to measurement by other SSBs which are transmitted in N consecutive SSB/SMTC occasions. The LBT test may be not verified. So we suggest to test the worst case where none of SSB in N consecutive SSB/SMTC occasions is received by UE when designing the LBT model for FR2-2. |

### Sub-topic 3-2: Test-cases

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 3-2-1: Test-cases for RRM performance requirements without CCA

- Proposal 1 (Nokia, CATT): RAN4 to discuss the list of test cases for RRM performance without CCA based on Table below:

| Group of requirements | Test cases | Sub-test | 120 kHz SCS | 480/960 kHz SCS |
| --- | --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR2-2 -> FR2-2 | intra-frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | inter-frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | yes |
| HO (delay and interruptions) | FR2-2->FR2-2 | intra-frequency, unknown target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | inter-frequency, unknown target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  | FR1->FR2-2 | unknown target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| RRC Re-establishment | FR2-2->FR2-2 | intra frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | inter frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | intra frequency without serving cell timing | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| Random access | 4-step RACH | Contention based | Reuse FR2-1 | Yes |
|  |  | Non-contention based | Reuse FR2-1 | Yes |
| RRC Connection Release with Redirection | FR2-2->FR2-2 |  | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
| Timing | UE Transmit Timing |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes |
|  | Timing advance adjustment accuracy |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes |
| Active BWP switch | DCI-based and timer-based active BWP switch | FR2-2->FR2-2 | Reuse FR2-1 | Yes |
|  |  | FR1->FR2-2 | Reuse FR2-1 | Low priority |
|  |  | One FR2-2 cell | Reuse FR2-1 | Low priority |
|  | RRC-based active BWP switch |  | Reuse FR2-1 | Yes |
|  | SCell dormancy switch | FR2-2->FR2-2 | Reuse FR2-1 | Low priority |
|  |  | FR1->FR2-2 | Reuse FR2-1 | Low priority |
|  | RRC-based Active BWP Switch on multiple CCs |  | Reuse FR2-1 | Yes |
| Radio link monitoring | FR2-2 PCell, SSB-based | Out-of-sync, in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | In-sync, in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | Out-of-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | In-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | FR2-2 PCell, CSI-RS-based | Out-of-sync, in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | In-sync, in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | Out-of-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | In-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  | Scheduling restrictions |  | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| Beam failure detection and link recovery | FR2-2 PCell | SSB-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | SSB-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | CSI-RS-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | CSI-RS -based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | Scheduling restrictions | SSB-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| SCell activation/deactivation delay | SCell in FR2 | intra-band in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | inter-band in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  | FR1+FR2 inter-band with target SCell in FR2 |  | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| PSCell addition and release delay | Known PSCell |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | No |
|  | Unknown PSCell |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes |
| Active TCI state switching delay | MAC-CE based | PCell in FR2-2 | Reuse FR2-1 | Yes |
|  | RRC based | PCell in FR2-2 | Reuse FR2-1 | Low priority |
| Interruptions | during measurements on deactivated NR SCC in FR2-2 |  | Reuse FR2-1 | Yes |
| Intra-frequency measurement procedure | SA event triggered reporting test without gap | under non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | under DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  | SA event triggered reporting test with per-UE gap | under non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | under DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| Inter-frequency measurement procedure | SA event triggered reporting tests For FR2 without SSB time index detection | PCell in FR2-2, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | PCell in FR2-2, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | PCell in FR1, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | PCell in FR1, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | SA event triggered reporting tests For FR2 with SSB time index detection | PCell in FR2-2, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | PCell in FR2-2, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | PCell in FR1, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | PCell in FR1, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
| L1-RSRP measurement for beam reporting | SSB based | DRX not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | CSI-RS based | DRX not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
| Accuracy for measurements | SS-RSRP | Intra-frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | Inter frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | Inter frequency FR1 serving and FR2-2 target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | SS-RSRQ | Intra-frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | Inter frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | SS-SINR | Intra-frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | Inter frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  | L1-RSRP | SSB based | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | CSI-RS based | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |

- Recommended WF

  - Companies to check if any cases are to be added/dropped

  - Work split for the agreed test-cases would be discussed in the second round

| Company | Comments |
| --- | --- |
| XXX |  |
| Nokia | We agree with Proposal 1 |

Issue 3-2-2: Test-cases for RRM performance requirements with CCA

- Proposal 1 (Nokia, CATT): RAN4 to discuss the list of test cases for RRM performance with CCA based on Table below:

| Group of requirements | Test cases | Sub-test | Needed / Not needed |
| --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR2-2 -> FR2-2 (CCA) | intra-frequency | Yes, at least with 120 kHz and 480 kHz |
|  |  | inter-frequency | Yes, at least with 120 kHz and 480 kHz |
| HO (delay and interruptions) | FR2-2->FR2-2 (CCA) | intra-frequency, unknown target cell | Yes |
|  |  | inter-frequency, unknown target cell | Low priority |
|  | FR1->FR2-2 (CCA) | unknown target cell | Yes |
| RRC Re-establishment | FR2-2->FR2-2 (CCA) | intra frequency | Low priority |
|  |  | inter frequency | Yes |
|  |  | intra frequency without serving cell timing | Yes |
| Random access | 4-step RACH | Contention based | Yes |
|  |  | Non-contention based | Yes |
| RRC Connection Release with Redirection | FR2-2->FR2-2 (CCA) |  | Low priority |
| Timing | UE Transmit Timing |  | Yes |
|  | Timing advance adjustment accuracy |  | Yes |
| Active BWP switch | DCI-based and timer-based active BWP switch | FR2-2 ->FR2-2 (CCA) | Yes, but this test case can be same as the one defined for operation without CCA |
|  |  | FR1->FR2-2 (CCA) | Low priority |
|  |  | One FR2-2 (CCA) cell | Low priority |
|  | RRC-based active BWP switch |  | Yes, but this test case can be same as the one defined for operation without CCA |
|  | UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA | FR2-2 (CCA) | Yes |
|  |  | FR1->FR2-2 | Not needed |
| Radio link monitoring | FR2-2 Pcell (CCA), SSB-based | Out-of-sync, in non-DRX | Yes |
|  |  | In-sync, in non-DRX mode | Low priority |
|  |  | Out-of-sync, in DRX mode | Low priority |
|  |  | In-sync, in DRX mode | Yes |
|  | Scheduling restrictions |  | Yes, but this test case can be same as the one defined for operation without CCA |
| Beam failure detection and link recovery | FR2-2 PCell (CCA) | SSB-based in non-DRX mode | Yes |
|  |  | SSB-based in non-DRX mode | Low priority |
|  |  | CSI-RS-based in non-DRX mode | Low priority |
|  |  | CSI-RS -based in non-DRX mode | Yes |
|  | Scheduling restrictions | SSB-based in non-DRX mode | Yes, but this test case can be same as the one defined for operation without CCA |
| SCell activation/deactivation delay | SCell in FR2 (CCA) | intra-band in non-DRX | Yes |
|  |  | inter-band in non-DRX | Low priority |
|  | FR1+FR2 (CCA) inter-band with target SCell in FR2 |  | Yes |
| PSCell addition and release delay | Known PSCell |  | No |
|  | Unknown PSCell |  | Yes |
| Active TCI state switching delay | MAC-CE based | PCell in FR2-2 (CCA) | Yes |
|  | RRC based | PCell in FR2-2 (CCA) | Low priority |
| Interruptions | during measurements on deactivated NR SCC in FR2-2 |  | Yes |
| Intra-frequency measurement procedure | SA event triggered reporting test without gap | under non-DRX | Yes |
|  |  | under DRX | Low priority |
|  | SA event triggered reporting test with per-UE gap | under non-DRX | Low priority |
|  |  | under DRX | Yes |
| Inter-frequency measurement procedure | SA event triggered reporting tests For FR2 without SSB time index detection | PCell in FR2-2, DRX is not used | Yes |
|  |  | PCell in FR2-2, DRX is used | Low Priority |
|  |  | PCell in FR1, DRX is not used | Low Priority |
|  |  | PCell in FR1, DRX is used | Yes |
|  | SA event triggered reporting tests For FR2 with SSB time index detection | PCell in FR2-2, DRX is not used | Low Priority |
|  |  | PCell in FR2-2, DRX is used | Yes |
|  |  | PCell in FR1, DRX is not used | Yes |
|  |  | PCell in FR1, DRX is used | Low Priority |
| L1-RSRP measurement for beam reporting | SSB based | DRX not used | Low Priority |
|  |  | DRX is used | Yes |
| Accuracy for measurements | SS-RSRP | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  |  | Inter frequency FR1 serving and FR2-2 target cell | covered by the requirements without CCA |
|  | SS-RSRQ | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  | SS-SINR | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  | L1-RSRP | SSB based | covered by the requirements without CCA |
|  |  | CSI-RS based | covered by the requirements without CCA |
|  | RSSI | Intra-frequency | Needed in all SCS |
|  |  | Inter-frequency | Needed in all SCS |

- Recommended WF

  - Companies to check if any cases are to be added/dropped

  - Work split for the agreed test-cases would be discussed in the second round

| Company | Comments |
| --- | --- |
| XXX |  |
| Nokia | We agree with Proposal 1. |

### Sub-topic 3-3: Work-plan

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 3-3-1: Work-plan for RRM performance requirements

- Proposal 1 (Qualcomm): RAN4 to follow the following work-plan for RRM performance requirements specification:

  - RAN4 #103e (May 2022)

    - Discussion and approval of the work-plan

    - Way forward on general framework and test cases split

  - RAN4 #104 (Aug 2022)

    - CR endorsement and agreement

  - RAN4 #104bis-e (Oct 2022)

    - CR endorsement and agreement

  - RAN4 #105 (Nov 2022)

    - CR endorsement and agreement

    - Performance part completion

- Recommended WF

  - Companies to check if the above work-plan is agreeable.

  - Draft work-plan will be available in the inbox for review.

| Company | Comments |
| --- | --- |
| XXX |  |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 General | Issue 3-1-1: FR2-2 performance scenarios Tentative agreements: RAN4 to define RRM performance requirements for the following scenarios: - FR2-2 only standalone - FR1+FR2-2 CA (FR1 is PCell) - FR1+FR2-2 DC (FR1 is PCell) Candidate options: Recommendations for 2nd round: No further discussion in 2nd round. Issue 3-1-2: Performance requirements for FR2-2 Tentative agreements: - Performance requirements for 120kHz SCS in FR2-2 - Attempt to reuse FR2-1 RRM performance requirements for 120 kHz SCS that do not depend on the Rx Beam sweeping scaling factor. - If Rx beam sweeping scaling factor is changed in RRM core requirements, define new RRM performance requirements for 120 kHz SCS considering the decision on the number of Rx Beam for RRM requirements. - RAN4 not to reuse FR2-1 RRM performance requirements for 120 kHz SCS with SSB using 240 kHz SCS. - Define RRM performance requirements for 480 kHz and 960 kHz SCS - For requirements that are the same for operation with CCA and without CCA, tests defined for operation without CCA can be reused to test the requirements with CCA Candidate options: Recommendations for 2nd round: Companies to confirm the tentative agreement. Issue 3-1-3: Performance requirements for 480/960 kHz SCS Moderator’s note: Tentative agreement captured under second bullet of Issue 3-1-2. Recommendations for 2nd round: No further discussion needed in 2nd round. Issue 3-1-4: Specification structure for test-cases Tentative agreements: - Proposal 1 (Huawei): Define test cases for operation in FR2-2 in dedicated sections. - Create in TS 38.133 the following new top-level sections for NR-test cases with CCA in FR2-2 - A.14 NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR2-2 - A.15 NR Standalone Tests with NR Scell under CCA and All Other Cells in FR2-2 Candidate options: Recommendations for 2nd round: Further discuss 2nd round, how to capture the non CCA related test cases for FR2-2 Issue 3-1-5: Baseline for test-cases Tentative agreements: - RAN4 to use as a baseline for the RRM performance without CCA the Rel-15 test cases. - Note: Whether certain Rel-15 test cases are applicable for FR2-2 or not can be further discussed. Candidate options: Recommendations for 2nd round: Since the proposal is very broad, a note has been added to keep the discussion open. Companies to check if the tentative agreement is acceptable. Issue 3-1-6: UE transmit timing error test Tentative agreements: - The rate of UE meeting the Te requirement observed during repeated tests shall be at least 90%. Candidate options: Recommendations for 2nd round: No further discussion needed in the second round. Issue 3-1-7: SSB pattern for 480/960 kHz SCS Tentative agreements: - The following SSB configuration to be used in RRM test cases for the new SSB pattern introduced for 480kHz and 960kHz SCS in FR2-2 band. SSB pattern number; SSB SCS; Channel bandwidth; Number of SSB per SS-burst; Symbol numbers containing SSBs; SSB index; Slot number containing SSBs. 1; 480kHz; 400MHz; 2; 2-5 9-12; 0 1; 0 0. 2; 960kHz; 400MHz; 2; 2-5 9-12; 0 1; 0 0. 3; 480kHz; 400MHz; 1; 2-5; 0; 0. 4; 960kHz; 400MHz; 1; 2-5; 0; 0. 5; 480kHz; 400MHz; 2; 2-5 9-12; 2 3; 1 1. 6; 960kHz; 400MHz; 2; 2-5 9-12; 2 3; 1 1. 7; 480kHz; 400MHz; 1; 9-12; 1; 0. 8; 960kHz; 400MHz; 1; 9-12; 1; 0. Candidate options: Recommendations for 2nd round: Companies to confirm the tentative agreement. Issue 3-1-8: Modelling unavailable SSB/SMTC Tentative agreements: Candidate options: - Proposal 1a: For test cases where no particular behaviour to be verified, an unavailable SSB/SMTC group can be modelled as that there is one SSB not transmitted by TE in N consecutive SSB/SMTC occasions - Proposal 1b: If particular UE behaviour is to be verified when exceeding the limitation of unavailable SBS/SMTC groups, the unavailable SSB/SMTC group can be modelled as none of SSB is transmitted by TE Recommendations for 2nd round: Continue discussion on proposal 1a and 1b in the 2nd round |
| Sub-topic #3-2: Test cases | Issue 3-2-1: Test-cases for RRM performance requirements without CCA Issue 3-2-2: Test-cases for RRM performance requirements with CCA Tentative agreements: No agreement Candidate options: Recommendations for 2nd round: Continue discussion on Issue 3-2-1 and 3-2-2 in the 2nd round |
| Sub-topic#3-3 Work-plan | Issue 3-3-1: Work-plan for RRM performance requirements Tentative agreements: Candidate options: Recommendations for 2nd round: Continue discussion in the second round. |

## Discussion on 2nd round

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### Sub-topic 3-1: General

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 3-1-4: Specification structure for test-cases

- Recommended WF

  - Further discuss 2nd round, how to capture the non CCA related test cases for FR2-2

| Company | Comments |
| --- | --- |
| Ericsson | Add bottom directory for non CCA related test cases for FR2-2 in existing top-level sections of FR2. |
| Nokia | Considering that either NRxBeam or the measurement requirements will have a change, it will be very hard to reuse FR2-1 performance requirements for 120 kHz. Given that, we are fine with a new session for non-CCA related test cases for FR2-2. |
| vivo | We prefer to define the new sections for non CCA related test cases for FR2-2 to distinguish the difference between FR2-1 and FR2-2, including the Rx beam sweeping factor, the frequency groups, the SSB configuration and the SCS configuration. Meanwhile, we understand this is more readable. |
| CATT | It is related to the Rx beam sweeping scaling factor. If a new beam scaling factor is introduced, we prefer capture the non CCA related test cases for FR2-2 in the new sections. Otherwise, we consider to add test configurations for FR2-2 in the existing sections only when necessary since it is a more concise method. |
| Huawei | Same views as Nokia and Vivo. |

Issue 3-1-8: Modelling unavailable SSB/SMTC

- Proposal 1a: For test cases where no particular behaviour to be verified, an unavailable SSB/SMTC group can be modelled as that there is one SSB not transmitted by TE in N consecutive SSB/SMTC occasions

- Proposal 1b: If particular UE behaviour is to be verified when exceeding the limitation of unavailable SBS/SMTC groups, the unavailable SSB/SMTC group can be modelled as none of SSB is transmitted by TE

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Ericsson | Agree on Proposal 1a. Regarding issue which Proposal 1b targets to, quoting comment in 1st round‘. If all SSB of SSB1 within N SSBs are muted, the then it is proposal 1b, that some particular UE behavior is to be verified.’, if UE fails to detect all SSB of SSB1, the failure occurs with LBT and non-LBT both. In other words, LBT behavior cannot be verfied. |
| Nokia | We think this can be further evaluated. For example, in proposal 1a, what does one SSB mean? One SSB index, or a candidate position? We think that the CCA model can follow the general lines of the existing CCA model, but adding the concept of the beam sweeping group that is being discussed as part of the core requirements. |
| vivo | We agree with Proposal 1b. As commented in the first round, if the unavailable SSB/SMTC group can be modelled as the method from Proposal 1a, the UE may complete to measurement by other SSBs which are transmitted in N consecutive SSB/SMTC occasions. The LBT test may be not verified. So it may be reasonable for test the worst case where none of SSB in N consecutive SSB/SMTC occasions is received by UE. Then the LBT behavior must be verified. |
| CATT | For proposal 1a, we consider it may mean ‘one SSB candidate position’? It is ok for us. For proposal 1b, we are not sure Huawei’s comment has the same meaning with vivo’s comment in the first round. From our understanding, ‘all SSB of SSB1 within N SSBs are muted’ commented by Huawei is different with ‘none of SSB in N consecutive SSB/SMTC occasions’ commented by vivo. Did Huawei mean that all SSB occasions with the same index which are in different N consecutive SSB/SMTC occasions ? And did vivo mean that all SSB occasions with different index which are in the same N consecutive SSB/SMTC occasions ? We’d like Huawei and vivo to provide the further clarify about proposal 1b. |
| Huawei | To Ericsson and CATT: Particular UE behaviour means the action triggered by consecutive unavailable SSB defined in NR-U. For instance, Report RSRP_0 upon L1 exceeding L1,max for L1-RSRP measurement. Due to beam sweeping, the LBT failure may not happen in the main beam, and UE may not able to trigger the behaviour. To Nokia, one SSB means one SSB index in one SSB burst. For instance, in N consecutive SSB bursts, SSB#1 (two candidate positions) in one of the SSB burst are not available. |
| Apple | Both proposals seem reasonable. Perhaps we don’t need to formally endorse them at this meeting and companies have more time to think before the next meeting. |

### Sub-topic 3-2: Test-cases

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 3-2-1: Test-cases for RRM performance requirements without CCA

- Proposal 1 (Nokia, CATT): RAN4 to discuss the list of test cases for RRM performance without CCA based on Table below:

| Group of requirements | Test cases | Sub-test | 120 kHz SCS | 480/960 kHz SCS |
| --- | --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR2-2 -> FR2-2 | intra-frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | inter-frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | yes |
| HO (delay and interruptions) | FR2-2->FR2-2 | intra-frequency, unknown target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | inter-frequency, unknown target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  | FR1->FR2-2 | unknown target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| RRC Re-establishment | FR2-2->FR2-2 | intra frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | inter frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | intra frequency without serving cell timing | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| Random access | 4-step RACH | Contention based | Reuse FR2-1 | Yes |
|  |  | Non-contention based | Reuse FR2-1 | Yes |
| RRC Connection Release with Redirection | FR2-2->FR2-2 |  | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
| Timing | UE Transmit Timing |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes |
|  | Timing advance adjustment accuracy |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes |
| Active BWP switch | DCI-based and timer-based active BWP switch | FR2-2->FR2-2 | Reuse FR2-1 | Yes |
|  |  | FR1->FR2-2 | Reuse FR2-1 | Low priority |
|  |  | One FR2-2 cell | Reuse FR2-1 | Low priority |
|  | RRC-based active BWP switch |  | Reuse FR2-1 | Yes |
|  | SCell dormancy switch | FR2-2->FR2-2 | Reuse FR2-1 | Low priority |
|  |  | FR1->FR2-2 | Reuse FR2-1 | Low priority |
|  | RRC-based Active BWP Switch on multiple CCs |  | Reuse FR2-1 | Yes |
| Radio link monitoring | FR2-2 PCell, SSB-based | Out-of-sync, in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | In-sync, in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | Out-of-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | In-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | FR2-2 PCell, CSI-RS-based | Out-of-sync, in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | In-sync, in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | Out-of-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | In-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  | Scheduling restrictions |  | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| Beam failure detection and link recovery | FR2-2 PCell | SSB-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | SSB-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | CSI-RS-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | CSI-RS -based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | Scheduling restrictions | SSB-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| SCell activation/deactivation delay | SCell in FR2 | intra-band in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | inter-band in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  | FR1+FR2 inter-band with target SCell in FR2 |  | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| PSCell addition and release delay | Known PSCell |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | No |
|  | Unknown PSCell |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes |
| Active TCI state switching delay | MAC-CE based | PCell in FR2-2 | Reuse FR2-1 | Yes |
|  | RRC based | PCell in FR2-2 | Reuse FR2-1 | Low priority |
| Interruptions | during measurements on deactivated NR SCC in FR2-2 |  | Reuse FR2-1 | Yes |
| Intra-frequency measurement procedure | SA event triggered reporting test without gap | under non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | under DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  | SA event triggered reporting test with per-UE gap | under non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |
|  |  | under DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
| Inter-frequency measurement procedure | SA event triggered reporting tests For FR2 without SSB time index detection | PCell in FR2-2, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | PCell in FR2-2, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | PCell in FR1, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | PCell in FR1, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | SA event triggered reporting tests For FR2 with SSB time index detection | PCell in FR2-2, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | PCell in FR2-2, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | PCell in FR1, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | PCell in FR1, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
| L1-RSRP measurement for beam reporting | SSB based | DRX not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | CSI-RS based | DRX not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
| Accuracy for measurements | SS-RSRP | Intra-frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | Inter frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | Inter frequency FR1 serving and FR2-2 target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | SS-RSRQ | Intra-frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  |  | Inter frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  | SS-SINR | Intra-frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | Inter frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |
|  | L1-RSRP | SSB based | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |
|  |  | CSI-RS based | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |

- Recommended WF

  - Companies to check if any cases are to be added/dropped

  - Work split for the agreed test-cases would be discussed in the second round

| Company | Comments |
| --- | --- |
| Ericsson | We can take reselection and HO upon work split. |
| Nokia | We think considering the status of the discussion on Subtopic 1-1 and 1-3 that 120 cant be reused for the cases indicated as “Can be reused from FR2-1 if RX beam sweeping is not changed” Additionally, we volunteer for - Timing requirements - UE transmit timing - Random access - Contention based |
| vivo | We would like to volunteer for the intra-frequency measurement procedure and accuracy requirements for SS-RSRP. In addition, as mentioned in the issue 3-1-7, the SSB patten for 480kHz and 960kHz need to be defined in the spec. We can also take this part’s CR. |
| Huawei | We would like to volunteer for RLM and Beam failure detection and link recovery. |

Issue 3-2-2: Test-cases for RRM performance requirements with CCA

- Proposal 1 (Nokia, CATT): RAN4 to discuss the list of test cases for RRM performance with CCA based on Table below:

| Group of requirements | Test cases | Sub-test | Needed / Not needed |
| --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR2-2 -> FR2-2 (CCA) | intra-frequency | Yes, at least with 120 kHz and 480 kHz |
|  |  | inter-frequency | Yes, at least with 120 kHz and 480 kHz |
| HO (delay and interruptions) | FR2-2->FR2-2 (CCA) | intra-frequency, unknown target cell | Yes |
|  |  | inter-frequency, unknown target cell | Low priority |
|  | FR1->FR2-2 (CCA) | unknown target cell | Yes |
| RRC Re-establishment | FR2-2->FR2-2 (CCA) | intra frequency | Low priority |
|  |  | inter frequency | Yes |
|  |  | intra frequency without serving cell timing | Yes |
| Random access | 4-step RACH | Contention based | Yes |
|  |  | Non-contention based | Yes |
| RRC Connection Release with Redirection | FR2-2->FR2-2 (CCA) |  | Low priority |
| Timing | UE Transmit Timing |  | Yes |
|  | Timing advance adjustment accuracy |  | Yes |
| Active BWP switch | DCI-based and timer-based active BWP switch | FR2-2 ->FR2-2 (CCA) | Yes, but this test case can be same as the one defined for operation without CCA |
|  |  | FR1->FR2-2 (CCA) | Low priority |
|  |  | One FR2-2 (CCA) cell | Low priority |
|  | RRC-based active BWP switch |  | Yes, but this test case can be same as the one defined for operation without CCA |
|  | UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA | FR2-2 (CCA) | Yes |
|  |  | FR1->FR2-2 | Not needed |
| Radio link monitoring | FR2-2 Pcell (CCA), SSB-based | Out-of-sync, in non-DRX | Yes |
|  |  | In-sync, in non-DRX mode | Low priority |
|  |  | Out-of-sync, in DRX mode | Low priority |
|  |  | In-sync, in DRX mode | Yes |
|  | Scheduling restrictions |  | Yes, but this test case can be same as the one defined for operation without CCA |
| Beam failure detection and link recovery | FR2-2 PCell (CCA) | SSB-based in non-DRX mode | Yes |
|  |  | SSB-based in non-DRX mode | Low priority |
|  |  | CSI-RS-based in non-DRX mode | Low priority |
|  |  | CSI-RS -based in non-DRX mode | Yes |
|  | Scheduling restrictions | SSB-based in non-DRX mode | Yes, but this test case can be same as the one defined for operation without CCA |
| SCell activation/deactivation delay | SCell in FR2 (CCA) | intra-band in non-DRX | Yes |
|  |  | inter-band in non-DRX | Low priority |
|  | FR1+FR2 (CCA) inter-band with target SCell in FR2 |  | Yes |
| PSCell addition and release delay | Known PSCell |  | No |
|  | Unknown PSCell |  | Yes |
| Active TCI state switching delay | MAC-CE based | PCell in FR2-2 (CCA) | Yes |
|  | RRC based | PCell in FR2-2 (CCA) | Low priority |
| Interruptions | during measurements on deactivated NR SCC in FR2-2 |  | Yes |
| Intra-frequency measurement procedure | SA event triggered reporting test without gap | under non-DRX | Yes |
|  |  | under DRX | Low priority |
|  | SA event triggered reporting test with per-UE gap | under non-DRX | Low priority |
|  |  | under DRX | Yes |
| Inter-frequency measurement procedure | SA event triggered reporting tests For FR2 without SSB time index detection | PCell in FR2-2, DRX is not used | Yes |
|  |  | PCell in FR2-2, DRX is used | Low Priority |
|  |  | PCell in FR1, DRX is not used | Low Priority |
|  |  | PCell in FR1, DRX is used | Yes |
|  | SA event triggered reporting tests For FR2 with SSB time index detection | PCell in FR2-2, DRX is not used | Low Priority |
|  |  | PCell in FR2-2, DRX is used | Yes |
|  |  | PCell in FR1, DRX is not used | Yes |
|  |  | PCell in FR1, DRX is used | Low Priority |
| L1-RSRP measurement for beam reporting | SSB based | DRX not used | Low Priority |
|  |  | DRX is used | Yes |
| Accuracy for measurements | SS-RSRP | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  |  | Inter frequency FR1 serving and FR2-2 target cell | covered by the requirements without CCA |
|  | SS-RSRQ | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  | SS-SINR | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |
|  | L1-RSRP | SSB based | covered by the requirements without CCA |
|  |  | CSI-RS based | covered by the requirements without CCA |
|  | RSSI | Intra-frequency | Needed in all SCS |
|  |  | Inter-frequency | Needed in all SCS |

- Recommended WF

  - Companies to check if any cases are to be added/dropped

  - Work split for the agreed test-cases would be discussed in the second round

| Company | Comments |
| --- | --- |
| Ericsson | We can take reselection and HO upon work split. |
| Nokia | We think a row for the CCA model should be included as well. We volunteer for - Timing requirements - UE transmit timing - Random access - Contention based - CCA model |
| vivo | We would like to volunteer the intra-frequency measurement procedure and accuracy requirements for SS-RSRP. In addition, the SSB patten under CCA for FR2-2 need to be defined in the spec. We can also take this part’s CR. |
| Huawei | We would like to volunteer for RLM and Beam failure detection and link recovery. |

### Sub-topic 3-3: Work-plan

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 3-3-1: Work-plan for RRM performance requirements

- Proposal 1 (Qualcomm): RAN4 to follow the following work-plan for RRM performance requirements specification:

  - RAN4 #103e (May 2022)

    - Discussion and approval of the work-plan

    - Way forward on general framework and test cases split

  - RAN4 #104 (Aug 2022)

    - CR endorsement and agreement

  - RAN4 #104bis-e (Oct 2022)

    - CR endorsement and agreement

  - RAN4 #105 (Nov 2022)

    - CR endorsement and agreement

    - Performance part completion

- Recommended WF

  - Companies to check if the above work-plan is agreeable.

| Company | Comments |
| --- | --- |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on NR extension to 71 GHz RRM requirements (Part 1) | Qualcomm | WF to capture the agreements and open issues |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2208811 |  | Draft CR - Introduction of scheduling restriction requirements in FR2-2 | Vivo | Agreeable |  |
| R4-2209050 |  | Draft CR to TS 38.133 Corrections on scheduling availability in FR2-2 | Nokia | Agreeable |  |
| R4-2208812 |  | Draft CR – Introduction of deriveSSB-IndexFromCell tolerance requirements in FR2-2 | Vivo | Agreeable |  |
| R4-2209386 |  | Draft CR adding timing requirements for FR2-2 | Nokia | To be revised |  |
| R4-2207782 |  | Draft CR on UE transmit timing for NR operation in 52.6GHz – 71GHz | Apple | To be revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2209386 | R4-2211031 | Draft CR adding timing requirements for FR2-2 | Nokia | Agreeable |  |
| R4-2207782 | R4-2211032 | Draft CR on UE transmit timing for NR operation in 52.6GHz – 71GHz | Apple | Not pursued |  |
| R4-2210590 |  | WF on NR extension to 71 GHz RRM requirements (Part 1) | Qualcomm | Agreeable | Capture the new agreements |
| R4-2210224 |  | FR2-2 RRM Performance Work Plan | Qualcomm | Agreeable | Treated in 2nd round |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Nokia | Rafael Paiva | Rafael.paiva@nokia.com |
| MediaTek | Hsuanli Lin | Hsuanli.Lin@mediatek |
| Qualcomm | Prashant Sharma | prasshar@qti.qualcomm.com |
| Ericsson | Magnus Larsson | Magnus.k.larsson@ericsson.com |
| AT&T | Ron Borsato | ronald.borsato@att.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)