3GPP TSG-RAN WG4 Meeting # 103-e												R4-2210522

Electronic Meeting, 09 – 20 May 2022

Agenda item:			9.8.4.1

Source:	Moderator (Samsung)

Title:	Email discussion summary for [103-e][319] NR_HST_FR2_Demod_Part1

Document for:	Information

# Introduction

In RAN Plenary #89-e, the RAN4-led work item of NR support for high speed train (HST) scenario in FR2 has been approved [RP-202118] (which has been further revised to [RP-210800] with editorial revisions and updates on time schedule).

Based on the agreement captured in WF [R4-2203093], the test setup of UE demodulation was under discussion. For this meeting, companies are encouraged to further discuss the remaining issue for UE demodulation test set setup based on the FR2 HST deployment scenarios

In this email thread, the following agenda items will be discussed:

- 9.8.4.1 UE demodulation requirements

- 9.8.4.1.1 PDSCH requirements under Uni-directional scenario

- 9.8.4.1.2 PDSCH requirements under Bi-directional scenario

It is suggested to have the following target of 1st and 2nd round email discussion

- 1st round: Further discussion the remaining issue for UE demodulation test setup

- 2nd round: draft CR discussion and revised

# Topic #1: PDSCH requirement

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2208074 | Samsung | Simulation results summary for UE demodulation |
| R4-2208076 | Samsung | Simulation results |
| R4-2208079 | Samsung | Big CR for 38.101-4 |
| R4-2209071 | Ericsson | Proposal 1: Specify Noc power level for PC6 in TS38.101-4 Table 4.5.3.2-1 as follows: Operating bands; n257; n258; n261. Noc level (dBm/Hz); -162.4; -162.6; -162.4. |
| R4-2209072 | Ericsson | CR on FRC for PDSCH demodulation requirement for FR2 HST |
| R4-2209862 | Huawei | CR on minimum requirements for FR2 PDSCH HST-DPS requirement |
| R4-2209863 | Huawei | For the test setup for PDSCH allocation timeline for Uni-directional scenario, the agreement achieved in last meeting should be modified and then the square brackets can be removed, i.e. - Step 1: Two RRHs of RRH#(2k), RRH#(2k+1) are assumed, and SSB#(2k mod 4) and SSB#((2k+1 )mod 4) are transmitted for each TRPs, separately, where k is the RRH number with k =0, 1, 2, …. - UE is configured with TCI#(2k mod 4) and TCI #((2k+1)mod 4) that are associated with TRS #(2k mod 4) and TRS#((2k+1)mod 4) transmitted from RRH#(2k) and RRH#(2k+1) respectively by RRC signalling tci-StatesToAddModList in the PDSCH-Config and tci-PresentInDCI is not configured; - All the configured TCI states are known to UE. UE is configured with NZP-CSI-RS resource for L1-RSRP measurements by RRC signalling nzp-CSI-RS-ResourceSet within the CSI-ResourceConfig and periodic CSI reporting by setting reportConfigType to periodic and reportQuantity to cri-RSRP (Note: reported L1-RSRP mesurements are not tested) - Step 2: TE actives TCI #0 for PDCCH by “TCI State Indication for UE-specific PDCCH MAC CE”; - Step 3: PDSCH associated with TCI #0 is transmitted during the slots from 0 to n+ THARQ + TMAC - Step4 : In slot n TE start triggering TCI state switching command to TCI #1 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4”; - Step 5: PDSCH associated with TCI #1 is transmitted in slots from n+2 + THARQ + TMAC to N. - PDSCH associated with TCI#(k mod 4) (k=1) is transmitted in slot from 0 to n + THARQ + TMAC - PDSCH associated with TCI #(k mod 4) (k=2, 3,…) is transmitted in slot from (k-1)*n+2 + THARQ + TMAC to (k*n+ THARQ + TMAC, where n =57600 is the number of slots between the location of (k-1)Ds- DS_offset and the location of (k)⋅DS-DS_offset. And k is the RRH number in the channel model. - PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered |
| R4-2209864 | Huawei | - For the test setup for PDSCH allocation timeline for Bi-directional scenario, the agreement achieved in last meeting should be confirmed and the square brackets should be removed directly, i.e. - Step 1: Three RRHs of RRH#(k-1), RRH#(k), RRH#(k+1) are assumed, and SSB#((2(k-1)+l)mod8), SSB#((2k+l)mod8), and SSB#((2(k+1)+l)mod8) are transmitted from each TRPs, separately, where k is the RRH number with k=1,2,3,…, l is the SSB index with l=0,1 - UE is configured with TCI#((2(k-1)+1) mod 8) (l=0,1) , TCI #((2k+1) mod 8) (l=0,1) and TCI#(((2k+1)+1)mod 8) (l=0,1) transmitted from RRH#(k-1), RRH#(k) and RRH#(k+1) respectively by RRC signalling tci-StatesToAddModList in the PDSCH-Config and tci-PresentInDCI is not configured; - All the configured TCI states are known to UE. UE is configured with NZP-CSI-RS resource for L1-RSRP measurements by RRC signalling nzp-CSI-RS-ResourceSet within the CSI-ResourceConfig and periodic CSI reporting by setting reportConfigType to periodic and reportQuantity to cri-RSRP (Note: reported L1-RSRP measurements are not tested) - Step 2: TE actives TCI #2 for PDCCH by “TCI State Indication for UE-specific PDCCH MAC CE”; - Step 3: PDSCH associated with TCI #2 is transmitted during the slots from 0 to n + THARQ + TMAC proc ; - Step 4: In slot n TE start triggering TCI state switching command to TCI #1 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4”; - Step 5: PDSCH associated with TCI #1 is transmitted in slots from n+1 + THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc to 2n+ THARQ + TMAC proc - Step 6: In slot 2n TE start triggering TCI state switching command to TCI# 4 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4” - Step 7: PDSCH associated with TCI #4 is transmitted in slots from 2n+1 + THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TSSB proc to 3n+THARQ + TMAC proc ; - PDSCH associated with TCI#(2k mod 8) (k=1) is transmitted in slot from 0 to n+ THARQ + TMAC proc - PDSCH associated with TCI #(2k mod 8) (k=2,3, …) is transmitted in slot from (2k-2)n +1 + THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc to (2k-1)n + THARQ + TMAC proc - PDSCH associated with TCI #((2k+1)mod 8) (k=0,1,2,…) is transmitted in slot from (2k+1)n +1+ THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc to (2(k+1)n+ THARQ + TMAC proc) where n =28800 slots is the half of the number of slots between two RRHs. - PDCCH and PDSCH are DTXed in other slots in which throughput statistic are not considered |
| R4-2210083 | Qualcomm | CR FR2 HST channel model |

## Open issues summary

### Sub-topic 1-1: Common setup

Issue 1-1-1: Noc power level

- Proposals

  - Option 1(Ericsson):

| Operating bands | n257 | n258 | n261 |
| --- | --- | --- | --- |
| Noc level (dBm/Hz) | -162.4 | -162.6 | -162.4 |

- Recommended WF

  - Option 1?

### Sub-topic 1-2: PDSCH requirement for Uni-directional scenario

Issue 1-2-1: Test setup for PDSCH allocation timeline for Uni-directional scenario

- Proposals

  - Option 1(Huawei): For the test setup for PDSCH allocation timeline for Uni-directional scenario, the agreement achieved in last meeting should be modified and then the square brackets can be removed, i.e.

    - Step 1: Two RRHs of RRH#(2k), RRH#(2k+1) are assumed, and SSB#(2k mod 4) and SSB#((2k+1 )mod 4) are transmitted for each TRPs, separately, where k is the RRH number with k =0, 1, 2, ….

- UE is configured with TCI#(2k mod 4) and TCI #((2k+1)mod 4) that are associated with TRS #(2k mod 4) and TRS#((2k+1)mod 4) transmitted from RRH#(2k) and RRH#(2k+1) respectively by RRC signalling tci-StatesToAddModList in the PDSCH-Config and tci-PresentInDCI is not configured;

- All the configured TCI states are known to UE. UE is configured with NZP-CSI-RS resource for L1-RSRP measurements by RRC signalling nzp-CSI-RS-ResourceSet within the CSI-ResourceConfig and periodic CSI reporting by setting reportConfigType to periodic and reportQuantity to cri-RSRP (Note: reported L1-RSRP mesurements are not tested)

    - Step 2: TE actives TCI #0 for PDCCH by “TCI State Indication for UE-specific PDCCH MAC CE”;

    - Step 3: PDSCH associated with TCI #0 is transmitted during the slots from 0 to n+ THARQ + TMAC

    - Step4: In slot n TE start triggering TCI state switching command to TCI #1 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4”;

    - Step 5: PDSCH associated with TCI #1 is transmitted in slots from n+2 + THARQ + TMAC to N.

    - PDSCH associated with TCI#(k mod 4) (k=1) is transmitted in slot from 0 to n + THARQ + TMAC

    - PDSCH associated with TCI #(k mod 4) (k=2, 3,…) is transmitted in slot from (k-1)*n+2 + THARQ + TMAC  to (k*n+ THARQ + TMAC, where n =57600 is the number of slots between the location of (k-1)Ds- DS_offset and the location of (k)⋅DS-DS_offset. And k is the RRH number in the channel model.

    - PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered

- Recommended WF

  - Option 1?

Issue 1-2-2: Requirement for Uni-directional scenario

- Proposals

  - Option 1: 	SNR=[13.7]dB

- Recommended WF

  - Simulation results summary in R4-2208074

| Samsung | Qualcomm | Huawei | Ericsson | Intel | Average | Span | STD | Extra Margin | 38101-4 req |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 |  | 12.9 | 12.5 | 13.3 | 12.925 | 0.8 | 0.286138 | 0.8 | 13.7 |
| 13.7 |  | 12.9 | 12.5 | 13.3 | 13.1 | 1.2 | 0.447214 | 0.8 | 13.9 |

### Sub-topic 1-3 PDSCH requirement for Bi-directional

Issue 1-3-1: Test setup for PDSCH allocation timeline for Bi-directional scenario

- Proposals

  - Option 1(Huawei): For the test setup for PDSCH allocation timeline for Bi-directional scenario, the agreement achieved in last meeting should be confirmed and the square brackets should be removed directly, i.e

    - Step 1: Two RRHs of RRH#(2k), RRH#(2k+1) are assumed, and SSB#(2k mod 4) and SSB#((2k+1 )mod 4) are transmitted for each TRPs, separately, where k is the RRH number with k =0, 1, 2, ….

- UE is configured with TCI#(2k mod 4) and TCI #((2k+1)mod 4) that are associated with TRS #(2k mod 4) and TRS#((2k+1)mod 4) transmitted from RRH#(2k) and RRH#(2k+1) respectively by RRC signalling tci-StatesToAddModList in the PDSCH-Config and tci-PresentInDCI is not configured;

- All the configured TCI states are known to UE. UE is configured with NZP-CSI-RS resource for L1-RSRP measurements by RRC signalling nzp-CSI-RS-ResourceSet within the CSI-ResourceConfig and periodic CSI reporting by setting reportConfigType to periodic and reportQuantity to cri-RSRP (Note: reported L1-RSRP measurements are not tested)

    - Step 2: TE actives TCI #2 for PDCCH by “TCI State Indication for UE-specific PDCCH MAC CE”;

    - Step 3: PDSCH associated with TCI #2 is transmitted during the slots from 0 to n + THARQ + TMAC proc;

    - Step 4: In slot n TE start triggering TCI state switching command to TCI #1 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4”;

    - Step 5: PDSCH associated with TCI #1 is transmitted in slots from n+1 + THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc to 2n+ THARQ + TMAC proc

    - Step 6: In slot 2n TE start triggering TCI state switching command to TCI# 4 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4”

    - Step 7: PDSCH associated with TCI #4 is transmitted in slots from 2n+1 + THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TSSB proc to 3n+THARQ + TMAC proc;

    - PDSCH associated with TCI#(2k mod 8) (k=1) is transmitted in slot from 0 to n+ THARQ + TMAC proc

    - PDSCH associated with TCI #(2k mod 8) (k=2,3, …) is transmitted in slot from (2k-2)n +1 + THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc to (2k-1)n + THARQ + TMAC proc

    - PDSCH associated with TCI #((2k+1)mod 8) (k=0,1,2,…) is transmitted in slot from (2k+1)n +1+ THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc to (2(k+1)n+ THARQ + TMAC proc) where n =28800 slots is the half of the number of slots between two RRHs.

    - PDCCH and PDSCH are DTXed in other slots in which throughput statistic are not considered

- Recommended WF

  - Option 1?

Issue 1-3-2: Requirement for Bi-directional scenario

- Proposals

  - Option 1: [13.9] dB

- Recommended WF

  - Simulation results summary in R4-2208074

| Samsung | Qualcomm | Huawei | Ericsson | Intel | Average | Span | STD | Extra Margin | 38101-4 req |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 |  | 12.9 | 12.5 | 13.3 | 12.925 | 0.8 | 0.286138 | 0.8 | 13.7 |
| 13.7 |  | 12.9 | 12.5 | 13.3 | 13.1 | 1.2 | 0.447214 | 0.8 | 13.9 |

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1 |
| Ericsson | Issue 1-1-1: Support Option 1. |
| Huawei | Issue 1-1-1: Noc power level We are OK with the recommended WF. |
| Samsung | Issue 1-1-1: Support Option 1. |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2-1 Issue 1-2-2 |
| Ericsson | Issue 1-2-1 Support Option 1 Issue 1-2-2: Support the recommended WF. |
| Huawei | Issue 1-2-1: Test setup for PDSCH allocation timeline for Uni-directional scenario We are OK with the recommended WF. Issue 1-2-2: Requirement for Uni-directional scenario We are OK with the recommended WF. |
| QC | Issue 1-2-1: The TCI state switch time doesn’t align to RRM TCI state switch delay for uni-directional deployment: UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the symbol m of the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ + TOk*(Tfirst-SSB + TSSB-proc + Trs + Trs-proc) / NR slot length. The UE shall be able to receive PDCCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ . - TOk = 1, m = 0 if target TCI state is not in the active TCI state list for PDSCH; otherwise TOk = 0, m = 1. - Trs is time to the first TRS or SSB transmission after the SSB transmission in the definition of Tfirst-SSB is processed by the UE; PDSCH transmission starts at THARQ + 3Nslotsubframe,µ + TOk*(Tfirst-SSB + TSSB-proc + Trs + Trs-proc) / NR slot length after slot n, when the target TCI state is not in the active TCI state list. Issue 1-2-2: We still have one more meeting to finish the WI, could we finalize the SNR in the next meeting? We need more time to derive the reliable simulation results. |
| Huawei | Reply to QC: As per agreement in R4-2203093, RAN4 agreed to define o Case 1: Uni-directional scenario A with DPS scheme 1b o Case 2: Bi-directional scenario B with DPS scheme 1a For Case 1, UE needs to support two active TCI states, i.e. TOk = 0, m = 1, so the TCI switching delay should be 2 + THARQ + TMAC For Case 2, UE just needs to support one active TCI states, i.e. TOk = 1, m = 0, so the TCI switching delay should be THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TSSB proc, no more one slot needs to be added (CR needs to be revised to modify this). |
| Samsung | Issue 1-2-1 In general, the TCI state delay should be followed with RRM core requirement, For Unidirectional scenario, one additional symbol is needed. Considering the PDSCH scheduling is per slot, skipping the remaining 13 PDSCH symbols statistics, there should be no impact on demodulation requirement Maybe we add a note to indicate the TCI state switch occurs at the symbol 1 in slot n+1 + THARQ + TMAC ? Issue 1-2-2 Maybe we can capture with [], and can be updated in the next meeting? |

Sub topic 1-3

| Company | Comments |
| --- | --- |
| XXX | Issue 1-3-1 Issue 1-3-2 |
| Ericsson | Issue 1-3-1 Support Option 1 Issue 1-3-2: Support the recommended WF. |
| Huawei | Issue 1-3-1: Test setup for PDSCH allocation timeline for Bi-directional scenario We are OK with the recommended WF. Issue 1-3-2: Requirement for Bi-directional scenario We are OK with the recommended WF. |
| QC | Issue 1-3-2: We still have one more meeting to finish the WI, could we finalize the SNR in the next meeting? We need more time to derive the reliable simulation results. |
| Samsung | Issue 1-3-1 OK with recommended WF Issue 1-3-2 Maybe we can capture with [], and can be updated in the next meeting? |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2209072 (draft CR: FRC for PDSCH demodulation requirement for FR2 HST, Ericsson) | Company A |
|  | Company B |
| R4-2209862 (Draft CR on minimum requirements for FR2 PDSCH HST-DPS requirements (38.101-4, Rel-17, Huawei) | Huawei: Revision is needed to apply the Step 5 of test setup for Uni-directional deployment and also to remove the square brackets for the requirements. |
|  | Company B |
| R4-2210083 (FR2 HST channel model, Qualcomm | Ericsson: It is good to put name 'HST-DPS-FR2-BI-B' and 'HST-DPS-FR2-UNI-A' in the Tables B.3.4.1-1 and B.3.4.2-1 as follows, since it is expected to define new scenario in Rel-18. Example: Table B.3.4.1-1: FR2 HST-DPS Unidirectional scenario Parameter; Value. HST-DPS-FR2-UNI-A. Ds; 700 m. Ds_offset; 10 m. Dmin; 10 m. v; 350 km/h. fd; 9722 Hz. Example: Table B.3.4.2-1: FR2 HST-DPS Bidirectional scenario Parameter; Value. HST-DPS-FR2-BI-B. 700 m. 150 m. 350 km/h. 9722 Hz. |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1-1: Noc power level Tentative agreements: Operating bands; n257; n258; n261. Noc level (dBm/Hz); -162.4; -162.6; -162.4. |
| Sub-topic #1-2 | Issue 1-2-1: Test setup for PDSCH allocation timeline for Uni-directional scenario Candidate options: - Introduce PDSCH requirement with PDSCH allocation timeline for Uni-directional scenario - Option 1 - Step 1: Two RRHs of RRH#(2k), RRH#(2k+1) are assumed, and SSB#(2k mod 4) and SSB#((2k+1 )mod 4) are transmitted for each TRPs, separately, where k is the RRH number with k =0, 1, 2, …. - UE is configured with TCI#(2k mod 4) and TCI #((2k+1)mod 4) that are associated with TRS #(2k mod 4) and TRS#((2k+1)mod 4) transmitted from RRH#(2k) and RRH#(2k+1) respectively by RRC signalling tci-StatesToAddModList in the PDSCH-Config and tci-PresentInDCI is not configured; - All the configured TCI states are known to UE. UE is configured with NZP-CSI-RS resource for L1-RSRP measurements by RRC signalling nzp-CSI-RS-ResourceSet within the CSI-ResourceConfig and periodic CSI reporting by setting reportConfigType to periodic and reportQuantity to cri-RSRP (Note: reported L1-RSRP mesurements are not tested) - Step 2: TE actives TCI #0 for PDCCH by “TCI State Indication for UE-specific PDCCH MAC CE”; - Step 3: PDSCH associated with TCI #0 is transmitted during the slots from 0 to n+ THARQ + TMAC - Step4: In slot n TE start triggering TCI state switching command to TCI #1 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4”; - Step 5: PDSCH associated with TCI #1 is transmitted in slots from n+2 + THARQ + TMAC to N. - PDSCH associated with TCI#(k mod 4) (k=1) is transmitted in slot from 0 to n + THARQ + TMAC - PDSCH associated with TCI #(k mod 4) (k=2, 3,…) is transmitted in slot from (k-1)*n+2 + THARQ + TMAC to (k*n)+ THARQ + TMAC, where n =57600 is the number of slots between the location of (k-1)Ds- DS_offset and the location of (k)⋅DS-DS_offset. And k is the RRH number in the channel model. - PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered - Option 2 - Step 1: Two RRHs of RRH#(2k), RRH#(2k+1) are assumed, and SSB#(2k mod 4) and SSB#((2k+1 )mod 4) are transmitted for each TRPs, separately, where k is the RRH number with k =0, 1, 2, …. - UE is configured with TCI#(2k mod 4) and TCI #((2k+1)mod 4) that are associated with TRS #(2k mod 4) and TRS#((2k+1)mod 4) transmitted from RRH#(2k) and RRH#(2k+1) respectively by RRC signalling tci-StatesToAddModList in the PDSCH-Config and tci-PresentInDCI is not configured; - All the configured TCI states are known to UE. UE is configured with NZP-CSI-RS resource for L1-RSRP measurements by RRC signalling nzp-CSI-RS-ResourceSet within the CSI-ResourceConfig and periodic CSI reporting by setting reportConfigType to periodic and reportQuantity to cri-RSRP (Note: reported L1-RSRP mesurements are not tested) - Step 2: TE actives TCI #0 for PDCCH by “TCI State Indication for UE-specific PDCCH MAC CE”; - Step 3: PDSCH associated with TCI #0 is transmitted during the slots from 0 to n+ THARQ + TMAC - Step4: In slot n TE start triggering TCI state switching command to TCI #1 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4”; - Step 5: PDSCH associated with TCI #1 is transmitted in slots from n+2 + THARQ + TMAC to N. - PDSCH associated with TCI#(k mod 4) (k=1) is transmitted in slot from 0 to n + THARQ + TMAC - PDSCH associated with TCI #(k mod 4) (k=2, 3,…) is transmitted in slot from (k-1)*n+2 + THARQ + TMAC to (k*n)+ THARQ + TMAC, where n =57600 is the number of slots between the location of (k-1)Ds- DS_offset and the location of (k)⋅DS-DS_offset. And k is the RRH number in the channel model. - PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered - Note: UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the symbol 1 in slot of (k-1)*n +1 THARQ + TMAC Recommendations for 2nd round - For Uni-directional, UE needs to support two active TCI states, the target TCI state is the active TCI state list, i.e., TOk = 0, m = 1, additional 1 symbol delay is needed for TCI switching delay. Considering PDSCH is scheduling per slot, there should be no impact on demodulation requirement with skipping the remaining PDSCH scheduling in the slot of (k-1)*n + THARQ + TMAC,, encourage companies can check whether option 2 with additional note is acceptable ? Issue 1-2-2: Requirement for Uni-directional scenario Tentative agreements: Candidate options: - Option 1: Capture SNR = [13.7] dB into draft CR based on simulation summary R4-2208074 in RAN4#103-e meeting, interest companies are encouraged to provide updated simulation results in RAN4#104-e meeting - Option 2: TBD in RAN4#103-e meeting Recommendations for 2nd round - Companies are encouraged to check whether option1 is acceptable? |
| Sub-topic #1-3 | Issue 1-3-1: Test setup for PDSCH allocation timeline for Bi-directional scenario Tentative agreements: - Introduce PDSCH requirement with PDSCH allocation timeline for Bi-directional scenario - Step 1: Two RRHs of RRH#(2k), RRH#(2k+1) are assumed, and SSB#(2k mod 4) and SSB#((2k+1 )mod 4) are transmitted for each TRPs, separately, where k is the RRH number with k =0, 1, 2, …. - UE is configured with TCI#(2k mod 4) and TCI #((2k+1)mod 4) that are associated with TRS #(2k mod 4) and TRS#((2k+1)mod 4) transmitted from RRH#(2k) and RRH#(2k+1) respectively by RRC signalling tci-StatesToAddModList in the PDSCH-Config and tci-PresentInDCI is not configured; - All the configured TCI states are known to UE. UE is configured with NZP-CSI-RS resource for L1-RSRP measurements by RRC signalling nzp-CSI-RS-ResourceSet within the CSI-ResourceConfig and periodic CSI reporting by setting reportConfigType to periodic and reportQuantity to cri-RSRP (Note: reported L1-RSRP measurements are not tested) - Step 2: TE actives TCI #2 for PDCCH by “TCI State Indication for UE-specific PDCCH MAC CE”; - Step 3: PDSCH associated with TCI #2 is transmitted during the slots from 0 to n + THARQ + TMAC proc; - Step 4: In slot n TE start triggering TCI state switching command to TCI #1 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4”; - Step 5: PDSCH associated with TCI #1 is transmitted in slots from n+1 + THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc to 2n+ THARQ + TMAC proc - Step 6: In slot 2n TE start triggering TCI state switching command to TCI# 4 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4” - Step 7: PDSCH associated with TCI #4 is transmitted in slots from 2n+1 + THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TSSB proc to 3n+THARQ + TMAC proc; - PDSCH associated with TCI#(2k mod 8) (k=1) is transmitted in slot from 0 to n+ THARQ + TMAC proc - PDSCH associated with TCI #(2k mod 8) (k=2,3, …) is transmitted in slot from (2k-2)n +1 + THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc to (2k-1)n + THARQ + TMAC proc - PDSCH associated with TCI #((2k+1)mod 8) (k=0,1,2,…) is transmitted in slot from (2k+1)n +1+ THARQ + TMAC proc + TfirstSSB + TSSB proc + TfirstTRSafterSSB + TTRS proc to (2(k+1)n+ THARQ + TMAC proc) where n =28800 slots is the half of the number of slots between two RRHs. - PDCCH and PDSCH are DTXed in other slots in which throughput statistic are not considered Issue 1-3-2: Requirement for Bi-directional scenario Candidate options: - Option 1: Capture SNR = [13.9] dB into draft CR based on simulation summary R4-2208074 in RAN4#103-e meeting, interest companies are encouraged to provide updated simulation results in RAN4#104-e meeting - Option 2: TBD in RAN4#103-e meeting Recommendations for 2nd round - Companies are encouraged to check whether option1 is acceptable? |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2209862 (Draft CR on minimum requirements for FR2 PDSCH HST-DPS requirements (38.101-4, Rel-17, Huawei) | To be revised |
| R4-2210083 (FR2 HST channel model, Qualcomm | To be revised |

## Discussion on 2nd round (if applicable)

Issue 1-2-1: Test setup for PDSCH allocation timeline for Uni-directional scenario

Proposals

- Introduce PDSCH requirement with PDSCH allocation timeline for Uni-directional scenario

  - Option 1

    - Step 1: Two RRHs of RRH#(2k), RRH#(2k+1) are assumed, and SSB#(2k mod 4) and SSB#((2k+1 )mod 4) are transmitted for each TRPs, separately, where k is the RRH number with k =0, 1, 2, ….

- UE is configured with TCI#(2k mod 4) and TCI #((2k+1)mod 4) that are associated with TRS #(2k mod 4) and TRS#((2k+1)mod 4) transmitted from RRH#(2k) and RRH#(2k+1) respectively by RRC signalling tci-StatesToAddModList in the PDSCH-Config and tci-PresentInDCI is not configured;

- All the configured TCI states are known to UE. UE is configured with NZP-CSI-RS resource for L1-RSRP measurements by RRC signalling nzp-CSI-RS-ResourceSet within the CSI-ResourceConfig and periodic CSI reporting by setting reportConfigType to periodic and reportQuantity to cri-RSRP (Note: reported L1-RSRP mesurements are not tested)

    - Step 2: TE actives TCI #0 for PDCCH by “TCI State Indication for UE-specific PDCCH MAC CE”;

    - Step 3: PDSCH associated with TCI #0 is transmitted during the slots from 0 to n+ THARQ + TMAC

    - Step4: In slot n TE start triggering TCI state switching command to TCI #1 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4”;

    - Step 5: PDSCH associated with TCI #1 is transmitted in slots from n+2 + THARQ + TMAC to N.

    - PDSCH associated with TCI#(k mod 4) (k=1) is transmitted in slot from 0 to n + THARQ + TMAC

    - PDSCH associated with TCI #(k mod 4) (k=2, 3,…) is transmitted in slot from (k-1)*n+2 + THARQ + TMAC  to (k*n)+ THARQ + TMAC, where n =57600 is the number of slots between the location of (k-1)Ds- DS_offset and the location of (k)⋅DS-DS_offset. And k is the RRH number in the channel model.

    - PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered

  - Option 2

    - Step 1: Two RRHs of RRH#(2k), RRH#(2k+1) are assumed, and SSB#(2k mod 4) and SSB#((2k+1 )mod 4) are transmitted for each TRPs, separately, where k is the RRH number with k =0, 1, 2, ….

- UE is configured with TCI#(2k mod 4) and TCI #((2k+1)mod 4) that are associated with TRS #(2k mod 4) and TRS#((2k+1)mod 4) transmitted from RRH#(2k) and RRH#(2k+1) respectively by RRC signalling tci-StatesToAddModList in the PDSCH-Config and tci-PresentInDCI is not configured;

- All the configured TCI states are known to UE. UE is configured with NZP-CSI-RS resource for L1-RSRP measurements by RRC signalling nzp-CSI-RS-ResourceSet within the CSI-ResourceConfig and periodic CSI reporting by setting reportConfigType to periodic and reportQuantity to cri-RSRP (Note: reported L1-RSRP mesurements are not tested)

    - Step 2: TE actives TCI #0 for PDCCH by “TCI State Indication for UE-specific PDCCH MAC CE”;

    - Step 3: PDSCH associated with TCI #0 is transmitted during the slots from 0 to n+ THARQ + TMAC

    - Step4: In slot n TE start triggering TCI state switching command to TCI #1 by “TCI State Indication for UE-specific PDCCH MAC CE with MCS 4”;

    - Step 5: PDSCH associated with TCI #1 is transmitted in slots from n+2 + THARQ + TMAC to N.

    - PDSCH associated with TCI#(k mod 4) (k=1) is transmitted in slot from 0 to n + THARQ + TMAC

    - PDSCH associated with TCI #(k mod 4) (k=2, 3,…) is transmitted in slot from (k-1)*n+2 + THARQ + TMAC  to (k*n)+ THARQ + TMAC, where n =57600 is the number of slots between the location of (k-1)Ds- DS_offset and the location of (k)⋅DS-DS_offset. And k is the RRH number in the channel model.

    - PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered

    - Note: UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the symbol 1 in slot of (k-1)*n +1 THARQ + TMAC

Recommended WF

- For Uni-directional, UE needs to support two active TCI states, the target TCI state is the active TCI state list, i.e., TOk = 0, m = 1, additional 1 symbol delay is needed for TCI switching delay. Considering PDSCH is scheduling per slot, there should be no impact on demodulation requirement with skipping the remaining PDSCH scheduling in the slot of (k-1)*n + THARQ + TMAC,, encourage companies can check whether option 2 with additional note is acceptable ?

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2-1 |
| Huawei | The only difference for the two options is that whether to add an additional note. We don’t think this note is necessary since the updated test setup in above options comply with the RRM specification from the demodulation point of view. Actually more relaxed requirements (1 slot interruption) is defined for demodulation comparing to the RRM specification (1 symbol interruption). For the recommended WF: As per RRM specification, UE shall apply the target slot after symbol 1 of slot n + THARQ + TMAC, so the skipping PDSCH scheduling should be in the slot of (k-1)*n + 1 + THARQ + TMAC, i.e. 1 slot interruption. |
| QC | Agree with Huawei on the comment to recommended WF. And we also don’t see what is the difference with and without the note. |

Issue 1-2-2: Requirement for Uni-directional scenario

- Proposals

  - Option 1: Capture SNR = [13.7] dB into draft CR based on simulation summary R4-2208074 in RAN4#103-e meeting, interest companies are encouraged to provide updated simulation results in RAN4#104-e meeting

  - Option 2: TBD in RAN4#103-e meeting

- Recommended WF

  - Companies are encouraged to check whether option1 is acceptable?

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2-2 |
| Ericsson | Option 1. Four companies have provided the results and the results are well aligned. We prefer to set the requirements with [] in this meeting. Interested companies can provide the results in the next meeting and update the requirements if necessary. |
| Huawei | We are OK with Option 1. |
| QC | We can compromise to option 1. |

Issue 1-3-2: Requirement for Bi-directional scenario

- Proposals

  - Option 1: Capture SNR = [13.9] dB into draft CR based on simulation summary R4-2208074 in RAN4#103-e meeting, interest companies are encouraged to provide updated simulation results in RAN4#104-e meeting

  - Option 2: TBD in RAN4#103-e meeting

- Recommended WF

  - Companies are encouraged to check whether option1 is acceptable?

| Company | Comments |
| --- | --- |
| XXX | Issue 1-3-2 |
| Ericsson | Option 1. Same comment as 1-2-2. |
| Huawei | We are OK with Option 1. |
| QC | We can compromise to option 1. |

| CR/TP number | Comments collection |
| --- | --- |
| R4-2209862 (Draft CR on minimum requirements for FR2 PDSCH HST-DPS requirements (38.101-4, Rel-17, Huawei) | Huawei: Revision is needed to apply the Step 5 of test setup for Uni-directional deployment and also to remove the square brackets for the requirements. The revised CR has been uploaded for your review at: draft R4-2210910 Draft CR on minimum requirements for FR2 PDSCH HST-DPS requirements (38.101-4, Rel-17).docx |
|  | Company B |
| R4-2210083 (FR2 HST channel model, Qualcomm | Ericsson: It is good to put name 'HST-DPS-FR2-BI-B' and 'HST-DPS-FR2-UNI-A' in the Tables B.3.4.1-1 and B.3.4.2-1 as follows, since it is expected to define new scenario in Rel-18. Example: Table B.3.4.1-1: FR2 HST-DPS Unidirectional scenario Parameter; Value. HST-DPS-FR2-UNI-A. Ds; 700 m. Ds_offset; 10 m. Dmin; 10 m. v; 350 km/h. fd; 9722 Hz. Example: Table B.3.4.2-1: FR2 HST-DPS Bidirectional scenario Parameter; Value. HST-DPS-FR2-BI-B. 700 m. 150 m. 350 km/h. 9722 Hz. |
|  | Samsung: Please add the 2210083 into cover sheet as revision history and use latest version of CR template as v12.2 The FR2 HST DPS Bidirectional Deployment Channel Profile is a single tap propagation channel, switching transmission point between adjacent RRHs as illustrated in Figure -> B.3.4.2-1: |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| R4-XXXXX | WF on UE demodulation requirement for FR2 HST | Samsung | for approve |
| R4-XXXXX | draftCR to TS 38.101-4: Applicability rules for HST FR2 PDSCH requirements | Intel | Resubmitted CR for 38.101-4 Cat. B Current version: 17.4.0F For endorsed |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2209072 |  | (draft CR: FRC for PDSCH demodulation requirement for FR2 HST, Ericsson) | Ericsson | Agreeable |  |
| R4-2208074 |  | Simulation results summary for Rel-17 FR2 HST UE demod | Samsung | Noted |  |
| R4-2208076 |  | Simulation results of PDSCH requirement for Rel-17 FR2 HST | Samsung | Noted |  |
| R4-2209071 |  | PDSCH demodulation requirements for HST FR2 | Ericsson | Noted |  |
| R4-2209863 |  | Discussion on UE demodulation requirements for FR2 HST Uni-directional | Huawei | Noted |  |
| R4-2209864 |  | Discussion on UE demodulation requirements for FR2 HST Bi-directional | Huawei | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2210653 |  | WF on UE demodulation requirement for FR2 HST | Samsung | Agreeable |  |
| R4-2210654 |  | draftCR to TS 38.101-4: Applicability rules for HST FR2 PDSCH requirements | Intel | Agreeable |  |
| R4-2210910 |  | Draft CR on minimum requirements for FR2 PDSCH HST-DPS requirements (38.101-4, Rel-17) | Huawei | Agreeable |  |
| R4-2210911 |  | CR: FR2 HST channel model | Qualcomm | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Moderator (Samsung) | Yunchuan Yang | yc0301.yang@samsung.com |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)