3GPP TSG-RAN WG4 Meeting #103e	R4-2210586

Electronic Meeting, 9 – 20 May, 2022

Title: 	WF on R17 NR MG enhancements – Pre-configured MG

Agenda Item:	9.10.1.1, 9.10.2.1

Source: 	Intel

Document for:	Approval

# Topic #1: Core requirement maintenance (AI 9.10.1.1)

## Sub-topic 1-1 Pre-MG activation/deactivation

### Issue 1-1-1 Additional trigger events for pre-MG activation/deactivation

Agreements:

- Remove LocationMeasurementIndication from the trigger events in clause 9.1.7.3.1. [TS38.133 v17.5.0]

“9.1.7.3. ….. The UE can autonomously change the Pre-MG status from activation to deactivation or vice versa based on any of the following triggering conditions: - DCI, timer or RRC based active BWP switching, - Activation/deactivation of SCell(s), - Addition/removal of any measurement object(s) - Addition/release/change of a SCell in carrier aggregation, - Initiation of LocationMeasurementIndication procedure specified in clause 5.5.6 [2].

## Sub-topic 1-2 Pre-MG activation/deactivation under CA

### Issue 1-2-1: Clarifications on simultaneous BWP switches on multiple

Agreements:

- From RAN4 perspective, there is no any requirements applied to simultaneous BWP switches on multiple CCs in Rel17 when pre-configured MG is configured to UE.

### Issue 1-2-2 Whether need indicate the state of pre-configured MG when switching to initial DL BWP

Agreements:

- BWP switching between the initial BWP and other types of BWP is possible to trigger the pre-MG (de)activation by:

    - Timer based BWP switching

    - RRC based BWP switching

    - DCI based BWP switching: FFS

- The current RAN2 signaling design (deactivatedMeasGapList-r17 in BWP-DownlinkDedicated) for Pre-MG on/off is problematic if the initial BWP(BWP#0) was not configured by B2-2 [TS38.331 v17.5.0. B.2]

- RAN4 can forward a LS to RAN2 to check this problem and how to resolve it shall be up to RAN2.

## Sub-topic 1-3 RRM requirements: Activation/Deactivation delay

### Issue 1-3-1 Pre-MG activation/deactivation delay trigged by Scell activation/deactivation

Agreements:

- Modify the requirement on the Pre-MG activation/deactivation delay trigged by Scell activation/deactivation as:

“UE shall be able to finish pre-configured activation or deactivation within [5] ms after the report of the latest first valid CQI among all Scell(s) being activated.”

### Issue 1-3-2 How to determine pre-MG (de)activation status when multiple trigger events happened simultaneously

Agreements:

- The general principle on how to determine Pre-MG (de)activation status when multiple trigger events happened simultaneously can be:

    - “The UE shall determine the status of the Pre-MG pattern based on all trigger events”

### Issue 1-3-3 Pre-MG activation/deactivation delay after pre-MG being configured initially

Agreements:

- Define delay for activation/deactivation after pre-MG configuration as [5]ms after the RRC processing time

# Topic #2: Performance requirement maintenance (AI 9.10.2.1)

## Sub-topic 2-1 Test case configuration

### Issue 2-1-1 Test cases for the different deployment scenarios

Agreements:

- Only need to define the test cases for SA

### Issue 2-1-2 Non-DRX or DRX

Agreements:

- Only define the test cases for Non-DRX cases

### Issue 2-1-3 CA or single carrier

Agreements:

- Define the test cases for single carrier cases firstly. The final agreements on TC list in issue 2-2-5 can be updated accordingly.

- Whether the other additional test cases for CA can be FFS in the next meeting.

### Issue 2-1-4 Pre-MG activation mechanism

Agreements:

- Both UE autonomous and network-controlled pre-MG (de)activation mechanisms shall be verified.

### Issue 2-1-5 Which types of measurement reference signal measured

Agreements:

- The SSB reference signal can be configured for Pre-MG test cases.

### Issue 2-1-6 Which types of trigger events

Agreements:

- Define the test cases for BWP switching trigger event firstly.

- Whether the other additional test cases for MO addition/remove can be FFS in the next meeting.

### Issue 2-1-7 per-UE/per-FR gap patterns

Agreements:

- per-UE gap pattern can be configured in Pre-MG testing

### Issue 2-1-8 Gap pattern

Agreements:

- #0 in FR1 and #13 for FR2

### Issue 2-1-9 Testing procedure

Agreements:

- Both the activation/deactivation and measurement reporting can be fulfilled within one testing procedure.

### Issue 2-1-10 Test cases on FR1/FR2

Agreements:

- Define the different TCs for FR1 and FR2

## Sub-topic 2-2 Test case list

### Issue 2-2-1 Test cases for Pre-configuration MG activation/deactivation delay

Agreements:

- Define TCs for both activation and deactivation delay

### Issue 2-2-2 Test cases for the reporting delay of intra-frequency measurement with gap

Agreements:

- Combine the test cases for the intra-frequency measurement reporting with that for Pre-MG activation/deactivation delay which is triggered by BWP switching

### Issue 2-2-3 Test cases for the reporting delay of inter-frequency measurement with gap

Agreements:

- Whether the other additional test cases for the inter-frequency measurement report event can be FFS in the next meeting.

### Issue 2-2-4 Test cases for data Tx/Rx when pre-MG deactivated

Agreements:

- Define TCs for pre-MG to verify whether UE can perform the data Tx/Rx when pre-MG deactivated.

### Issue 2-2-5 Test cases list and work split

Agreements:

| No. | Test case | Testing setup and scenarios | Purpose of test | Clause in TS38.133 | draftCR Responsible company |
| --- | --- | --- | --- | --- | --- |
| 1-1 | Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR1 | - Intra-frequency - FR1 - SA - Non-CA - No network indication on Pre-MG activation/deactivation Pre-MG - DCI-based BWP switching trigger events - SSB - Per-UE gap - #0 | - When pre-MG being deactivated at the beginning of testing, UE can report the results of intra-frequency measurement without gap within the required period and receives data from the serving cell - Pre-MG activation/deactivation delay - After pre-MG being activated by UE autonomously, UE can report the results of intra-frequency measurement with gap within the required period | A6.6.x1 | Intel |
| 1-2 | Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2 | - Intra-frequency - FR2 - SA - Non-CA - No network indication on Pre-MG activation/deactivation Pre-MG - DCI-based BWP switching trigger events - SSB - Per-UE gap - #13 | - When pre-MG being deactivated at the beginning of testing, UE can report the results of intra-frequency measurement without gap within the required period and receives data from the serving cell - Pre-MG activation/deactivation delay - After pre-MG being activated by UE autonomously, UE can report the results of intra-frequency measurement with gap within the required period | A7.6.x1 | Apple |
| 1-3 | Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR1 | - Intra-frequency - FR1 - SA - Non-CA - Network indication on Pre-MG activation/deactivation Pre-MG - DCI-based BWP switching trigger events - SSB - Per-UE gap - #0 | - When pre-MG being activated at the beginning of testing, UE can report the results of intra-frequency measurement with gap within the required period and can’t transmit data to the serving cell during the activated gap - Pre-MG deactivation delay - After pre-MG being deactivated by network signalling, UE can report the results of intra-frequency measurement without gap within the required period and can receive the data from the serving cell | A6.6.x2 | Huawei |
| 1-4 | Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2 | - Intra-frequency - FR2 - SA - Non-CA - Network indication on Pre-MG activation/deactivation Pre-MG - DCI-based BWP switching trigger events - SSB - Per-UE gap - #13 | - When pre-MG being activated at the beginning of testing, UE can report the results of intra-frequency measurement with gap within the required period and can’t transmit data to the serving cell during the activated gap - Pre-MG deactivation delay - After pre-MG being deactivated by network signalling, UE can report the results of intra-frequency measurement without gap within the required period and can receive the data from the serving cell | A7.6.x2 | MTK |
| [1-5] (TBD) | Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR1 | - Intra-frequency - FR1 - SA - Non-CA - No network indication on Pre-MG activation/deactivation Pre-MG - MO addition/release trigger events - Per-UE gap - #0 | - When pre-MG being deactivated, UE can report the results of intra-frequency measurement without gap within the required period and receives data from the serving cell - Pre-MG activation delay - When pre-MG being activated by UE autonomously, UE can report the results of intra-frequency measurement with gap within the required period | A6.6.x3 | OPPO |