3GPP TSG-RAN WG4 Meeting #103-e	R4-2210591

E-meeting, 9 – 20 May, 2022

Agenda item:	10.16.8

Source:	Intel Corporation

Title:	WF on LBT impacts on RRM requirements for FR2-2

Document for:	Approval

# Introduction

In this contribution we capture the agreements achieved during the email discussion on the topic Rel-17 NR ext. to 71GHz RRM core requirements (AI 10.16.8) in RAN4#103-e under email thread [103-e][213] NR_ext_to_71GHz_RRM_2. This email thread covers following sub-agenda items:

- AI 10.16.8.6	LBT impacts on RRM requirements

The details on the discussion can be found at the [213] thread discussion summary [1].

# Way forward

## LBT impacts on RRM requirements

### General

- Consider in the maintenance stage between option 1 and option 2 on how to specify the maximum separation between two consecutive measurements.

  - Option 1: The requirement apply provided any two measurement shall not be separated in time by more than the maximum time requirement for the cell to remain known.

  - Option 2: the requirement only applies when

    - Within the set of measurements any two measurements shall not be separated in time by more than 2 seconds when no DRX is configured, and

    - Within the set of measurements any two measurements shall not be separated in time by more than max(1 DRX occasion group duration, 2 seconds) when DRX is configured.

### Signalling characteristics

- For operation with LBT, extend the RLM out-of-sync evaluation period by a fixed factor in FR2-2

| Configuration | TEvaluate_out_SSB,CCA (ms) |
| --- | --- |
| no DRX | Max(200, Ceil([12]*P* N)*TSSB) |
| DRX cycle≤320 | Max(200, Ceil(1.5*[10]*P* N)*Max(TDRX,TSSB)) |
| DRX cycle>320 | Ceil([10]*P* N)*TDRX |

- Extend BFD OOS evaluation periods the same as those of RLM OOS

- The same requirements apply between different side conditions SNR <-7dB or >= -7dB

### Measurement procedure

- Specify RSSI measurement requirements and reuse the ones specified for NR-U.

- The scheduling availability during intra-frequency measurements in carrier frequencies with CCA in FR2-2 is the same as the scheduling availability defined for operation without CCA in FR2-2.

- Identify in the maintenance stage the differences between FR2-2 and FR1 NR-U regarding whether the UE assumes LBT when it carries out measurements. Identify if any, impact on RRM spec due to those differences.

### Signaling of CCA configuration of neighbour cells

- Send an LS to RAN1 and RAN2 asking about their inputs on signalling of CCA configuration of neighbour cells

# References

- R4-2210482, Email discussion summary for [103-e][213] NR_ext_to_71GHz_RRM_2, Intel Corporation