3GPP TSG-RAN WG4 Meeting #103-e                	 R4-2210620

Electronic Meeting, May. 9- May. 20, 2022

Title: 	WF on SL relay test cases

Agenda Item:	9.23

Source: 	OPPO

Document for:	Approval

<Topic 1> Test case scope

  - <Sub-topic 1-1> Scope of test cases of SL relay RRM

< Agreement>:

  - Introduce two test cases:

- Verify interruption requirements at NR sidelink discovery configuration.

- Verify selection/reselection of sidelink relay UE in NR

  - Define test applicability rule: the new test for interruption at sidelink discovery configuration is not applicable to UE passed legacy sidelink configuration interruption test A.9.1.6.1.

<Topic 2>Test case for selection/reselection of sidelink relay UE in NR

  - <Sub-topic 2-1> Test case for interruption requirements at NR sidelink discovery configuration

< Agreement >: Introduce test for interruption requirements at NR sidelink discovery configuration

- Option 1:

- During T1, the UE is in idle state and monitoring NR sidelink discovery announcements from other active sidelink UEs on the NR sidelink discovery resources.

- During T2, the UE is expected to transmit SidelinkUEInformationNR indicating sl-RxInterestedFreqListDisc and receive RRC reconfiguration message. No PDSCH is scheduled for UE.

- During T3, the UE S-RSRP is scheduled with PDSCH on Pcell downlink. The missed ACK/NACKs during T3 shall be counted to verify the allowed interruptions during NR sidelink discovery.

  - <Sub-topic 2-2> Test case for selection/reselection of sidelink relay UE in NR

< Agreement >: Introduce test for relay selection/reselection measurement delay requirement, by using LTE ProSe A.7.5.12 test as a reference.

  - Confirm to use SL resource allocation mode 2 for UE in the test

  - Configure separate resource pool configuration of SL discovery and SL communication for UE in the test.

Issue 2-2-2: Whether UE needs to request serving cell for sidelink communication resource in T2

< Agreement >: The resource pool for remote UE has been configured by Pcell before T1 starts. During T2, remote UE is not expected to request the serving cell for sidelink communication resources for communicating with a candidate sidelink relay UE.

Issue 2-2-3: Whether the discovery resource configuration and allocation can be skipped.

< Agreement >: The reference resource pool configurations for R17 SL discovery need to be introduced.

Reference

[1] R4-2210501, Email discussion summary for [103-e][231] NR_SL_relay, Moderator (OPPO), RAN4#103e