3GPP TSG-RAN WG4 Meeting #103-e	R4-2210662

# Electronic Meeting, May 9th – 20th, 2022

Title:	WF on NTN SAN demodulation requirements

Source:	Huawei, HiSilicon

Agenda item:			9.12.8.2

Document for:	Approval

Introduction

This WF capture all agreements and open issues for the following topics discussed in [103-e][323] NR_NTN_Demod_Part2.

- Satellite Access Node demodulation requirements

  - PUSCH requirements

  - PUCCH requirements

  - PRACH requirements

Satellite Access Node demodulation requirements

PUSCH performance requirements

Scope of PUSCH requirements

Agreements

- Not consider 2-step RACH case

NTN normal PUSCH requirements

SCS/CBW set

Agreements

- One test cases with few PRBs for each SCS

- Reuse Rel-15 test applicability rule defined in clause 8.1.2.1.2 Applicability of requirements for different channel bandwidths

Candidate options

- Option 1: 15kHz SCS: 5MHz/10MHz/20MHz, 30kHz SCS: 10MHz/20MHz

- Option 2: 4 PRBs for all SCS

- Other options are not precluded.

Additional antenna configuration besides 2Rx

Agreements

- Both 1Rx and 2Rx shall be considered for NTN SAN demodulation requirements.

- FFS for the test applicability rules.

Modulation order

Agreements

- MCS4 unless any technical issues figured out

Other test parameters

Agreements:

- Use the following test parameters for NTN PUSCH performance requirements definition unless any issues figured out

| Parameter |  | Value |
| --- | --- | --- |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0, group hopping and sequence hopping are disabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

NTN UL timing adjustment requirements

UE speed

Agreements

- 120km/h

Additional DM-RS position:

Agreements

- pos1

Transform precoding

Candidate options

- Option 1: Same as normal PUSCH

- Option 2: CP-OFDM only

Other test parameters for UL timing adjustment requirement

Agreements

- Agree to use the following test parameters for NTN PUSCH UL TA performance requirements definition unless any issues figured out

| Parameter |  | Value |
| --- | --- | --- |
| Channel bandwidth |  | Same as normal PUSCH |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
| Time domain resource assignment | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
|  | PUSCH mapping type | Both A and B |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Depends on normal PUSCH |
|  | Starting PRB index | Based on RB assignment |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | slot #1 in radio frames |
|  | SRS resource allocation | Based on RB assignment |
| NOTE 1. The transmission of SRS is optional. And the transmission comb and SRS periodic are configured as KTC = 2, and TSRS = 10 respectively. |  |  |

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: Same as PUSCH for NTN-TDLA |
| CP length | Normal |
| A | 15 kHz: 10 s 30 kHz: 5 s |
|  | Depends on UE speed |

NTN PUSCH repetition type A requirements

MCS for PUSCH repetition type A

Agreements

- MCS5 in Table 3 same as legacy TN PUSCH repetition type A

Transform precoding for PUSCH repetition type A

Candidate options

- Option 1: Same as normal PUSCH

- Option 2: CP-OFDM only

Other test parameters for PUSCH repetition type A

Agreements

- Agree to use the following parameters for NTN PUSCH repetition type A requirements.

| Parameter |  | Value |
| --- | --- | --- |
| Channel model |  | NTN-TDL-A, Doppler and delay same as normal PUSCH |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 2] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource assignment | RB assignment | Same as normal PUSCH |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

PUCCH requirements

Scope of PUCCH requirements

Agreements

- PUCCH format 2/3/4: additional note should be added that the UCI information does not contain CSI part 1 and CSI part 2.

- PUCCH format 0: select 2 symbol case from legacy BS PUCCH format 0 requirements.

- PUCCH format 1: Same as the legacy BS PUCCH format 1

- PUCCH format 2: select both two cases with different UCI information bits from legacy BS requirements

- PUCCH format 3: only select 1 PRB with 14 symbol case from legacy BS requirements

- PUCCH format 4: Same as the legacy BS PUCCH format 4

- For PUCCH format 3/4, FFS DMRS configuration:

  - Option 1: DM-RS 1+0

  - Option 2: Both DM-RS 1+0 and 1+1

- PUCCH format 1(multi-slot): Same as the legacy BS PUCCH format 1 (multi-slot)

SCS/CBW set

Agreements

- If only a few of PRBs can be used for UL, only one requirement will be applied for all bandwidths.

- If full bandwidth can be used for UL, only requirements for minimum bandwidths could be introduced.

- The PUCCH PRB number of PF2/3 need to wait for the conclusion of PUSCH part

- Reuse Rel-15 test applicability rule defined in clause 8.1.2.2.3 Applicability of requirements for different channel bandwidths

Antenna configuration for PUCCH

Agreements

- Align with the antenna configuration for PUSCH

Channel model

Agreements

- Align with the channel model for normal PUSCH requirements

PUCCH format 0

Agreements

- Detailed test parameters for NTN PUCCH format 0 as shown below:

Table 2.2-1 PUCCH format 0

| Parameter | Test |
| --- | --- |
| PUCCH format | 0 |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 |
| Number of OFDM symbols | 2 |
| Test metric | ACK missed detection probability <1% |

PUCCH format 1

Agreements

- Detailed test parameters for NTN PUCCH format 1 as shown below:

Table 2.2-2 PUCCH format 1

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Cyclic Prefix | Normal |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

PUCCH format 2

Agreements

- Detailed test parameters for NTN PUCCH format 2 as shown below:

Table 2.2-3 PUCCH format 2 for ACK missed detection

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

Table 2.2-4 PUCCH format 2 for UCI BLER

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | [9] |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Test metric | UCI block error probability<1% |

PUCCH format 3

Agreements

- Detailed test parameters for NTN PUCCH format 3 as shown below:

Table 2.2-5 PUCCH format 3

| Parameter | Value |
| --- | --- |
| PUCCH format | 3 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DM-RS |
| Test metric | UCI block error probability<1% |

PUCCH format 4

Agreements

- Detailed test parameters for NTN PUCCH format 4 as shown below:

Table 2.2-6 PUCCH format 4

| Parameter | Value |
| --- | --- |
| PUCCH format | 4 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DMRS |
| Test metric | UCI block error probability<1% |

Multi-slot PUCCH format 1

Agreements

- Detailed test parameters for NTN multi-slot PUCCH format 1 as shown below:

Table 2.2-7 PUCCH format 1 (multi-slot)

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 (multi-slot) |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |
| Cyclic Prefix | Normal |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

PRACH requirements

Preamble Format

Agreements

- Including format 0, B4 and C2

- FFS for format 2 and A2

SCS

Agreements

- Long preamble format (if agreed): 1.25kHz

- Short preamble format (if agreed): 15kHz and 30kHz SCS.

Sequence length for short preamble format

Agreements

- Only consider LRA=139 for short preamble format.

Antenna configuration

Agreements

- Align with the PUSCH

Time error tolerance

Agreements

- Use same time error tolerance of AWGN as Rel-15 FR1 for 1.25kHz, 15kHz and 30kHz SCS.

- Use the same method to calculate time error tolerance of multi-path channel as Rel-15: 0.522μ+Tdelay where Tdelay is corresponding to the tap delay of the propagation channel

    - Option 1: The largest delay of the propagation channel

    - Option 2: The second largest delay of the propagation channel

Frequency offset for PRACH Channel model

Agreements

- AWGN: 0Hz

- Fading channel: 200Hz (0.1ppm@2GHz)

Ncs

Agreements

- Use the same Ncs, Logical sequence index and v value as Rel-15 normal PRACH requirement, i.e.

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | 1.25 | 13 | 22 | 32 |
| [A2],B4,C2 | 15 | 23 | 0 | 0 |
|  | 30 | 46 | 0 | 0 |

Reference

- R4-2210329, Email discussion summary for [103-e][323] NR_NTN_Demod_Part2, RAN4#103-e, Moderator (Huawei)

- R4-2207198, WF on NTN SAN demodulation requirements, RAN4#102-e, Huawei, HiSilicon

- R4-2207464, WF on general and NTN UE demodulation requirements, RAN4#102-e, Qualcomm

- R4-2207169, Email discussion summary for [102-e][325] NR_NTN_Demod, RAN4#102-e, Moderator (Qualcomm Incorporated)

- R4-2203043, WF on NTN SAN demodulation requirements, RAN4#101bis-e, Huawei, HiSilicon