3GPP TSG-RAN WG4 Meeting #103-e	  R4-2210746

# Electronic Meeting, 9th May to 20th May, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38101-1-h50 Further note and correction for SUL combos |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SUL_combos_R17-Core |  |  |  |  |  | Date: |  |  | 2022-05-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The UL/DL frequency separation shall be kept consistent for FDD bands and its corresponding SUL band, such as n1_SUL_n84, n8_SUL_n81. The DL center frequency shall keep same TX/RX separatoin as correspoinding FDD band or MSD need to be characterized due to the change on T/R separation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add missing superscript on SUL_n24-n99 - Improving note 1 with adding sentence “This restriction applies also for these carriers when applicable SUL configuration is part of a higher order configuration.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Sensitivity would be degraded if T/R separation is not consistent with SUL band’s corresponding FDD band or MSD need to be characterized due to the change on T/R separation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

Table 7.3C.2-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n80 | 15 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |
| n1 | n841 | 15 | 25 | 50 | 75 | 100 | 128 | 128 | 128 | 128 |  |  |  |  |  |
| n3 | n801 | 15 | 25 | 50 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |  |  |
| n241 | n991 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |
| n28 | n831 | 15 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |  |  |  |
| n41 | n80 | 15 |  | 160 | 160 | 160 |  | 160 | 160 | 160 | 160 |  | 160 | 160 | 160 |
| n41 | n81 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n41 | n83 | 15 |  | 100 | 100 | 100 |  | 100 | 100 | 100 | 100 |  | 100 | 100 | 100 |
|  |  | 30 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n41 | n95 | 15 |  | 75 | 75 | 75 |  | 75 | 75 | 75 | 75 |  | 75 | 75 | 75 |
| n41 | n97 | 30 |  | 216 | 216 | 216 |  | 216 | 216 | 216 | 216 | 216 | 216 | 216 | 216 |
| n41 | n98 | 15 |  | 216 | 216 | 216 |  | 216 | 216 | 216 | 216 |  | 216 | 216 | 216 |
| n41 | n99 | 15 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n48 | n99 | 15 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n77 | n80 | 15 |  | 160 | 160 | 160 |  |  | 160 | 160 | 160 |  | 160 | 160 | 160 |
| n77 | n84 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n77 | n99 | 15 |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| n78 | n80 | 15 |  | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |
| n78 | n81 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n78 | n82 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n78 | n83 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | n84 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | n86 | 15 |  | 216 | 216 | 216 |  |  | 216 | 216 | 216 |  | 216 | 216 | 216 |
| n79 | n80 | 15 |  |  |  |  |  |  | 160 | 160 | 160 |  | 160 |  | 160 |
| n79 | n83 | 15 |  |  |  |  |  |  | 100 | 100 | 100 |  | 100 |  | 100 |
|  |  | 30 |  |  |  |  |  |  | 50 | 50 | 50 |  | 50 |  | 50 |
| n79 | n81 | 15 |  |  |  |  |  |  | 100 | 100 | 100 |  | 100 |  | 100 |
| n79 | n84 | 15 |  |  |  |  |  |  | 100 | 100 | 100 |  | 100 |  | 100 |
| n79 | n95 | 15 |  |  |  |  |  |  | 75 | 75 | 75 |  | 75 |  | 75 |
| n79 | n97 | 15 |  |  |  |  |  |  | 270 | 270 | 270 |  | 270 |  | 270 |
| n79 | n98 | 15 |  |  |  |  |  |  | 216 | 216 | 216 |  | 216 |  | 216 |
| NOTE 1: The Tx-Rx carrier center frequency separation between SUL band and DL band is the same as the Tx-Rx carrier center frequency separation of DL band specified in table 5.4.4-1 from TS 38.101-1. The channel bandwidth of SUL band is the same as DL band. This restriction of REFSENS configurations applies also for these carriers when applicable SUL configuration is part of a higher order configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<< End of changed sections >>>