3GPP TSG-RAN WG4 Meeting # 103-e	R4-2210878

# Electronic Meeting, May 9 – May 20, 2022

Source: 	MediaTek Inc.

Title: 	TP to update TS 38.101-5 clause 7.6.3 on OOBB

Agenda Item:	9.12.5.2

Document for:	Approval

Introduction

As indicated in WF [1] from RAN4#102-e, there was still necessary evaluation for n256 OOBB requirements. In the contribution, with the aid of evaluation from [2], the updates of out-of-band blocking requirements to TS 38.101-5 clause 7.6.3 are proposed.

Reference

[1] R4-2207416, “WF on NTN UE RF requirements”, ZTE

[2] R4-2208378, “Discussion on UE RX REFSENS and OOBB for band n256”, MediaTek Inc.

- 3 Text Proposal to TS 38.101-5

<<< Start of change >>>

- 7.6	Blocking characteristics

<<< Unchanged sections omitted >>>

7.6.3	Out-of-band blocking

For NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 15 MHz below or above the UE receive band.

The throughput of the wanted signal shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 38.101-1 Annexes A.2.2, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.3-1 and Table 7.6.3-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6.3-1: Out-of-band blocking parameters for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration2 | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + 9 dB |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Power in transmission bandwidth configuration shall be rounded to the next higher 0.5dB value. |  |  |  |  |

Table 7.6.3-2: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

|  | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | [-30] | -15 |
| n255, n256 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: For band n256 in Range 2 requirement, the applicable lower frequency range should be modified as -145 < f – FDL_low ≤ -[100]. NOTE 2: For band n256 in Range 3 requirement, the applicable lower frequency range should be modified as 1 ≤ f ≤ FDL_low –[145] |  |  |  |  |  |

For interferer frequencies across ranges 1, 2 and 3 in Table 7.6.3-1, a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz withthe number of resource blocks in the downlink transmission bandwidth configuration, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

<<< Unchanged sections omitted >>>

<<< End of change >>>