3GPP TSG-RAN WG4 Meeting # 103-e	R4-2210911

# Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR for FR2 HST High speed Train Scenarios (Annex B.3.4) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2-Perf |  |  |  |  |  | Date: |  |  | 04/25/2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce FR2 HST Channel model |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce FR2 HST Channel model |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing FR2 HST Channel Model |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2210083 |  |  |  |  |  |  |  |  |

<Start of Change #1>

## B.3.4	FR2 HST-DPS Channel Profile

There is an infinite number of RRHs distributed equidistantly along the railway track with the same Cell ID as illustrated in Figure B.3.4.1-1 for Unidirectional and Figure B.3.4.2-1 for Bidirectional.

The location of RRH k is given as:

xk=k*Ds+j*Dmin	(B.3.4.1)

where:	k∈[-∞,∞], j=sqrt(-1) and Dminis the distance between the RRHs and railway track, while Ds is the distance of two RRHs, both in meters.

The train location is denoted as:

y=a+j*0	(B.3.4.2)

where:	a∈[0,∞] and a means distance in meters, which means the train is right on the track. a=vt where v (m/s) is the moving speed of the train.

### B.3.4.1 Unidirectional Deployment Channel Profile

The FR2 HST DPS Unidirectional Deployment Channel Profile is a single tap propagation channel, switching transmission point between adjacent RRHs when the UE reaches a distance equal to DS_offset from the serving RRH as illustrated in Figure B.3.4.1-1:

Figure B.3.4.1-1: Unidirectional deployment of FR2 HST-DPS

RRH k is visible for the train only in the range:

(k-2)*Ds- Ds_offset≤a<k*Ds	(B.3.4.1.1)

However, RRH k is considered for PDSCH and PDCCH signal transmission only in the range:

(k - 1)*Ds-Ds_offset≤a<k*Ds- Ds_offset	(B.3.4.1.2)

Propagation delay difference are not considered between signals from different RRHs.

Power level Pk (dB) for the signal from each RRH equals to 0. Doppler shift fst=fd cosθt (Hz) is given by:

cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2,  0<t≤Dsv

cosθt=cosθt modDsv,  t>Ds/v											(B.3.4.1.3)

Doppler shift is given by equation B.3.4.1.3, where the required input parameters listed in table B.3.4.1-1 and the resulting Doppler shift shown in Figures [TBD] are applied for all frequency bands.

Table B.3.4.1-1: FR2 HST-DPS Unidirectional scenario

| Parameter | Value |
| --- | --- |
|  | HST-DPS-FR2-UNI-A |
| Ds | 700 m |
| Ds_offset | 10 m |
| Dmin | 10 m |
| v | 350 km/h |
| fd | 9722 Hz |

Static channel matrix will be used as defined in Annex B.1.

### B.3.4.2 Bidirectional Deployment Channel Profile

The FR2 HST DPS Bidirectional Deployment Channel Profile is a single tap propagation channel, switching transmission point between adjacent RRHs as illustrated in Figure B.3.4.2-1:

Figure B.3.4.2-1: Bidirectional deployment of FR2 HST-DPS

As shown in Figures B.3.4.2-1, RRH k is visible for the train only in the range:

k*Ds-Ds≤a<k*Ds+Ds	(B.3.4.2.1)

However, as shown in Figures B.3.3-3 and B.3.3-5, RRH k is considered for PDSCH and PDCCH signal transmission only in the range:

k-1*Ds≤a<k*Ds-Ds2; k*Ds+Ds2≤a<k+1*Ds	(B.3.4.2.2)

Note that UE is receiving PDSCH and PDCCH from the 2nd-nearest RRH. Propagation delay difference are not considered between signals from different RRHs.

Power level  (dB) for the signal from each RRH equals to 0. Doppler shift fst=fd cosθt (Hz) is given by:

cosθt=Ds-vtDmin2+Ds-vt2,  0<t≤(0.5*Ds)/v

cosθt=-vtDmin2+vt2,  (0.5*Ds)/v<t≤Ds/v

cosθt modDsv,    t>Ds/v													 (B.3.4.2.3)

Doppler shift is given by equation B.3.4.2.3, where the required input parameters listed in table B.3.4.2-1 and the resulting Doppler shift shown in Figures [TBD] are applied for all frequency bands.

Table B.3.4.2-1: FR2 HST-DPS Bidirectional scenario

| Parameter | Value |
| --- | --- |
|  | HST-DPS-FR2-BI-B |
|  | 700 m |
|  | 150 m |
|  | 350 km/h |
|  | 9722 Hz |

Static channel matrix will be used as defined in Annex B.1.

<End of Change #1>