3GPP TSG-RAN4 Meeting #103-e	R4-2211053

# Electronic meeting, May 9 – May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2304 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on PRS-RSRPP measurement period requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The measurement capability and reporting requirements for PRS-RSRPP are missing. - The PRS-RSRP measurement period can be reused including the requirements of reduced number of samples and Rx beam sweeping factor. No need to have the repeated description. - PRS-RSRP measurement requirements without gap can be reused to PRS-RSRPP measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the measurement capability and reporting requirements for PRS-RSRPP measurement. - Remove the repeated description and editor note in 9.9.6.5. - Update the PRS-RSRPP measurement requirements without gaps by referring to PRS-RSRP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement period requirements for the PRS-RSRPP measurement are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2208213 |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.9.6	PRS-RSRPP measurements

9.9.6.1	Introduction

The requirements in clause 9.9.6.5 shall apply when UE is performing PRS measurement in the configured MG and provided the UE has received a message from LMF via LPP requesting the UE to measure and report PRS-RSRPP measurements defined in TS 38.215 [4].

The requirements in clause 9.9.6.6 shall apply when UE is performing PRS measurement without gap and provided the UE has received a message from LMF via LPP requesting the UE to measure and report PRS-RSRPP measurements defined in TS 38.215 [4].

9.9.6.2	Requirements applicability

The requirements in clause 9.9.6 apply for periodic and triggered PRS-RSRPP measurements, provided:

-	PRS-RSRPP related side conditions given in clause 10.1.X are met for a corresponding Band.

9.9.6.3	Measurement capability

UE PRS-RSRPP measurement capability is as indicated by the UE in NR-DL-AoD-ProvideCapabilities according to TS 37.355 [34].

9.9.6.4	Measurement reporting requirements

This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes the delay uncertainty caused by inserting the measurement report into the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported PRS-RSRPP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.1.24.3.

The PRS-RSRPP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy requriements specified in the clauses 10.1.24.X.

9.9.6.5	Measurement period requirements

For PRS measurement within MG configured to UE, measurement period requirements for PRS-RSRP defined in 9.9.3.5 is re-used for PRS-RSRPP.

9.9.6.6	Measurement Period Requirements without Measurement Gaps

For PRS measurement without MG configured to UE, measurement period requirements for PRS-RSRP defined in 9.9.3.6 is re-used for PRS-RSRPP.

The PRS-RSRPP measurement requirements in this section apply for the first path PRS-RSRP measurement.

TPRS-RSRPP,total

TPRS-RSRPP,total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i

Teffect,i TPRS-RSRPP,i

TPRS-RSRPP,i=CSSFi*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

CSSFiCSSFi = FFS

NRxBeam,i NRxBeam,i NRxBeam,i

Lavailable_PRS,iTavailable_PRS,iLavailable_PRS,i

NPRS,islot

{N,T}

N’

NsampleNsample

TlastTiTavailable_PRS,i

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i

Ti

Tavailable_PRS,i= LCMTPRS,i,PPWRPiTPRS,iPPWRPi

TPRS,i

PPWRPi

TperPRS with muting=Nmuting*TperPRSTperPRS with mutingTPRS,i

TperPRS

Nmutingb1NmutingNmuting=TmutingPRS*LmutingTmutingPRSLmutingb1

TPRS-RSRPP,total

- Lavailable_PRS,i

<End of Change 1>