3GPP TSG-RAN WG4 Meeting #103-e	R4-2211223

Electronic Meeting, 9 ‒ 20 May, 2022

Title: 	WF on NR FR1/FR2 repeater measurement system set-up

Agenda Item:	     9.5.5.2, 9.5.5.3

Source: 	ZTE

Document for:	Approval

Topic #1: Measurement system set-up

Sub-topic#1-1 FR1 set-up baseline

Recommended agreement for FR1 set-up baseline:

- Take option 2 as the baseline for the 2nd round discussion.:

Recommended Agreement for out of band gain:

- to remove the 30dB precision attenuation

Sub-topic#1-2 FR1 TDD set-up

Recommended Agreement:

Only one link direction at a time is being tested

UL/DL timing could be provided through DL signals or cable to repeater under the test

- Other methods not precluded;

- FFS whether and if needed how to specify UL/DL timing into specification

Sub-topic#1-4: FR2 set-up baseline

Recommended agreement:

- For EIRP based measurement, option 1 (R4-2209606) could be starting point;

- Reverberation chamber is not applicable for FR2 repeater conformance testing; FFS for CATR, Near field chamber

- ACRR/out of band gain requirement, the output should be done with TRP measurement

- FFS on TRP measurement of FR2 repeater on OTA chamber;

Open issues:

Further discuss the following issues:

- TRP measurement with input antenna rotating together with repeater ;

- Testing chamber size with testing antenna ;

- Two calibration antenna for repeater within the OTA chamber ;

- Other approach should not be precluded e.g. moving measurement antenna or measure with measure with multiple test antennas.

Sub-topic# 1-5: FR2 set-up specific issues compared with baseline

Open issues:

Further discuss the following issues:

- TDD off power measurement;

- Potential issue/solution: Input signal may need to turn off.

- spurious emissions testing

- Potential issue/solution: isolation between input and output may need to be considered.

- Feasibility of the DUT moving together testing antenna

- The impact of feeding signal to measured signal, such as two calibrated directional paths in the chamber, the signal power impact, etc.