3GPP TSG-RAN WG4 Meeting #104-e	R4-2211640

# Electronic Meeting, August 15 – 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev | <Rev#> | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases for Intra- and inter-frequency CHO for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Perf |  |  |  |  |  | Date: |  |  | 2022-8-1 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test case for Intra- and inter-frequency SAN time-based conditional handover from FR1 to FR1 should be introduced for The test case for Intra- and inter-frequency SAN distance-based conditional handover from FR1 to FR1 should be introduced for |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing TC for Intra-frequency SAN time-based conditional handover from FR1 to FR1 in clause A.6.3.3.3. Introducing TC for inter-frequency SAN time-based conditional handover from FR1 to FR1 in clause A.6.3.3.4. Introducing TC for Intra-frequency SAN distance-based conditional handover from FR1 to FR1 in clause A.6.3.3.5. Introducing TC for inter-frequency SAN distance-based conditional handover from FR1 to FR1 in clause A.6.3.3.6. Only FDD 10MHz bandwidth is used in these test cases. Set UE position (N, S, H)=(0, 0, 0) by GNSS The ephemeris informatione is set as: - LEO 1200km height SAN by position (x, y, z)=( (6372.797+1200)*103 m, 0, 0) and velocity (Vx, Vy, Vz)=(0, 7560 m/s, 0) Set (TACommon, TACommonDrift, TACommonDriftVariation) = (0, 0, 0), i.e. the timing referent point is at satellite to simple test. Koffset =8ms coressponding to 1200km satellite height. Kmac = 0ms coressponding to the timing referent point is at satellite to simple test. For time-based conditional handover, Event CondEvent T1 is configured to UE. T1-1 = T2 start + 0.8 (Tidentify_intra_without_index) and T1-2 = T2 start + 1.2s. UE will complete identification of cell 2 at T1-1. For distance-based conditional handover, Event D1 is configured to UE. The UE moving speed is set to 50km/h by GNSS. The reference location is configured UE position at T1 start. D1-1 = 100m and D1-2 = 200m. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for Intra- and inter-frequency SAN conditinal handover will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section A.6.3.3.3, A.6.3.3.4, A.6.3.3.5, A.6.3.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

A.6.3.3.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1

A.6.3.3.3.1	Test Purpose and Environment

This test is to verify the requirement for Intra-frequency SAN time-based conditional Handover from FR1 to FR1 specified in clause 6.1C.2.

A.6.3.3.3.2	Test Parameters

The test scenario comprises of 1 NR FDD carrier and 2 cells as given in table A.6.3.3.3.2-1, and A.6.3.3.3.2-2. Both handover delay and interruption length are tested.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2. During T1, the UE is configured to measure intra-frequency neighbour cell. The RRC message implying time-based handover to cell 2 with Event CondEvent T1 shall be sent to UE, at a time earlier than TRRC before the beginning of T2.

Starting T2, cell 2 becomes detectable and offset better than cell 1.

Table A.6.3.3.3.2-1: General test parameters for Intra-frequency SAN time-based conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | FDD duplex mode cell |
|  | Neighbouring cell |  | Cell 2 | FDD duplex mode cell |
| Final condition | Active cell |  | Cell 2 |  |
| UE position (N,S, H) |  |  | (0, 0, 0) | Set by GNSS |
| T1-1 of CondEvent T1 |  | s | T2 + 0.8 | Entering condition |
| T1-2 of CondEvent T1 |  | s | T2 + 1.2 | Leaving condition |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

Table A.6.3.3.3.2-2: Cell specific test parameters for Intra-frequency SAN time-based conditional handover from FR1 to FR1

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
| BWP BW |  | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
| SAN positionX |  | m | (6372.797+1200)*103 |  | (6372.797+1200)*103 |  |
| SAN positionY |  | m | 0 |  | 0 |  |
| SAN positionZ |  | m | 0 |  | 0 |  |
| SAN velocityVX |  | m/s | 0 |  | 0 |  |
| SAN velocityVY |  | m/s | 7560 |  | 7560 |  |
| SAN velocityVZ |  | m/s | 0 |  | 0 |  |
| TACommon |  | s | 0 |  | 0 |  |
| TACommonDrift |  | s | 0 |  | 0 |  |
| TACommonDriftVariation |  | s | 0 |  | 0 |  |
| Koffset |  | ms | 8 |  | 8 |  |
| Kmac |  | ms | 0 |  | 0 |  |
| DRx Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  | SR.1.1 FDD |  |  |  |
| CORESET Reference Channel |  |  | CR.1.1 FDD |  |  |  |
| TRS configuration |  |  | TRS.1.1 FDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  |  | SSB.1 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 15 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 15 kHz |  |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  | dBm/ 15kHz | -98 |  |  |  |
| Note2 |  | dBm/SCS | -98 |  |  |  |
|  |  | dB | 8 | -3.3 | -Infinity | 2.36 |
|  |  | dB | 8 | 8 | -Infinity | 11 |
| SSB_RP |  | dBm/SCS | -90 | -90 | -Infinity | -87 |
| IoNote3 |  | dBm/ 9.36MHz | -61.41 | -57.06 | -61.41 | -57.06 |
| Propagation condition |  | - | AWGN |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.3.3.3.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 872 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay is defined in clause 6.1C.2, can be expressed as:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

TEvent_DU = start of T2

Tmeasure = 600 + 200 ms; Tinterrupt = 62ms; TCHO_execution = 10ms.

This gives a total of 872 ms.

A.6.3.3.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1

A.6.3.3.4.1	Test Purpose and Environment

This test is to verify the requirement for Inter -frequency SAN time-based conditional Handover from FR1 to FR1 specified in clause 6.1C.2.

A.6.3.3.4.2	Test Parameters

The test scenario comprises of 2 NR FDD carrier and one cell on each carrier as given in table A.6.3.3.4.2-1, and A.6.3.3.4.2-2. Both handover delay and interruption length are tested.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2. During T1, the UE is configured to measure inter-frequency neighbour cell and Gap pattern ID gp0. The RRC message implying time-based handover to cell 2 with Event CondEvent T1 shall be sent to UE, at a time earlier than TRRC before the beginning of T2.

Starting T2, cell 2 becomes detectable and offset better than cell 1.

Table A.6.3.3.4.2-1: General test parameters for Inter-frequency SAN time-based conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | FDD duplex mode cell |
|  | Neighbouring cell |  | Cell 2 | FDD duplex mode cell |
| Final condition | Active cell |  | Cell 2 |  |
| UE position (N,S, H) |  |  | (0, 0, 0) | Set by GNSS |
| T1-1 of CondEvent T1 |  | s | T2 + 0.8 | Entering condition |
| T1-2 of CondEvent T1 |  | s | T2 + 1.2 | Leaving condition |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

Table A.6.3.3.4.2-2: Cell specific test parameters for Inter-frequency SAN time-based conditional handover from FR1 to FR1

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  | 2 |  |
| BWchannel |  | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
| BWP BW |  | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
| SAN positionX |  | m | (6372.797+1200)*103 |  | (6372.797+1200)*103 |  |
| SAN positionY |  | m | 0 |  | 0 |  |
| SAN positionZ |  | m | 0 |  | 0 |  |
| SAN velocityVX |  | m/s | 0 |  | 0 |  |
| SAN velocityVY |  | m/s | 7560 |  | 7560 |  |
| SAN velocityVZ |  | m/s | 0 |  | 0 |  |
| TACommon |  | s | 0 |  | 0 |  |
| TACommonDrift |  | s | 0 |  | 0 |  |
| TACommonDriftVariation |  | s | 0 |  | 0 |  |
| Koffset |  | ms | 8 |  | 8 |  |
| Kmac |  | ms | 0 |  | 0 |  |
| DRx Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  | SR.1.1 FDD |  |  |  |
| CORESET Reference Channel |  |  | CR.1.1 FDD |  |  |  |
| TRS configuration |  |  | TRS.1.1 FDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  |  | SSB.1 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 15 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 15 kHz |  |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  | dBm/ 15kHz | -98 |  |  |  |
| Note2 |  | dBm/SCS | -98 |  |  |  |
|  |  | dB | 4 | 4 | -Infinity | 9 |
|  |  | dB | 4 | 4 | -Infinity | 9 |
| SSB_RP |  | dBm/SCS | -94 | -94 | -Infinity | -89 |
| IoNote3 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -70.05 | -60.53 |
| Propagation condition |  | - | AWGN |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.3.3.4.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 872 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay is defined in clause 6.1C.2, can be expressed as:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

TEvent_DU = start of T2

Tmeasure = 600 + 200 ms; Tinterrupt = 62ms; TCHO_execution = 10ms.

This gives a total of 872 ms.

A.6.3.3.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1

A.6.3.3.5.1	Test Purpose and Environment

This test is to verify the requirement for Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 specified in clause 6.1C.2.

A.6.3.3.5.2	Test Parameters

The test scenario comprises of 1 NR FDD carrier and 2 cells as given in table A.6.3.3.5.2-1, and A.6.3.3.5.2-2. Both handover delay and interruption length are tested.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2. During T1, the UE is configured to measure intra-frequency neighbour cell. The RRC message implying distance-based handover to cell 2 with Event D1 shall be sent to UE, at a time earlier than TRRC before the beginning of T2.

Starting T2, cell 2 becomes detectable and offset better than cell 1.

Table A.6.3.3.5.2-1: General test parameters for Intra-frequency SAN distance-based conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | FDD duplex mode cell |
|  | Neighbouring cell |  | Cell 2 | FDD duplex mode cell |
| Final condition | Active cell |  | Cell 2 |  |
| UE position (N,S, H) at T1 start |  |  | (0, 0, 0) | Set by GNSS |
| UE moving speed |  | km/s | 50 | Set by GNSS |
| reference location |  |  | (0, 0, 0) |  |
| D1-1 of Event D1 |  | m | 100 | Entering condition |
| D1-2 of Event D1 |  | m | 200 | Leaving condition |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 10 |  |
| T2 |  | s | 2 |  |

Table A.6.3.3.5.2-2: Cell specific test parameters for Intra-frequency SAN distance-based conditional handover from FR1 to FR1

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
| BWP BW |  | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
| SAN positionX |  | m | (6372.797+1200)*103 |  | (6372.797+1200)*103 |  |
| SAN positionY |  | m | 0 |  | 0 |  |
| SAN positionZ |  | m | 0 |  | 0 |  |
| SAN velocityVX |  | m/s | 0 |  | 0 |  |
| SAN velocityVY |  | m/s | 7560 |  | 7560 |  |
| SAN velocityVZ |  | m/s | 0 |  | 0 |  |
| TACommon |  | s | 0 |  | 0 |  |
| TACommonDrift |  | s | 0 |  | 0 |  |
| TACommonDriftVariation |  | s | 0 |  | 0 |  |
| Koffset |  | ms | 8 |  | 8 |  |
| Kmac |  | ms | 0 |  | 0 |  |
| DRx Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  | SR.1.1 FDD |  |  |  |
| CORESET Reference Channel |  |  | CR.1.1 FDD |  |  |  |
| TRS configuration |  |  | TRS.1.1 FDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  |  | SSB.1 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 15 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 15 kHz |  |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  | dBm/ 15kHz | -98 |  |  |  |
| Note2 |  | dBm/SCS | -98 |  |  |  |
|  |  | dB | 8 | -3.3 | -Infinity | 2.36 |
|  |  | dB | 8 | 8 | -Infinity | 11 |
| SSB_RP |  | dBm/SCS | -90 | -90 | -Infinity | -87 |
| IoNote3 |  | dBm/ 9.36MHz | -61.41 | -57.06 | -61.41 | -57.06 |
| Propagation condition |  | - | AWGN |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.3.3.5.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 872 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay is defined in clause 6.1C.2, can be expressed as:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

TEvent_DU = start of T2

Tmeasure = 600 + 200 ms; Tinterrupt = 62ms; TCHO_execution = 10ms.

D1-1 time = (50km/h*1000/3600)/(100m +50m) = 10.8s from T1 start.

This gives a total of 10.8s – 10s + 62ms + 10ms = 872 ms.

A.6.3.3.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1

A.6.3.3.6.1	Test Purpose and Environment

This test is to verify the requirement for Inter -frequency SAN distance-based conditional Handover from FR1 to FR1 specified in clause 6.1C.2.

A.6.3.3.6.2	Test Parameters

The test scenario comprises of 2 NR FDD carrier and one cell on each carrier as given in table A.6.3.3.6.2-1, and A.6.3.3.6.2-2. Both handover delay and interruption length are tested.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2. During T1, the UE is configured to measure inter-frequency neighbour cell and Gap pattern ID gp0. The RRC message implying distance-based handover to cell 2 with Event D1 shall be sent to UE, at a time earlier than TRRC before the beginning of T2.

Starting T2, cell 2 becomes detectable and offset better than cell 1.

Table A.6.3.3.6.2-1: General test parameters for Inter -frequency SAN distance-based conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | FDD duplex mode cell |
|  | Neighbouring cell |  | Cell 2 | FDD duplex mode cell |
| Final condition | Active cell |  | Cell 2 |  |
| UE position (N,S, H) at T1 start |  |  | (0, 0, 0) | Set by GNSS |
| UE moving speed |  | km/s | 50 | Set by GNSS |
| reference location |  |  | (0, 0, 0) |  |
| D1-1 of Event D1 |  | m | 100 | Entering condition |
| D1-2 of Event D1 |  | m | 200 | Leaving condition |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 10 |  |
| T2 |  | s | 2 |  |

Table A.6.3.3.6.2-2: Cell specific test parameters for Inter-frequency SAN distance-based conditional handover from FR1 to FR1

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  | 2 |  |
| BWchannel |  | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
| BWP BW |  | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
| SAN positionX |  | m | (6372.797+1200)*103 |  | (6372.797+1200)*103 |  |
| SAN positionY |  | m | 0 |  | 0 |  |
| SAN positionZ |  | m | 0 |  | 0 |  |
| SAN velocityVX |  | m/s | 0 |  | 0 |  |
| SAN velocityVY |  | m/s | 7560 |  | 7560 |  |
| SAN velocityVZ |  | m/s | 0 |  | 0 |  |
| TACommon |  | s | 0 |  | 0 |  |
| TACommonDrift |  | s | 0 |  | 0 |  |
| TACommonDriftVariation |  | s | 0 |  | 0 |  |
| Koffset |  | ms | 8 |  | 8 |  |
| Kmac |  | ms | 0 |  | 0 |  |
| DRx Cycle |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  | SR.1.1 FDD |  |  |  |
| CORESET Reference Channel |  |  | CR.1.1 FDD |  |  |  |
| TRS configuration |  |  | TRS.1.1 FDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  |  | SSB.1 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 15 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 15 kHz |  |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  | dBm/ 15kHz | -98 |  |  |  |
| Note2 |  | dBm/SCS | -98 |  |  |  |
|  |  | dB | 4 | 4 | -Infinity | 9 |
|  |  | dB | 4 | 4 | -Infinity | 9 |
| SSB_RP |  | dBm/SCS | -94 | -94 | -Infinity | -89 |
| IoNote3 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -70.05 | -60.53 |
| Propagation condition |  | - | AWGN |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.3.3.6.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 872 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay is defined in clause 6.1C.2, can be expressed as:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

TEvent_DU = start of T2

Tmeasure = 600 + 200 ms; Tinterrupt = 62ms; TCHO_execution = 10ms.

D1-1 time = (50km/h*1000/3600)/(100m +50m) = 10.8s from T1 start.

This gives a total of 10.8s – 10s (T1) + 62ms + 10ms = 872 ms.