3GPP TSG-RAN WG4 Meeting #104-e	R4-2211642

Electronic Meeting, August 15 – 26, 2022

Title: 	Discussion on Rel-18 multi Rx pannel RRM

Source: 	CATT

Agenda item:	11.8.3

Document for:	Discussion

- Introduction

A new RAN4 leading work item for requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception was approved in RAN#95 [1] and the revised WID was approved in RAN#96 [2]. The core part objectives of the work item are as following:

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO

  - Enhanced RF requirements:

    - Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs

    - The legacy spherical coverage requirement for reception from a single direction will be kept

    - PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier

  - Enhanced RRM requirements:

    - The following requirements should be studied and specified if necessary:

      - L1-RSRP measurement delay

      - L3 measurement delay (both cell detection delay and measurement period can be considered)

        - The starting point is the enhancements related to L1-RSRP measurement enhancements

      - RLM and BFD/CBD requirements

      - Scheduling/measurement restrictions

      - TCI state switching delay with dual TCI

      - Receive timing difference between different directions (different QCL Type D RSs)

NOTE:

  - The case of single TCI is handled as a second priority. Additional aspects related to single TCI can be further revisited.

This document will preliminary discuss the RRM requirements for FR2 multi-Rx chain DL reception, and give our understanding and proposals.

- Discussion

To our understanding, the mainly impact on RRM requirements is cell search delay for FR2-1 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO. That is, since the UE can simultaneously perform DL reception in two different directions, the scaling factor of 8 for FR2-1 for beam sweeping can be reduced to 4.

Observation1: The scaling factor for FR2-1 for beam sweeping can be reduced because the UE can simultaneously perform DL reception in two different directions.

In RRC-IDLE state, the scaling factor N1 is defined for FR2 for following requirements:

- Measurement and evaluation of serving cell in RRC-IDLE state

- Measurements of intra-frequency NR cells

- Measurements of inter-frequency NR cells

- Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion

- Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion

- Idle Mode CA/DC Measurements

They are defined as following and N1 may be modified according to the following example:

Table 4.2.2.2-1: Nserv

| DRX cycle length [s] | Scaling Factor (N1) |  | Nserv [number of DRX cycles] |
| --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |
| 0.32 | 1 | 8 -> [4] | M1*N1*4 |
| 0.64 |  | 5 -> [3] | M1*N1*4 |
| 1.28 |  | 4 -> [2] | N1*2 |
| 2.56 |  | 3 – [2] | N1*2 |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1 or 5, N1 = 8 -> [4] for all DRX cycle length. |  |  |  |

The scaling factor N1 is also defined for RRM requirements for RRC_INACTIVE state as for RRC-IDLE state, which can be similarly modified may be modified similar.

Observation 2: The scaling factor N1 = 8/5/4/3/2 defined for FR2 RRM requirements for RRC-IDLE state and for RRC_INACTIVE state in different DRX cycle length may be modified as N1 = [4/3/2/2/1] for UE supporting simultaneous DL reception in two different directions.

Whether to apply the reduced N1 for high speed train UE needs further discussion. To our understanding, it should apply if the high speed train UE indicates it supporting simultaneous DL reception in two different directions.

Observation 3: Whether to apply the reduced N1 for high speed train UE needs further discussion.

In section of RRC_CONNECTED state mobility, the search time requirements for handover to FR2 unknown cell are defined as “If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 8* Trs  ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 8*3* Trs  ms”. Here the 8 is for beam sweep, it may be reduced to 4.

Observation 4: The search time requirements for handover to FR2 unknown cell may be reduced for UE supporting simultaneous DL reception in two different directions.

For SSB based radio link monitoring, the scaling factor N=8 is also defined for TEvaluate_out_SSB and TEvaluate_in_SSB requirement for FR2 SSB based radio link monitoring. It may be reduced to [4].

For CSI-RS based RLM, such as following measurement restrictions are defined:

“For FR2, when the CSI-RS for RLM measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band,

-	In the following cases, UE is required to measure one of but not both CSI-RS for RLM and the other CSI-RS. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined.

-	The CSI-RS for RLM or the other CSI-RS in a resource set configured with repetition ON, or

-	The other CSI-RS is configured in q1 and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE,

-	Otherwise, UE shall be able to measure the CSI-RS for RLM without any restriction.”

The above measurement restriction may not be need for same CC for UE supporting simultaneous DL reception with two different directions.

Observation 5: The scaling factor N=8 may be reduced to [4] for TEvaluate_out_SSB and TEvaluate_in_SSB requirement for FR2 SSB based radio link monitoring for UE supporting simultaneous DL reception in two different directions.

Observation 6: The measurement restrictions for measurement two CSI-RSs in same OFDM symbol on same CC may not be need for FR2 CSI-RS based radio link monitoring for UE supporting simultaneous DL reception in two different directions.

For SCell activation and deactivation delay, the activation delay requirements for FR2 unknown SCell cell are including cell search time as 8*Trs. It may be reduced to [4] for for UE supporting simultaneous DL reception with two different directions.

Observation 7: The search time requirements for FR2 unknown cell in SCell activation may be reduced for UE supporting simultaneous DL reception in two different directions.

Also same justification as RLM, scaling factor N=8 may be reduced in requirements of evaluation period TEvaluate_BFD_SSB for FR2 for SSB based beam failure detection. The measurement restrictions for measurement two CSI-RSs in same OFDM symbol on same CC may not be need for FR2 CSI-RS based beam failure detection for UE supporting simultaneous DL reception with two different directions.

Observation 8: The scaling factor N=8 may be reduced in requirements of evaluation period TEvaluate_BFD_SSB for FR2 for SSB based beam failure detection for UE supporting simultaneous DL reception in two different directions.

Observation 9: The measurement restrictions for measurement two CSI-RSs in same OFDM symbol on same CC may not be need for FR2 CSI-RS based beam failure detection for UE supporting simultaneous DL reception in two different directions.

In Requirements for SSB based candidate beam detection; Requirements for CSI-RS based candidate beam detection, scaling factor N=8 also may be reduced.

Observation 10: In Requirements for SSB based candidate beam detection, the scaling factor N=8 in the requirements for CSI-RS based candidate beam detection may also be reduced.

In PSCell Addition and Release Delay, the search time requirements for FR2 unknown cell may be reduced for UE supporting simultaneous DL reception with two different directions.

Observation 11: In PSCell Addition and Release Delay, the search time requirements for FR2 unknown cell may be reduced for UE supporting simultaneous DL reception in two different directions.

In measurement requirements, they include also scaling factors for beam sweep, such as Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps. For UE supporting simultaneous DL reception with two different directions, they should also be considered to reduce.

Observation 12: The scaling factors for beam sweeping in FR2 measurement requirements, such as Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps, should also be considered to reduce for UE supporting simultaneous DL reception in two different directions.

- Conclusion

This document discussed preliminary what RRM requirements will be impacted for NR FR2 UE with multi-Rx chain DL reception and presented following observations:

Observation1: The scaling factor for FR2-1 for beam sweeping can be reduced because the UE can simultaneously perform DL reception in two different directions.

Observation 2: The scaling factor N1 = 8/5/4/3/2 defined for FR2 RRM requirements for RRC-IDLE state and for RRC_INACTIVE state in different DRX cycle length may be modified as N1 = [4/3/2/2/1] for UE supporting simultaneous DL reception in two different directions.

Observation 3: Whether to apply the reduced N1 for high speed train UE needs further discussion.

Observation 4: The search time requirements for handover to FR2 unknown cell may be reduced for UE supporting simultaneous DL reception in two different directions.

Observation 5: The scaling factor N=8 may be reduced to [4] for TEvaluate_out_SSB and TEvaluate_in_SSB requirement for FR2 SSB based radio link monitoring for UE supporting simultaneous DL reception in two different directions.

Observation 6: The measurement restrictions for measurement two CSI-RSs in same OFDM symbol on same CC may not be need for FR2 CSI-RS based radio link monitoring for UE supporting simultaneous DL reception in two different directions.

Observation 7: The search time requirements for FR2 unknown cell in SCell activation may be reduced for UE supporting simultaneous DL reception in two different directions.

Observation 8: The scaling factor N=8 may be reduced in requirements of evaluation period TEvaluate_BFD_SSB for FR2 for SSB based beam failure detection for UE supporting simultaneous DL reception in two different directions.

Observation 9: The measurement restrictions for measurement two CSI-RSs in same OFDM symbol on same CC may not be need for FR2 CSI-RS based beam failure detection for UE supporting simultaneous DL reception in two different directions.

Observation 10: In Requirements for SSB based candidate beam detection, The scaling factor N=8 in the requirements for CSI-RS based candidate beam detection may alsobe reduced.

Observation 11: In PSCell Addition and Release Delay, the search time requirements for FR2 unknown cell may be reduced for UE supporting simultaneous DL reception with two different directions.

Observation 12: The scaling factors for beam sweeping in FR2 measurement requirements, such as Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps should also be considered to reduce for UE supporting simultaneous DL reception in two different directions.

- Reference

[1]	RP-220974, New WI: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm

[2]	RP-221753, Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL, Qualcomm