3GPP TSG-RAN WG4 Meeting #104-e                                                                                                      R4-2211645

Electronic Meeting, August 15 – August 26, 2022

Source: 	CATT

Title: 	Discussion on general part of conformance requirements for 52.6-71GHz

Agenda item:	9.14.5.1

Document for:	Approval

- Introduction

In RAN4#103-e, a WF [1] BS RF conformance testing for FR2-2 was approved. There are some the remaining issues in the WF need to be further discussed

In this contribution, we provide our views on the remaining issues for conformance testing in the WF.

- Discussion

Test configuration signal for FR2-2

As per the WF [1], the remaining issues concerning test configuration signal for FR2-2 are shown as below:

Issue 3-5-1: test configuration signal for FR2-2 - Option 1: to define the TC signals for 52.6-71GHz as shown in Table Table 4.7.2.1-2a. (R4-2209590) Table 4.7.2.1-2a: Signal to be used to build NR TCs for FR2-2 BS type 2-O Operating band characteristics; FDL_high – FDL_low ≤ 5000 MHz. TC signal; BWchannel; 400 MHz (Note 1, Note 2). characteristics; Subcarrier spacing; Smallest supported subcarrier spacing declared per operating band (D.7). NOTE 1: BS vendor can decide to test with 100 MHz BS channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 400 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 100 MHz. NOTE 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used.. NOTE: 400MHz has already been agreed as mandatory channel bandwidth for FR2-2 in main session. - Option 2: other options

Referring to transmission bandwidth configuration NRB for FR2-2 defined in TS38.104, the 100MHz CBW wasn’t supported by 480kHz and 960kHz SCS, if smallest supported SCS declared per operating band is 480kHz or 960kHz, note 1 in option 1 isn’t applicable, so it need to be updated as follows:

NOTE 1:    If smallest supported SCS declared per operating band (D.7) is120kHz, BS vendor can decide to test with 100 MHz BS channel bandwidth instead of 400 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 100 MHz.

Table 9.7.1-1: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics (MHz) | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-O | FDL,high – FDL,low < 100 | 10 |
|  | 100 MHz ≤ FDL,high – FDL,low ≤ 900 | 40 |
| BS type 2-O | FDL,high – FDL,low ≤ 4000 | 1500 |
|  | 4000 < FDL,high – FDL,low ≤ 14000 | 3500 |

About operating band characteristics in table of option1, we can refer to Operating band characteristics for ΔfOBUE in above Table 9.7.1-1 of TS 38.104, FDL,high – FDL,low≤ 14000 is applicable for FR2-2. So, we propose,

Proposal 1: Define independent TC signals for FR2-2 as shown in following Table 4.7.2.1-2a.

Table 4.7.2.1-2a: Signal to be used to build NR TCs for BS type 2-O in FR2-2

| Operating band characteristics |  | FDL_high – FDL_low ≤ 14000 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 400 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: If smallest supported SCS declared per operating band (D.7) is 120 kHz, BS vendor can decide to test with 100 MHz BS channel bandwidth instead of 400 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 100 MHz. NOTE 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |

Applicable test configurations for FR2-2

As per the WF [1], the remaining issues concerning applicable test configurations for FR2-2 are shown as below:

Issue 3-5-2: applicable test configurations for FR2-2 - Option 1: only NRTC1 and NRTC2 is applicable for 52.6-71GHz (R4-2209590) - Option 2: other options

According decision of RAN meeting, non-contiguous spectrum operation in FR2-2 is not supported. Currently, the multi-band is not supported by BS type 2-O in FR2-1 and FR2-2. In existing test configuration, NRTC3 is for non-contiguous spectrum operation, and NRTC4 and NRTC5 are for multi-band operation, so NRTC3, NRTC4 and NRTC5 are not applicable for FR2-2. And only NRTC1 and NRTC2 in existing test configurations can be applicable for FR2-2. So option 1 is reasonable.

Proposal 2: Adopt option 1: only NRTC1 and NRTC2 is applicable for 52.6-71GHz (R4-2209590)

Test cases for FR2-2

As per the WF [1], the remaining issues concerning test cases for FR2-2 are shown as below:

Issue 3-5-3: test cases for FR2-2 - Option 1: agree to reuse the existing test case for FR2-1 NR BS conformance testing for FR2-2. (R4-2209590) Moderator: In the Tdoc this proposals refers to placement of RF channels - Option 2: other options

For FR2-2, the single carrier, multi-carrier, and contiguous spectrum operation are supported in current specification, RF channels defined in TS 28.141 support test cases for single carrier, multi-carrier and contiguous spectrum operation for FR2-2.  So option 1 is reasonable.

Proposal 3: Support Option 1: agree to reuse the existing test case for FR2-1 NR BS conformance testing for FR2-2. (R4-2209590)

Test model

As per the WF [1], the remaining issues concerning test model for FR2-2 are shown as below:

Issue 3-6-2: Test model data length Agreement: - Have the same length on EVM measurement time length and Test model data length

In TS 38.141-2, the duration for all NR FR2 test model is 2 radio frames for TDD (20 ms), and maximum supported SCS is 120kHz. The NR test model for120kHz has been used for BS testing, the testing time and storage memory of data content for 120kHz is acceptable.  We can scale down the duration for all NR FR2-2 test models for 480 kHz SCS and 960 kHz SCS to make number of sampling points in data content for 480kHz SCS and 960kHz SCS equal to that for120kHz.  Because the sampling frequency of 480kHz and 960kHz is 4 and 8 times that of 120kHz, respectively. So the duration for 480kHz and 960kHz for test model can scale down to 5ms and 2.5ms, respectively.

Proposal 4: Adopt 5ms for test model data length for FR2-2.

- Conclusion

This contribution provides analysis on general part of conformance testing for 52.6-71GHz. The following proposals are concluded as follows:

Proposal 1: Define independent TC signals for FR2-2 as shown in following Table 4.7.2.1-2a.

Table 4.7.2.1-2a: Signal to be used to build NR TCs for BS type 2-O in FR2-2

| Operating band characteristics |  | FDL_high – FDL_low ≤ 14000 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 400 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: If smallest supported SCS declared per operating band (D.7) is 120 kHz, BS vendor can decide to test with 100 MHz BS channel bandwidth instead of 400 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 100 MHz. NOTE 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |

Proposal 2: Adopt option 1: only NRTC1 and NRTC2 is applicable for 52.6-71GHz (R4-2209590)

Proposal 3: Support Option 1: agree to reuse the existing test case for FR2-1 NR BS conformance testing for FR2-2. (R4-2209590)

Proposal 4: Adopt 5ms for test model data length for FR2-2.

- References

- R4-2210577, WF on system parameters for NR extension to 71GHz, Intel Corporation, RAN4#103-e

- TS 38.104 v17.6.0