3GPP TSG-RAN WG4 Meeting #104-e                                                                                      R4-2211662

Electronic Meeting, August 15 –August 26 2022

Source: 	CATT

Title: 	Discussion on Satellite Access Node output power testing

Agenda item:	9.11.3.2.1

Document for:	Approval

- Introduction

In RAN4 #103-e meeting, A WF [1] on NTN SAN RF conformance was agreed. There are some agreements and some the remaining issues concerning test procedure in the WF are shown as below:

| Issue 2-1-5 | Test procedure for SAN type 1-H. It seems common understanding to start from the test procedures described in TS 38.141-1 Way forward: Discussions will be started at the time where we will see some concrete text proposals. |
| --- | --- |

In this contribution, we provide our views on satellite access node output power test procedure.

- Discussion

Concrete text proposals for test configurations in Annex

Based on our proposed manufacturer declarations and measurement uncertainties for conducted SAN testing, our concrete text proposals for satellite access node output power are attached in Annex for agreement.

Proposal 1: Our concrete text proposals for satellite access node output power based on our proposed manufacturer declarations and measurement uncertainties for conducted SAN testing are attached in Annex for agreement.

- Conclusion

This contribution provides our general consideration on satellite access node output power. The following proposals are concluded as follows:

Proposal 1: Our concrete text proposals for satellite access node output power based on our proposed manufacturer declarations and measurement uncertainties for conducted SAN testing are attached in Annex for agreement.

- References

- R4-2210634, WF on NTN SAN RF conformance, Ericsson, RAN4#103-e

- Annex

6	Conducted transmitter characteristics

6.1	General

6.1.2	SAN type 1-H

General test conditions for conducted transmitter tests are given in clause 4, including interpretation of measurement results and configurations for testing. SAN configurations for the tests are defined in clause 4.5.

If a number of single-band connectors have been declared equivalent (D.13), only a representative one is necessary to be tested to demonstrate conformance.

6.2 	Satellite Access Node output power

6.2.1	Definition and applicability

The conducted SAN output power requirements are specified at single-band connector.

The rated carrier output power of the SAN type 1-H shall be as specified in table 6.2.1-1.

Table 6.2.1-1: SAN type 1-H rated output power limits for SAN classes

| SAN class | Prated,c,sys (NOTE) | Prated,c,TABC (NOTE) |
| --- | --- | --- |
| SAN GEO class | Prated,c,sys,GEO | Prated,c,TABC,GEO |
| SAN LEO class | Prated,c,sys,LEO | Prated,c,TABC,LEO |
| NOTE: Prated,c,sys or Prated,c,TABC of SAN shall be based on manufacturer declaration and comply with regulation requirement. |  |  |

The output power limit for the respective SAN classes in table 6.2.1.-1 shall be compared to the rated output power and the declared SAN class. It is not subject to testing.

6.2.2	Minimum requirement

The minimum requirement applies per single-band connector supporting transmission in the operating band.

The minimum requirement for SAN type 1-H is defined in TS 38.108 [2], clause 6.2.2.

6.2.3	Test purpose

The test purpose is to verify the accuracy of the maximum carrier output power across the frequency range and under normal and extreme conditions.

6.2.4	Method of test

6.2.4.1	Initial conditions

Test environment:

-	Normal, see annex B.2,

-	Extreme, see annexes B.3 and B.5.

RF channels to be tested for single carrier:	B, M and T; see clause 4.9.1

SAN RF Bandwidth positions to be tested for multi-carrier:

-	BRFBW, MRFBW and TRFBW for single-band connector(s), see clause 4.9.1.

Under extreme test environment, it is sufficient to test on one NR-ARFCN or one RF bandwidth position, and with one applicable test configuration defined in clauses 4.7 and 4.8. Testing shall be performed under extreme power supply conditions, as defined in Annex B.5.

NOTE:	Tests under extreme power supply conditions also test extreme temperatures.

6.2.4.2	Procedure

For SAN type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.3.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the power measuring equipment to single-band connector(s) or to multi-band connector(s) under test as shown in annex D.3.1 for SAN type 1-H. All connectors not under test shall be terminated.

2)	For single carrier set the connector under test to transmit according to the applicable test configuration in clause 4.8 using the corresponding test models or set of physical channels in clause 4.9.2 at rated carrier output power Prated,c,TABC for SAN type 1-H (D.10).

For a connector under test declared to be capable of multi-carrier (D.8) set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models or set of physical channels in clause 4.9.2.

3)	Measure the maximum carrier output power (Pmax,c,TABC for SAN type 1-H) for each carrier at each connector under test.

6.2.5	Test requirement

For each single-band connector under test, the power measured in clause 6.2.4.2 in step 3 shall remain within the values provided in table 6.2.5-1 for normal and extreme test environments, relative to the manufacturer's declared Prated,c,TABC for SAN type 1-H (D.10):

Table 6.2.5-1: Test requirement for conducted SAN output power

|  | Normal test environment | Extreme test environment |
| --- | --- | --- |
| SAN type 1-H, | f ≤ 3.0 GHz: ± 2.7 dB | f ≤ 3.0 GHz: ± 3.2 dB |