3GPP TSG-RAN WG4 Meeting # 104-e		R4-2211675

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.7.2.3

Source: 	CATT

Title: 	Discussion on maintenance for measurement procedure requirements for HST FR2

Document for:	Discussion

Introduction

RAN4 has completed UE RRM requirements for HST FR2. However, there are still some open issues for RRM core requirements for HST FR2. WF on RRM requirements for NR HST FR2 was approved in [1].

In this contribution, we provide our consideration of open issues and give our proposals.

Discussion

In last meeting, SMTC limit related CR was discussed. Companies have different views on whether to add the SMTC limit in the heading of the table.

Issue 2-1-3: SMTC limit in HST FR2 enhanced requirements GtW Agreement: Option 1 agreed as starting point and further work on the drafting CR revision including table heading and note 3. Way forward: Further discussion is needed on how to define HST FR2 requirements if SMTC periodicity is > 40 ms. - Option 1: Apply the FR2 HST enhanced requirement only when SMTC <=40ms cases. When SMTC period > 40ms, requirements in Table 9.2.5.2-2 apply. - Option 2: Delete NOTE 3, keep table titles without changes and set M2 = 1.5 - Other options are not precluded

In previous meetings, it is agreed that HST requirements are enhanced when DRX≤80ms by using M1= 6 and 18 for set1 and set2 respectively.

And it is also agreed that HST requirements are enhanced when SMTC<=40ms.

In RAN4@102-e meeting, it is agreed that no enhanced requirement should be applied to other than PC6 UEs even when HST FR2 flags are configured.

Agreement: - No enhanced requirement should be applied to other than PC6 UEs even when HST FR2 flags are configured. - PC6 shall be used to identify the feature support of HST FR2 operation.

Therefore, the table 9.2.5.1-11&9.2.5.1-2 should be only applicable for power class 6.

Table 9.2.5.1-11: Time period for PSS/SSS detection when [highSpeedMeasFlagFR2-r17] is configured, (Frequency range FR2) when SMTC period <= 40ms

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 80ms | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80ms< DRX cycle≤ 320ms | ceil(1.5 x Mpss/sss_sync_w/o_gaps Note 3 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps Note 3 x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For UE supporting power class 6, M1 = 6 if [highSpeedMeasFlagFR2-r17 = set1] or M1 = 18 if [highSpeedMeasFlagFR2-r17 = set2] NOTE 3: Mpss/sss_sync_w/o_gaps =24. |  |

Table 9.2.5.2-7: Measurement period for intra-frequency measurements without gaps when [highSpeedMeasFlagFR2-r17] is configured (FR2) when SMTC period <= 40ms

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 80ms | max(400ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80ms< DRX cycle≤ 320ms | ceil(1.5 x Mmeas_period_w/o_gaps Note 3 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps Note 3 xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For UE supporting power class 6, M1 = 6 if [highSpeedMeasFlagFR2-r17 = set1] or M1 = 18 if [highSpeedMeasFlagFR2-r17 = set2] NOTE 3: Mmeas_period_w/o_gaps =24. |  |

In previous meetings, it is agreed that HST requirements are enhanced when DRX≤80ms by using M1= 6 and 18 for set1 and set2 respectively. When DRX>80ms, we think the legacy requirements should be used. In legacy requirements, Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps are used. They are defined by different power class such as:

Mpss/sss_sync_w/o_gaps : For a UE supporting FR2 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2 power class 4, Mpss/sss_sync_w/o_gaps =24 Mmeas_period_w/o_gaps : For a UE supporting power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24.

There is no such definition for power class 6 in HST FR2 in legacy requirements.

Proposal 1: In Table 9.2.5.1-11 & Table 9.2.5.2-7, use Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps as 24 for power class 6 when DRX cycle > 80ms. The same principle is applied in intra-frequency measurement with measurement gaps.

For SMTC limitation,

Proposal 2: support option 1: Apply the FR2 HST enhanced requirement only when SMTC <=40ms cases. When SMTC period > 40ms, requirements in Table 9.2.5.2-2 apply. The same principle is applied in intra-frequency measurement with measurement gaps.

In RAN4@102-e meeting, it is agreed that no enhanced requirement should be applied to other than PC6 UEs even when HST FR2 flags are configured.

Agreement: - No enhanced requirement should be applied to other than PC6 UEs even when HST FR2 flags are configured. - PC6 shall be used to identify the feature support of HST FR2 operation.

Therefore, when HST FR2 flags are configured for other power classes other than PC6, the legacy tables should be used.

Proposal 3: When HST FR2 flags are configured for other power classes other than PC6, the legacy requirements should be used.

The CR can be found in [2].

In last meeting, L1-SINR measurements are still open as listed in [1]:

Issue 2-1-6: L1-SINR measurements with SSB-based CMR and dedicated IMR Way forward: Open issue needs further discussion - Option 1: For L1-SINR measurements with SSB-based CMR and dedicated IMR configured for FR2 HST, the same enhancements as SSB-based L1-RSRP measurements should be applied. - Option 2: Do not define enhancement for L1-SINR - Other options are not precluded

SSB-based L1-RSRP measurements are enhanced for the measurement periods. The behavior for the measurement periods for L1-RSRP and L1-SINR are similar. So we think the same enhancements can be applied for L1-SINR measurements with SSB-based CMR and dedicated IMR configured for FR2 HST.

Proposal 4: For L1-SINR measurements with SSB-based CMR and dedicated IMR configured for FR2 HST, the same enhancements as SSB-based L1-RSRP measurements should be applied

Conclusion

In this contribution, we provide our consideration of open issues for HST FR2 RRM requirements and our proposals are:

Proposal 1: In Table 9.2.5.1-11 & Table 9.2.5.2-7, use Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps as 24 for power class 6 when DRX cycle > 80ms. The same principle is applied in intra-frequency measurement with measurement gaps.

Proposal 2: support option 1: Apply the FR2 HST enhanced requirement only when SMTC <=40ms cases. When SMTC period > 40ms, requirements in Table 9.2.5.2-2 apply. The same principle is applied in intra-frequency measurement with measurement gaps.

Proposal 3: When HST FR2 flags are configured for other power classes other than PC6, the legacy requirements should be used.

Proposal 4: For L1-SINR measurements with SSB-based CMR and dedicated IMR configured for FR2 HST, the same enhancements as SSB-based L1-RSRP measurements should be applied

Reference

[1] R4-2210608, WF on HST FR2 RRM Core Requirement Maintenance, Nokia

[2] R4-2211676, CR on RRM core requirements for measurement procedure requirements for HST FR2, CATT