3GPP TSG-RAN4 Meeting #104-e	R4-2211738

# Electronic meeting, Aug. 15 – Aug. 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test set 4-23: UE Rx-Tx accuracy test case with reduced number of samples in FR1 in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-08-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - UE Rx-Tx time difference accuracy requirements with reduced number of samples in RRC_INACTIVE are defined in R17 ePOS WI and the corresponding test case needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce test set 4-23, UE Rx-Tx time difference measurement accuracy test case with reduced number of samples in FR1 in RRC_INACTIVE in TS 38.133. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE Rx-Tx time difference measurement accuracy test case with reduced number of samples in FR1 in RRC_INACTIVE is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New A.6.X2.Y3.Z2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.6.X2	Measurement performance requirements in RRC_INACTIVE state

A.6.X2.Y3	UE Rx-Tx time difference measurements

A.6.X2.Y3.Z2	UE Rx-Tx time difference measurement accuracy with reduced number of samples

A.6.X2.Y3.Z2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy with reduced number of samples in RRC_INACTIVE state is within the specified limits. This test will verify the requirements in clause 10.1.25.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in Table A.6.X2.Y3.Z2.1-1.

Table A.6.X2.Y3.Z2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE before the start of the test. The UE is configured to measure UE Rx-Tx time difference using reduced number of samples via requestedDL-PRS-ProcessingSamples in NR-Multi-RTT-RequestLocationInformation.

UE shall be configured to enter into RRC_INACTIVE state before the start of the test.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

A.6.X2.Y3.Z2.2	Test parameters

The UE Rx-Tx time difference accuracy test parameters are given in Table A.6.X2.Y3.Z2.2-1.

Table A.6.X2.Y3.Z2.2-1: UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit |  | Test configuration | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| RF Channel Number |  |  | 1,2,3 | 1 | 1 | 1 | 1 |
| DRX |  |  | 1,2,3 | DRX.8 |  | DRX.8 |  |
| Time offset with Cell 1 | s |  | 1, 2, 3 | N/A | 3 | N/A | 3 |
| TDD configuration |  |  | 1 | N/A | N/A | N/A | N/A |
|  |  |  | 2 | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  |  | 3 | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  |  | 1 | SR.1.1 FDD | N/A | SR.1.1 FDD | N/A |
|  |  |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  |  | 1 | CR.1.1 FDD | N/A | CR.1.1 FDD | N/A |
|  |  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  |  | 1 | CCR.1.1 FDD | N/A | CCR.1.1 FDD | N/A |
|  |  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  | 1, 2, 3 | OP.1 | OP.1 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  |  | 1 | TRS.1.1 FDD | N/A | TRS.1.1 FDD | N/A |
|  |  |  | 2 | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  |  | 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP configuration |  |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A | DLBWP.0.1 ULBWP.0.1 | N/A |
| PRS configuration |  |  | 1 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  |  | 2 | PRS.1.1 FR1 | PRS.1.1 FR1 | PRS.1.2 FR1 | PRS.1.2 FR1 |
|  |  |  | 3 | PRS.2.1 FR1 | PRS.2.1 FR1 | PRS.2.2 FR1 | PRS.2.2 FR1 |
| PRS Resource slot offset | slot |  | 1, 2, 3 | 0 | 4 | 0 | 4 |
| SRS configuration |  |  | 1 | POS-SRS.1 | N/A | POS-SRS.1 | N/A |
|  |  |  | 2 | POS-SRS.1 | N/A | POS-SRS.1 | N/A |
|  |  |  | 3 | POS-SRS.2 | N/A | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS |  | 1 | -98 |  | -98 |  |
|  |  |  | 2 | -98 |  | -98 |  |
|  |  |  | 3 | -95 |  | -95 |  |
| Note 2 | dBm/15 kHz |  | 1 | -98 |  | -98 |  |
|  |  |  | 2 |  |  |  |  |
|  |  |  | 3 |  |  |  |  |
| PRS | dB |  | 1 | -1.76 | -6.01 | -1.76 | -6.01 |
|  |  |  | 2 |  |  |  |  |
|  |  |  | 3 |  |  |  |  |
| PRS | dB |  | 1 | 0 | -3 | 0 | -3 |
|  |  |  | 2 |  |  |  |  |
|  |  |  | 3 |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz |  | 1 | -98 | -101 | -98 | -101 |
|  |  |  | 2 | -98 | -101 | -98 | -101 |
|  |  |  | 3 | -95 | -98 | -95 | -98 |
| Io | dBm/9.36 MHz |  | 1 | -66.07 | -66.07 | -66.07 | -66.07 |
|  | dBm/9.36 MHz |  | 2 | -66.07 | -66.07 | -66.07 | -66.07 |
|  | dBm/38.16 MHz |  | 3 | -59.97 | -59.97 | -59.97 | -59.97 |
| Propagation Condition |  |  | 1, 2, 3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.X2.Y3.Z2.3	Test requirements

The UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE state fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.25.2 for both Cell 1 and Cell 2.

<End of Change 1>