3GPP TSG-RAN4 Meeting #104e	R4-2211786

# Electronic Meeting, August 15 – August 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PDSCH 4Rx demod requirements for MU-MIMO IRC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-Perf |  |  |  |  |  | Date: |  |  | 2022-08-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has agreed to define requirements for PDSCH 4Rx demod in intra-cell inter-user interference |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced sections for requirements for 4Rx demod requirements for MU-MIMO IRC for FDD and TDD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements will not be defined for 4Rx demod requirements for MU-MIMO IRC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.1.16 (New) 5.2.3.2.17 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The changes with change track nemed ‘Endorsed changes’ have been endorsed in the last meeting big CR R4-2209828. Additional changes that removed the [] and add SNR requirement values, are made with change track name ‘RAN4#104e-CTC’. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

##### 5.2.3.1.16	Minimum requirements for PDSCH with intra-cell inter-user interference

The performance requirements are specified in Table 5.2.3.1.16-3 and Table 5.2.3.1.16-4, with the addition of test parameters in Table 5.2.3.1.16-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.16-1.

Table 5.2.3.1.16-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions, when the PDSCH transmission of target UE is interfered by co-scheduled UE. | 1-1, 2-1 |

Table 5.2.3.1.16-2: Test parameters

| Parameter |  | Unit | Target UE | Co-scheduled UE |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | {1000} for test 1-1 {1000, 1001} for test 2-1 | {1001} for test 1-1 {1002, 1003} for test 2-1 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for test 1-1 2 for test 2-1 | 1 for test 1-1 2 for test 2-1 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity | Single Panel Type I, Random precoder selection updated per slot and with PRB bundling granularity. Any column of precoder matrix is not equal to any column of precoder matriax of Target UE for test 1-1 Select the precoder to ensure orthogonality with the precoder for the target PDSCH for test 2-1 |
| MU-MIMO Beamforming Model |  |  | As specified in B.4.2 |  |
| Number of HARQ Processes |  |  | 4 | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 | N/A |

Table5.2.3.1.16-3: Minimum performance for target UE with Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | Random 16QAM symbols | TDLC300-100 | 2x4, ULA Low | 70 |  |

Table5.2.3.1.16-4: Minimum performance for target UE with Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.x-1.1 FDD | 10 / 15 | 16QAM, 0.48 | Random 16QAM symbols | TDLA30-10 | 4x4, ULA Low | 70 |  |

<End of change 1>

<Start of change 2>

##### 5.2.3.2.17	Minimum requirements for PDSCH with intra-cell inter-user interference

The performance requirements are specified in Table 5.2.3.2.17-3 and and Table 5.2.3.2.17-4, with the addition of test parameters in Table 5.2.3.2.17-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.17-1.

Table 5.2.3.2.17-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions, when the PDSCH transmission of target UE is interfered by co-scheduled UE. | 1-1, 2-1 |

Table 5.2.3.2.17-2: Test parameters

| Parameter |  | Unit | Target UE | Co-scheduled UE |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | {1000} for test 1-1 {1000, 1001} for test 2-1 | {1001} for test 1-1 {1002, 1003} for test 2-1 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for test 1-1 2 for test 2-1 | 1 for test 1-1 2 for test 2-1 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity | Single Panel Type I, Random precoder selection updated per slot and with PRB bundling granularity. Any column of precoder matrix is not equal to any column of precoder matriax of Target UE for test 1-1 Select the precoder to ensure orthogonality with the precoder for the target PDSCH for test 2-1 |
| MU-MIMO Beamforming Model |  |  | As specified in B.4.2 |  |
| Number of HARQ Processes |  |  | 8 | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 | N/A |

Table5.2.3.1.17-3: Minimum performance for target UE with Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | Random 16QAM symbols | FR1.30-1 | TDLC300-100 | 2x4, ULA Low | 70 |  |

Table 5.2.3.2.17-4: Minimum performance for target UE with Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.y-1.1 TDD | 40 / 30 | 16QAM, 0.48 | Random 16QAM symbols | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 |  |

<End of change 2>