3GPP TSG-RAN WG4 Meeting # 104-e	R4-2211807

Electronic Meeting, 15 – 26 August 2022

Agenda Item:	9.14.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on finalizing BS receiver requirements for extending current NR operation to 71 GHz

Document for:	Approval

# 1.	Introduction

The work item for extending current NR operation to 71 GHz was extended at TSG RAN#96 [1]. One of the incomplete tasks listed in the approved exception sheet are RF core requirements in 38.104.

This contribution provides further proposals on finalizing BS receiver requirements based on the agreed WF in RAN4#103-e [2].

# 2.	Discussion

2.1	Reference sensitivity and FRC

The following points were agreed in [2]:

- There is no need to define a new dedicated FRC for 960 kHz SCS and 800 MHz ChBW.

Table A.1-2: FRC parameters for FR2 OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity

| Reference channel | G-FR2-A1-1 | G-FR2-A1-2 | G-FR2-A1-3 | G-FR2-A1-4 | G-FR2-A1-5 | G-FR2-A1-6 | G-FR2-A1-7 | G-FR2-A1-8 | G-FR2-A1-9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 120 | 120 | 60 | 120 | 480 | 960 | 480 | 960 |
| Allocated resource blocks | 66 | 32 | 66 | 33 | 16 | 66 | 33 | 33 | 17 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 5632 | 2792 | 5632 | 2856 | 1416 | 5632 | 2856 | 2856 | 1480 |
| Transport block CRC (bits) | 24 | 16 | 24 | 16 | 16 | 24 | 16 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | - | - | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) (Note 3) | 5656 | 2808 | 5656 | 2872 | 1432 | 5656 | 2872 | 2872 | 1496 |
| Total number of bits per slot | 19008 | 9216 | 19008 | 9504 | 4608 | 19008 | 9504 | 9504 | 4896 |
| Total symbols per slot | 9504 | 4608 | 9504 | 4752 | 2304 | 9504 | 4752 | 4752 | 2448 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0 = 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size. NOTE 3: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |  |  |  |  |

Note that the payload size of G-FR2-A1-9 has been reduced from 1608 bits (in the agreed WF in RAN4#102-e [3]) to 1480 bits. We have performed simulation for the new FRCs G-FR2-A1-6 to G-FR2-A1-9 (with payload size of 1480 bits) together with the simulation assumptions recorded in TR 38.817-02 Annex B [4], and the simulation results are provided in Figure 1 below, where results for the existing FRCs G-FR2-A1-3 to G-FR2-A1-5 are also provided as references.

Figure 1: BLER Vs SNR for FRCs in FR2-2

It can be seen from Figure 1 that the required SNR to achieve 95% of maximum throughput (i.e., 5% BLER) are similar (within 1 dB) among the existing and new FRCs. Therefore, the scaling factor of 9 dB of EISREFSENS relevant for FR2-2 is justifiable.

The transmission bandwidth configuration for FR2-2 in the agreed WF in RAN4#103-e [5] are shown in Table 1 below.

Table 1: Transmission bandwidth configuration NRB for FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 264 | N/A | N/A | N/A |
| 480 | N/A | 66 | 124 | 248 | N/A |
| 960 | N/A | 33 | 62 | 124 | 148 |

It can be seen from Table 1 that the new FRC parameters for G-FR2-A1-6 to G-FR2-A1-9 are aligned with the corresponding transmission bandwidth configurations for 100 MHz and 400 MHz channel bandwidths in the agreed WF [5]. Therefore, there is no need to further update the new FRC parameters, and the reference sensitivity and FRC in the agreed WF [2] can be finalized.

Proposal 1: No need to further update the new FRC parameters, and the reference sensitivity and FRC in the agreed WF in R4-2210637 can be finalized.

2.2	ICS

The following points were agreed in [2]:

- Update ICS interfering signal RB for 800MHz, 1600MHz and 200MHz channel bandwidth to ensure both wanted and interfering signal RB are within channel bandwidth.

Table 10.9.3-1: OTA in-channel selectivity requirement for BS type 2-O

| Frequency Range | BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
| FR2-1 | 50 | 60 | G-FR2-A1-4 | EISREFSENS_50M + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 32 RB |
|  | 100,200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 64 RB |
|  | 50 | 120 | G-FR2-A1-5 | EISREFSENS_50M + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 16 RB |
|  | 100,200,400 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 32 RB |
| FR2-2 | 100,400 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 32 RB |
|  | 400 | 480 | G-FR2-A1-8 | EISREFSENS_50M + 9 + ΔFR2_REFSENS | EISREFSENS_50M + 19+ ΔFR2_REFSENS | DFT-s-OFDM NR signal, 480 kHz SCS, 32 RB |
|  | 800, 1600 | 480 | G-FR2-A1-6 | EISREFSENS_50M + 12 + ΔFR2_REFSENS | EISREFSENS_50M + 22 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 480 kHz SCS, [54] RB |
|  | 400 | 960 | G-FR2-A1-9 | EISREFSENS_50M + 9 + ΔFR2_REFSENS | EISREFSENS_50M + 19+ ΔFR2_REFSENS | DFT-s-OFDM NR signal, 960 kHz SCS, 16 RB |
|  | 800, 1600, 2000 | 960 | G-FR2-A1-7 | EISREFSENS_50M + 12 + ΔFR2_REFSENS | EISREFSENS_50M + 22+ ΔFR2_REFSENS | DFT-s-OFDM NR signal, 960 kHz SCS, [27] RB |
| NOTE 1: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. NOTE 2: EISREFSENS_50M is defined in clause 10.3.3. |  |  |  |  |  |  |

Note that the ICS interfering signal RB for 800MHz, 1600MHz and 200MHz channel bandwidth (54 and 27) are in square brackets because they are smaller than the nominal ones (64 and 32) to ensure both wanted and interfering signal RBs are within channel bandwidth. The interfering signal mean power can be calculated using the formula in the agreed WF in RAN4#101-bis-e [6]:

EISREFSENS_wanted = EISREFSENS_50M +10*log10(BWwanted/ BW50M)+( SNRwanted - SNR50M)+3

EISREFSENS_interfer = EISREFSENS_50M +10*log10(BWinterfer/ BW50M)+ ICS-0.9

Note: ICS in above equation = 14 dBc

It can be seem that the impact on the interfering signal mean power is 10log(64/54) = 10log(32/27) ≈ 0.74dB which should not make the ICS requirements considerably tighter. Therefore, the square brackets around the ICS interfering signal RB for 800MHz, 1600MHz and 200MHz channel bandwidth can be removed to finalize the ICS requirements. The corresponding draft CR to remove the square brackets around the ICS interfering signal RB for 800MHz, 1600MHz and 200MHz channel bandwidth is provided in [7].

Proposal 2: The square brackets around the ICS interfering signal RB for 800MHz, 1600MHz and 200MHz channel bandwidth can be removed to finalize the ICS requirements.

# 3.	Conclusion

This contribution has provided on finalizing BS receiver requirements based on the agreed WF in RAN4#103-e [2]. They are summarized as follows.

Proposal 1: No need to further update the new FRC parameters, and the reference sensitivity and FRC in the agreed WF in R4-2210637 can be finalized.

Proposal 2: The square brackets around the ICS interfering signal RB for 800MHz, 1600MHz and 200MHz channel bandwidth can be removed to finalize the ICS requirements.

The corresponding draft CR to remove the square brackets around the ICS interfering signal RB for 800MHz, 1600MHz and 200MHz channel bandwidth is provided in [7].

# References

[1]	RP-221783, “Exception request for Extending current NR operation to 71GHz”, Qualcomm CDMA

Technologies, Intel Corp.

[2]	R4-2210637, “WF on BS RF requirements for FR2-2”, Nokia, Nokia Shanghai Bell.

[3]	R4-2207217, “WF on >52.6 GHz BS RF Rx Requirements”, Ericsson.

[4]	3GPP TR 38.817-02 v15.9.0: “NR; General aspects for Base Station (BS) Radio Frequency (RF) for NR”.

[5]	R4-2210577, “WF on system parameters for NR extension to 71GHz”, Intel Corporation.

[6]	R4-2203017, “WF on BS RF Rx requirements”, Ericsson.

[7]	R4-2211808, “Draft CR to TS 38.104 on finalization of ICS interfering signal RB”, Nokia, Nokia Shanghai Bell.