3GPP TSG-RAN WG4 Meeting # 104-e	R4-2211831

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.18.5.1.1

Source:	Apple Inc.

Title:	On PDSCH/SDR Demodulation Requirements for Reduced Capability Devices in NR

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#103-e RedCap UE demodulation and CSI reporting requirements were discussed and a way forward [1] was agreed. In this contribution we present our views on open issues and provide the requested simulation results for PDSCH demodulation requirements for RedCap UE.

# 2. Discussion

In [1] the simulation assumptions for PDSCH demodulation requirements for RedCap UE were agreed. The scope of opens in PDSCH demodulation requirements are converging quickly.

Define 256QAM demodulation requirements for FR1

- Option 1: Define 256QAM demodulation requirements for 1Rx in FR1

  - Option 1a: MCS20

  - Option 1b: MCS is based on the operating SNR

- Option 2: Not to define 256QAM demodulation requirements for 1Rx in FR1

To reduce the simulation effort and eventually the testing burden, we don’t see the need of introducing requirements for 256QAM, which is an optional UE feature for RedCap.

Nonetheless, in RAN4#103e it was agreed to introduce this requirement to avoid concerns by network operators on actual achievable cell spectral efficiency with 256QAM RedCap UEs with 2Rx. Following the same rationale, we agree to extend this requirement also to 1Rx UEs.

To keep things simple, we support the straightforward selection of MCS20 since it corresponds to the lowest coderate in the 256QAM MCS table.

Proposal #1: Define an additional PDSCH demodulation requirement for 1Rx 256QAM RedCap UEs set at MCS20.

Simulation Results

For the agreed simulation parameters stated in [1], we contribute with our performance results for PDSCH demodulation. For clarity of exposition, we present the full table requested in [1] but omitting the results of those test cases that are reused from Rel.15.

FR1 FDD 15kHz

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference TS38.101-4 | SNR[dB] / Reuse | Impair Res. [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | 2.61 | 4.55 |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | 13.9 | 15.7 |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | 13.2 | 15.3 |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | Reuse | Reuse |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | 6.6 | 8.5 |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | Reuse | Reuse |
| 2Rx UE | 10MHz / 15kHz | 256QAM 0.82 Rank 1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-3 | Reuse | Reuse |

FR1 TDD 30kHz

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 | SNR[dB] / Reuse | Impair Res. [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz/ 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | 2.6 | 4.7 |
| 1Rx UE | 20MHz/ 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | 10.8 | 12.6 |
| 1Rx UE | 20MHz/ 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | 15.8 | 17.9 |
| 2Rx UE | 20MHz/ 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | -1.3 | 0.5 |
| 2Rx UE | 20MHz/ 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | 6.7 | 8.5 |
| 2Rx UE | 20MHz/ 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | 10.9 | 12.8 |
| 2Rx UE | 20MHz/ 30kHz | 256QAM 0.82 Rank 1 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-3 | 23.7 | 25.9 |

FR2 TDD 120kHz

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 | New / Reuse | Impair Res. [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-3 Test 1-1 | Reuse | Reuse |
| 2Rx UE | 100MHz / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-1 | TDLA30-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-2 | Reuse | Reuse |
| 2Rx UE | 100MHz / 120kHz | 64QAM 0.46 Rank 2 | FR2.120-2 | TDLA30-75 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-6 | Reuse | Reuse |

# 3. Conclusion

In this contribution we provide our position on the remaining open issues and we provide the requested simulation results for PDSCH demodulation requirements for RedCap UE. Our observations and proposals are captured below:

Proposal #1: Define an additional PDSCH demodulation requirement for 1Rx 256QAM RedCap UEs set at MCS20.

# Reference

- R4-2210672, “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.