3GPP TSG-RAN WG4 Meeting # 104-e	R4-2211867

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.11.7.3.1

Source:	Apple Inc.

Title:	Discussion on PDSCH demod requirements for NTN

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#103-e UE demodulation requirements for NTN were discussed and way forward [1] was agreed. In this contribution we present our views on the open issues and simulation assumptions for requirements.

# 2. Discussion

## General Aspects

In RAN4#103-e the following general aspects were agreed

Issue 1-2-1: Maximum Doppler shift due to UE motion - Do not explicitly specify the UE speed and to consider the Doppler shift that reflects a reasonable UE speed. Issue 1-2-2: Doppler shift due to satellite motion for DL in service link - Do not verify the UE compensation prior to the baseband processing. The maximum doppler shift is residual frequency offset, i.e., 0.1ppm - Under the assumption UE compensation functionality will be covered by other requirements i.e., RF requirements. - It’s FFS whether this already covered by other requirement (RF and or RRM); RAN4 can further discuss whether this need to be verified by demodulation requirements if RAN4 conclude it’s not covered by other requirements (RF and or RRM). Issue 1-2-4: Doppler shift due to satellite for UL in service link - Consider the UE pre-compensation for UL. The residual frequency shift of ±200Hz (i.e., 0.1ppm.) is assumed for UL in service link. Issue 1-2-5: Doppler shift for DL in feeder link - Do not consider Doppler shift for feeder link for DL Issue 1-2-6: Doppler shift for UL in feeder link - Do not consider Doppler shift for feeder link for UL Issue 1-3-1: Frequency drift - Do not consider the Frequency Drift by assuming UE compensation functionality covered by other requirements i.e., RF requirements (frequency error) Issue 1-3-2: Timing drift and sampling frequency offset - Option 1: Do not define sampling offset model - by assuming UE compensation functionality covered by other requirements i.e., RF requirements - It’s FFS whether this already covered by other requirement (RF and or RRM); RAN4 can further discuss whether this need to be verified by demodulation requirements if RAN4 conclude it’s not covered by other requirements (RF and or RRM). Issue 1-4-1: Depolarization loss between satellite and UE - Do not consider depolarization loss since it is not relevant to receiver algorithm.

The remaining open issues on general assumptions are:

Issue 1-1-1: Scenarios for NTN demodulation requirements

- Proposals

  - Option 1: Define the NTN demodulation requirement only based on the worst case of elevation angle in LEO600 deployment. The delay and Doppler configuration should be based on the agreed worst case.

  - Option 2: Consider the maximum delay spread of 100ns (Agreements from RAN4#102e). The doppler shift will depend on the outcome of Issue 1-2-2 – Issue 1-2-6

Issue 1-1-2: Elevation angle

- Proposals

  - Option 1: Consider the following elevation angles and the corresponding delay spread, k-factor, etc.,

    - Dense urban LOS scenario with elevation angle 20o, 50o and 90o

    - Urban NLOS scenario with elevation angle 20o, 50o and 90o

  - Option 2: Whether to consider the different elevation angles and corresponding k-factors depend on the outcome of Issue 1-2-2 – Issue 1-2-6

Issue 1-2-3: Doppler shift modelling

- Proposals

  - Option 1: TE should generate the channel model by TE simulated satellite ephemeris and TE GNSS without any extra UE reporting for the UE location during the test if option 2 in Issue 1-2-2 is agreed.

  - Option 2: Other option if any

Issue 1-5-1: Channel model parameter combination

- Proposals

  - Option 1: Take following channel parameter combination for NTN demodulation:

    - NTN-TDLX <DS>-<Fd>-<Doppler>

    - Where, NTN-TDLX represents the tap delay profile based on a certain TDL-X channel model with a certain satellite elevation angle, DS is the desired delay spread, Fd is frequency shift in both service link and feed link, and Doppler is the maximum Doppler shift caused by UE motion.

  - Option 2: other option if any

In RAN4#103-e we agreed that Doppler shift due to satellite motion for DL in service link, frequency drift, timing drift will not be modelled in demod requirements provided they are covered in other RF and/or RRM requirements.

For delay spread, the max value of delay spread of NTN scenarios was decided in RAN4#102-e as 100ns. There was a proposal to consider the worst-case elevation angle and resulting Doppler shift and delay spread. In our understanding if Doppler shift compensation, frequency drift, time drift compensation are done prior to baseband processing, we don’t need to introduce requirements considering the worst case elevation angle. We only need to introduce requirement with previously agreed max delay spread of 100ns and nominal Doppler shift to account for residual frequency compensation.

Proposal #1: Do not consider delay spread / Doppler shift due to worst case elevation angle since we have agreed to define requirements with assumption of compensation of Doppler shift, frequency drift and time drift prior to baseband processing.

Proposal #2: Define requirements with max delay spread of 100ns and nominal residual frequency offset.

With Doppler shift not modelled as part of the DL UE demodulation requirements, we don’t need to discuss Doppler shift modelling for demod requirements.

Observation #1: We don’t need to discuss Doppler shift modelling since it is no longer modelled as part of UE demod requirements for NTN.

For the channel model definition, we would need the desired delay spread and Doppler due to UE speed. Hence the channel model parameter combination can be similar to what we have in NR TDL channels defined in Rel-15. Define channel model parameter combination as NTN-TDLX <DS>-<Doppler>.

Proposal #3: Define channel model parameter combination as NTN-TDLX <DS>-<Doppler>.

## PDSCH Demodulation

The agreements for PDSCH demodulation requirements in [1] are:

Issue 2-1-2: Applicability rules for LEO requirements - If define the requirements for LEO only, the following applicability rules are aggregable. Issue 2-2-1: K_offset value - Select the K_offset value equal to or greater than twice the satellite-UE one-way delay - Further discuss the specific K_offset values based on the elevation angle. Issue 2-3-1: Modulation order - Further consider 64QAM as 2nd priority Issue 2-5-1: Antenna configuration - Consider SAN 1Tx-UE 2Rx for PDSCH demodulation

Issue 2-1-1: PDSCH requirements for GEO and LEO

- Proposals

  - Option 1: Define the requirements for LEO only

  - Option 2: Define the requirements for LEO and GEO separately if time-varying propagation delay is assumed

With respect to UE demod requirements the fundamental difference between GEO and LEO is the propagation delay and Doppler shift. With the assumption that Doppler shift / frequency drift are compensated prior to UE baseband processing, we don’t see any difference in UE processing for LEO and GEO. Hence, we don’t see the necessity to introduce different requirements for GEO and LEO. To reduce the number of test cases and requirements we propose to introduce requirements only for LEO. For different propagation delay, different K_offsets would need to be considered for LEO and GEO. To cover GEO and LEO with the same requirements, a large enough K_offset value should be chosen to cover both types. We propose to further discuss this approach.

Proposal #4: Define PDSCH demod requirements for LEO and discuss how requirements can be extended to GEO by defining large enough k-offset value. Do not define separate set of requirements for GEO.

Issue 2-4-1: SCS/CBW set

- Proposals

  - Option 1: Further consider 20MHz CBW for 30kHz SCS

  - Option 2: Do not consider new case for 20MHz CBW for 30kHz SCS

For defining requirements with additional SCS of 30KHz, we don’t see a strong necessity. The requirements in 38.101-4 already cover 30KHz SCS and that would be sufficient. We don’t see strong need to define requirements with 30KHz SCS with NTN channel model and condition.

Proposal #5: Do not define requirements with 30KHz SCS for NTN.

The operating SNR in NTN is expected to be low. Hence, we should target requirements with low MCS for UE demod.

Proposal #6: Target low MCS for NTN UE demod requirements for low SNR conditions.

We recommend considering the following test cases as starting point for UE demod requirements

| CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric |
| --- | --- | --- | --- | --- |
| 10MHz / 15kHz | MCS 4 Rank 1 | NTN-TDLA30-30 | 1x2 low | 70% of peak rate |
| 10MHz / 15kHz | 16QAM MCS 11 Rank 1 | NTN-TDLC100-10 | 1x2 low | 70% of peak rate |

# 3. Conclusion

In this paper, we provide our views NTN UE demod requirements. Our observations and proposals are captured below:

General aspects

Proposal #1: Do not consider delay spread / Doppler shift due to worst case elevation angle since we have agreed to define requirements with assumption of compensation of Doppler shift, frequency drift and time drift prior to baseband processing.

Proposal #2: Define requirements with max delay spread of 100ns and nominal residual frequency offset.

Observation #1: We don’t need to discuss Doppler shift modelling since it is no longer modelled as part of UE demod requirements for NTN.

Proposal #3: Define channel model parameter combination as NTN-TDLX <DS>-<Doppler>.

PDSCH Demod

Proposal #4: Define PDSCH demod requirements for LEO and discuss how requirements can be extended to GEO by defining large enough k-offset value. Do not define separate set of requirements for GEO.

Proposal #5: Do not define requirements with 30KHz SCS for NTN.

Proposal #6: Target low MCS for NTN UE demod requirements for low SNR conditions.

# Reference

- R4-2210661, “WF on NTN demodulation - general and PDSCH”, Qualcomm Incorporated.